3GPP TSG RAN WG1 #120	   	 	  	                	      R1-2500565

Athens, Greece, February 17th – 21st, 2025

Agenda item:	9.1.1

Source:	LG Electronics

Title:	Discussions on AI/ML for beam management

Document for:	Discussion and Decision

Introduction

In the new WID on AI/ML for NR air interface [1], objectives for beam management were listed as below.

- Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

In this contribution, we discuss enhancements on AI/ML for beam management. Specifically, we provide our view on specification support for spatial domain beam prediction, temporal domain beam prediction, LCM operations for beam management, and consistency between training and inference.

Discussion

Data collection

NW-sided AI/ML model

For NW-sided AI/ML model, UE measurement/reporting should be accompanied for data collection for training/inference/monitoring. In this section, we discuss specification support regarding data collection for NW-sided AI/ML in case of spatial domain DL Tx beam prediction and temporal DL Tx beam prediction.

FL summary in RAN1 #119 ##### For #120 (onhold) Proposal 6.1: For NW-sided model, for BM-Case 2, support to report the measurement results of multiple transmission occasions of resources for channel measurement at least for the RS types including P CSI-RS, SP CSI-RS and SSB in one L1 report. - Differential L1-RSRP reporting over multiple time instances is used - FFS on how to determine the multiple occasion(s) - FFS whether include the time stamp information explicitly or implicitly in the report - FFS on the max number of RS resources to be reported for one transmission occasion - FFS on the max number of the multiple transmission occasions. - Note: Purpose, such as above “For NW-sided model, for BM-Case 2”, will not be specified in RAN 1 specifications

For NW-sided AI/ML models, NW can use its own measurement, e.g., via receiving SRS, as training/validation/testing dataset for AI/ML model. In addition, beam related UE reporting quantity can be used as input parameter for the AI/ML model. Especially for temporal DL Tx beam prediction(BM-case 2), for example, legacy beam reporting and/or CSI reporting can be used as input or assistance information for NW-sided AI/ML model to predict future DL beams. Also, positioning related UE reporting can also be used for DL beam prediction, since UE position is highly related to DL beam to be served for the UE. Additional UE reporting can also be considered for improving the DL Tx beam prediction performance. For example, UE data attained from sensors such as velocity, orientation, or rotation can be formatted to a non-proprietary information and reported to NW so that it can be used as input for the NW-side AI/ML model. Also, more refined or detailed beam reporting, e.g., tendency/variance of best N beam(s) and/or past/present best N beam(s) per time stamp, can be considered as discussed in previous meetings, which can also be used as input data for NW-side AI/ML model.

In several previous meetings, enhancement on beam report for NW-sided AI/ML model especially in case of BM-case 2 had been discussed. At least for inference in case of BM-case 2, beam report for multiple measurement instances can be introduced wherein multiple measurement samples would be needed for NW-sided AI/ML model input. The benefit of reporting multiple measurement instances is to prevent frequent beam report when the report includes only a single measurement instance, which brings lower UL resource occupation. Furthermore, when it comes to SP/AP CSI reporting for dynamic measurement/reporting, report of multiple measurement instances can be efficiently used by NW as demand basis if NW is about to infer NW-sided AI/ML model, instead of periodic measurement/reporting.

Proposal #1: For NW-sided AI/ML in temporal DL Tx beam prediction, support the following UE reporting enhancements for data collection:

- Past/present best N beam(s) per time stamp

- Tendency/variance of best N beam(s)

UE-sided AI/ML model

For UE-sided AI/ML model, beam measurement for Set A can be configured for UE to collect training data, and beam measurement/report for Set B can be configured for UE to collect input data for inference. In this section, we discuss specification support regarding data collection for UE-sided AI/ML in case of spatial domain DL Tx beam prediction. For the case of temporal DL Tx beam prediction, multiple instance of Set A/B beam measurement can be used for training/inference data collection.

FL summary in RAN1 #117 ##### (low)Proposal 4.2A: Support UE to request for RS resources of Set A, FFS on details

In RAN1 #117 meeting, there was a discussion on UE request for (preferred) RS resources of Set A but we couldn’t have any conclusion due to lack of time. Regarding the data collection, a UE can report the supported size of Set A/B via UE capability. In addition, if assistance information on DL RS is provided to the UE, the UE can select/report preferred Set A among a candidate RS set for Set A to NW in order to maintain UE complexity for prediction and feedback overhead within a reasonable level. Assuming that potential beams for Set A are measurable with a periodicity much longer than that of Set B beams, UE assistance/reporting for determining Set A can be considered.

Proposal #2: Support reporting of UE assistance information for determining Set A, e.g., UE to report preferred Set A among candidate beams of Set A.

For efficient beam measurement/reporting procedure for data collection, it is also considerable to extend Rel-18 NES CSI reporting framework into BM. In Rel-18 NES, a CSI-ReportConfig can contain a list of sub-configurations, UE can report CSI regarding one or more sub-configurations for the CSI-ReportConfig. Currently, beam reporting is not supported for this sub-configuration based CSI reporting but for flexible beam report on varying Set B, it is considerable to extend this framework to BM. For this, each sub-configuration, i.e., Set B, may correspond to different Set A on which UE needs to predict beam(s). In addition, extension of Rel-18 NES CSI framework into BM can also achieve variable Set B configuration and/or Set B pattern configuration, for example, different sub-configurations can be associated with different subset of Set B wherein UE can report the measurement of configured/preferred subset of Set B. In this case, specification effort to support variable Set B patterns for a specific Set B can be very small.

Proposal #3: Consider extending sub-configuration based Rel-18 NES mechanism for Set B beam measurement and reporting.

- Different Set A and/or Set B may be associated with each sub-configuration

- Different Set B patterns for a specific Set B may be associated with each sub-configuration

Inference

NW-sided AI/ML model

For NW-sided AI/ML model, Set B beams should be measured/reported by UE for inference. In this section, we discuss specification support regarding inference for NW-sided AI/ML in case of spatial domain DL Tx beam prediction and temporal domain DL Tx beam prediction.

Agreement For NW-sided model, for inference, in a beam report initiated by network, based on one measurement resource set, support the report of more than 4 beam related information in L1 signaling - Note: Purpose, such as above “For NW-sided model, for inference”, will not be specified in RAN 1 specifications - FFS on the report content for beam related information - FFS on max number of reported beam related information in one report Agreement For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signaling, support - L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB - If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set - FFS: L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, X and M are configured by gNB, and whether/how to report number of reported beams - FFS on the maximum value of M (where M can be larger than 4) based on UE capability (M may or may not be different for different reporting contents) - FFS on beam information - Note: Purpose, such as above “For NW-sided model, for inference report, at least for BM-Case 1”, will not be specified in RAN 1 specifications

In RAN1 #116 meeting, it was agreed to support L1 reporting for more than 4 beam related information for NW-sided model.  In RAN1 #117 meeting, it was agreed to support the contents of more than 4 beam related information report which includes L1-RSRPs and corresponding beam information of Top M beam(s). Especially in case of M = the size of the measurement resource set(e.g., Set B) for the measurement report, beam index can be omitted except for Top 1 beam.

There are currently several FFS points regarding the details on more than 4 beam related information report. For the first FFS on additional condition of the measurement report(i.e., report up to M beams within X dB gap to the largest measured value of L1-RSRP), there should be an additional UE report on the number of reported beams which can be varying up to the value of X. It only consumes reporting payload overhead and increases NW side decoding complexity due to two part encoding, so we don’t think that the additional condition is needed. For the second FFS point, we agree that the maximum value of M can be based on UE capability and NW configures M value for the more than 4 beam related information report. Regarding the last FFS point, there are several alternatives on how to report beam information on the L1 reporting for more than 4 beam related information. For instance, it had been discussed for UE to report beam pattern/group ID or bitmap for beam ID information [2][3], where reported beam IDs can also be implicitly indicated by beam pattern/group ID or bitmap for beam ID information which represents specific subset of Set B beams. In our view, the main purpose of these options is to report preferable subset of Set B from UE side with less overhead compared to legacy CRI/SSBRI report mechanism. However, similar functionality can be achieved by sub-configuration based approach, which is simple extension of Rel-18 NES CSI reporting framework into BM as discussed in the previous section 2.1.2. In this approach, we can still use legacy beam reporting framework. In addition, more than 4 beam related L1 reporting can be used not only for inference but also for other purposes, e.g., performance monitoring. Therefore, explicit indication of beam ID could be beneficial in order to support flexible usage of the report. Lastly, more than 4 beam related L1 reporting may also be exploited for legacy non-AI/ML beam management, such that it has benefit to align the report content with legacy beam reporting for NW to compare best DL Tx beams with corresponding L1-RSRP values. So, we propose to support CRI/SSBRI + L1-RSRP for the report content in more than 4 beam related L1 reporting, for NW-sided AI/ML model.

Proposal #4. Regarding the report of more than 4 beam related information in L1 signaling for NW-sided model, support CRI/SSBRI for beam information as legacy, e.g., CRI/SSBRI(s) of Top M beam(s) and the corresponding M L1-RSRP(s) are reported.

For NW-sided AI/ML models, spatial domain predicted beam on Set A can be found from inference output, and this beam can be used as gNB Tx beam for later transmission. In RAN1 #116, it was agreed that beam indication is based on unified TCI state framework as captured below.

Agreement - For NW-sided model and for UE-sided model, beam indication is based on unified TCI state framework - FFS on whether/how potential enhancement is needed

To align UE Rx beam for the newly found gNB Tx beam based on AI/ML prediction model, gNB needs to inform UE about the Tx beam, i.e., via TCI indication for UE to determine its Rx beam. In case of ‘beam indication of beams in Set A not in Set B’, UE can measure and maintain its Rx beam only for Set B beams, and it is practically not possible for all the Set A beams. Of course, NW may ensure transmission of non-outdated Set A beams for the UE, e.g., aperiodically for UE to find Rx beam for it but this approach requires extra RS overhead for transmitting Set A beams and extra delay/latency on beam application. Note that in RAN4 specification, different TCI application delay values are defined for known TCI and unknown TCI, respectively. For unknown TCI, it was RAN4’s assumption that there should be aperiodic beam RS transmission for P-3 (Rx beam refinement) before the TCI application, resulting in much larger delay value for the unknown TCI. Thus, it is much better to perform beam indication based on Set B beams, not Set A beams. Since one beam from Set A can be in a middle point of multiple Set B beams from boresight beam angle perspective, we may consider indicating the multiple neighboring beam IDs from Set B for indicating a beam from Set A for helping the UE to find an Rx beam for the Set A beam. How to find the Rx beam would be up to UE implementation e.g., via a composite/combined Rx beam for the resources or selecting an Rx beam from known optimal Rx beam for each of the resource.

Observation #1: In case of ‘beam indication of beams in Set A not in Set B’, it is practically not possible for UE to measure and maintain its Rx beam for all the Set A beams. So, for beam indication, TCI/QCL RS should be based on Set B beams of which UE can measure and maintain its Rx beam.

Proposal #5: In order to indicate one beam in Set A not in Set B, support indicating multiple neighboring beams from Set B for helping UE to find its Rx beam for the Set A beam.

For inference in case of temporal DL Tx beam prediction, NW can indicate temporal predicted beam to UE as a form of TCI indication. For higher efficiency, multi-TCI indication can be considered where each TCI corresponds to different time duration. In addition, future TCI indication with broadcast/multicast signaling can also be considered to reduce beam indication signaling overhead further.

UE-sided AI/ML model

For UE-sided AI/ML model, UE can be configured to measure Set B beams and report predicted Set A beams. In this section, we discuss specification support regarding inference for UE-sided AI/ML in case of spatial domain DL Tx beam prediction and temporal domain DL Tx beam prediction.

Agreement For UE-sided model at least for BM Case-1, CSI-ReportConfig is used for the configuration of inference results reporting - FFS on the details in the CSI-ReportConfig, at least considering: - Alt 1: one CSI-ResourceConfigId is configured for Set B - FFS: how UE can determine the information about set A - Alt 2: one CSI-ResourceConfigId is configured for both Set A and Set B - FFS: How to configure resource set(s) for Set A and Set B in CSI-ResourceConfig - Alt 3: two CSI-ResourceConfigId s are configured for Set A and Set B separately - Alt 4: one CSI-ResourceConfigId is configured for Set B, Set A is configured using separate resource set(s) other than that represented by CSI-ResourceConfigId - FFS: how to configure/indicate separate resource set(s) for Set A - Note: separate CSI-ReportConfig for Set A and Set B are not precluded. - Note: Not perform measurement for Set A and only perform measurement for Set B subject to the CSI-ReportConfig - FFS on the association between Set A and Set B with or without additional IE - Other necessary configuration are not precluded. Agreement For UE-sided model at least for BM Case-1, for inference results report - Two resource sets can be configured for Set A and Set B separately in the CSI report configuration for the report - FFS whether support only resource set for Set B is configured - UE performs measurement on the resource set for Set B for inference, and UE is not expected to measure resource set for Set A for inference, - The beam information in the inference report refers to the resource set for Set A Agreement For both BM-Case 1 and BM-Case 2, for UE-sided model for inference, when Set A and Set B are configured within CSI report configuration, - Two CSI-ResourceConfigId s are configured for Set A and Set B separately

In RAN1 #116bis meeting, 4 alternatives were listed for CSI report/resource configuration to report UE-sided AI/ML inference result. And, in RAN1 #119bis meeting, it was agreed to support that two CSI-ResourceConfigIds are configured for Set A and Set B separately in the CSI report configuration for the inference result report. Meanwhile, the Alt 1 does not require any bit for Set A configuration for inference result report, which can achieve huge signaling overhead reduction. So, we think that Alt 1 should be supported with high priority to achieve overhead reduction from the perspective of resource configuration for inference result report.

For UE-sided AI/ML models, it should be noted that the DL Tx beams applied to DL resources are totally transparent to UE by current specification. In the case that Set B is a subset of Set A, for example, UE has no idea about the beams included in Set A but not in Set B since there may be no recent measurement on those beams and no additional information from NW neither. According to the conclusion made in RAN1#112, on the other hand, providing explicit information about NW-side beam shape to UE was failed to achieve consensus due to the issues on vendor-specific proprietary information. In order to meet these competing demands, we can consider approaches where information of Set A/Set B beams are represented in a logical domain/space which may provide ‘relative’ information between beams. For an example, NW may provide 2D/3D coordinate information about Tx beam boresight direction, e.g. (x1, y1, z1) for RS#a, (x2, y2, z2) for RS#b, where the coordinate axes may be decided by NW implementation (i.e., not necessarily same as global coordinate axes) or may be defined in a logical space so that it does not provide ‘physical’ implementation of beams, rather it is about ‘relative’ Tx beam angle difference of RSs comprising Set A and/or Set B. For another example, Set A beams can be represented as a linear combination of Set B beams, analogous to type-II CSI codebook.

Proposal #6: For UE-sided model, support to configure only resource set for Set B for CSI-ResourceConfig which is used for the configuration of inference results reporting (i.e., Alt 1), considering huge resource overhead reduction obtained from not configuring Set A.

Proposal #7: In order to support only resource set for Set B for CSI-ResourceConfig (Alt 1) for Set A and Set B configuration, assistance information on relation/association between Set A beams and Set B beams should be provided to UE for the UE-side AI/ML model training and inference. To represent beams in Set A and/or Set B while preserving sensitive proprietary information, consider following exemplary methods:

- Set A beams are represented by linear combining coefficients of Set B beams

- Tx beam directions are represented as ordered numbers on a 2D or 3D coordinate

Agreement For UE-sided model, at least for BM-Case1, for content in the report of inference results, support - Opt 1: Beam information on predicted Top K beam(s) among a set of beams - Opt 2: Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams - At least K=1 and more, FFS on max value - FFS on beam information - FFS on the definition of predicted Top K beam(s) - FFS on definition of reported RSRP when applicable - FFS on other information in the report with potential down selection among the following options - Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams - FFS on the quantization method of probability information - Probability information is the probability of the beam to be the Top 1 or Top K beam - Opt 4: Beam information on predicted Top K beam(s) among a set of beams, RSRP of predicted Top K beam(s) among a set of beams, and confidence information of the RSRP - FFS on definition of reported RSRP - FFS on the definition and quantization method of confidence information - Other options are not precluded. where the set of beams is Set A, i.e., the beams for UE prediction.

In RAN1 #116 meeting, two options were agreed to support predicted beam reporting from UE-sided AI/ML. Option 1 is to report beam information on predicted Top K beam(s) among Set A beams and Option 2 is to report beam information on predicted Top K beam(s) among Set A beams and corresponding RSRP values. Option 1 is suited design for AI/ML classification model, so that the definition of predicted Top K beam(s) in Option 1 can be “the beam(s) classified as Top K beam(s) in Set A”. On the other hand, Option 2 is mainly for AI/ML regression model so the definition of predicted Top K beam(s) in case of Option 2 can be “the beam(s) corresponding to the K-th highest predicted RSRP value in Set A” and naturally the definition of reported RSRP value can be “the predicted RSRP value”. Predicted RSRP value is useful for NW to compare beam quality of multiple UEs and/or multiple beams of a same UE. The max number of K can be 4 as legacy beam reporting, in order to have sufficient flexibility for NW to choose optimal predicted Set A beam for beam indication in consideration of intra/inter-cell interference circumstance. Lastly, regarding beam information, considering the case that predicted beam is in Set A but not in Set B, predicted Set A beams can be represented by linear combining coefficients of Set B beams or represented as ordered numbers on a 2D or 3D coordinate if Set A is not explicitly configured to UE. If it is configured to UE, relation/association information between Set A beams and Set B beams should also be provided to UE.

In addition to above two options, option 3 and option 4 need further discussion. For the predicted beam report with RSRP, one challenge would be how NW can rely on the reported beam ID and/or RSRP value. This issue could be addressed by option 3 and option 4 if UE reports confidence/probability information about the predicted Set A beam(s). It is also beneficial for temporal DL Tx beam prediction, since NW can choose future predicted beam with higher confidence/probability, for beam indication. For this information, it needs to be clarified whether the confidence/probability is calculated per model/functionality, per report, per time instance, or per report parameter (e.g., predicted beam ID, predicted RSRP).

Proposal #8: For supported Option 1 and Option 2, support K=4 for the max value.

- Considering the case that predicted beam is in Set A but not in Set B, reported beam information can be based on the relation/association between Set A beams and Set B beams

Proposal #9: For predicted RSRP report, confidence/probability information may be helpful for NW to decide whether/how to use the reported RSRP. Further study whether the information is per model/functionality, per report, per time instance, or per report parameter.

Agreement For UE-side AI/ML model inference, for BM-Case2, support to report inference results of N(N>=1, FFS on N) future time instance(s) in one report - wherein information of inference results of one time instance is as in one report for BM-Case 1 - Note: overhead reduction is not precluded - FFS on details Agreement For UE-sided model, for the quantization of a RSRP value at least for the report of inference results, support - Support differential RSRP reporting with legacy quantization step and range for L1-RSRP reporting - For BM-Case 1, support differential RSRP report among multiple beams - For BM-Case 2, support differential RSRP report among multiple beams over multiple time instances - FFS details Agreement For UE-sided model for BM-Case 2, for inference results report, support to configure UE with N future time instance(s) for inference by NW when applicable - FFS: how to determinate reference time for the time instance(s) - FFS: duration values of the N time instance(s) that can be predicted. Agreement For BM-Case 2 of UE-side model, for the reference time of the earliest time instance for the predicted results, consider at least the following alternatives for potential down-selection: - Option 1: Based on the uplink slot for the report - Option 2: Based on the CSI reference resource corresponding to the report - Option 3: Based on the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report, wherein the transmission occasion is no later than the CSI reference resource Agreement For UE-sided model, at least for BM-Case 1, the beam information in inference result report is CRI/SSBRI of resource in Set A.

In RAN1 #116bis meeting, it was agreed to support reporting of inference results of N(N>=1, FFS on N) future time instance(s) in one report, for temporal DL Tx beam prediction. In our view, N value more than 1 can reduce PUCCH resource overhead and reporting overhead from UE side. However, in order to report inference results of multiple future time instances in one report, overhead reduction for UCI payload size should be considered. For this, enhancement on RSRP quantization can achieve overhead reduction to express beam IDs + RSRPs for multiple time instances. For example, the only best predicted RSRP across every multiple time instances can be expressed by absolute RSRP value (e.g., 7 bit) and the others can be expressed by differential RSRP value (e.g., 4 bit), rather than having the best predicted RSRP as absolute RSRP value per each future time instance. In order to achieve this, an indicator for the time instance containing the largest predicted RSRP in addition to the best beam indicator should be included in inference result report for BM-case 2. Furthermore, In this case, information on time-variation of predicted RSRP can also be considered to be included in the report for helping intra-/extra-polation at NW side as well as for UCI overhead reduction.

In RAN1 #118bis meeting, there was a discussion regarding the reference time to determine the earliest time instance for the predicted results. For this, we believe that re-using Rel-18 MIMO CSI prediction framework seems sufficient. In the current TS 38.214[5], the earliest slot for CSI prediction is specified as captured below. Similarly, for BM-case 2, the reference time to determine the earliest time instance for beam prediction can be the uplink slot for the report and slot offset value can be configured by NW to determine the earliest time instance, wherein candidate value of slot offset also includes the value to indicate CSI reference resource as Rel-18 MIMO.

TS38.214 S5.2.1.4.2 Subject to UE capability, a UE configured with a CSI-ReportConfig with the higher layer parameter N4 and reportQuantity set to 'cri-RI-PMI-CQI' is assumed to support UE-side CSI prediction. The reported PMI indicates predicted precoder matrices associated with N4 consecutive slot intervals, each with duration of d slots, where the value of N4∈{1,2,4,8} is configured by higher layer parameter N4. If the UE is configured with an aperiodic CSI-RS resource set for channel measurement, the value, in number of slots, of the time unit d∈{1,m} is configured by higher layer parameter d, where m is defined in Clause 5.2.1.4.1. If the UE is configured with a periodic or semi-persistent CSI-RS resource set for channel measurement, the value of d is equal to the periodicity of the CSI-RS resource. The earliest of the N4 slot intervals starts at slot l=n+δ, where n is the uplink slot in which the CSI is reported and the slot offset δ∈{-nCSI_ref,0,1,2} is configured by higher layer parameter delta, where nCSI_ref defined in Clause 5.2.2.5 and the value δ=-nCSI_ref can be configured subject to UE capability.

Proposal #10: Support to report inference results of N(N>=1) future time instance(s) in one report.

- Maximum value of N can be more than 1

- Maximum value of N can be reported by UE capability, and M(M<=N) value for inference results reporting can be configured by gNB

Proposal #11: For BM-Case 2, support differential RSRP report among multiple beams over multiple time instances.

- In the inference result report, an indicator for the time instance containing the largest predicted RSRP in addition to the best beam indicator is reported by UE

- Other than largest predicted RSRP value across every multiple time instances, predicted RSRP value for each beam is expressed by differential RSRP value (e.g., 4 bit)

Proposal #12: Regarding the reference time to determine the earliest time instance for the predicted results in BM-case 2, support to re-use Rel-18 MIMO CSI prediction framework.

- The reference time is based on the uplink slot for the report and slot offset value can be configured by NW to determine the earliest time instance, wherein candidate value of slot offset also includes the value to indicate CSI reference resource, e.g., δ=-nCSI_ref

Proposal #13: For temporal DL Tx beam prediction, information on time-variation of RSRP can also be included in the report.

Proposal #14: For temporal DL Tx beam prediction with UE-sided models, following beam reporting enhancements can be considered:

- Report of beam dwelling time for each future time instance

- Report of beam(s) for current time instance for fallback operation

Agreement For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, when applicable, further study the following options: - Option A: Predicted RSRP - Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement - Where the predicted RSRP is based on AI/ML output - Note: Support both Option A and Option B is not precluded. Agreement Following Working Assumption is confirmed. Working Assumption For report content of inference results for UE-sided model for BM-Case 2, the RSRP of predicted beam(s) in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output.

In previous meetings, there had been a discussion on how to report the RSRP of predicted Top K beam(s), and it was agreed to support predicted RSRP for the RSRP of predicted Top K beam(s) in case of BM-case 2, in RAN1 #117 meeting. With regard to Option A and Option B listed for the RSRP of predicted Top K beam(s) in case of BM-case 1, we think that predicted RSRP and measured L1-RSRP shall be the same or very similar up to specific regression algorithm so that it is not critical for NW to distinguish these two. So, Option A, which is simpler than Option B, is preferred. Alternatively, we may not specify this case, i.e., leave this case up to UE implementation.

Proposal #15: Support Option A or Option C (new) for the RSRP of predicted Top K beam(s) in the report of inference results.

- Option A: Predicted RSRP

- Option C: Not specify whether to report predicted RSRP or measured L1-RSRP when both RSRPs are available at UE side, i.e., leave this case up to UE implementation

Agreement For UE-side AI/ML model, for BM-Case1, at least for inference, at least for Set B, support the following CSI-RS resource types for CMR: - Periodic (P) CSI-RS - Semi-persistent (SP) CSI-RS - Aperiodic (AP) CSI-RS For UE-side AI/ML model, for BM-Case 2, at least for inference, at least for Set B, support the following CSI-RS resource types for CMR: - Periodic (P) CSI-RS - Semi-persistent (SP) CSI-RS - FFS: Aperiodic (AP) CSI-RS Note: above CSI-RS resource refers to that used for beam management.

In RAN1 #118bis meeting, we had agreement on supported time domain behavior of the CSI-RS resource for Set B, at least for inference, for UE-side AI/ML model. Specifically, all of P/SP/AP CSI-RS is supported for the CSI-RS resource of Set B in case of BM-case 1, while support of AP CSI-RS as CMR for BM-case 2 is a pending issue. We think that AP CSI-RS is not appropriate in this case since BM-case 2 requires multiple transmission with a periodicity/interval as input data for UE-side AI/ML model inference thus it requires quite a huge DL resource overhead for NW to transmit a set of beam RSs multiple times in a short period. The burst AP CSI-RS introduced in Rel-18 MIMO CSI prediction may be considered for this case, however, the feature does not support multi-beam RS transmission thus quite a big specification enhancement is needed to support the burst AP CSI-RS for this case. For example, the number of resources, K∈4,8,12 CSI-RS resources for Rel-18, needs to be increased with considering both multi-beam transmission and the same beam repetitions in different slots. In addition, it needs to be considered how to support a large number of beams in one or multiple slots, e.g., more than 14 beams in two consecutive slots, and how to support the repetition of the set of beams with a periodicity/interval. Comparing its spec impact, practical benefit of AP CSI-RS is doubtful as it requires quite a big DL resource overhead for NW, which is only for a predicted beam report.

Regarding the supported time domain behavior of the CSI-RS resources for Set A for inference, we should note that Set A beams will be measured by UE very rarely compared to Set B thus the actual measurements will be easily outdated. Thus, it will be very typical assumption that RSs for Set A is configured with a long periodicity, i.e., periodic CSI-RS. If Set A is triggered dynamically with SP/AP CSI-RS, the whole motivation of ‘spatial-domain prediction’ becomes weaker as the motivation is to ‘reduce’ DL RS overhead for beam measurement.

Proposal #16: For UE-side AI/ML model, for BM-case 2, do not support AP CSI-RS for set B configuration for inference.

Proposal #17: For UE-side AI/ML model, for BM-case 2, at least for inference, support only P CSI-RS for Set A configuration.

Agreement For UE-side model, existing CPU mechanism is used as a starting point for AI/ML-based CSI processing. - FFS whether the overall CPU should be shared or separately counted between legacy CSI reporting and AI/ML-based CSI reporting, and among AI/ML features/functionalities. - FFS whether it is fully applicable for BM-Case 1 and/or BM-Case 2

In RAN1 #118bis meeting, it was agreed that existing CPU mechanism is used as a starting point for AI/ML-based CSI processing. We think that general framework of existing CPU mechanism including CPU occupancy, timeline, and priority rule can be inherited for UE-side AI/ML model inference. However, typical implementation of AI/ML algorithms is on hardware processer, i.e., GPU, which is different from typical CSI calculation processor. In this regard, it is desirable to have separate mechanism to manage AI/ML related processes from UE side, i.e., AI/ML processing units(APU). This APU can include not only BM cases but also the other use cases of CSI prediction and positioning for unified management of AI/ML related processes.

The APU management criteria can be similar to the legacy CPU management criteria. If a UE supports NAPU simultaneous AI/ML inference operations, it can be said that the UE has NAPU AI/ML processing units for inference reports. The required APU(s) can be different for different functionalities or (sub-)use-cases according to AI/ML model implementation at UE-side, thus this information needs to be reported to NW. For the same (sub-)use-case, the value of APU occupancy for AI/ML model inference can be different according to input/output data size, algorithm type/structure, etc. from different UE vendors.

Proposal #18: For UE-side AI/ML operations, support to introduce AI/ML processing units(APU) for CSI reports for inference result report.

- APU is occupied for all AI/ML inference operations at UE-side, and separately managed from CPU

- Further study on APU capability, e.g., supported maximum APUs per UE, required APUs per functionality/sub-use-case.

AI/ML Model performance monitoring

NW-sided AI/ML model

When it comes to NW-sided AI/ML model, NW needs to compare predicted Set A beams and actual Set A beams for AI/ML model monitoring. This can be achieved via UE reporting of actual Set A beam measurement(s). NW can maintain performance of AI/ML model by model switching/activation/deactivation/fallback from its own determination and UE-transparently.

UE-sided AI/ML model

In the RAN1 #117 meeting, it was agreed to support Type 1 performance monitoring (i.e., NW-side model monitoring) for a UE-sided AI/ML model, as captured below.

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including: - Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring - Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring - Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring - Note: resources for Set B for monitoring are not precluded and can be study. - Note: this is only applicable when the model can predict RSRP - Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam - Note: this is only applicable when the model can generate probability information - FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference. - For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window) Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), - At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation - FFS: on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric - FFS other alternatives Agreement At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), consider the following options with potential down selection for the configuration for monitoring: - Option 1: The resource set(s) for monitoring and report configuration for monitoring are configured (when applicable) within CSI report configuration used for inference - FFS: the resource set(s) for monitoring - UE measures the resource set(s) for monitoring. - FFS how/when to report the monitoring results. - Option 2: Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The dedicated report configuration used for monitoring links to an inference report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - UE measures the resource set(s) for monitoring. - FFS when to report the monitoring results. Agreement At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling: - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report - FFS on the timing related issues - UE measures the dedicated resource set(s) for monitoring.

For Type 1 performance monitoring, both Option 1 (NW-side performance monitoring) and Option 2 (UE-assisted performance monitoring) are agreed to support. In our view, Option 1 can be up to NW implementation and may have no specification impact, which means that NW can configure for UE to report measurement results from CSI resource set for performance monitoring in order to calculate performance metric at NW side. In this case, there is no need to report other contents other than L1-RSRP and/or RS index since more than 4 beam related beam report can be exploited for performance monitoring purpose.

Observation #2: For Option 1 (NW-side performance monitoring) of Type 1 performance monitoring, there may be no specification impact, e.g., NW can exploit report of more than 4 beam related information in L1 signaling for performance monitoring purpose.

On the other hand, performance metric should be discussed firstly for Option 2(UE-assisted performance monitoring). In RAN1 #118bis meeting, it was agreed to support Alt 1: Top 1 or Top K beam prediction accuracy. We think that Alt 1 can be the unified solution because it can be applied to any of AI/ML inference algorithm (e.g., classification model, regression model) for beam prediction. Remaining issues are the definition of performance metric with Alt 1 and configuration details on the CSI report configuration for monitoring result report and the resource set for monitoring, which can be jointly discussed.

Definition of performance metric with Alt 1: Top 1 or Top K beam prediction accuracy should be firstly discussed and specified. In our view, performance metric report for every single time when resource set(s) for monitoring is transmitted is too instant, so performance metric could be calculated with multiple Set A transmission/reception in order to obtain sufficient amount of samples. For this, periodicity of performance metric report can be associated to multiple transmissions of resource set(s) for monitoring, and the accuracy metric can be calculated based on comparison between multiple inference instances and multiple Set A transmissions. To be more specific, potential solution is to introduce counter for success/failure of inference within a specific time window which corresponds to the periodicity of performance metric report. When it comes to BM-case 2, it should be further investigated how to measure/calculate performance metric for multiple prediction instances.

In the last meeting, it was agreed to support dedicated resource set(s) for monitoring and dedicated CSI report configuration for monitoring. There are several remaining issues to be resolved with performance monitoring related configurations, we provide our views on the following issues; details on the dedicated resource set(s) for monitoring, linkage between an inference result report and a monitoring result report, and combination on time domain behavior of the reportConfigType for inference report and the reportConfigType for monitoring report. Firstly, regarding the configuration of the dedicated resource set(s) for monitoring, support to configure full set A for the dedicated resource set for monitoring. It seems that there is no clear benefit to configure subset of Set A or resource set for monitoring which is not Set A itself, because additional information/assumption is needed to figure out relationship between resource set for monitoring and exact Set A, which causes unnecessary signaling overhead in addition to the dedicated resource set its own. So, we propose to support at least full set A for the dedicated resource set for monitoring.

Proposal #19: For Option 2 (UE-assisted performance monitoring) of Type 1 performance monitoring, support at least full set A for the dedicated resource set for monitoring.

Secondly, linkage between an inference result report and a monitoring result report should be considered. According to the agreement, the ID of an inference report configuration can be configured in the CSI report configuration for monitoring, in order to have linkage between an inference report and a performance monitoring report. One important issue of this linkage is that UE cannot expect/predict when NW will activate/trigger monitoring RS/report. Therefore, UE has to buffer inference results until the ‘unknown time’ that could be very long time depending on NW’s decision.

Observation #3: Once a UE starts the CSI measurement/report for inference, the UE has to buffer the inference results until NW activates/triggers associated CSI measurement/report for monitoring.

Noting that UE do not need to buffer CSI if it is reported to NW in legacy system, this feature will create a serious issue for UE vendors if above issue is not resolved. Therefore, when an inference report is activated/triggered, and if the report is linked to a monitoring report, the monitoring report should be activated/triggered within a certain time limit, in order to limit the max buffering window size, where the time limit may be a specified value or a UE capability. Alternatively, the buffering window can be specified so that UE does not have to wait for the activation/triggering of monitoring report from NW.

Proposal #20: When an inference report is activated/triggered, and if the report is linked to a monitoring report, the monitoring report should be activated/triggered within a certain time limit or a buffering window for the inference results should be specified.

Lastly, there is FFS on whether to support all the combination on time domain behavior of the reportConfigType for inference report and the reportConfigType for monitoring report. In the current specification, supported time domain behavior of CSI reporting per P/SP/AP CSI-RS is described respectively, as captured below Table 1 from TS 38.214. This is to eliminate any possibility of CSI report instance without transmission of associated NZP CSI-RS, which can be ambiguous to UE behavior. Similarly, supported time domain behavior for monitoring result report should be restricted per P/SP/AP CSI report for UE-sided AI/ML inference result, so that UE can report performance monitoring result only if there is inference result report instance. So, we propose:

Proposal #21: Regarding supported combination on time domain behavior of the reportConfigType for inference report and the reportConfigType for performance monitoring report, support the following combination:

- Periodic report of inference + P/SP/AP report of performance monitoring

- Semi-persistent report of inference + SP/AP report of performance monitoring

- Aperiodic report of inference + AP report of performance monitoring

Table 1. Triggering/Activation of CSI Reporting for the possible CSI-RS Configurations from TS38.214

| CSI-RS Configuration | Periodic CSI Reporting | Semi-Persistent CSI Reporting | Aperiodic CSI Reporting |
| --- | --- | --- | --- |
| Periodic CSI-RS | No dynamic triggering/activation | For reporting on PUCCH, the UE receives an activation command, as described in clause 6.1.3.16 of [10, TS 38.321]; for reporting on PUSCH, the UE receives triggering on DCI | Triggered by DCI; additionally, subselection indication as described in clause 6.1.3.13 of [10, TS 38.321] possible as defined in Clause 5.2.1.5.1. |
| Semi-Persistent CSI-RS | Not Supported | For reporting on PUCCH, the UE receives an activation command, as described in clause 6.1.3.16 of [10, TS 38.321]; for reporting on PUSCH, the UE receives triggering on DCI | Triggered by DCI; additionally, subselection indication as described in clause 6.1.3.13 of [10, TS 38.321] possible as defined in Clause 5.2.1.5.1. |
| Aperiodic CSI-RS | Not Supported | Not Supported | Triggered by DCI; additionally, subselection indication as described in clause 6.1.3.13 of [10, TS 38.321] possible as defined in Clause 5.2.1.5.1. |

Meanwhile, from resource overhead perspective, periodic reporting may not be appropriate to report result of performance monitoring since it always consume periodic resource even when there is no performance issue at UE sided AI/ML model. If calculated performance metric at UE side derived from comparing predicted Top-K/1 beam and actual Top-K/1 beam is larger/lower than a threshold, UE can report the result of performance monitoring to NW as event-triggered basis, which is more efficient in terms of power consumption and signaling overhead than the periodic reporting based performance monitoring. When there is an issue on current AI/ML performance, UE cannot provide reliable and useful information about the SD/TD predicted beam to NW so UE behavior in this case should be taken into account, e.g., request for changing set A configuration, fallback to legacy beam report, holding the report for a while, etc.

Proposal #22: Support event-triggered UE reporting for UE-sided AI/ML performance monitoring.

- Further consider UE report via UCI or SR to request change of Set A configuration, fallback to legacy beam report, holding the report for a while, etc.

Consistency between training and inference

Four alternative ways to ensure consistency between training and inference for UE-sided AI/ML model have been identified during the SI phase [4]. Among these, we prefer to maintain functionality-based framework for one-sided models, and use the following two approaches together, which are also listed as Opt 1 and Opt 2 in RAN1 #116bis meeting:

- Providing information to assist UE training/inference (i.e., Opt 1: Based on associated ID)

  - Proprietary information such as antenna layout and beamforming coefficients should be kept hidden

- Performance monitoring of functionality(ies) (i.e., Opt 2: Performance monitoring based)

For the second approach(Opt 2), necessary specification impacts have already been discussed in previous sections. It is also considerable to consider performance monitoring of functionality(ies) related to inactive model(s).

Agreement Further study, for the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, where the NW-side additional condition may at least impact UE assumption on beams of Set A/Set B: - Opt1: Based on associated ID (Referring to AI 9.1.3.3) - FFS on what can be assumed by UE with the same associated ID across training and inference - FFS on how associated ID is introduced, e.g., within CSI framework, or outside of CSI framework - Opt 2: Performance monitoring based - FFS details Other options are not precluded. Agreement For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

For the first approach(Opt 1), the associated ID is to ensure the consistency on beams of Set A/Set B. In RAN1 #118 meeting, it was agreed to support associated ID, and with the same associated ID, UE may assume the similar properties of a DL Tx beam or beam set/list. We support that the associated ID can be configured within CSI framework, but the details (e.g., configured in CSI-ReportConfig, CSI-ResourceConfig, CSI-ResourceSet, or CSI-RS resource) should be discussed later after UE assumption with the same associated ID is concluded.

Proposal #23: Confirm the following working assumption:

- [Working Assumption]

  - The associated ID at least can be configured within CSI framework

    - FFS on details

    - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s)

Regarding the UE assumption with the same associated ID, “similar properties of a DL Tx beam or beam set/list” may assumed but it is still not clear. Firstly, we think that the definition of ‘similar’ should be clearly defined. If not, it is highly likely that different NW vendors may have different understanding on the definition of ‘similarity’. Therefore, a gNB may say that they are similar in a certain condition but another gNB belonging to different NW vendor may say that they are not similar in the same condition. Due to this inconsistency, the performance of UE-side model cannot be ensured and is very hard to be tested. Secondly, we need to list ‘the properties of DL Tx beam or beam set/list’ which affects to the consistency between training and inference. From DL Tx beam perspective, there are so many physical properties such as beam angle, beam width, power of main lobe, and power/angle of side lobe, etc. Therefore, it should be clearly define which ‘property’ is referred.

##### 5.1.6.1.2 CSI-RS for L1-RSRP and L1-SINR computation If a UE is configured with a NZP-CSI-RS-ResourceSet configured with the higher layer parameter repetition set to 'on', the UE may assume that the CSI-RS resources, described in Clause 5.2.2.3.1, within the NZP-CSI-RS-ResourceSet are transmitted with the same downlink spatial domain transmission filter, where the CSI-RS resources in the NZP-CSI-RS-ResourceSet are transmitted in different OFDM symbols. If repetition is set to 'off', the UE shall not assume that the CSI-RS resources within the NZP-CSI-RS-ResourceSet are transmitted with the same downlink spatial domain transmission filter.

As captured above, in TS 38.214[5], UE behavior when a NZP-CSI-RS-ResourceSet configured with the higher layer parameter repetition set to 'on' is described. According to the specification, with the parameter repetition set to ‘on’, UE may assume that the CSI-RS resources within the NZP-CSI-RS-ResourceSet are transmitted with the same downlink spatial domain transmission filter. Note that this does not mandate gNB to use ‘same’ physical beam because this is just UE assumption. In our view, it is clear if we extend the above assumption across different CSI-RS transmission instances. So, we suggest to define similar properties of a DL Tx beam set (for Set A and/or Set B) that at least the same downlink spatial domain transmission filters are maintained for each beams in the DL Tx beam set by NW.

Observation #4: The definition of ‘similar properties of a DL Tx beam or beam set/list’ should be clearly defined, otherwise different NW vendors may have different understanding on the definition which may affect performance of UE-sided AI/ML model inference due to inconsistency between training and inference.

Observation #5: In the current specification, there is clear description of UE assumption that the same downlink spatial domain transmission filter is maintained by NW for different CSI-RS resources.

Proposal #24: Define similar properties of a DL Tx beam set (for Set A and/or Set B) that at least the same downlink spatial domain transmission filters are maintained for each beam in different transmission instances.

Applicable functionality report (related with RAN2 LS reply)

In RAN1 #118bis, RAN1 did intensive discussion on functionality based LCM based on RAN2’s LS[6]. The overall procedure from RAN2 is captured below.

- Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - (Between “Step 3” and “Step 4”) UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. - “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration). - Step 5: 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not.

Based on above, the following agreements were made in RAN1.

Agreement RAN 1 further study the following options for applicability for inference for UE-side model: Option 1: - In Step 3, following configurations are provided from NW to UE: - 1) UE is allowed to do UAI reporting via OtherConfig, - 2)+3) NW configures one or more CSI-ReportConfig for inference configuration, where the associated ID may be configured in CSI framework as working assumption applied. - FFS on whether some IEs in the CSI report configuration can be removed or modified - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - In Step 4, UE reports applicability(ies) of the above CSI-ReportConfig - FFS on one or more of the above CSI-ReportConfig to be reported - FFS on activation (including when/how) of inference report after obtaining the applicability from UE Step 4 - FFS: whether Step 5 is needed, Option 2: - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - NW configures one set or multiple sets of inference related parameters - Note: the set of inference related parameters is not configured by CSI-ReportConfig - FFS on the set of inference related parameters, at least including: - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - The associated ID(s) may be configured - wherein the associated ID(s) may be - FFS: a) part of one set of the inference related parameters, or - FFS: b) independently from the one set of the inference related parameters. - In Step 4, UE reports applicability of the above one or multiple sets of inference related parameters, where the associated ID information may be associated. - In Step 5, NW configures configuration(s) for CSI report for inference Option 3: - In Step 3, following configurations are provided from NW to UE: - 1) UE is allowed to do UAI reporting via OtherConfig, - 2) The associated ID(s) may be provided to UE, e.g., a new RRC parameter. - In Step 4, UE reports by UAI - the applicable one or multiple sets of inference related parameters may be included. - FFS on the set of inference related parameters, at least including: - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - Note: not applicable may also be replied by UE - Note: if the inference related parameters are not supported for reporting, only the applicability(ies) or not is reported in Step 4. - the associated ID(s) may be included - FFS: a) as part of the inference related parameters, or - FFS: b) independently from the set of the inference related parameters. - In Step 5, NW configures configuration(s) for CSI report for inference. Note: There is no impact of configuring CSI report configuration for non-AI beam management in RRCReconfiguration. Agreement Incorporating below text into the general part of the LS In RAN1’s discussion of RAN 2 terminologies on beam management, - The concept/terminology “functionality“ of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-specific FGs - The concept/terminology “ functionality“ of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters or information/parameters indicated by UE - The Activated functionalities may be enabled based on CSI framework. Therefore, the meaning and the granularity of “functionality“ for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same, depends on certain option in RAN1, and the discussion is still ongoing. Agreement Answer to Q1 in R1-2407604 as below, In Step 2, RAN1 expects that UE reports its UE-capability information/parameters, i.e., Rel-19 AI/ML-specific FGs (including components and corresponding value ranges). These AI/ML-specific UE capability information/parameters will depend on how FGs are defined including the granularity, that will be discussed in RAN1 later in the WI.. Agreement For UE-side model, for beam management, for inference report, support periodic CSI report, aperiodic CSI report, and semi-persistent CSI report. Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability. Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3 Conclusion For the CSI-ReportConfig for inference configuration provided in Step 5, - aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete. - periodic CSI Report is considered as activated after RRCReconfigurationComplete. - Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig - Any specification impact is a separate discussion

For applicability report, the main difference between Option 1 and Option 2 is on whether to re-use CSI-ReportConfig or not. In our view, whether to re-use existing RRC IE is a minor issue which may be up to RAN2’s decision based on the entire design and procedure in the end of WI phase. Thus, we think that it is much beneficial to discuss on what information to deliver in each step. In this regard, we think that it is also possible to merge Option 1 and Option 2 because it is possible to remove or modify some existing IEs in the CSI report configuration for Option 1.

Proposal #25: Focus on what information to deliver in each step rather than re-using existing IE or not for applicable functionality reporting. Consider merging Option 1 and Option 2.

As agreed, UE would report AI/ML-specific FGs including components and value ranges in Step 2. These components and value ranges would correspond to functionalities that UE may be able to support under specific condition, which is different from legacy UE capability report. Let’s assume one exemplary FG considering aspects on Set A and Set B sizes.

- FG: BM-Case1

  - Component#1: the max size of Set B

    - Values: 16, 32, 64, 128, …

  - Component#2: the max size of Set A

    - Values: 32, 64, 128, 256, …

  - Component#3: the supported ratio(s) of Set B size divided by Set A size

    - Values: 1/2, 1/4, 1/8, …

Component#1 is related to memory requirement at UE side thus the reported value in Step 2 may not be updated later according to applicable functionality updates. However, the supported values related to component#2 and component#3 can be updated according to applicable functionality updates, e.g. based on updates on NW or UE-side additional condition.  For example, a UE may report the following in Step 2.

- Support of the FG

- Component#1: 32

- Component#2: 64, 128

- Component#3: 1/2, 1/4

Based on configuration and assistance information provided by NW in Step 3, UE may report which set(s) of components, e.g. (64, 1/2) and/or (128, 1/4), are/is currently applicable in Step 4.

Proposal #26: For the components of an FG, UE reports value ranges that the UE potentially supports in Step 2 and reports value(s) that the UE actually supports in Step 4 under configuration/information provided in Step 3.

Conclusion

In this contribution, we have discussed enhancements on AI/ML for beam management, and provided the following proposals.

Observation #1: In case of ‘beam indication of beams in Set A not in Set B’, it is practically not possible for UE to measure and maintain its Rx beam for all the Set A beams. So, for beam indication, TCI/QCL RS should be based on Set B beams of which UE can measure and maintain its Rx beam.

Observation #2: For Option 1 (NW-side performance monitoring) of Type 1 performance monitoring, there may be no specification impact, e.g., NW can exploit report of more than 4 beam related information in L1 signaling for performance monitoring purpose.

Observation #3: Once a UE starts the CSI measurement/report for inference, the UE has to buffer the inference results until NW activates/triggers associated CSI measurement/report for monitoring.

Observation #4: The definition of ‘similar properties of a DL Tx beam or beam set/list’ should be clearly defined, otherwise different NW vendors may have different understanding on the definition which may affect performance of UE-sided AI/ML model inference due to inconsistency between training and inference.

Observation #5: In the current specification, there is clear description of UE assumption that the same downlink spatial domain transmission filter is maintained by NW for different CSI-RS resources.

Proposal #1: For NW-sided AI/ML in temporal DL Tx beam prediction, support the following UE reporting enhancements for data collection:

- Past/present best N beam(s) per time stamp

- Tendency/variance of best N beam(s)

Proposal #2: Support reporting of UE assistance information for determining Set A, e.g., UE to report preferred Set A among candidate beams of Set A.

Proposal #3: Consider extending sub-configuration based Rel-18 NES mechanism for Set B beam measurement and reporting.

- Different Set A and/or Set B may be associated with each sub-configuration

- Different Set B patterns for a specific Set B may be associated with each sub-configuration

Proposal #4. Regarding the report of more than 4 beam related information in L1 signaling for NW-sided model, support CRI/SSBRI for beam information as legacy, e.g., CRI/SSBRI(s) of Top M beam(s) and the corresponding M L1-RSRP(s) are reported.

Proposal #5: In order to indicate one beam in Set A not in Set B, support indicating multiple neighboring beams from Set B for helping UE to find its Rx beam for the Set A beam.

Proposal #6: For UE-sided model, support to configure only resource set for Set B for CSI-ResourceConfig which is used for the configuration of inference results reporting (i.e., Alt 1), considering huge resource overhead reduction obtained from not configuring Set A.

Proposal #7: In order to support only resource set for Set B for CSI-ResourceConfig (Alt 1) for Set A and Set B configuration, assistance information on relation/association between Set A beams and Set B beams should be provided to UE for the UE-side AI/ML model training and inference. To represent beams in Set A and/or Set B while preserving sensitive proprietary information, consider following exemplary methods:

- Set A beams are represented by linear combining coefficients of Set B beams

- Tx beam directions are represented as ordered numbers on a 2D or 3D coordinate

Proposal #8: For supported Option 1 and Option 2, support K=4 for the max value.

- Considering the case that predicted beam is in Set A but not in Set B, reported beam information can be based on the relation/association between Set A beams and Set B beams

Proposal #9: For predicted RSRP report, confidence/probability information may be helpful for NW to decide whether/how to use the reported RSRP. Further study whether the information is per model/functionality, per report, per time instance, or per report parameter.

Proposal #10: Support to report inference results of N(N>=1) future time instance(s) in one report.

- Maximum value of N can be more than 1

- Maximum value of N can be reported by UE capability, and M(M<=N) value for inference results reporting can be configured by gNB

Proposal #11: For BM-Case 2, support differential RSRP report among multiple beams over multiple time instances.

- In the inference result report, an indicator for the time instance containing the largest predicted RSRP in addition to the best beam indicator is reported by UE

- Other than largest predicted RSRP value across every multiple time instances, predicted RSRP value for each beam is expressed by differential RSRP value (e.g., 4 bit)

Proposal #12: Regarding the reference time to determine the earliest time instance for the predicted results in BM-case 2, support to re-use Rel-18 MIMO CSI prediction framework.

- The reference time is based on the uplink slot for the report and slot offset value can be configured by NW to determine the earliest time instance, wherein candidate value of slot offset also includes the value to indicate CSI reference resource, e.g., δ=-nCSI_ref

Proposal #13: For temporal DL Tx beam prediction, information on time-variation of RSRP can also be included in the report.

Proposal #14: For temporal DL Tx beam prediction with UE-sided models, following beam reporting enhancements can be considered:

- Report of beam dwelling time for each future time instance

- Report of beam(s) for current time instance for fallback operation

Proposal #15: Support Option A or Option C (new) for the RSRP of predicted Top K beam(s) in the report of inference results.

- Option A: Predicted RSRP

- Option C: Not specify whether to report predicted RSRP or measured L1-RSRP when both RSRPs are available at UE side, i.e., leave this case up to UE implementation

Proposal #16: For UE-side AI/ML model, for BM-case 2, do not support AP CSI-RS for set B configuration for inference.

Proposal #17: For UE-side AI/ML model, for BM-case 2, at least for inference, support only P CSI-RS for Set A configuration.

Proposal #18: For UE-side AI/ML operations, support to introduce AI/ML processing units(APU) for CSI reports for inference result report.

- APU is occupied for all AI/ML inference operations at UE-side, and separately managed from CPU

- Further study on APU capability, e.g., supported maximum APUs per UE, required APUs per functionality/sub-use-case.

Proposal #19: For Option 2 (UE-assisted performance monitoring) of Type 1 performance monitoring, support at least full set A for the dedicated resource set for monitoring.

Proposal #20: When an inference report is activated/triggered, and if the report is linked to a monitoring report, the monitoring report should be activated/triggered within a certain time limit or a buffering window for the inference results should be specified.

Proposal #21: Regarding supported combination on time domain behavior of the reportConfigType for inference report and the reportConfigType for performance monitoring report, support the following combination:

- Periodic report of inference + P/SP/AP report of performance monitoring

- Semi-persistent report of inference + SP/AP report of performance monitoring

- Aperiodic report of inference + AP report of performance monitoring

Proposal #22: Support event-triggered UE reporting for UE-sided AI/ML performance monitoring.

- Further consider UE report via UCI or SR to request change of Set A configuration, fallback to legacy beam report, holding the report for a while, etc.

Proposal #23: Confirm the following working assumption:

- [Working Assumption]

  - The associated ID at least can be configured within CSI framework

    - FFS on details

    - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s)

Proposal #24: Define similar properties of a DL Tx beam set (for Set A and/or Set B) that at least the same downlink spatial domain transmission filters are maintained for each beam in different transmission instances.

Proposal #25: Focus on what information to deliver in each step rather than re-using existing IE or not for applicable functionality reporting. Consider merging Option 1 and Option 2.

Proposal #26: For the components of an FG, UE reports value ranges that the UE potentially supports in Step 2 and reports value(s) that the UE actually supports in Step 4 under configuration/information provided in Step 3.

References

[1] RP-234039, “New WID on AI/ML for NR Air Interface”, QualComm

[2] R1-2400232, “Specification support for beam management”, vivo

[3] R1-2401002, “Discussion on AI/ML beam management”, Apple

[4] TR 38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”

[5] TS 38.214, “NR; Physical layer procedures for data”

[6] R1-2407604, “LS on applicable functionality reporting for beam management UE-sided mode”, RAN2