3GPP TSG RAN WG1 #120		     	R1-2500587

Athens, Greece, Feb 17th-21st, 2025

Source:	NEC

Title:	Discussion on random access for subband non-overlapping full duplex

Agenda Item:	9.3.2

Document for: 	Discussion and Decision

- Introduction

During the RAN #102 meeting, a new WI on the evolution of NR duplex operation was approved [1]. In the RAN1#119 meeting, the agreements reached are as follows [2].

Agreement

Introduce explicit indications (1-bit SIB1 indication and/or dedicated signalling to convey cell-specific configuration) to indicate whether RACH configuration Option 1 for SBFD random access operation is enabled or not from network side.

Agreement

For RACH configuration Option 1, for support of separate power control parameters for PRACH transmission in additional-ROs and legacy-ROs, support separate preambleReceivedTargetPower for additional-ROs.

- FFS: powerRampingStep, preamble maximum output power PCMAX, preambleTransMax, powerRampingStepHighPriority

Agreement

For RACH configuration Option 2, the association period and association pattern period of additional-ROs are determined separately from legacy-ROs.

- The legacy rules for determining association period and association pattern period are reused in principle for additional-ROs.

Agreement

Confirm the following working assumption

Working Assumption

For a PRACH transmission with Npreamblerep preamble repetition within additional-ROs, support reusing current specification rules for the determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-ROs.

- the time period of the set(s) of Npreamblerep valid additional-ROs is determined separately from the time period for legacy-ROs

Agreement

It is up to RAN2 to determine the detailed specified/configured conditions/prioritizations for RO selection for initial PRACH transmission attempt in one random access procedure.

Agreement

For CFRA triggered by PDCCH order for SBFD aware-UEs, SBFD-aware UE is explicitly indicated to use additional-RO or legacy-RO via a 1-bit field in the DCI of PDCCH order

Agreement

For RACH configuration Option 2, and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration, down-select from the following options:

- Alt-1: introduce a subframe/slot offset (similar as prach-ConfigurationSOffset-IAB) to determine the subframe/slot number for ROs in SBFD symbols for FR1/FR2

  - FFS: details of the subframe/slot offset

- Alt-2: introduce a new parameter indicating from an entry from a pre-defined/configured table to derive additional subframe/slot numbers.

  - Details are FFS.

- Alt-3: introduce a bitmap indicating or puncturing subframe(s)/slot(s) for additional ROs in SBFD symbols.

  - Details are FFS.

- Alt-4: no enhancement

Agreement

For RACH configuration Option 2, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 and msg1-RepetitionNum for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.

- msg1-RepetitionNum is configured in RACH-ConfigCommon in current specification, i.e., for RACH configuration Option 2, msg1-RepetitionNum can already be separately configured for legacy-ROs and additional-ROs.

Agreement

For RACH configuration Option 1, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.

- Same msg1-RepetitionNum is used for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.

Agreement

For SBFD aware UEs, for Msg3 PUSCH transmission,

- the number of NUL,hop hopping bits is based on the number of PRBs in initial UL BWP as in legacy specification.

- the frequency offset for the 2nd hop is reserved for the case when the value of NUL,hop= 11 as in legacy specification.

Agreement

For an SBFD aware UE, if addition RO is selected for Msg1 transmission, only Configuration 1 is supported for Msg3 repetition

Agreement

For determining PUCCH resource sets in SBFD symbols before dedicated PUCCH resource configuration, reuse the Table 9.2.1-1 in TS 38.213, with the following modifications:

- use the size of UL usable PRBs to replace the size of initial UL BWP to determine the PRB offset in row index 15.

Agreement

For RACH configuration Option 1 with Alt 1-1, for determination of lowest RO of additional-ROs in SBFD symbols, Alt 2-1 is supported.

- Alt 2-1: The parameter msg1-FrequencyStart in rach-ConfigCommon is reinterpreted as the frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs.

- ROs that are outside of UL usable PRBs are invalid

Agreement

For SBFD aware UEs, update the following equations to determine the lowest PRB indices of the PUCCH transmission in the first hop and second hop in SBFD symbols before dedicated PUCCH resource configuration:

- if rPUCCH/8=0

  - RBUL SB start+RBBWPoffset∙NRB+rPUCCH/NCS∙NRB for the first hop

  - RBUL SB start+NUL SBsize-RBBWPoffset∙NRB-(1+rPUCCH/NCS) ∙NRB for the second hop

- if rPUCCH/8=1

  - RBUL SB start+NUL SBsize-RBBWPoffset∙NRB-(1+(rPUCCH-8)/NCS) ∙NRB for the first hop

  - RBUL SB start+RBBWPoffset∙NRB+(rPUCCH-8)/NCS∙NRB for the second hop

Where

- RBUL SB start is the starting PRB index of UL usable PRBs with reference to the start of UL active BWP.

- NUL SBsize is the number of UL usable PRBs.

- RBBWPoffset is the RB offset with reference to the lowest PRB of UL usable PRBs.

Note: Definitions of NRB, rPUCCH and NCS are unchanged from existing specifications.

Agreement

For RACH configuration Option 2, for the case that additional ROs overlap with legacy ROs, down-select from the following alternatives:

- Alt-1: do not optimize this case

- Alt-2: the additional-ROs are considered invalid when overlapping with legacy-ROs

Agreement

For Msg3 PUSCH repetition Configuration 1, the valid symbol type is determined based on the symbol type of the first transmission occasion.

- UE does not expect the first transmission occasion includes different symbol types (i.e., SBFD symbols and non-SBFD symbols)

- FFS: Available slot counting on SBFD symbols

Agreement

Study how to determine the Msg3 PUSCH power control in SBFD symbols and non-SBFD symbols based on at least the following parameters.

- preambleReceivedTargetPower, msg3-DeltaPreamble/deltaPreamble, TPC Command in RAR

This contribution will discuss and provide views on UE random access procedures for RRC_CONNECTED and RRC_IDLE/INACTIVE mode for subband non-overlapping full duplex.

- Discussion

4-step RACH Procedure Enhancements for SBFD Operation

From the physical layer perspective, the Type-1 L1 random access procedure includes the transmission of random access preamble (Msg1) in a PRACH, random access response (RAR) message with a PDCCH/PDSCH (Msg2), and when applicable, the transmission of a PUSCH scheduled by a RAR UL grant, and PDSCH for contention resolution.

2.1.1 RO configuration

For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported.

In RAN1#119 meeting, below agreements were reached for power control parameters configuration for PRACH transmission.

Agreement

For RACH configuration Option 1, for support of separate power control parameters for PRACH transmission in additional-ROs and legacy-ROs, support separate preambleReceivedTargetPower for additional-ROs.

- FFS: powerRampingStep, preamble maximum output power PCMAX, preambleTransMax, powerRampingStepHighPriority

We think for RACH configuration option 1 with Alt 1-1(use one single RACH configuration, and only based on the existing parameters of the single RACH configuration), the configured Max number of RA preamble transmission preambleTransMax is applied to additional RO and legacy RO, and it includes the preamble transmission times performed on SBFD symbols and non-SBFD symbols. And for RACH configuration option 2(one legacy RACH configuration and one additional RACH configuration), preambleTransMax is separately configured in additional configuration for preamble transmission counting on SBFD symbols.

Proposal 1:

- For RACH configuration Option 1 with Alt 1-1, preambleTransMax is applied to RA preamble transmission performed on SBFD symbols and non-SBFD symbols counting.

- For RACH configuration option 2(one legacy RACH configuration and one additional RACH configuration), preambleTransMax is separately configured in additional configuration for preamble transmission counting on SBFD symbols.

Furthermore, power ramping for PRACH/preamble retransmission should consider inter/intra subband CLI, and power ramping should be controlled for PRACH transmission on SBFD symbols for SBFD-aware UE. The power ramping for PRACH on SBFD symbols can be changed to one of the following:

Alt1: the same counter value of powerRampingStep is used for PRACH transmission on SBFD symbols and non-SBFD symbols.

Each beam has a separate power ramping counter. If the beam is not changed on SBFD symbols, then the power ramping counter for the PRACH/preamble retransmission will be reset, and if the beam is changed, then the counter is kept for the PRACH retransmission on SBFD symbols.

Or, for the first two PRACH transmissions after beam change, the power ramping counter is not changed if the transmission is on SBFD symbols, and from the third retransmission on the SBFD symbols, the power ramping counter is counted.

Or, if the beam is not changed, the power ramping counter for the PRACH retransmission on the SBFD symbols will be reset. If the retransmission number reaches 2 or 3 times, then UE will do power ramping for the PRACH transmission. Otherwise, UE will keep the power for PRACH retransmission on the SBFD symbols.

Alt2: powerRampingStep is configured separately and uses different counters for PRACH transmission on SBFD and non-SBFD symbols.

Alt3: powerRampingStep is always equal to dB0 for the PRACH retransmission on the SBFD symbols. UE does not perform power ramping for PRACH retransmission on SBFD symbols.

For RACH configuration Option 2, separate power control parameters for PRACH transmission in SBFD symbols and non-SBFD symbols should also be supported, such as separate powerRampingStep, preamble maximum output power PCMAX and power ramping counter.

Proposal 2:

For RACH configuration Option 1 and Option 2,

- powerRampingStep and preamble maximum output power P_CMAX are configured separately and uses different counters for PRACH transmission on SBFD and non-SBFD symbols.

For RACH configuration Option-2, there is always a risk that ROs configured by additional RACH configuration may (partially) overlap in the time domain with ROs configured by legacy RACH configuration. For example, consider the overlap scenarios:

- The ROs configured by additional configuration may overlap in the time domain with ROs configured by legacy RACH configuration during Flexible symbols (note that legacy RACH configuration can configure ROs during Flexible SBFD symbols as discussed during RAN1#116bis meeting).

And the following agreement was reached in #119 meeting and two alts were provided for down selection.

Agreement

For RACH configuration Option 2, for the case that additional ROs overlap with legacy ROs, down-select from the following alternatives:

- Alt-1: do not optimize this case

- Alt-2: the additional-ROs are considered invalid when overlapping with legacy-ROs

In the above case, if the RO configured by additional RACH configuration is mapped to a different set of SSBs as compared to the RO configured by legacy RACH configuration, then the gNB might not be able to simultaneously receive both ROs (as gNB may use different Rx beams to receive the corresponding ROs). Although there have been arguments that such kind of conflict can be addressed by gNB implementation (which is assumed for RACH configuration for different features like RedCap), as the number of features (which may require additional RACH configurations) which are supported by a gNB keeps increasing, addressing such kind of conflict scenarios through implementation becomes even harder.

Hence, there is a need to specify validity rules for such overlap scenarios. One simple mechanism to address this case is that if an RO configured by an additional RACH configuration overlaps in the time domain with an RO of legacy RACH configuration, then the RO configured by an additional RACH configuration is considered invalid.

Proposal 3:

- For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration), support Alt-2: an RO configured by an additional RACH configuration is considered invalid if it overlaps in the time domain with a valid RO configured by legacy RACH configuration.

Another discussion which has been ongoing is how to interpret PRACH configuration index for additional RACH configuration in Option-2, where following options are being considered for further discussion based on RAN1#119.

Agreement

For RACH configuration Option 2, and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration, down-select from the following options:

- Alt-1: introduce a subframe/slot offset (similar as prach-ConfigurationSOffset-IAB) to determine the subframe/slot number for ROs in SBFD symbols for FR1/FR2.

  - FFS: details of the subframe/slot offset.

- Alt-2: introduce a new parameter indicating from an entry from a pre-defined/configured table to derive additional subframe/slot numbers.

  - Details are FFS.

- Alt-3: introduce a bitmap indicating or puncturing subframe(s)/slot(s) for additional ROs in SBFD symbols.

  - Details are FFS.

- Alt-4: no enhancement.

The above options (from Alt-1 to Alt-3) have been identified to provide the following gains for the system level operation:

- Allow gNB to configure RACH resource for additional RACH configuration in such a manner to minimize time domain overlap with legacy RACH resources thus avoiding different conflicting scenarios

- More flexibility for the network in terms of time occurrence of additional RACH resources

However, the first motivation can also be resolved with the previous proposal we discussed which updates the UE behaviour on how to determine valid RACH resource. While the benefits of Alt-1 to Alt-3 for higher network RO allocation flexibility is doubtful as the existing PRACH configuration indices already enable sufficient control at the network side in terms of time occurrence of RACH occasions within a system frame. Hence, we think that Alt-1 to Alt-3 may not bring sufficient gains, and we can simplify the SBFD design considerably by following legacy specification for PRACH configuration index.

Proposal 4:

- For RACH configuration Option 2, and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration, support Alt-4 (no enhancement).

2.1.2 RO type indication

In RAN1#119, the agreement that CFRA triggered by PDCCH order for SBFD aware-UEs, SBFD-aware UE is explicitly indicated to use additional-RO or legacy-RO via a 1-bit field in the DCI of PDCCH order were reached.  For other CFRA configuration, the configured value of ra-ssb-OccasionMaskIndex in CFRA , SI-RequestConfig, and BeamFailureRecoveryConfig and the configured value ssb-SharedRO-MaskIndex in FeatureCombinationPreambles not given the RO type(the additional ROs or legacy ROs), therefore UE does not known which type of symbol this parameter applied to or UE does not known how to determine the RO index in SBFD symbols. Some RO type configuration or indication or rule should be defined for SBFD aware-UE to determine the RO index on SBFD symbols and non-SBFD symbols. For example, new table can be defined for SBFD aware UE to determine the RO resource of SSB for PRACH transmission on SBFD symbols or new column is added in PRACH Mask Index/msgA-SSB-SharedRO-MaskIndex/ssb-SharedRO-MaskIndex values table in TS 38.321 to indicate the RO index of SSB on SBFD symbols.

Furthermore, for CFRA procedure triggered by BFR or SI request, the other ra-ssb-OccasionMaskIndex parameter can be included in BeamFailureRecoveryConfig IE or SI-RequestConfig IE for UE to determine the RO resource of SSB on SBFD symbols or gNB configure the applied symbol type (SBFD symbol or non-SBFD symbol or both) or RO Type (legacy RO or additional RO or both) for on-demand SI triggered or BFR triggered CFRA resource for preamble transmission. Otherwise, UE does not know the RO resource of SSB on SBFD symbols.

Proposal 5:

- For SBFD-aware UEs and CFRA triggered by BFR or SI request, support one of the following for UE to determine the RO resource

- New table can be defined for SBFD aware UE to determine the RO resource of SSB for PRACH transmission on SBFD symbols

- New column is added in PRACH Mask Index/msgA-SSB-SharedRO-MaskIndex/ssb-SharedRO-MaskIndex values table for SBFD aware UE to determine the RO resource of SSB for PRACH transmission on SBFD symbols

- gNB configure the applied symbol type (SBFD symbol or non-SBFD symbol or both) or RO Type for CFRA

2.1.3 Msg3 PUSCH enhancement

The intra-slot frequency hopping for Msg3 PUSCH enabled or disabled for SBFD slots can be separately configured by RRC or indicated in the RAR UL grant. The value of hopping bits in RAR/RRC to indicate which frequency offset to use for the second hop for SBFD symbols can be separately configured/indicated.To avoid the frequency hopping resource overlapping with the DL subband, below agreement were reached in #118bis and #119 meeting.

Agreement

For SBFD aware UEs, for Msg3 PUSCH transmission, reuse Table 8.3-1 in TS 38.213 for determination of the frequency offset for the 2nd hop, with the update that the frequency offset for 2nd hop is based on the size of UL usable PRBs.

- FFS the case when the value of NUL,hop= 11

- FFS the definition of UL usable PRBs for Msg3 PUSCH transmission

Agreement

For SBFD aware UEs, for Msg3 PUSCH transmission,

- the number of NUL,hop hopping bits is based on the number of PRBs in initial UL BWP as in legacy specification.

- the frequency offset for the 2nd hop is reserved for the case when the value of NUL,hop= 11 as in legacy specification.

We think the definition of UL usable PRBs for Msg3 PUSCH transmission can be the PRB number part or whole within the UL subband.Furthermore, for Msg3 PUSCH repetition transmission, below agreement was reach in #119 meeting.

Agreement

For an SBFD aware UE, if addition RO is selected for Msg1 transmission, only Configuration 1 is supported for Msg3 repetition

We think if legacy RO is selected for Msg1 transmission, Configuration 2 can be supported for Msg3 repetition transmission that is UE can transmit the repetition of Msg3 PUSCH across the SBFD and non-SBFD slots. And for the symbols of the Msg3 PUSCH repetition transmission, the configured UL subband on the SBFD symbols can be determined as the available resource if the allocated FDRA in RAR is within the UL subband BW; otherwise, UE defers the Msg3 PUSCH repetition transmission.

Proposal 6:

For Msg3 PUSCH enhancements, the following aspects can be supported.

- The definition of UL usable PRBs for Msg3 PUSCH transmission can be the PRB number part or whole within the UL subband.

- PUSCH repetition transmission can across SBFD symbols and non-SBFD symbols if legacy RO is selected for Msg1 transmission

2-step RACH Procedure Enhancements for SBFD Operation

For SBFD-aware UEs in the RRC_CONNECTED state, the Type-1 random access procedure (4-step RACH) in SBFD symbols has been supported, and whether the Type-2 random access procedure (2-step RACH) can also be supported is FFs. Considering most solutions of 4-step RACH can be applied to 2-step RACH, e.g., RO configuration, additional work on 2-step RA can be considered if time allows.

Proposal 7:

- Both 4-step RACH and 2-step RACH can be supported on SBFD symbols.

From the physical layer perspective, the Type-2 L1 random access procedure includes the transmission of random-access preamble in a PRACH and of a PUSCH (MsgA) and the reception of a RAR message with a PDCCH/PDSCH (MsgB), and when applicable, the transmission of a PUSCH scheduled by a fallback RAR UL grant, and PDSCH for contention resolution. The following aspects should be considered for Type-1 L1 random access procedure with SBFD operation.

- How to configure the PO and determine its validity.

- The association relationship between PRACH and the PUSCH occasion associated with a DMRS resource in SBFD symbols should be defined.

- Power ramping for the PRACH and the PUSCH may produce inter-subband CLI on SBFD symbols.

For PO configuration in SBFD symbols as the RO configuration, additional configurations can be added for POs determination or MsgA PUSCHs transmission on SBFD symbols in existing MsgA-PUSCH-Config or MsgA-PUSCH-Resource IE. UE can determine PO numbers, frequency positions, symbol positions, MCS, mapping types, or DMRS configurations in SBFD symbols. Some offset values relative to the non-SBFD symbols can be predefined, configured, or indicated for UE PUSCH transmission on the SBFD symbols, such as the frequency position or MCS determination for the PUSCH transmission on SBFD symbols.

Currently, the time-frequency resources used for PUSCH are configured by SIB1. In legacy, valid PO is determined based on semi-static TDD pattern configuration. PO overlapping with a semi-static DL symbol would be regarded as an invalid PO. For SBFD operation, the PO is valid if the PO is within the UL subband in the SBFD symbols and the PO is not overlapped with a preamble of valid PRACH occasions in the SBFD symbols.

In the existing spec, the preamble mapping relationship with PRU between one or multiple PRACH preambles and a PUSCH occasion associated with a DMRS resource is per PUSCH configuration provided by msgA-PUSCH-Resource-r16. Suppose MsgA PUSCH configuration is common for SBFD symbols and non-SBFD symbols. Both the RO and the PO indices are numbered together for SBFD symbols and non-SBFD symbols in increasing order of slot indexes. The RARCH mapping to PRU uses the existing procedure for SBFD symbols and non-SBFD. Suppose two preamble groups and two PUSCH resource configurations are always configured for SBFD-aware UE. The predefined mapping relationship for PRACH and the PUSCH can be changed to the preamble group A associated with the PUSCH resource on the SBFD symbols and preamble group B associated with the PUSCH resource on the non-SBFD symbols. The mapping is separately performed for each group, as illustrated in Figure 3. For each group, the preambles are mapped to the PRU in increasing order of time resource indexes t_id for time-multiplexed PUSCH occasions within a PUSCH slot on the SBFD or non-SBFD symbols.

Figure 3

Besides, for SBFD-aware UE, powerRampingStep can be separately configured, and different counters can be used for MsgA (re)transmission on SBFD and non-SBFD symbols.

Proposal 8:

- The following aspects should be considered for 2-step RACH enhancements on SBFD symbols:

  - PO configuration and valid determination in the UL subband of SBFD symbols

  - The RO mapping relationship with PRU in SBFD symbols

  - Power control for PUSCH for MsgA

  - Random access in IDLE/INACTIVE mode and SDT transmission

    - SDT transmission

In Release R17, CG-based SDT, and RACH-based PUSCH transmission was introduced. A UE indicated to release a dedicated RRC connection can be provided with one or more configurations by one or more ConfiguredGrantConfigs for configured grant Type 1 PUSCH transmissions on the initial UL BWP. For the remaining clause, PUSCH transmissions refer to configured grant Type-1 PUSCH transmissions for a configuration provided by ConfiguredGrantConfig. One ConfiguredGrantConfig corresponds to one SSB mapping relationship. Besides, sdt-SSB-Subset can provide a UE with a number of SS/PBCH block indexes N_PUSCH^(SS/PBCH) to map to several valid PUSCH occasions for PUSCH transmissions over an association period. If the UE is not provided sdt-SSB-Subset, the UE determines N_PUSCH^(SS/PBCH) from the value of ssb-PositionsInBurst in SIB1 or by ServingCellConfigCommon. A PUSCH occasion for a PUSCH transmission is defined by a time resource and a frequency resource. It is associated with a DM-RS provided by cg-DMRS-Configuration for the configuration of PUSCH transmissions. A UE is provided several SS/PBCH block indexes associated with a PUSCH occasion and a DM-RS resource by sdt-SSB-PerCG-PUSCH. PUSCH occasions and associated DMRS resources not associated with SS/PBCH block indexes after an integer number of association periods, if any, are not used for PUSCH transmissions.

For an RRC_INACTIVE UE, using SBFD symbols to transmit SDT can provide more UL resources to increase the SDT PUSCH transmission reliability and reduce the SDT latency. The aspects below should be considered for CG-based SDT PUSCH transmission on SBFD symbols.

- How to configure or determine the SDT CG PUSCH resource on SBFD symbols

- How do you determine the validity of the PO on SBFD symbols?

- The association relationship between the SSB index and the SDT CG PUSCH occasion in SBFD symbols should be defined.

- CG PUSCH retransmission on SBFD symbols.

For RACH-based SDT PUSCH transmission, some enhancement can also considered on SBFD symbols. For example, a dedicated CSS set can be configured so that UE can monitor the PDCCH on SBFD symbols.

Proposal 9:

- The aspects below should be considered for SDT enhancements on SBFD symbols:

  - PUSCH allocation/configuration in the UL subband of SBFD symbols

  - Valid PO determination on SBFD symbols

  - The PO mapping relationship with SSB for PO in SBFD symbols

    - Paging enhancement

For idle/inactive state UE configured with SBFD operation, UE needs to receive a paging message. To receive paging messages, UE should do PDCCH monitoring on a PO that is scrambled by P-RNTI. A PO is a set of PDCCH monitoring occasions and can consist of multiple time slots (e.g., subframe or OFDM symbol) where paging DCI can be sent. It is very likely that PO may fully or partly overlap with SBFD durations. In this case, if UE transmits UL (such as PRACH or msg3) on the UL subband of SBFD symbols, UE cannot detect PDCCH for paging and cannot receive a paging message; the paging message will be missed. Therefore, the performance of paging for these UEs will be impacted. Some methods should be considered to resolve the missing paging issue. For example, more PDCCH monitoring occasions can be configured for SBFD-aware UE. If all original PDCCH monitoring occasions overlap with SBFD symbols or UL transmission in the UL subband, gNB can still use the additional PDCCH monitoring occasions to send DCI for paging to UE, as illustrated in Figure 4 below. Then, a paging message will not be missed.

Figure 4

Proposal 10:

- Study paging enhancement on SBFD symbols.

- 2.4 Timer adjustments for SBFD slots

When considering PRACH in cross-slot or normal within-slot configurations. The timer value can run out during PRACH if the UE receives no return signal from the gNB. When this happens, the UE should try PRACH again during dedicated UL slots instead of continued access during SBFD slots. The more chaotic nature of SBFD transmissions means they have less favorable channel characteristics for transmission, so it would be better for the UE to simply wait until more favorable transmission channels re-attempt the PRACH process.

Proposal 11:

- For failed PRACH attempts in SBFD slots, the UE will wait for dedicated UL slots to re-attempt PRACH.

- Conclusion

In this contribution, we discussed SBFD for NR duplex operation. Observations and Proposals are summarized as follows:

Proposal 1:

- For RACH configuration Option 1 with Alt 1-1, preambleTransMax is applied to RA preamble transmission performed on SBFD symbols and non-SBFD symbols counting.

- For RACH configuration option 2(one legacy RACH configuration and one additional RACH configuration), preambleTransMax is separately configured in additional configuration for preamble transmission counting on SBFD symbols.

Proposal 2:

For RACH configuration Option 1 and Option 2,

- powerRampingStep and preamble maximum output power P_CMAX are configured separately and uses different counters for PRACH transmission on SBFD and non-SBFD symbols.

Proposal 3:

- For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration), support Alt-2:an RO configured by an additional RACH configuration is considered invalid if it overlaps in the time domain with a valid RO configured by legacy RACH configuration.

Proposal 4:

- For RACH configuration Option 2, and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration, support Alt-4 (no enhancement).

Proposal 5:

- For SBFD-aware UEs and CFRA triggered by BFR or SI request, support one of the following for UE to determine the RO resource

- New table can be defined for SBFD aware UE to determine the RO resource of SSB for PRACH transmission on SBFD symbols

- New column is added in PRACH Mask Index/msgA-SSB-SharedRO-MaskIndex/ssb-SharedRO-MaskIndex values table for SBFD aware UE to determine the RO resource of SSB for PRACH transmission on SBFD symbols

- gNB configure the applied symbol type (SBFD symbol or non-SBFD symbol or both) or RO Type for CFRA

Proposal 6:

For Msg3 PUSCH enhancements, the following aspects can be supported.

- The definition of UL usable PRBs for Msg3 PUSCH transmission can be the PRB number part or whole within the UL subband.

- PUSCH repetition transmission can across SBFD symbols and non-SBFD symbols if legacy RO is selected for Msg1 transmission

Proposal 7:

- Both 4-step RACH and 2-step RACH can be supported on SBFD symbols.

Proposal 8:

- The following aspects should be considered for 2-step RACH enhancements on SBFD symbols:

  - PO configuration and valid determination in the UL subband of SBFD symbols

  - The RO mapping relationship with PRU in SBFD symbols

  - Power control for PUSCH for MsgA

Proposal 9:

- The aspects below should be considered for SDT enhancements on SBFD symbols:

  - PUSCH allocation/configuration in the UL subband of SBFD symbols

  - Valid PO determination on SBFD symbols

  - The PO mapping relationship with SSB for PO in SBFD symbols

Proposal 10:

- Study paging enhancement on SBFD symbols.

Proposal 11:

- For failed PRACH attempts in SBFD slots, the UE will wait for dedicated UL slots to re-attempt PRACH.

- References

- RP-234035, ‘New WI: Evolution of NR duplex operation: Sub-band full duplex (SBFD)’CMCC.

- Draft_Minutes_report_RAN1#119