3GPP TSG RAN WG1 #120	R1-2500855

Athens, Greece, February 17th – 21st, 2025

Agenda item:	9.5.2

Source:	Samsung

Title:	On-demand SIB1 for idle/inactive mode UEs

Document for:	Discussion and decision

- Introduction

To reduce energy consumption of common signal and channel for UEs in idle/inactive mode, in the WID of Rel-19 network energy saving [1], the following objective has been specified:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3]

  - Specify procedures and signaling method(s) for Case 2 [RAN1/2]

    - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell

    - Triggering method by UL WUS using PRACH

  - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3]

  - Note 1: No modification of SSB will be discussed under this objective

  - Note 2:

    - UL WUS: Uplink wake-up signal

    - Cell A: A cell that is periodically transmitting at least its own SIB1

    - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE

  - Note 3:

    - RAN1 strives to minimize impact to legacy UE

    - RAN1 specification impact to support this feature should be minimized

In this contribution, we discuss details on supporting on-demand SIB1 operation for UEs in idle/inactive mode.

- How to use pdcch-configSIB1 provided by NES cell

In the previous meeting [3], the following agreement has been made to define how UE identifies a NES cell with on-demand SIB1.

Agreement On how to use PDCCH-ConfigSIB1 for R19 NES-capable UE in MIB of NES Cell when K_SSB = 30 in FR1 or K_SSB = 14 in FR2 on NES cell if SSB of NES cell is on the sync raster, down-select from the following options: - Option 1: Use it to indicate frequency assistance information to search SSB for Cell A - FFS: Details on the range/granularity of the frequency assistance information - FFS: Potential impact to RAN3 - Option 2: Use it to indicate searchSpaceZero and controlResourceSetZero of OD-SIB1 - Option 3: Above feature is not supported.

There is still a remaining issue on UE behavior after first detecting SSB transmitted by NES cell, when K_SSB = 30 in FR1 or K_SSB = 14 in FR2. Even if a UE can determine whether or not the SSB is associated with SIB1 from NES cell, the UE can’t know which Cell A provides information of the target NES cell for on-demand SIB1 operation. It would be a burden for the UE to search the Cell A to acquire information for on-demand SIB1, and it would also bring additional latency. To solve the burden and latency to search the Cell A, pdcch-ConfigSIB1 can be used to indicate information of Cell A, like the frequency location of SSB associated with the Cell A.

When K_SSB is set to 30 for FR1 or 14 for FR2, there isn’t legacy UE behavior using pdcch-ConfigSIB1. For the usage of pdcch-ConfigSIB1, the following options can be taken into account for Rel-19 NES capable UE as captured in agreement.

- Option 1: Use it to indicate frequency assistance information to search SSB for Cell A

  - FFS: Details on the range/granularity of the frequency assistance information

  - FFS: Potential impact to RAN3

- Option 2: Use it to indicate searchSpaceZero and controlResourceSetZero of OD-SIB1

- Option 3: Above feature is not supported.

There are pros. and cons. for each option. With Option 2, to reduce the payload size of UL WUS configuration on Cell A, pdcch-ConfigSIB1 can be used to indicate type 0 PDCCH monitoring occasion. Option 3 has no spec impact since it will be same as legacy UE behavior. But, with both Option 2 and Option 3, UE burden to search Cell A remains. In terms of Option 1, to solve issue on UE complexity and latency to search Cell A, pdcch-ConfigSIB1 can be used to indicate frequency location of CD-SSB for Cell A with overhead to indicate type 0 PDCCH monitoring occasion in UL WUS configuration. Regarding the range/granularity of the frequency assistance information, to support a scenario that Cell A has large coverage with low carrier frequency and NES cell provides good data rate with high carrier frequency, at least it should be used to indicate out of band of frequency location, and it can have GSCN level of granularity to indicate SSB of Cell A supporting initial access.

Proposal 1: Support to use pdcch-ConfigSIB1 to indicate frequency assistance information for searching SSB for Cell A, when K_SSB is set to 30 for FR1 or 14 for FR2.

- Type0-PDCCH monitoring occasion for on-demand SIB1

In the previous meeting [3], the following agreements have been made to define reference time point for the window starting time and duration of the time window for monitoring Type0-PDCCH occasion.

Agreement The reference time point to determine the window starting time for on-demand SIB1 is based on the RAR window for UL WUS wherein UE successfully received a RAR. - FFS: Use starting or ending slot of the RAR window as reference time Agreement The duration of the time window for on-demand SIB1 is indicated in the UL WUS configuration. - Unit of the duration is in slot - FFS: Starting time

For reference time point, remaining issue is whether to use starting or ending slot of the RAR window. From our understanding on the purpose of RAR, it would be acknowledgement response to UL WUS as legacy RAR operation for OSI and RACH procedure. After receiving RAR within the RAR window, UE will start to monitor Type0-PDCCH occasions at least after reference time point. Rather than designing a new mechanism, it is good to count on what current spec supports for OSI and RACH procedure. Hence, we support the ending slot of the RAR window as reference time.

Proposal 2: Support the ending slot of the RAR window as reference time.

Regarding the starting time, it is necessary to reflect RAR processing time. Taking into account the reference time point and start time, the window start time for On-Demand SIB1 can be determined. We think pre-determined value for RAR processing time seems sufficient as starting time without additional bits in UL WUS configuration similar as legacy PDSCH processing time (e.g., NT,1).

Proposal 3: Support pre-determined value for starting time without indication in UL WUS configuration.

- Details of switching of on-demand SIB1 transmission state

In the previous meeting [4], the following proposal has been discussed to study the details of switching of on-demand SIB1 transmission state.

FL Proposal 7-1-6 (Revised by Huawei) RAN1 study the details of switching of OD-SIB1 transmission state between “SIB1 is currently not being broadcasted”, and the following cases of “SIB1 is currently being broadcasted” and visa versa - Case 1: NES cell becomes a normal cell. It means that the OD-SIB1 function is switched off. - Case 2: SIB1 is currently being broadcasted due to the UL WUS sent by another UE, and the broadcasted SIB1 is limited to a time window with an indicated duration - Case 3: NES cell is updating the SIB1, by broadcast SIB1 in the SI modification period, but the OD-SIB1 function is still on. - Case 4: normal cell becomes NES cell. It means that the OD-SIB1 function is switched ON.

As stated above, according to SIB1 transmission, multiple gNB states are considered for discussion purpose. From the gNB perspective, at least three different states, e.g. State A) No SIB1 transmission, State B) OD-SIB1 transmission and State C) periodic always-on SIB1 transmission as legacy, can be taken into account as depicted in Figure 1. First, regarding the State A, gNB will not transmit any SIB1 as NES cell to save energy. Second, while State B, gNB can transmit OD-SIB1 within time window as a response to UL WUS by UE. Last, in State C, gNB can always transmit SIB1 periodically as legacy non-NES operation. Each state can be switched by gNB and/or UE-triggering. However, in UE perspective, depending on states, different UE behaviors should be applied in order to avoid unnecessary SIB1 decoding and determine whether to request OD-SIB1. Therefore, RAN1 need to further clarity the UE behavior and study on how/whether to indicate switching of states described in Figure 1.

Figure 1: Illustration of SIB1 transmission states, including no SIB1 transmission/on-demand SIB1 transmission for NES cell and periodic SIB1 transmission for non-NES cell (legacy)

Proposal 4: RAN1 further studies the UE behavior and how/whether to indicate switching of SIB1 transmission states between the following states

- State A: No SIB1 transmission

- State B: OD-SIB1 transmission within time window

- State C: Periodic always-on SIB1 transmission as legacy SIB1 operation

- Contents of UL WUS configuration

In previous meetings [2], [3], the following agreements have been made for the required parameters/contents for UL WUS configuration.

Agreement For the purpose of “to which cell does the configuration applies”, at least the following parameters are included in the UL-WUS configuration. Purpose; Parameters. To which cell does the configuration applies; NES-CellId; PhysCellId. ARFCN-ValueNR. Note: ARFCN-ValueNR is used to indicate the absolute radio frequency channel number (ARFCN) for SSB of NES cell. Agreement For the purpose of “WUS transmission”, at least the following parameters are included in the UL-WUS configuration: - rsrp-ThresholdSSB - prach-RootSequenceIndex - msg1-SubcarrierSpacing - restrictedSetConfig Note: In legacy spec, the parameters above are under the IE RACH-ConfigCommon Agreement For the purpose of “WUS transmission”, at least the following parameters to indicate “frequency information for UL-WUS transmission” are included in the UL-WUS configuration. Purpose; Parameters. WUS transmission; frequencyBandList. absoluteFrequencyPointA. offsetToCarrier. p-Max. frequencyShift7p5khz (working assumption)ULSubCarrierSpacing. Agreement For the purpose of “WUS transmission”, at least the following parameters are included in the UL-WUS configuration. Purpose; Parameters. WUS transmission; SIB1-RequestConfig; ss-PBCH-BlockPower. SSB-positionInBurst. tdd-UL-DL-ConfigurationCommon. rach-OccasionsSIB1; Prach-ConfigurationIndex. msg1-FDM. msg1-FrequencyStart. zeroCorrelationZoneConfig. preambleReceivedTargetPower. preambleTransMax. powerRampingStep. ra-ResponseWindow. ssb-perRACH-Occasion. sib1-RequestPeriod. sib1-RequestResources; ra-PreambleStartIndex. ra-AssociationPeriodIndex. ra-ssb-OccasionMaskIndex. Note: - In legacy spec, ss-PBCH-BlockPower/SSB-positionInBurst/tdd-UL-DL-ConfigurationCommon are under the IE ServingCellConfigCommon/ServingCellConfigCommonSIB Msg1-FrequencyStart is with respect to the first RB of the UE carrier determined by offsetToCarrier and absoluteFrequencyPointA Agreement The parameter “n-TimingAdvanceOffset” is included in the UL-WUS configuration. Agreement At least for RO validation determination for WUS transmission, the parameter “ssb-PeriodicityServingCell” is included in the UL-WUS configuration at least for TDD system. - FFS: FDD system Agreement Confirm the working assumption below from RAN1 #118-bis to include it in the UL-WUS configuration. - (working assumption) ULSubCarrierSpacing

To reduce unnecessary UE behavior to determine accessibility on NES cell, UL WUS configuration can include accessibility information of NES cell, like cell selection information and accessible UE type. By informing the accessibility to UE, it can prevent not only for UE to request and receive on-demand SIB1 on NES cell, which doesn’t support the UE, but also for gNB to save energy consumption for transmission triggered by non-accessible UE.

Furthermore, the following remaining RRC parameters will be needed for supporting on-demand SIB1 operation:

- For initial UL BWP for the configuration of RO for UL WUS, the following additional RRC parameter is still needed with rach-ConfigCommon agreed in previous meeting at least for FDD case:

  - genericParameters in initialUplinkBWP

Proposal 5: Support the following additional RRC parameters in UL WUS configuration:

- Remaining configuration related to initial UL BWP for the configuration of RO:

  - genericParameters in initialUplinkBWP

- Configurations related to accessibility, e.g. cell selection information and accessible UE type

Note: details are up to RAN2

- List of RRC parameters

In the previous meetings, the several RRC parameters have been agreed for UL WUS configuration. In general, RRC parameters for UL WUS configuration should be newly added in a new SIB but they would be inherited as legacy RRC parameters related to RACH procedure. Additionally, the list of RRC parameters newly introduced for Rel-19 NES is proposed as below:

| New or existing? | Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | Required for initial access or IDLE/INACTIVE | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| New | odsib1-WindowDuration | Duration of the time window for on-demand SIB1 | INTEGER (20…160) | 160ms | Per cell | Yes |  |  |
| New | odsib1-kSSB-FR1 | Frequency domain offset between SSB and the overall resource block grid in number of subcarriers. | INTEGER (0~23) |  | Per cell | Yes |  |  |
| New | odsib1-kSSB-FR2 | Frequency domain offset between SSB and the overall resource block grid in number of subcarriers. | INTEGER (011) |  | Per cell | Yes |  |  |
| New | odsib1-searchSpaceZero | Parameters of the common SearchSpace#0. The values are interpreted like the corresponding bits in MIB pdcch-ConfigSIB1. | INTEGER (0~15) |  | Per cell | Yes |  |  |
| New | odsib1-controlResourceSetZero | Parameters of the common CORESET#0. The values are interpreted like the corresponding bits in MIB pdcch-ConfigSIB1. | INTEGER (0~15) |  | Per cell | Yes |  |  |

- Conclusion

In this contribution, we discussed details on supporting on-demand SIB1 operation for UEs in idle/inactive mode. The following proposals were made:

Proposal 1: Support to use pdcch-ConfigSIB1 to indicate frequency assistance information for searching SSB for Cell A, when K_SSB is set to 30 for FR1 or 14 for FR2.

Proposal 2: Support the ending slot of the RAR window as reference time.

Proposal 3: Support pre-determined value for starting time without indication in UL WUS configuration.

Proposal 4: RAN1 further studies the UE behavior and how/whether to indicate switching of SIB1 transmission states between the following states

- State A: No SIB1 transmission

- State B: OD-SIB1 transmission within time window

- State C: Periodic always-on SIB1 transmission as legacy SIB1 operation

Proposal 5: Support the following additional RRC parameters in UL WUS configuration:

- Remaining configuration related to initial UL BWP for the configuration of RO:

  - genericParameters in initialUplinkBWP

- Configurations related to accessibility, e.g. cell selection information and accessible UE type

Note: details are up to RAN2

- References:

- RP-242354, “Revised WID: Enhancements of network energy savings for NR”

- Chairman’s notes, RAN1#118bis, October 2024.

- Chairman’s notes, RAN1#119, November 2024.

- R1-2410681, “FL summary 5 for on-demand SIB1 in idle/inactive mode”, Moderator (MediaTek), RAN1#119, November 2024.