3GPP TSG RAN WG1 #120			R1-2501062

Athens, Greece, February 17th – 21st, 2025

Source:	Sharp

Title:	Support of (e)RedCap UEs with NR NTN

Agenda Item:	9.11.2

Document for:	Discussion and Decision

# Introduction

Supporting (e)RedCap UEs was discussed in the previous meetings and the following was agreed [1-6]:

RAN1#116: Agreement Study at least the following scenarios for (e)RedCap HD-FDD UEs for NTN: - Whether existing handling rules for the following cases should be reused or updated when taking into account TA mismatch between actual TA used by UE and assumed TA at the gNB based on available TA report: - […] - Case 3: Semi-statically configured DL reception collides with semi-statically configured UL transmission - Case 4: Dynamically scheduled DL reception collides with dynamic scheduled UL transmission RAN1#116-bis: Observation To avoid the occurrence of error cases 3 and 4 through network scheduling, there are less resources available for a scheduled HD-FDD RedCap/eRedCap UE in NTN compared to TN when there is TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB. RAN1#117: Conclusion For Rel-19 HD-FDD RedCap/eRedCap UE in NTN, the issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB should be mitigated for collision cases 3 and 4. RAN1#118: Agreement The collision case 3 (Semi-statically configured DL reception collides with semi-statically configured UL transmission) can’t be assumed as error case. When the collision happens at UE side for collision case 3, one of the following options on priority rules is supported. - Option A: the priority of keeping one direction and overriding another direction is based on different use cases (e.g. the collision happening in different channel types or different scheduling cases) without network signalling - It is not precluded that some or all use cases are left to UE implementation regarding whether DL reception or UL transmission is dropped - Note: it is not precluded that the same direction is prioritized for all different use cases - Option B: network indicates the priority configuration to the UE - It is not precluded that some use cases are left to UE implementation regarding whether DL reception or UL transmission is dropped - It is not precluded to define default rules, if necessary RAN1#118-bis: Conclusion The different use cases (e.g. the collision happening in different channel types or different scheduling cases) from RAN1#118 agreement for collision case 3 (Semi-statically configured DL reception collides with semi-statically configured UL transmission) consist of pairs of one semi-statically configured DL reception and one semi-statically configured UL transmission, among: - Semi-statically configured DL reception includes Type-0/0A/0B/1/2-PDCCH CSS and dedicated semi-statically configured PDCCH USS/PDSCH SPS/CSI-RS/PRS/[Type 3 PDCCH CSS] - Semi-statically configured UL transmission includes semi-statically configured PUSCH/PUCCH/SRS RAN1#119: Agreement For Rel-19 HD-FDD (e)Redcap UE in RRC connected mode, for collision case 3, - Handling of collision with Type-0/0A/1/2-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note. - For other use cases, default priority rule for collision case 3 in RRC-Connected mode is that DL is prioritized. - Network is allowed to indicate UL overriding DL for all the other use cases above - This is signaled by RRC configuration Agreement The collision case 4 (Dynamically scheduled DL reception collides with dynamic scheduled UL transmission) is not considered as an error case for Rel-19 HD-FDD (e)Redcap UE in RRC connected mode. - FFS whether to prioritize UL or prioritize DL

In this paper, we provide views on outstanding issues on collision resolution due to TA mismatch.

# Discussion

## Priority rules

In the previous meetings, several collision cases were discussed. For cases 1, 2, 5, and 6, existing priority rules can be reused, while cases 3 and 4 were determined to require further attention. Later, in RAN1#119, case 3 was agreed to be handled by a combination of statis priority rules, RRC configured priority rules, and UE implementation.

Furthermore, TA reporting for (e)RedCap UEs was argued to be beneficial in mitigating TA mismatches, although the impact on complexity and power consumption was not studied. It was argued that further investigation could be needed on complexity, power consumption, and signalling overhead for TA reporting enhancements; whether to establish default rules or allow UE implementation flexibility for prioritization in collision cases 3 and 4; additional solutions or schemes to enhance TA reporting, such as finer granularity, smaller offset thresholds, and new triggering mechanisms; and benefit assessments for each TA reporting enhancement.

As suggested in the previous meeting, discussing and deciding about static priority rules for different channel types or scheduling cases may require a significant effort to decide whether DL should be prioritized over UL, or vice versa, particularly if such a priority rule is specified in the standard and unchangeable. Different scenarios with different characteristics of the traffic and QoS requirements do demand different priority rules, and hence a simple static priority rule for DL over UL or UL over DL is overly simplified and not acceptable. Therefore, we strongly support network configuration of priority rules. The configuration may be applicable to a whole cell or to specific UEs.

Proposal 1: Support network configuration of priority rules for collision case 3 (semi-static DL-UL collision) and collision case 4 (dynamic DL-UL collision). The configuration may be applicable to a whole cell or to specific UEs.

With respect to the priority rules, simple priority rules such as prioritizing DL over UL or vice versa are overly simplified and inflexible. RAN1 should consider indication of priority rules by the network based on DL and UL signal/channel types in a DL-UL collision. In addition, the network may indicate a default prioritization of DL or UL for any channels/signals for which no priority is indicated by the configuration.

Proposal 2: Support network configuration of different priorities for different channels and signals for collision case 3 and collision case 4. A default priority by configuration can also be considered.

As an additional or alternative mechanism, the timing of the DCI messages scheduling DL and UL channels can be used by the UE to determine the priority between DL and UL. For example, if the UE receives the DCI message scheduling the DL later than the DCI message scheduling the UL, then the UE can prioritize the DL over the UL, or vice versa. This approach simplifies the specification compared to fully configurable priority rules, while still allowing the network to have dynamic control over UE prioritization.

Proposal 3: Consider the timing of receiving the DCI messages by the UE for determining the priority of the DL or UL scheduled by the DCI messages in collision case 4.

## TA reporting enhancement

It was further discussed in the previous meetings that TA reporting could be enhanced to avoid DL-UL collisions or reduce their probability. We support TA reporting enhancements. For example, reporting only the difference between the latest reported TA and the current TA allows the UE to report the TA more frequently with smaller overhead. Furthermore, DL-UL collisions resulting from TA mismatch can be reported along with the TA reporting.

Proposal 4: Support TA reporting enhancements to avoid, or reduce the probability of, DL-UL collisions due to TA mismatch. For example, consider reporting the TA difference and indication of DL-UL collisions.

# Conclusion

In this paper, we discussed some of the outstanding issues for support of (e)RedCap UEs with NR NTN and proposed the following:

Proposal 1: Support network configuration of priority rules for collision case 3 (semi-static DL-UL collision) and collision case 4 (dynamic DL-UL collision). The configuration may be applicable to a whole cell or to specific UEs.

Proposal 2: Support network configuration of different priorities for different channels and signals for collision case 3 and collision case 4. A default priority by configuration can also be considered.

Proposal 3: Consider the timing of receiving the DCI messages by the UE for determining the priority of the DL or UL scheduled by the DCI messages in collision case 4.

Proposal 4: Support TA reporting enhancements to avoid, or reduce the probability of, DL-UL collisions due to TA mismatch. For example, consider reporting the TA difference and indication of DL-UL collisions.

# References

- Chairman’s notes, RAN1#116, February 2024.

- Chairman’s notes, RAN1#116-bis, April 2024.

- Chairman’s notes, RAN1#117, May 2024.

- Chairman’s notes, RAN1#118, August 2024.

- Chairman’s notes, RAN1#118-bis, October 2024.

- Chairman’s notes, RAN1#119, November 2024.