3GPP TSG RAN WG1 #120						                       	      R1-2501273

Athens, Greece, Feb 17th – Feb 21st , 2025

Agenda item:       9.1.2

Source:                CEWiT

Title                     Discussion on specification support for AI/ML positioning accuracy  enhancement

Document for:    Discussion and Decision

__________________________________________________________________

# Introduction

In RAN#102 plenary meeting, the following have been identified as the objectives of normative work for enhancing positioning accuracy using AI/ML:

Provide specification support for the following aspects: - AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]: - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback - Identification related signalling is part of the above objective - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models - Signalling mechanism of applicable functionalities/models - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2 Provide specification support for Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

This contribution discusses our views on how to select the no. of input samples, starting point of the sampling window,  issues related to timing information representation of the training samples, necessity of including quality indicator of the measurement and labels and feasibility of different cases for label based model monitoring metric calculation for UE sided models.

# 2. Discussion

## 2.1 Sample vs Path based reporting

During the study item phase, the majority of the results submitted by companies were related to time domain sample based channel measurements where the measurement samples are collected at a fixed sampling rate.

The path-based measurement as defined, is based on the detected channel paths from the raw channel, which is itself challenging to estimate the actual paths in real deployment scenario. Although in path-based method of measuring channel, the reporting overhead will be reduced but the positioning accuracy will be affected due to loss of channel information. Whereas in sample-based channel measurement depending on the sampling rate, samples are collected across constant interval of sampling time and hence will carry redundant channel information.

Additionally, the performance of the Path based method  will also depend on the path detection algorithm. Different path detection algorithm (e.g., based on strongest power etc.) will have different AI/ML model performance depending on how accurately it can capture the channel.

Proposal 1: In Rel-19 AI/ML based positioning, regarding the time domain channel measurements, time domain sample-based measurement reporting is preferred.

In RAN1#117 meeting, companies suggested an unified input measurement approach where measurement times of observed time domain channel response are integral multiples of timing granularity T, where T=2kxTc, ,Tc  being the basic time unit for NR  which equals1NFFT∗Δfmax = 0.5086ns  for NFFT = 4096  and Δfmax= 480kHz and k representing the timing reporting granularity factor or  the timing resolution for the channel samples.  Tc can thus be seen as the sampling time for the maximum allowable subcarrier spacing of 480 kHz with an FFT size equal to 4096.

Input samples to AI/ML model are further selected from within a  sampling window of size T * Nt   where Nt being the consecutive time samples within the window from which Nt’ samples of highest power are  selected .The selection of Nt’ samples can be based on methods such as thresholding of power  etc. However, the effective information content within  the sampling window will differ depending on the delay spread  of the channel .

Positioning use cases where model training is performed at the UE side (Case 1 and Case 2a), there is often possibility of lack of computational resources or memory capacity while  training. Therefore, it is advantageous to find efficient methods of preprocessing the model input to minimize resource consumption without compromising on training accuracy and signalling overhead.

Similarly, for the LMF side  model (Case 2b , Case 3b), where channel measurements needs to be transferred from UE to  LMF via LPP  and gNB/TRP via NRPPa, reducing reporting overhead would be beneficial.

One potential solution is to request for capability information and supported functionalities, including details about the UE’s processing power and available memory, by LMF to UE. Based on the capability report and scenario related information,  LMF can recommend sample selection criteria and optimal range for Nt' and k to UE.

The value of k will determine the timing reporting granularity. For smaller value of k, the reporting granularity will be smaller, hence  overhead will be more. Similarly for higher value, the overhead will be less  but fewer no. of actual channel paths will be captured. Release 16 specifications supports value range of  ‘k’ from  0 to 5  allowing timing reporting granularity T to vary between 0.50863 ns and 16.27 ns.  The LMF can leverage this existing range and provide recommendations to the UE/gNB.

Additionally, the UE can calculate the appropriate 'k' value based on the SCS and RS bandwidth and then notify the LMF. However, the final decision on the value of 'k' should be left to the UE, as it is specific to each UE's requirements.

Proposal 2: Based on the UE capability report and scenario related information, in Case 2b,  the LMF can recommend sample selection criteria,  optimal ranges for Nt' and k to UE.

Proposal 3: Support reusing the existing value range for ‘k’ from {0..5} specified in the specification and provide recommendations  by LMF to  UE/gNB.

Observation 1: The final decision of selection of ‘Nt’ and ‘k’ should be  upto UE’s decision and specific to each UE.

During RAN1#118-bis meeting,  issues related to defining the channel grid and identifying a starting point of the Nt consecutive samples are discussed.

Assuming, first detected path is in one of the channel samples, the first path detection time  with respect to a reference time should be the starting of the sampling window for the training/inference data. The detection of the first path at a receiver (UE/gNB) generally depends on the receiver's              implementation. This includes factors such as the synchronization level needed between the receiver and transmitter clocks, as well as the specific point in the IQ samples where the receiver initiates first-path detection.

Proposal 4: Support   considering time at which the first path is detected  with respect to a reference time  as the starting of the sampling window for  model input data.

## 2.2 Training Data Collection

### 2.2.1 Content of Training data samples

Agreement For training data collection of AI/ML based positioning, if a training data sample contains both Part A and Part B, RAN1 assumes that Part A and Part B in one training data sample are: - for a same UE (PRU or Non-PRU UE), and - for a same location associated with Part B. Note: the association can be discussed

The AI/ML positioning training dataset consists of two parts: Part A, which includes channel measurements and their associated information, and Part B, which contains ground truth labels or UE positions (PRU or Non-PRU UE) along with related information. Given the limited availability of ground truth (GT) labels, the AI/ML model is initially trained using the labeled data from PRUs. The trained model can then be used to predict labels for the unlabeled channel measurements in the same scenario, after which the model is retrained using the entire dataset ( semi-supervised learning). This will enable both PRU and non-PRU UEs to perform both labeled and unlabeled data collection.

Additionally, the specific types of information a non-PRU UE can share should be defined to obtain the necessary signaling and information exchange with the network (LMF) for accurate positioning.

Typically, LMF knows the positions of PRUs in a region and can perform conventional positioning method to calculate the labels from the collected channel measurements and associated information from non PRU UEs (assuming its privacy is preserved) and estimate its position.

Association of measurement and labels

To associate the measurement (and its related data) to collected labels (and its related data), the data generating entities can record its ID (UE ID/ TRP ID/ LMF ID), RS configurations and IDs, Area ID along with the collected measurement, labels and its corresponding time stamps. In this way, channel measuring entity can map the channel measurements to the associated labels collected by label generating entity.

Proposal 5: Permissions/Privacy and positioning implementation related to non PRU UE needs to be defined and discussed.

Proposal 6:  Channel Measurements can be performed by both PRU and non PRU UE.

Proposal 7:  Semi supervised learning can be used for positioning Non PRU UE in scenarios where lesser PRUs are deployed in an area.

Proposal 8: To associate the measurement and its related data to collected labels and 	its related data, the data generating entity can record its ID (UE ID/ TRP ID/ LMF ID), RS configurations and its corresponding IDs,  Area ID along with the collected measurement, labels and its corresponding time stamps.

### 2.2.2 Timing information related to training samples

The timestamp associated with channel measurements and labels can be used to create a link  between them, especially when the entities generating the measurements and labels differ or when the measurement and labels are generated at different time instance.

Existing specification for legacy NR positioning supports reporting the time stamps from UE to LMF using field ‘measurementReferenceTime’  where either of SFN time  and UTC time is reported.

Similarly for NPPa reporting from gNB to LMF, supports measurement time reporting  in terms of relative time with respect to  00:00:00 on 1 January 1900  along with SFN information. The legacy IE NR-TimeStamp from TS 37.355, reported by the UE to the LMF, specifies  the UE measurement associated  with a time stamps using NR system frame numbers (SFN). This timestamp includes the NR slot number within the SFN and the NR symbol index within the slot. Similarly, the TimeStamp reported by the gNB to the LMF includes the SFN, SlotIndex, and measurement time.

SFN is monitored by UE to keep track of time with respect to network.. The range of SFN in NR is 0-1023 and as the length of a single frame is 10ms, total SFN range is 10.24s. After every 10.24s, SFN starts a new SFN cycle. When timestamp is represented only in terms of  SFN, it becomes less reliable as the SFN would possibly wrap around more than once while the time stamp is recorded.

Therefore it is preferred to capture UTC time everytime SFN completes its cycle .The UTC time as defined in the specification takes the form of YYMMDDhhmmssZ . This information should be reported in comparable format with the SFN time.

Proposal 9:  Support reporting UTC time to LMF everytime SFN completes its cycle.

### 2.2.3 Assistance Information for Data Collection

The channel measurements for both uplink and downlink are obtained through the Reference Signals (either Downlink PRS or Uplink SRS), depending on the use case for AI/ML positioning. These measurements can be performed by the UE itself, whether it is a PRU or non-PRU UE, in Case 1, Case 2a, or Case 2b, or by the gNB in Case 3a or Case 3b. Prior to the measurement collection process, it is essential to exchange assistance information/signals to the data collecting entity to gather the required quality information.

As discussed in RAN1#116-bis and RAN1#117 meetings, for different use case in AI/ML positioning, the following entities are assumed for measurement generation and label generation.

| Use cases | Measurement collecting entity | Label collecting entity | RS |
| --- | --- | --- | --- |
| Case 1 | PRU, Non-PRU UE | PRU, Non-PRU UE, LMF | DL-PRS |
| Case 2a | PRU, Non-PRU UE | PRU, Non-PRU UE, LMF | DL-PRS |
| Case 2b | PRU, Non-PRU UE | PRU, Non-PRU UE, LMF | DL-PRS |
| Case 3a | gNB/TRP | LMF | UL-SRS |
| Case 3b | gNB/TRP | PRU, Non-PRU UE, LMF | UL-SRS |

For all the above use cases, RS configuration and Area validity from which training data is to be collected should be provided as assistance to the data-collecting entity. The existing Information Element (IE) AreaID-CellList (from TS 37.355) can be reused to define the validity of the area as it provides the list of Cell-IDs for the TRPs in a specific network area. These information can be signalled via LPP for Case 1, Case 2a and Case 2b from LMF to UE and NRPP from LMF to NgRAN.

​​

Additionally, depending on the deployment scenario, the network should also provide a range/margin for quality-indicating metrics (e.g., SNR, channel estimation error, etc., for channel measurement), UE's available storage/ UE capability (if the UE is collecting data), and the type of input given to the model for any required preprocessing of dataset.

NR-PositionCalculationAssistance, which is used by the location server (LMF) to enable UE based downlink positioning, can be reused to provide assistance to UE for training datcollection for Case 1, Case 2a,  and Case 2b.

Proposal 10: For all the use cases (Case 1, Case2a, Case 2b, Case 3a, Case 3b),  RS configuration, related UE and gNB associations and Area validity from which training data is collected should be provided as assistance information to the data collecting entity.

Proposal 11: Depending on the deployment scenario, network should provide a range/margin for quality-indicating metrics (e.g., SNR, channel estimation error, etc., for channel measurement), entity’s available storage/ entity’s capability (data collecting entity), and the type of input given to the model for any required preprocessing of the dataset as assistance.

Observation 2:  Use of   IE  AreaID-CellList  for defining the validity of an area for data collection can be supported.

### 2.2.4 Quality indicators for Training Data

In previous RAN1 meetings, companies agreed on providing atleast channel measurement, related quality indicator and time stamps to LMF for Case 3b, but sending quality indicators with every measurement will increase reporting overhead, hence steps such as reducing resolution of the quality indication can be considered while reporting.

Quality indicator for training data can be for measurements and labels respectively. For channel measurements, it can be of  two type : timing quality and power quality. The availability of both of these parameters will ensure more confidence to the measurement.

The necessity to include quality indicator with training data is to ensure proper training of the AI/ML model.

As specified from RAN1#118-bis agreement, the existing IEs “NR-TimingQuality” for UE-to-LMF reporting and “Timing Measurement Quality” for gNB/TRP-to-LMF reporting can be used in Case 2b and Case 3b for timing quality reporting in Part A.

However, for reporting measurement power quality, SNR relative to a predefined threshold can be specified.

Proposal 12 :  For reporting measurement power quality, SNR relative to a predefined threshold can be specified.

## 2.3  Model monitoring

### 2.3.1 Model monitoring in Case 3a

Agreement For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least: - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: whether other network entities can generate label for Case 3a is out of RAN1 scope. Agreement For AI/ML positioning Case 3a, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility of the following options. To provide information on how to generate information on ground truth label for each option. - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. - Note: Final selection of Option A and Option B is out of RAN1 scope, but RAN1 can make recommendation about the option(s), and potential support of Option A and/or Option B is pending RAN3 confirmation. - Note: Exact method to perform the monitoring metric calculation is up to implementation.

Monitoring metric for a model is the mathematical formulation that can determine or quantify the performance of the model.

As agreed in RAN1#116-bis, in Case 3a, NGRAN assisted, gNB side model, for AI/ML assisted positioning,  gNB/TRP does the channel measurements and its related generation of data using UL- SRS from UEs. The model output for AI/ML assisted positioning are  enhancement of input channel measurements such as LOS/NLOS indicator, likelihood of measurements or timing/angle information of  the measurements. Monitoring of AI/ML model is performed either  by monitoring the model output or model input depending on the implementation and signalling overhead.

If model monitoring is label based, LMF should calculate the monitoring metric in Case 3a, since the LMF 	has a wider view of the scenario and  can calculate the estimated ground truth labels.

If monitoring is label free, gNB is preferred to calculate the monitoring metric in order to reduce the reporting overhead. However, LMF can provide all the necessary assistance information for monitoring and performing model management.

| Model monitoring method | Measurement generating entity | Ground truth label generation entity | Monitoring Metric calculating entity |
| --- | --- | --- | --- |
| Label free | gNB/TRP | N/A | gNB/TRP |
| Label based | gNB/TRP | LMF | LMF |
|  |  | gNB/TRP | gNB using assistance from LMF |

Proposal 13 : How LMF generates the ground truth labels using model output for AI/ML assisted positioning should be discussed.

Proposal 14: For label free monitoring for Case 3a, gNB/TRP is preferred to perform the model monitoring metric calculation.

Proposal 15: For label-based monitoring for Case 3a, LMF is preferred to perform the model monitoring metric calculation.

After the entity calculates the monitoring metric depending on the model performance, management decisions such as model activation/deactivation, fine-tuning or the type of legacy method to falling back etc. are required to be made. These necessitates determining the decision-making flow for model management among different entities for various positioning use cases.

In Case 3a, the gNB can either decide or recommend lifecycle management (LCM) decisions for label-free model monitoring and communicate these decisions to the LMF.

For label-based model monitoring, decisions can be made either by gNB or  LMF.   Monitoring approach in this Case requires transferring of labels or information related to labels  from LMF to gNB.  gNB  with assistance from LMF, will have the knowledge about the training data along with its own model and hence can determine if the current AI/ML model needs to be updated. The  final decision made by gNB is notified to LMF.

Additionally, LMF can provide its recommendations such as which model to activate or which positioning method to fallback etc after receiving the management related decision from gNB.

Proposal 16: In case 3a, gNB is preferred to make model management decisions for its own model and notifies its decision to LMF.

Proposal 17: LMF can provide its assistance such as threshold for performance degradation of the AI/ML model  and recommendations such as which model to activate or which positioning method to fallback etc to gNB after receiving the management related decision from gNB .

### 2.3.2 Model monitoring for LMF side models

In positioning use cases such as UE-assisted/LMF-based positioning, the LMF side model (Case 2b) and NgRAN-assisted positioning, LMF side model (Case 3b), the LMF receives channel measurements from the UE and gNB for model training. In addition to the knowledge of PRU locations, LMF can also perform legacy positioning methods to compute ground truth labels from these measurements.  Additionally, the LMF is aware of the network side conditions and scenario for which the model is being trained and monitored.  Hence, LMF can take model management decisions based on its calculated monitoring metric and its determined threshold for measuring the performance degradation of the model for Case 2b and Case 3b.

Proposal 18: LMF is preferred to take model management decisions for Case 2b and Case 3b.

### 2.3.3 Model monitoring for UE side models

Following are the agreements from RAN1#116b and RAN1#118 meetings for model monitoring metric  calculation related to Case 1.

Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded.

Conclusion For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, - Option A-4 can be realized by implementation in a manner transparent to specification specification if the PRU sends information to the target UE side in a proprietary method. No further discussion on Option A-4.

| Option A | Monitoring metric calculating entity | Procedure | Requirements |
| --- | --- | --- | --- |
| A-1 | UE | Atleast Information on Ground truth label is generated by LMF and sent to target UE. Eg: target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. | LMF to generate information on GT labels from measurements received from target UE and/or gNB is through implementing legacy positioning methods to estimate the labels. Feasibility Check: Feasible but the quality of the estimated labels cannot be guaranteed for a NLOS scenario. |
| A-2 |  | At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. | Target UE receives assistance data from LMF related to existing UE based positioning. UE generates GT with UE based positioning using assistance data. Feasiblility check: Feasible but the quality of the GT labels cannot be guaranteed for NLOS scenario. |
| A-3 |  | Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. | Target UE can receive PRU measurement and location as assistance from LMF to target UE Feasible Check: Feasible in scenarios where PRU data is available. |

Agreement For model performance monitoring of AI/ML positioning Case 1, support at least: - Option A. The target UE side performs monitoring metric calculation. - The target UE may signal the monitoring outcome to the LMF. - FFS: content of monitoring outcome - FFS: Option B

In RAN1#119 meeting, The monitoring outcome for Case 1,  label based model monitoring method can be position error between the AI/ML estimated labels and ground truth (GT) labels or a monitoring decision informing the validity of the AI/ML model during inference.

| Option B | Monitoring metric calculating entity | Channel Measuring entity | Procedure | Requirements |
| --- | --- | --- | --- | --- |
| B-1 | LMF | UE | At least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. | Model Output labels are sent to LMF. 1. Assuming LMF has knowledge about the GT labels, LMF can calculate the monitoring metric calculation. 2. LMF calculates GT labels using legacy method. For NLOS scenarios, GT labels can be inaccurate in heavy non LOS scenario. B-1 is not accurate method for NLOS scenarios. |
| B-2 |  | PRU | PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. | The input channel measurement is transferred via LMF, LMF will have knowledge of inference channel measurement along with the corresponding model output. Hence it can compare the model output and PRU’s locations. B-2 is feasible in scenarios where PRU’s are available. |

Out of Option A and Option B for model monitoring metric calculation for UE sided models, it is always preferred for UE to do the metric calculation due to lesser overhead and easy access of data.

Proposal 19: Out of Option A and Option B for model monitoring metric calculation for UE sided models,  Option A is preferred for monitoring metric calculation.

Proposal 20: The monitoring outcome for Case 1,  label based model monitoring method can be quantified value such as position error between the AI/ML estimated labels and ground truth (GT) labels or a monitoring decision informing the validity of the AI/ML model during inference.

# 3. Conclusion

Proposal 1: In Rel-19 AI/ML based positioning, regarding the time domain channel measurements as model input, time domain sample-based measurement reporting is preferred.

Proposal 2: Based on the UE capability report and scenario related information, in Case 2b,  the LMF can recommend sample selection criteria,  optimal ranges for Nt' and k to UE.

Proposal 3: Support reusing the existing value range for ‘k’ from {0..5} specified in the specification and provide recommendations  by LMF to  UE/gNB.

Observation 1: The final decision of selection of ‘Nt’ and ‘k’ should be  upto UE’s decision and specific to each UE.

Proposal 4: Support   considering time at which the first path is detected  with respect to a reference time  as the starting of the sampling window for  model input data.

Proposal 5: Permissions/Privacy and positioning implementation related to non PRU UE needs to be defined and discussed.

Proposal 6:  Channel Measurements can be performed by both PRU and non PRU UE.

Proposal 7:  Semi supervised learning can be used for positioning Non PRU UE in scenarios where lesser PRUs are deployed in an area.

Proposal 8: To associate the measurement and its related data to collected labels and 	its related data, the data generating entity can record its ID (UE ID/ TRP ID/ LMF ID), RS configurations and its corresponding IDs,  Area ID along with the collected measurement, labels and its corresponding time stamps.

Proposal 9:  Support reporting UTC time to LMF everytime SFN completes its cycle.

Proposal 10: For all the use cases (Case 1, Case2a, Case 2b, Case 3a, Case 3b),  RS configuration, related UE and gNB associations and Area validity from which training data is collected should be provided as assistance information to the data collecting entity.

Proposal 11: Depending on the deployment scenario, network should provide a range/margin for quality-indicating metrics (e.g., SNR, channel estimation error, etc., for channel measurement), entity’s available storage/ entity’s capability (data collecting entity), and the type of input given to the model for any required preprocessing of the dataset as assistance.

Observation 2:  Use of   IE  AreaID-CellList  for defining the validity of an area for data collection can be supported.

Proposal 12:  For reporting measurement power quality, SNR relative to a predefined threshold can be specified.

Proposal 13 : How LMF generates the ground truth labels using model output for AI/ML assisted positioning should be discussed.

Proposal 14: For label free monitoring for Case 3a, gNB/TRP is preferred to perform the model monitoring metric calculation.

Proposal 15: For label-based monitoring for Case 3a, LMF is preferred to perform the model monitoring metric calculation.

Proposal 16: In case 3a, gNB is preferred to make model management decisions for its own model and notifies its decision to LMF.

Proposal 17: LMF can provide its assistance such as threshold for performance degradation of the AI/ML model  and recommendations such as which model to activate or which positioning method to fallback etc to gNB after receiving the management related decision from gNB .

Proposal 18: LMF is preferred to take model management decisions for Case 2b and Case 3b.

Proposal 19: Out of Option A and Option B for model monitoring metric calculation for UE sided models,  Option A is preferred for monitoring metric calculation.

Proposal 20: The monitoring outcome for Case 1,  label based model monitoring method can be quantified value such as position error between the AI/ML estimated labels and ground truth (GT) labels or a monitoring decision informing the validity of the AI/ML model during inference.

# 4. References

[1] R1-2410717,“Summary #4 of specification support for positioning accuracy enhancement

[2] R1-2410571,“Specification support for AI/ML for positioning accuracy enhancement, CEWiT”

[3] TS 37.355, “LTE Positioning Protocol”

[4] TS 38.455 ,“ NR Positioning Protocol A (NRPPa) ”