3GPP TSG-RAN WG1 Meeting #120	R1-2501326

Athens, Greece, February 17 – 21, 2025

Agenda Item:	7

Source:	Huawei, HiSilicon

Title:	Discussion on Type0-PDCCH CSS set monitoring

Document for:	Discussion and Decision

Introduction

In this contribution, we clarify the UE behavior related to Type0-PDCCH CSS monitoring.

Discussion

According to TS 38.213 [1], a UE can be configured with a Type0-PDCCH CSS set by different higher-layer parameters

Section 10.1, TS 38.213 A set of PDCCH candidates for a UE to monitor is defined in terms of PDCCH search space sets. A search space set can be a CSS set or a USS set. A UE monitors PDCCH candidates in one or more of the following search spaces sets - a Type0-PDCCH CSS set on the primary cell of the MCG configured by - pdcch-ConfigSIB1 in MIB or by searchSpaceSIB1 in PDCCH-ConfigCommon or by searchSpaceZero in PDCCH-ConfigCommon for a DCI format 1_0 with CRC scrambled by a SI-RNTI, or - searchSpaceZero by providing searchSpaceID=0 for searchSpaceMCCH or searchSpaceMTCH for a DCI format 4_0 with CRC scrambled by a MCCH-RNTI or a G-RNTI for broadcast, or - searchSpaceZero by providing searchSpaceID=0 for searchspaceMulticastMCCH for a DCI format 4_0 with CRC scrambled by a Multicast MCCH-RNTI, or by searchSpaceMulticastMTCH for a DCI format 4_1 with CRC scrambled by a G-RNTI for multicast in RRC_INACTIVE state

As illustrated in Figure 1, a UE may be configured as follows for an active BWP

- searchSpaceSIB1 is not provided by PDCCH-ConfigCommon for the active DL BWP

- CORESET#0 bandwidth is within the active DL BWP, and the active DL BWP has the same SCS configuration and cyclic prefix as the initial DL BWP

Figure 1 Configuration of active DL BWP, initial DL BWP and CORESET#0

In this case, according to the description highlighted in yellow in Section 10.1 of TS38.213 [1], the UE does not monitor the PDCCH candidates in the Type0-PDCCH CSS set since searchSpaceSIB1 is not configured.

However, according to the description highlighted in green in Section 12 of TS38.213 [1], the UE shall monitor the PDCCH candidates for the Type0-PDCCH CSS set since the CORESET#0 bandwidth is within the active DL BWP, and the active DL BWP has the same SCS configuration and cyclic prefix as the initial DL BWP.

Therefore, the UE behaviors appear to be inconsistent in different parts of the specification

Section 10.1, TS 38.213 For a DL BWP, if a UE is not provided searchSpaceSIB1 for Type0-PDCCH CSS set by PDCCH-ConfigCommon, the UE does not monitor PDCCH candidates for a Type0-PDCCH CSS set on the DL BWP. The Type0-PDCCH CSS set is defined by the CCE aggregation levels and the number of PDCCH candidates per CCE aggregation level given in Table 10.1-1.

Section 12, TS 38.213 If a UE is provided controlResourceSetZero and searchSpaceZero in PDCCH-ConfigSIB1 or PDCCH-ConfigCommon, the UE determines a CORESET for a search space set from controlResourcesetZero as described in clause 13 and for Tables 13-0 through 13-10, and determines corresponding PDCCH monitoring occasions as described in clause 13 and for Tables 13-11 through 13-15. If the active DL BWP is not the initial DL BWP, the UE determines PDCCH monitoring occasions for the search space set only if the CORESET bandwidth is within the active DL BWP and the active DL BWP has same SCS configuration and same cyclic prefix as the initial DL BWP.

Furthermore, in RRC_CONNECTED mode, a UE is required to monitor PDCCH candidates for the DCI format 0_0/1_0 scrambled by C-RNTI in CSS set according to the following description. Therefore, the ambiguity in Type0-PDCCH CSS monitoring may impact the reception of unicast DCI in this case.

Section 10.1, TS 38.213 If a UE is provided - one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, and - a C-RNTI, an MCS-C-RNTI, or a CS-RNTI the UE monitors PDCCH candidates for DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI, the MCS-C-RNTI, or the CS-RNTI in the one or more search space sets in a slot where the UE monitors PDCCH candidates for at least a DCI format 0_0 or a DCI format 1_0 with CRC scrambled by SI-RNTI, RA-RNTI, MsgB-RNTI, or P-RNTI.

Observation: According to the current specification, it is unclear whether UE shall monitor PDCCH candidates for a Type0-PDCCH CSS set in the following case

- searchSpaceSIB1 is not provided by PDCCH-ConfigCommon for the active DL BWP

- CORESET#0 bandwidth is within the active DL BWP, and the active DL BWP has the same SCS configuration and cyclic prefix as the initial DL BWP

According to the agreement in RAN1#94bis, when the CORESET#0 bandwidth is within the active DL BWP, and the active DL BWP has the same SCS configuration and cyclic prefix as the initial DL BWP, the UE shall monitor PDCCH candidates in the Type0-PDCCH CSS. In this case, our understanding is that searchSpaceSIB1 should also be configured for the active DL BWP in this case. Otherwise, the gNB will not configure the active DL BWP containing the CORESET#0 bandwidth.

Agreements: (RAN1#94bis) - If a UE is provided with the higher layer parameters controlResourceSetZero and searchSpaceZero in PDCCH-ConfigSIB1 or PDCCH-ConfigCommon on the initial DL BWP, the UE determines the corresponding control resource set and search space set according to the procedure described in Subclause 13 of TS 38.213 - For a search space set configured for the active DL BWP other than the initial DL BWP, - The UE monitors PDCCH candidates in the search space set associated with controlResourceSetZero only if the bandwidth determined for controlResourceSetZero in Subclause 13 of TS 38.213 is contained in the active DL BWP and the active DL BWP is configured with the same SCS and CP as controlResourceSetZero - The UE does not expect to be configured with more than 2 CORESETs in a DL BWP if the UE is required to monitor PDCCH candidates in the search space set associated with controlResourceSetZero in the DL BWP

Therefore, we propose to make the following clarification in RAN1

Proposal: For an active DL BWP, if the CORESET#0 bandwidth is within the active DL BWP, and the active DL BWP has the same SCS configuration and cyclic prefix as the initial DL BWP, searchSpaceSIB1 shall be provided by PDCCH-ConfigCommon for the active DL BWP.

Conclusion

In this contribution, we discussed an ambiguity issue of monitoring type0-PDCCH CSS. Based on the discussion, we have the following observations and proposals:

Observation: According to the current specification, it is unclear whether UE shall monitor PDCCH candidates for a Type0-PDCCH CSS set in the following case

- searchSpaceSIB1 is not provided by PDCCH-ConfigCommon for the active DL BWP

- CORESET#0 bandwidth is within the active DL BWP, and the active DL BWP has the same SCS configuration and cyclic prefix as the initial DL BWP

Proposal: For an active DL BWP, if the CORESET#0 bandwidth is within the active DL BWP, and the active DL BWP has the same SCS configuration and cyclic prefix as the initial DL BWP, searchSpaceSIB1 shall be provided by PDCCH-ConfigCommon for the active DL BWP.

- References

- TS 38.213, Physical layer procedures for control (Release 18).