3GPP TSG RAN WG1 #120			R1-250XXXX

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.4

Source:	Ad-Hoc Chair (Huawei)

Title:	Session notes for 9.4 Solutions for Ambient IoT in NR

Document for:	Discussion, Decision

## Solutions for Ambient IoT (Internet of Things) in NR

Please refer to RP-243326 for detailed scope of the WI.

[120-R19-A-IoT] Email discussion on Rel-19 Ambient IoT – Jingwen (CMCC)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2500286	Ambient IoT Work Item work plan	CMCC, Huawei, T-Mobile USA

### Physical channels design – modulation aspects

Including discussions on CP handling for R2D.

R1-2500033	Modulation aspects for Ambient IoT	Ericsson

R1-2500044	Discussion on modulation aspects for A-IoT physical channel	FUTUREWEI

R1-2500124	Discussion on Ambient IoT modulation	ZTE Corporation, Sanechips

R1-2500153	Physical channels design on modulation	Huawei, HiSilicon

R1-2500169	Discussion on modulation aspects of physical channels design for Ambient IoT	Spreadtrum, UNISOC

R1-2500224	Ambient IoT physical channel design and modulation	CATT

R1-2500259	Discussion on physical channels design about modulation aspects for Ambient IoT	China Telecom

R1-2500287	Discussion on modulation aspects of physical channel design	CMCC

R1-2500349	Discussion on Modulation Aspects of Physical Channels Design	vivo

R1-2500394	AIoT Physical channels design - modulation aspects	Nokia

R1-2500424	 Discussion on modulation aspects for Ambient IoT physical design	TCL

R1-2500450	Discussion on modulation aspects of A-IoT	OPPO

R1-2500479	Discussion on modulation aspects for A-IoT physical channel	Tejas Network Limited

R1-2500497	Discussion on Physical Channel Modulation Aspects for Ambient-IoT	EURECOM

R1-2500583	A-IoT Modulation Aspects	Panasonic

R1-2500610	Discussion on modulation aspects of ambient IoT	NEC

R1-2500651	Modulation aspects for Ambient IoT	Sony

R1-2500732	Discussion on modulation aspects for Ambient IoT	Xiaomi

R1-2500778	On modulation aspects for Ambient IoT	Apple

R1-2500850	Views on Physical channels design – modulation aspects	Samsung

R1-2500936	Modulation for R2D and D2R	Fujitsu

R1-2500947	Discussion on D2R modulation schemes for A-IoT	IIT Kharagpur

R1-2500949	A-IoT PHY layer design - waveform and modulation aspects	LG Electronics

R1-2501016	A-IoT - PHY modulation aspects	MediaTek Inc.

R1-2501071	Discussion on modulation aspects for Ambient IoT physical channel design	Lenovo

R1-2501092	Discussion on modulation aspects	Sharp

R1-2501108	Discussion on Physical channels design for Ambient IoT-modulation aspects	HONOR

R1-2501117	Modulation aspects of physical channel design for Ambient IoT	InterDigital, Inc.

R1-2501156	Physical channels design – modulation aspects	Qualcomm Incorporated

R1-2501202	Discussion on modulation aspects of physical channel design for Ambient IoT	NTT DOCOMO, INC.

R1-2501287	Discussion on modulation aspects of physical channels design for Ambient IoT	WILUS Inc.

R1-2501310	Discussion on modulation related aspects of AIoT	IIT Kanpur

R1-2501351	Discussion on Physical channels design for Ambient IoT	Indian Institute of Tech (M)

R1-2501407	FL summary #1 for Ambient IoT: “9.4.1 Physical channels design – modulation aspects	Moderator (Huawei)

Agreement

For D2R, only 2SB (Double sideband) modulation is supported.

Agreement

For R2D transmission, only subcarrier spacing of 15 kHz is supported (as per TR 38.769), and no other SCS are further considered.

Agreement

For a D2R transmission, it is up to device implementation to use OOK or BPSK modulation. It is assumed that a device operates consistently with the same D2R modulation in the same inventory/command round.

Proposal

For D2R BPSK modulation

- The data “1” is mapped to the real-valued modulation symbol {1} and data bit “0” is mapped to the real-valued modulation symbol {-1} in the baseband.

[Open][H] Proposal 5.2.1a-v1:

For R2D CP handling, Alt M2-2 (as per TR 38.769) is not considered in Rel-19.

R1-2501408	FL summary #2 for Ambient IoT: “9.4.1 Physical channels design – modulation aspects	Moderator (Huawei)

R1-2501409	FL summary #3 for Ambient IoT: “9.4.1 Physical channels design – modulation aspects	Moderator (Huawei)

### Physical channels design – line coding, FEC, CRC, repetition aspects

Including discussions on small frequency shift

R1-2500034	Coding aspects for Ambient IoT	Ericsson

R1-2500045	Discussion on coding aspects for A-IoT physical channel	FUTUREWEI

R1-2500125	Discussion on Ambient IoTcoding	ZTE Corporation, Sanechips

R1-2500154	Physical channel design on channel coding	Huawei, HiSilicon

R1-2500170	Discussion on line coding, FEC, CRC, repetition aspects for Ambient IoT	Spreadtrum, UNISOC

R1-2500225	Ambient IoT channel coding and small frequency shift	CATT

R1-2500260	Discussion on physical channels design about line coding, FEC, CRC, repetition aspects for Ambient IoT	China Telecom

R1-2500288	Discussion on coding aspects of physical channel design	CMCC

R1-2500350	Discussion on line coding, FEC, CRC and repetition for A-IoT	vivo

R1-2500395	AIoT Physical channels design - line coding, FEC, CRC, repetition aspects	Nokia

R1-2500425	Discussion on other aspects for Ambient IoT physical design	TCL

R1-2500451	Discussion on physical channels design for A-IoT	OPPO

R1-2500487	Discussion on Physical Channel Designs for A-IoT	Panasonic

R1-2500611	Physical layer design – line coding, FEC, CRC, repetition aspects	NEC

R1-2500733	Discussion on line coding, FEC, CRC and repetition aspects for Ambient IoT	Xiaomi

R1-2500779	On coding aspects for Ambient IoT	Apple

R1-2500851	Views on Physical channels design – line coding, FEC, CRC, repetition aspects	Samsung

R1-2500894	Discussion on general aspects of physical layer design of A-IoT	IIT, Kharagpur

Late submission

R1-2500937	Discussion on coding aspects	Fujitsu

R1-2500950	A-IoT PHY layer design - line coding, FEC, CRC and repetition aspects	LG Electronics

R1-2501017	A-IoT - PHY line coding, FEC, CRC, repetition aspects	MediaTek Inc.

R1-2501072	Discussion on the Ambient IoT physical layer design aspects for Line coding, FEC, CRC, Repetition 	Lenovo

R1-2501093	Discussion on coding aspects	Sharp

R1-2501109	Discussion on Physical channels design for Ambient IoT-other aspects	HONOR

R1-2501118	Coding aspects of physical channel design for Ambient IoT	InterDigital, Inc.

R1-2501124	Discussion on A-IoT Physical channels design	ASUSTeK

R1-2501157	Physical channels design – line coding, FEC, CRC, repetition aspects	Qualcomm Incorporated

R1-2501203	Discussion on coding and CRC aspects of physical channel design for Ambient IoT	NTT DOCOMO, INC.

R1-2501269	Coding aspects for Ambient IoT	ITL

R1-2501311	Discussion on other aspects of Phy design for AIoT	IIT Kanpur

R1-2501435	Summary #1 for coding aspects of physical channel design	Moderator (CMCC)

Agreement

For R2D Manchester line code, the specification is according to TR 38.769 as follows:

- A chip corresponds to one modulated symbol.

- For Manchester encoding, the bit-to-chip mapping is: bit 0→chips{10}, bit 1→chips{01}

Agreement

For D2R transmission with small frequency shift +/- R/Tb Hz, where time duration Tb corresponding to a bit (that is after FEC if applied) and R = Tb/(2 × D2R chip length):

- the transmitted 2R chips for bit 1 and bit 0 are [0 1 0 1 …] and [1 0 1 0 …], respectively, for OOK modulation

- the transmitted 2R chips for bit 1 and bit 0 are [-1 +1 -1 +1 …] and [+1 -1 +1 -1 …], respectively, for BPSK modulation.

- Note: The case of R=1 is equivalent to using a Manchester line code without repeating each Manchester codeword.

R1-2501436	Summary #2 for coding aspects of physical channel design	Moderator (CMCC)

R1-2501437	Summary #3 for coding aspects of physical channel design	Moderator (CMCC)

### Timing acquisition and synchronization

Including discussions on the necessity/functionalities of preamble/midamble/postamble for R2D and D2R, respectively.

R1-2500035	Timing acquisition and synchronization for Ambient IoT	Ericsson

R1-2500046	Discussion on timing acquisition and synchronization aspects for A-IoT	FUTUREWEI

R1-2500126	Discussion on Ambient IoT signals	ZTE Corporation, Sanechips

R1-2500140	Discussion on timing acquisition and synchronization for Ambient IoT	Lenovo

R1-2500155	Physical signals design	Huawei, HiSilicon

R1-2500171	Discussion on  timing acquisition and synchronization for Ambient IoT	Spreadtrum, UNISOC

R1-2500226	Ambient IoT Timing and synchronization	CATT

R1-2500261	Discussion on timing acquisition and synchronization for Ambient IoT	China Telecom

R1-2500289	Discussion on timing acquisition and synchronization	CMCC

R1-2500351	Discussion on Timing acquisition and synchronization for AIoT	vivo

R1-2500396	AIoT Timing acquisition and synchronization	Nokia

R1-2500426	Discussion on timing acquisition and synchronization functionalities for Ambient IoT	TCL

R1-2500452	Discussion on timing acquisition and synchronization for A-IoT	OPPO

R1-2500480	Discussion on timing acquisition and synchronization aspects	Tejas Network Limited

R1-2500493	Discussion on timing acquisition and synchronization	InterDigital, Inc.

R1-2500584	A-IoT Timing acquisition and synchronization	Panasonic

R1-2500625	Discussion on timing acquisition and synchronization	NEC

R1-2500652	Timing acquisition and synchronisation for Ambient IoT	Sony

R1-2500734	Discussion on timing acquisition and synchronization for Ambient IoT	Xiaomi

R1-2500780	On timing acquisition & synchronization aspects for Ambient IoT	Apple

R1-2500852	Views on timing acquisition and synchronization	Samsung

R1-2500911	Discussion on timing acquisition and synchronization	ETRI

R1-2500938	Discussion on timing acquisition and synchronization	Fujitsu

R1-2500951	Timing acquisition and synchronization for A-IoT	LG Electronics

R1-2501018	A-IoT - Timing acquisition and synchronization	MediaTek Inc.

R1-2501094	Discussion on timing acquisition and synchronization	Sharp

R1-2501110	Discussion on timing acquisition and synchronization	HONOR

R1-2501125	Discussion on end timing acquisition	ASUSTeK

R1-2501158	Timing acquisition and synchronization	Qualcomm Incorporated

R1-2501204	Discussion on timing acquisition and synchronization for Ambient IoT	NTT DOCOMO, INC.

R1-2501275	Discussion on timing acquisition and synchronization aspects for Ambient IoT	CEWiT

R1-2501312	Discussion on timing and synchronization aspects for AIoT	IIT Kanpur

Withdrawn

R1-2500782	FL Summary#1 on timing acquisition & synchronization for Ambient IoT	Apple

Agreement

For the SIP of R-TAS, for providing the start of the R2D transmission, one single design based on Option 1 is supported and further down-selection to be done among Alt 1 and Alt 2 :

- Option 1: ON-OFF transmission with following alternatives:

  - Alt 1: A single ON-OFF transmission with pre-defined duration for each of the ON-OFF, where ON and OFF may have same or different durations

    - Continue discussion to clarify the duration of each of the ON and OFF

    - Continue discussion to list the different candidate proposals under Alt1

  - Alt 2: A multi-ON-OFF transmission with pre-defined duration for each of the ON(s)-OFF(s), where different ON and different OFF may have same or different durations and different parts may have same or different duration

    - Continue discussion to clarify the duration of each of the ON and OFF

    - Continue discussion to list the different candidate proposals under Alt2

- Only a single fixed value for entire duration of SIP of R-TAS is supported, which is independent of the value of “M” used in CAP and PRDCH

- Note: Specific design and duration for SIP of R-TAS are further discussed, and companies are encouraged to evaluate the designs in terms of target MDR of [10%] for a FAR up to [1%] and at least following assumptions are used:

  - MDR refers to the probability that SIP is not detected when it was actually transmitted

  - FAR probability that the receiver incorrectly detects SIP when SIP was not transmitted

  - Energy/edge detection-based method is the baseline assumption for evaluation purpose

  - Continue discussion on necessary details for simulation assumptions

Agreement

For the CAP of R-TAS, the starting chip has a different voltage level compared to the end of the SIP of R-TAS.

Agreement

R2D transmission does not include a midamble.

Proposal

For D2R preamble, further consider the following sequence types for the binary signal:

- Option 1: M-sequence

- Option 2: Golay sequence

- Option 3: an all 1 or an all 0 sequence

Note: Specific design and length will be discussed separately, and companies are encouraged to evaluate the designs in terms of:

- Autocorrelation property, and cross-correlation property (if applicable)

- SFO estimation

- Timing accuracy

- Number of sequences in a D2R preamble

- Continue discussion on necessary details for simulation assumptions (e.g. target SNR)

R1-2500783	FL Summary#2 on timing acquisition & synchronization for Ambient IoT	Apple

R1-2500784	FL Summary#3 on timing acquisition & synchronization for Ambient IoT	Apple

### Other aspects incl. multiplexing/multiple access, scheduling information, and timing relationships

Including discussions on L1 control information and any other L1 procedural aspects.

R1-2500036	Other aspects for Ambient IoT	Ericsson

R1-2500047	Discussion on multiple access, scheduling and timing aspects for A-IoT	FUTUREWEI

R1-2500127	Discussion on Ambient IoT multiple access and timing	ZTE Corporation, Sanechips

R1-2500141	Discussion on multiple access, scheduling and timing aspects for Ambient IoT	Lenovo

R1-2500156	Multiplexing, scheduling, and other physical-layer procedures	Huawei, HiSilicon

R1-2500172	Discussion on other aspects for Ambient IoT	Spreadtrum, UNISOC

R1-2500227	Ambient IoT frame structure, system control and resource allocation	CATT

R1-2500262	Discussion on other aspects for Ambient IoT	China Telecom

R1-2500290	Discussion on access, scheduling and timing relationships	CMCC

R1-2500352	Discussion on other aspects for Rel-19 Ambient IoT	vivo

R1-2500397	AIoT Other aspects incl. multiplexing/multiple access, scheduling information, and timing relationships	Nokia

R1-2500453	Discussions on other aspects of A-IoT communication	OPPO

R1-2500481	Discussion on multiple access, scheduling and timing aspects for A-IoT	Tejas Network Limited

R1-2500488	Discussion on other aspects of A-IoT	Panasonic

R1-2500494	Discussion on other aspects including multiplexing/multiple access, scheduling information, and timing relationships	InterDigital, Inc.

R1-2500517	Views on other aspects for AIoT	Ofinno

R1-2500563	Discussion on multiple access, scheduling and timing for Ambient IoT	Lekha Wireless Solutions

R1-2500612	Discussion on control and other aspects of ambient IoT	NEC

R1-2500653	Multiplexing, multiple access and timing relationships for Ambient IoT	Sony

R1-2500735	Discussion on other aspects for Ambient IoT	Xiaomi

R1-2500781	On multiple Access, scheduling and control for Ambient IoT	Apple

R1-2500853	Views on aspects including multiplexing/multiple access, scheduling information, and timing relationships	Samsung

R1-2500912	Discussion on other aspects of Ambient IoT	ETRI

R1-2500939	Discussion on other aspects for Ambient IoT	Fujitsu

R1-2500952	Other aspects for A-IoT	LG Electronics

R1-2501019	A-IoT - Other aspects	MediaTek Inc.

R1-2501095	Discussion on other aspects	Sharp

R1-2501111	Discuss on L1 control information and L1 procedural aspects	HONOR

R1-2501126	Discussion on control information and procedural aspects	ASUSTeK

R1-2501159	Discussion on other aspects for Rel-19 Ambient IoT	Qualcomm Incorporated

R1-2501205	Discussion on other aspects for Ambient IoT	NTT DOCOMO, INC.

R1-2501276	Discussion on multiple access, scheduling and timing aspects for Ambient IoT	CEWiT

R1-2501288	Discussion on multiplexing/multiple access and timing relationships for Ambient IoT	WILUS Inc.

R1-2501298	Discussion on multiple access, scheduling information and timing relationships for Ambient IoT	TCL

R1-2501313	Discussion on other aspects of AIoT	IIT Kanpur

R1-2501336	Discussion on multiplexing, multiple access, scheduling information, and timing relationships	Google

R1-2501347	Discussion on aspects for Ambient IoT	Sequans Communications

R1-2501355	FL summary #1 on other aspects for Rel-19 Ambient IoT	Moderator (vivo)

FL1 High Priority Proposal 2.1.1-1a

For following according to TR 38.769, support X=1 and X>1.

- The maximum value of X is down selected between [2] and [4] in [RAN1#120].

- [It is up to device implementation to support X>1]

A R2D transmission triggering random access determines X time domain resource(s) for D2R transmission(s) for Msg1, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s). The study includes: - X=1 and X>1 and X>=1. The maximum value of X>1 should be set considering the device implementation complexity, device power consumption, the resource usage efficiency affected at least by SFO, and inventory latency.

Agreement

Support FDMA with Y≥1 D2R transmissions for Msg.1 from multiple devices in response to a R2D transmission triggering random access.

- The maximum value of Y >1 is determined in AI 9.4.2, based on feasible values of small frequency shift to be decided in AI 9.4.2.

- Signalling details should be discussed and determined in AI 9.4.4.

Agreement

Support following two options for Msg2 transmission in response to multiple Msg1 transmissions, which are initiated by a R2D transmission triggering random access.

- Option 1: A PRDCH for Msg2 transmission corresponds to a A-IoT Msg1 received from one device

- Option 2: A PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices

- FFS the maximum number of Msg1 responses that can be carried within a PRDCH for Msg2.

FL1 High Priority Proposal 2.2.2-1a

For following according to TR 38.769, down-select between option 1 and option 2 for X>1 (if supported).

- Note: When X = 1, option 1 and option 2 are the same.

For a device to determine the reference time for the starting time for monitoring Msg2 in response to multiple Msg1 transmissions initiated by a R2D transmission triggering random access, the following options have been identified: Option 1: End of time domain resource where the device transmitted the Msg1 Option 2: End of the last of X time domain resource(s) determined for Msg1 transmission(s) Option 3: End of the R2D transmission triggering random access Option 4: End of a R2D transmission carrying Msg2 Note: The reference time is used to determine the starting time for Msg2 monitoring, it does not mean that the starting time always equals the reference time.

R1-2501356	FL summary #2 on other aspects for Rel-19 Ambient IoT	Moderator (vivo)

R1-2501357	FL summary #3 on other aspects for Rel-19 Ambient IoT	Moderator (vivo)