3GPP TSG RAN WG1 #120bis																R1-2501744

Wuhan, China, April 7th – 11th, 2025

Agenda Item:		9.2.3

Source:				InterDigital, Inc.

Title:						On Specifications of Rel-19 CB-based UL for 3TX UE

Document for:		Discussion and Decision

# Introduction

For Rel-19, a WID was proposed to further enhance the MIMO feature [1], where the fourth objective targets enhancements to support 3TX UL transmission. The initial WID scope of 3TX was completed in RAN1#117.

After the RAN#105 plenary meeting, the WID was revised [2] and expanded to include additional enhancements to support 3TX with ‘antennaSwitching’ and ‘nonCodebook’ usage. Enhancements to ‘nonCodebook’ were introduced. After RAN#106 plenary meeting, there were further clarifications that 3T3R antenna switching can be discussed and supported as part of Rel-19 MIMO in RAN1 [3]:

- RAN clarifies that for ‘3T3R’ antenna switching, SRS configuration i.e., SRS resource/set definition for ‘3T3R’ antenna switching can be discussed and supported as part of Rel-19 MIMO in RAN1. - where ‘3T3R’ is only applicable for the UE equipped with 4Rx, 6Rx, or 8Rx antenna ports. - No RAN4 impact.

The Rel-19 work has been completed with all agreements finalized on supporting 3TX. In this contribution, we provide our views on the CRs to specify the feature.

# CR for 38.214

The draft CR was discussed offline in [4] that includes,

### 6.2.1 UE sounding procedure <unchanged text omitted> [The UE does not transmit SRS on antenna port 1003 if [RRCparameter] is configured.]

However, once the discussion was finalized, there was a suggestion to not add the above change in Section 6.2.1 and instead incorporate it into 6.1.1.1 with the following CR edit:

Original Capture:

6.1.1.1 Codebook based UL transmission When only one SRS resource set is configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook', SRI and TPMI are given by the DCI fields of one SRS resource indicator and one Precoding information and number of layers in clauses 7.3.1.1.2, 7.3.1.1.3 and 7.3.1.1.4 of [5, TS 38.212] for DCI format 0_1, 0_2 and 0_3 or given by srs-ResourceIndicator and precodingAndNumberOfLayers according to clause 6.1.2.3. A UE does not expect two SRS resource sets are configured in srs-ResourceSetToAddModList with higher layer parameter usage in SRS-ResourceSet set to 'codebook' for a serving cell, when the serving cell is included in schedulingCellListDCI-0-3-r18 for a set of serving cells provided by mc-DCI-SetOfCellsToAddModList-r18. The TPMI is used to indicate the precoder to be applied over the layers {0…ν-1} and that corresponds to the SRS resource selected by the SRI when multiple SRS resources are configured, or if a single SRS resource is configured TPMI is used to indicate the precoder to be applied over the layers {0…ν-1} and that corresponds to the SRS resource. If a UE is configured with higher layer parameter [RRC_parameter] in the SRS-ResourceSet set to ‘enabled’, the transmission precoder is selected from the uplink codebook that has the number of antenna ports equal to 3, otherwise the transmission precoder is selected from the uplink codebook that has a number of antenna ports equal to higher layer parameter nrofSRS-Ports or nrofSRS-Ports-n8 in SRS-Config, as defined in Clause 6.3.1.5 of [4, TS 38.211]. When the UE is configured with the higher layer parameter txConfig set to 'codebook', the UE is configured with at least one SRS resource. The indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI.

Suggested edits:

6.1.1.1 Codebook based UL transmission When only one SRS resource set is configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook', SRI and TPMI are given by the DCI fields of one SRS resource indicator and one Precoding information and number of layers in clauses 7.3.1.1.2, 7.3.1.1.3 and 7.3.1.1.4 of [5, TS 38.212] for DCI format 0_1, 0_2 and 0_3 or given by srs-ResourceIndicator and precodingAndNumberOfLayers according to clause 6.1.2.3. A UE does not expect two SRS resource sets are configured in srs-ResourceSetToAddModList with higher layer parameter usage in SRS-ResourceSet set to 'codebook' for a serving cell, when the serving cell is included in schedulingCellListDCI-0-3-r18 for a set of serving cells provided by mc-DCI-SetOfCellsToAddModList-r18. The TPMI is used to indicate the precoder to be applied over the layers {0…ν-1} and that corresponds to the SRS resource selected by the SRI when multiple SRS resources are configured, or if a single SRS resource is configured TPMI is used to indicate the precoder to be applied over the layers {0…ν-1} and that corresponds to the SRS resource. If a UE is configured with higher layer parameter [RRC_parameter] set to ‘enabled’ in athe SRS-ResourceSet with usage set to ‘codebookenabled’, the transmission precoder is selected from the uplink codebook that has the number of antenna ports equal to 3 and the SRS port 1003 of the SRS resource(s) in the SRS-ResourceSet is disabled, otherwise the transmission precoder is selected from the uplink codebook that has a number of antenna ports equal to higher layer parameter nrofSRS-Ports or nrofSRS-Ports-n8 in SRS-Config, as defined in Clause 6.3.1.5 of [4, TS 38.211]. When the UE is configured with the higher layer parameter txConfig set to 'codebook', the UE is configured with at least one SRS resource. The indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI.

The paragraph starts with: “When only one SRS resource set is configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook'  ”. The original wording before the change states “with higher layer parameter [RRC_parameter] in the SRS-ResourceSet set to ‘enabled’ ” , which clearly refers to the SRS-ResourceSet  configured with usage set to ‘codebook’. Therefore, we do not see the need for the proposed change to reiterate that the SRS-ResourceSet is configured with usage set to ‘codebook’.

As for the statement “The UE does not transmit SRS on antenna port 1003 if [RRCparameter] is configured ”, in 6.2.1, we agree that the current wording may be too general, but we think it is still preferrable to keep it in Section 6.2.1 since that section discusses the mapping of antenna ports to SRS resources. To further clarify the applicability of the statement, we propose to modify the wording as follows,

Proposal 1: Adopt the following changes in the CR for 38.214,

### 6.2.1 UE sounding procedure <unchanged text omitted> For the SRS resource set(s) configured with higher layer parameter usage set to ‘codebook’ or ‘antennaSwitching, Tthe UE does not transmit SRS on antenna port 1003 if [RRCparameter] is configured.

# Conclusions

In this contribution, we discussed our views with respect to the Rel-19 CRs for specifying 3TX. We make the following observation and proposal:

Proposal 1: Adopt the following changes in the CR for 38.214,

### 6.2.1 UE sounding procedure <unchanged text omitted> For the SRS resource set(s) configured with higher layer parameter usage set to ‘codebook’ or ‘antennaSwitching, Tthe UE does not transmit SRS on antenna port 1003 if [RRCparameter] is configured.

# References

- RP-234007, “New WID: NR MIMO Phase 5”, 3GPP TSG RAN Meeting #102, December 2023

- RP-242394, “WID revision: NR MIMO Phase 5”, 3GPP TSG RAN Meeting #105, September 2024

- RP-243265, “Way forward on REL-19 MIMO 3T3R”, 3GPP TSG RAN Meeting #106, December 2024

- R1-2501666, “Summary of the discussion concerning the 3Tx draft CR for Release 19”, 3GPP TSG RAN1 Meeting #120, February 2025