3GPP TSG RAN WG1 Meeting #120bis	R1-2501774  Wuhan, China, April 7th - 11th, 2025

Agenda item:		9.5.3

Source:	Nokia, Nokia Shanghai Bell

Title:		Adaptation of Common Signal Channel Transmissions

Document for:		Discussion and Decision

# Introduction

In the RAN1#119 meeting, the following agreements were made:

Conclusion There is no RAN1 consensus to support the following in Rel-19 - New SSB burst periodicity values other than the legacy values (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms). - New UE trigger - Adapting the transmitted number of SSBs within a SSB burst - Adaptation of the time domain positions of SSBs within a burst Conclusion There is no RAN1 consensus to support time domain adaptation for CD-SSB in Rel-19 for Scell Agreement For adaptation of SSB in time-domain, support the following to adapt SSB burst periodicity for an SCell - UE is configured with SSB burst periodicity using legacy signalling for the SCell - UE is configured with X additional SSB burst periodicity for the SCell - FFS: Value of X - SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity - FFS: Whether there is specification impact - Note: This does not impact the discussion on OD-SSB - For switching the periodicity, down-select between - (MAC-CE) - MAC-CE details for SSB burst periodicity adaptation is up to RAN2. - (DCI) - Alt 1-3: DCI based signalling is used Agreement For adaptation of SSB in time-domain, for adapting SSB burst periodicity for an SCell - Support group common DCI signalling for switching the SSB burst periodicity using DCI format 2_9 with [cellDTRX-RNTI] - FFS: which scenario(s) is this applicable for (e.g. as defined in 9.5.1) Note: Above does not prevent RAN2 from designing a MAC CE based on OD-SSB feature and also used for SSB burst adaptation Agreement For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain, - When an additional RO is overlapped with legacy valid RO in both time and frequency domain, the additional RO is invalid before the SSB-RO mapping - Note: the overlapped RO for legacy resource is not impacted - FFS: Clarification on configuration of legacy ROs Agreement For adaption of PRACH in time-domain, for a connected mode UE, support a 1-bit field in DCI 1_0 with C-RNTI used to trigger PRACH (i.e. PDCCH order) to indicate whether the additional PRACH resource(s) is available for the triggered PRACH. - FFS: UE behaviour (e.g. applicable resources for PRACH mask index) when it is indicated of additional PRACH resource(s) - FFS: Details on how to reuse existing bit for the 1-bit indication Agreement For DCI-based adaptation for additional PRACH resources, DCI 1_0 with P-RNTI indicates the availability information for additional PRACH resource from a reference point and for a validity time duration - FFS: Validity time duration for availability is configured by higher layer signaling or predefined - FFS: Location of the reference point defined in the specification - FFS: Value/granularity of the validity time duration. - FFS: Whether DCI can be used to explicitly deactivate the additional PRACH resources Agreement - Separate configuration of Msg1-FDM for the additional PRACH resources at least for 4-step RACH is supported - UE is not expected to be configured such that there are more than 8 FDM-ed valid ROs (legacy + additional ROs) - FFS: When there is no configuration of Msg1-FDM - Separate configuration of number of SSB per RO is supported Agreement - Study the following options for the reference point (for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI in a PF) for RRC idle/inactive mode UE and RRC connected mode UE, - Option 1: SFN of the first PF from the next I-DRX cycle - Option 2: SFN of the first PF from the current I-DRX cycle - Option 3: From the first frame of the first PRACH association period after UE receives the DCI - Option 4: From the first frame of the current SI modification period - Option 5: From the first frame of the next SI modification period Agreement - For DCI-based adaptation for additional PRACH resources, support optional semi-static signalling of a single PRACH mask to identify the subset of the additional PRACH resources - The mask is applicable at unit of - Alt 1: PRACH association period - Alt 2: PRACH association pattern period - Alt 3: SFN level - The PRACH association period is determined based on valid additional ROs only. - The mask is applied after valid RO determination and SSB-RO mapping. - Note: The existing behaviour in TS 38.213 "An association pattern period includes one or more association periods and is determined so that a pattern between PRACH occasions and SS/PBCH block indexes repeats at most every 160 msec." is not impacted due to application of the mask. - This is applicable at least for adaptation for DCI 1_0 with P-RNTI - The DCI does not indicate PRACH mask selection - FFS: how the mask is identified - Option 1: The PRACH mask is from a PRACH mask table - Pre-defined table with N=[4 or 8 or 16] rows - The semi-static signalling indicates a PRACH mask index - Option 2: The PRACH mask is based on configuration parameters e.g. bitmap at SFN-level, periodic time domain window, …

In this contribution, we will discuss the issues related to the adaptation of common signal/channel transmissions. And in our companion contributions, we will discuss the issues related to the on-demand SSB SCell operation and on-demand SIB1 for idle/inactive mode UEs respectively in [1][2].

# Adaptation of common signal/channel transmissions

## Adaptation of SSB in time domain

Based on the agreements of the RAN1#120 meeting, SSB adaptation in time domain refers to adaptation of SSB burst periodicity of NCD-SSBs (either on the sync raster or non-sync raster) in a Rel-19 NES-capable UE’s SCell.

### Scenarios

A scenario where adaptation of the SSB burst periodicity can be applied is the activation of an SCell, e.g., when the load in the PCell is high and more capacity is needed. In an SCell operating with e.g., long SSB burst periodicity, the SSB burst periodicity can be adapted to a value smaller than the prior one for a period of time, aiming at the activation of the SCell. In this way, the UE can perform measurements, detect and access the cell in a faster manner. The SCell can revert to the initial SSB burst periodicity after a certain period of time for NES purposes.

In Rel.19 NES, the SCell activation has been studied under AI 9.5.1 with OD-SSB. Considering the scenarios studied under AI 9.5.1, Scenario #2 and Scenario #2A are relevant for the SCell activation procedure, where

- Scenario#2 refers to

  - SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321),

- Scenario #2A refers to

  - “When UE receives SCell activation command (e.g., as defined in TS 38.321)”

For this objective, AI 9.5.3, it is also relevant to consider SSB burst periodicity adaptation in the post-SCell activation phase, i.e., in Scenario #3B of AI 9.5.1, which refers to:

- “When SCell activation is completed and SCell is activated” or

- “After SCell activation is completed and SCell is activated”

Exemplary illustration of the OD-SSB scenarios with SSB burst periodicity adaptation are provided in Figure 1.

A use case specifically for Scenario #3B, is if the network operates with long SSB burst periodicity after completion of SCell activation, i.e., time point reference T3 in Figure 1. Then, it would be useful for UEs in the cell to measure denser SSBs, e.g., for improved time and frequency synchronization or to use those SSBs as reference for more frequent CSI reporting. In these cases, the network may adapt the SSB burst periodicity value to a value smaller than the prior legacy one.

If all UEs in the SCell are R19 UEs supporting SSB adaptation in time domain, then, it is also possible that the network adapts the SSB burst periodicity to longer value than the default one and obtain NES gains.

Proposal-1: RAN1 considers the scenarios for SSB burst periodicity adaptation at least with Scenarios #2, #2A and #3B in AI 9.5.1.

Figure 1: Timeline of OD-SSB SCell activation scenario with SSB adaptation.

### Signalling aspects

During the RAN1#120 meeting, the following agreements were reached for SSB adaptation in time domain.

Agreement For adaptation of SSB in time-domain, support the following to adapt SSB burst periodicity for an SCell - UE is configured with SSB burst periodicity using legacy signalling for the SCell - UE is configured with X additional SSB burst periodicity for the SCell - FFS: Value of X - SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity - FFS: Whether there is specification impact - Note: This does not impact the discussion on OD-SSB - For switching the periodicity, down-select between - (MAC-CE) - MAC-CE details for SSB burst periodicity adaptation is up to RAN2. - (DCI) - Alt 1-3: DCI based signalling is used Agreement For adaptation of SSB in time-domain, for adapting SSB burst periodicity for an SCell - Support group common DCI signalling for switching the SSB burst periodicity using DCI format 2_9 with [cellDTRX-RNTI] - FFS: which scenario(s) is this applicable for (e.g. as defined in 9.5.1) Note: Above does not prevent RAN2 from designing a MAC CE based on OD-SSB feature and also used for SSB burst adaptation

Following on the RAN1#120 agreements, we believe that a list of candidate SSB burst periodicity values for SSB adaptation can be configured and the values can be any of the legacy periodicity values. Depending on the number of supported SSB burst periodicity values for adaptation, a corresponding number of bits should be included within the signalling command indicating the adapted SSB burst periodicity.

Based on the agreements, for the signalling command indicating the adapted SSB burst periodicity, both DCI signalling and more specifically the group common DCI format 2_9 and MAC-CE may be specified. As MAC-CE details for SSB burst periodicity adaptation is up to RAN2, in this contribution we focus on the content of the DCI, for which we believe that a maximum of three (3) bits is sufficient to indicate all the legacy SSB periodicity values.

Proposal-2: The number of bits required for indicating the adapted SSB periodicity in DCI format 2_9 depends on the number of configured candidate SSB burst periodicity values in RRC.

Regarding the agreed group common DCI signalling, DCI format 2_9, we should take into account that, as per the current specifications, it is used for

- activating or de-activating the cell DTX and/or DRX configuration of one or multiple serving cells for one or more UEs,

- and/or for providing NES-mode indication of the primary cell for one or more UEs (NES-specific CHO execution condition).

DCI 2_9 can contain one or more information blocks configured for the UE, where each block corresponds to a cell. The information blocks may contain bit information about the afore mentioned features (i.e., Cell DTX/DRX (de-)activation indication and/or NES-mode (CHO) indication) and/or indication indicating adaptation of SSB burst periodicity. This enables that SSB adaptation can be indicated jointly or independently from e.g., Cell DTX/DRX operation. We believe that UE can be configured to monitor DCI 2_9 with CRC scrambled by cellDTRX-RNTI for the SSB adaptation as well.

Proposal-3: Support indication of SSB adaptation jointly or independently from Cell DTX/DRX operation.

Proposal-4: Support that UE is configured to monitor DCI 2_9 with cellDTRX-RNTI for SSB adaptation.

In the case of SSB adaptation indication, we should also consider that UEs need time to process the SSB adaptation indication message they have received before they can follow the adapted SSB burst periodicity. If the SSB adaptation is indicated jointly in the DCI 2_9 with the Cell DTR/DRX operation, then, R18 UEs will receive the DCI 2_9 indication of both Cell DTX/DRX and SSB adaptation, but they will be able to understand only the Cell DTX/DRX part, whereas R19 UEs will understand both indications. We believe thus that the processing time of DCI 2_9 when it indicates only SSB adaptation and when it indicates both Cell DTX/DRX and SSB adaptation should be further analysed.

Proposal-5: Discuss the processing time of DCI 2_9 when it indicates only SSB adaptation and when it indicates both Cell DTX/DRX and SSB adaptation.

Inclusion of SSB adaptation in DCI format 2_9 should not change the current indication operation for Cell DTX/DRX. In detail, currently, DCI format 2_9 is used for activating or de-activating the cell DTX and/or DRX configuration. Thus, for SSB adaptation, the same indication operation should be preserved, i.e., DCI format 2_9 is sent when SSB adaptation is activated, and another DCI format 2_9 is sent when SSB adaptation is de-activated. Timer-based de-activation was not introduced for cell DTX/DRX operation, and we prefer not to introduce it now for SSB adaptation. The current method of selecting suitable aggregation level for PDCCH to manage reliability of DCI format 2_9 is sufficient. Thus, we believe that it is not needed to introduce in Rel.19 new methods to handle a missed DCI.

Proposal-6: New methods to handle missed DCI is not introduced in R19.

Regarding the scenarios to which DCI 2_9 can be used for SSB adaptation, we believe that DCI 2_9 can be considered to be used after completion of the SCell activation, i.e., in the Scenario#3B and after time instant T3 of Figure 1. For the same Scenario#3B, MAC-CE signalling can also be considered, if RAN2 agrees on it.

From our perspective, MAC-CE can be assumed to enable SCell activation with SSB adaptation, i.e., in Scenarios #2 and #2A, if RAN2 agrees on it. MAC-CE design currently discussed under AI 9.5.1 could be commonly used to enable SSB adaptation in time domain. The details of the MAC-CE design for SSB adaptation in time domain, including any differentiation aspects from OD-SSB MAC-CE design, can be further defined by RAN2.

Proposal-7: DCI or MAC-CE signalling can be considered for indicating SSB adaptation in time domain after completion of the SCell activation.

Proposal-8: Signaling of SSB adaptation in time domain for SCell should be considered jointly with the MAC-CE specified for OD-SSB at least for SCell activation. Design details, including any differentiation aspects from the OD-SSB design, can be further considered in RAN2.

It should be noted that, direct SCell activation has been considered in RAN4 for OD-SSB. Direct SCell activation should be possible with SSB adaptation in time domain too. To that end, MAC-CE or RRC signaling should be able to activate/deactivate SSB burst periodicity adaptation with SCell activation.

Proposal-9: Support direct SCell activation with SSB adaptation in time domain based on RRC signalling.

### Impact from SSB adaptation in time domain

UE behavior regarding CSI/beam measurements and reporting corresponding to SSBs that are dynamically adapted, should be defined.

For rate matching purposes RRC configuration of the serving cell includes SSB periodicity. When SSB periodicity is adapted dynamically, it should be clarified that R19 UEs change their rate matching operation while legacy UEs still apply the periodicity configured in RRC. Otherwise, when periodicity is adapted to longer value than in the legacy RRC configuration, there would be resources that all UEs assume to carry SSB even though the resources are in fact empty.

CSI measurements and reporting can be based on SSB. When the SSB periodicity changes the effect to the CSI reporting should be defined. For example, if a UE is configured with periodic CSI reporting based on SSB so that both SSB and CSI reporting have 20ms periodicity and then SSB periodicity is adapted to 40ms, it should be clarified, if e.g., UE should drop some CSI reports or change the reporting periodicity.

Proposal-10: RAN1 to discuss and decide how adaptation of SSB periodicity affects rate matching operation and CSI measurements and reporting.

## Adaptation of PRACH in time domain

From our perspective, current agreements in Rel-19 seem to imply that the agreed PRACH adaptation based on additional PRACH resources is applicable to both 4-step RACH and 2-step RACH (if configured in the cell). However, for 2-step RACH, further clarifications and considerations would be needed. For instance, it is not clear whether additional PUSCH resources for NES-capable shall be considered for MsgA payload transmission or the same PUSCH as configured for legacy UEs shall be considered.

Proposal-11: RAN1 to clarify whether Rel-19 additional ROs could be used for 2-step PRACH procedure and, if yes, how.

Furthermore, PRACH transmission can be performed also in the context of CFRA. Given that RRC connected Rel-19 NES UEs will be able to use additional PRACH resource as per Rel-19 PRACH adaptation solution, it is rather straightforward to see that the impact of the PRACH adaptation on CFRA procedures should be studied as well, if time allows.

Proposal-12: PRACH adaptation applied to CFRA should also be investigated, if time allows, however no CFRA-specific PRACH adaptation solutions should be considered.

### Configuration of additional PRACH resources

In Rel-19, NW will be able to choose to configure additional PRACH resources with same PRACH configuration index as or different from the PRACH configuration index used to configure the legacy PRACH resources.

For both cases, additional adjustments can be applied time and/or frequency to mitigate potential RO overlapping issues between legacy and additional PRACH resources.

- Frequency domain adjustments

According to existing agreements, msg1-FrequencyStart and Msg1-FDM, can be configured separately for the additional PRACH resources at least for 4-step RACH.

- Time domain adjustments

According to RAN1 #120, it was agreed to support a single PRACH mask to identify the subset of the additional PRACH resources, details are provided in the following:

For DCI-based adaptation for additional PRACH resources, support optional semi-static signalling of a single PRACH mask to identify the subset of the additional PRACH resources - The mask is applicable at unit of - Alt 1: PRACH association period - Alt 2: PRACH association pattern period - Alt 3: SFN level - The PRACH association period is determined based on valid additional ROs only. - The mask is applied after valid RO determination and SSB-RO mapping. - Note: The existing behaviour in TS 38.213 "An association pattern period includes one or more association periods and is determined so that a pattern between PRACH occasions and SS/PBCH block indexes repeats at most every 160 msec." is not impacted due to application of the mask. - This is applicable at least for adaptation for DCI 1_0 with P-RNTI - The DCI does not indicate PRACH mask selection - FFS: how the mask is identified - Option 1: The PRACH mask is from a PRACH mask table - Pre-defined table with N=[4 or 8 or 16] rows - The semi-static signalling indicates a PRACH mask index - Option 2: The PRACH mask is based on configuration parameters e.g. bitmap at SFN-level, periodic time domain window, …

We start from the last aspect, i.e., the FFS on how the mask is identified. Two fundamental differences exists between Option 1(The PRACH mask is from a PRACH mask table) and Option 2 (The PRACH mask is based on configuration parameters e.g. bitmap at SFN-level, periodic time domain window, …):

- Option 1: offers a simple and natural way of indicating patterns of association periods, pattern periods, or SFNs, by design. Indeed, in NR a mask for PRACH indicates one of more resources according to a specific periodicity. Furthermore, it requires RAN1 only to agree on

  - The granularity of its application, i.e., the mask indicates one or more association periods, pattern periods, or SFNs.

  - Whether 2,3 or 4 bits are used for the mask.

The spec impact would be rather limited, and the corresponding RAN1 discussion would be simple. Finally, Option 1 would always guarantee that, the same number of ROs per SSB exists in each mapping cycle, ensuring the same access probability to all UEs across different SSBs.

- Option 2: it requires the definition of a new tool which was never discussed so far in Rel-19 and for which detailed technical solutions have not been studied. To indicate patterns of association periods, pattern periods or SFNs a more convoluted design would be needed, since it would depend on how Option 2 is designed eventually. Furthermore, Option 2 would not always guarantee that the same number of ROs per SSB exists in each mapping cycle, ensuring the same access probability to all UEs across different SSBs. Indeed, this would depend on how and how many configuration parameters are designed. It is unclear, for instance, how Option 2 could indicate patterns of association periods or pattern periods, if its configuration would use subframes, or system frames, of slots, as granularity for the indication. It is also unclear how many bits could be needed to achieve the same descriptive power and flexibility of the PRACH mask implement using the PRACH max table. Finally, its specification impact would also be larger than Option 1.

Proposal-13: The Rel-19 PRACH mask for NES is from a PRACH mask table.

Concerning the granularity, a good approach would be for the mask to indicate association periods of additional ROs (Alt 1), i.e., after valid RO determination and SSB-RO mapping, to ensure that sufficient flexibility exists at scheduler level to accommodate different design targets, e.g., maximize NES gain, maximum number of UL resources dedicated to initial access per subframe and so on. For instance, a very simple mask would be needed to obtain effects like:

- Additional ROs occur only in even/odd association periods w.r.t. association pattern period, when the association pattern period consists of more than one association period.

- Additional ROs occur only in parts of the association periods w.r.t. association pattern period, when the association pattern period consists of more than one association period.

Of course, finer effects could also be achieved, depending on the structure and size of the mask.

Indicating association pattern periods would offer only a rather coarse granularity, which may not be suitable in case additional and legacy ROs are configured using different PRACH configuration indices.

Indicating SFNs would go against the principle that the same number of ROs per SSB exists in each mapping cycle, ensuring the same access probability to all UEs across different SSBs. In our view, this is a paramount principle that should be at the core of any contention-based initial access procedure.

Proposal-14: The Rel-19 PRACH mask should apply the unit of association periods.

Switching the focus on the size the of the PRACH mask table, this has been subject of discussion during RAN1 #119 and #120 meetings. Some companies expressed concerns related to the complexity at the UE if the mask size is too big. This observation assumes that a larger mask would increase the amount of actual PRACH configurations the UE would need to support on top of the legacy PRACH configurations as per TS 38.211. This argument does not seem corroborated by evidence and might be based on a debatable modeling of how the mask would work in practice. According to existing specification, all the masks defined in NR operate on the existing ROs after the application of both the PRACH configuration index and the SSB-to-RO mapping and do not “mute” ROs but rather indicate ROs that can be used. In this sense, no new (virtual) PRACH configuration is introduced when the mask is applied but rather a restriction on the use of certain PRACH resources is provided to the UE. At the same time, it is in the best interest of RAN1 for interested companies to work on and propose solutions which are in the comfort zone of as many companies as possible. In this context, we will propose three examples of possible masks supporting Option 1, where a pre-defined PRACH mask table requiring 2, 3 and 4 bits (i.e. 4, 8 and 16 rows respectively) is to be configured to the UE.

We start from the 4-bits case. In this case, the structure and size of existing PRACH mask could be reused, and only the interpretation associated to the different codepoints could be changed, i.e., the codepoint of the PRACH mask for NES would correspond to different indications as compared to their existing legacy PRACH mask counterparts. Only NES-capable UEs would be able to interpret this mask to identify a subset of additional ROs that can be used by such UEs, together with the legacy ROs, for performing the PRACH transmission. The applicability of such PRACH mask for NES would not depend on how the additional ROs are created, e.g., via same or different PRACH configuration index, to allow NW to use it to increase the scheduler’s flexibility with no specific constraint. We observe that legacy PRACH mask resets at each mapping cycle, when N<1 (up to 1/8), hence a natural extension of the legacy logic in case of mask adaptation for NES would be for the mask to reset every association pattern period, or every N association periods, where N could be equal to 8 for simplicity (but could also be smaller/larger, if needed). In this context, association periods may be easily indexed from 0 starting from:

- The first association period of an association pattern period, or

- SFN0, where a reference period of N association periods may or may not be considered as well.

According to these principles, a possible 4-bit mask signalled in the SIB1 could be as the one below, which is based on Table 7.4-1: PRACH Mask Index values of TS 38.321, and where N could be the number of association periods in an association pattern period (N=8 in this example).

| PRACH Mask Index/ msgA-SSB-SharedRO-MaskIndex/ ssb-SharedRO-MaskIndex | Association periods with additional valid ROs in a sequence of N association periods or in an association pattern period |
| --- | --- |
| 0 | All |
| 1 | mod(association period index, N)=0 |
| 2 | mod(association period index, N)=1 |
| 3 | mod(association period index, N)=2 |
| 4 | mod(association period index, N)=3 |
| 5 | mod(association period index, N)=4 |
| 6 | mod(association period index, N)=5 |
| 7 | mod(association period index, N)=6 |
| 8 | mod(association period index, N)=7 |
| 9 | Every even association period |
| 10 | Every odd association period |
| 11 | Reserved |
| 12 | Reserved |
| 13 | Reserved |
| 14 | Reserved |
| 15 | Reserved |

Alternatively, if smaller masks is to be adopted, the granularity of one association period, used in the first 8 rows of the previous examples could be made coarser, while keeping a practical relevance. In particular, the RO density could be controlled in a meaningful way also if:

- NW could provide a mask configuration such as the PRACH transmission can be only the additional ROs of the first or second half of the association periods in a reference period of length N association periods (or in an association pattern period).

- NW could provide a mask configuration such as the PRACH transmission can be only the additional ROs of the first, second, third or fourth quarter of the association periods in a reference period of length N association periods (or in an association pattern period).

Depending on the situation and requirements, NW could decide whether to condense ROs or have a sparser RO layout in time domain, depending on its scheduler and NES targets. Possible 3-bits and 2-bits masks according to these principles could be as the ones below. Other mask structures could also be considered.

| PRACH Mask Index/ msgA-SSB-SharedRO-MaskIndex/ ssb-SharedRO-MaskIndex | Association periods with additional valid ROs in a sequence of N association periods or in an association pattern period |
| --- | --- |
| 0 | The first half of N association periods |
| 1 | The second half of N association periods |
| 2 | The first quarter of N association periods |
| 3 | The second quarter of N association periods |
| 4 | The third quarter of N association periods |
| 5 | The fourth quarter of N association periods |
| 6 | Every even association period |
| 7 | Every odd association period |

| PRACH Mask Index/ msgA-SSB-SharedRO-MaskIndex/ ssb-SharedRO-MaskIndex | Association periods with additional valid ROs in a sequence of N association periods or in an association pattern period |
| --- | --- |
| 0 | The first half association periods |
| 1 | The second half association periods |
| 2 | Every even association period |
| 3 | Every odd association period |

Obviously, if the mask is specified as using the association pattern period as reference time for counting the association periods, then the mask would be meaningful when the number of association periods in an association pattern period is larger than 1. If the mask was configured when the association pattern period consists of only one association period, it would be a bad NW configuration. In this sense, NW would never configure it, as much as other options and parameters are not configured in a cell if certain conditions are not met. Differently, if the reference time is N association periods, then the mask would always be meaningful, as long as N>1. In this regard, having N representing a number of association periods, irrespective of how many of them are in an association pattern period would be simpler and more effective, especially if we consider that in many instances of FR1 deployments, there could be only one association period per association pattern period, due to the low number of SSB beams used by gNB in this case. Hence, using a reference time of N association periods is our preferred approach.

Proposal-15: The PRACH mask operates over association periods, and is reset every N association periods, or every association pattern periods. Details about N, if applicable, are FFS.

Proposal-16: If the semi-static configuration of a 4-bit mask is supported, the following Rel-19 NES PRACH mask is supported:

| PRACH Mask Index/ msgA-SSB-SharedRO-MaskIndex/ ssb-SharedRO-MaskIndex | Association periods with additional valid ROs in a sequence of N association periods or in an association pattern period |
| --- | --- |
| 0 | All |
| 1 | mod(association period index, N)=0 |
| 2 | mod(association period index, N)=1 |
| 3 | mod(association period index, N)=2 |
| 4 | mod(association period index, N)=3 |
| 5 | mod(association period index, N)=4 |
| 6 | mod(association period index, N)=5 |
| 7 | mod(association period index, N)=6 |
| 8 | mod(association period index, N)=7 |
| 9 | Every even association period |
| 10 | Every odd association period |
| 11 | Reserved |
| 12 | Reserved |
| 13 | Reserved |
| 14 | Reserved |
| 15 | Reserved |

Proposal-17: If the semi-static configuration of a 3-bit mask is supported, the following Rel-19 NES PRACH mask is supported:

| PRACH Mask Index/ msgA-SSB-SharedRO-MaskIndex/ ssb-SharedRO-MaskIndex | Association periods with additional valid ROs in a sequence of N association periods or in an association pattern period |
| --- | --- |
| 0 | The first half of N association periods |
| 1 | The second half of N association periods |
| 2 | The first quarter of N association periods |
| 3 | The second quarter of N association periods |
| 4 | The third quarter of N association periods |
| 5 | The fourth quarter of N association periods |
| 6 | Every even association period |
| 7 | Every odd association period |

Proposal-18: If the semi-static configuration of a 2-bit mask is supported, the following Rel-19 NES PRACH mask is supported:

| PRACH Mask Index/ msgA-SSB-SharedRO-MaskIndex/ ssb-SharedRO-MaskIndex | Association periods with additional valid ROs in a sequence of N association periods or in an association pattern period |
| --- | --- |
| 0 | The first half association periods |
| 1 | The second half association periods |
| 2 | Every even association period |
| 3 | Every odd association period |

### Further signalling details

It is natural to expect that additional PRACH resources are configured semi-statically in Rel-19. This includes the indication of a mask codepoint to the UE in e.g. SIB1 (as explained in the previous subsection), to be used by the UE in conjunction with the PRACH configuration index to know which of the configured additional ROs can be used.

Whether adaptation of the resources will be performed only via DCI-based signalling, as per RAN1#118 agreements, or also via semi-static signalling is to be further discussed by RAN1. Indeed, system Information (SI), e.g. SIB1, could also be considered for indicating PRACH adaptation to UEs that have missed the DCI-based indication of additional PRACH resources for NES-capable UEs. The SIB1 would then include the information about the configuration of additional PRACH resources and their activation status, e.g., reference point and availability timer, if applicable. Such procedure would not require any SI change.

For what concern the dynamic indication of additional PRACH resources, we observe that the latter indicates whether the additional PRACH resources (provided by semi-static signalling) are available or not. In this context, if a mask is provided to the UE, the mask selection is provided via semi-static signalling as discussed in the previous section, not in DCI. This would have the advantage of ensuring that the dynamic adaptation, which may be eventually agreed to be subject to UE capability reporting, would only pertain the activation of the additional resources but not their determination. This should reduce significantly complexity at the UE to support the PRACH adaptation feature. This would also serve the purpose of minimizing the number of bits in the DCI for the PRACH adaptation, since the mask could simply be activated with 1 bit.

Observation-1: Dynamic adaptation of PRACH resources may be eventually agreed to be subject to UE capability reporting. A semi-static indication of the mask selection could then serve as baseline supported by all UEs, with processing of dynamic indication supported only by UEs who reported the corresponding capability, if any.

Proposal-19: RAN1 to also consider higher-layer signalling based on SI, e.g. SIB1, for PRACH adaptation, in addition to the DCI-based adaptation, especially if a UE capability related to the DCI-based adaptation is introduced.

We switch now the focus on the DCI used to carry the activation indication. During previous Rel-19 discussions, it was agreed that at least DCI format 1_0 scrambled with Paging RNTI can carry the adaptation indication for UEs in RRC idle/inactive and connected mode. The payload size of 1 bit for this indication was also included in tan agreement, although this will have to be further confirmed by RAN1. Indeed, DCI 1_0 with P-RNTI would allow for larger indications to take place, given that several reserved bits that are currently not used exist in this DCI. The following working assumption elaborates on such possibilities.

Working Assumption For DCI-based adaptation for additional PRACH resources, Select from the following options for carrying the adaptation indication in DCI format 1_0 with P-RNTI - Option 1: Use reserved bits in the DCI format - FFS: relation (if any) to TRS availability bits / short message indicator in the DCI format - Option 2: Use Bits 5-8 within the Short Message (from upper layers) - Note: Availability should be confirmed by checking with RAN2. - Option 3: Use bits available for both Option 1 and Option 2 FFS: Payload size for adaptation for additional PRACH resources

Now, it is important to note that PRACH adaptation indication would be provided along with other information, e.g. Paging, Short Message, and it should minimize the impact to legacy UEs. For example, it is important to avoid unnecessary reception and decoding of the paging record in the PDSCH by legacy UEs when the paging DCI is meant to deliver only the PRACH adaptation indication for NES-capable UEs.

By selecting Option 1, the reserved bits in Paging DCI are (8-M) or (6-M), where M is the number of bits for the field ‘TRS availability indication’ which ranges from 1 to 6. In some scenarios, the reserved number of bits could be zero, so option 1 cannot be considered. Option 2 offers always the same number of bits available for carrying the one-bit indication in DCI for PRACH adaptation.

Proposal-20: RAN1 to confirm the size of the PRACH adaptation indication in the DCI, i.e., 1 bit, and to select option 2: where one of the reserved bits 5-8 within the Short Message indication is used.

During RAN1 #120, it was also agreed that DCI 1_0 with P-RNTI indicates the availability information for additional PRACH resource from a reference point and for a validity time duration. The validity time duration for availability of additional PRACH resources could be expressed in association (pattern) period and signalled at higher layer, e.g. SIB1, and we don’t see the need to support explicit deactivation of the additional PRACH resources. Indeed, this would come with additional energy consumption at the NW by transmitting a dedicated DCI and UE monitoring (and corresponding energy consumption), while not offering any meaningful added value from the procedure perspective.

Proposal-21: The validity time duration of additional PRACH resources shall be expressed in association (pattern) period and provided to the UE via semi-static signalling, e.g. SIB1.

Proposal-22: RAN1 does not support DCI-based deactivation of additional PRACH resources.

On the reference point for the application of the PRACH adaptation indication provided in DCI 1_0 with P-RNTI, several options were proposed in the previous RAN1 meeting:

- Option 1: SFN of the first PF from the next I-DRX cycle

- Option 2: SFN of the first PF from the current I-DRX cycle

- Option 3: From the first frame of the first PRACH association period after UE receives the DCI

- Option 4: From the first frame of the current SI modification period

- Option 5: From the first frame of the next SI modification period

From our perspective, the expected timescale for the PRACH adaptation to be meaningful may not be in the order of the milliseconds. Additionally, RAN1 should also avoid, if possible, to specify solutions which may result in unnecessary operations to be performed by idle or inactive or connected UEs, if alternatives exist. In this session, Option 4 and Option 5 seem the wisest course of action and are, in our view, identical. Indeed, RO determination at the UE would not change depending on whether Option 4 or Option 5 is supported, only the position of the reference point for the additional ROs would change.

Proposal-23: RAN1 to support Option 4 (From the first frame of the current SI modification period) or Option 5 (From the first frame of the next SI modification period) for determining the reference point for the application of the PRACH adaptation indication provided in DCI 1_0 with P-RNTI.

Agreement

For adaption of PRACH in time-domain, for a connected mode UE, support a 1-bit field in DCI 1_0 with C-RNTI used to trigger PRACH (i.e. PDCCH order) to indicate whether the additional PRACH resource(s) is available for the triggered PRACH.

- FFS: UE behaviour (e.g. applicable resources for PRACH mask index) when it is indicated of additional PRACH resource(s)

- FFS: Details on how to reuse existing bit for the 1-bit indication

Finally, during RAN1 #120, RAN1 agreed on the support of a 1-bit field in DCI 1_0 with C-RNTI used to trigger PRACH (i.e. PDCCH order) to indicate whether the additional PRACH resource(s) is available for the triggered PRACH.

UE behaviour is still to be defined w.r.t. the application resources for PRACH mask index when additional PRACH resource(s) are indicated as available. The design will depend on whether the PRACH resources will be considered as whole by the UE (i.e., legacy and additional together) or as two different and independent sets of resources. If the former scenario is assumed, then the PRACH mask could be applied jointly to additional and legacy ROs (details would need to be discussed and agreed on). Conversely, if the latter scenario is assumed, then RAN1 will have to discuss about which set of ROs, i.e., legacy or additional, the mask applies to, and how. A new PRACH mask table may also be defined, although this depends on which of the two approaches is agreed on for the previous aspect.

Proposal-24: RAN1 to discuss whether additional and legacy ROs are considered jointly (or separately) for the application of the PRACH mask index indicated in DCI 1_0 with C-RNTI, when applicable. Details of possible additional indications or PRACH mask indication application are FFS.

## 2.3 Adaptation of paging occasions

In RAN2 #127bis the following was agreed:

Agreements on paging adaptation - Select option-b as baseline for R19 NES paging enhancement. - R2 should aim at signaling overhead minimization (e.g., Ns=8 and FFS for other larger values) - Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration. - Legacy UE is not barred. - Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration.

In RAN2 #128 it was further agreed:

Agreements on paging adaptation - Introduce value for Ns=8. FFS on 16. - Introduce value for N= T/32. FFS on T/64, T/128, T/256.

In release 17, 3GPP specified the Paging Early Indication (PEI). The PEI can be used to indicate to UEs whether they can expect at least one UE will be paged in the PO addressed by the PEI. Specifically, the PEI is transmitted in advance of the PO, such that the UE can determine whether to prepare to monitor the PO including whether to perform actions to achieve synchronization. In short, the PEI can save UE energy, because the UE can skip some actions, when it knows it will not be paged in the upcoming PO. The PEI is therefore an important UE energy saving feature, which must also be supported, when the release 19 NES paging enhancements are applied.

The PEI is provided via DCI format 2_7 and consists of a paging indication field, which is the product of the number of POs indicated by the PEI and the number of subgroups per PO, and a TRS availability indication bitmap. Currently, the number of POs per PEI is in the range [1, 2, 4, 8] and since the legacy number of POs per PF is 4, the configuration of 8 POs per PEI implies two PFs are addressed by the PEI. This seems unlikely in the low paging load scenario currently targeted for paging enhancements, because the PF interval would be long, which would then result in a very long time-distance between the PEI and the second PF. Therefore, we expect the number of POs per PEI to be configured as maximum 4.

In RAN1 #120 a proposal was discussed (R1-2501600) regarding the use of PEI for the additional release 19 POs.

## Proposal 4.1.1 From RAN1 perspective, study based on following options for PEI bits (in DCI 2_7) handling in case of legacy and additional Rel-19 POs - Option 1: PEI bits for legacy POs and additional Rel-19 POs are always multiplexed in the same DCI 2_7. - Option 2: PEI bits for legacy POs and additional Rel-19 POs are not multiplexed in the same DCI 2_7. - Option 3: Option 1 and Option 2 are supported via gNB configuration - [Option 4: No additional handling is required]

According to our understanding, the network energy saving potential is maximized if the network can address both legacy POs and additional release 19 POs using a single PEI with DCI format 2_7. If the network is required to transmit two DCIs using format 2_7 it will increase both the physical resource consumption and impact the network sleeping opportunities. Furthermore, if the network is required to transmit two DCIs it will limit the network scheduling flexibility as two PDCCH monitoring occasions will be consumed, which essentially means less scheduling options for other use cases.

Observation-2: The network energy saving potential is maximized if the network can address both legacy POs and additional release 19 POs using a single PEI with DCI format 2_7.

At RAN1 #120 the options 1 and 2 proposed to always or never multiplex the legacy POs and additional release 19 POs. In our view there is however a third option, where a subset of the additional release 19 POs are multiplexed with the legacy POs in the legacy DCI format 2_7 bit positions, while the remaining release 19 POs are not multiplexed and instead allocated the legacy spare bits, such that all addressing is done in a single DCI format 2_7.

Observation-3: A subset of the additional release 19 POs can be multiplexed with the legacy POs in a DCI format 2_7, while the remaining release 19 POs can be addressed by spare bits in the same DCI format 2_7.

For example, assuming the legacy number of POs is 4 (0-3) and the additional release 19 POs is 8 (0-7), it would be feasible to address the first 4 (0-3) of the additional release 19 PO together with the legacy POs, while the remaining 4 POs (4-7) can be addressed in the bits that appear as spare bits to the legacy UEs. If the number of POs per PEI is 4 there may be spare bits in the DCI format 2_7, because it has a maximum size of 43 bits including up to 6 bits for the TRS availability indication. Assuming 4 subgroups per PO, up to 4*4+6=22 bits will be used.

With spare 22 bits the network can for example

- accommodate 8 additional release 19 POs each having 2 subgroups per PO, in total 16 bits, or

- accommodate the additional release 19 POs #5-8 each having 4 subgroups PO, in total 16 bits, while the additional release 19 POs #1-4 can be multiplexed with the legacy POs.

Observation-4: There may be more than 20 spare bits in the PEI DCI format 2_7, when the number of legacy POs per PEI is 4.

It is still an open question whether the legacy POs and the first 4 additional release 19 POs (0-3) correspond to the same PDCCH monitoring occasions for the DCI format 1_0 scrambled with P-RNTI. Independently of this aspect we do not see a problem in multiplexing the 4 legacy POs and the first 4 additional release 19, because the paging enhancement is assumed to be applied in a low load scenario and therefore the impact to the false alarm rate will be limited.

Observation-5: Multiplexing legacy POs and additional release 19 POs is not problematic for the false alarm rate since the paging load is low.

Proposal-25: A single PEI with DCI format 2_7 can address both legacy POs and additional release 19 POs using a mixture of multiplexing and separate signalling.

If RAN1 cannot reach consensus on using a single PEI with DCI format 2_7, it will be beneficial for network energy saving to provide a second PEI for additional release 19 POs only when such a second PEI (DCI format 2_7) is needed.

Proposal-26: If RAN1 decides to use two PEIs, a second PEI for additional release 19 POs shall only be transmitted when needed.

## 2.3.1 PRACH Adaptation in time domain in conjunction with adaptation of paging occasions

The PRACH load level will inherently increase if the paging occasions are confined in time domain based on the release 19 paging enhancement. This is because the UEs will have to be paged during such a confined time period, thereby causing the UEs to respond to the paging also in a confined time interval. The response will require PRACH resources independently of whether it is for RRC Connection setup or Small Data Transmission.

Therefore, it will be beneficial for RAN1 to consider PRACH adaptation in the time domain, when the paging enhancement is applied. For example, means to distribute the load will be relevant to avoid PRACH preamble collisions, when many UEs react to paging in a confined time period.

Proposal-27: RAN1 to consider PRACH adaptation in time domain to spread PRACH load when paging occasion adaptation is used.

# Conclusions

In this contribution, we have the following observations and proposals:

Proposal-1: RAN1 considers the scenarios for SSB burst periodicity adaptation at least with Scenarios #2, #2A and #3B in AI 9.5.1.

Proposal-2: The number of bits required for indicating the adapted SSB periodicity in DCI format 2_9 depends on the number of configured candidate SSB burst periodicity values in RRC.

Proposal-3: Support indication of SSB adaptation jointly or independently from Cell DTX/DRX operation.

Proposal-4: Support that UE is configured to monitor DCI 2_9 with cellDTRX-RNTI for SSB adaptation.

Proposal-5: Discuss the processing time of DCI 2_9 when it indicates only SSB adaptation and when it indicates both Cell DTX/DRX and SSB adaptation.

Proposal-6: New methods to handle missed DCI is not introduced in R19.

Proposal-7: DCI or MAC-CE signalling can be considered for indicating SSB adaptation in time domain after completion of the SCell activation.

Proposal-8: Signaling of SSB adaptation in time domain for SCell should be considered jointly with the MAC-CE specified for OD-SSB at least for SCell activation. Design details, including any differentiation aspects from the OD-SSB design, can be further considered in RAN2.

Proposal-9: Support direct SCell activation with SSB adaptation in time domain based on RRC signalling.

Proposal-10: RAN1 to discuss and decide how adaptation of SSB periodicity affects rate matching operation and CSI measurements and reporting.

Proposal-11: RAN1 to clarify whether Rel-19 additional ROs could be used for 2-step PRACH procedure and, if yes, how.

Proposal-12: PRACH adaptation applied to CFRA should also be investigated, if time allows, however no CFRA-specific PRACH adaptation solutions should be considered.

Proposal-13: The Rel-19 PRACH mask for NES is from a PRACH mask table.

Proposal-14: The Rel-19 PRACH mask should apply the unit of association periods.

Proposal-15: The PRACH mask operates over association periods, and is reset every N association periods, or every association pattern periods. Details about N, if applicable, are FFS.

Proposal-16: If the semi-static configuration of a 4-bit mask is supported, the following Rel-19 NES PRACH mask is supported:

| PRACH Mask Index/ msgA-SSB-SharedRO-MaskIndex/ ssb-SharedRO-MaskIndex | Association periods with additional valid ROs in a sequence of N association periods or in an association pattern period |
| --- | --- |
| 0 | All |
| 1 | mod(association period index, N)=0 |
| 2 | mod(association period index, N)=1 |
| 3 | mod(association period index, N)=2 |
| 4 | mod(association period index, N)=3 |
| 5 | mod(association period index, N)=4 |
| 6 | mod(association period index, N)=5 |
| 7 | mod(association period index, N)=6 |
| 8 | mod(association period index, N)=7 |
| 9 | Every even association period |
| 10 | Every odd association period |
| 11 | Reserved |
| 12 | Reserved |
| 13 | Reserved |
| 14 | Reserved |
| 15 | Reserved |

Proposal-17: If the semi-static configuration of a 3-bit mask is supported, the following Rel-19 NES PRACH mask is supported:

| PRACH Mask Index/ msgA-SSB-SharedRO-MaskIndex/ ssb-SharedRO-MaskIndex | Association periods with additional valid ROs in a sequence of N association periods or in an association pattern period |
| --- | --- |
| 0 | The first half of N association periods |
| 1 | The second half of N association periods |
| 2 | The first quarter of N association periods |
| 3 | The second quarter of N association periods |
| 4 | The third quarter of N association periods |
| 5 | The fourth quarter of N association periods |
| 6 | Every even association period |
| 7 | Every odd association period |

Proposal-18: If the semi-static configuration of a 2-bit mask is supported, the following Rel-19 NES PRACH mask is supported:

| PRACH Mask Index/ msgA-SSB-SharedRO-MaskIndex/ ssb-SharedRO-MaskIndex | Association periods with additional valid ROs in a sequence of N association periods or in an association pattern period |
| --- | --- |
| 0 | The first half association periods |
| 1 | The second half association periods |
| 2 | Every even association period |
| 3 | Every odd association period |

Observation-1: Dynamic adaptation of PRACH resources may be eventually agreed to be subject to UE capability reporting. A semi-static indication of the mask selection could then serve as baseline supported by all UEs, with processing of dynamic indication supported only by UEs who reported the corresponding capability, if any.

Proposal-19: RAN1 to also consider higher-layer signalling based on SI, e.g. SIB1, for PRACH adaptation, in addition to the DCI-based adaptation, especially if a UE capability related to the DCI-based adaptation is introduced.

Proposal-20: RAN1 to confirm the size of the PRACH adaptation indication in the DCI, i.e., 1 bit, and to select option 2: where one of the reserved bits 5-8 within the Short Message indication is used.

Proposal-21: The validity time duration of additional PRACH resources shall be expressed in association (pattern) period and provided to the UE via semi-static signalling, e.g. SIB1.

Proposal-22: RAN1 does not support DCI-based deactivation of additional PRACH resources.

Proposal-23: RAN1 to support Option 4 (From the first frame of the current SI modification period) or Option 5 (From the first frame of the next SI modification period) for determining the reference point for the application of the PRACH adaptation indication provided in DCI 1_0 with P-RNTI.

Proposal-24: RAN1 to discuss whether additional and legacy ROs are considered jointly (or separately) for the application of the PRACH mask index indicated in DCI 1_0 with C-RNTI, when applicable. Details of possible additional indications or PRACH mask indication application are FFS.

Observation-2: The network energy saving potential is maximized if the network can address both legacy POs and additional release 19 POs using a single PEI with DCI format 2_7.

Observation-3: A subset of the additional release 19 POs can be multiplexed with the legacy POs in a DCI format 2_7, while the remaining release 19 POs can be addressed by spare bits in the same DCI format 2_7.

Observation-4: There may be more than 20 spare bits in the PEI DCI format 2_7, when the number of legacy POs per PEI is 4.

Observation-5: Multiplexing legacy POs and additional release 19 POs is not problematic for the false alarm rate since the paging load is low.

Proposal-25: A single PEI with DCI format 2_7 can address both legacy POs and additional release 19 POs using a mixture of multiplexing and separate signalling.

Proposal-26: If RAN1 decides to use two PEIs, a second PEI for additional release 19 POs shall only be transmitted when needed.

Proposal-27: RAN1 to consider PRACH adaptation in time domain to spread PRACH load when paging occasion adaptation is used.

# References

- R1-2501772, “On-demand SSB SCell Operation”, Nokia, Nokia Shanghai Bell, RAN1#120bis, Wuhan, China, April 7th – 11th, 2025.

- R1-2501773, "On-demand SIB1 for Idle/Inactive mode UEs", Nokia, Nokia Shanghai Bell, RAN1#120bis, Wuhan, China, April 7th – 11th, 2025.