3GPP TSG RAN WG1 #120bis		R1-2502003

Wuhan, China, April 7th – 11th, 2025

Source:	CATT, CICTCI

Title:	Discussion on ISAC channel modelling

Agenda Item:	9.7.2

Document for:	Discussion and Decision

Introduction

Integrated Sensing and Communication (ISAC) represents an innovative field of study that focuses on merging wireless communication capabilities with sensing functionalities within a unified framework. Utilizing the same set of hardware and signal-processing techniques, ISAC has the potential to unlock novel applications and services. These include precise location tracking, gesture detection, intruder detection, and the reconstruction of physical environments. Additionally, ISAC offers the prospect of enhancing both the efficiency and effectiveness of wireless network systems. As a cutting-edge technology, ISAC is poised to play a pivotal role in the advancement of future mobile communication generations, including 5.5G and 6G. Therefore, it is important to establish a solid channel modelling framework to enable evaluation of sensing techniques. In [1], a Rel-19 study on channel modelling for ISAC is approved, focusing on object detection and/or tracking.  Specifically, the following objectives have been agreed to be the high-level guidelines:

- Define channel modelling details for sensing using 38.901 as a starting point, and taking into account relevant measurements, including:

- modelling of sensing targets and background environment, including, for example (if needed by the above use cases), radar cross-section (RCS), mobility and clutter/scattering patterns;

- spatial consistency.

In this contribution, we discuss the details related to channel modelling methodology. Our views on deployment scenario and initial simulation results for calibration can be found in our companion paper [2].

Target channel modelling

Reference TR

In RAN1#120 meeting, several agreements were achieved regarding reference TRs to generate TRP-TRP channel, UE-UE channel and TRP-UE channel [3]. However, there is still ambiguity in the references for UAV-UE channels. Two options may be considered: One is to used UE-UE channel defined in TR 38.858 [4], which is mainly used to generate channel for between cellular UEs. The other is to use TRP-UE channel defined in TR 36.777 [5], which is designed for TRP-UAV channel. Simulation results of TRP-UE bistatic coupling loss of these two options along with the result of TRP-TRP bistatic coupling loss are provided in Figure 1, where one cellular UE as sensing TX/RX is random distributed in each cell.

Figure 1: Target channel coupling loss of TRP-UE and TRP-TRP bistatic sensing mode

It can be observed that the coupling loss of TRP-UE bistatic sensing mode is comparable with TRP-TRP bistatic sensing mode if TR 36.777 is used to generate UAV-UE channel. However, when using TR 38.858, nearly 20 dB gap appears for 40% CDF point , which is unreasonable considering the only difference is the height between TRP and UE. Therefore, it is proposed to use TRP-UAV channel defined in TR 36.777 when modelling the channel between UAV and cellular UE.

- TRP-UAV channel defined in TR 36.777 is used to model the channel between UAV and cellular UE.

For highway and urban grid scenario, since RSU-type UE can be considered as sensing TX/RX, channel model between RSU and vehicle, between TRP and RSU, and between two RSUs should be defined. According to TR 36.885[9], channel model between a UE and a RSU and between two UE-type RSUs can reuse the V2V channel model with antenna height at RSU changed to 5m. Channel model between a TRP and a RSU should reuse B2R link modelling in TR 37.885 [10].

- For highway and urban grid scenario,

- Channel model between a UE and a RSU and between two UE-type RSUs reuse the V2V channel model with antenna height at RSU changed to 5m, as defined in TR 37.885.

- Channel model between a TRP and a RSU should reuse the B2R link modelling in TR 37.885.

Large scale fading

Large scale fading refers to the attenuation and fluctuations of signal when the signal is transmitted over a long distance. It also covers the attenuation of signal power due to obstacles between the transmitter and receiver. In the modelling of integrated sensing and communication channels, large-scale fading primarily describes the power variation at the sensing receiver located at a certain distance from the sensing transmitter. Additionally, the impact of EO type-2 on pathloss needs further investigation.

LOS/NLOS probability

In RAN1#119 meeting, the following agreements have been reached for LOS/NLOS probability [7]:

Agreement(RAN1#119) When the EO type-2 is modelled in the target channel, down select between the following options to determine the LOS condition of the Tx-target link and target-Rx link - Option A: If type-2 EO is in the LOS ray of one link, the link is determined as NLOS condition, and otherwise use the LOS probability equation to determine the LOS/NLOS condition - FFS changes to the LOS probability defined in existing TRs - FFS details on blockage by EO type-2 - Option B: Use the LOS probability equation to determine the LOS/NLOS condition of one link, and then the impacts of type-2 EO is modeled by a blockage model

When the EO type-2 is modelled in the target channel, down select between the following options to determine the LOS condition of the Tx-target link and target-Rx link

- Option A: If type-2 EO is in the LOS ray of one link, the link is determined as NLOS condition, and otherwise use the LOS probability equation to determine the LOS/NLOS condition

  - FFS changes to the LOS probability defined in existing TRs

  - FFS details on blockage by EO type-2

- Option B: Use the LOS probability equation to determine the LOS/NLOS condition of one link, and then the impacts of type-2 EO is modeled by a blockage model

Given that the existing TRs (e.g., 3GPP TR 38.901) already take into account the effects of EO, and noting that the Rx could be far from EO type-2, which is considerably larger than the wavelength (leading to negligible diffraction effects), it is proposed to only use the LOS probability equation to determine the LOS/NLOS condition of one link, i.e. support a simplified Option B that do not consider the impacts of blockage model.

- Support simplified Option B, i.e. using the existing LOS probability equations in TRs without any blockage model.

Radar cross-section (RCS) modelling

    - Single scattering point RCS modelling

In RAN1#120 meeting, the following agreements have been reached for RCS modelling [3].

Agreement(RAN1#120) Agreement For bistatic/monostatic RCS - RCS values/pattern for a scattering point of a target for bistatic sensing is generated by A*B1*B2 (i.e., Option 3 from the agreement in RAN1 #118bis) - RCS values/pattern obtained by setting the same incident/scattered angle in the RCS model for bistatic sensing should be aligned with RCS for monostatic sensing Agreement RCS model and application in ISAC channel generation - To define the RCS model (RCS=A*B1*B2) for a scattering point of a target, when the target type is vehicle, large size UAV, human with RCS model 2, AGV - The values/pattern of the product of component A and B1, i.e., A*B1 is given per target type, expressed in dBsm scale - Component A is expressed in dBsm scale. B1 is dependent on A*B1 and value of component A. - A is equal to a single value per target type - FFS: this allows different values for the same target type with different size, if needed - FFS: this allows different values for monostatic and bistatic sensing, if needed - Component B2 follows log-normal distribution. The mean μB2_dB and variance σB2_dB2 used to characterize 10lgB2 satisfied a fixed relation μB2_dB=-ln1020σB2_dB2. - In the procedure of generating ISAC target channel, B1*B2 is applied after coupling of rays for a STX-SPST link and the corresponding SPST-SRX link before path dropping - In the procedure of generating ISAC target channel, the following power scaling factor is applied in the last step in target channel generation (i.e., step 14 in the running CR). LTX-SPST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2 Where, - PLdBd1 is pathloss between Tx and SPST, where d1 is the distance between Tx and SPST - PLdBd2 is pathloss between Rx and SPST, where d2 is the distance between SPST and Rx - σRCS,A is the value of RCS component A - SFdB,1,SFdB,2 are shadow fading respectively generated for the Tx- SPST link and SPST -Rx link referring to step 4 in section 7.5, TR 38.901 - Note: for monostatic sensing, PLdBd1=PLdBd2,SFdB,1=SFdB,2 Agreement RCS upper bound: k equals to 3 is adopted to derive the upper bound of RCS component B2, kσ, where σ is the standard deviation of B2 in dB Agreement For vehicle with single/multiple scattering points: - For mono-static, the RCS=A*B=A*B1*B2 for a scattering point of a vehicle is generated by - The values/pattern A*B1, i.e., rcsdB(θ,φ) is deterministic based on incident/scattered angles rcsdB(θ,φ)=Gmax-min-σVdBθ+σHdB φ,σmax Where, σVdBθ=-min12θ-θcenterθ3dB2, σmax, σHdB φ=-min12φ-φcenterφ3dB2, σmax, For example, in case of vehicle with multiple scattering points: φcenter; φ3dB, n; θcenter; θ3dB,n; Gmax; σmax; Applicable Range of θ; Applicable Range of φ. Left; [ ]; [ ]; [ ]; [ ]; [ ]; [ ]; [ ]; [ ]. Back; [ ]; [ ]; [ ]; [ ]; [ ]; [ ]; [ ]; [ ]. Right; [ ]; [ ]; [ ]; [ ]; [ ]; [ ]; [ ]; [ ]. Front; [ ]; [ ]; [ ]; [ ]; [ ]; [ ]; [ ]; [ ]. Roof; [ ]; [ ]; [ ]; [ ]; [ ]; [ ]; [ ]; [ ]. - Note: the applicable angular range is 360 degrees per row in horizontal domain in case of vehicle with multiple scattering points, and the applicable angular range is < 360 degrees per row in horizontal domain in case of vehicle with a single scattering point. - FFS: angular continuity Agreement For mono-static, the following values of component A, B2 are agreed for UAV of small size - Component A: -12.81 dBsm - Component B1: 0 dB (already agreed in RAN1#118bis) - Component B2: - standard deviation: 3.74 dB Agreement For mono-static, the following values of component A, B2 are agreed for RCS model 1 of human - Component A: -1.37 dBsm - Component B1: 0 dB (already agreed in RAN1#118bis) - Component B2: - standard deviation: 3.94 dB

In RAN1#120 meeting, RCS values/pattern for bistatic and monostatic sensing should be aligned when setting the same incident/scattered angle. With consideration of the consistency premise of bistatic and monostatic RCS modelling method, the following three Options are given in FL summary R1-2501001 [8].

[H][FL3] Proposal 4.1-2 Study the following options for A*B1 of RCS for a scattering point of a target bistatic sensing: - Option 1: RCSbi=RCSmo∙abscosβ2, β is bistatic angle [0, 360] degree - Option 2: bistatic RCS is constructed into 3 angular regions according to scattered angles - Region 1: derive the RCS based on monostatic RCS - This may or may not be Option 1 - Region 2: derive the RCS around the angles near to the specular reflection angle - Region 3: RCS value including large effect of diffraction - Option 3: bistatic RCS is constructed into 3 angular regions according to scattered angles - Region 1+2: σB(f,θ,θ+β)=4π λ2S2cos2(θ+β)∙sinc22πaλsin(θ+β/2)∙cos(β/2) - Where a is the length of a rectangle (cf R1-2501212) - The formula may be further updated - Region 3: RCS value including large effect of diffraction - Distance dependent RCS is not considered in any incident/scattered angles Which option is selected/updated will be based on validation result

Option 2 and Option 3 are too complicated to realize and lacks relevant measurement data support. Option 1 of Proposal 4.1-2 in FL summary R1-2501001 [8], which bistatic RCS can be obtained based on monostatic RCS and a multiplier related to bistatic angle is more reasonable. Therefore, A multiplier related to bistatic angle is enough to complete the conversion between bistatic and monostatic RCS, such as

RCSbi=min{RCSmo∙abscosβ2,  RCS_MIN}.                                                        (1)

In addition, B2 is a random variable following a log-normal distribution. If B2 for different symbol is generated independently, the sensing performance will be degraded by the fluctuant amplitude of RCS, e.g. the velocity sensing results of independent B2 per slot as shown in Appendix. Thus, B2 of RCS remain unchanged even if target position changes during simulation by default. If spatial consistency is considered, B2 of RCS may be further added in the correlation parameter list.

- A multiplier related to bistatic angle is enough to complete the conversion between bistatic and monostatic RCS, such as RCSbi=min{RCSmo∙abscosβ2, RCS_MIN}.

- By default, B2 of RCS remains unchanged even if target position changes during simulation. If spatial consistency is considered, B2 of RCS may be further added in the correlation parameter list.

In RAN1#120 meeting, the following agreements have been reached for polarization effect modelling.

Agreement(RAN1#120) Agreement To model polarization matrix CPMsp,i=αi,1expjΦsp,iθθβi,1expjΦsp,iθϕβi,2expjΦsp,iϕθαi,2expjΦsp,iϕϕ of a direct/indirect path i of a scattering point of a target - in Rel-19 study item (e.g., UAV, human, vehicle, AGV), αi,1=αi,2=1, βi,1=βi,2=κsp,i-1, i.e., CPMsp,i=expjΦsp,iθθκsp,i-1expjΦsp,iθϕκsp,i-1expjΦsp,iϕθexpjΦsp,iϕϕ Where, - κsp,i is XPR ratio is randomly generated by log-normal distribution per target type - The initial random phase Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ is uniformly distributed within -π,π - FFS: spatial consistency of random phase when a scattering point moves - FFS: whether the base station rotation procedure in 38.901 can be reused to support rotation of the target

In RAN1#120 meeting, the polarization matrix of a scattering point of a target is modelled by XPR ratio and initial random phases. The remaining issue is spatial consistency of random phase when a scattering point moves. The variation of random phase due to the mobility of target is reasonable. However, the variation of phase will impact the velocity sensing which is shown in Appendix. The changing speed of correlation parameters can be controlled by correlation distance. As in section 7.6.3 in TR 38.901, the correlation parameters include XPR and initial random phase which is a good start. In total, initial random phase of target CPM remains unchanged even if target position changes during simulation. If spatial consistency is considered, initial random phase of target CPM may be further added in the correlation parameter list. Another issue is whether the base station rotation procedure in TR 38.901 can be reused to support rotation of the target. The base station rotation procedure for CPM of target is not necessary because the XPR and random phase are generated statistically. In addition, the angle dependent RCS of target is determined by incident and scattering angle, which should be modelled in LCS centered on the sensing target. The rotation process includes the conversion of the sensing target to the transmitting and receiving node respectively. The base station rotation procedure in TR 38.901 can be reused.

- By default, initial random phase of target CPM remains unchanged even if target position changes during simulation. If spatial consistency is considered, initial random phase of target CPM may be further added in the correlation parameter list.

- The base station rotation procedure for CPM of target is not necessary because the XPR and random phase are generated statistically.

Multiple scattering points RCS modelling

In RAN1#120 meeting, the following agreements have been reached for multiple scattering points RCS modelling [3].

Agreement To generate the LOS ray and NLOS clusters for the multiple scattering points, each scattering point is separately handled as if a different target with single scattering point. Agreement - The LOS condition between Tx/Rx and each of the multiple scattering points of a same target are individually generated - The pathloss between Tx/Rx and each of the multiple scattering points of a same target are individually generated Agreement For a target with single/multiple scattering points, the 3D location of each scattering point is defined in the evaluation assumptions. Agreement If a target is modelled with multiple scattering points, - The number of scattering points of the target is generated in the beginning of the simulation and kept unchanged in the whole simulation - The number and locations of the scattering points of the target (if it is a vehicle) are common to each pair of sensing Tx/Rx - RAN1 assumes no ray is scattered from one scattering point to another scattering point of the same target RCS values of the multiple scattering points are individually determined

For the exact 3D location of the multiple scattering points, one possible solution is the location is at the center of each plane. To further simplify the simulation, the number of scattering points of a target keeps unchanged in the whole simulation. However, the problem is that there may be some points invisible to the BS. These invisible points will cause the RCS value to drop sharply. According to the agreements of RAN1#120 meeting, RCS values of the multiple scattering points are individually determined. One possible remedy is that the invisible scattering point from an incident or scattered direction can modelled by angular dependent RCS component B1 at the target. In this way, the overall number can be fixed during the movement, which is more feasible for the actual modeling of simulation.

- An invisible scattering point from an incident or scattered direction is modelled by angular dependent RCS component B1 at the target.

Small scale fading

In RAN1#119 and RAN1#120 meeting, the following agreements have been reached for the sensing target channel modelling [3] [7].

Agreement(RAN1#119) - To generate indirect paths of NLOS ray + NLOS ray in the target channel - Option 0 is recommended, i.e., ray level full convolution between Tx-target link and target-Rx link for radio propagation Case 1/2/3/4 - Option 3 to generate a reduced number of indirect paths of NLOS ray + NLOS ray is recommended, i.e., ray level 1-by-1 random coupling between Tx-target link and target-Rx link is supported for radio propagation Case 1/2/3/4 - If number of rays in the two links are different, e.g., M1, M2 respectively for link 1 and link 2, - If M1<M2, randomly M1 rays are selected in link 2, otherwise randomly M2 rays are selected in link 1 - Other methods are up to company choice for complexity reduction - Both option 0 and 3 will be calibrated independently. Company should report which option is used in calibration - The power threshold for path dropping is X=[-25] dB - X is relative to the strongest indirect path in the target channel - FFS: further power normalization of target channel is performed after path dropping, - Note: power normalization when target channel and background channel are combined can be discussed separately FFS The set of remaining indirect paths can be updated during movement of Tx, target or Rx Agreement(RAN1#120) - For target channel - The power threshold for removing clusters in step 6 in section 7.5, TR 38.901, i.e., -25 dB is reused to generate Tx-target link and target-Rx link - The power threshold for path dropping after concatenation is relaxed to [X=-40] dB - For background channel - The power threshold for removing clusters in step 6 in section 7.5, TR 38.901, i.e., [-25 dB] is reused to generated the background channel - FFS: whether to add additional very low power clusters - FFS: The reference power for removing cluster is the min (max. Tx-target link cluster power, max. target-Rx link cluster power)

Issue 1: The power threshold for path dropping X after concatenation

The following rules can be considered for determining the path dropping power threshold of indirect paths after concatenation.

- Rule 1: Taking the RMS delay spread (DS) τRMS and RMS angle spread (AS, e.g., ASD/ZSD/ASA/ZSA) θRMS without path dropping as baseline, the relative loss of RMS DS τ'RMS with path dropping and the loss of  RMS AS θ'RMS with path dropping are both less than a given threshold. For example, the mean relative error (MRE) [19] of RMS DSMREτPth is 5% and MRE of RMS AS  MREθPth is 5%.

MREτPth=Eτ'RMS-τRMSτRMS                                                                   (2)

MREθPth=Eθ'RMS-θRMSθRMS                                                                  (3)

- Rule 2: Taking the RMS DS τRMS without path dropping as baseline, the relative loss of RMS DS τ'RMS with path dropping is less than a threshold. For example, the MRE of RMS DS MREτPth is 5%.

Rule 1 can achieve more accurate matching of baseline channel models without path dropping compared to Rule 2 at the cost of higher computational complexity. Rule 2 reuses the same rule used in the channel model step 6 of section 7.5 of TR 38.901, where the RMS angle has little impact on the equivalent channel gain of spatially multi stream channel. The simulation results in R1-2409693 shows that the computational complexity of Rule 2 is lower than that of Rule 1 as the path dropping power threshold of Rule 2 is generally higher than that of Rule 1. Therefore, Rule 1 is recommended.

- It is recommended to use Rule 1 to determine the power threshold X for path dropping.

    - Rule 1: Taking the RMS DS τRMS and RMS AS θRMS without path dropping as baseline, the relative loss of RMS DS τ'RMS with path dropping and the loss of  RMS AS θ'RMS with path dropping are both less than a given threshold. For example, the mean relative error (MRE) of RMS DS MREτPth is 5% and MRE of RMS AS MREθPth is 5%.

Issue 2: Whether to update the scaling factors for AOA, AOD, ZOA, ZOD generation in step 7 after removing clusters with -25 dB threshold in step 6 in section 7.5, TR 38.901

In step 6 in section 7.5, TR 38.901, the cluster powers are normalized so that the sum of all cluster powers is equal to one by Equation (7.5-6), i.e., . After removing clusters with less than -25 dB power compared to the maximum cluster power, the scaling factors need not be changed after cluster elimination. There may be two interpretation of scaling factors in step 6: 1) the first one is Equation (7.5-6); 2) the second one is the scaling factor  and  in step 7 in section 7.5, TR 38.901. For the first interpretation of scaling factors in step 6, there will be no definition of scaling factor  and if the total number of remaining clusters after removing clusters with less than -25 dB power is not the same as the value in Table 7.5-2. For the second interpretation of scaling factors in step 6, the scaling factors for AOA, AOD, ZOA, ZOD generation need not be changed in step 7. In our opinion, the second interpretation should be used.

- The scaling factors in step 6 also apply to the scaling factor  and  in step 7 in section 7.5, TR 38.901, where the scaling factors need not be changed after cluster elimination in step 6 in section 7.5.

Another issue is about determination of the maximum power for removing clusters before target channel concatenation. Based on the agreement for path dropping after target channel concatenation, the maximum power is determined by the strongest indirect path in the target channel. However, the current description for removing clusters in TR 38.901 is pretty vague, different companies should align the understanding. Whether the maximum cluster power includes the power of the single LOS ray should be clarified.

- For the step of removing clusters before channel concatenation, whether the maximum cluster power includes the power of the single LOS ray should be clarified.

Issue 3: Whether to update the set of remaining indirect paths during movement of Tx, target or Rx

For spatial consistency procedure A in Section 7.6.3.2 in TR 38.901, the updated distance of UT/BS should be limited within a given distance D = min{1m, d_3D/10} with the use of the absolute time of arrival modeling, where d_3D is the distance between BS and UE in 3D. In our opinion, the same method can be reused to make the channel model of target channel for tracking simple. The set of indirect paths generated by Option 3 is not updated during movement of Tx, target or Rx, within a given distance D = min{1m, d_3D_min /10}, where d_3D_min is the minimum 3D distances between pairs of Tx/Rx and sensing target. The set of remaining indirect paths after path dropping after concatenation is not updated during movement of Tx, target or Rx, within a given distance D = min{1m, d_3D_min /10}, where d_3D_min is the minimum 3D distances between pairs of Tx/Rx and sensing target.

- The set of indirect paths generated by Option 3 is not updated during movement of Tx, target or Rx, within a given distance D = min{1m, d_3D_min /10}, where d_3D_min is the minimum 3D distances between pairs of Tx/Rx and sensing target.

- The set of remaining indirect paths after path dropping after concatenation is not updated during movement of Tx, target or Rx, within a given distance D = min{1m, d_3D_min /10}, where d_3D_min is the minimum 3D distances between pairs of Tx/Rx and sensing target.

Issue 4: Whether to do power normalization of NLOS+NLOS indirect paths before path dropping for Option 3

In RAN1#119, one reason for supporting power normalization of NLOS+NLOS indirect paths for Option 3 is that, the performance of the RMS DS and RMS AS CDF distribution curves is consistent with the that of Option 0 as baseline only after power normalization. However, from the perspective of reality channel and sensing detection performance, it is unreasonable. The reason lies in that the power of a certain indirect path relative to the direct path has been artificially raised. Take CASE 4 (NLOS+NLOS) as an example, power of reserved indirect paths may be increased by 400 times. Both the rule of RMS DS and RMS AS consistence compared to the baseline and the rule that conforms to the reality channel model should be considered. Therefore, no power normalization of NLOS+NLOS indirect paths before path dropping for Option 3.

- No power normalization of NLOS+NLOS indirect paths before path dropping for Option 3.

Issue 5: Remaining issues of Mono-static target channel modelling

It has been agreed for Mono-static target channel modelling that the same LOS condition, pathloss, shadow fading and Δτ are used for Tx-target link and target-Rx link. But we still have no agreement on the determination of delay, angle, K factor and polarization matrix for Tx-target link and target-Rx link. Because Tx-target link and target-Rx link have the same LOS condition, same K factor should be used for two separate links, LOS AOA and LOS ZOA of target-Rx link should be the same as LOS AOD and LOS ZOD of Tx-target link, and LOS AOD and LOS ZOD of target-Rx link should be the same as LOS AOA and LOS ZOA of Tx-target link. For the left parts such as delay spread, angle spread and polarization, which mainly affect the generation of NLOS clusters. The NLOS clusters of Tx-target link and target-Rx link can be totally different in the environment, so the NLOS clusters (DS, angle spread and polarization) for the two separate links should be generated independently. Besides, special handling of angle spread generation may be required. The ASA and ZSA of target-Rx link should obey the same statistical distribution characteristics of ASD and ZSD of Tx-target link , respectively, and the ASD and ZSD of target-Rx link should obey the same statistical distribution characteristics of ASA and ZSA of Tx-target link, respectively.

- For the Mono-static target channel modelling, the determination of delay, angle, K factor and polarization matrix for Tx-target link and target-Rx link should follow the following rules:

- Same K factor should be used for two separate links.

- LOS AOA and LOS ZOA of target-Rx link should be the same as LOS AOD and LOS ZOD of Tx-target link.

- LOS AOD and LOS ZOD of target-Rx link should be the same as LOS AOA and LOS ZOA of Tx-target link.

- Delay spread, angle spread and polarization for the two separate links should be generated independently.

  - The ASA and ZSA of target-Rx link should obey the same statistical distribution characteristics of ASD and ZSD of Tx-target link, respectively.

  - The ASD and ZSD of target-Rx link should obey the same statistical distribution characteristics of ASA and ZSA of Tx-target link, respectively.

Issue 6: Remaining issues of Bi-static target channel modelling

Regarding to Bi-static target channel modelling, Tx-target link and target-Rx link should be generated independently. However, for some special cases, additional special handling is required. If both Tx and Rx are the same type devices such as TRPs or UEs, the relationship between Tx-target and Rx-target is quite similar as that of Uplink and downlink, the angle spread of Tx and Rx should be very similar or even same. To simplify the process, it is recommended that the ASA and ZSA of target-Rx link should obey the same statistical distribution characteristics of ASD and ZSD of Tx-target link respectively, the ASD and ZSD of target-Rx link should obey the same statistical distribution characteristics of ASA and ZSA of Tx-target link respectively.

- When the sensing Tx and sensing Rx are of the same type in Bi-static mode,

      - The ASA and ZSA of target-Rx link should obey the same statistical distribution characteristics of ASD and ZSD of Tx-target link, respectively.

      - The ASD and ZSD of target-Rx link should obey the same statistical distribution characteristics of ASA and ZSA of Tx-target link, respectively.

EO type-2 model

In RAN1#118 meeting, it was agreed that specular reflection is considered to model EO type-2 using section 7.6.8 of TR 38.901 as reference. However, it should be noted that only LOS condition is considered when modelling ground reflection in TR 38.901 and only ground-type object is considered in section 7.6.8. When modelling EO type-2, wall of buildings is also typical object besides ground, and NLOS condition may exist, but the modelling method is absent in legacy TRs.

- Only LOS condition is considered when modelling ground reflection in TR 38.901. Whether/how to model the impact of deterministic reflection in NLOS condition is absent in legacy TRs.

When EO type-2 is modelled in the case that LOS condition is determined for Tx-Target link or Target-Rx link, a similar modelling method can be considered as defined in section 7.6.8 of TR 38.901. The modelling of ground reflection in TR 38.901 implements the impact of pathloss by adding a scaling factor d3DdGR when generating channel coefficients, instead of introducing a new pathloss formula. Accordingly, for TX-EO-Target path and Target-EO-Rx path, the impact of EO type-2 on pathloss can also be implemented by the scaling factor. Modification of the original pathloss PLd1 and PLd2, which are modeled using the existing pathloss formulas in 3GPP TRs, is not needed. The channel impulse response equation can refer to equation (7.6-32) of explicit ground reflection model as following:

Hu,sLOS with EO-2=1KR+1Hu,sNLOS+KRKR+1(Hu,s,1LOS+d3DdEO-2Hu,sEO-2)                                           (4)

- When EO type-2 is modelled in the case that LOS condition is determined for Tx-Target link or Target-Rx link the impact of EO type-2 on pathloss can be implemented by adding a scaling factor when generating channel coefficients. Modification of the pathloss formula is not needed.

Regarding NLOS condition, two options may be considered when modelling EO type-2. One is not to model EO type-2 when the path between Tx to Target or between Target to Rx is in NLOS condition. That is, only when the path is in LOS condition, EO type-2 is modelled using the method in section 7.6.8 of TR 38.901. The other is still modelling EO type-2 when the path between Tx to Target or between Target to Rx is in NLOS condition, and the modelling method should be further studied.

For a NLOS link, EO type-2 actually creates a reflected LOS ray. Thus, a candidate method to model EO type-2 in NLOS condition is to consider the original NLOS link as LOS condition, by replacing the LOS ray with EO type-2 reflection path. Therefore, the pathloss of target channel in NLOS condition with EO type-2 is calculated based on the LOS condition pathloss model. The channel impulse response equation can refer to equation 7.6-32 in TS 38.901 of explicit ground reflection model but eliminating the channel impulse response of LOS ray as following:

Hu,sNLOS with EO-2=1KR+1Hu,sNLOS+KRKR+1d3DdEO-2Hu,sEO-2                                           (5)

- If EO type-2 is modelled when NLOS condition is determined for Tx-Target link or Target-Rx link:

- The pathloss of Tx-Target link or Target-Rx link in NLOS condition with EO type-2 is calculated based on the LOS condition pathloss model

- The channel impulse response equation of Tx-Target link or Target-Rx link in NLOS condition with EO type-2 should be modified as following:

Hu,sNLOS with EO-2=1KR+1Hu,sNLOS+KRKR+1d3DdEO-2Hu,sEO-2

Background channel modelling

In RAN1#120 meeting, the following agreements have been reached for background channel [3].

Agreement (RAN1#120) - For target channel - The power threshold for removing clusters in step 6 in section 7.5, TR 38.901, i.e., -25 dB is reused to generate Tx-target link and target-Rx link - The power threshold for path dropping after concatenation is relaxed to [X=-40] dB - For background channel - The power threshold for removing clusters in step 6 in section 7.5, TR 38.901, i.e., [-25 dB] is reused to generated the background channel - FFS: whether to add additional very low power clusters FFS: The reference power for removing cluster is the min (max. Tx-target link cluster power, max. target-Rx link cluster power)

In our opinion, in order to keep consistency with existing communication channel and background channel, the same power threshold should be used for removing clusters in step 6 in section 7.5, TR 38.901 with no additional very low power clusters.

- Update the agreement for power threshold for background channel in RAN1#120.

- For background channel

  - The power threshold for removing clusters in step 6 in section 7.5, TR 38.901, i.e., [-25 dB] is reused to generated the background channel.

    - FFS: whether to add additional very low power clusters

    - FFS: The reference power for removing cluster is the min (max. Tx-target link cluster power, max. target-Rx link cluster power)

Combining target channel and background channel

In RAN1#119 meeting, the following agreements have been reached for combining target channel and background channel [7].

Agreement(RAN1#119) - The following options are supported to generate the combined ISAC channel - Option 1: The ISAC channel of a pair of sensing Tx/Rx is obtained by summing the target channel(s) and background channel, i.e., power normalization is not performed. - Option 2: As an additional modelling component, power normalization is performed when summing the target channel(s) and background channel, to keep the same/similar channel power as the background channel without target. Down select between - Alt 1: Power normalization on both target channel and background channel - Alt 2: Power normalization on background channel only - Alt 3: the target channel of a target will replace one cluster in the background channel - FFS Blockage is modelled for the background channel due to sensing target and/or EO type-2 - FFS condition to select option, e.g. depending on scenario, sensing mode, number of target/EO type-2

Down selection needs to be made for Option 2. The NLOS clusters in both the target channel and the background channel share similarities, as they represent indirect paths between the Tx and Rx nodes. From this perspective, normalization is necessary for the NLOS clusters in both the target and background channels to comply with the power requirements outlined in existing TRs (e.g., 3GPP TR 38.901). Furthermore, the target channel possesses unique characteristics, such as different RCS properties compared to the environment. Therefore, the power normalization issue should be addressed from both the target and background channel. If Option 2 is adopted, alternative 1 is preferred: Power normalization on both target channel and background channel.

- If Option 2 is adopted, alternative 1 is preferred: Power normalization on both target channel and background channel.

Spatial consistency

A spatial consistency procedure is defined in TR 38.901 [6], where random variables are generated in various channel generation steps, so that they are spatially consistent for drop-based simulations. The procedure can be considered as a 2D random process (in the horizontal plane) given the target locations based on the parameter-specific correlation distance values for spatial consistency. Furthermore, due to the varying nature of parameters, such as whether a user is indoors or the indoor distance, which is only related to the user's position and not to the base station's location, the channel's large-scale and small-scale parameters are related not only to the user's position but also to the base station's location. This distinction in parameters defines different types of correlations. For parameters like cluster delay and cluster power, which are related to the link, they are determined to have site-level correlation. This means for these parameters, when generating sectors within a site and the users in the network, the spatial consistency of these link channel parameters needs to be considered. However, for links between stations at different sites and users, spatial consistency is not required. Additionally, for parameters not related to the link, such as indoor distance, they are determined to have network-wide correlation. This implies that in generating these parameters, spatial consistency should be considered for all correlated.

It is suggested that this modelling methodology should be reused for ISAC. However, unlike the channel from the base station to the user, in the modelling of the sensing channel, the impact of the sensing target must also be considered. This means that the sensing channel model includes three nodes: the sensing transmitter, the sensing target, and the sensing receiver. Applying the same parameter correlation type as site-specific, as in the base station-user channel, may not be appropriate in this context. For instance, under this configuration, it might be feasible to maintain spatial consistency in the channel from the sensing transmitter to the sensing target. However, introducing spatial consistency in the channel from the sensing target to the sensing receiver could be challenging, potentially leading to significant changes in the overall sensing channel when movement occurs.

In RAN1#120 meeting, the following agreement was achieved regarding the spatial consistency model between TRP and different STs/UEs [3].

Agreement The existing spatial consistency model in TR 38.901 is reused to model correlation of links between one TRP and different STs/UEs.

Considering the distance between RSU-type UEs is comparable with ISD and much larger than the correlation distance. The existing spatial consistency model in TR 38.901 (i.e., site-specific correlation) can also be reused to model correlation of links between one RSU and different STs/UEs from the perspective of simplification and reasonability, instead of using the newly defined link correlation.

- The existing spatial consistency model in TR 38.901 (i.e., site-specific correlation) is reused to model correlation of links between one RSU and different STs/UEs, instead of using the newly defined link correlation.

In RAN1#120 meeting, the following topics regarding spatial consistency were discussed and initial observations are provided in the FL summary R1-2501001 [8]:

- Option 1: global grid (i.e., Unified grid) + shifting UT/ST locations.

- Option 2: direct modelling on the correlation of grids of different UT/ST, e.g., Cholesky decomposition.

- Option 3: UT specific grid only in Option 2.

- Option 4: ST specific grid only in Option 2.

Regarding Option 2 (correlation of grids for different UT / ST ), as the number of UT / ST deployed in the simulation increases, the grid associated with each UT / ST can lead to significant storage overhead and computational complexity. In comparison, Option 1 is simpler to implement and easier to calibrate, thereby providing a more efficient framework for system evaluation while maintaining the required spatial correlation characteristics. Therefore, regarding the spatial consistency design for ISAC channel, it is proposed to support Option 1 by generating global grid first, and obtaining either UE-specific or ST-specific grid by shifting the UT/ST locations.

- Support Option 1: Generating global grid first, and obtaining either UE-specific or ST-specific grid by shifting the UT/ST locations.

Conclusion

In this contribution, we discuss the details related to channel modelling methodology. We have the following observation and proposals:

- Only LOS condition is considered when modelling ground reflection in TR 38.901. Whether/how to model the impact of deterministic reflection in NLOS condition is absent in legacy TRs.

- TRP-UAV channel defined in TR 36.777 is used to model the channel between UAV and cellular UE.

- For highway and urban grid scenario,

    - Channel model between a UE and a RSU and between two UE-type RSUs reuse the V2V channel model with antenna height at RSU changed to 5m, as defined in TR 37.885.

    - Channel model between a TRP and a RSU should reuse the B2R link modelling in TR 37.885.

- Support simplified Option B, i.e. using the existing LOS probability equations in TRs without any blockage model.

- A multiplier related to bistatic angle is enough to complete the conversion between bistatic and monostatic RCS, such as RCSbi= min{RCSmo∙abscosβ2, RCS_MIN}.

- By default, B2 of RCS remains unchanged even if target position changes during simulation. If spatial consistency is considered, B2 of RCS may be further added in the correlation parameter list.

- By default, initial random phase of target CPM remains unchanged even if target position changes during simulation. If spatial consistency is considered, initial random phase of target CPM may be further added in the correlation parameter list.

- The base station rotation procedure for CPM of target is not necessary because the XPR and random phase are generated statistically.

- An invisible scattering point from an incident or scattered direction is modelled by angular dependent RCS component B1 at the target.

- It is recommended to use Rule 1 to determine the power threshold X for path dropping.

    - Rule 1: Taking the RMS DS τRMS and RMS AS θRMS without path dropping as baseline, the relative loss of RMS DS τ'RMS with path dropping and the loss of  RMS AS θ'RMS with path dropping are both less than a given threshold. For example, the mean relative error (MRE) of RMS DS MREτPth is 5% and MRE of RMS AS MREθPth is 5%.

- The scaling factors in step 6 also apply to the scaling factor  and  in step 7 in section 7.5, TR 38.901, where the scaling factors need not be changed after cluster elimination in step 6 in section 7.5.

- For the step of removing clusters before channel concatenation, whether the maximum cluster power includes the power of the single LOS ray should be clarified.

- The set of indirect paths generated by Option 3 is not updated during movement of Tx, target or Rx, within a given distance D = min{1m, d_3D_min /10}, where d_3D_min is the minimum 3D distances between pairs of Tx/Rx and sensing target.

- The set of remaining indirect paths after path dropping after concatenation is not updated during movement of Tx, target or Rx, within a given distance D = min{1m, d_3D_min /10}, where d_3D_min is the minimum 3D distances between pairs of Tx/Rx and sensing target.

- No power normalization of NLOS+NLOS indirect paths before path dropping for Option 3.

- For the Mono-static target channel modelling, the determination of delay, angle, K factor and polarization matrix for Tx-target link and target-Rx link should follow the following rules:

- Same K factor should be used for two separate links.

- LOS AOA and LOS ZOA of target-Rx link should be the same as LOS AOD and LOS ZOD of Tx-target link.

- LOS AOD and LOS ZOD of target-Rx link should be the same as LOS AOA and LOS ZOA of Tx-target link.

- Delay spread, angle spread and polarization for the two separate links should be generated independently.

  - The ASA and ZSA of target-Rx link should obey the same statistical distribution characteristics of ASD and ZSD of Tx-target link, respectively.

  - The ASD and ZSD of target-Rx link should obey the same statistical distribution characteristics of ASA and ZSA of Tx-target link, respectively.

- When the sensing Tx and sensing Rx are of the same type in Bi-static mode,

      - The ASA and ZSA of target-Rx link should obey the same statistical distribution characteristics of ASD and ZSD of Tx-target link, respectively.

      - The ASD and ZSD of target-Rx link should obey the same statistical distribution characteristics of ASA and ZSA of Tx-target link, respectively.

- When EO type-2 is modelled in the case that LOS condition is determined for Tx-Target link or Target-Rx link the impact of EO type-2 on pathloss can be implemented by adding a scaling factor when generating channel coefficients. Modification of the pathloss formula is not needed.

- If EO type-2 is modelled when NLOS condition is determined for Tx-Target link or Target-Rx link:

      - The pathloss of Tx-Target link or Target-Rx link in NLOS condition with EO type-2 is calculated based on the LOS condition pathloss model.

      - The channel impulse response equation of Tx-Target link or Target-Rx link in NLOS condition with EO type-2 should be modified as following:

Hu,sNLOS with EO-2=1KR+1Hu,sNLOS+KRKR+1d3DdEO-2Hu,sEO-2

- Update the agreement for power threshold for background channel in RAN1#120.

- For background channel

  - The power threshold for removing clusters in step 6 in section 7.5, TR 38.901, i.e., [-25 dB] is reused to generated the background channel.

    - FFS: whether to add additional very low power clusters

    - FFS: The reference power for removing cluster is the min (max. Tx-target link cluster power, max. target-Rx link cluster power)

- If Option 2 is adopted, alternative 1 is preferred: Power normalization on both target channel and background channel.

- The existing spatial consistency model in TR 38.901 (i.e., site-specific correlation) is reused to model correlation of links between one RSU and different STs/UEs, instead of using the newly defined link correlation.

- Support Option 1: Generating global grid first, and obtaining either UE-specific or ST-specific grid by shifting the UT/ST locations.

Appendix

1) Range-Doppler Map (RDM) when B2 and initial random phase remain unchanged for all slots.

Figure 2: RDM spectrum

2) RDM when B2 is generated per slot independently.

Figure 3: RDM spectrum

Figure 4: velocity cut of RDM spectrum

3) RDM when initial random phase is generated per slot independently.

Figure 5: RDM spectrum

Figure 6: velocity cut of RDM spectrum

References

- RP-234069, New SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR, RAN#102, Edinburgh, Scotland, December 11th – 15th, 2023.

- R1-2502002, Discussion on ISAC deployment scenarios, CATT, CICTCI, RAN1#120bis, Wuhan, China, April 7th – 11th, 2025.

- Chairman’s Notes RAN1#120, Athens, Greece, February 17th – 21st, 2025

- TR 38.858, “Study on Evolution of NR Duplex Operation”, V18.1.0.

- TR 36.777, “Study on Enhanced LTE Support for Aerial Vehicles”, V15.0.0.

- TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz, V17.0.0.

- Chairman’s Notes RAN1#119, Orlando, US, November 18th – 22nd, 2024.

- R1-2501001, Summary #3 on ISAC channel modelling, Moderator (Xiaomi), RAN1#120, Athens, Greece, February 17th – 21st, 2025.

- 3GPP TR 36.885, “Study on LTE-based V2X Services”, V14.0.0.

- 3GPP TR 37.885, “Study on evaluation methodology of new Vehicle-to-Everything (V2X) use cases for LTE and NR”, V15.3.0.