3GPP TSG RAN WG1 #120-bis		R1-2502697

Wuhan, China, April 7th – 11st, 2025

Agenda Item:	9.11.3

Source:	HONOR

Title:	Discussion on NR-NTN UL capacity/throughput enhancement

Document for:	Discussion and Decision

Introduction

In RAN1#120, the following agreements were made.

Conclusion For OCC time synchronization / alignment of multiplexed UEs to maintain orthogonality of the codes used for OCC, OCC group alignment is handled by network scheduling without specification impact. Conclusion OCC with Msg3 PUSCH is not in scope of Release 19 NR NTN Ph3. Agreement For RV cycling for OCC with DG-PUSCH, support Option 1 - Option 1: RV cycling is used across OCC groups. - Note: when option 1 is applied, RV cycling is applied when the number of repetitions is greater than the OCC length No optimization in Rel-19 for pairing UEs with OCC2 and UEs with OCC4.

In this contribution, we provide our views on improving uplink capacity/throughput by utilizing orthogonal cover code (OCC) on PUSCH, along with the specification aspects associated with OCC techniques.

- Specification aspects of OCC

## 2.1 UCI multiplexing

In RAN1#119, the following was agreed:

Agreement If PUCCH without repetition overlaps with inter-slot OCC with any PUSCH repetitions in an OCC group, the following options to be considered: - Option 1: UCI is dropped - FFS: whether all UCI is dropped - Option 2: UCI is transmitted on PUCCH, and all PUSCH repetitions within the OCC group are dropped. - Option 3: UCI is multiplexed on PUSCH with inter-slot OCC - Option 3-a: UCI is multiplexed on all PUSCH repetitions within an OCC group with inter-slot OCC - FFS: which OCC group - Option 3-b: UCI is multiplexed on PUSCH and OCC is not applied within the OCC group - Option 3-c: UCI is multiplexed on PUSCH and OCC is not applied within the PUSCH repetitions Note: combination of the above can be considered FFS Details timeline of PUCCH and PUSCH FFS: handling of PUCCH with repetition FFS: handling of different UCI types

For Option 2, if all PUSCH repetitions within the OCC group overlapped with PUCCH are dropped, the uplink throughput will be affected and reduced. Since the OCC technology is introduced to enhance the uplink coverage and throughput, we do not support Option 2.

For Option 3-b, if UCI is multiplexed on PUSCH and OCC is not applied within the OCC group, the network needs to allocate additional resources for the PUSCH that does not apply OCC, instead of multiplexing it with other users. This will increase scheduling overhead and reduce spectrum efficiency. For Option 3-c, UCI is multiplexed on PUSCH and OCC is not applied within the PUSCH repetitions, which would cause more PUSCH transmissions to be unable to be multiplexed with other users. Therefore, we support Option 3-a: UCI is multiplexed on all PUSCH repetitions within an OCC group with inter-slot OCC.

For Option 1, we think that whether UCI should be dropped can depend on the UCI timeline. As shown in Figure 1, if the scheduling DCI is received earlier than a time offset from the first slot of the OCC period, the UCI is able to be multiplexed on all PUSCH repetitions within an OCC group. However, if the scheduling DCI is received later than a time offset from the first slot of the OCC period, Option 3-a could not be applied in the current OCC period. In this case, the UCI could be postponed to the next OCC period or dropped.

Figure 1. Timeline of DCI and UCI multiplexing on all repetitions of an OCC period

Proposal 1: Support the combination of Option 1 and Option 3-a, i.e. the UCI is multiplexed on PUSCH with inter-slot OCC or dropped.

Proposal 2: If the scheduling DCI is received later than a time offset from the first slot of the OCC period, the UCI could be postponed to the next OCC group or dropped.

For UCI dropping rules, whether to drop the UCI can depend on the type of UCI. For instance, HARQ-ACK has a higher priority than CSI. In addition, whether a UCI is dropped can depend on the time when UCI occurs. For example, if the scheduling DCI is received later than a time offset from the first slot of the OCC period, and meanwhile the OCC period is the last OCC period, the UCI should be dropped.

Proposal 3: The dropping rules for UCI can be related to the type of UCI.

If UCI is multiplexed on every slot of the OCC group, the resources occupied by UCI will grow by OCC length, leading to resource waste. To reduce resource waste, the resources occupied by UCI in a slot need to be appropriately scaled down. For example, a scaling factor can be introduced to scale down βoffsetPUSCH, which adjusts the number of coded modulation symbols of UCI in the current specification.

Proposal 4: Introduce a scaling factor to scale down βoffsetPUSCH, thereby reducing the number of resources occupied by UCI when UCI is multiplexed on all slots of the OCC group.

## 2.2 OCC indication/configuration

For the configured grant Type 1 PUSCH transmission, the OCC-related parameters can be configured by RRC, as it is semi-statically configured to operate upon the reception of RRC without the detection of an UL grant in a DCI.

Proposal 5: For the configured grant Type 1 PUSCH transmission, the OCC-related parameters should be configured by RRC.

For the configured grant Type 2 PUSCH transmission and the dynamic granted based PUSCH transmission, OCC-related parameters can be configured by RRC or DCI. To minimize signaling overhead, the indication of the OCC sequence index can be associated with the antenna port in the DCI, while the OCC length can be configured by RRC.

Proposal 6: For the configured grant Type 2 PUSCH transmission and the dynamic granted based PUSCH transmission, the OCC sequence index can be associated with the antenna port indicated by DCI, while the OCC length can be configured by RRC.

In OCC-based user multiplexing, distinct OCC sequence index must be assigned to ensure orthogonality. When the OCC index is bound to a specific antenna port (as indicated by DCI), and the DMRS port is exclusively mapped to a single OCC index, different users will not be able to use the same DMRS port. This leads to rigid resource allocation and reduced scheduling adaptability. To enhance flexibility, an additional DMRS port to OCC index mapping relationship can be established, with the RRC configuring the specific mapping for each UE.

Proposal 7: If the OCC sequence index is associated with antenna port indicated by DCI, additional indication can be added in the RRC to indicate different associations for different users in order to increase the flexibility of the OCC sequence index indication.

## 2.3 OCC with TBoMS

For inter-slot OCC with TBoMS, the following figure shows three options for the mapping relationship between OCC codewords and TBoMS repetition.

Figure 2. Examples of mapping of inter-slot OCC with TBoMS

For Option 1, the TBoMS mapping follows the current specification, but the number of slots within an OCC group increases from 2 to 4. This change is necessary because the content covered by each of the OCC code element in an OCC group needs to be the same. As the number of TBoMS slots increases, the extension of slots within an OCC group will also grow, which will negatively impact the OCC orthogonality due to time and frequency offsets.

For Option 2, each part of TBoMS is repeated a repetition number of times in sequence. This approach ensures that the number of slots within an OCC group is equal to the OCC length. However, it requires 6 or 8 slots for the receiver to obtain a complete TB. Additionally, the time delay in receiving a complete TB will increase as the number of TBoMS slots and the repetition number grow, because the reception of the complete TB is concentrated in the latter part of the PUSCH transmissions.

For Option 3, each part of TBoMS is repeated OCC length times in sequence. This option also ensures that the number of slots within an OCC group is equal to the OCC length. And the complete TB can be received every 4 slots. This option offers a relatively low time delay in receiving the complete TB while ensuring OCC orthogonality.

Proposal 8: Support inter-slot OCC with TBoMS for PUSCH and each part of TBoMS is repeated OCC length times in sequence, and one OCC group includes the repetitions of one part of TBoMS.

Conclusions

- In this contribution, we provide our views on NR-NTN Uplink Capacity/Throughput Enhancement with the following and proposals.

Proposal 1: Support the combination of Option 1 and Option 3-a, i.e. the UCI is multiplexed on PUSCH with inter-slot OCC or dropped.

Proposal 2: If the scheduling DCI is received later than a time offset from the first slot of the OCC period, the UCI could be postponed to the next OCC group or dropped.

Proposal 3: The dropping rules for UCI can be related to the type of UCI.

Proposal 4: Introduce a scaling factor to scale down βoffsetPUSCH, thereby reducing the number of resources occupied by UCI when UCI is multiplexed on all slots of the OCC group.

Proposal 5: For the configured grant Type 1 PUSCH transmission, the OCC-related parameters should be configured by RRC.

Proposal 6: For the configured grant Type 2 PUSCH transmission and the dynamic granted based PUSCH transmission, the OCC sequence index can be associated with the antenna port indicated by DCI, while the OCC length can be configured by RRC.

Proposal 7: If the OCC sequence index is associated with antenna port indicated by DCI, additional indication can be added in the RRC to indicate different associations for different users in order to increase the flexibility of the OCC sequence index indication.

Proposal 8: Support inter-slot OCC with TBoMS for PUSCH and each part of TBoMS is repeated OCC length times in sequence, and one OCC group includes the repetitions of one part of TBoMS.

- References

- Chair notes RAN1#120

- Chair notes RAN1#119

- TS 38.214 Physical layer procedures for data v18.3.0

- R1-2410407 Discussion on NR-NTN uplink capacity/throughput enhancement, NTT DOCOMO, INC.