3GPP TSG RAN WG1 #120bis			R1-2502889

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	9.2.4

Source:	Google

Title:	Discussion on enhancement for asymmetric DL sTRP and UL mTRP scenarios

Document for: 	Discussion

- Introduction

In this contribution, we discuss some aspects related to asymmetric DL sTRP and UL mTRP scenarios. In RAN#102 meeting, RAN Plenary has agreed to a working item for MIMO phase 5 as shown in revised Rel-19 MIMO WID [1]. In previous RAN1 meeting(s), RAN1 has agreed some agreements related to asymmetric DL sTRP and UL mTRP scenarios as shown below [2][3][4]. In the following sections, we provide some thinking and observations from our side on enhancements for a scenario comprising a TRP serving DL and/or UL transmission, and a TRP serving only UL transmission.

RAN1 #118b Agreement For indicating PL offset for PDCCH-order PRACH, introduce a new 1-bit DCI field in DCI format 1_0: - This DCI field exists when the corresponding RRC parameter (which is a new RRC used to configure the presence of this 1-bit DCI field) is enabled and at least one TCI state is configured with PL offset. - When one joint/UL TCI state is indicated in Rel-17 unified TCI, - the bit field index 0 of this field indicates that PL offset is not included in the PRACH transmission power calculation - the bit field index 1 of this field indicates that the PL offset associated with the indicated TCI state is included in the PRACH transmission power. - FFS: Whether the bit field can be used to indicate other information - FFS: When two joint/UL TCI states are indicated in Rel-18 unified TCI Agreement For indicating PL offset for PDCCH-order PRACH, when two joint/UL TCI states are indicated in Rel-18 unified TCI, down-select one from the following Alts for the 1-bit DCI field in DCI 1_0 which indicates the application of PL offset on PRACH transmission: - Alt2: - the bit field index 0 of this field indicates that the PL offset associated in the first indicated joint/UL TCI state is included in the PRACH transmission power calculation - the bit field index 1 of this field indicates that the PL offset associated in the second indicated joint/UL TCI state is included in the PRACH transmission power calculation. - FFS: Whether a restriction that only one of the indicated joint/UL TCI states can be configured with PL offset is needed. FFS: If other information can be indicated by this same 1-bit DCI field for the PDCCH-order PRACH transmission RAN1 #119 Agreement Support to apply PL offset on PDCCH-order PRACH in FR2 - The design for applying PL offset on PDCCH-order PRACH in FR1 is fully reused here. Note: there is no extra enhancement for Tx beam determination for PDCCH-order PRACH in FR2 in Rel-19. RAN1 #120 Agreement For a UE provided with SSB-MTC-AddtionalPCI and not configured with multi-DCI based mTRP, support to reuse the DCI field ‘PRACH association indicator’ in DCI format 1_0 to indicate PL RS for PDCCH-order PRACH: - The bit field index 0 of this field indicates the DL RS that DMRS of PDCCH order DCI is QCLed with is used as PL RS for PRACH; - The bit field index 1 of this field is mapped to the additional PCI associated with the active TCI states and indicates the indicated SSB in this DCI is used as PL RS for PRACH: - In this case, the PRACH configuration associated with addition PCI is used. - This DCI field is present when the corresponding RRC parameter is configured and multi-DCI based mTRP is not configured. Conclusion There is no RAN1 consensus to support the following proposal: Support the UE to report either Type1 PHR or Type 3 PHR in a serving cell configured with one UL carrier and two separate SRS CLPC adjustment states. The UE determines to report Type1 PHR or Type 3 PHR according to: - UE provides the Type 1 PHR if both the Type 1 PHR and the Type 3 PHR are based on respective actual transmissions or on respective reference transmissions. - UE provides the PHR that is based on a respective actual transmission if either the Type 1 PHR or the Type 3 PHR is based on a respective reference transmission. This is subject to UE capability. This is feature is enabled by one new RRC parameter. Conclusion There is no consensus to introduce the restriction that only one of the indicated joint/UL TCI states can be configured with PL offset. Conclusion There is no consensus to include PL offset in Type 3 PHR calculation. Conclusion In asymmetric DL sTRP/UL mTRP deployment scenario, the PDCCH-order PRACH to UL TRP is CFRA.

- Discussion

Over previous RAN1 meetings, it has been a tough nut to crack that whether to support 2TA for asymmetric DL sTRP/UL mTRP scenarios in Rel-19. Several companies have claimed that introduction of 2TA is critical for performance of asymmetric DL sTRP/UL mTRP. Then, in RAN#105, it has been agreed to specify 2TA for M-TRP S-DCI or asymmetric DL sTRP/UL mTRP as being one of objective in the WID of Rel-19 MIMO as quoted below.

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

Currently, the asymmetric DL sTRP/UL mTRP scenarios are based on M-TRP S-DCI scenarios. For M-TRP S-DCI, some M-TRP PUSCH transmission schemes requiring two indicated joint/UL TCI states may be enabled/configured by the network. Also, a TAG or TAG pointer is actually associated with an indicated joint/UL TCI state. Then, it is unclear that how to handle if one of indicated joint/UL TCI is associated with a TAG with expired TA timer. The issue would become more severe when SDM and SFN UL transmissions are enabled/configured by the network.

Observation 1: If 2TA for S-DCI is introduced, it is unclear how to transmit an UL transmission with two indicated TCI states applied, and one of indicated TCI states is associated with a TAG with expired TA timer.

For such issue, we suggest UE should just skip an UL transmission, if one of the indicated joint/UL TCI state applied for the UL transmission is associated with an expired TA value. Otherwise, there is no point to perform SDM or SFN UL transmission with only one joint/UL TCI state valid.

Proposal 1: For the asymmetric DL sTRP/UL mTRP deployment scenarios, if one of the indicated joint/UL TCI state applied for an UL transmission (e.g., UL channel with SDM/SFN scheme) is associated with an expired TA value, UE skips the UL transmission.

RAN1 #119 Agreement For an SRS resource set not configured with followUnifiedTCI-StateSRS, regarding how to determine the PL offset and the CLPC adjustment state, down-select from the following Alts: - Alt1: The PL offset and CLPC adjustment state indicated by the TCI state configured to the SRS resource with lowest ID (i.e., reusing the rule specified in Rel-17) and no extra enhancement. - Alt2: The PL offset and CLPC adjustment state indicated by the TCI state configured to the SRS resource with lowest ID (i.e., reusing the rule specified in Rel-17), the SRS with lowest ID is always configured with one joint/UL TCI state, the “referenceSignal” in a UL TCI state is optional. - Alt3: - When the SRS resource with lowest ID is configured with a TCI state, The PL offset and CLPC adjustment state indicated by that TCI state are applied to those SRS resources (i.e., reusing the rule specified in Rel-17) - Otherwise, one of the two separate SRS CLPC adjustment states and one PL offset are configured in the SRS resource set, and they are applied on the SRS resources. - Alt4: - When the SRS resource with lowest ID is configured with a TCI state, The PL offset and CLPC adjustment state indicated by that TCI state are applied to those SRS resources (i.e., reusing the rule specified in Rel-17) - Otherwise, one PL offset and one CLPC adjustment state are applied on the SRS resources. - FFS how to define the PL offset and CLPC adjustment state

In previous RAN1 meeting(s), we discussed how to determine the PL offset and the CLPC adjustment state for an SRS resource set not configured with followUnifiedTCI-StateSRS. We still prefer Alt 3. Alt 1 and Alt 2 are infeasible. For Alt 1, it is possible that no TCI state is configured to the SRS resource with lowest ID, especially for SRS for beam measurement. For Alt 2, it would be more problematic to change a mandatory field to optional field. Alt 3 and Alt 4 are similar. However, we expect that for determining the one/default PL offset and CLPC, the decision would be unavoidably fall on to one of the two separate SRS CLPC adjustment states and one PL offset, which is equal to Alt 3.

Revised Proposal 2.1: For an SRS resource set not configured with followUnifiedTCI-StateSRS, regarding how to determine the PL offset and the CLPC adjustment state: - [If the SRS resource with lowest ID is configured with a TCI state,] The PL offset and CLPC adjustment state indicated by the TCI state configured to the SRS resource with lowest ID (i.e., reusing the rule specified in Rel-17) are applied on the SRS resources - Subject to UE capability, if the SRS resource with lowest ID is not configured with a TCI state: - One of the two separate SRS CLPC adjustment states and one PL offset are configured in the SRS resource set. - The P0, alphas, PL RS, CPLC adjustment state and PL offset configured in the SRS resource set are applied on the SRS resources. - [Note: If the UE does not report such capability, NW will always configure a TCI state to the SRS resource with the lowest ID.] - When the UE is configured with Rel-19 2 TA, the above design is also applied to TAG.

After a long debate, RAN1 had a compromised way forward as shown above. Due to short remaining time, RAN1 was not able to agree this revised proposal. We consider this is a decent proposal, which takes a good balance. From our perspective, it is no harm to take out the bracket on the first sub-bullet and the note.

Proposal 2: For an SRS resource set not configured with followUnifiedTCI-StateSRS, regarding how to determine the PL offset and the CLPC adjustment state:

- If the SRS resource with lowest ID is configured with a TCI state,] The PL offset and CLPC adjustment state indicated by the TCI state configured to the SRS resource with lowest ID (i.e., reusing the rule specified in Rel-17) are applied on the SRS resources

- Subject to UE capability, if the SRS resource with lowest ID is not configured with a TCI state:

  - One of the two separate SRS CLPC adjustment states and one PL offset are configured in the SRS resource set.

  - The P0, alphas, PL RS, CPLC adjustment state and PL offset configured in the SRS resource set are applied on the SRS resources.

  - Note: If the UE does not report such capability, NW will always configure a TCI state to the SRS resource with the lowest ID.

- When the UE is configured with Rel-19 2 TA, the above design is also applied to TAG.

RAN1 #120 Agreement Study whether/how to reset the CLPC of PUSCH/PUCCH/SRS based on the following alternatives (including if any enhancement is necessary and other alternatives are not precluded) - Alt1: For a UE configured with two TAGs, when the UE receives a RAR of a PDCCH-order PRACH, according to the new 1-bit DCI field PL offset indication in DCI format 1_0: - When there is one indicated joint/UL TCI state in Rel-17 Unified TCI: - If the 1-bit DCI field is 0, the UE resets the CLPC that is different from the CLPC associated with the indicated joint/UL TCI state. - If the 1-bit DCI field is 1, the UE resets the CLPC associated with the indicated joint/UL TCI state. - When there are two indicated joint/UL TCI states in Rel-18 Unified TCI: - If the 1-bit DCI field is 0, the UE resets the CLPC associated with the first indicated joint/UL TCI state; - If the 1-bit DCI field is 1, the UE resets the CLPC associated with the second indicated joint/UL TCI state - Alt 2: For a UE configured with two TAGs, one TAG ID is indicated in the RAR - The UE resets the closed loop power control adjustment state associated with a joint/UL TCI state associated with the indicated TAG.

Another one issue is related to the resetting of the CLPC of PUSCH/PUCCH/SRS, in response to receiving RAR of a PDCCH-order PRACH. Two alternatives are available from previous meeting(s). To us, Alt 1 would be more comprehensive with considering Rel-17 and Rel-18 unified TCI framework. For Alt 2, the relation of TAGs may not be always the same as that of the CLPC. With that said, we propose the following proposal.

Proposal 3: For resetting the CLPC of PUSCH/PUCCH/SRS, support the following alternative:

- Alt1: For a UE configured with two TAGs, when the UE receives a RAR of a PDCCH-order PRACH, according to the new 1-bit DCI field PL offset indication in DCI format 1_0:

  - When there is one indicated joint/UL TCI state in Rel-17 Unified TCI:

    - If the 1-bit DCI field is 0, the UE resets the CLPC that is different from the CLPC associated with the indicated joint/UL TCI state.

    - If the 1-bit DCI field is 1, the UE resets the CLPC associated with the indicated joint/UL TCI state.

  - When there are two indicated joint/UL TCI states in Rel-18 Unified TCI:

    - If the 1-bit DCI field is 0, the UE resets the CLPC associated with the first indicated joint/UL TCI state;

    - If the 1-bit DCI field is 1, the UE resets the CLPC associated with the second indicated joint/UL TCI state.

- Conclusion

According to the above discussion(s), we have the following observation(s) and/or proposal(s).

Observation 1: If 2TA for S-DCI is introduced, it is unclear how to transmit an UL transmission with two indicated TCI states applied, and one of indicated TCI states is associated with a TAG with expired TA timer.

Proposal 1: For the asymmetric DL sTRP/UL mTRP deployment scenarios, if one of the indicated joint/UL TCI state applied for an UL transmission (e.g., UL channel with SDM/SFN scheme) is associated with an expired TA value, UE skips the UL transmission.

Proposal 2: For an SRS resource set not configured with followUnifiedTCI-StateSRS, regarding how to determine the PL offset and the CLPC adjustment state:

- If the SRS resource with lowest ID is configured with a TCI state,] The PL offset and CLPC adjustment state indicated by the TCI state configured to the SRS resource with lowest ID (i.e., reusing the rule specified in Rel-17) are applied on the SRS resources

- Subject to UE capability, if the SRS resource with lowest ID is not configured with a TCI state:

  - One of the two separate SRS CLPC adjustment states and one PL offset are configured in the SRS resource set.

  - The P0, alphas, PL RS, CPLC adjustment state and PL offset configured in the SRS resource set are applied on the SRS resources.

  - Note: If the UE does not report such capability, NW will always configure a TCI state to the SRS resource with the lowest ID.

- When the UE is configured with Rel-19 2 TA, the above design is also applied to TAG.

Proposal 3: For resetting the CLPC of PUSCH/PUCCH/SRS, support the following alternative:

- Alt1: For a UE configured with two TAGs, when the UE receives a RAR of a PDCCH-order PRACH, according to the new 1-bit DCI field PL offset indication in DCI format 1_0:

  - When there is one indicated joint/UL TCI state in Rel-17 Unified TCI:

    - If the 1-bit DCI field is 0, the UE resets the CLPC that is different from the CLPC associated with the indicated joint/UL TCI state.

    - If the 1-bit DCI field is 1, the UE resets the CLPC associated with the indicated joint/UL TCI state.

  - When there are two indicated joint/UL TCI states in Rel-18 Unified TCI:

    - If the 1-bit DCI field is 0, the UE resets the CLPC associated with the first indicated joint/UL TCI state;

    - If the 1-bit DCI field is 1, the UE resets the CLPC associated with the second indicated joint/UL TCI state.

# Reference

- RP-242394 WID revision: NR MIMO Phase 5, Samsung

- Chair notes of 3GPP TSG RAN WG1 #118-bis

- Chair notes of 3GPP TSG RAN WG1 #119

- Chair notes of 3GPP TSG RAN WG1 #120