3GPP TSG RAN WG1 #120bis		 R1-2502902

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	9.11.1

Source:	Google

Title:	Discussion on Downlink Coverage Enhancement for NR NTN

Document for:	Discussion/Decision

# Introduction

In this contribution, we provide our view on NR NTN downlink coverage enhancement.

# Discussion

## PDCCH enhancement

Agreement Support only inter-slot repetition for Type0 PDCCH CSS. Agreement For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1, consider the following: - Option 1: Support repeated PDCCH candidates in the two consecutive slots n0 and n0+1 associated with the same SSB index ( n0 as defined in section 13 of TS 38.213) - Repeated PDCCH candidates share the same aggregation level (AL), coded bits and same candidate index - FFS: Details including how the two PDCCH candidates are counted toward the BD limits - Note: with option 1, if the network repeats the Type 0 PDCCH across two consecutive slots, a legacy UE might decode the PDCCH and associated PDSCH in one slot and skip PDCCH monitoring in the other slot. - FFS: whether/how option 1 can be applicable for M=1 and M= ½ - Option 2: Support repeated PDCCH candidates in the two slots n0 and n0+X [or n0+1 and n0+1+X ] associated with the same SSB index ( n0 as defined in section 13 of TS 38.213) - Value of X>1, predefined or configured - FFS: Value of X - Repeated PDCCH candidates share the same aggregation level (AL), coded bits and same candidate index - FFS: Backward compatibility to legacy UE - Option 3: - The PDCCH candidates in slots n0 associated respectively with different SSB indexes are repetitions of each other and share the same aggregation level, coded bits and same candidate index - For M=1/2 and M=1, the repeated PDCCH candidates in two consecutive slots associated with different SSB indexes; - For M=2, the PDCCH candidates in slots n0+1 associated respectively with different SSB indexes are repetitions of each other and share the same aggregation level, coded bits and same candidate index - Option 4: Option 2 with cross SSB beam repetition support - The PDCCH candidates in slots n0 associated respectively with different SSB indexes are repetitions of each other and share the same aggregation level, coded bits and same candidate index

In the last meeting, RAN 1 has agreed to support only the inter-slot repetition for Type0-CSS coverage enhancement, where 4 implementation options were listed for further discussion. From our perspective, we still see the benefits from intra-slot repetition for Type0-CSS, which provides minimum satellite dwell time to cells with common message only.

Observation 1: Although RAN1 has agreed to support only the inter-slot repetition for Type0-CSS, we still see the benefits of intra-slot repetition in terms of a shorter satellite dwell time for cells with common message only.

Regarding the options in the agreement, there are no critical issues from our perspective. Option 1 introduces less spec impact. New UE expects the base station to transmit PDCCH repetitions in each Type0-CSS monitoring candidate slots (i.e., in n0 and n0+1) according to the current spec. Figure 1 provides an example in FR1 with 1 Type0-CSS per slot (e.g., M=1) according to the Table 13-11 in 38.214, where each SSB associates with 2 Type0-CSS candidates, one in slot n0 and another one in slot n0+1. For example, SSB 0 in the slot 0 associates with Type0-CSS candidates in slot 2 and slot 3, SSB1 in slot 0 associates with Type0-CSS candidates in slot 3 and slot 4, etc. In the current specification, the UE expects the base station to transmit PDCCH in one of the candidates (e.g., in either slot 2 or slot 3).

Figure 1. Example of SSB and Type0-CSS monitoring association with M=1.

To be noted, for adjacent SSBs, the candidate slots overlapped (e.g., slot 3 for Type0-CSS of SSB0 and SSB1) based on to the formula and tables from 38.214. It would cause problems if the base station is to transmit all SSBs in the SSB burst with Type0-CSS repetitions in all candidate slots. In our views, this can be solved by transmit a single SSB or proper SSB combinations in the SSB burst to avoid the problem, which can be handled by base station implementation. Figure 2 shows some available SSB combinations with associated Type0-CSS repetition. In Figure 2(a), the base station transmits SSB0 and SSB 2 in the slot 0 and slot 1, respectively. Thus, the Type0-CSS repetitions associated with SSB0 are transmitted in slot 2 and 3. Likewise, the Type0-CSS repetitions associated with SSB2 are transmitted in slot 4 and 5. In this case, the overlapping can be avoid. Figure 2(b) and 2(c) illustrates the SSB combination of SSB0 and SSB3, and SSB1 and SSB3, respectively. Of course, the base station can avoid the overlapping by sending only one SSB in the SSB burst.

Figure 2. Example of SSB selections to avoid monitoring candidate overlapping with M=1.

The same UE behavior can apply to the case where 2 Type0-CSS per slot is configured (i.e., M=1/2). As shown in the Figure 3, the Type0-CSS associated with SSB1 starts right after the Type-0 CSS associated with SSB 0. However, the second Type0-CSS candidate of SSB0 still collides with the first Type0-CSS candidate of SSB 2, and the second Type0-CSS candidate of SSB1 collides with the first Type0-CSS candidate of SSB 3.

Figure 3. Example of SSB and Type0-CSS monitoring association with M=1/2.

To avoid the collision when the base station is to transmit Type0-CSS repetition, same solutions as Figure 2 can be applied. As shown in the Figure 4(a), the base station transmits only SSB0 and SSB1 in the SSB burst, thus there are no Type0-CSS repetition collision occurs. Likewise, the base station can avoid the Type0-CSS repetition collision by transmitting only one SSB in the SSB burst, or SSB combinations such as {SSB0, SSB3} or {SSB1, SSB2}

Figure 4. Example of SSB selections to avoid monitoring candidate overlapping with M=1/2.

One concern addressed in the Option-1 is to support legacy UE. However, from our perspective, this feature is for compensating coverage gap discovered from the LLS simulation scenario set 1-3. A legacy UE without the knowledge of Type0-CSS repetition has relative low probability to camp on the satellite due to insufficient link budget. In this regard, it is not mandatory to support legacy UE in this scenario. However, for examples showed in Figure 2 and 4, there are no issues for legacy UE to receive SSB and monitor associated Type0-CSS.

Observation 2: Considering set 1-3 scenario, a legacy UE without the knowledge of Type0-CSS repetition has relative low probability to camp on the satellite due to insufficient link budget. In this regard, it is not mandatory to support legacy UE in this scenario.

For other options, Option-2 provides an additional 1 or 2 Type0-CSS monitoring occasions (n0+X and/or n0+1+X) to a SSB. From our perspective, the option creates too many Type0-CSS monitoring occasions to a cell, which increases the dwell time to a single cell with only common messages, thereby not beneficial for other active cells in the same footprints.

As for Option-3 and Option-4, where cross SSB repetition is assumed. Figure 5(a) show an example in FR1 with M=1, where the base station transmits SSB0 and SSB1 in beam 0, and transmits SSB2 and SSB3 in beam 1. The base station only transmit the associated Type0-CSS in monitoring occasion n0, thus the 1st and 2nd repetitions in slot 2 and 3 are associated with SSB0 and SSB1, respectively. The result is similar to the Figure 2(a) (i.e., Option-1), except that additional SSBs are transmitted. However, because the SSB0 and SSB1 are transmitted in the same slot with the same beam, the performance gain provided by transmitting the additional SSB is marginal.

Figure 5(b) shows an example in FR1 with M=1/2. The base station transmits SSB0 and SSB1 in beam 0, and the Type0-CSS in slot 2 and 3 are repetitions which associated with SSB 0 and SSB1, respectively. Likewise, SSB2 and SSB 3 associate with Type0-CSS in slot 4 and 5, respectively. The outcome is similar to Figure 4(a), expect that 1 more SSB is transmitted.

Figure 5. Example of cross SSB repetition with M=1 and M=1/2.

Proposal 1: For Type0-CSS, support repeated PDCCH candidates in the two consecutive slots  n0 and n0+1 associated with the same SSB index. (Option-1)

- Base station applies parameter M=1 and M= ½ by selecting a SSB or SSB combination to be transmitted. (No spec impact)

# Conclusion

Observation 1: Although RAN1 has agreed to support only the inter-slot repetition for Type0-CSS, we still see the benefits of intra-slot repetition in terms of a shorter satellite dwell time for cells with common message only.

Observation 2: Considering set 1-3 scenario, a legacy UE without the knowledge of Type0-CSS repetition has relative low probability to camp on the satellite due to insufficient link budget. In this regard, it is not mandatory to support legacy UE in this scenario.

Proposal 1: For Type0-CSS, support repeated PDCCH candidates in the two consecutive slots  n0 and n0+1 associated with the same SSB index. (Option-1)

- Base station applies parameter M=1 and M= ½ by selecting a SSB or SSB combination to be transmitted. (No spec impact)

# Reference

- “Draft Report of 3GPP TSG RAN WG1 #120 v0.2.0,” MCC Support, Athens, Greece, February 17th – 21st, 2025.

- R1-2500042, “FL Summary #4: NR-NTN downlink coverage enhancements,” Moderator (Thales), Athens, Greece, February 17th – 21st, 2025.