3GPP TSG-RAN WG1 Meeting #120bis	R1-2503088

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	9.6.3

Title:	FL summary #3 on LP-WUS operation in CONNECTED mode

Source:	Moderator (NTT DOCOMO)

Document for:	Discussion, Decision

1	Introduction

The core part of the Rel-19 LP-WUS/WUR WI [1] has the following objective:

The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode. - Specify the necessary RAN4 core requirement(s) to support the feature (RAN4). - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance - Define guard RBs for ACS and ASCS cases - Study testability of above requirements - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS. - Current NR BS requirements is baseline - Specify necessary RRM requirements

This document summarizes contributions [4] – [26] submitted to agenda item 9.6.3 (LP-WUS operation in CONNECTED mode).

For this meeting, considering the limited time before Stage 3 (Sep. 2025) and ASN.1 freeze (Dec. 2025), any issues which may affect higher layer signaling/parameter and UE feature are prioritized, which are tagged [MAC], [RRC], and [UEF], respectively.

2	Proposals for Online Sessions

2.1	Proposals for Monday Online

[UEF]Proposal 3.2-1:

- For the UE capability report on the minimum time gap for each SCS, support X=3 candidate values corresponding to {V1, 10, V3}ms

  - Note: V1 is smaller than 10ms, V3 is larger than 10ms, FFS exact values

[RRC][UEF]Proposal 5-1:

- For the case when UE is configured with CA without dual DRX groups in RRC CONNECTED mode,

  - LP-WUS can be configured only on PCell

- For the case when UE is configured with NR-DC with CA without dual DRX groups or without CA in RRC CONNECTED mode,

  - LP-WUS can be configured only on PCell/PSCell per cell-group

  - Introduce serarate UE capability for monitoring LP-WUS on one cell group or both cell groups

[RRC]Proposal 5-2:

- For the case when UE is configured with CA with dual DRX groups in RRC CONNECTED mode, at least for LP-WUS procedure Option 1-1, down select one of the following in RAN1#120bis

  - Opt.1

    - LP-WUS can be configured on a serving cell

    - LP-WUS carries separate indications for each DRX group(s) to trigger PDCCH monitoring of all serving cells

      - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation

  - Opt.2

    - LP-WUS can be configured on a serving cell per DRX group

    - LP-WUS indication is applicable to all serving cells in the DRX group

      - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation

  - Opt.3

    - LP-WUS can be configured on a serving cell

    - LP-WUS indication is applicable to all serving cells of all DRX groups.

      - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation

  - Opt.4

    - Up to RAN2 to decide whether and how to support this case

  - Opt.5

    - LP-WUS cannot be configured on any serving cells for this case

  - FFS for LP-WUS procedure Option 1-2

- FFS: Whether the serving cell on which the LP-WUS is configured is only PCell or can be RRC configured cell other than PCell

[RRC]Proposal 3.3-1:

- Update previous agreement in red:

Agreement

- For LP-WUS MOs in connected mode for Option 1-1, support Approach 1:

  - LP-WUS MOs, including periodicity and time offset1, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s).

  - Time offset2 indicates a time prior to a slot where the drx-onDurationTimer would start

    - UE monitors the first Z LP-WUS MO(s) at or after Time offset2.

      - FFS: How to determine the value of Z is RRC configured

    - Time offset2 is RRC configured

  - UE is not required to monitor LP-WUS during the X [slots/symbols] prior to the beginning of a slot where the UE would start the drx-onDurationTimer. FFS: How X is determined given by the minimum time gap reported by UE capability.

2.2	Proposals for Tuesday Online

Proposed conclusion 4-2:

- From RAN1 perspective, when LP-WUS monitoring Option 1-2 is enabled for RRC CONNECTED mode, it is supported that SR, PRACH and CG-PUSCH triggers MR PDCCH monitoring according to the legacy UE behaviour

[RRC][UEF]Proposal 5-2b:

- For the case when UE is configured with CA with dual DRX groups in RRC CONNECTED mode, at least for LP-WUS procedure Option 1-1, down select one of the following in RAN1#120bis

  - Alt1

    - Basic capability

      - LP-WUS can be configured only on PCell

      - LP-WUS indication is applicable to all serving cells of all DRX groups.

        - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation

    - Advanced capability

      - LP-WUS can be configured on PCell for primary DRX group and an activated SCell for secondary DRX group

      - LP-WUS indication is applicable to all serving cells in the DRX group where LP-WUS is monitored

        - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation

      - This capability has prerequisite of the above basic capability

  - Alt2

    - LP-WUS can be configured on PCell for primary DRX group and/or an activated SCell for secondary DRX group

    - LP-WUS indication is applicable to all serving cells in the DRX group where LP-WUS is monitored

      - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation

    - This capability has candidate values for the LP-WUS configuration with {primary DRX group, secondary DRX, both}

  - Alt3: There is no consensus in RAN1 to support this case in Rel-19

[RRC]Proposal 3.3-1:

- Update previous agreement in red:

Agreement

- For LP-WUS MOs in connected mode for Option 1-1, support Approach 1:

  - LP-WUS MOs, including periodicity and time offset1, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s).

  - Time offset2 indicates a time prior to a slot where the drx-onDurationTimer would start

    - UE monitors the first Z LP-WUS MO(s) at or after Time offset2.

      - FFS: How to determine the value of Z is RRC configured

    - Time offset2 is RRC configured

  - UE is not required to monitor LP-WUS during the X [slots/symbols] prior to the beginning of a slot where the UE would start the drx-onDurationTimer. FFS: How X is determined given by the minimum time gap reported by UE capability.

[RRC]Proposal 3.3-2:

- Update previous agreement in red:

Agreement

- For LP-WUS MOs in connected mode for Option 1-2, support Approach 1:

  - LP-WUS MOs, including periodicity and time offset3, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s).

    - FFS one or multiple MOs per periodicity

  - UE monitors LP-WUS in the first Z LP-WUS MOs for the same slot that PDCCH monitoring would start and, if triggered to wake up, starts a new timer for PDCCH monitoring triggered by LP-WUS, after a time offset4.

    - The time offset4 configured by the network indicating a time from the first LP-WUS MO, after which the UE starts PDCCH monitoring via starting the new timer.

    - FFS: Definition of time offset4

    - The time gap from the last LP-WUS MO of the Z LP-WUS MOs and the associated slot where the new timer would start by the Z LP-WUS MOs is not smaller than the minimum time gap given by UE capability

[RRC][MAC]Proposal 3.4-2:

- For LP-WUS monitoring in RRC CONNECTED mode, support TCI framework for CORESET as baseline for LP-WUS:

Rel-15 TCI framework: RRC configured K TCI states and MAC-CE activates one of them

Rel-17 unified TCI framework: LP-WUS follows the activated/indicated TCI-state by MAC-CE/DCI, same as other DL channels/signals

2.3	Proposals for Thursday Morning Online

[RRC]Proposal 3.3-2b:

- Update previous agreement in red:

Agreement

- For LP-WUS MOs in connected mode for Option 1-2, support Approach 1:

  - LP-WUS MOs, including periodicity and time offset3, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s).

    - FFS one or multiple MOs per periodicity

    - UE is configurd with the number of MOs per periodicity by RRC

  - UE monitors LP-WUS in the LP-WUS MOs and, if triggered to wake up, starts a new timer for PDCCH monitoring triggered by LP-WUS, after a time offset4.

    - FFS: Definition of time offset4

    - Alt1:

      - The time offset4 configured by the network indicating a time from the first LP-WUS MO per periodicy, after which the UE starts PDCCH monitoring via starting the new timer.

      - The time gap from the last LP-WUS MO per periodicity and the start of new timer is not smaller than the minimum time gap given based on UE capability

    - Alt2:

      - The time offset4 configured by the network indicating a time from the LP-WUS MO indicating UE to wake-up, after which the UE starts PDCCH monitoring via starting the new timer.

      - The time offset4 is not smaller than the minimum time gap given based on UE capability

[UEF]Proposal 3.2-1b:

- For the UE capability report on the minimum time gap between the last symbol of LP-WUS and the symbol to start corresponding timer regardless of SCS, support X=3 candidate values corresponding to {V1=[3,5,6], V2=[10,13,20], V3=[33,42]}ms where V1<V2<V3.

  - FFS: Whether to support different set of values for different receiver types

Proposed conclusion 3.1-2a:

- There is no consensus in RAN1 whether to introduce any enhancements of PDCCH skipping indication for the interaction with LP-WUS in RRC connected mode in Rel-19.

[RRC][MAC]Proposal 3.4-2:

- For LP-WUS monitoring in RRC CONNECTED mode, support TCI framework for CORESET as baseline for LP-WUS:

Rel-15 TCI framework: RRC configured K TCI states and MAC-CE activates one of them

Rel-17 unified TCI framework: LP-WUS follows the activated/indicated TCI-state by MAC-CE/DCI, same as other DL channels/signals

Proposal 6-1a:

- For RRC CONNECTED mode when LP-WUS is configured with Cell DTX, during Cell DTX inactive time,

For Option 1-1, the UE is expected to monitor LP-WUS

For Option 1-2, the UE is not expected to monitor LP-WUS both for Option 1-1 and 1-2

3	LP-WUS Procedures to trigger PDCCH monitoring

3.1	High-level procedures

This issue has been discussed since RAN1#116 and following agreements were made so far.

Agreement Update the following agreement in RAN1#116 in red: Agreement - For RRC CONNECTED mode, from RAN1 perspective, further study following LP-WUS procedures to trigger PDCCH monitoring: - Case 1: PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - This option may replace DCP functionality - Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - PDCCH monitoring possibly irrespective of drx-onDurationTimer - Option 1-2-1: PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS - If this is adopted, it should be configured together with Option 1-1 to achieve power saving gain compared to legacy C-DRX - Option 1-2-2: PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS - Option 1-3: LP-WUS monitoring inside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - Case 2: PDCCH monitoring is triggered by LP-WUS without C-DRX configuration. LP-WUS can be monitored at any time according to the LP-WUS monitoring configuration - FFS duty-cycled and/or continuous LP-WUS monitoring - Combination of options in Case 1 and combination of options in Case 1 and Case 2 are not precluded should be considered. - RAN1 does not discuss C-DRX related timers other than drx-onDurationTimer, this topic is up to RAN2 - Note: Above does not preclude to support fallback mechanism to trigger PDCCH monitoring, if any Agreement - For Option 1-1 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective. LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer UE is configured with legacy C-DRX configurations as Rel-18 UE behaviors related to CDRX active time triggered by all legacy DRX timers are not affected unless included in this proposal No impact on RRM/RLM/BFD measurement requirements is assumed For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP) - Periodic CSI/L1-RSRP is not reported during the time given by the configured drx-onDurationTimer if UE is not indicated to wake-up - Periodic CSI/L1-RSRP is periodically reported during the time given by the configured drx-onDurationTimer regardless if UE is indicated to wake-up or not - Note: Periodic CSI/L1-RSRP is reported during CDRX active time as in legacy specification Agreement - For option 1-2 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective. LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. UE is configured with legacy C-DRX configurations as Rel-18 UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration) - FFS potential restriction for LP-WUS configuration in relation with C-DRX configuration LP-WUS triggers the start of a timer during which UE monitors PDCCH - FFS the timer is existing timer and/or new timer UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected - drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL No impact on RRM/RLM/BFD measurement requirements is assumed For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1) - Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up - Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS Agreement Confirm following working assumption Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Note: Above can be revisited considering RAN2 decisions. Agreement - For LP-WUS CONNECTED mode operation, Option 1-2 can be configured separately from Option 1-1 - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Agreement For LP-WUS CONNECTED mode operation, the following case is not supported: - Both Option 1-1 and Option 1-2 are simultaneously configured for the same UE in a given cell Agreement - For Option 1-2 of LP-WUS CONNECTED mode operation, for explicit indication to stop the new timer for PDCCH monitoring in Option 1-2 and drx-InactivityTimer for the DRX group, RAN1 identified the following potential solution - Use existing DRX Command MAC CE and Long DRX Command MAC CE - When UE is not in C-DRX active time, UE monitors LP-WUS - Send LS to RAN2 to ask the feasibility of the above potential solution. LS in R1-2410909. Conclusion - For Option 1-1 of LP-WUS CONNECTED mode operation, RAN1 assumes existing DRX Command MAC CE and Long DRX Command MAC CE can be used to stop drx-onDurationTimer and drx-InactivityTimer for the DRX group - When UE is not in C-DRX active time, UE monitors LP-WUS - Send LS to RAN2 to ask whether RAN2 has same understanding or not. LS in R1-2410909. Agreement - For Option 1-2, for LP-WUS CONNECTED mode operation, the periodic CSI/L1-RSRP reporting operation is same as Rel-16 DCP, that is the UE can be configured with a parameter to enable/disable periodic CSI/L1-RSRP reporting, respectively: - If the parameter is NOT configured: - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is not reported. - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time. - If the parameter is configured: - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the time given by the configured drx-onDurationTimer in DRX-Config for the case when UE is outside C-DRX active time. - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time. - FFS: Whether RAN1 spec impact is required. Agreement - For RRC connected mode, UE does not monitor LP-WUS during C-DRX active time - FFS FAR and MDR impact

Note that RAN2 have made following agreements

- In RRC_CONNECTED mode, RAN2 to further discuss the impacts of LP-WUS operation methods identified in RAN1. - For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1. - For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline. - RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE. - The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message. For option 1-2, - After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.) - The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring. The range of time offset is left for RAN1. For Option 1-2, LP-WUS monitoring is performed at least outside legacy C-DRX Active Time. FFS if the legacy drx-onDurationTimer is started or not if the new timer is configured in Option 1-2. In option 1-2, a new timer triggered by LPWUS is introduced. When this new timer is running, UE is in C-DRX active time. When UE is not in C-DRX active time, UE goes back to LPWUS monitoring. When UE is in C-DRX active time, UE PDCCH monitoring behaviors related to other legacy DRX timers (except for drx-onDurationTimer) are not affected. - The drx-onDurationTimer is not started with Option 1-2 LP-WUS. - For Option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the configured drx-onDurationTimer, for the case when UE is outside C-DRX active time. - For option 1-2, if UE receives DRX command MAC CE or Long DRX command MAC CE, UE stops the new timer triggered by LP-WUS. - Don’t support Option 1-1 and Option 1-2 simultaneously configured for the same UE. For Option 1-1, UE monitors LP-WUS outside C-DRX active time at least when long DRX cycle is used. FFS whether short DRX cycle is used. RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-1 to stop drx-onDurationTimer and drx-InactivityTimer. RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-2 to stop the new timer and drx-InactivityTimer.

Regarding MR and LP-WUR interaction, followng agreement/conclusion were made in previous RAN1 meetings.

Agreement - For RRC connected mode, UE does not monitor LP-WUS during C-DRX active time - FFS FAR and MDR impact Conclusion From RAN1 perspective, when LP-WUS monitoring is enabled for RRC CONNECTED mode, it is supported that SR, PRACH and CG-PUSCH triggers MR PDCCH monitoring according to the legacy UE behaviour - Above applies at least for Option 1-1

Related to overview on LR-MR interaction, a number of companies provided their view in this meeting as below:

- QC

  - UE does not monitor LP-WUS when MR is active outside C-DRX Active Time, e.g.:

    - the UE transmits SR, PRACH or CG-PUSCH outside C-DRX Active Time, or;

    - the UE monitors PDCCH outside C-DRX Active time (e.g., for CSS sets), or;

    - the UE is in measurement gap, or etc

  - For both Option 1-1 and Option 1-2, if all the LP-WUS MO(s), associated with a slot where PDCCH monitoring would start, are not monitored due to the MR activity, the UE starts drx-ondurationTimer or the new timer at the slot for PDCCH monitoring

- MTK

  - The specification should clearly mandate that LP-WUS monitoring is skipped whenever the main receiver is active, ensuring the low-power receiver never disrupts primary operations while preserving maximum power-saving benefits.

- HW

  - For the purpose of future discussion, RAN1 assumes at least the following as baseline.

    - LP-WUR is NOT able to receive signal(s) during the time where MR transmits/receives signals/channels.

      - If LP-WUS monitoring collides with measurements and/or periodic CSI/L1-RSRP transmission, measurements are prioritized.

      - If LP-WUS monitoring collides with SPS/CG resources, SPS/CG reception/transmission is prioritized.

      - FFS how to let gNB and UE have the common understanding on where the UE performs measurement.

  - FFS: additional consideration of simultaneous operation between LP-WUR and MR.

    - FFS: UE capability.

- Pana

  - During the time when UE is not in C-DRX active time according to drx-HARQ-RTT-TimerDL or drx-HARQ-RTT-TimerUL, UE is not required to monitor LP-WUS.

- ZTE

  - Outside new or legacy active time, reusing legacy mechanism as DCP for Searchspace monitoring

As discussed in previous meetings, it may be difficult to make further progress on basic assumption of LR and MR operations since those are highly related to each individual MR Tx/Rx. Rather, here we can focus on whether/which specific MR behabior (such as CSI/RRM measurments, PDCCH monitoring in CSS, etc.) needs to be further discussed, and following question is made:

[Old]Question 3.1-1:

- Companies are encouraged to provide views on whether/which specific MR behabior (such as CSI/RRM measurments, PDCCH monitoring in CSS, etc.) needs to be further discussed in this meeting, in addition to what have been agreed so far.

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm |  | In principle, our view is that MR should be prioritized as follows: - UE does not monitor LP-WUS in C-DRX active time (already agreed) - UE does not monitor LP-WUS outside C-DRX active time when MR is active - UE monitors LP-WUS outside C-DRX active time when MR is not active There are various cases where MR is active outside C-DRX active time overlapping LP-WUS MO, such as (1) PDCCH monitoring for paging; (2) SR/SRS/CG-PUSCH/PRACH; (3) measurements. We think MR activities should be prioritized in these cases. Prioritizing MR activity over LP-WUS monitoring ourside C-DRX active time should be fine as long as gNB knows the overlap. There are two ways of handling the case: (a) gNB does not transmit LP-WUS on the unavailable MO hence UE does not trigger the timer for PDCCH monitoring due to the absence of the LP-WUS; (b) gNB does not transmit LP-WUS on the unavailable MO and UE triggers the timer for PDCCH monitoring as if LP-WUS is detected in the MO. In Rel-16 DCP, 38.321 specifies the above (b) for the above (2) where DCP cannot be monitored due to MR activities. We think we can do the same for LP-WUS. |
| CMCC |  | Support to discuss the scenarios in QC’s comment, i.e., UE behaviour when LP-WUS MO/LP-SS transmission occasion overlaps with PDCCH MO for CSS/UL signal transmission occasion/measurement. The other we think worth noting is the definiton of overlap. For instance, in current NR deployment: - A UE may not transmit in the uplink earlier than a time interval after the end of the last received downlink symbol; - A UE may consider a RO as invalid if the time interval between the RO and SSB transmission/DL symbol is smaller than a threshold; Such kind of scenarios may also occur between LP-WUS MO/LP-SS transmission occasion and existing NR transmission. We suggest to also take them into consideration. |
| ZTE, Sanechips |  | We are OK to discuss the exact scenarios when MR is active and overlapped with LP-WUS. But we feel this is not a urgent issue, which also can be fixed in later phase. |
| Samsung |  | In our view, the current focus on PDCCH monitoring should be the baseline one and shall be discussed with higher priority. Other behavior can be addressed later after the current focus is completed. |
| Nokia1 |  | Needs to be discussed to avoid/minimise UE-gNB misalignment/wasted MO attempts. Note, proposed conclusion 4.2 is already continuing at least one aspect of this discussion. |
| vivo |  | We think the UE behaviour when LP-WUS monitoring is collided with NR DL transmission such as CSI/RRM measurments, PDCCH monitoring in CSS can be handled by gNB, that is gNB can prioritize the mentioned MR transmission and delay the LP-WUS transmission accordingly. Besides, the type of SS set controlled by the LP-WUS monitoring can be further confirmed, e.g., LP-WUS monitoring will not impact on the PDCCH monitoring on type 0 1 2 CSS and only apply to the trigger the type 3 CSS set and USS set PDCCH monitoring. But these issues can be discussed in the later stage. |
| LGE |  | We agree that MR should be prioritized. We would like to think one more way other than QC suggested. gNB can transmit LP-WUS on the later available MO. Just for the clarification, if the UE transmits SR/CG-PUSCH/PRACH, DRX Active Time is started. So, we do not have to discuss this case. |
| Spreadtrum |  | We agree with ZTE’s views. |
| OPPO |  | If the MR is activated, the LP-WUS monitoring should be disabled. This principle was generally acceptable to us. However, there is no agreements on it, due to it is not clear how the spec. can be impact. We think it is still unclear if spec. can capture this. We can further discuss this. |
| Panasonic |  | We support QC’s comment. Regarding the case of “UE monitors LP-WUS outside C-DRX active time when MR is not active” in QC’s comments, during the time drx-HARQ-RTT-TimerDL/drx-HARQ-RTT-TimerUL is running, we think UE is not required to monitor LP-WUS as UE knows when the time will expire without additional trigger. |
| Xiaomi |  | As we have presented in our contribution. P/SP SRS should not be transmitted during non-active time, just as we defined for C-DRX. |
| Moderator |  | According to the input, a number of companies propose to make further progress in this meeting, while some others think it is not urgent. Since the proposal from QC may have RRC impact, this can be discussed as a starting point. Note that following was agreed in AI9.6.1 in Monday online and some of the overlap between LP-WUS MO and MR activity may be avoided. [H][FL1] Proposal 3.2-2: Support maximum length for a LP-WUS (actual OFDM symbols used) which is larger than one slot - Non-consecutive OFDM symbols at least across multiple slots can be used for a LP-WUS transmission |

[RRC]Proposal 3.1-1a:

- For both Option 1-1 and Option 1-2 in RRC connected mode, if all the LP-WUS MO(s), associated with a slot where PDCCH monitoring would start, are not monitored due to the MR activity, the UE starts drx-ondurationTimer or the new timer at the slot for PDCCH monitoring

  - FFS which MR activity is appricable

  - Introduce a new RRC parameter to enable the above UE behavior

| Company | Y/N | Comments |
| --- | --- | --- |
| Ericsson1 |  | From our perspective, no additional restrictions need to be specified. If needed, UE can follow approach (b) indicated in QC comment by implementation. |
| CMCC |  | For Option 1-1, seems this is to reuse the legacy UE behaviour on DCP. We are Okay for it. For Option 1-2, we do not think it is feasible. We also do not think the overlap between LP-WUS MO and MR activity can be totally avoided by properly configures the LP-WUS MO resource consider there are various kinds of MR activity in current NR system (e.g. PDCCH MO for CSS/UL signal transmission occasion/measurement). We share the same view with Ericsson and QC that we can define a unify rule to easily handle all of them. |

In the last RAN1 meeting, following note was writtern as remainder that this aspects can be further discussed after receiving RAN2 reply.

Moderator’s note: following can be further discussed after receiving RAN2 reply to R1-2410909 - FFS: Enhanced PDCCH skipping indication can be used to - Alt1: indicate UE to terminate the DRX active time and start LP-WUS monitoring - Alt2: indicate UE to start LP-WUS monitoring without terminate the DRX active time

Then, RAN2 sent reply LS in R1-2501685 answering that

RAN2 confirmed as the following RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-1 to stop drx-onDurationTimer and drx-InactivityTimer. RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-2 to stop the new timer and drx-InactivityTimer. RAN2 will continue to discuss whether short DRX can be configured and used with Option 1-1 and Option 1-2.

Therefore, one company (DCM) assumed Enhanced PDCCH skipping indication is not necessary since similar feature as Alt1 can be covered by DRX command MAC CE. On the other hand, following was proposed by companies:

- ETRI

  - Option 1: Utilize the reserved ‘11’ value of the 2-bit PDCCH monitoring adaptation field in the DCI as an explicit indication for the UE to terminate the DRX active time and transition to the sleep state for LP-WUS monitoring.

  - Option 2: Support an approach where the existing PDCCH skipping mechanism is utilized to indicate the UE to start LP-WUS monitoring without terminating the DRX active time. In this case, the UE may perform LP-WUS monitoring during the skipping duration and return to PDCCH monitoring upon LP-WUS detection

- vivo

  - At least for LP-WUS connected mode operation Option 1-2, support enhanced PDCCH skipping indication that is used to indicate UE to terminate the DRX active time (e.g., stop PDCCH monitoring on one or multiple serving cells) and starts LP-WUS monitoring

[Old]Question 3.1-2:

- Companies are encouraged to provide views on whether any enhancements of PDCCH skipping indication are necessary for the interaction with LP-WUS in RRC connected mode.

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm | N | We agree this is potentially a good solution, but also think this is beyond the scope, since this is an enhancement of PDCCH skipping indication for PDCCH DCI formats. |
| CMCC |  | This may make the mechanism complicate. For instance, if the duration of PDCCH skipping is within the on-duration timer, should UE stop/suspend the timer, and re-trigger/resume it again afterwards? |
| ZTE, Sanechips | N | Based on what we agreed, it is not worthy to spend much time on this issue. |
| Apple | N | Prefer to one solution only which is the already agreed MAC CE |
| Nokia1 | N | This is not an essential enhancement, hence given limited time, we think this should be deprioritized. |
| vivo | Y | We want to clarify that the enhanced PDCCH skipping is not a standalone feature, instead, as we mentioned, it is combinedly used with LP-WUS monitoring. Hence, it is not out of the scope. Besides, the intention of enhanced PDCCH skipping is to let UE immedicately back to LP-WUS monitoring, but since we have agreed that MAC CE based DRX command is already used to terminate DRX active time, so if companies give less interest on this further enhancement, we can also go with the majority. |
| LGE | N | It is already discussed in Rel-17 and beyond the sopce of LP-WUS. We don’t need to spend time to discuss. |
| OPPO | Y | We think this is not out scope, as we discussed if the skipping can indicated LP-WUS monitoring, although we said no in last meeting. We think vivo’s proposal is also a reasonable behaviour for skipping configured with LP-WUS. |
| Xiaomi | Y | For example, during PDCCH skipping duration ,UE can keep on monitoring LP-WUS, thus UE can wake up to monitor PDCCH whenever data arrives rather than wait until the end of the PDCCH skipping duration, which is beneficial for reducing latency. |
| ETRI | Y | We are also supportive for PDCCH skipping. It can support dynamic change between PDCCH moniting and LP-WUS monitoring and reduce latency. However, if majority of companies do not support the enhancement, we can go with the majority. |
| Moderator |  | According to the comments, a number of companies does not think this is essential or in the scope of WI, and also some supporting companies showed flexibility to go with majority view. Therefore, following conclusion is made |

Proposed conclusion 3.1-2a:

- There is no consensus in RAN1 whether to introduce any enhancements of PDCCH skipping indication for the interaction with LP-WUS in RRC connected mode in Rel-19.

| Company | Y/N | Comments |
| --- | --- | --- |

3.2	Timeline between LP-WUS reception/detection and PDCCH monitoring

Regarding the timeline LP-WUS reception/detection and PDCCH monitoring, following agreements were made in previous RAN1 meetings:

Agreement For RRC CONNECTED mode, minimum time gap between LP-WUS reception and MR to start PDCCH monitoring is introduced considering at least following - LP-WUS processing time - MR transition time for ramp up - Time/frequency synchronization of MR - FFS whether UE can report supported minimum time gap from candidate values FFS: Whether the minimum time gap values can be more than one Agreement For RRC CONNECTED mode, support UE capability report for determination of minimum time gap between LP-WUS reception and MR to start PDCCH monitoring. - FFS: exact value(s) of the minimum time gap - FFS: support of multiple minimum time gaps - FFS whether the reported value includes the duration for time/frequency synchronization of MR Agreement Select one of the following alternatives in RAN1#118bis Alt 1 For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. FFS: X FFS: definition of reported value Alt2 For RRC CONNECTED mode, UE reports multiple values for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. FFS: X FFS: definition of reported value Different minimum time gaps correspond to different sleep states Companies are encouraged to details on how the reported values are to be used by the network Agreement For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. FFS: X FFS: definition of reported candidate value UE can indicate its preference for configured time offset between LP-WUS and PDCCH reception using UAI mechanism. FFS details. UAI is optional. FFS: A single time offset is configured for a UE at a given time Agreement - For the X = [2, 3] candidate values for the UE capability report on the minimum time gap for each SCS, consider {0.25, 0.5, 1, 2, 3, 6, 10, 20, 30, 40}ms - Note: Other values are not precluded - Only for reported value of [20ms,] 30ms and 40ms (if supported), the reported values assume SSB periodicity of 20ms and ensures SSB(s) before PDCCH reception and during time gap - FFS: translation of minimum time gap for different SSB periodicities including different candidates for different SSB periodicities - FFS whether the same set of candidates are used for different SCSs - FFS whether different values are selected for OOK-based WUR and OFDM-based WUR - FFS whether/how to consider the case of CA - FFS: candidate values that can be indicated by UAI

This issue has been discussed for multiple RAN1 meetings while details of UE capability report have not been converged yet. A number of companies provided their view on this issue:

- Candidate values

  - X=2: E///, TCL, LGE, Samsung, Nokia, CATT

  - X=3: DCM, Apple, Xiaomi, HW

  - At least 10ms (for deep sleep): QC

  - {Y+0, Y+6, Y+20} ms (for micro/light/deep sleep): DCM

  - {6, 13, 33}ms (for light/deep sleep and deep sleep w/ additional T/F sync): Apple

  - {6, 20, 40}ms (for light/deep sleep and deep sleep w/ additional T/F sync): Apple

  - {2, 6, 20}ms (for light/deep sleep and advanced capability): OPPO

  - {2, 30, 42}ms (for micro/light/deep sleep): HW

  - {3, 10}ms (light/deep sleep): CMCC

  - {1, 3, 10}ms (micro/light/deep sleep): CMCC

  - At least 20ms (for SSB Rx): CATT

  - Up to 20ms (for deep sleep): vivo

- FFS: translation of minimum time gap for different SSB periodicities including different candidates for different SSB periodicities

  - Not support: IDC

  - For larger SSB burst periodicity than 20ms, the translation of delay wake-up is used; For 5ms and 10ms SSB periodicity, the minimum value of 42ms/30ms assuming 20ms SSB periodicity should be kept: HW

- FFS whether different values are selected for OOK-based WUR and OFDM-based WUR

  - Support: QC, E///, LGE, IDC, ZTE

  - Not support: Xiaomi, HW, Nokia, ETRI, vivo

- FFS whether/how to consider the case of CA

  - Support/report different values: QC

  - Not support/report different values: E///, DCM, IDC, HW, ETRI, vivo, ZTE

- FFS: candidate values that can be indicated by UAI

  - up to UE to report a larger or a smaller value than that reported by UE capability: Apple, Nokia

  - larger value than that reported by UE capability: TCL, HW, vivo

  - smaller value than that reported by UE capability: OPPO

Regardigng the candidate values for the minimum time gap, companies have divergent views while those can be considered as 1) around 10ms (for deep sleep state or advanced capability), 2) for shorter gap (e.g. for micro/light sleep), and 3) for longer gap (e.g. for additional T/F sync using SSB) on high level. Therefore, following can be considered as starting point according to companies input. Other FFS can be discussed after this issue is resolved.

[Old][UEF]Proposal 3.2-1:

- For the UE capability report on the minimum time gap for each SCS, support X=3 candidate values corresponding to {V1, 10, V3}ms

  - Note: V1 is smaller than 10ms, V3 is larger than 10ms, FFS exact values

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm | Y | We believe “10ms” is a good number to take a balance between power saving benefit and latency. “20ms”, next to “10ms” in the list of agreed candidate values, is too large as a balanced value from latency perspective. “6ms” is too short as a balanced value from power saving perspective. |
| CMCC | Y | Fine with the proposal |
| CATT | N | I do not know why X=3 in the proposal, we can see more companies support X=2. Also, the values of minimum time gap considered for LP-WUS is different to those minimum time gap in Rel-16 DCP since the UE in Rel-16 DRX adaptation already wake up early to prepare for the coherent demodulation of DCP. In other words, the minimum time gap of LP-WUS needs to take into account of the time MR in preparation of channel tracking for PDCCH coherent demodulation when LP-WUS triggers UE wakeup. Even UE is in-sync (the C-DRX cycle has the periodicity less than 160ms), UE still needs at least 1 SSB to get the latest channel estimation. Thus, 20ms would be the delay for MR ramping up time in preparation of PDCCH demodulation. |
| ZTE, Sanechips | N | The following issues needs to be clarified firstly - Whether the candidate values are based on different SCS - Whether the candidate values are based on different WUR type - the candidate values are based on CA or non-CA |
| Apple |  | We support X=3, and also support V1 and V2. However, for the 10ms value, we propose to support 13ms, the reason is that the 10ms only covers the MR transition time for ramp up from deep sleep state, another 3ms is needed for the LP-WUS processing time. |
| Nokia1 |  | We support the following aspects of this proposal: SCS does not impact the values WUR type does not impact the values CA type does not impact the values [we will accept the majority view] We would like more discussion/conclusion on X=2 or X=3 For example, Support up to X=3 candidate values corresponding to {V1, 10, V3}ms |
| vivo | Y | 10ms may be suitable as the median value of the candidate value. Besides, for the case that MR is enter micro sleep (which there is no ramp up time of MR) during LP-WUS monitoring, we think 0~3ms is resonable to be considered as V1. And for V2, from our view, 20ms is enough, which can aslo contain one SSB used for sync before resuming PDCCH monitoring as companies proposed. |
| LGE | N | The issues of different WUR types and CA/non-CA should be considered for the number and values of candidates. |
| OPPO |  | We think the 10ms is somehow too large. The it can be covered by 20ms. The middle value would be good to be 6 ms, as it was used for power saving light-sleep during the study. |
| Xiaomi |  | Generally OK with the proposal. And we it as a trade-off that the value larger than 10ms contains synchronization delay while other values do not. |
| Moderator |  | This proposal was discussed in Monday online while no consensus was achieved. Following proposal made by RAN1 chair can be further discussed. |

[Old][UEF]Proposal 3.2-1a:

- For the UE capability report on the minimum time gap regardless of SCS, support X=3 candidate values corresponding to {V1, V2, V3}ms where V1<V2<V3.

  - FFS: Whether to support different set of values for different receiver types

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | Discussion points - Whether V1 can be different for different receiver types for adcanced capability - Whether V2 can be [10] ms - Whether V3 can be [20] ms or larger - Whether UAI has same candidate values or not |
| Ericsson1 | N | We support X=2. |

[UEF]Proposal 3.2-1b:

- For the UE capability report on the minimum time gap between the last symbol of LP-WUS and the symbol to start corresponding timer regardless of SCS, support X=3 candidate values corresponding to {V1=[3,5,6], V2=[10,13,20], V3=[33,42]}ms where V1<V2<V3.

  - FFS: Whether to support different set of values for different receiver types

3.3 LP-WUS monitoring occasions

For LP-WUS monitoring occasions in connected mode, following agreements were made in previous RAN1 meetings:

Agreement LP-WUS monitoring occasions (MOs) are configured by RRC, where UE can monitor for LP-WUS transmission in RRC CONNECTED mode. - FFS whether to define a time window for MOs - It is at least supported that a UE can monitor MOs with a periodicity. - FFS details of the periodicity, e.g. derived from DRX cycle, separately configured Agreement - Regarding the potential restriction for LP-WUS configuration in relation with C-DRX configuration for option 1-2 of LP-WUS CONNECTED mode operation, from RAN1 perspective, - Opt2: At least LP-WUS periodicity is no larger than long C-DRX cycle, FFS other restrictions - Introducing monitoring window is not precluded Agreement - For LP-WUS MOs in connected mode for Option 1-1, following is considered further discussion and possible down-selection - Approach 1: - LP-WUS MOs, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s). - UE selectively monitors LP-WUS in the LP-WUS MOs identified by other parameters such as time offset prior to the start of drx-onDurationTimer. - Approach 2: - LP-WUS MOs, including periodicity and offset, are identified based on the slot that drx-onDurationTimer would start and a new RRC parameter indicating the time offset until the slot that drx-onDurationTimer would start. - FFS: An RRC parameter indicates the duration where the UE monitors LP-WUS [consecutively in time] - FFS: the consecutive LP-WUS MOs in time identified by the RRC parameter are associated with a same C-DRX cycle; - FFS: LP-WUS MOs in time identified by the RRC parameter provides consistent indication for the same C-DRX cycle of the UE, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS MOs associated with the same C-DRX cycle. - For LP-WUS MOs in connected mode for Option 1-2, following is considered further - Approach 1: - LP-WUS MOs, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s). - UE monitors LP-WUS in the LP-WUS MOs and starts a new timer for PDCCH monitoring triggered by LP-WUS, after a time offset. - Approach 2: - Periodicity/offset of the slot that the new timer for PDCCH monitoring may be potentially triggered by LP-WUS are configured by RRC parameters. - LP-WUS MOs, including periodicity and offset, are identified based on the slot that new timer would start and a new RRC parameter indicating the time offset until the slot that new timer would start. - FFS: An RRC parameter indicates the duration where the UE monitors LP-WUS [consecutively in time] - FFS: the consecutive LP-WUS MOs in time identified by the RRC parameter are associated with the same slot that the new timer would start; - FFS: LP-WUS MOs in time identified by the RRC parameter provides consistent indication for the same slot that the new timer would start, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS MOs associated with the same slot that the new timer would start. Agreement For LP-WUS MOs in connected mode for Option 1-1, support Approach 1: - LP-WUS MOs, including periodicity and time offset1, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s). - Time offset2 indicates a time prior to a slot where the drx-onDurationTimer would start - UE monitors the first Z LP-WUS MO(s) at or after Time offset2. - FFS: How to determine the value of Z - Time offset2 is RRC configured - UE is not required to monitor LP-WUS during the X [slots/symbols] prior to the beginning of a slot where the UE would start the drx-onDurationTimer. FFS: How X is determined. Agreement For LP-WUS MOs in connected mode for Option 1-2, support Approach 1: - LP-WUS MOs, including periodicity and time offset3, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s). - FFS one or multiple MOs per periodicity - UE monitors LP-WUS in the LP-WUS MOs and, if triggered to wake up, starts a new timer for PDCCH monitoring triggered by LP-WUS, after a time offset4. - The time offset4 configured by the network indicating a time, after which the UE starts PDCCH monitoring via starting the new timer. - FFS: Definition of time offset4

Note that RAN2 also made some agreements related to LP-WUS monitoring occasions

- In RRC_CONNECTED mode, RAN2 to further discuss the impacts of LP-WUS operation methods identified in RAN1. - For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1. - For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline. - RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE. - The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message. For option 1-2, - After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.) - The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring. The range of time offset is left for RAN1.

Related to remaining FFS, companies provide views as follows:

- Option 1-1

  - FFS: How to determine the value of Z

    - Configured by RRC: QC, E///, DCM, Apple, TCL, LGE, Xiaomi, Samsung, Sony, OPPO, Nokia, CATT, SPRD, vivo

      - QC, Samsung, vivo: If a UE is configured with Z > 1, the Z LP-WUS MOs shall be associated with the same C-DRX cycle and provide consistent indication for the same C-DRX cycle of the UE. A UE does not expect to detect more than one LP-WUS with different indications for the given C-DRX cycle in the associated Z LP-WUS MOs.

      - Z could be [2, 3, 4] and one of them is RRC-configured: E///

      - Z is configured in the LP-WUS MO configuration, if not configured, UE monitors all the LP-WUS MO at or after Time offset2 until the X [slots/symbols] prior to the beginning of a slot where the UE would start the drx-onDurationTimer: Apple

      - if more than one MO is configured for triggering PDCCH monitoring of the same onDuration, those MOs are restricted to the period/window between time offset 2 and the time set by X. FFS: Which MOs within this time window the UE monitors, e.g. all or the first few defined by a rule/parameter: Nokia

      - For LP-WUS MOs in connected mode for both Option 1-1 and Option 1-2, support a maximum number of M=4 MOs per periodicity: vivo

  - FFS: How X is determined

    - Minimum time gap reported in UE capability: QC, E///, DCM, TCL?, Sony, HW, Nokia, CATT, SPRD

    - Configured by RRC based on UE’s minimum wake-up capability and also the UAI value for wake-up delay reported by UE: Xiaomi

  - Time offset2

    - same as ps-offset or can be configured as a separate value: LGE

- Option 1-2

  - FFS one or multiple MOs per periodicity

    - one or multiple MOs configured by RRC: E///, DCM, Apple, TCL, Sony, CATT

    - Same Z as Option 1-1 is applied to trigger the new timer starting at the same slot: QC, LGE?, Samsung, Nokia?, ZTE

      - QC, Samsung: If a UE is configured with Z > 1, the Z LP-WUS MOs shall be associated with the same slot where the new timer would start and provide consistent indication for the new timer. A UE does not expect to detect more than one LP-WUS with different indications for the new timer for the slot in the associated Z LP-WUS MOs.

  - FFS: Definition of time offset4

    - The time gap from the last LP-WUS MO of the Z LP-WUS MOs and the associated slot where the new timer would start by the Z LP-WUS MOs is not smaller than the minimum time gap ‘X’: QC, Samsung

    - Offset4 is an offset indicating a time where the UE starts PDCCH monitoring after the last slot of the MO window in which the UE detects LP-WUS: E///, Apple, LGE?, Sony, CATT, vivo

    - Time offset4 is configured so that the gap between the end of last MO in the duration and the start of new timer is not smaller than the minimum time gap: DCM, TCL?, Sony, vivo

    - after configured offset4 slots from positive detection of wake-up the new timer would start: OPPO

  - Offset3

    - Offset 3 is the starting slot of the first MO in a periodicity: Apple

Regarding Option 1-1, following proposal can be considered based on companies input

[Close][RRC]Proposal 3.3-1:

- Update previous agreement in red:

Agreement

- For LP-WUS MOs in connected mode for Option 1-1, support Approach 1:

  - LP-WUS MOs, including periodicity and time offset1, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s).

  - Time offset2 indicates a time prior to a slot where the drx-onDurationTimer would start

    - UE monitors the first Z LP-WUS MO(s) at or after Time offset2.

      - FFS: How to determine the value of Z is RRC configured

    - Time offset2 is RRC configured

  - UE is not required to monitor LP-WUS during the X [slots/symbols] prior to the beginning of a slot where the UE would start the drx-onDurationTimer. FFS: How X is determined given by the minimum time gap reported by UE capability.

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm | Y | Agree with the proposal. |
| CMCC | Y | Fine with the proposal. |
| CATT | Y |  |
| ZTE, Sanechips | Y |  |
| Apple | Y |  |
| Nokia1 |  | We think it would be beneficial for both the Network and the device and also more akin to search space operation, if we change:  UE monitors the first Z LP-WUS MO(s) at or after Time offset2. to.  UE monitors the LP-WUS MO(s) between Time offset2 and X |
| vivo | Y |  |
| LGE | Y |  |
| Spreadtrum | Y |  |
| OPPO | Y | We also would like to have offset1 and offset3 a single parameter. |
| Panasonic | Y |  |
| Xiaomi |  | For the ‘X’, it is the minimum gap between PDCCH monitoring and LP-WUS, and it is the key factor that impact the power saving mode that the UE could apply. Previously, we have agreed to report a UAI value by UE to reflect the UE’s expectation of the time gap between PDCCH monitoring and LP-WUS, and that’s a trade-off that UE only report one wake-up delay value. So we think it is not fair to let X is given by the minimum time gap reported by UE capability, UAI value should also be considered. We prefer X is configured by gNB, based on UAI value and the minimum time gap capability. |
| Sharp |  | Like in DCP, The Z can be derived based on LP-WUS periodicity and duration between offset 2 and X |
| ETRI | Y |  |
| Ericsson1 | Y | Maybe also add FFS: the value of Z |
| Moderator |  | Following was agreed in Tuesday Online Agreement Update previous agreement in red: Agreement - For LP-WUS MOs in connected mode for Option 1-1, support Approach 1: - LP-WUS MOs, including periodicity and time offset1, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s). - Time offset2 indicates a time prior to a slot where the drx-onDurationTimer would start - UE monitors the first Z LP-WUS MO(s) at or after Time offset2. - FFS: How to determine the value of Z is RRC configured - Time offset2 is RRC configured UE is not required to monitor LP-WUS during the X [slots/symbols] prior to the beginning of a slot where the UE would start the drx-onDurationTimer. FFS: How X is determined given based on the minimum time gap reported by UE capability. |

Regarding Option 1-2, as highlighted above, some companies proposed unified configuration between Option 1-1 and Option 1-2 (an example figure is shown below), which can be discussed at first. If this is adopted, previous agreement can be updated as:

[Old][RRC]Proposal 3.3-2:

- Update previous agreement in red:

Agreement

- For LP-WUS MOs in connected mode for Option 1-2, support Approach 1:

  - LP-WUS MOs, including periodicity and time offset3, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s).

    - FFS one or multiple MOs per periodicity

  - UE monitors LP-WUS in the first Z LP-WUS MOs for the same slot that PDCCH monitoring would start and, if triggered to wake up, starts a new timer for PDCCH monitoring triggered by LP-WUS, after a time offset4.

    - The time offset4 configured by the network indicating a time from the first LP-WUS MO, after which the UE starts PDCCH monitoring via starting the new timer.

    - FFS: Definition of time offset4

    - The time gap from the last LP-WUS MO of the Z LP-WUS MOs and the associated slot where the new timer would start by the Z LP-WUS MOs is not smaller than the minimum time gap given by UE capability

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm | Y | Agree with the proposal. There seem proposals to enable multiple LP-WUS occasions within a LP-WUS MO identified by configured periodicity and time offset3. We proposed this way in RAN1#120, but we thought the agreement excludes this approach. The proposal, “Z LP-WUS MOs to trigger PDCCH monitoring from the same slot”, would be an alternative approach to the “multiple LP-WUS occasions within a LP-WUS MO identified by configured periodicity and time offset3”. We think both offers similar or same benefit and hence are fine. |
| CMCC | Y | Fine with the proposal. |
| CATT |  | We are OK with introdicing the Z to algin with Option 1-2 since both ways can bring multiple LP-WUS occasions per periodicity. However, the time offset4 is related with the last LP-WUS MO of the Z LP-WUS MOs is more simpler for us. |
| ZTE, Sanechips | Y | We are fine with this unified solution. Currently, Z is aligned and offset4 and offset 2 are aligned. |
| Apple |  | We are supportive of multiple LP-WUS MOs, however, with the current wording, it is not clear how to determine the start of PDCCH monitoring slot. Does this mean that every Z LP-WUS MOs are a group? When the LP-WUS triggered timer is running and overlaps with some LP-WUS MOs, does this mean that in the next cycle, the LP-WUS MOs that a UE monitors is smaller than Z? |
| Nokia1 |  | We support multiple MOs/windowed MOs for option 1-2. We also support a more aligned approach of defining MOs used by options 1-1 and 1-2. Some wording clarifications are suggested below: “The time offset4 configured by the network indicating a time from the start of the slot of the first LP-WUS MO, after which the UE starts PDCCH monitoring via starting the new timer.” Also, should RAN1 formally define/clarify: (1) a parameter X (minimum time gap for option 1-1) (2) whether ALL or just Z MOs, between time-offset4 and X are monitored (we prefer ALL) |
| vivo | N | We are confuse on how to determine the Z MOs for option 1-2. As we agreed in the last meeting, UE monitors LP-WUS according to the MO configuration (i.e., MO periodicity and time offset3 ) for option 1-2. So, If Z MOs here is tend to be determined relative to the start of the new timer, then how to determine the start time of the new timer? Since we have agreed that the new timer is start at the end of the time offset 4 and the time offset 4 starts based on the trigger of detecting LP-WUS, there is no intention to reopen the discussion. Besides, for the FFS: one or mulitple MOs per MO periodicity, we support to introduce consecutive mulitple MOs which can be regarded as a WUS window or WUS set per periodicity to give the flexibilty for the network to transmit more than one codepoints to wake up multiple UEs. |
| LGE |  | Generally fine with the unified solution. However, we think offset4 is more related to X in the early agreement. So, Z MOs before offset4 priori to the start of new timer can trigger the start of PDCCH monitoring (i.e., new timer) in the same timing, and these MOs can be considered as group. |
| OPPO | N | We don’t think this is needed. The previous agreements for option1-2 is special for the new timer, which is not like DRX ON timer. It works in floating way. If the LP-WUS not detected, then UE try to monitor the LP-WUS in the next period. Thus, no ’Z’ MOs need to be defined. |
| Panasonic | Y |  |
| Xiaomi | N | We think Z is not necessary. If it is for reliability issue, LP-WUS is already RM encoded, and the reliability can be guaranteed. If it is to handle resource conflicting between LP-WUS and legacy channels, gNB can simple transmit LP-WUS in the next available LP-WUS MO. |
| ETRI | N | We agree with OPPO. No Z MOs are necessary. |
| Ericsson1 |  | Proposed changes are not clear to us. Is the intention of introducing Z to resolve “FFS one or multiple MOs per periodicity”? If yes, our preference is to add “Z1 MOs per periodicity can be configured” to replace the FFS in the first main bullet. The Z1 MOs would then be the ‘MO window’. Then for time offset4 our preference is that it starts at the end of MO window. |

[Old][RRC]Proposal 3.3-2a:

- Update previous agreement in red:

Agreement

- For LP-WUS MOs in connected mode for Option 1-2, support Approach 1:

  - LP-WUS MOs, including periodicity and time offset3, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s).

    - FFS one or multiple MOs per periodicity

  - UE monitors LP-WUS in the first Z LP-WUS MOs for the same slot that PDCCH monitoring would start and, if triggered to wake up, starts a new timer for PDCCH monitoring triggered by LP-WUS, after a time offset4.

    - The time offset4 configured by the network indicating a time from the first LP-WUS MO, after which the UE starts PDCCH monitoring via starting the new timer.

    - FFS: Definition of time offset4

    - The time gap from the last LP-WUS MO of the Z LP-WUS MOs and the associated slot where the new timer would start by the Z LP-WUS MOs is not smaller than the minimum time gap given based on UE capability

[RRC]Proposal 3.3-2b:

- Update previous agreement in red:

Agreement

- For LP-WUS MOs in connected mode for Option 1-2, support Approach 1:

  - LP-WUS MOs, including periodicity and time offset3, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s).

    - FFS one or multiple MOs per periodicity

    - UE is configurd with the number of MOs per periodicity by RRC

  - UE monitors LP-WUS in the LP-WUS MOs and, if triggered to wake up, starts a new timer for PDCCH monitoring triggered by LP-WUS, after a time offset4.

    - FFS: Definition of time offset4

    - Alt1:

      - The time offset4 configured by the network indicating a time from the first LP-WUS MO per periodicy, after which the UE starts PDCCH monitoring via starting the new timer.

      - The time gap from the last LP-WUS MO per periodicity and the start of new timer is not smaller than the minimum time gap given based on UE capability

    - Alt2:

      - The time offset4 configured by the network indicating a time from the LP-WUS MO indicating UE to wake-up, after which the UE starts PDCCH monitoring via starting the new timer.

      - The time offset4 is not smaller than the minimum time gap given based on UE capability

3.4 QCL/TCI-state for LP-WUS

Regarding the QCL/TCI state for LP-WUS in CONNECTED mode, following agreements were made in previous RAN1 meetings:

Agreement For LP-WUS monitoring in RRC CONNECTED mode, a LP-WUS is QCLed with existing NR signal/channel/CORESET for the TCI state - FFS which existing NR signal/channel/CORESET is the QCL source of LP-WUS - FFS exact definition of QCL relationship between LP-WUS and existing NR signal/channel/CORESET Agreement - For LP-WUS monitoring in RRC CONNECTED mode, SSB and/or CSI-RS can be the QCL source of LP-WUS FFS applicable QCL type(s) Agreement For LP-WUS monitoring in RRC CONNECTED mode, SSB and/or CSI-RS can be the QCL source of LP-WUS QCL type(s) is {Type A or Type C – for down-selection} and Type D, when applicable Agreement For LP-WUS monitoring in RRC CONNECTED mode, select one of the options on how to define/configure/indicate QCL source of LP-WUS - Opt1: Separate indication/configuration for QCL source of LP-WUS - Opt2: Association of QCL source between LP-WUS and PDCCH/CORESET

Following views are provide in the contributions:

- FFS applicable QCL type(s)

  - Type A:IDC (LP-SS), Xiaomi (LP-SS), QC (CSI-RS)

    - LP-SS is supported for RRC CONNECTED mode LP-WUS monitoring

    - Reuse QCL Type(s) defined for CORESET/PDCCH (i.e. Type A w/ CSI-RS)

  - Type C:vivo, IDC (SSB/CSI-RS), Xiaomi (SSB), Pana

    - no channel estimation and channel equalization operations are expected

    - SSB is QCLed with CSI-RS for tracking in Type C and D not Type A

    - Type C was agreed for idle/inactive mode

- How to define/configure/indicate QCL source of LP-WUS

  - Opt1: QC, DCM, Apple, LGE?, Samsung, IDC, CMCC?, ETRI, vivo, ZTE

    - TCI-state configuration by RRC for LP-WUS: QC, DCM, Apple, LGE?, Samsung, IDC, CMCC?, ETRI, vivo, ZTE

      - MAC-CE for LP-WUS activation/deactivation indicates active TCI-state for the LP-WUS being activated: QC

      - RRC configured K TCI states and MAC-CE activates one of them: DCM, vivo

      - If TCI-state is configured but not indicated, a UE assumes that LP-WUS is QCLed with SSB that the UE recently received for random access or initial access: LGE

      - If a dynamic adaptation of the QCL source for LP-WUS is needed, the QCL source for LP-WUS can follow the latest indication by DCI format using the existing TCI framework: Samsung

    - Support unified TCI framework for LP-WUS: QC, DCM, LGE?

      - LP-WUS follows the activated/indicated TCI-state by MAC-CE/DCI, same as other DL channels/signals: QC, DCM

  - Opt2: E///, OPPO, CATT, HW, Nokia

    - define a rule for the UE that the LP-WUS QCL is the same as the one used to decode PDCCH in the coreset directly following the LPWUS: E///

    - LP-WUS Monitoring Occasions can be associated with CORESETs for the TCI states determination. The activated TCI state of the CORESET is applied for the associated LP-WUS Monitoring Occasion: OPPO, CATT

    - If LP-WUS monitoring is activated by PDCCH, UE can assume the same QCL source as the CORESET to receive the PDCCH activating LP-WUS monitoring.: HW

    - If LP-WUS monitoring is activated by timer, the QCL source can be the same as the CORESET to receive the last PDCCH before the timer expires.: HW

    - The network identifies a specific PDCCH coreset in the active BWP that the LP-WUS is using, for the UE to determine the QCL source to apply for the LP-WUS: Nokia

Regarding the QCL type, this was discussed in the last RAN1 meeting while no consensus was made. Let’s resume the discussion from the latest one discussed in the last RAN1 meeting.

[Old]Question 3.4-1:

- Companie are invited to provide view whether any QCL relation should be defined between LP-SS and LP-WUS

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | There is interesting view from IDC to define QCL type A between LP-SS and LP-WUS in addition to QCL type C between SSB and LP-WUS. Moderator is not sure whether LP-SS is used in connected mode, since there is no agreement or WID objective corresponding to it. |
| Qualcomm |  | We see benefits of enabling LP-SS for LP-WUS, even in connected mode. However, we agree with FL that LP-SS is not part of objective for connected mode. |
| CMCC |  | Okay but we already agreed QCL Type C between LP-SS/LP-WUS and SSB. Whether LR can use the information under QCL Type A may need some further discuss. |
| ZTE, Sanechips | N | But LP-SS is not available in connected mode. I do not understand why we need to defined this. If LP-SS introduced, more spec impacts would be expected. |
| Nokia1 | N | Same comment as ZTE. Per WID, LP-SS is only specified to support Idle/Inactive mode. It is outside the scope of essential functionality for CONNECTED mode. |
| LGE |  | We also think this is interesting. But from our understanding, typically if some signal other than from mTRP has multiple types from multiple QCL source, one QCL type is type D, i.e., related to beam, and the other QCL type is type A, B, or C. So, we are not sure this is valid. |
| OPPO | N | It was not even mandated for WUR measuring LP-SS in CONNECTED mode. Then we may not even configure that QCL difference. |
| Xiaomi |  | For the QCL relation between LP-SS and LP-WUS, we think it is better to be handled in RRC idle/inactive mode. For connected mode, sine LP-SS is not applied, it may be not necessary to define QCL relation between LP-SS and LP-WUS. |
| Moderator |  | Most companies assume LP-SS is not supported in RRC connected mode. Therefore, SSB and/or CSI-RS can be the QCL source for LP-WUS (as agreed previously). As following was agreed in RRC idle/inactive mode in the previous RAN1 meeting, same approach is applied here |

Proposal 3.4-1a:

- Update previous agreement in red:

Agreement

For LP-WUS monitoring in RRC CONNECTED mode, SSB and/or CSI-RS can be the QCL source of LP-WUS QCL type(s) is {Type A or Type C – for down-selection} and Type D, when applicable

| Company | Y/N | Comments |
| --- | --- | --- |

Regarding how to define/configure/indicate TCI-state of LP-WUS, there are divergent views on preferred options between Opt1 and Opt2, while slightly more companies support Opt1, and hence, following can be considered as starting point for further discussion

[RRC][MAC]Proposal 3.4-2:

- For LP-WUS monitoring in RRC CONNECTED mode, support TCI framework for CORESET as baseline for LP-WUS:

Rel-15 TCI framework: RRC configured K TCI states and MAC-CE activates one of them

Rel-17 unified TCI framework: LP-WUS follows the activated/indicated TCI-state by MAC-CE/DCI, same as other DL channels/signals

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm | Y | We agree with the FL proposal. Firstly, we need to support both legacy (non-unified) TCI and unified TCI for LP-WUS. Secondly, for legacy (non-unified) TCI, there must be a way for gNB to indicate/activate TCI state in dynamic manner, since TCI state can be dynamically indicated/activated for all other DL channels including PDCCH. For unified TCI, LP-WUS TCI must follow unified TCI. To meet these, we think the FL proposal is the most reasonable approach. |
| CMCC | Y | Fine with the proposal. |
| CATT | N | Since the LP-WUS is used to trigger PDCCH monitoring and the TCI-state is configured per CORESET for PDCCH, the LP-WUS can be QCLed with the RS associated with the TCI state of the CORESET. |
| ZTE, Sanechips | Y | Association with CORESET is not clear to us. Which CORESET is associated with LP-WUS? How to determine this? Why do not we reuse the legacy mechanism as baseline? |
| Apple | Y | OK with the proposal |
| vivo |  | Does this propose means support option 1 ”Opt1: Separate indication/configuration for QCL source of LP-WUS” or just focus on the framework structure of LP-WUS QCL indication? From our view, we want to support separate configuration for QCL source of LP-WUS. For example, the network configure a ceratin QCL source for a MO configuration by RRC signal. However, we don’t see the need to use dynamic signalling to update the QCL source of LP-WUS. Instead, if there is the need to have different QCL sources for LP-WUS, we think the network can configure mulitple MO configurations (a QCL source associated with a MO config.) to achieve this which is similar as the design of R16 DCP. |
| LGE | Y |  |
| Spreadtrum | Y |  |
| OPPO | Y | We agree the proposal. |
| Panasonic | Y | Fine with the proposal. |
| ETRI | Y |  |
| Ericsson1 | N | Is the proposal based on Option 1 or Option 2? If Option 1, the proposal seems to require two separate MAC CE to be sent in order to set the beams for LP WUS and PDCCH, which will take a lot of time. Our preference is Option 2, i.e., a rule to determine the TCI state based on the upcoming PDCCH TCI state would be simpler. |

4	Activation/deactivation of LP-WUS monitoring

This issue has been discussed in previous RAN1 meetings and following agreements were made:

Agreement For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by Gnb RRC signaling - FFS whether to support UE assistance. - LP-WUS monitoring by UE is known to Gnb. - FFS whether implicit/explicit indication from UE is necessary - In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by Gnb L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission. Agreement - For Option 1-2 of LP-WUS CONNECTED mode operation, for explicit indication to stop the new timer for PDCCH monitoring in Option 1-2 and drx-InactivityTimer for the DRX group, RAN1 identified the following potential solution - Use existing DRX Command MAC CE and Long DRX Command MAC CE - When UE is not in C-DRX active time, UE monitors LP-WUS - Send LS to RAN2 to ask the feasibility of the above potential solution. LS in R1-2410909. Conclusion - For Option 1-1 of LP-WUS CONNECTED mode operation, RAN1 assumes existing DRX Command MAC CE and Long DRX Command MAC CE can be used to stop drx-onDurationTimer and drx-InactivityTimer for the DRX group - When UE is not in C-DRX active time, UE monitors LP-WUS - Send LS to RAN2 to ask whether RAN2 has same understanding or not. LS in R1-2410909. Agreement - LP-WUS can be configured with legacy PDCCH skipping. - Note: There is no intention to modify legacy PDCCH skipping Conclusion From RAN1 perspective, when LP-WUS monitoring is enabled for RRC CONNECTED mode, it is supported that SR, PRACH and CG-PUSCH triggers MR PDCCH monitoring according to the legacy UE behaviour - Above applies at least for Option 1-1

Related to Option 3, a number of companies also propose autonomous fallback to PDCCH monitoring when UE monitors LP-WUS

- Autonomous fallback to PDCCH monitoring when UE monitors LP-WUS

  - Support: QC, vivo, ZTE, Apple, ETRI, LGE, DCM, MTK, Sharp, HW?, CATT

  - Not support: E///

    - Contradicting with previous RAN1 agreement: LP-WUS monitoring by UE is known to gNB

  - Condition to fallback

    - when the channel quality measured by LP-SS for a certain time duration and/or a certain number of times is below the threshold: LGE, ETRI

    - When UE does not received LP-WUS for a long time: ZTE, LGE, MTK

    - Timer since last actual DL reception: MTK, Sharp?

    - Without specified condition (by UE implementation): vivo

  - Fallback to

    - Option 1-1

      - Legacy C-DRX: QC, vivo, Apple, DCM

    - Option 1-2

      - Legacy C-DRX: QC, Apple, DCM

      - Always-on PDCCH monitoring: vivo

      - PDCCH monitoring according to LP-WUS cycle: Apple

  - How to avoid misalignment between gNB and UE for Option 1-2

    - network can configure LP-WUS parameters such that the slots where the new MAC timer for PDCCH monitoring would start are in a superset of the slots where the drx-onDurationTimer would start. With this, at least some PDCCH monitoring windows are guaranteed, no matter whether the UE monitors LP-WUS or the UE fallbacks to the legacy C-DRX operation: QC

    - indicates the fallback state to NW: Apple, DCM, IDC, ETRI

[Old]Proposal 4-1:

- When LP-WUS monitoring is enabled for RRC CONNECTED mode, support UE fallback to PDCCH monitoring when UE monitors LP-WUS

  - FFS the condition for the fallback

  - For LP-WUS operation option 1-1, UE fallbacks to legacy C-DRX behavior

  - FFS For LP-WUS operation option 1-2

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm |  | We feel the proposal implicitly encourages companies to consider condition(s) for fallback. So, we recommend to update the proposal. Our understanding is that for connected mode, “specifying entry/exit conditions” is an objective for idle/inactive modes but not for connected mode, and hence we think this is beyond the WID scope. Could we firstly clarify or conclude: “entry/exit conditions for LP-WUS monitoring are not specified for connected mode”? |
| CMCC |  | Since RRM/RLM/BFD/CSI measurements are still performed by MR, it can up to gNB to control the activation/deactivation of LP-WUS monitoring. E.g., if gNB find out that a measurement report from a UE is not that good, it can deactivate the LP-WUS monitoring via RRC signaling and vise versa. Therefore, we think RRC signaling for activation/deactivation of LP-WUS monitoring is enough. |
| CATT | Y | OK with the proposal, a fallback mechanism to the legacy behavior of PDCCH monitoring can be specified to battle the MDR impact to the PDCCH decoding for UEs in CONNECTED mode. If there is miss detection on LP-WUS, the MR may not be waken-up for a long time due to the LP-WUR continuously monitors LP-WUS. This will lead to excessive delay of the PDCCH decoding triggered by LP-WUS. |
| ZTE, Sanechips |  | We can consider to update the FFS to address Qualcomm’s concern. - FFS whether/how the condition is considered for the fallback Additionally, we support option 1-2 also adopt the same behavior, fallback to C-DRX. Different UE behavior is not expected. |
| Apple | Y | Support the proposal. For Option 1-2, we think falling back to the new timer is a unified solution. |
| Nokia1 |  | As discussed by several companies, some UE indication of fallback could be useful in minimizing gNB-UE misalignment. Should the wording of this proposal include an additional FFS Need for UE indication of fallback? |
| vivo |  | For the first FFS the condition for the fallback, we don’t think the condition for UE fallback must to have, as we mentioned in previous meeting, in RRC connceted mode, it can be up to UE implementation to judge whether it is out of LP-WUS coverage, if yes, then UE fallbacks to PDCCH monitoring. So, we prefer to modify the first FFS as suggested by ZTE. |
| LGE | Y | Fallback operation for Option 1-1 and Option 1-2 can eb unified. |
| OPPO | Y | We consider the timer as one of the fallback condition. Another condistion could be the measurement by UE. E.g. in ceratain threshold, the fallback triggered. For option 1-2, the fall back should be just not monitoring, same as the main bullet. |
| Panasonic | Y | Fine with the proposal. |
| ETRI | Y | We support the proposal. |
| Huawei, HiSilicon | Y | We support the proposal. LP-WUS monitoring may be not reliable and can be missed, a fallback soluiton should be supported. |

Proposal 4-1a:

- When LP-WUS monitoring is enabled for RRC CONNECTED mode, support UE fallback to PDCCH monitoring when UE monitors LP-WUS

  - FFS whether/how the condition is considered for the fallback

  - For LP-WUS operation option 1-1, UE fallbacks to legacy C-DRX behavior

  - FFS For LP-WUS operation option 1-2

| Company | Y/N | Comments |
| --- | --- | --- |
| Ericsson1 | N | Based on the previous agreement, UE monitoring LP-WUS is known to gNB, therefore automonous fallback should not be allowed. gNB should be aware of UE behavior to avoid any potential connection lost. |

Related to Option 4, in the previous conclusion on some UL TX can activate PDCCH monitoring (deactivate LP-WUS monitoring), it is still open question whether the same conclusion can be applied to Option 1-2, and following views are provided:

- Applicability to Option 1-2

  - Yes: DCM (send LS to RAN2 to ask for potential RAN2 spec impact), TCL, Pana, ETRI, vivo

    - Pana: How to avoid the collision between PDCCH monitoring triggered by SR, PRACH, and CG-PUSCH and PDCCH monitoring triggered by LP-WUS should be considered for both Option 1-1 and 1-2 of LP-WUS monitoring procedure

[Close]Proposed conclusion 4-2:

- From RAN1 perspective, when LP-WUS monitoring Option 1-2 is enabled for RRC CONNECTED mode, it is supported that SR, PRACH and CG-PUSCH triggers MR PDCCH monitoring according to the legacy UE behaviour

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm | Y |  |
| CMCC | Y, and | We suggest the scenarios mentioned in our comment for Question 3.1-1 are also worth noting. |
| ZTE, Sanechips | Y | OK as conclusion. |
| Apple | Y |  |
| Nokia1 | Y |  |
| vivo | Y |  |
| LGE | Y |  |
| OPPO | Y |  |
| Panasonic | Y |  |
| ETRI | Y |  |
| Ericsson1 | Y |  |
| Moderator |  | Following was concluded in Tuesday online Conclusion From RAN1 perspective, when LP-WUS monitoring Option 1-2 is enabled for RRC CONNECTED mode, it is supported that SR, PRACH and CG-PUSCH triggers PDCCH monitoring according to the legacy UE behaviour |

5	CA/DC support of LP-WUS

Regarding whether wake-up indication is applicable to one or more serving cells for CA/DC cases, following agreements were made in previous RAN1 meetings

Agreement - Study whether/how LP-WUS works when UE is configured with CA in RRC CONNECTED mode FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable Option 1: one or more serving cells based on Gnb indication/configuration Option 2: all activated serving cells Note: other options are not precluded Agreement LP-WUS is at least supported for the case where a UE is configured with CA in RRC CONNECTED mode - FFS: DC Agreement LP-WUS is supported when UE is configured with NR-DC in RRC CONNECTED mode - Above is supported for the case PDCCH monitoring is triggered by LP-WUS in the same cell group - FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable Agreement - For the case when UE is configured with CA without dual DRX groups in RRC CONNECTED mode, - LP-WUS can be configured on a serving cell - LP-WUS indication is applicable to all serving cells - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - FFS: Whether the serving cell on which the LP-WUS is configured is only PCell or can be RRC configured cell other than PCell Agreement - For the case when UE is configured with NR-DC with CA without dual DRX groups or without CA in RRC CONNECTED mode, - LP-WUS can be configured on a serving cell per cell-group - This does not imply that LP-WUS has to be configured on both cell groups - LP-WUS indication is applicable to all serving cells in the cell-group - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - FFS whether to support this case as separate UE capability from CA case - FFS UE capability for monitoring LP-WUS on one cell group or both cell groups - FFS: Whether the serving cell on which the LP-WUS is configured is only PCell/PSCell or can be RRC configured cell other than Pcell/PSCell

Also, following proposal was discussed in the last RAN1 meeting while no consensus was achieved:

[Open]Proposal 5-1e: - For the case when UE is configured with CA with dual DRX groups in RRC CONNECTED mode, at least for LP-WUS procedure Option 1-1 - Opt.1 - LP-WUS can be configured on a serving cell - LP-WUS carries separate indications for each DRX group(s) to trigger PDCCH monitoring of all serving cells - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - Opt.2 - LP-WUS can be configured on a serving cell per DRX group - LP-WUS indication is applicable to all serving cells in the DRX group - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - Opt.3 - LP-WUS can be configured on a serving cell - LP-WUS indication is applicable to all serving cells of all DRX groups. - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - Opt.4 - Up to RAN2 to decide whether and how to support this case - Opt.5 - LP-WUS cannot be configured on any serving cells for this case - FFS for LP-WUS procedure Option 1-2 - FFS: Whether the serving cell on which the LP-WUS is configured is only PCell or can be RRC configured cell other than PCell

Many companies provided their view on the remaining issues:

- CA without dual DRX groups

  - FFS: Whether the serving cell on which the LP-WUS is configured is only PCell or can be RRC configured cell other than PCell

    - Pcell: QC, E///, MTK, Sharp, Xiaomi, OPPO, CMCC, CATT, ZTE

    - PCell or an activated Scell: DCM, HW, Nokia, SPRD, vivo

- NR-DC with CA without dual DRX groups

  - FFS: Whether the serving cell on which the LP-WUS is configured is only PCell/PSCell or can be RRC configured cell other than Pcell/PSCell

    - PCell/PCell: E///, MTK, OPPO, CMCC, CATT, ZTE

    - PCell/PCell or an activated Scell: DCM, Nokia, SPRD, vivo

  - FFS UE capability for monitoring LP-WUS on one cell group or both cell groups

    - Support: MTK, Apple, Xiaomi, Nokia

- NR-DC without CA

  - FFS: Whether the serving cell on which the LP-WUS is configured is only PCell/PSCell or can be RRC configured cell other than Pcell/PSCell

    - PCell/PCell: E///, MTK, OPPO, CMCC, CATT, ZTE

    - PCell/PCell or an activated Scell: DCM, Nokia, SPRD, vivo

  - FFS UE capability for monitoring LP-WUS on one cell group or both cell groups

    - Support: MTK, Apple, Xiaomi, Nokia

- CA with dual DRX groups

  - Opt1: Apple, Xiaomi, Samsung

  - Opt2: QC, DCM (Option 1-1), MTK, Xiaomi, CMCC

  - Opt3: LGE, HW, ETRI, vivo, ZTE

  - Opt4: E///

  - Opt5: IDC, ZTE

  - FFS for LP-WUS procedure Option 1-2

    - Send LS to RAN2: DCM (whether the new timer can be configured per DRX group)

    - Similar to legacy C-DRX with dual DRX group, the same periodicity and starting offset is configured for the LP-WUS triggered new timer, while the duration can be different: Apple

    - The same LP-WUS triggered timer is applied on both DRX groups: Apple, Samsung

    - LP-WUS Option 1-2 is not supported with CA with dual DRX group: Apple

Following proposals are made based on the input.

[Close][RRC][UEF]Proposal 5-1:

- For the case when UE is configured with CA without dual DRX groups in RRC CONNECTED mode,

  - LP-WUS can be configured only on PCell

- For the case when UE is configured with NR-DC with CA without dual DRX groups or without CA in RRC CONNECTED mode,

  - LP-WUS can be configured only on PCell/PSCell per cell-group

  - Introduce serarate UE capability for monitoring LP-WUS on one cell group or both cell groups

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm | Y | For the last sub-bullet on UE capability, we suggest to update as ”Introduce UE capability for LP-WUS operation on one cell group or both cell groups for NR-DC. Details to be discussed in UE feature session”. |
| CMCC | Y | Fine with the proposal. |
| CATT |  | OK witht the proposal. |
| ZTE, Sanechips | Y |  |
| Apple | Y | Support the proposal |
| Nokia1 |  | Ok with the proposal |
| vivo |  | It may be not necessary to force the LP-WUS must be configured in the Pcell. To give the more flexible of LP-WUS monitoring and the network LP-WUS configuration, we think LP-WUS monitroing can be configured in Pcell/PScell or an activated Scell. |
| LGE | Y |  |
| OPPO | Y |  |
| Panasonic | Y |  |
| Sharp | y |  |
| ETRI | Y |  |
| Moderator |  | Following was agreed in Monday online. Whether to introduce serarate UE capability will be further discussed in UE feature session. Agreement - For the case when UE is configured with CA without dual DRX groups in RRC CONNECTED mode, LP-WUS can be configured only on PCell - For the case when UE is configured with NR-DC with CA without dual DRX groups or without CA in RRC CONNECTED mode, LP-WUS can be configured only on PCell/PSCell per cell-group |

[Close][RRC]Proposal 5-2:

- For the case when UE is configured with CA with dual DRX groups in RRC CONNECTED mode, at least for LP-WUS procedure Option 1-1, down select one of the following in RAN1#120bis

  - Opt.1

    - LP-WUS can be configured on a serving cell

    - LP-WUS carries separate indications for each DRX group(s) to trigger PDCCH monitoring of all serving cells

      - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation

  - Opt.2

    - LP-WUS can be configured on a serving cell per DRX group

    - LP-WUS indication is applicable to all serving cells in the DRX group

      - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation

  - Opt.3

    - LP-WUS can be configured on a serving cell

    - LP-WUS indication is applicable to all serving cells of all DRX groups.

      - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation

  - Opt.4

    - Up to RAN2 to decide whether and how to support this case

  - Opt.5

    - LP-WUS cannot be configured on any serving cells for this case

  - FFS for LP-WUS procedure Option 1-2

- FFS: Whether the serving cell on which the LP-WUS is configured is only PCell or can be RRC configured cell other than PCell

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm | Y | Our preference is Opt.2. Power saving via LP-WUS is beneficial for CA in connected mode. NR supports CA with various carrier types. To accommodate power saving for different carrier characteristics, dual DRX group has been introduced in the past. There is no reason not to enable functional support of LP-WUS for CA with dual DRX group. |
| CMCC | Y | Fine with the proposal and our perference is Opt.2 which is striaghtforward. |
| CATT | N | Based on above summary, CATT should be added in Opt5. For Rel-16 DCP, the secondary DRX group and DCP cannot be configured simultaneously due to the longer duration of drx-onDurationTimer or drx-InactivityTimer may overlap with the next DCP occasion. For LP-WUS, if dual DRX configured, same issue may be arisen. Thus, the dual DRX group with LP-WUS simultaneously for Rel-19 LP-WUS should not be considered. |
| ZTE, Sanechips |  | Prefer option5 which is similar as DCP. Could live with option3. |
| Apple | Y | We think Option 1 could be beneficial for UE power saving, since LP-WUS could separately trigger PDCCH on the two DRX groups. For Option 2, we think this should also based on UE capability whether LR can simultaneously work on two frequencies. If the LR supports operating on only one FR, then the additional UE power saving gain can not be achieved. |
| Nokia1 | Y | Our preference is Opt 2 |
| vivo | Y | We supprot option2. It is up to the network to decide whether configure LP-WUS monitoring in both two DRX groups, if yes, then UE monitor the LP-WUS indication in each DRX group, and the LP-WUS indication is applied within the corresponding DRX group. |
| LGE | Y | We prefer Opt 3 and can consider Opt 5. |
| OPPO | Y | Option 2 is OK for us. And we’d better to restrict it to be a Pcell/Pscell. We are also fine to have Option 5, if no agreement. |
| Panasonic | Y |  |
| Sharp |  | The start of On-durationtimer for two DRX group is aligned, we prefer option 1 to reduce UE capability to monitor two LP-WUS in same time. |
| ETRI | Y | We prefer Opt 3. |
| Huawei, HiSilicon | Y | We support option 3. Option 4 should be removed. This is clearly related with RAN1. |
| Moderator |  | This proposal was discussed in Monday online while no consensus was achieved. Following proposal can be further discussed |

[Close][RRC]Proposal 5-2a:

- For the case when UE is configured with CA with dual DRX groups in RRC CONNECTED mode, at least for LP-WUS procedure Option 1-1, down select one of the following in RAN1#120bis

  - Opt.1

    - LP-WUS can be configured only on PCell

    - LP-WUS carries separate indications for each DRX group(s) to trigger PDCCH monitoring of all serving cells

      - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation

  - Opt.2

    - LP-WUS can be configured on a serving cell per DRX group

    - LP-WUS indication is applicable to all serving cells in the DRX group

      - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation

    - FFS whether to introduce serarate UE capability for monitoring LP-WUS on one DRX group or both DRX groups, and details when UE supports LP-WUS monitoring only on one DRX group

  - Opt.3

    - LP-WUS can be configured only on PCell

    - LP-WUS indication is applicable to all serving cells of all DRX groups.

      - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation

  - Opt.4

    - Up to RAN2 to decide whether and how to support this case

  - Opt.5

    - LP-WUS cannot be configured on any serving cells for this case

  - FFS for LP-WUS procedure Option 1-2

- FFS: Whether the serving cell on which the LP-WUS is configured is only PCell or can be RRC configured cell other than PCell

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | Following was concluded in Tuesday Online Conclusion - For the case when UE is configured with CA with dual DRX groups in RRC CONNECTED mode, at least for LP-WUS procedure Option 1-1, there is no consensus in RAN1 which of the following options to support for this case in Rel-19. - Alt1 - Basic capability - LP-WUS can be configured only on PCell - LP-WUS indication is applicable to all serving cells of all DRX groups. - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - Advanced capability - LP-WUS can be configured on PCell for primary DRX group and an activated SCell for secondary DRX group - LP-WUS indication is applicable to all serving cells in the DRX group where LP-WUS is monitored - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - This capability has prerequisite of the above basic capability - Alt2 - LP-WUS can be configured on PCell for primary DRX group and/or an activated SCell for secondary DRX group - LP-WUS indication is applicable to all serving cells in the DRX group where LP-WUS is monitored - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - This capability has candidate values for the LP-WUS configuration with {primary DRX group, secondary DRX, both} |

6	Coexistence with existing UE power saving features

This issue has been discussed in previous RAN1 meetings, and following agreements were made:

Agreement For RRC CONNECTED mode, LP-WUS can be configured without following existing features. Rel-16 DCP Rel-17 PDCCH skipping Rel-17 SSSG switching Rel-18 cell DTX Further study whether/how LP-WUS works with following existing features (PDCCH skipping, SSSG switching, cell DTX) Agreement - LP-WUS can be configured with legacy PDCCH skipping. - Note: There is no intention to modify legacy PDCCH skipping Agreement - For RRC CONNECTED mode, LP-WUS can be configured with Rel-17 SSSG switching Agreement - For RRC CONNECTED mode, LP-WUS can be configured with Cell DTX - FFS During Cell DTX inactive time, whether the UE is expected to monitor LP-WUS or not

Note that RAN2 have made following agreements, and hence simultanoeus configuration of LP-WUS and DCP is no longer considered.

- In RRC_CONNECTED mode, RAN2 to further discuss the impacts of LP-WUS operation methods identified in RAN1. - For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1. - For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline. - RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE. - The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message.

Many companies provided their view on the FFS in Cell DTX as follows. Since companies views are split, one way forward is to support different behavior between Option 1-1 and Option 1-2

- Cell DTX

  - FFS During Cell DTX inactive time, whether the UE is expected to monitor LP-WUS or not

    - UE is expected to monitor LP-WUS: DCM, HW, Nokia, SPRD, vivo (Option 1-1)

    - UE is not expected to monitor LP-WUS: E///, Apple, TCL, IDC, OPPO, vivo (Option 1-2), ZTE

      - UE does not expect the case when all LP-WUS MOs are configured within cell DTX non-active period: Apple

[Old]Proposal 6-1:

- For RRC CONNECTED mode when LP-WUS is configured with Cell DTX, during Cell DTX inactive time,

For Option 1-1, the UE is expected to monitor LP-WUS

For Option 1-2, the UE is not expected to monitor LP-WUS

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | The background to support different behavior between Option 1-1 and Option 1-2 in vivo’s contribution: --- - For LP-WUS connected mode operation Option 1-1, similar as Rel-16 DCP, UE is expected to monitor LP-WUS during the Cell DTX inactive time. Once LP-WUS is detected, UE starts the drx-onDurationTimer for the corresponding C-DRX cycle. It was agreed RAN2#123 for Rel-18 NES that gNB should ensure that there is at least partial overlapping between UE C-DRX on-duration and cell DTX/DRX on-duration and the configured cellDTXDRX-Cycle is an integer multiple of configured drx-longCycle or vice versa. - For LP-WUS connected mode operation Option 1-2, different from Option 1-1 where the typical WUS configuration for Option 1-1 has some alignment with C-DRX cycle, e.g., the periodicity of WUS configuration for Option 1-1 is one or multiple times of long C-DRX cycle(s), resulting in less LP-WUS monitoring occasions. Therefore, for WUS option 1-1, it makes sense to support LP-WUS monitoring during the Cell DTX inactive time. However, for option 1-2, the main purpose is to enable more frequent LP-WUS monitoring for faster and more flexible triggering of PDCCH monitoring that is not restricted by C-DRX configuration. The periodicity for WUS configuration for option 1-2 should be much shorter compared to WUS configuration for option 1-1. Therefore, for LP-WUS operation option 1-2, it is reasonable that UE monitor WUS only within cell DTX active time for both network and UE energy savings. When WUS is detected, the UE starts a NEW Timer and enters the C-DRX active time. |
| Qualcomm |  | In principle we think better not to differentiate the behavior between option 1-1 and option 1-2, unless necessary. We understand the rationale of the proposal for Option 1-1. However, it is not clear why in the proposal, LP-WUS monitoring (hence PDCCH triggering) has to be restricted within Cell-DTX active time for Option 1-2. The purpose of Option 1-2 is low latency by enabling PDCCH monitoring from anytime. The whole benefit of LP-WUS Option 1-2 is gone if LP-WUS monitoring is only within Cell-DTX active time. |
| CATT |  | Simlar with Qualcomm, perfer not to differentiate the behavior between option 1-1 and option 1-2. |
| ZTE, Sanechips |  | the UE is not expected to monitor LP-WUS should be applicable for both option 1-1 and option 1-2. - The reason for option 1-2 not monitoring LP-WUS during inactive time is that there are lots of MOs for LP-WUS and the NW does not worry about all the MOs are occupied. Then in this case, why the gNB would configure option 1-1 and let UE to receive LP-WUS, instead of directly configuring option 1-2 ? - Then in summary, There is no need to differentiate the UE behavior for both options. |
| Apple |  | We also prefer to not differentiate between Option 1-1 and Option 1-2. We support UE does not monitor LP-WUS during cell DTX inactive period. For Option 1-1, NW ensures that LP-WUS is within cell DTX active period. |
| Nokia1 |  | Unified behavior between option 1-1 and option 1-2 is preferred. |
| vivo | Y | We agree the explaination of FL. Since UE only monitor a few LP-WUS MOs prior to DRX onduration for option 1-1, it can be a compromise for UE to monitor LP-WUS outside cell DTX active time for option 1-1. But for option 1-2, since UE needs to monitor plenty of MOs before it detects the LP-WUS, considering not impact on the network power saving, we prefer UE will not expect to monitor LP-WUS during cell DTX off time for option 1-2. But if the majority view is to align the UE behavior for option 1-1 and option 1-2, then we are OK that UE is not expected to monitor LP-WUS during cell DTX off. |
| LGE |  | We do not prefer different behavior between Option 1-1 and Option 1-2. Valid MOs that the UE should monitor for Option 1-2 during cell DTX inactive period can be restricted, then we can expect NW energy saving and some latency reduction. |
| Spreadtrum |  | We also perfer not to differentiate the behavior between option 1-1 and option 1-2. |
| OPPO |  | We’d better to have same solution for Option1-1/1-2 |
| Xiaomi |  | Prefer Option 2, since for cell-DTX non-active time, gNB power saving should be considered, and LP-WUS should not be transmitted for gNB energy saving. |
| Moderator |  | Seems the WF does not work for companies. Then, companies have split views. We can take 2nd one which has slightly more support as starting point - UE is expected to monitor LP-WUS: DCM, HW, Nokia, SPRD, vivo (Option 1-1), QC - UE is not expected to monitor LP-WUS: E///, Apple, TCL, IDC, OPPO, vivo (Option 1-2), ZTE, Xiaomi - UE does not expect the case when all LP-WUS MOs are configured within cell DTX non-active period: Apple |
| Ericsson1 |  | We think there is no need to differentiate between the two options and we support UE does not monitor LP-WUS during cell DTX inactive period. |

Proposal 6-1a:

- For RRC CONNECTED mode when LP-WUS is configured with Cell DTX, during Cell DTX inactive time,

For Option 1-1, the UE is expected to monitor LP-WUS

For Option 1-2, the UE is not expected to monitor LP-WUS both for Option 1-1 and 1-2

| Company | Comments |
| --- | --- |

7	Other aspects

In addition to the discussion points in previous section, following proposal was made in the last RAN1 meeting, while there was not enough time to discuss:

[Open]Proposal 7-1a: - Consider the overlapping handling between LP-WUS reception and existing NR transmission/reception (e.g., UL Tx, configured DL Rx, TDD DL/UL patterns).

Related to this aspect, companies provide following views:

- DCM: Consider the overlapping handling of PDCCH as baseline for the overlapping handling between LP-WUS reception and existing NR transmission/reception

- MTK: Establish clear scheduling restrictions, guard intervals, and priority rules to ensure LP-WUS does not interfere with essential UL or DL operations. If overlaps occur, prioritize normal NR procedures; skip or postpone WUS and rely on fallback as necessary, thereby preserving network reliability and power-saving benefits.

- CMCC: RAN1 to discuss the UE behaviour when LP-SS/LP-WUS transmission overlap with existing pattern (e.g. UL slot/symbol, common signal transmission, configuration related to L1/L3 measurement), or when the time interval between LP-SS/LP-WUS transmission and existing pattern is smaller than the RF retuning time (e.g. when LR/MR is deployed in FR2, or when SCS of PRACH is equal or larger than 15kHz)

[Close]Proposal 7-1:

- Consider the overlapping handling between LP-WUS reception and existing NR transmission/reception (e.g., UL Tx, configured DL Rx, TDD DL/UL patterns).

| Company | Comments |
| --- | --- |
| Qualcomm | We think we have provided our view in Question 3.1-1. |
| CMCC | Please see our comment under Question 3.1-1. |
| ZTE, Sanechips | OK to consider |
| LGE | We can consider. But it seems similar issue with Question 3.1-1, so it can be merged. |
| Panasonic | We are fine to consider this related to Question3.1-1. |
| Xiaomi | OK |
| Moderator | This aspect can be discussed together with Question 3.1-1 |

As captured in “Summary of observations/proposals in contributions” in this document, companies proposed aspects other than those discussed in previous sections. Here we can discuss which additional aspects should be discussed in this meeting considering the remaining RAN1 meetings before Rel-19 RAN1 functional freeze. Companies are invited to provide view in the following question.

[Old]Question 7-2:

- Companies are invited to provide view on which aspects, other than those discussed in previous sections,  should be discussed in this meeting.

| Company | Comments |
| --- | --- |
| Qualcomm | For Rel-16 DCP, it is possible to configure whether drx-onDurationTimer is triggered when DCP is not detected at the MO by RRC parameter ps-Wakeup. For Rel-19 LP-WUS in connected mode, we think the same RRC parameter is beneficial at least for Option 1-1. We recommend to discuss in RAN1. This has RRC impact. |
| Xiaomi | Similar as defined for C-DRX, P/SP SRS does not need to be transmitted during non-active time, since no dynamic UL scheduling |
| Huawei, HiSilicon | We have agreed that gNB should know whether UE monitors LP-WUS or not. Therefore, the feedback from UE after UE wakes-up should be discussed. |
| Moderator | The proposal from QC has already been considered in Proposal 3.1-1a. Following proposal by Xiaomi can be discussed. The proposal from HW/HiSi can be discussed together with Proposal 4-1a |
| Ericsson1 | Certain PDCCH ALs can not be supported by LP-WUS as discussed in our contribution. The PDCCH search spaces/ALs applicable for LP-WUS should be discussed. |

Proposal 7-2:

For RRC connected mode, the same behaviour as periodic CSI/L1-RSRP report in both Option 1-1 and Option 1-2 can be applied to periodic/semi-persistent SRS transmission

| Company | Comments |
| --- | --- |

8	Conclusions

Following agreements and conclusions were made in this meeting:

Agreement

- For the case when UE is configured with CA without dual DRX groups in RRC CONNECTED mode, LP-WUS can be configured only on PCell

- For the case when UE is configured with NR-DC with CA without dual DRX groups or without CA in RRC CONNECTED mode, LP-WUS can be configured only on PCell/PSCell per cell-group

Conclusion

From RAN1 perspective, when LP-WUS monitoring Option 1-2 is enabled for RRC CONNECTED mode, it is supported that SR, PRACH and CG-PUSCH triggers PDCCH monitoring according to the legacy UE behaviour

Conclusion

- For the case when UE is configured with CA with dual DRX groups in RRC CONNECTED mode, at least for LP-WUS procedure Option 1-1, there is no consensus in RAN1 which of the following options to support for this case in Rel-19.

  - Alt1

    - Basic capability

      - LP-WUS can be configured only on PCell

      - LP-WUS indication is applicable to all serving cells of all DRX groups.

        - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation

    - Advanced capability

      - LP-WUS can be configured on PCell for primary DRX group and an activated SCell for secondary DRX group

      - LP-WUS indication is applicable to all serving cells in the DRX group where LP-WUS is monitored

        - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation

      - This capability has prerequisite of the above basic capability

  - Alt2

    - LP-WUS can be configured on PCell for primary DRX group and/or an activated SCell for secondary DRX group

    - LP-WUS indication is applicable to all serving cells in the DRX group where LP-WUS is monitored

      - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation

    - This capability has candidate values for the LP-WUS configuration with {primary DRX group, secondary DRX, both}

Agreement

Update previous agreement in red:

Agreement

- For LP-WUS MOs in connected mode for Option 1-1, support Approach 1:

  - LP-WUS MOs, including periodicity and time offset1, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s).

  - Time offset2 indicates a time prior to a slot where the drx-onDurationTimer would start

    - UE monitors the first Z LP-WUS MO(s) at or after Time offset2.

      - FFS: How to determine the value of Z is RRC configured

    - Time offset2 is RRC configured

UE is not required to monitor LP-WUS during the X [slots/symbols] prior to the beginning of a slot where the UE would start the drx-onDurationTimer. FFS: How X is determined given based on the minimum time gap reported by UE capability.

References

| [1] | RP-241824 | Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR) | vivo, NTT DOCOMO |
| --- | --- | --- | --- |
| [2] | R1-2400640 | Work plan for Low-power wake-up signal and receiver for NR | vivo, NTT DOCOMO |
| [3] | R1-2410907 | FL summary #5 on LP-WUS operation in CONNECTED mode | Moderator (NTT DOCOMO) |
| [4] | R1-2501769 | Discussion on LP-WUS operation in CONNECTED mode | ZTE Corporation, Sanechips |
| [5] | R1-2501816 | Remaining issues on LP-WUS operation in CONNECTED modes | vivo |
| [6] | R1-2501877 | Discussion on LP-WUS operation in CONNECTED modes | Spreadtrum, UNISOC |
| [7] | R1-2502001 | System design and procedure of LP-WUS operation for UE in CONNECTED Modes | CATT |
| [8] | R1-2502009 | LP-WUS operation in CONNECTED mode | Nokia |
| [9] | R1-2502060 | Discussion on LP-WUS operation in CONNECTED mode | Panasonic |
| [10] | R1-2502077 | Discussion on LP-WUS operation in RRC CONNECTED mode | NEC |
| [11] | R1-2502169 | Discussion on LP-WUS operation in CONNECTED mode | CMCC |
| [12] | R1-2502239 | Procedures and functionalities of LP-WUS in CONNECTED mode | Huawei, HiSilicon |
| [13] | R1-2502259 | Further consideration on LP-WUS operation in connected mode | OPPO |
| [14] | R1-2502304 | Discussion on RRC CONNECTED mode LP-WUS monitoring | InterDigital, Inc. |
| [15] | R1-2502324 | LP-WUS operation in CONNECTED mode | Sony |
| [16] | R1-2502377 | Discussion on LP-WUS operation in CONNECTED modes | Samsung |
| [17] | R1-2502450 | Discussion on LP-WUS operation in Connected mode | Xiaomi |
| [18] | R1-2502483 | Discussion on LP-WUS operation in CONNECTED modes | LG Electronics |
| [19] | R1-2502517 | Discussion on LP-WUS operation in CONNECTED modes | ETRI |
| [20] | R1-2502576 | Discussion on LP-WUS procedures in Connected mode | TCL |
| [21] | R1-2502617 | LP-WUS operation in CONNECTED modes | Apple |
| [22] | R1-2502677 | Discussion on LP-WUS operation in CONNECTED modes | Sharp |
| [23] | R1-2502713 | LP-WUS operation in CONNECTED modes | MediaTek Inc. |
| [24] | R1-2502775 | Discussion on LP-WUS operation in CONNECTED mode | NTT DOCOMO, INC. |
| [25] | R1-2502807 | LP-WUS operation in CONNECTED mode | Ericsson |
| [26] | R1-2502848 | LP-WUR operation in connected mode | Qualcomm Incorporated |

Summary of observations/proposals in contributions

R1-2501769	ZTE Corporation, Sanechips

Observations: Observation 1: Monitoring DCI for LP-WUS activation, compared with RRC activation, causes more power consumption. Observation 2: OFDM WUR has smaller minimum time gap than OOK WUR - OFDM WUR has less processing time than OOK WUR - OFDM WUR has less ramp-up time than OOK WUR Proposals: Proposal 1: RRC activation/deactivation is the baseline - DCI deactivation and fallback mechanism based on that UE does not received LP-WUS for a long time can be considered. - The motivation for L2 activation/deactivation should be further clarified if L1 deactivation is introduced. - Option 4 is not considered for activation/deactivation of LP-WUS monitoring. Proposal 2: A time window defined as number of MOs is needed for both option 1-1 and option 1-2 - Time offset 4 could be defined between the window start and new timer start Proposal 3: Outside new or legacy active time, reusing legacy mechanism as DCP for Searchspace monitoring. Proposal 4: The LP-WUS is used to indicate wake-up for PDCCH monitoring - Target all the configured searchspaces in the active DL BWP Proposal 5: Use RRC configuration to indicate the TCI state for LP-WUS. Proposal 6: LP-WUS can be transmitted in PCell or PScell. Proposal 7: It is considered to support option 3 or not support dual DRX group for LP-WUS similar as DCP Proposal 8: The candidate value is defined based on LP-WUS processing time and ramp-up time from LR to MR - The candidate value should differ from different WUR type - There is no need to differentiate the CA case Proposal 9: During Cell-DTX off period, UE does not need to monitor LP-WUS.

R1-2501816	vivo

Proposal 1: For LP-WUS operation Option 1-2, LP-WUS periodicity is no larger smaller than long C-DRX cycle. - No other restrictions for LP-WUS monitoring configuration in relation with C-DRX configuration are needed. Proposal 2: For the case when UE is configured with CA with dual DRX groups in RRC CONNECTED mode, for both LP-WUS procedure Option 1-1 and Option 1-2, a UE monitors LP-WUS only on one serving cell per DRX group. - The serving cell where to monitor LP-WUS is configured by the network. - LP-WUS indication is applicable to all serving cells in the DRX group where the LP-WUS is monitored - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation Proposal 3 For LP-WUS operation option 1-1 and option 1-2 in RRC CONNECTED mode, the serving cell on which the LP-WUS can be RRC configured cell other than Pcell/PSCell. Proposal 4: For the candidate values for the UE capability report on the minimum time gap for each SCS, it is unnecessary to restrict the candidates to the same set for different SCSs. Proposal 5: Allow a UE to report different values of the minimum time gap for OOK-based WUR and OFDM-based WUR, and for non-CA and CA scenarios. Up to 20ms can be considered as the maximum value for the minimum time gap reported by the UE. Proposal 6: - A single time offset is configured for a UE at a given time. - The UAI value(s) should be equal to or longer than the minimum time gap reported by UE capability. - More UAI details should be decided in RAN2. Proposal 7: - For Option 1-1 of LP-WUS CONNECTED mode operation, the minimum time gap is defined as the time between the end of the slot of the LP-WUS MO where LP-WUS is detected and the beginning of the slot where the UE would start the drx_onDurationTimer. - For Option 1-2 of LP-WUS CONNECTED mode operation, the minimum time gap is defined as the time between the end of the slot of the LP-WUS MO where LP-WUS is detected and the beginning of the first slot where the UE would start PDCCH monitoring. Conclusion: From RAN1 perspective, when LP-WUS monitoring is enabled for RRC CONNECTED mode for Option 1-2, it is supported that SR, PRACH and CG-PUSCH triggers MR PDCCH monitoring according to the legacy UE behavior. Proposal 8: When LP-WUS monitoring is enabled for RRC CONNECTED mode, support UE fallback to PDCCH monitoring when UE monitors LP-WUS. - No need to specify any condition for the fallback - No need to have indication from UE when it falls back to PDCCH monitoring. - For LP-WUS operation option 1-1, the fallback behavior for PDCCH monitoring is according to the C-DRX configuration - For LP-WUS operation option 1-2, the fallback behavior for PDCCH monitoring is always-on PDCCH monitoring. Proposal 9: At least for LP-WUS connected mode operation Option 1-2, support enhanced PDCCH skipping indication that is used to indicate UE to terminate the DRX active time (e.g., stop PDCCH monitoring on one or multiple serving cells) and starts LP-WUS monitoring. Proposal 10: For LP-WUS connected mode operation Option 1-1 configured with Cell DTX, UE is expected to monitor LP-WUS during the Cell DTX inactive time. Proposal 11: For LP-WUS connected mode operation Option 1-2 configured with Cell DTX, a UE does not expect to monitor the LP-WUS during the non-active periods of cell DTX of a serving cell. Proposal 12: For LP-WUS MOs in connected mode for both Option 1-1 and Option 1-2, support a maximum number of M=4 MOs per periodicity. - The M value is configured by RRC with the value range of INTEGER (1..4) Proposal 13: For LP-WUS MOs in connected mode for both Option 1-1 and Option 1-2, support more than one LP-WUS configurations where the QCL configuration/indication is on a per LP-WUS configuration. Proposal 14: For LP-WUS MOs in connected mode for Option 1-1, support Approach 1: - LP-WUS MOs, including periodicity and time offset1, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s). - The periodicity and time offset1 provide the periodicity and offset of the LP-WUS MO(s), following candidate values can be considered: - sl1, NULL, - sl2, INTEGER {0..1}, - sl4, INTEGER {0..3}, - sl5, INTEGER {0..4}, - sl8 INTEGER {0..7} - sl10, INTEGER {0..9} - sl16, INTEGER {0..15} - sl20, INTEGER {0..19} - sl40, INTEGER {0..39} - sl80, INTEGER {0..79} - sl160, INTEGER {0..159} - sl320, INTEGER {0..319} - sl640, INTEGER {0..639} - sl1280, INTEGER {0..1279} - sl2560, INTEGER {0..2559} - The number of MOs per periodicity “M” is configured by RRC, the candidate values for M is INTEGER {1..4} - Time offset2 i.e., Time_offset_CONNECTED_Option1-1 configured by a new RRC parameter, indicates a time offset where the UE starts monitoring LP-WUS at LP-WUS MO(s), prior to a slot where the drx-onDurationTimer would start - The values for Time_offset_CONNECTED_Option1-1 is in multiples of 0.125ms (milliseconds), INTEGER (3..640), where 3 corresponds to 0.375ms, 4 corresponds to 0.5ms and so on. - Z configured by a new RRC parameter provides a number of consecutive LP-WUS period(s) for LP-WUS monitoring for a C-DRX cycle. - The candidate values on the Z can be: INTEGER {1.. [2]} - The UE monitors LP-WUS at the WUS MOs in the first LP-WUS monitoring period(s) indicated by Z. - The LP-WUS MOs associated with the same C-DRX cycle provides consistent indication for the same C-DRX cycle of the UE, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS MOs associated with the same C-DRX cycle. - UE is not required to monitor LP-WUS during the X slots prior to the beginning of a slot where the UE would start the drx-onDurationTimer, where X corresponds to the UE capability signaling report of the minimum time gap between the end of the slot of the LP-WUS MO where LP-WUS is detected and the beginning of the slot where the UE would start the drx_onDurationTimer. Proposal 15: For LP-WUS MOs in connected mode for Option 1-2, support Approach 1: - LP-WUS MOs, including periodicity and time offset3, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s). - The periodicity and time offset3 provide the periodicity and offset of the LP-WUS MO(s). following candidate values can be considered: - sl1, NULL, - sl2, INTEGER {0..1}, - sl4, INTEGER {0..3}, - sl5, INTEGER {0..4}, - sl8 INTEGER {0..7} - sl10, INTEGER {0..9} - sl16, INTEGER {0..15} - sl20, INTEGER {0..19} - sl40, INTEGER {0..39} - sl80, INTEGER {0..79} - The number of MOs per periodicity “M” is configured by RRC, the candidate values for M is INTEGER {1..4} - UE monitors LP-WUS in the LP-WUS MOs and, if triggered to wake up, starts a new timer for PDCCH monitoring triggered by LP-WUS, after a time offset4, i.e., Time_offset_CONNECTED_Option1-2. - The Time_offset_CONNECTED_Option1-2 configured by the network indicating a time between the end of the slot of the LP-WUS MO where LP-WUS is detected and the beginning of the first slot where the UE starts PDCCH monitoring via starting the new timer. - The values for Time_offset_CONNECTED_Option1-2 is in multiples of 0.125ms (milliseconds), INTEGER (2..160), where 2 corresponds to 0.25ms, 3 corresponds to 0.375ms and so on. - UE is not expected to be configured with a time offset4 that is smaller than the minimum value of the minimum time gap(s) reported by UE capability signaling. Proposal 16: For LP-WUS monitoring in RRC CONNECTED mode, SSB and/or CSI-RS can be the QCL source of LP-WUS QCL type(s) is Type C and Type D, when applicable Proposal 17: For LP-WUS monitoring in RRC CONNECTED mode, support separate indication/configuration for QCL source of LP-WUS MO(s). - Separate RRC configuration and separate MAC CE are introduced to configure/indicate TCI state of LP-WUS monitoring.

R1-2501877	Spreadtrum, UNISOC

Proposal 1: For the determination of the value of Z and X, the same mechanism as DCP can be reused. Proposal 2: Activation/deactivation of LP-WUS monitoring by gNB L1 signaling (e.g., DCI) can be considered. Proposal 3: For the case when UE is configured with CA without dual DRX groups, the serving cell on which the LP-WUS is configured can be RRC configured cell other than PCell. Proposal 4: For the case when UE is configured with NR-DC with CA without dual DRX groups or without CA, the serving cell on which the LP-WUS is configured can be RRC configured cell other than Pcell/PSCell. Proposal 5: During Cell DTX inactive time, the UE is expected to monitor LP-WUS. Proposal 6: Separate indication/configuration for QCL source of LP-WUS should be supported.

R1-2502001	CATT

Proposal 1: The minimum time gap of LP-WUS needs to take into account the time of MR in preparation of channel tracking for PDCCH coherent demodulation when LP-WUS triggers UE wakeup. Proposal 2: For RRC CONNECTED mode, support 2 candidate value for the UE capability report on the minimum time gap with considering at least {20ms}. Proposal 3: For LP-WUS MO in for UE in CONNECTED mode, the periodicity and offset is configured in slot-level and the first symbol for LP-WUS MO in a slot can be determined by an RRC parameter. Proposal 4: For Option 1-1, 1, Z can be larger than 1 and configured by RRC. Proposal 5: X is the value of UE capability report with ‘ms’ unit. Proposal 6: In Option1-2, support multiple LP-WUS MOs per periodicity and the definition of time offset4 is related with the LP-WUS MO window. Proposal 7: One LP-WUS is configured to be QCLed with the RS associated with each of the TCI states of the CORESETs in order to trigger the PDCCH monitoring at the associated CORESETs when LP-WUS is detected. Proposal 8: For the case when UE is configured with NR-DC and/or CA without dual group in RRC CONNECTED mode, LP-WUS can be configured in the PCell or on the SpCell per cell-group. Proposal 9: For Rel-19 LP-WUS, the dual DRX group and LP-WUS cannot be configured simultaneously. Proposal 10: A fallback mechanism to the legacy behavior of PDCCH monitoring can be specified to battle the MDR impact to the PDCCH decoding for UEs in CONNECTED mode.

R1-2502009	Nokia

OBSERVATIONS Observation 1: For options 1-1 and 1-2, having multiple MOs that can trigger the same onDuration period, is an effective mechanism to handle collisions and reduce service latency. Observation 2: To increase the network opportunity to multiplex LP-WUS for different UEs, it is preferable to minimise the number of options for the minimum time gap available for UEs. Observation 3: The total amount of time required to wake up all the radios in a CA scenario to a state when they can all monitor PDCCH, can for certain CA configurations and UE implementations, be longer than it takes to wake up a single radio for a single cell. Observation 4: TS38.214 specifications require updates to reflect that LP-WUS can affect the periodic CSI/L1-RSRP reporting. Observation 5: Legacy DCI format 2_6 based WUS can be monitored outside active time of cell DTX. Observation 6: For CONNECTED mode, LP-WUR time and frequency synchronisation can be provided by the MR. PROPOSALS Proposal 1: For options 1-1, X corresponds to the configured minimum time gap. Proposal 2: For option 1-1, if more than one MO is configured for triggering PDCCH monitoring of the same onDuration, those MOs are restricted to the period/window between time offset 2 and the time set by X. FFS: Which MOs within this time window the UE monitors, e.g. all or the first few defined by a rule/parameter. Proposal 3: For option 1-1, the maximum number of MOs the UE is expected to monitor for a given onDuration is [4]. Proposal 4: For option 1-1, the UE is expected to blindly monitor all the MOs within the window. Note: The network can select which MO(s) are used. Proposal 5: For option 1-2, new parameters time_offset6 and option1-2_periodicity define where LP-WUS triggered onDurations can start. Proposal 6: For option 1-2, the UE starts monitoring for LP-WUS at a time-offset5 prior to the slot to when the onDurationTimer can start. Proposal 7: For option 1-2, the UE is not required to monitor LP-WUS for time-offset4 prior to the slot when the onDurationTimer can start. FFS: If this is the same as the minimum time gap Proposal 8: For option 1-2, the time between time-offset5 and time-offset4, corresponds to a time window where the UE is expected to monitor all MOs. Proposal 9: For option 1-2, the maximum number of MOs the UE is expected to monitor for a given onDuration is [4]. Proposal 10: For option 1-2, the UE is expected to blindly monitor all the MOs within the window. Note: The network can select which MO(s) is uses. Proposal 11: The number, X, of candidate minimum time gap values, in a UE capability report, is limited to 2. Proposal 12: The candidate values for the minimum time gap, are not varied for different SCS. Proposal 13: The candidate values for the minimum time gap, are not varied for different WUR types. Proposal 14: RAN1 determine if the singe cell candidate set of minimum time gap values is sufficient for CA scenarios. Proposal 15: The UE can use the UAI procedure to signal any value from the full candidate set. Proposal 16: For NR-DC and/or CA scenarios without dual-drx, LP-WUS can be configured on any serving cell within a cell group with an active DL BWP. Proposal 17: For NR-DC configuration with or without dual-drx, a new UE capability/capability component, is supported to indicate to the network the UEs ability to monitor 2 LP-WUS signals on separate component carriers. Proposal 18: The network identifies a specific PDCCH coreset in the active BWP that the LP-WUS is using, for the UE to determine the QCL source to apply for the LP-WUS (option 2). Proposal 19: LP-WUS can be monitored outside the cell DTX Active time. Proposal 21: For CONNECTED mode, there is no change to how existing MR based measurements (RRM/RLM/BFD/CSI) are determined when LP-WUS is enabled. Proposal 22: Option 1-3 is deprioritised.

R1-2502060	Panasonic

Proposal 1: During the time when UE is not in C-DRX active time according to drx-HARQ-RTT-TimerDL or drx-HARQ-RTT-TimerUL, UE is not required to monitor LP-WUS. Proposal 2: For the reported values, Larger SSB periodicities than 20 ms should be also assumed. Proposal 3: For the candidate values that can be indicated by UAI, the possible cases are: - The definition of the candidate values indicated by UAI is similar as the definition of the reported value via the UE capability report. - The definition of the candidate values indicated by UAI can be separate from the reported value via the UE capability report and frequently changed depending on the situation. Proposal 4: Opt2 should be supported to define/configure/indicate QCL source of LP-WUS. Proposal 5: QCL Type C should be applied for RRC CONNECTED mode. Proposal 6: MR PDCCH monitoring triggered by SR, PRACH, and CG-PUSCH can also be applied for Option 1-2 of LP-WUS monitoring procedure. Proposal 7: How to avoid the collision between PDCCH monitoring triggered by SR, PRACH, and CG-PUSCH and PDCCH monitoring triggered by LP-WUS should be considered for both Option 1-1 and 1-2 of LP-WUS monitoring procedure.

R1-2502077	NEC

Proposal 1: support LP-WUS in combination with DCP to further reduce power consumption without introducing too much overhead.

R1-2502169	CMCC

Proposal 1. For the candidate values of minimum time gap: - If X=2 is supported, the candidate values of minimum time gap are {3, 10}ms. - If X=3 is supported, the candidate values of minimum time gap are {0/1, 3, 10}ms. - Above candidate values are applicable for different SCSs, and there is no need to introduce separate values for OOK-based WUR and OFDM-based WUR. Proposal 2. Regarding QCL information for LP-WUS, support: - Opt 1: Separate indication/configuration for QCL source of LP-WUS in previous agreement. - SSB and/or CSI-RS can be the QCL source of LP-WUS QCL type(s) is Type C, and Type D when applicable. Proposal 3. Support not to introduce additional activation/deactivation mechanism for LP-WUS monitoring. Proposal 4. Regarding LP-WUS monitoring in CA/DC, support: - LP-WUS is configured only in PCell/PSCell. - For CA/DC with dual DRX groups, LP-WUS can be separately configured for each DRX cell group. Observation 1. LP-SS occasion/LP-WUS MO may go across multiple slots and contiguous in time. Moreover, a LO/LP-SS burst may also contiguous in time. Proposal 5. RAN1 to discuss the UE behaviour when LP-SS/LP-WUS transmission overlap with existing pattern (e.g. UL slot/symbol, common signal transmission, configuration related to L1/L3 measurement), or when the time interval between LP-SS/LP-WUS transmission and existing pattern is smaller than the RF retuning time (e.g. when LR/MR is deployed in FR2, or when SCS of PRACH is equal or larger than 15kHz).

R1-2502239	Huawei, HiSilicon

- DCI format 2_6 is monitored outside active time of cell DTX, which leaves gNB the flexibility to wake up UE earlier. - Monitoring LP-WUS also outside active time of cell DTX has marginal impact on power saving gain, since the power consumption for LP-WUS monitoring is expected to be low. - For the purpose of future discussion, RAN1 assumes at least the following as baseline. - LP-WUR is NOT able to receive signal(s) during the time where MR transmits/receives signals/channels. - If LP-WUS monitoring collides with measurements and/or periodic CSI/L1-RSRP transmission, measurements are prioritized. - If LP-WUS monitoring collides with SPS/CG resources, SPS/CG reception/transmission is prioritized. - FFS how to let gNB and UE have the common understanding on where the UE performs measurement. - FFS: additional consideration of simultaneous operation between LP-WUR and MR. - FFS: UE capability. - The minimum time gap reported by the UE capability is defined between the end of the detected LP-WUS/the monitored last LP-WUS MO and the start of the PDCCH monitoring in the new timer or on duration timer respectively for Option 1-1 and Option 1-2. - For the X = 3 candidate values for the UE capability report on the minimum time gap for each SCS, consider {2, 30, 42} ms - For reported value of 30ms and 42ms, the reported values assume SSB periodicity of 20ms and ensures SSB(s) before PDCCH reception and during time gap - For larger SSB burst periodicity than 20ms, the translation of delay wake-up is used; For 5ms and 10ms SSB periodicity, the minimum value of 42ms/30ms assuming 20ms SSB periodicity should be kept; - The same set of candidates are used for different SCSs - Same values are used for OOK-based WUR and OFDM-based WUR - The case of CA should be considered and the minimum time gap is the same between CA case and non-CA case - Candidate values for UAI are same as candidate values for UE capability - UE does not expect gNB to configure a time offset that is shorter than the duration reported from the X candidate values, which already considers the time duration required by synchronization: - The value indicated by UAI must be larger than the duration reported from the X candidate values, which already considers the time duration required for synchronization; - If a LP-WUS triggering a PDCCH monitoring received by the UE ends at symbol n, the UE is not expected to monitor a PDCCH before symbol n+Y where Y is the configured time offset. - In CONNECTED mode, maximum number of LP-WUS information bits is up to 16 bits. - In CONNECTED mode, if LP-WUS monitoring is enabled, at least the following options are further supported. - Deactivation of LP-WUS monitoring based on timer. - Note: Activation of LP-WUS monitoring based on timer has been supported by the agreement for UE behavior Option 1-2 in RAN1#118 and the agreement in RAN2 #127bis. Activation of LP-WUS monitoring by gNB L2 signaling has been supported by the agreements in RAN1#119 and RAN2#128. And Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission has been supported in RAN1#120. - Note: PDCCH monitoring of a UE is suspended if LP-WUS monitoring of the UE is activated, and vice versa. PDCCH monitoring of a UE is resumed if LP-WUS monitoring of the UE is deactivated. - The impact due to miss-detection/falsely-detection of LP-WUS activation/deactivation signaling should be minimized/avoided for CONNECTED mode UEs by e.g. explicit feedback signaling from UE to gNB, and the explicit feedback signaling can reuse SR. - For the LP-WUS monitoring occasion configuration - For Option 1-1, - UE monitors all the MOs between the time point determined by Time offset2 and the time point determined by X. - Time offset2 is configured to be no smaller than the minimum wake-up delay reported by UE capability and UAI (optional). - X is reported by UE capability. - For Option 1-2, - Time offset4 is configured to be no smaller than the minimum wake-up delay reported by UE capability and UAI (optional). - The basic capability for the maximum number of codepoints checked per MO by a UE is 2. - If DCP is configured, after LP-WUS monitoring is activated, the UE should not monitor DCP in MR. - After LP-WUS monitoring is activated, the UE should stop/suspend the Rel-17 PDCCH adaption mechanism, if configured. - LP-WUS monitoring outside legacy cell DTX active time according to the LP-WUS monitoring configuration can trigger the PDCCH monitoring. - When UE is configured with CA, LP-WUS monitoring is configured on one cell, where the cell can be either PCell or SCell. - If LP-WUS is configured to be monitored on a SCell for a UE, the UE expects that the SCell is not deactivated. - For the case when UE is configured with CA with dual DRX groups in RRC CONNECTED mode, - LP-WUS can be configured on a serving cell in default DRX group. - LP-WUS indication is applicable to all serving cells - UE does not monitor LP-WUS during C-DRX active time, where the C-DRX active time is defined as - For option1-1, the C-DRX active time is defined as the union set of the legacy C-DRX active time from both DRX groups - For option1-2, the C-DRX active time is defined as the union set of the legacy C-DRX active time from both DRX groups, as well as the running time of a single new timer for both DRX groups. - Support association of QCL source between LP-WUS and PDCCH/CORESET(Opt2). - If LP-WUS monitoring is activated by PDCCH, UE can assume the same QCL source as the CORESET to receive the PDCCH activating LP-WUS monitoring. - If LP-WUS monitoring is activated by timer, the QCL source can be the same as the CORESET to receive the last PDCCH before the timer expires. - Capture the parameter in appendix in the RRC parameter list. - In CONNECTED mode, monitoring parameters of LP-WUS can be adjusted by MR signaling and/or pre-defined rules at least for different coverage requirement.

R1-2502259	OPPO

Proposal 1: A UE can report supported number of detecting codepoints for monitoring wake-up indication in CONNECTED mode, from 2 candidates {4, 8}. A UE can be indicated by LP-WUS codepoints, in which each codepoint can be corresponding to one or more UEs in CONNECTED mode. UE in CONNECTED mode is UE-specifically configured with the LP-WUS resources and codepoints. Proposal 2: For RRC CONNECTED mode and Option 1-1/1-2, the LP-WUS monitoring occasions use one set of parameters for periodicity and offset. For option 1-1, configured Z as a number of detecting MO started from configured offset2 slots before the start of drx-onDurationTimer. For option 1-2, after configured offset4 slots from positive detection of wake-up the new timer would start. Proposal 3: When Cell DTX is configured with LP-WUS, LP-WUS is assumed to be disabled during the Cell DTX inactive time. Proposal 4: When the LP-WUS is configured with CA in CONNECTED mode, only PCell/PSCell can be configured with LP-WUS. Proposal 5: For the LP-WUS activation and deactivation by pre-configured condition(s), timer-based monitoring of LP-WUS is the baseline. Proposal 6: RAN1 can assume that MR measurements are one of the UE assistant information for the LP-WUS activation and deactivation. RAN1 discuss the introduction LP-SS measurement in CONNECTED mode as another UE assistant information. Proposal 7: For the LP-WUS the LR/MR activation assumption may be needed. We are fine for concluding the design assumption with clear definition of LR and MR behavior. The specification impact is to be further discussed. Proposal 8: LP-WUS Monitoring Occasions can be associated with CORESETs for the TCI states determination. The activated TCI state of the CORESET is applied for the associated LP-WUS Monitoring Occasion. SSB and/or CSI-RS is the QCL source of LP-WUS and the QCL type(s) is Type A, and Type D when applicable Proposal 9: In RRC CONNECTED mode, support UE capability report one value among 3 candidate values for minimum time gap between LP-WUS reception and MR to start PDCCH monitoring. 20 ms, 6 ms and 2 ms are the candidature values. UE could additional report another smaller gap than its capability report as UAI. Proposal 10: RAN1 may specify that the monitoring occasion of LP-WUS and associated monitoring occasion of PDCCH should meet the reported minimum gap.

R1-2502304	InterDigital, Inc.

Observation 1: LP-WUS monitoring configuration should be selected to match the conditions UE is experiencing. Observation 2: Higher power saving can be achieved by activating LP-WUS monitoring only when UE is in appropriate conditions. Observation 3: gNB is unable to select all the appropriate LP-WUS monitoring configuration and activate/deactivate LP-WUS monitoring without UE assistance. Observation 4: Supporting dual DRX groups with RRC CONNECTED mode LP-WUS monitoring can reduce achievable power saving gains through LP-WUS monitoring and impact LP-WUS coverage. Observation 5: Compact LP-WUS payload size is important to support coverage comparable with PUSCH for Msg3. Observation 6: LP-WUS payload can be reduced by implicitly indicating some of information of LP-WUS. Observation 7: Time/frequency synchronization of MR is not impacted by RRC CONNTECTED mode LP-WUS monitoring. Observation 8: Configuration of periodic CSI/L1-RSRP reporting mechanism can be inefficient in case of long inactive durations. Proposal 1: PDCCH monitoring triggered by LP-WUS is enabled/disabled or activated/deactivated with UE assistance. Proposal 2: For a LP-WUS monitoring enabled UE, support activating/deactivating LP-WUS monitoring based on following preconfigured conditions. - For activating LP-WUS monitoring: level of UE activity (e.g., timer and/or counter associated with UE activity), channel/signal quality, mobility. - For deactivating LP-WUS monitoring: level of UE activity (e.g., timer and/or counter associated with UE activity), channel/signal quality, mobility, LP-WUS reception quality. Proposal 3: For OFDM based LP-WUR, both NR-SS (existing SSB) and LP-SS can be used to determine activation/deactivation of LP-WUS monitoring. Proposal 4: Introduce a mechanism to report UE decision of LP-WUS monitoring deactivation potentially with reason for the decision. Proposal 5: A UE is not configured with dual DRX groups and LP-WUS monitoring simultaneously. Proposal 6: For RRC CONNTECTE mode LP-WUS monitoring, when UE is configured with CA, LP-WUS is only configured on the PCell. Proposal 7: For RRC CONNTECTE mode LP-WUS monitoring, when UE is configured with NR-DC, LP-WUS is only configured on the PCell/PSCell. Proposal 8: A subset of LP-WUS information can be informed to UE implicitly using LP-WUS monitoring resource (e.g., LO and MO) association and structure of LP-WUS. Proposal 9: For RRC CONNECTED mode LP-WUS monitoring, translation of reported minimum time gap for different SSB periodicities is not supported. Proposal 10: In RRC CONNECTED mode LP-WUS monitoring, for minimum time gap between LP-WUS reception and MR to start PDCCH monitoring, different sets of candidates are not supported for different SCSs. Proposal 11: In RRC CONNECTED mode LP-WUS monitoring, for minimum time gap between LP-WUS reception to starting PDCCH monitoring, different sets of candidates are used for OOK-based WUR and OFDM-based WUR. Proposal 12: In RRC CONNECTED mode LP-WUS monitoring, for minimum time gap between LP-WUS reception to starting PDCCH monitoring, different set of candidates or reported values are not defined based on supporting/not supporting CA. Proposal 13: For RRC CONNECTED mode LP-WUS monitoring, introduce a mechanism to support CSI/L1-RSRP reporting triggered when inactivity periods exceed a preconfigured threshold. Proposal 14: LP-SS or LP-WUS is QCLed with one SSB and/or CSI-RS in QCL Type C and QCL Type D (when applicable). Proposal 15: LP-SS is supported for RRC CONNECTED mode LP-WUS monitoring. Proposal 16: LP-WUS is QCLed with LP-SS in QCL Type A and QCL Type D (when applicable). Proposal 17: For LP-WUS monitoring in RRC CONNECTED mode, Option 1 is supported for defining/configuring/indicating QCL source of LP-WUS. - Option 1: Separate indication/configuration for QCL source of LP-WUS. Proposal 18: UE is not expected to monitor for LP-WUS during Cell DTX inactive time.

R1-2502324	Sony

Proposal 1: The first Z LP-WUS MO(s) is determined based on the number of MO(s) at or after Time offset2 (and before the switching time-gap). Proposal 2: the time-gap duration: X [slots/symbols] is determined based on the UE capability and also may depend on the numerology. Proposal 3: Support LP-WUS option 1-2 with the configuration with multiple MOs per LP-WUS periodicity. Proposal 4: The time offset 4 is the time offset from the LP-WUS MO reception to the starting of On-duration Time offset4 should be greater than the time-gap with X [slot/symbol].

R1-2502377	Samsung

Proposal 1: For Option 1-1: - Support a configurable value of Z by RRC; - A UE assumes the same indication by LP-WUS in the Z MOs. Proposal 2: For Option 1-2: - Support one or multiple (denoted by Z) MOs per periodicity; - Support a configurable value of Z by RRC; - Support a configurable time interval between two consecutive MOs by RRC; - A UE assumes the same indication by LP-WUS in the Z MOs; - The UE expects the time offset4 is no less than the minimum wake-up delay; - The UE may not need to monitor MOs within the time offset4 after the reception of LP-WUS indicating wake-up. Proposal 3: For the time gap between LP-WUS reception and MR to start PDCCH monitoring: - Support two set of minimum gaps (i.e., X=2) per SCS and per LP-WUR type; - Candidate values for the indicated preferred gap value by UAI shall be same as the candidate values for the UE capability reporting. Proposal 4: For QCL source of LP-WUS: - If a dynamic adaptation of the QCL source for LP-WUS is not needed, the QCL source for LP-WUS can be provided to the UE as part of the configuration for LP-WUS; - If a dynamic adaptation of the QCL source for LP-WUS is needed, the QCL source for LP-WUS can follow the latest indication by DCI format using the existing TCI framework. Proposal 5: For CA with two DRX groups: - LP-WUS can be configured on a serving cell for both Option 1-1 and Option 1-2; - For Option 1-1, LP-WUS carries separate indications for each DRX group(s) to trigger PDCCH monitoring of all serving cells; - For Option 1-2, LP-WUS carries a single indication to trigger PDCCH monitoring of all serving cells.

R1-2502450	Xiaomi

Proposal 1: Support multiple transmission occasions for a LP-WUS MO. Proposal 2: To support LP-WUS configuration in NR-DC, UE should be able to receive LP-WUS on two carriers simultaneously by LR. Proposal 3: For NR-DC, if UE does not have the capability to receive the two LP-WUS simultaneously on two cells, gNB should only configure LP-WUS on MCG or on SCG. Proposal 4: For the case when UE is configured with CA without dual DRX groups in RRC CONNECTED mode, LP-WUS is only configured on Pcell. Proposal 5: In CA with dual DRX group, wake-up indication is per DRX group. Proposal 6: For CA with dual DRX group, the following alternatives for LP-WUS resources configuration can be considered, Alt 1, configured separately in each DRX group; Alt 2, configured only on Pcell. Proposal 7: For LP-WUS MOs in connected mode for Option 1-1, Z and X are configured by gNB. The ending time of the Z MOs is not later than X prior to the beginning of a slot where the UE would start the drx-onDurationTimer. Proposal 8: For LP-WUS MOs in connected mode for Option 1-2, time offset 4 is configured by gNB. Proposal 9: For RRC CONNECTED mode, there are at least 3 candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. Proposal 10: For RRC CONNECTED mode, synchronization delay is not included in the wake-up delay. Proposal 11: For the UE capability report on the minimum time gap for each SCS, consider {0.25, 0.5, 1, 2, 3, 6, 10}ms. Proposal 12: For OOK-based LR and OFDM-based LR, the minimum time gap reported by UE for each SCS is the same. Proposal 13: If the UE receives a LP-WUS indicating PDCCH monitoring, and the ending time of LP-WUS is just before or overlapped with SPS PDSCH in time domain, the wake-up delay needed before starting PDCCH monitoring can be 0. Proposal 14: LP-WUS in connected state can be QCLed with LP -SS by QCL type A, and can be QCLed with SSB by QCL type C. Proposal 15: For RRC connected, the same behaviour as periodic CSI/L1-RSRP report in both Option 1-1 and Option 1-2 can be applied to periodic/semi-persistent SRS transmission. Proposal 16: For RRC connected, for a UE with basic capability, if BWP switching happens and leads to the case that LP-WUS resource is outside of the new DL active BWP, UE will fall back to legacy C-DRX operation behaviors for both Option 1-1 and Option 1-2.

R1-2502483	LG Electronics

Proposal #1: For LP-WUS MOs in connected mode for Option 1-1, - time offset2 is the same as ps-offset or can be configured as a separate value. - a number of consecutive slots that MO lasts per periodicity can be introduced to define Z. Proposal #2: For LP-WUS MOs in connected mode for Option 1-2, - multiple MOs per periodicity should be considered for scheduling flexibility. - time offset4 is defined as the time gap between the reference MO and the starting of new timer. Proposal #3: Multiple MOs per periodicity should be considered for both Option 1-1 and Option 1-2, and the unified design should be adopted. - The number of MOs per periodicity is defined considering FAR. Proposal #4: Support that the UE reports two values on the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring. - Different values are selected for OOK-based WUR and OFDM-based WUR. Proposal #5: For LP-WUS monitoring in RRC CONNECTED mode, support separate indication/configuration for QCL source of LP-WUS - Re-use existing TCI-state framework to indicate TCI state of LP-WUS. - If TCI-state is configured but not indicated, a UE assumes that LP-WUS is QCLed with SSB that the UE recently received for random access or initial access. Proposal #6: For the case when UE is configured with NR-DC and/or CA with dual DRX groups in RRC CONNECTED mode, - LP-WUS is configured on a serving cell per cell-group (not per DRX group). - LP-WUS indication is applicable to all activated serving cells in the cell-group. Proposal #7: When LP-WUS is configured with Cell DTX, discuss whether the UE is expected to monitor LP-WUS during Cell DTX inactive period by considering whether the time while the new timer is running is included in cell DTX active period or not. Proposal #8: Support UE fallback to PDCCH monitoring, - when the channel quality for a certain time duration and/or a certain number of times is below the threshold. - when the UE does not receive LP-WUS for a certain time duration. Proposal #9: Do not support UE fallback by implementation without any condition and additional implicit/explicit indication from UE when it falls back.

R1-2502517	ETRI

Proposal 1: It is proposed to support only duty-cycled monitoring mode for LP-WUS in Rel-19. Proposal 2: For the purpose of future discussions, RAN1 assumes at least the following - For RRC connected mode, - LP-WUR and MR cannot support simultaneous operation at a given time instance. - FFS: additional considerations for simultaneous operation between LP-WUR and MR Proposal 3: For UE capability report on the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring - For the X = 2 candidate values for the UE capability report on the minimum time gap for each SCS, consider {0.25, 0.5, 1, 2, 3, 6, 10, 20, 30, 40}ms - Support different candidate values per SCS - Not support different values for OOK-based WUR and OFDM-based WUR and support candidate values only for OOK-based WUR - Not support special consideration for the case of CA Proposal 4: It is proposed to configure the QCL source in advance as in Opt1, as part of the LP-WUS configuration. - Opt1: Separate indication/configuration for QCL source of LP-WUS Proposal 5: It is proposed to support the use of PDCCH skipping and monitoring adaptation mechanisms as a means to trigger LP-WUS monitoring for the UE. The following two options can be considered: - Option 1: Utilize the reserved ‘11’ value of the 2-bit PDCCH monitoring adaptation field in the DCI as an explicit indication for the UE to terminate the DRX active time and transition to the sleep state for LP-WUS monitoring. - Option 2: Support an approach where the existing PDCCH skipping mechanism is utilized to indicate the UE to start LP-WUS monitoring without terminating the DRX active time. In this case, the UE may perform LP-WUS monitoring during the skipping duration and return to PDCCH monitoring upon LP-WUS detection. Proposal 6: It is proposed to apply the conclusion that uplink transmissions such as SR, PRACH, and CG-PUSCH trigger MR PDCCH monitoring by deactivating LP-WUS monitoring to both Option 1-1 and Option 1-2. Proposal 7: It is proposed to support fallback mechanisms that allow the MR to switch to legacy C-DRX operation when the channel condition of LP-WUS is not sufficient, such as when it falls below a pre-configured threshold. Proposal 8: It is proposed to define an indication mechanism for the UE to notify the gNB when fallback from LP-WUS monitoring to PDCCH monitoring is triggered due to poor channel conditions. - FFS for detail mechanism Proposal 9: It is proposed not to require explicit indication from the UE to inform the gNB of successful LP-WUS monitoring, except for the fallback case. Proposal 10: It is proposed to adopt Option 3 for the CA with dual DRX groups case. - Opt.3 - LP-WUS can be configured on a serving cell - LP-WUS indication is applicable to all serving cells of all DRX groups. - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation

R1-2502576	TCL

Observation 1: For the minimum time gap needed for different sleep states, it can be indicated by UAI. Observation 2: The current agreement implies that LP-WUS monitoring can be deactivated based on C-DRX timers. Proposal 1: Two candidate values, i.e., X=2, can be the baseline. Proposal 2: The preference offset indicated by UAI is not smaller than the value reported by UE capability signaling. Proposal 3: Based on the preference offset indicated by UAI, gNB configures time offset between the end of LP-WUS reception and the start of PDCCH monitoring. Proposal 4: the value of X should be satisfied with the minimum time gap reported by UE capability signaling and the time offset between the end of LP-WUS reception and the start of PDCCH monitoring indicated by gNB (if the preference offset is indicated by UAI). Proposal 5: For option 1-1, the value of Z can be one or multiple. Proposal 6: For option 1-2, one or multiple LP-WUS MO(s) can be configured within a periodicity of LP-WUS. Proposal 7: For option 1-2, the value of time offset4 should be satisfied with the minimum time gap reported by UE capability signaling and the time offset between the end of LP-WUS reception and the start of PDCCH monitoring indicated by gNB (if the preference offset is indicated by UAI). Proposal 8: For option 1-2, the existing timer (e.g., drx-onDurationTimer or drx-InactivityTimer) duration can be used for the length of the new timer. Proposal 9: For Option 1-2, support that SR, PRACH and CG-PUSCH triggers MR PDCCH monitoring according to the legacy UE behavior. Proposal 10: Deactivation of LP-WUS monitoring can be based on: - Timer, e.g., if the UE doesn’t detect a LP-WUS before the expiration of the timer - UL signaling including SR, PRACH, CG PUSCH. Proposal 11: When Cell DTX is configured and activated, UE monitors LP-WUS during the active time of cell DTX (i.e., celldtxdrx-onDurationTimer) and does not monitor LP-WUS during the non-active time of Cell DTX.

R1-2502617	Apple

LP-WUS monitoring occasions Observation 1: Z is important to determine the FAR probability and should be jointly considered with the number of codepoint a UE monitors as well as the UE grouping. Proposal 1: Z is configured in the LP-WUS MO configuration, if not configured, UE monitors all the LP-WUS MO at or after Time offset2 until the X [slots/symbols] prior to the beginning of a slot where the UE would start the drx-onDurationTimer. Proposal 2: For Option 1-2, support configuring multiple LP-WUS MOs in one periodicity, which can be consecutive in time domain. Proposal 3: Offset 3 is the starting slot of the first MO in a periodicity, offset 4 indicates the time gap of the last MO and the start of the new timer triggered by LP-WUS. QCL source of LP-WUS Proposal 4: Support Option 1: Separate indication/configuration for QCL source of LP-WUS. LP-WUS operation with CA/DC Proposal 5: LP-WUS can be configured in only one cell group when DC is configured. Configuring LP-WUS on both cell groups is subject to another UE capability. Proposal 6: For LP-WUS Option 1-2 when UE is configured with dual DRX group, consider the following three alternatives: - Alt 1: Similar to legacy C-DRX with dual DRX group, the same periodicity and starting offset is configured for the LP-WUS triggered new timer, while the duration can be different. - Alt 2: The same LP-WUS triggered timer is applied on both DRX groups. - Alt 3: LP-WUS Option 1-2 is not supported with CA with dual DRX group. Proposal 7: Support monitoring LP-WUS in only one serving cell when dual DRX group is configured. Proposal 8: Support UE monitoring LP-WUS in SpCells. Proposal 9: Support Option 1 for the case when UE is configured with CA with dual DRX groups in RRC CONNECTED mode, at least for LP-WUS procedure Option 1-1 - LP-WUS can be configured on a serving cell per cell-group - LP-WUS carries separate indications for each DRX group(s) to trigger PDCCH monitoring of all serving cells - A UE monitors 2 codepoints for triggering dual DRX group, details FFS - Alt 1: One codepoint indicates triggering of a UE’s 1st DRX group, the other indicates triggering of the 2nd DRX group - Alt 2: One codepoint indicates triggering of the UE’s 1st DRX group/DRX group with PCell/DRX group with LP-WUS, the other codepoint indicates triggering of both DRX groups. Minimum time gap Proposal 10: Consider X = 3 candidate values for the UE capability report on the minimum time gap for each SCS, consider the following options: - Option 1: {6, 13, 33}ms; - Option 2: {6, 20, 40}ms. Proposal 11: For the preferred configured time offset between LP-WUS and PDCCH reception using UAI mechanism, it is up to UE to report a larger or a smaller value than that reported by UE capability. Coexistence with cell DTX Proposal 11: During Cell DTX non-active period, UE is not expected to monitor LP-WUS. Proposal 12: UE does not expect the case when all LP-WUS MOs are configured within cell DTX non-active period. UE fallback behavior Proposal 13: Support UE fall back to PDCCH monitoring: - For Option 1-1, UE falls back to legacy C-DRX behaviors - For Option 1-2, the following alternatives can be considered: - Alt 1: UE falls back to C-DRX behavior, and indicates the fallback state to NW - Alt 2: UE falls back to PDCCH monitoring in the duration defined by the LP-WUS triggered new timer

R1-2502677	Sharp

Proposal 1: Confirm the working assumption on using codepoint-based LP-WUS information for connected UE. Proposal 2: The basic UE capability is to detect at least 2 codepoints per MO. Proposal 3: Support LP-WUS resource configured in PCell when UE is configured with CA. Proposal 4: Support Option 1, which provides separate indications for one or more serving cells within an LP-WUS. Proposal 5: Support timer-based LP-WUS monitoring activation and deactivation for UEs in connected mode.

R1-2502713	MediaTek Inc.

Observation 1: The UE’s active state takes priority, effectively bypassing LP-WUS whenever primary communications or measurements are ongoing, thus preventing conflicts or interference. Proposal 1: The specification should clearly mandate that LP-WUS monitoring is skipped whenever the main receiver is active, ensuring the low-power receiver never disrupts primary operations while preserving maximum power-saving benefits. Observation 2: The fallback requirement is most critical for Option 1-2, especially 1-2-2, where the UE otherwise relies solely on LP-WUS. Clear UE/network signaling of wake-up timing constraints helps ensure reliability. Proposal 2: RAN1 further discuss and decide the following: - A periodic PDCCH check if the UE relies solely on WUS (e.g., Option 1-2-2), configured via RRC. - Explicit UE capability/advisory signals about fallback needs and acceptable timings. - A parallel DRX onDuration as safety net for scenarios demanding higher reliability. Observation 3: Dual/multiple-group scenarios necessitate flexible LP-WUS configurations (e.g., one WUS per DRX group), yet simplicity in signaling and consideration of UE capability (single vs. multiple WUS monitoring) remain key factors. Proposal 3: RAN1 to further discuss and decide the following: - Use one LP-WUS per DRX domain, typically on the PCell for single-group scenarios. - In dual-group setup, configure separate WUS per group to reduce unnecessary wake-ups. - Define a UE capability bit for multiple simultaneous WUS monitoring, ensuring that UEs lacking this feature only receive a single WUS configuration. Observation 4: Overlaps with UL or DL transmissions can block WUS reception, but existing awake states or exclusion windows help mitigate these conflicts. Proposal 4: Establish clear scheduling restrictions, guard intervals, and priority rules to ensure LP-WUS does not interfere with essential UL or DL operations. If overlaps occur, prioritize normal NR procedures; skip or postpone WUS and rely on fallback as necessary, thereby preserving network reliability and power-saving benefits.

R1-2502775	NTT DOCOMO, INC.

Proposal 1: - For LP-WUS in RRC CONNECTED mode in Rel-19, following is not supported - Option 1-3: LP-WUS monitoring inside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. Proposal 2: - For LP-WUS in RRC CONNECTED mode in Rel-19, enhancement of PDCCH skipping is not supported to terminate the DRX active time and start LP-WUS monitoring Proposal 3: - For RRC CONNECTED mode, the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring - does not include the duration for time/frequency synchronization of MR - does not have different capability for CA - has X=3 candidate values corresponding to {Y+0, Y+6, Y+20} ms, which are defined in the unit of “slots” and scaled with SCS - FFS: common value of Y Proposal 4: - For LP-WUS MOs in connected mode for Option 1-1 - UE monitors the first Z LP-WUS MO(s) at or after Time offset2. - Z is one or multiple MOs configured by RRC - UE is not required to monitor LP-WUS during the X [slots/symbols] prior to the beginning of a slot where the UE would start the drx-onDurationTimer. X is given by the minimum time gap reported as UE capability. Proposal 5: - For LP-WUS MOs in connected mode for Option 1-2 - LP-WUS MOs, including periodicity and time offset3, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s). - Introduce RRC parameter “Duration” to configure one or multiple MOs per periodicity - UE monitors LP-WUS in the LP-WUS MOs and, if triggered to wake up, starts a new timer for PDCCH monitoring triggered by LP-WUS, after a time offset4. - The time offset4 configured by the network indicating a time, after which the UE starts PDCCH monitoring via starting the new timer. - Time offset4 is configured so that the gap between the end of last MO in the duration and the start of new timer is not smaller than the minimum time gap Proposal 6: - Support TCI framework for CORESET as baseline for LP-WUS: - Rel-15 TCI framework: RRC configured K TCI states and MAC-CE activates one of them - Rel-17 unified TCI framework: The indicated TCI state (to legacy NR DL/UL channels/signals) is also applied to LP-WUS Proposal 7: - For the case when UE is configured with CA without dual DRX groups, LP-WUS can be configured on PCell or an activated SCell - For the case when UE is configured with NR-DC with CA without dual DRX groups, LP-WUS can be configured on PCell/PScell or an activated SCell - For the case when UE is configured with NR-DC without CA, LP-WUS can be configured on PCell/PScell Proposal 8: - For the case when UE is configured with CA with dual DRX groups in RRC CONNECTED mode, at least for LP-WUS procedure Option 1-1 - LP-WUS can be configured on PCell or an activated SCell per DRX group - LP-WUS indication is applicable to all serving cells in the DRX group - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - For LP-WUS procedure Option 1-2, send LS to RAN2 to ask whether the new timer can be configured per DRX group Proposal 9: - Support UE autonomous fallback to PDCCH monitoring when UE monitors LP-WUS - FFS the condition for the fallback - For LP-WUS operation Option 1-1, UE monitors PDCCH during drx-onDurationTimer - For LP-WUS operation Option 1-2, UE monitors PDCCH during drx-onDurationTimer and indicates it to gNB Proposal 10: - From RAN1 perspective, when LP-WUS monitoring is enabled for RRC CONNECTED mode, it is supported that SR, PRACH and CG-PUSCH triggers MR PDCCH monitoring according to the legacy UE behaviour - Above applies both for Option 1-1 and Option 1-2 - Send LS to RAN2 to ask for potential RAN2 spec impact. Proposal 11: - For RRC CONNECTED mode, when LP-WUS is configured with Cell DTX, irrespective of inside/outside Cell DTX inactive time, the UE is expected to monitor LP-WUS Proposal 12: - Consider the overlapping handling of PDCCH as baseline for the overlapping handling between LP-WUS reception and existing NR transmission/reception Proposal 13: - LPWUS_offset_CONNECTED is split into two parameters for Option 1-1 and Option 1-2 - Option 1-1: Time_offset_CONNECTED_Option1-1 - For LP-WUS operation Option 1-1, the time offset determines the start of LP-WUS monitoring relative to the start of drx-onDurationTimer. - This parameter shall be configured for LP-WUS operation Option 1-1 in RRC CONNECTED - Option 1-2: Time_offset_CONNECTED_Option1-2 - For LP-WUS operation Option 1-2, the time offset determines the start of UE PDCCH monitoring via starting the LPWUS_PDCCHMonitoringTimer after LP-WUS is detected. - This parameter shall be configured for LP-WUS operation Option 1-2 in RRC CONNECTED Proposal 14: - LPWUS_MO_CONNECTED is split into two parameters for Option 1-1 and Option 1-2 - Option 1-1: Time_MO_CONNECTED_Option1-1 - Configuration for LP-WUS MOs for Option 1-1, including at least the following: - Periodicity and time offset1 - Number of first MOs where UE monitors LP-WUS at or after Time_offset_CONNECTED_Option1-1. - This parameter shall be configured for LP-WUS operation Option 1-1 in RRC CONNECTED - Option 1-2: Time_MO_CONNECTED_Option1-2 - Configuration for LP-WUS MOs for Option 1-2, including at least the following: - Periodicity and time offset3. The configured periodicity is no larger than long C-DRX cycle - Duration to configure one or multiple MOs per periodicity - This parameter must be configured for LP-WUS operation option 1-2 in RRC CONNECTED

R1-2502807	Ericsson

Observation 1 OOK-based LP-WUS (with ≤2 slots duration) is less reliable than PDCCH for following cases - PDCCH 1Rx reception for PDCCH aggregation levels 8,16 - PDCCH 2Rx reception for PDCCH aggregation levels 4,8,16 - PDCCH 4Rx reception for PDCCH aggregation levels 2,4,8,16 Proposal 1 Considering the need to minimize overhead and latency, LP-WUS duration used for connected mode should be limited to at most 2 slots. Proposal 2 OOK-based LP-WUS in connected mode should only be used for triggering PDCCH monitoring for a limited set of PDCCH CCE aggregation levels. For example, - PDCCH aggregation levels 1,2,4 for PDCCH 1Rx reception - PDCCH aggregation levels 1,2 for PDCCH 2Rx reception - PDCCH aggregation level 1 for PDCCH 4Rx reception Proposal 3 UE PDCCH monitoring of at least certain configured PDCCH search space set(s) with higher ALs (e.g., used for scheduling RRC reconfiguration) should be independent of LP-WUS detection. Proposal 4 The number of candidate values for minimum time gap reporting X is 2. Different candidate values can be selected for different WUR types. However, only one set of min gap values should be reported for both single carrier and multi-carrier scenarios. Proposal 5 For Option 1-2, multiple MOs should be configured per periodicity (MO window) for scheduling flexibility. Proposal 6 Offset4 is an offset indicating a time where the UE starts PDCCH monitoring after the last slot of the MO window in which the UE detects LP-WUS. Proposal 7 Do not support UE autonomous switching between LP-WUS monitoring and no LP-WUS monitoring without gNB knowledge. Proposal 8 LP-WUS is configured only on PCell for CA without dual DRX groups and PCell/PSCell for NR-DC with CA without dual DRX groups or without CA in RRC CONNECTED mode. Proposal 9 Support Option2: Association of QCL source between LP-WUS and PDCCH/CORESET. Proposal 10 UE is not expected to monitor LP-WUS during Cell DTX active time.

R1-2502848	Qualcomm Incorporated

Proposal 1: - For Option 1-1, the Z is an RRC parameter provided by RRC signalling - If a UE is configured with Z > 1: - The Z LP-WUS MOs shall be associated with the same C-DRX cycle and provide consistent indication for the same C-DRX cycle of the UE. A UE does not expect to detect more than one LP-WUS with different indications for the given C-DRX cycle in the associated Z LP-WUS MOs. - The ‘X’ is the minimum time gap reported in the UE capability. Proposal 2: - For Option 1-2, support RRC parameter providing the value of Z - Z LP-WUS MOs are for triggering the new timer for PDCCH monitoring starting at the same slot - If a UE is configured with Z > 1: - The Z LP-WUS MOs shall be associated with the same slot where the new timer would start and provide consistent indication for the new timer. A UE does not expect to detect more than one LP-WUS with different indications for the new timer for the slot in the associated Z LP-WUS MOs. - The time gap from the last LP-WUS MO of the Z LP-WUS MOs associated with a slot where the new timer would start and the slot is not smaller than the minimum time gap ‘X’. Proposal 3: - Allow a UE to autonomously fallback from LP-WUS monitoring to C-DRX operation in both Option 1-1 and Option 1-2 - For Option 1-1, no matter whether the UE monitors the DCP before C-DRX active time or the UE fallbacks to the legacy C-DRX operation, the scheduling DCI within the C-DRX active time is receivable by the UE. - For Option 1-2, network can configure LP-WUS parameters such that the slots where the new MAC timer for PDCCH monitoring would start are in a superset of the slots where the drx-onDurationTimer would start. With this, at least some PDCCH monitoring windows are guaranteed, no matter whether the UE monitors LP-WUS or the UE fallbacks to the legacy C-DRX operation. Proposal 4: - UE does not monitor LP-WUS when MR is active outside C-DRX Active Time, e.g.: - the UE transmits SR, PRACH or CG-PUSCH outside C-DRX Active Time, or; - the UE monitors PDCCH outside C-DRX Active time (e.g., for CSS sets), or; - the UE is in measurement gap, or etc - For both Option 1-1 and Option 1-2, if all the LP-WUS MO(s), associated with a slot where PDCCH monitoring would start, are not monitored due to the MR activity, the UE starts drx-ondurationTimer or the new timer at the slot for PDCCH monitoring Proposal 5: - Support at least “10ms” as one of the X candidate values that a UE can report in the UE capability for all the SCSs. Proposal 6: - Allow a UE to report different values of the minimum time gap for OOK-based WUR and OFDM-based WUR - Allow a UE to report different values of the minimum time gap for non-CA and CA Proposal 7: - Support TCI-state configuration by RRC for LP-WUS - The TCI-state in the LP-WUS configuration indicates one or two DL RSs with the corresponding QCL types. - Support unified TCI framework for LP-WUS - If a UE is configured with dl-OrJointTCI-StateList, the UE obtains the QCL assumptions from a TCI state within the dl-OrJointTCI-StateList - The TCI state from which the UE obtains the QCL assumptions for LP-WUS is the one activated by MAC-CE activation command and is indicated by the codepoint of the DCI field “Transmission Configuration Indication” Proposal 8: - For a UE configured with NR-CA without secondary DRX group, LP-WUS can be configured on the PCell to trigger PDCCH monitoring on all the active serving cells. - For a UE configured with NR-CA with secondary DRX group, support following: - LP-WUS can be configured on the PCell to trigger PDCCH monitoring on all the active serving cells in the primary DRX group, and LP-WUS can be configured on an SCell in the secondary DRX group to trigger PDCCH monitoring on all the active serving cells in the secondary DRX-group. - RRC configuration for LP-WUS monitoring shall be provided to a secondary cell in the secondary DRX-group to enable this - FFS: LP-WUS can be configured on the PCell to trigger PDCCH monitoring on all the active serving cells in either or both of the DRX groups. Proposal 9: - For NR-CA, specify the minimum time gap as the time gap to wake up multiple active serving cells. - For NR-CA without secondary DRX group, the UE reports the minimum time gap to wake up all the active serving cells. - For NR-CA with secondary DRX group, the UE reports the minimum time gap to wake up all the active serving cells in the DRX group. Proposal 10: - Introduce MAC-CE for LP-WUS activation/deactivation - A UE can be configured with one or multiple LP-WUS configurations - The MAC-CE activates one (or none) of the LP-WUS configurations dynamically

RAN1 agreements

RAN1#116

Agreement

- For RRC CONNECTED mode, from RAN1 perspective, further study following LP-WUS procedures to trigger PDCCH monitoring:

  - Case 1: PDCCH monitoring is triggered by LP-WUS with C-DRX configuration

    - Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.

      - This option may replace DCP functionality

    - Option 1-2: LP-WUS monitoring outside C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

      - PDCCH monitoring possibly irrespective of drx-onDurationTimer

    - Option 1-3: LP-WUS monitoring inside C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

  - Case 2: PDCCH monitoring is triggered by LP-WUS without C-DRX configuration. LP-WUS can be monitored at any time according to the LP-WUS monitoring configuration

    - FFS duty-cycled and/or continuous LP-WUS monitoring

- Combination of options in Case 1 and combination of options in Case 1 and Case 2 are not precluded.

Agreement

For RRC CONNECTED mode, maximum number of LP-WUS information bits is up to X bits

- FFS value X, which is no more than [8 or 16]

Agreement

For RRC CONNECTED mode, minimum time gap between LP-WUS reception and MR to start PDCCH monitoring is introduced considering at least following

- LP-WUS processing time

- MR transition time for ramp up

- Time/frequency synchronization of MR

- FFS whether UE can report supported minimum time gap from candidate values

FFS: Whether the minimum time gap values can be more than one

Agreement

For RRC CONNECTED mode, from RAN1 perspective,

- PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling

  - FFS whether to support UE assistance.

- LP-WUS monitoring by UE is known to gNB.

  - FFS whether implicit/explicit indication from UE is necessary

- In case LP-WUS monitoring is enabled, following options are further studied

  - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring

  - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance.

  - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer.

  - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

RAN1#116bis

Agreement

Update the following agreement in RAN1#116 in red:

Agreement

- For RRC CONNECTED mode, from RAN1 perspective, further study following LP-WUS procedures to trigger PDCCH monitoring:

  - Case 1: PDCCH monitoring is triggered by LP-WUS with C-DRX configuration

    - Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.

      - This option may replace DCP functionality

    - Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

      - PDCCH monitoring possibly irrespective of drx-onDurationTimer

        - Option 1-2-1: PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

          - If this is adopted, it should be configured together with Option 1-1 to achieve power saving gain compared to legacy C-DRX

        - Option 1-2-2: PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

    - Option 1-3: LP-WUS monitoring inside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

  - Case 2: PDCCH monitoring is triggered by LP-WUS without C-DRX configuration. LP-WUS can be monitored at any time according to the LP-WUS monitoring configuration

    - FFS duty-cycled and/or continuous LP-WUS monitoring

- Combination of options in Case 1 and combination of options in Case 1 and Case 2 are not precluded should be considered.

- RAN1 does not discuss C-DRX related timers other than drx-onDurationTimer, this topic is up to RAN2

- Note: Above does not preclude to support fallback mechanism to trigger PDCCH monitoring, if any

RAN1#117

Agreement

For RRC CONNECTED mode, support UE capability report for determination of minimum time gap between LP-WUS reception and MR to start PDCCH monitoring.

- FFS: exact value(s) of the minimum time gap

- FFS: support of multiple minimum time gaps

- FFS whether the reported value includes the duration for time/frequency synchronization of MR

Agreement

For LP-WUS monitoring in RRC CONNECTED mode, a LP-WUS is QCLed with existing NR signal/channel/CORESET for the TCI state

- FFS which existing NR signal/channel/CORESET is the QCL source of LP-WUS

- FFS exact definition of QCL relationship between LP-WUS and existing NR signal/channel/CORESET

Agreement

LP-WUS monitoring occasions (MOs) are configured by RRC, where UE can monitor for LP-WUS transmission in RRC CONNECTED mode.

- FFS whether to define a time window for MOs

- It is at least supported that a UE can monitor MOs with a periodicity.

  - FFS details of the periodicity, e.g. derived from DRX cycle, separately configured

Agreement

- Study whether/how LP-WUS works when UE is configured with CA in RRC CONNECTED mode

FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable

Option 1: one or more serving cells based on gNB indication/configuration

Option 2: all activated serving cells

Note: other options are not precluded

Agreement

For RRC CONNECTED mode, LP-WUS can be configured without following existing features.

Rel-16 DCP

Rel-17 PDCCH skipping

Rel-17 SSSG switching

Rel-18 cell DTX

Further study whether/how LP-WUS works with following existing features (PDCCH skipping, SSSG switching, cell DTX)

RAN1#118

Agreement

- For option 1-2 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective.

LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

UE is configured with legacy C-DRX configurations as Rel-18

UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration)

    - FFS potential restriction for LP-WUS configuration in relation with C-DRX configuration

LP-WUS triggers the start of a timer during which UE monitors PDCCH

    - FFS the timer is existing timer and/or new timer

UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected

    - drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL

No impact on RRM/RLM/BFD measurement requirements is assumed

For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1)

    - Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up

    - Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not

UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

Working assumption

From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration

Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.

Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE

Note: Above can be revisited considering RAN2 decisions.

Agreement

Select one of the following alternatives in RAN1#118bis

Alt 1

For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting.

FFS: X

FFS: definition of reported value

Alt2

For RRC CONNECTED mode, UE reports multiple values for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting.

FFS: X

FFS: definition of reported value

Different minimum time gaps correspond to different sleep states

Companies are encouraged to details on how the reported values are to be used by the network

Agreement

LP-WUS is at least supported for the case where a UE is configured with CA in RRC CONNECTED mode

- FFS: DC

Agreement

- For LP-WUS monitoring in RRC CONNECTED mode, SSB and/or CSI-RS can be the QCL source of LP-WUS

FFS applicable QCL type(s)

RAN1#118bis

Agreement

- For Option 1-1 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective.

LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer

UE is configured with legacy C-DRX configurations as Rel-18

UE behaviors related to CDRX active time triggered by all legacy DRX timers are not affected unless included in this proposal

No impact on RRM/RLM/BFD measurement requirements is assumed

For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP)

    - Periodic CSI/L1-RSRP is not reported during the time given by the configured drx-onDurationTimer if UE is not indicated to wake-up

    - Periodic CSI/L1-RSRP is periodically reported during the time given by the configured drx-onDurationTimer regardless if UE is indicated to wake-up or not

      - Note: Periodic CSI/L1-RSRP is reported during CDRX active time as in legacy specification

Agreement

Confirm following working assumption

Working Assumption

From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration

Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.

Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE

Note: Above can be revisited considering RAN2 decisions.

Agreement

For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting.

FFS: X

FFS: definition of reported candidate value

UE can indicate its preference for configured time offset between LP-WUS and PDCCH reception using UAI mechanism. FFS details. UAI is optional.

FFS: A single time offset is configured for a UE at a given time

Agreement

LP-WUS is supported when UE is configured with NR-DC in RRC CONNECTED mode

  - Above is supported for the case PDCCH monitoring is triggered by LP-WUS in the same cell group

  - FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable

Agreement

- For LP-WUS CONNECTED mode operation, Option 1-2 can be configured separately from Option 1-1

  - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE

Agreement

- Regarding the potential restriction for LP-WUS configuration in relation with C-DRX configuration for option 1-2 of LP-WUS CONNECTED mode operation, from RAN1 perspective,

  - Opt2: At least LP-WUS periodicity is no larger than long C-DRX cycle, FFS other restrictions

  - Introducing monitoring window is not precluded

RAN1#119

Agreement

For LP-WUS monitoring in RRC CONNECTED mode, SSB and/or CSI-RS can be the QCL source of LP-WUS QCL type(s) is {Type A or Type C – for down-selection} and Type D, when applicable

Agreement

LP-WUS in RRC CONNECTED mode does not include Scell dormancy indication.

- Note: LP-WUS indication does not impact SCell dormancy behavior

Agreement

For LP-WUS CONNECTED mode operation, the following case is not supported:

- Both Option 1-1 and Option 1-2 are simultaneously configured for the same UE in a given cell

Agreement

- For LP-WUS MOs in connected mode for Option 1-1, following is considered further discussion and possible down-selection

  - Approach 1:

    - LP-WUS MOs, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s).

    - UE selectively monitors LP-WUS in the LP-WUS MOs identified by other parameters such as time offset prior to the start of drx-onDurationTimer.

  - Approach 2:

    - LP-WUS MOs, including periodicity and offset, are identified based on the slot that drx-onDurationTimer would start and a new RRC parameter indicating the time offset until the slot that drx-onDurationTimer would start.

  - FFS: An RRC parameter indicates the duration where the UE monitors LP-WUS [consecutively in time]

    - FFS: the consecutive LP-WUS MOs in time identified by the RRC parameter are associated with a same C-DRX cycle;

    - FFS: LP-WUS MOs in time identified by the RRC parameter provides consistent indication for the same C-DRX cycle of the UE, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS MOs associated with the same C-DRX cycle.

- For LP-WUS MOs in connected mode for Option 1-2, following is considered further

  - Approach 1:

    - LP-WUS MOs, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s).

    - UE monitors LP-WUS in the LP-WUS MOs and starts a new timer for PDCCH monitoring triggered by LP-WUS, after a time offset.

  - Approach 2:

    - Periodicity/offset of the slot that the new timer for PDCCH monitoring may be potentially triggered by LP-WUS are configured by RRC parameters.

    - LP-WUS MOs, including periodicity and offset, are identified based on the slot that new timer would start and a new RRC parameter indicating the time offset until the slot that new timer would start.

  - FFS: An RRC parameter indicates the duration where the UE monitors LP-WUS [consecutively in time]

    - FFS: the consecutive LP-WUS MOs in time identified by the RRC parameter are associated with the same slot that the new timer would start;

    - FFS: LP-WUS MOs in time identified by the RRC parameter provides consistent indication for the same slot that the new timer would start, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS MOs associated with the same slot that the new timer would start.

Agreement

- For Option 1-2 of LP-WUS CONNECTED mode operation, for explicit indication to stop the new timer for PDCCH monitoring in Option 1-2 and drx-InactivityTimer for the DRX group, RAN1 identified the following potential solution

  - Use existing DRX Command MAC CE and Long DRX Command MAC CE

  - When UE is not in C-DRX active time, UE monitors LP-WUS

- Send LS to RAN2 to ask the feasibility of the above potential solution. LS in R1-2410909.

Conclusion

- For Option 1-1 of LP-WUS CONNECTED mode operation, RAN1 assumes existing DRX Command MAC CE and Long DRX Command MAC CE can be used to stop drx-onDurationTimer and drx-InactivityTimer for the DRX group

  - When UE is not in C-DRX active time, UE monitors LP-WUS

- Send LS to RAN2 to ask whether RAN2 has same understanding or not. LS in R1-2410909.

Agreement

- For Option 1-2, for LP-WUS CONNECTED mode operation, the periodic CSI/L1-RSRP reporting operation is same as Rel-16 DCP, that is the UE can be configured with a parameter to enable/disable periodic CSI/L1-RSRP reporting, respectively:

  - If the parameter is NOT configured:

    - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is not reported.

    - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time.

  - If the parameter is configured:

    - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the time given by the configured drx-onDurationTimer in DRX-Config for the case when UE is outside C-DRX active time.

    - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time.

  - FFS: Whether RAN1 spec impact is required.

Companies are encouraged to consider [Fri]Combined Proposal 3.2-1/3.2-2c of R1-2410906 for further discussions in future meetings.

RAN1#120

Agreement

- For RRC connected mode, UE does not monitor LP-WUS during C-DRX active time

  - FFS FAR and MDR impact

Agreement

- LP-WUS can be configured with legacy PDCCH skipping.

  - Note: There is no intention to modify legacy PDCCH skipping

Agreement

- For RRC CONNECTED mode, LP-WUS can be configured with Rel-17 SSSG switching

Agreement

- For the case when UE is configured with CA without dual DRX groups in RRC CONNECTED mode,

  - LP-WUS can be configured on a serving cell

  - LP-WUS indication is applicable to all serving cells

    - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation

- FFS: Whether the serving cell on which the LP-WUS is configured is only PCell or can be RRC configured cell other than PCell

Agreement

- For the case when UE is configured with NR-DC with CA without dual DRX groups or without CA in RRC CONNECTED mode,

  - LP-WUS can be configured on a serving cell per cell-group

    - This does not imply that LP-WUS has to be configured on both cell groups

  - LP-WUS indication is applicable to all serving cells in the cell-group

    - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation

  - FFS whether to support this case as separate UE capability from CA case

  - FFS UE capability for monitoring LP-WUS on one cell group or both cell groups

- FFS: Whether the serving cell on which the LP-WUS is configured is only PCell/PSCell or can be RRC configured cell other than Pcell/PSCell

Agreement

- For RRC CONNECTED mode, LP-WUS can be configured with Cell DTX

  - FFS During Cell DTX inactive time, whether the UE is expected to monitor LP-WUS or not

Conclusion

From RAN1 perspective, when LP-WUS monitoring is enabled for RRC CONNECTED mode, it is supported that SR, PRACH and CG-PUSCH triggers MR PDCCH monitoring according to the legacy UE behaviour

- Above applies at least for Option 1-1

Agreement

For LP-WUS monitoring in RRC CONNECTED mode, select one of the options on how to define/configure/indicate QCL source of LP-WUS

- Opt1: Separate indication/configuration for QCL source of LP-WUS

- Opt2: Association of QCL source between LP-WUS and PDCCH/CORESET

Agreement

For LP-WUS MOs in connected mode for Option 1-1, support Approach 1:

  - LP-WUS MOs, including periodicity and time offset1, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s).

  - Time offset2 indicates a time prior to a slot where the drx-onDurationTimer would start

    - UE monitors the first Z LP-WUS MO(s) at or after Time offset2.

      - FFS: How to determine the value of Z

    - Time offset2 is RRC configured

  - UE is not required to monitor LP-WUS during the X [slots/symbols] prior to the beginning of a slot where the UE would start the drx-onDurationTimer. FFS: How X is determined.

Agreement

For LP-WUS MOs in connected mode for Option 1-2, support Approach 1:

- LP-WUS MOs, including periodicity and time offset3, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s).

  - FFS one or multiple MOs per periodicity

- UE monitors LP-WUS in the LP-WUS MOs and, if triggered to wake up, starts a new timer for PDCCH monitoring triggered by LP-WUS, after a time offset4.

  - The time offset4 configured by the network indicating a time, after which the UE starts PDCCH monitoring via starting the new timer.

  - FFS: Definition of time offset4

Agreement

- For the X = [2, 3] candidate values for the UE capability report on the minimum time gap for each SCS, consider {0.25, 0.5, 1, 2, 3, 6, 10, 20, 30, 40}ms

  - Note: Other values are not precluded

  - Only for reported value of [20ms,] 30ms and 40ms (if supported), the reported values assume SSB periodicity of 20ms and ensures SSB(s) before PDCCH reception and during time gap

    - FFS: translation of minimum time gap for different SSB periodicities including different candidates for different SSB periodicities

  - FFS whether the same set of candidates are used for different SCSs

  - FFS whether different values are selected for OOK-based WUR and OFDM-based WUR

  - FFS whether/how to consider the case of CA

  - FFS: candidate values that can be indicated by UAI