3GPP TSG RAN WG1 #120bis	R1-250xxxx

# Wuhan, China, April 7th – 11th, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.901 | CR | Draft | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to introduce channel model for ISAC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Sensing_NR |  |  |  |  |  | Date: |  |  | 2025-4-7 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN #102, the study item for channel modelling for ISAC was agreed. The output of the study item was agreed to be incorporated into TR 38.901. This CR captures the agreements/conclusions of the study item for channel modelling for ISAC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clauses, i.e., 7.9 and [8.1] are added for the channel model for ISAC. The existing sections 1, 2, 4, […] are also updated to reflect the introduction of channel model for ISAC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The existing channel model for communications in TR 38.901 is not sufficient to address evaluation of sensing techniques for Integrated Sensing and Communications use cases. The existing models in TR38.901 are not designed for sensing, in particular they do not address target modelling and sensing, background environment modelling and differentiation from targets. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Impact to all/most clauses expected |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1	Scope

The present document captures the findings of the study item, "Study on channel model for frequency spectrum above 6 GHz" [2] and from further findings of the study item, "Study on New Radio Access Technology [22]", the study item "Study on Channel Modeling for Indoor Industrial Scenarios [23]" and the study item "Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR [24]". The channel models in the present document address the frequency range 0.5-100 GHz. The purpose of this TR is to help TSG RAN WG1 to properly model and evaluate the performance of physical layer techniques using the appropriate channel model(s). Therefore, the TR will be kept up-to-date via CRs in the future.

This document relates to the 3GPP evaluation methodology and covers the modelling of the physical layer of both Mobile Equipment and Access Network of 3GPP systems.

This document is intended to capture the channel model(s) for frequencies from 0.5GHz up to 100GHz.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

< Unchanged text omitted >

[23]	3GPP TD RP-182138: "SID on Channel Modeling for Indoor Industrial Scenarios".

[26]	3GPP TD RP-242348: " Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR ".

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] apply.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

< Unchanged text omitted >

Angular displacement between two pairs of unit vectors

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

< Unchanged text omitted >

ZSD	Zenith angle Spread of Departure

4	Introduction

At TSG RAN #69 meeting the Study Item Description on "Study on channel model for frequency spectrum above 6 GHz" was approved [2]. This study item covers the identification of the status/expectation of existing information on high frequencies (e.g. spectrum allocation, scenarios of interest, measurements, etc), and the channel model(s) for frequencies up to 100 GHz. This technical report documents the channel model(s). The new channel model has to a large degree been aligned with earlier channel models for <6 GHz such as the 3D SCM model (TR 36.873) or IMT-Advanced (ITU-R M.2135). The new model supports comparisons across frequency bands over the range 0.5-100 GHz. The modelling methods defined in this technical report are generally applicable over the range 0.5-100 GHz, unless explicitly mentioned otherwise in this technical report for specific modelling method, involved parameters and/or scenario.

Subsequently, at the TSG RAN #81 meeting the Study Item Description "Study on Channel Modeling for Indoor Industrial Scenarios" was approved [23]. The findings from this study item is also captured in the present technical report. The Industrial channel model was developed by considering new measurements and information in the literature. An overview list of all such contributions and sources is available in tdoc R1-1909706.

At TSG RAN #102 meeting the Study Item Description “Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR” was approved. The findings from this study item are captured in Clause 7.9.

The channel model is applicable for link and system level simulations in the following conditions:

-	For system level simulations, supported scenarios are urban microcell street canyon, urban macrocell, indoor office, rural macrocell, and indoor factory.

-	Bandwidth is supported up to 10% of the center frequency but no larger than 2GHz.

-	Mobility of either one end of the link or both ends of the link is supported

-	For the stochastic model, spatial consistency is supported by correlation of LSPs and SSPs as well as LOS/NLOS state.

-	Large array support is based on far field assumption and stationary channel over the size of the array.

-    The sensing target is assumed in the far field of the sensing transmitter/receiver for the channel model(s) for ISAC in Clause 7.9

5	Void

6	Status/expectation of existing information on high frequencies

6.1	Channel modelling works outside of 3GPP

This clause summarizes the channel modelling work outside of 3GPP based on the input from companies.

< Unchanged text omitted >

QuaDRiGa (Fraunhofer HHI)

-	QuaDRiGa (QUAsi Deterministic RadIo channel GenerAtor) was developed at the Fraunhofer Heinrich Hertz Institute within the Wireless Communications and Networks Department to enable the modelling of MIMO radio channels for specific network configurations, such as indoor, satellite or heterogeneous configurations.

-	Besides being a fully-fledged 3D geometry-based stochastic channel model (well aligned with TR36.873), QuaDRiGa contains a collection of features created in SCM(e) and WINNER channel models along with novel modelling approaches which provide features to enable quasi-deterministic multi-link tracking of users (receiver) movements in changing environments. QuaDRiGa supports Massive MIMO modelling enabled through a new multi-bounce scattering approach and spherical wave propagation. It will be continuously extended with features required by 5G and frequencies beyond 6 GHz. The QuaDRiGa model is supported by data from extensive channel measurement campaigns at 10 / 28 / 43 / 60 / 82 GHz performed by the same group.

ETSI Industry specification group (ISG) Integrated sensing and communications (ISAC)

-	ETSI ISG ISAC plans to publish Group Report (GR) DGR/ISC-002 that provides information on developing advanced channel models and validation through measurement campaigns and emulations, information on evaluation methodology framework, and corresponding deployment scenarios and the potentially suitable frequency bands.

6.2	Scenarios of interest

Brief description of the key scenarios of interest identified (see note):

< Unchanged text omitted >

(7)	Indoor industrial scenarios

(9) e sensing and communication scenarios: as described in Clause 7.9.1.

Note:	The scenarios of interest are based on the plenary email discussion and different from the supported scenarios in clause 7. The indoor industrial scenarios were identified at a later stage in the TSG RAN #81 meeting. The integrated sensing and communication scenarios were identified at a later stage as part of Study on channel modelling for integrated sensing and communications for NR.

< Unchanged text omitted >

6.4	Modelling objectives

The requirements for channel modelling are as follows.

< Unchanged text omitted >

-	Support large antenna arrays

-	Support integrated sensing and communications

7	Channel model(s) for 0.5-100 GHz

< Unchanged text omitted >

7.9	Channel model(s) for ISAC

7.9.0	Introduction

The channel model for ISAC in this clause is designed based on the channel model defined in the previous clauses within Clause 7 taking into account the known properties, e.g., location, Radar Cross-Section (RCS), polarization and etc. of one or more physical objects. A physical object is categorized as a sensing target (ST) or an environment object (EO). The large scale and small scale parameters of the channel between any two of sensing transmitter (STX), ST and sensing receiver (SRX) in a sensing scenario are obtained from the TRs[X] for the same communication scenario unless updates on the parameter values are specially described. The ST is considered as a receiver or transmitter, respectively, in the determination of a proper channel model for a STX-ST link or a ST-SRX link.

The common framework for the ISAC channel between a pair of STX and SRX is composed of a component for one or multiple target channel(s) and a component for a background channel. A pair of STX and SRX can sense one or multiple STs. If blockage/forward scattering between sensing targets is not considered, a propagation path from STX to SRX interacting with more than one sensing target is not modelled. The target channel of ST k includes all [multipath] components impacted by ST k, 1≤k≤K, where K is the number of STs. Such [multipath] components may interact with stochastic cluster(s) in either of or both the STX-ST link and ST-SRX link. The background channel includes other [multipath] components not belonging to any of the K target channel(s).

7.9.1	Scenarios

The detailed sensing scenario description in this clause can be used for channel model calibration.

ISAC-UAV

Details on ISAC-UAV scenarios are listed in Table 7.9.1-1.

Table 7.9.1-1: Evaluation parameters for UAV sensing scenarios

| Parameters |  | Value |
| --- | --- | --- |
| Applicable communication scenarios |  | UMi, UMa, RMa [38.901] UMi-AV, UMa-AV, RMa-AV |
| Sensing transmitters and receivers properties | Rx/Tx Locations | Rx/Tx locations are selected among the TRPs and UEs locations in the corresponding communication scenarios. NOTE1: This may include aerial UEs for UMi-AV, UMa-AV, RMa-AV communication scenarios. In this case, other Rx/Tx properties (e.g. mobility) are also taken from the corresponding communication scenario. |
| Sensing target | LOS/NLOS | LOS and NLOS |
|  | Outdoor/indoor | Outdoor |
|  | 3D mobility | Horizontal velocity: uniform distribution between 0 and 180km/h, if horizontal velocity is not fixed to 0. Vertical velocity: 0km/h, optional {20, 40} km/h NOTE2: 3D mobility can be horizontal only or vertical only or a combination for each sensing target NOTE 3: time-varying velocity may be considered for future evaluations. |
|  | 3D distribution | Horizontal plane: Option A: N targets uniformly distributed within one cell. Option B: N targets uniformly distributed per cell. Option C: N targets uniformly distributed within an area not necessarily determined by cell boundaries. N = {1, 2, 3, 4, 5} NOTE4: N=0 may be considered for the evaluation of false alarm Vertical plane: Option A: Uniform between 1.5m and 300m. Option B: Fixed height value chosen from {25, 50, 100, 200, 300} m assuming vertical velocity is equal to 0. |
|  | Orientation | Random in horizontal domain |
|  | Physical characteristics (e.g., size) | Size: - Option 1: 1.6m x 1.5m x 0.7m - Option 2: 0.3m x 0.4m x 0.2m |
| Minimum 3D distances between pairs of Tx/Rx and sensing target |  | Min distances based on min. TRP/UE distances defined in TR36.777 as a starting point. NOTE5: the sensing target is assumed in the far field of sensing Tx/Rx |
| Minimum 3D distance between sensing targets |  | Option 1: At least larger than the physical size of a target Option 2: 10 meters |
| [Unintended/Environment objects, e.g., types, characteristics, mobility, distribution, etc.] |  | FFS |

NOTE: A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations.

ISAC-Automotive

Details on ISAC-Automotive scenarios are listed in Table 7.9.1-2.

Table 7.9.1-2: Evaluation parameters for Automotive sensing scenarios

| Parameters |  | Values |
| --- | --- | --- |
| Applicable communication scenarios |  | Highway, Urban Grid. NOTE1 |
| Sensing transmitters and receivers properties |  | Rx/Tx locations are selected among the TRPs and UEs (e.g., VRU, vehicle, RSU-type UEs) locations in the corresponding communication scenario. NOTE2 Additional option: ISD between TRPs of Urban Grid is 250m |
| Sensing target | LOS/NLOS | LOS and NLOS (including NLOSv) |
|  | Outdoor/indoor | Outdoor |
|  | Mobility (horizontal plane only) | Based on TR37.885 mobility for urban grid or highway scenario |
|  | Distribution (horizontal) | Based on dropping in TR37.885 per urban grid or highway communication scenario |
|  | Orientation | Lane direction in horizontal plane |
|  | Physical characteristics (e.g., size) | Type 1/2 (passenger vehicle) Type 3 (truck/bus) Vehicle type distribution per TR 37.885 as a starting point |
| Minimum 3D distances between pairs of Tx/Rx and sensing target |  | Min distances based on min. TRP/UE distances defined in TR37.885 as a starting point. NOTE3: the sensing target is assumed in the far field of sensing Tx/Rx |
| Minimum 3D distance between sensing targets |  | Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, 10 m. |
| Environment Objects, e.g., types, characteristics, mobility, distribution, etc. |  | EO Type 2 for Urban Grid - up to 4 walls modelled as EO type 2, per building of size 413m x 230m x 20m. FFS: number of buildings, how many walls are modelled, additional building sizes, etc. |

NOTE1: calibration for UMi, Uma, RMa is not performed for the automotive scenario, but UMi, Uma, RMa can be considered for future evaluations of the automotive sensing target scenarios. Calibration for UMi, Uma, RMa is expected to be performed for another sensing scenario.

NOTE2: A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations.

ISAC-Human

Details on ISAC-Human scenarios are listed in Table 7.9.1-3.

Table 7.9.1-3: Evaluation parameters for Human (indoor and outdoor) sensing scenarios

| Parameters |  | Indoor Values | Outdoor Values |
| --- | --- | --- | --- |
| Applicable communication scenarios NOTE1 |  | Indoor office, indoor factory [TR38.901] Indoor room [TR38.808] | UMi, Uma, RMa [TR38.901] |
| Sensing transmitters and receivers properties | Rx/Tx Locations NOTE 2 | Rx/Tx locations are selected among the TRPs and UE locations in the corresponding communication scenario | Rx/Tx locations are selected among the TRPs and UE locations in the corresponding communication scenario |
|  | Rx/Tx Mobility for UEs | Option 1: 0km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 3km/hr | Option 1: 0km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 10km/hr |
| Sensing target | LOS/NLOS | LOS and NLOS | LOS and NLOS |
|  | Outdoor/indoor | Indoor | Outdoor |
|  | 3D mobility | Option 1: 0km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 3km/hr (horizontal plane with random direction straight-line trajectory) | Option 1: 0km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 10km/hr (horizontal plane with random direction straight-line trajectory) |
|  | 3D distribution | N targets uniformly distributed over the horizontal area of the convex hull of the TRP deployment NOTE1: N=0 may be considered for the evaluation of false alarm | Option A: N targets uniformly distributed within one cell. Option B: N targets uniformly distributed per cell. Option C: N targets uniformly distributed within an area not necessarily determined by cell boundaries. NOTE1: N=0 may be considered for the evaluation of false alarm |
|  | Orientation | Random over the horizontal area | Random over the horizontal area |
|  | Physical characteristics (e.g., size) | Size (Length x Width x Height): - Child: 0.2m x 0.3m x 1m - Adult Pedestrian: 0.5m x 0.5m x 1.75m | Size (Length x Width x Height): - Child: 0.2m x 0.3m x 1m - Adult Pedestrian: 0.5m x 0.5m x 1.75m |
| Minimum 3D distances between pairs of Tx/Rx and sensing target |  | Min distances defined in TR 38.901 and TR36.843 and TR38.859as a starting point NOTE2: the sensing target is assumed in the far field of sensing Tx/Rx | Min distances defined in TR 38.901 and TR36.843 and TR38.859 as a starting point NOTE3: the sensing target is assumed in the far field of sensing Tx/Rx |
| Minimum 3D distance between sensing targets |  | Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, [x] m. value of x is FFS | Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, [x] m. value of x is FFS |
| Environment Objects, e.g., types, characteristics, mobility, distribution, etc. |  | FFS, based on outcome for AI 9.7.2 | FFS, based on outcome for AI 9.7.2 |

NOTE1: For the human (indoor and outdoor) sensing targets, additional communication scenarios can be considered for future evaluations. Channel model calibration for Urban Grid with outdoor humans is expected to be performed from Objects creating hazards on the road/railway sensing scenarios.

NOTE2: A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations.

ISAC-AGV

Details on ISAC-AGV are listed in Table 7.9.1-4.

Table 7.9.1-4: Evaluation parameters for Automated Guided Vehicles

| Parameters |  | Value |
| --- | --- | --- |
| Applicable communication scenarios NOTE1 |  | InF (TR38.901 including Table 7.8-7) |
| Sensing transmitters and receivers properties NOTE2 |  | Rx/Tx location are selected among the TRPs and UEs location in the corresponding communication scenario Rx/Tx Mobility for UEs - Option 1: 0 km/h - Option 2: 3km/h - Option 3: Uniform distribution between 0km/h and 3km/h |
| Sensing target | LOS/NLOS | LOS and NLOS |
|  | Outdoor/indoor | Indoor |
|  | 3D mobility | Horizontal velocity with random straight-line trajectory - Option 1: Uniform distribution in the range of up to 30 km/h - Option 2: Fixed velocities [3, 10] km/h |
|  | 3D distribution | Option A: Uniformly distributed in the convex hull of the horizontal BS deployment Option B: Uniformly distributed in horizontal plane |
|  | Orientation | Horizontal plane only |
|  | Physical characteristics (e.g., size) | Size (L x W x H) - Option 1: 0.5m x 1.0m x 0.5m - Option 2: 1.5 m x 3.0m x 1.5 m - FFS: Material, Additional sizes, and AGV size distribution |
| Minimum 3D distances between pairs of Tx/Rx and sensing target |  | Min distances based on min. TRP/UE distances defined in TR38.901 NOTE: the sensing target is assumed in the far field of sensing Tx/Rx |
| Minimum 3D distance between sensing targets |  | Option A: At least larger than the physical size of a target Option B: Fixed value, [x] m. value of x is FFS |
| Environment objects, e.g., types, characteristics, mobility, distribution, etc. |  | FFS |

NOTE1: For the AGV sensing targets, additional communication scenarios can be considered for future evaluations.

NOTE2: A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations.

NOTE3: RAN1 can further discuss narrowing down the number of sub-scenarios of InF

ISAC-Objects creating hazards

Details on ISAC-Hazards are listed in Table 7.9.1-5.

Table 7.9.1-5: Evaluation parameters for objects creating hazards

| Parameters |  | Value |
| --- | --- | --- |
| Applicable communication scenarios NOTE1 |  | Highway, Urban grid, HST (High Speed Train) |
| Sensing transmitters and receivers properties NOTE2 | Rx/Tx Locations | Rx/Tx locations are selected among the TRPs and UEs (e.g., VRU, vehicle, RSU-type UEs) locations in the corresponding communication scenarios. Additional option ISD between TRPs of Urban Grid is 250 m |
| Sensing target | LOS/NLOS | LOS and NLOS |
|  | Outdoor/indoor | Outdoor |
|  | 3D mobility | Horizontal velocity: up to [10] km/h for humans and animals FFS: Additional velocities, trajectory |
|  | 3D distribution | Uniformly distributed in horizontal plane |
|  | Orientation | Random distribution in horizontal plane |
|  | Physical characteristics (e.g., size) | For human/pedestrians: Child: 0.2m x 0.3m x 1m Adult: 0.5m x 0.5m x 1.75m For animals: Size: 1.5m x 0.5m x 1 m |
| Minimum 3D distances between pairs of Tx/Rx and sensing target |  | Min. distance is based on min TRP/UE distances defined in TR37.885 and TR38.802 and TR36.843 and TR38.859 NOTE: the sensing target is assumed in the far field of sensing Tx/Rx |
| Minimum 3D distance between sensing targets |  | Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, 10 m. |
| Environment objects, e.g., types, characteristics, mobility, distribution, etc. |  | EO Type 2 for Urban Grid - up to 4 walls modelled as EO type 2, per building of size 413m x 230m x 20m. FFS: number of buildings, how many walls are modelled, additional building sizes, etc. |

NOTE1: For the objects creating hazards sensing targets, additional communication scenarios can be considered for future evaluations.

NOTE2: A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations.

7.9.2	Physical object model

7.9.2.0	Introduction

A ST is an object of interest for sensing. An EO is a non-target object with known location. Two types of EO are supported in the ISAC channel model. A first type of EO (type-1 EO) has similar characteristic as a ST and is modelled in the same way as a ST. A second type of EO (type-2 EO, 7.9.5.2) is of large size and is modelled differently from a ST.

In the following descriptions in Clause 7.9, only the related details on ST are described, which is also applicable to type-1 EO.

A ST is modelled with one or multiple scattering points. Each scattering point of a ST (SPST) is used to model the total scattering effects of some adjacent scattering centres at the ST. The impact of a SPST to the channel is modelled from two aspects, i.e., the RCS (Radar Cross Section, 7.9.2.1) and the polarization matrix (7.9.2.2). The RCS of the SPST is a scalar value and is defined as the hypothetical area required to intercept the incident power at the SPST such that if the total intercepted power were re-radiated, the power density actually observed at the receiver would be produced. The polarization matrix of the SPST includes the effect of phase and amplitude of co-polarization and cross-polarization at the SPST. The polarization matrix of the SPST is separately modelled from other polarization effects introduced by stochastic clusters and/or EOs in the target channel.

7.9.2.1 RCS of a sensing target

The RCS of a SPST is defined in LCS of the ST and is dependent on both the incident angle and the scattered angle. A subset of the RCS values with same incident/scattered angles can be referred as monostatic RCS values.

The RCS related coefficient σRCS of a SPST for a pair of incident/scattered angles is composed of a first component σM which is included in the large-scale parameters, and a second component σD and third component σS which are both included in the small-scale parameters, i.e., σRCS=σMσDσS. σM is deterministic value for the SPST. σD can be fixed to 1 or can be angular dependent. σS follows log-normal distribution. The mean μσS_dB and standard deviation σσS_dB used to characterize 10lgσS satisfied a fixed relation μσσS_dB=-ln1020σσS_dB2. σSσS is separately determined for each pair of incident/scattered angels at the SPST. [correlation]

A first RCS model is to characterize a ST as single SPST with component σD of the monostatic RCS values fixed to 1. σM is the mean of linear monostatic RCS values at the SPST. The logarithmic values of σM,σD,σSof the monostatic RCS values for the sensing targets are provided in Table 7.9.2.1-1.

Table 7.9.2.1-1: Parameters on RCS for the targets with angular independent monostatic RCS values

| Sensing target | 10lgσM (dBsm) | 10lgσD (dB) | σσS_dB (dB) |
| --- | --- | --- | --- |
| UAV of small size | -12.81 | 0 | 3.74 |
| Human with RCS model 1 | -1.37 | 0 | 3.94 |

A second RCS model is to split a ST into single or multiple SPSTs, and adopts an angular dependent component σD of the monostatic RCS values for each SPST. For UAV with large size and human, single SPST is modelled. While for vehicle [and AGV], both models with single and multiple SPSTs are provided. For vehicle modelled with multiple scattering points, the recommended five scattering points are located at the front, left, back, right and roof side of the vehicle respectively. The orientation of a sensing target in LCS is provided as follows.

- The front of a vehicle, a UAV with large size or an AGV has azimuth angle ϕ=00 and zenith angle θ=900 in LCS.

- For a human, it faces the direction of azimuth angle ϕ=00 and zenith angle θ=900 in LCS.

For UAV of large size with single scattering point, human with RCS model 2, and AGV with single[/multiple SPSTs], the values/pattern 10lgσMσDσMσD, denoted as σMD_dBθ',ϕ', of the monostatic RCS values for a SPST is deterministic based on incident/scattered angles

σMD_dB(θ',ϕ')=Gmax-min-σVdBθ'+σHdBϕ',σmax

Where,

σVdBθ'=-min12θ'-θcenterθ3dB2, σmax

σHdBϕ'=-min12ϕ'-ϕcenterϕ3dB2, σmax

The parameters to define σMσD,σM,σS of the monostatic RCS values for the sensing targets are provided in Table 7.9.2.1-2/3/6/7.

For vehicle with single/multiple SPSTs, the values/pattern 10lgσMσDσMσD, denoted as σMD_dBθi,ϕi,θs,ϕsθ,ϕ, of the RCS for a SPST is deterministic based on the incident angle (θi,ϕi,) and the scattered angle (θs,ϕs).

σMD_dB(θi,ϕi,θs,ϕsθ,ϕ)=maxGmax-min-σVdBθ+σHdBϕ,σmax-AF,σFS(θi,ϕi,θs,ϕs)

With σVdBθ, σHdBϕ defined by,

σVdBθ=-min12θ-θcenterθ3dB2, σmax

σHdBϕ=-min12ϕ-ϕcenterϕ3dB2, σmax

Where,

  - (θ,ϕ) are the projection of the bisector angle between incident angle (θi,ϕi,) and scattered angle (θs,ϕs) on the vertical plane and the horizontal plane, respectivelyθ=θi+θs2,ϕ=ϕi+ϕs2.

  - AF=k1sink2β2,β∈0,180°. β is the bistatic angle between the incident ray and scattering ray within the plane defined by incident angle (θi,ϕi,) and scattering angle (θs,ϕs).

    - k1= 6, k2=1.65 for vehicle

  - σFS(θi,ϕi,θs,ϕs) is for the effect of forward scattering and is set to -∞

For a vehicle with single scattering point, Nsp≥1 sets of parameters φcenter, φ3dB, n, θcenter, θ3dB,n, Gmax, σmax, Range of θ and Range of ϕ are defined. The bisector angle (θ,ϕ) is used to index one set from the Nsp sets of parameters, and determine σMD_dB(θi,ϕi,θs,ϕs) of the vehicle consequently. If the vehicle is split into Nsp scattering points, with each scattering point characterized by one of Nsp sets of parameters, σMD_dB(θi,ϕi,θs,ϕs) are respectively determined for each scattering point.

The Nsp sets of parameters to define σMσD,σM,σS and the parameters σM,σS of the RCS for the vehicle are provided in Table 7.9.2.1-4/5.

Table 7.9.2.1-2: Parameters on RCS for UAV with large size

|  | 10lgσMσD (dBsm) |  |  |  |  |  |  |  | 10lgσM (dBsm) | σσS_dB (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ϕcenter in [°] | ϕ3dB, n in [°] | θcenter in [°] | θ3dB,n in [°] | Gmax | σmax | Range of θ in [°] | Range of ϕ in [°] |  |  |
| Left | 90 | 7.13 | 90 | 8.68 | 7.43 | 14.30 | [45,135] | [45,135] | [] | 2.50 |
| Back | 180 | 10.09 | 90 | 11.43 | 3.99 | 10.86 | [45,135] | [135,225] |  |  |
| Right | 270 | 7.13 | 90 | 8.68 | 7.43 | 14.30 | [45,135] | [225,315] |  |  |
| Front | 0 | 14.19 | 90 | 16.53 | 1.02 | 7.89 | [45,135] | [-45,45] |  |  |
| Bottom | / | / | 180 | 4.93 | 13.55 | 20.42 | [135,180] | [0,360] |  |  |
| Roof | / | / | 0 | 4.93 | 13.55 | 20.42 | [0,45] | [0,360] |  |  |

Note: When θ is in the range [0,45] or [135,180], σHdB φ=0.

Table 7.9.2.1-3: Parameters on RCS for human with RCS model 2

|  | 10lgσMσD (dBsm) |  |  |  |  |  |  |  | 10lgσM (dBsm) | σσS_dB (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ϕcenter in [°] | ϕ3dB, n in [°] | θcenter in [°] | θ3dB,n in [°] | Gmax | σmax | Range of θ in [°] | Range of ϕ in [°] |  |  |

Table 7.9.2.1-4: Parameters on  RCS for vehicle with single scattering point

|  | 10lgσMσD (dBsm) |  |  |  |  |  |  |  | 10lgσM (dBsm) | σσS_dB (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ϕcenter in [°]ϕcenter | ϕ3dB, n in [°]ϕ3dB, n | θcenter in [°]θcenter | θ3dB,n in [°]θ3dB,n | GmaxGmax | σmaxσmax | Range of θ in [°]θ | Range of ϕ in [°]ϕ |  |  |
| Left | 90 | 26.90 | 79.70 | 44.42 | 20.75 | 13.68 | [30,180] | (45,135] | [] | 3.41 |
| Back | 180 | 36.32 | 79.65 | 36.73 | 14.56 | 7.50 | [30,180] | (135,225] |  |  |
| Right | 270 | 26.90 | 79.70 | 44.42 | 20.75 | 13.68 | [30,180] | (225,315] |  |  |
| Front | 0 | 40.54 | 71.75 | 29.13 | 15.52 | 8.45 | [30,180] | (-45, 45] |  |  |
| Roof | - | - | 0.00 | 18.13 | 21.26 | 14.19 | [0,30) | [0,360) |  |  |

Note: When θ is in the range [0, 30), σHdB φ=0.

Table 7.9.2.1-5: Parameters on RCS for vehicle with multiple scattering points

|  | 10lgσMσD (dBsm) |  |  |  |  |  |  |  | 10lgσM (dBsm) | σσS_dB (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ϕcenter in [°]ϕcenter | ϕ3dB, n in [°]ϕ3dB, n | θcenter in [°]θcenter | θ3dB,n in [°]θ3dB,n | GmaxGmax | σmaxσmax | Range of θ in [°]θ | Range of ϕ in [°]ϕ |  |  |
| Left | 90 | 26.90 | 79.70 | 44.42 | 20.60 | 20.52 | [0,180] | [0,360] | [] | 3.41 |
| Back | 180 | 36.32 | 79.65 | 36.73 | 13.90 | 13.82 | [0,180] | [0,360] |  |  |
| Right | 270 | 26.90 | 79.70 | 44.42 | 20.60 | 20.52 | [0,180] | [0,360] |  |  |
| Front | 0 | 40.54 | 71.75 | 29.13 | 14.99 | 14.91 | [0,180] | [0,360] |  |  |
| Roof | - | - | 0.00 | 18.13 | 21.12 | 21.05 | [0,180] | [0,360] |  |  |

Note: For the scattering point associated with roof of the vehicle, σHdBϕ=0.

Table 7.9.2.1-6:  Parameters on RCS for AGV with single scattering point

|  | 10lgσMσD (dBsm) |  |  |  |  |  |  |  | 10lgσM (dBsm) | σσS_dB (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ϕcenter in [°]ϕcenter | ϕ3dB, n in [°]ϕ3dB, n | θcenter in [°]θcenter | θ3dB,n in [°]θ3dB,n | GmaxGmax | σmaxσmax | Range of θ in [°]θ | Range of ϕ in [°]ϕ |  |  |
| Front | 0 | 13.68 | 90 | 13.68 | 13.02 | 23.29 | [30,180] | [45,135] | [] | 2.51 |
| Left | 90 | 15.53 | 75 | 20.03 | 7.33 | 17.60 | [30,180] | [135,225] |  |  |
| Back | 180 | 12.49 | 90 | 11.89 | 11.01 | 21.28 | [30,180] | [225,315] |  |  |
| Right | 270 | 15.53 | 75 | 20.03 | 7.33 | 17.60 | [30,180] | [-45,45] |  |  |
| Roof | / | / | 0 | 11.44 | 11.79 | 22.06 | [0,30] | [0,360] |  |  |

Note: When θ is in the range [0,30), σHdB φ=0.

[Table 7.9.2.1-7: Parameters on  RCS for AGV with multiple scattering points]

|  | 10lgσMσD (dBsm) |  |  |  |  |  |  |  | 10lgσM (dBsm) | σσS_dB (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ϕcenter in [°]ϕcenter | ϕ3dB, n in [°]ϕ3dB, n | θcenter in [°]θcenter | θ3dB,n in [°]θ3dB,n | GmaxGmax | σmaxσmax | Range of θ in [°]θ | Range of ϕ in [°]ϕ |  |  |

7.9.2.2 Cross-polarization matrix of a sensing target

The cross-polarization matrix CPMsp,i CPMof a SPST for a pair i of incident/scattered angles is generally modelled by amplitude factors αi,1,αi,2,βi,1,βi,2, and initial random phases Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ, i.e.,

CPMsp,i=αi,1expjΦsp,iθθβi,1expjΦsp,iθϕβi,2expjΦsp,iϕθαi,2expjΦsp,iϕϕ								(7.9-xx)

CPM

For UAV, human, vehicle or AGV, αi,1=αi,2=1, βi,1=βi,2=κsp,i-1, i.e.,

CPMsp,i=expjΦsp,iθθκsp,i-1expjΦsp,iθϕκsp,i-1expjΦsp,iϕθexpjΦsp,iϕϕ						(7.9-xx)

where,

- κsp,i is the XPR of the pair i of incident/scattered angles. κsp,i is randomly generated by log-normal distribution per target type defined in Table 7.9.2.2-1.

- Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ is uniformly distributed within -π,π

Table 7.9.2.2-1: Parameters on XPR (dB) for the targets

| Target | XPR | XPR |
| --- | --- | --- |
| UAV | 13.75 | 7.07 |
| Human | 19.81 | 4.25 |
| Vehicle | 21.12 | 6.88 |
| AGV | [] | [] |

7.9.3 Reference channel models and required updates

A transmitter or receiver in the sensing operation can be TRP, terrestrial UE, vehicle UE, aerial UE, AGV UE or RSU-type UE. The reference TR(s) for each combination of transmitter and receiver for each sensing scenario are provided in Table 7.9.3-1, where the terrestrial UE, AGV and RSU-type UE are referred as terrestrial UE.

Table 7.9.3-1: Reference TRs to generate channel for ISAC

| Case | Tx/Rx | Rx/Tx | Reference TR to define the channel model |
| --- | --- | --- | --- |
| 1 | TRP | TRP | For sensing scenario UMi, UMa, RMa, InH, InF, UMi-AV, UMa-AV, and RMa-AV: - TRP-TRP link of scenario UMi, UMa, InH, and InF following the option based on TR 38.901 defined in Clause A.3 of TR 38.858 - For InF, hUE is changed to the same height as the BS - TRP-UE link of scenario RMa defined in Clause 7 of TR 38.901 by setting hUE=35m (NOTE 1) For sensing scenario Highway - TRP-UE link of scenario RMa in section 7 of TR 38.901 by setting hUE=35m for FR1(NOTE 1) - TRP-TRP link of scenario UMa following the option based on TR 38.901 defined in section A.3 of TR 38.858 For sensing scenario Urban grid - TRP-TRP link of scenario UMa following the option based on TR 38.901 defined in section A.3 of TR 38.858 For sensing scenario HST - TRP-UE link of scenario RMa in section 7 of TR 38.901 by setting hUE=35m for FR1 (NOTE 1) - TRP-TRP link of scenario UMa following the option based on TR 38.901 defined in section A.3 of TR 38.858 for FR2 |
| 2 | TRP | terrestrial UE | For sensing scenario UMi, UMa, RMa, InH, InF, UMi-AV, UMa-AV, and RMa-AV - TRP-UE link of scenario UMi, UMa, RMa, InH, and InF in Clause 7 of TR 38.901 For sensing scenario Highway and Urban grid - P2B link of scenario Highway and Urban grid in Clause 6 of TR 37.885 For sensing scenario HST - TRP-UE link of scenario RMa in Clause 7 of TR 38.901 for FR1 and TRP-UE link of scenario UMa in Clause 7 of TR 38.901 for FR2 |
| 3 | TRP | vehicle UE | For sensing scenario Highway and Urban grid - V2B link of scenario Highway and Urban grid in Clause 6 of TR 37.885 For sensing scenario UMi, UMa, and RMa - TRP-UE link of scenario UMi, UMa, and RMa in Clause 7 of TR 38.901 |
| 4 | TRP | aerial UE | For sensing scenario UMa-AV, UMi-AV, and RMa-AV - TRP-aerial UE link of scenario UMa-AV, UMi-AV, and RMa-AV in Clause Annex A and B of TR 36.777 for FR1 - Reuse the channel model of scenario UMa-AV, UMi-AV, and RMa-AV of FR1 for FR2 |
| 5 | terrestrial UE | terrestrial UE | For sensing scenario UMi, UMa, RMa, InH, InF, UMi-AV, UMa-AV, and RMa-AV: - UE-UE link of scenario UMi, UMa, InH, and InF following the option based on TR 38.901 defined in Clause A.3 of TR 38.858 - TRP-UE link of scenario RMa defined in Clause 7 of TR 38.901 by setting hBS =1.5m (NOTE 2) For sensing scenario Highway and Urban grid - For pedestrian type UE: - P2P link in section 6 of TR 37.885 For sensing scenario HST - TRP-UE link of scenario RMa in section 7 of TR 38.901 for FR1, e.g., hBS=1.5m, UE-UE link of scenario UMa following the option based on TR 38.901 defined in section A.3 of TR 38.858 for FR2 |
| 6 | terrestrial UE | vehicle UE | For sensing scenario UMi, UMa, RMa - UE-UE link of scenario UMi, UMa following the option based on TR 38.901 defined in section A.3 of TR 38.858 - TRP-UE link of scenario RMa defined in section 7 of TR 38.901 by setting hBS =1.5m For sensing scenario Highway and Urban grid - For pedestrian type UE: - V2P link in section 6 of TR 37.885 |
| 7 | terrestrial UE | aerial UE | For sensing scenario UMi-AV, UMa-AV, and RMa-AV - TRP-aerial UE link of UMi-AV in Annex A and B of TR 36.777 by setting hBS =1.5m for FR1 - LOS probability is not reused, FFS new LOS probability - FFS pathloss model, shadowing fading - Reuse the channel model of scenario UMa-AV, UMi-AV, and RMa-AV of FR1 for FR2 - The corresponding parameter values in FR2 are used |
| 8 | vehicle UE | vehicle UE | For sensing scenario Highway and Urban grid - V2V link of scenario Highway and Urban grid in section 6 of TR 37.885 For sensing scenario UMi, UMa, and RMa - UE-UE link of scenario UMi, UMa following the option based on TR 38.901 defined in section A.3 of TR 38.858 - TRP-UE link of scenario RMa defined in section 7 of TR 38.901 by setting hBS =1.5m |
| 9 | aerial UE | aerial UE | For sensing scenario UMi-AV, UMa-AV, RMa-AV - TRP-aerial UE link of UMi-AV in Annex A and B of TR 36.777 by setting height of TRP equal to the height of the first aerial UE for FR1 - LOS probability is not reused, FFS new LOS probability - FFS pathloss model, shadowing fading, angular spread - Reuse the channel model of scenario UMa-AV, UMi-AV, and RMa-AV of FR1 for FR2 - The corresponding parameter values in FR2 are used |

NOTE 1: ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0

NOTE 2: ASD and ZSD statistics updated to be the same as ASA and ZSA.

The large scale and small scale parameters of the STX-ST link and the ST-SRX link for a sensing scenario are determined according to Table 7.9.3-1 assuming the same sensing scenario. The targets of UAV, vehicle are respectively considered as an aerial UE, vehicle UE. Other targets are considered as terrestrial UE. The proper case for each combination of STX/STX and ST are provided in Table 7.9.3-2.

Table 7.9.3-2: Channel model for STX-ST and ST-SRX link

| STX/SRX | Target | Cases in Table 7.9.3-1 assuming the same sensing scenario |
| --- | --- | --- |
| TRP | UAV | Case 4: TRP-Aerial UE link |
|  | Human | Case 2: TRP-normal UE link |
|  | Vehicle | Case 3: TRP-Vehicle UE link |
|  | AGV | Case 2: TRP-normal UE link |
|  | Object (Hazard) | Case 2: TRP-normal UE link |
| Terrestrial UE | UAV | Case 7: normal UE - Aerial UE link |
|  | Human | Case 5: normal UE-normal UE link |
|  | Vehicle | Case 6: normal UE - Vehicle UE link |
|  | AGV | Case 5: normal UE-normal UE link |
|  | Object (Hazard) | Case 5: normal UE-normal UE link |
| Vehicle UE | Human | Case 6: normal UE - Vehicle UE link |
|  | Vehicle | Case 8: Vehicle UE - Vehicle UE link |
|  | Object (Hazard) | Case 6: normal UE - Vehicle UE link |
| Aerial UE | UAV | Case 9: Aerial UE - Aerial UE link |
| AGV UE | AGV | Case 5: normal UE - normal UE link |

The large scale and small scale parameters of the STX-SRX link, i.e., background channel for a sensing scenario are determined according to Table 7.9-1 assuming the same sensing scenario. The proper case for each combination of STX and SRX are provided in Table 7.9-3.

Table 7.9.3-3: Channel model for background channel

| STX/SRX | SRX/STX | Cases in Table 7.9.3-1 assuming the same sensing scenario |
| --- | --- | --- |
| TRP | TRP | Case 1: TRP-TRP link |
|  | Terrestrial UE | Case 2: TRP-normal UE link |
|  | Vehicle UE | Case 3: TRP-Vehicle UE link |
|  | Aerial UE | Case 4: TRP-Aerial UE link |
|  | AGV UE | Case 2: TRP-normal UE link |
| Terrestrial UE | Terrestrial UE | Case 5: normal UE-normal UE link |
|  | Vehicle UE | Case 6: normal UE - Vehicle UE link |
|  | Aerial UE | Case 7: normal UE - Aerial UE link |
|  | AGV UE | Case 5: normal UE -normal UE link |
| Vehicle UE | Vehicle UE | Case 8: Vehicle UE - Vehicle UE link |
| Aerial UE | Aerial UE | Case 9: Aerial UE - Aerial UE link |
| AGV UE | AGV UE | Case 5: normal UE - normal UE link |

The following updates to the reference TRs are identified to generate ISAC channel.

- For human as a sensing target with a single scattering point, the height of the scattering point is 1.5 m.

- For sensing scenario UMi, UMa, RMa, UMi-AV, UMa-AV and RMa-AV, the height of a scattering point of a target is used to calculate the LOS probability and pathloss, regardless of the lower bound of hUT in the reference TRs that are used to generate ISAC channel.

7.9.4	Fast fading model

7.9.4.0	Introduction

1≤k≤K

A stepwise procedure illustrated in Figure 7.9.4-1 is used to generate the channel model for ISAC referring to the procedure in Clause 7.5 with parameters derived by Table 7.9.3-2 and 7.9.3-3. Step 1 is commonly executed, followed by the respective steps for target channel (7.9.4.1) and background channel (7.9.4.2). Finally, the target channel and background channel are combined to form the final channel model for ISAC (7.9.4.3).

Figure 7.9.4-1 Channel coefficient generation procedure

Step 1: Set environment, network layout, and antenna array parameters

a)	Choose one of the sensing scenarios (ISAC-UAV, ISAC-Automative, ISAC-Human, ISAC-AGV, ISAC-Objects creating hazardsHaz) and related communication scenarios (e.g. UMa, UMi-Street Canyon, RMa, InH-Office, Urban grid, Highway, or InF). Choose a global coordinate system and define zenith angle θ, azimuth angle ϕ, and spherical basis vectors θ, ϕ as shown in Figure 7.5-2. Note: Scenario RMa is for up to 7GHz while others are for up to 100GHz.

b)	Give number of STX, ST and SRX. Give the number of the SPST(s) of each ST.

c)	Give 3D locations of STX and SRX, give 3D locations of SPST(s) of each ST, and determine LOS AOD (ϕLOS,AOD), LOS ZOD (θLOS,ZOD), LOS AOA (ϕLOS,AOA), and LOS ZOA (θLOS,ZOA) of each pair of STX and SPST, and each pair of SPST and SRX in the global coordinate system

d)	Give STX and SRX antenna field patterns Frx and Ftx in the global coordinate system and array geometries.

e)	Give STX and SRX array orientations with respect to the global coordinate system. STX array orientation is defined by three angles ΩSTX,α (STX bearing angle), ΩSTX,β (STX downtilt angle) and ΩSTX,γ (STX slant angle). SRX array orientation is defined by three angles ΩSRX,α (SRX bearing angle), ΩSRX,β (SRX downtilt angle) and ΩSRX,γ (SRX slant angle). Give the orientation of ST in the global coordinate system. ST orientation is defined by three angles ΩST,α (ST bearing angle), ΩST,β (ST downtilt angle) and ΩST,γ (ST slant angle).

f)	Give speed and direction of motion of STX, ST and SRX in the global coordinate system

g)	Specify system centre frequency fc and bandwidth B

Note:	In case wrapping is used, each wrapping copy of a STX or SRX should be treated as a separate STX/SRX considering channel generation.

For monostatic sensing mode, a STX and a corresponding SRX have the same 3D locations, antenna field pattern, array orientations, speed and direction of motion.

7.9.4.1	Target channel

Following Step 1 in Clause 7.9.4.0, the target channels for the one or multiple STs between a pair of STX and SRX is generated using the following procedure with parameters derived by Table 7.9.3-2. It assumes a ST k consists of P scattering points,  P≥1.

Large scale parameters:

Step 2: Assign propagation condition (LOS/NLOS) for each pair of STX and SPST, and each pair of SPST and SRX according to Table 7.4.2-1 updated as necessary in Clause 7.9.3..

Step 3: Calculate pathloss with formulas in Table 7.4.1-1 updated as necessary in Clause 7.9.3 for each STX-SPST link, and each SPST-SRX link.

Step 4: For each STX-SPST link and SPST-SRX link, generate large scale parameters, e.g. delay spread (DS), angular spreads (ASA, ASD, ZSA, ZSD), Ricean K factor (K) and shadow fading (SF) taking into account cross correlation according to Table 7.5-6 and using the procedure described in clause 3.3.1 of [14] with the square root matrixCMxM(0) being generated using the Cholesky decomposition and the following order of the large scale parameter vector: sM = [sSF, sK, sDS, sASD, sASA, sZSD, sZSA]T.

The LSPs for links from co-sited sectors to a STX/SPST/SRX are the same. In addition, these LSPs for the links of STX/SPST/SRX on different floors are uncorrelated.

Limit random RMS azimuth arrival and azimuth departure spread values to 104 degrees, i.e., ASA= min(ASA, 104), ASD = min(ASD, 104). Limit random RMS zenith arrival and zenith departure spread values to 52 degrees, i.e., ZSA = min(ZSA, 52), ZSD = min(ZSD, 52).

For monostatic sensing mode, the large scale parameters generated in step 2-4 are identical for a STX-SPST link and the corresponding SPST-SRX link of same SPST.

Small scale parameters:

Step 5: Generate cluster delays

The cluster delays of cluster n in a STX-SPST link are generated using Step 5 of Clause 7.5, i.e., τtx,nk,p=τn.

The cluster delays of cluster n' in the corresponding SPST-SRX link of same SPST are generated using Step 5 of Clause 7.5 by replacing subscript n with n’, i.e., τrx,n'k,p=τn'.

Step 6: Generate cluster powers.

The cluster power of cluster n in a STX-SPST link are generated using Step 6 fo Clause 7.5, i.e., Ptx,nk,p=Pn.

The cluster power of cluster n' in the corresponding SPST-SRX link of same SPST  are generated using Step 6 of Clause 7.5 by replacing subscript n with n’, i.e., Prx,n'k,p=Pn'.

Step 7: Generate arrival angles and departure angles for both azimuth and elevation.

The arrival angles and departure angles for both azimuth and elevation of a cluster n in a STX-SPST link are generated using Step 7 of Clause 7.5, i.e., ϕtx,n,m,AOAk,p=ϕn,m,AOA, ϕtx,n,m,AODk,p=ϕn,m,AOD, θtx,n,m,ZOAk,p=θn,m,ZOA, θtx,n,m,ZODk,p=θn,m,ZOD.

The arrival angles and departure angles for both azimuth and elevation of a cluster n’ in the corresponding SPST-SRX link of same SPST  are generated using Step 7 of Clause 7.5 by replacing subscript n, m with n’, m’, i.e., ϕrx,n',m',AOAk,p=ϕn',m',AOA, ϕrx,n',m',AODk,p=ϕn',m',AOD, θrx,n',m',ZOAk,p=θn',m',ZOA, θrx,n',m',ZODk,p=θn',m',ZOD.

For monostatic sensing mode, τtx,nk,p,Ptx,nk,p,ϕtx,n,m,AOAk,p,ϕtx,n,m,AODk,p,θtx,n,m,ZOAk,p,θtx,n,m,ZODk,p are respectively equal to τrx,n'k,p,Prx,n'k,p,ϕrx,n',m',AOAk,p, ϕrx,n',m',AODk,p,ϕrx,n',m',AOAk,p θrx,n',m',ZOAk,p,θrx,n',m',ZODk,p, θrx,n',m',ZOAk,p in Step 5-7, if n=n' and if applicable m=m'.

Step 8: Coupling of rays within a cluster for both azimuth and elevation.

The rays within a cluster n for both azimuth and elevation in a STX-SPST link are coupled using Step 8 of Clause 7.5.

The rays within a cluster n’ for both azimuth and elevation in the corresponding SPST-SRX link of same SPST  are coupled using Step 8 of Clause 7.5 by replacing subscript n, m with n’, m’.

For monostatic sensing mode, the same ray-coupling way apples to both the STX-SPST link and the SPST-SRX link.

Step 9: Coupling of rays for a STX-SPST link and the corresponding SPST-SRX link of same SPST.

In the STX-SPST link, a LOS ray is represented by n=0,m=0, while a NLOS ray m of a stochastic cluster n is represented by n>0,m>0. In the SPST-SRX link, a LOS ray is represented by n'=0,m'=0, while a NLOS ray m’ of a stochastic cluster n’ is represented by n'>0,m'>0.

A ray in the STX-SPST link and a ray in the corresponding SPST-SRX link that are coupled to each other construct a path.

The LOS ray (if present) in the STX-SPST link is coupled with the LOS ray (if present) in the SPST-SRX link. The resulting path is never dropped.

Further, a set R0 of paths are generated.

- The LOS ray in the STX-SPST link (if present) is coupled with each NLOS ray in the SPST-SRX link.

- Each NLOS ray in the STX-SPST link is coupled with the LOS ray (if present) in the SPST-SRX link.

- The NLOS rays in the STX-SPST link are coupled with the NLOS rays in the SPST-SRX link. Two Options for the coupling are recommended with other methods for complexity reduction up to company choice.

  - Option 1: Each NLOS ray in the STX-SPST link is coupled with each NLOS ray in the SPST-SRX link.

  - Option 2: The NLOS rays in the STX-SPST link are 1-by-1 randomly coupled with the NLOS rays in the ST-SRX link. If the number of rays in the two links M1, M2 are not equal, min(M1, M2) rays are randomly selected in the link with larger number of rays in the coupling operation.

Step 10: Obtain the power for all generated paths

The power of a path (k,p,n',m',n,m) is given by

Pn',m',n,mk,p=σD, n',m',n,mk,pminσS,n',m',n,mk,p,103σσS_dBμσσS_dB+3σσS_dB10Prx,n',m'k,pPtx,n,mk,p					(7.9-xx)

where,

σD, n',m',n,mk,p, σS,n',m',n,mk,p are the second and third component of RCS at the SPST p for a path are derived by the incident angle, i.e., AOA (ϕtx,n,m,AOAk,p), ZOA (θtx,n,m,ZOAk,p) of the ray in the STX-SPST link and the scattered angle, i.e., AOD (ϕrx,n',m',AODk,p), ZOD (θrx,n',m',ZODk,p) of the ray in the SPST-SRX link, referring to the RCS model defined in Clause 7.9.2.1 for the SPST. Note: the transformation between GCS and LCS for the incident/scattered angles is necessary to determine the RCS at the SPST p. Prx,n',m'k,p,Ptx,n,mk,p are respectively the ray power in the SPST-SRX link and the STX-SPST link.

- If the STX-SPST link is in LOS condition,

  - for a LOS ray,  Ptx,0,0k,p=Ktx,Rk,pKtx,Rk,p+1, Ktx,Rk,p is the Ricean K-factor of the STX-SPST link.

  - for a NLOS ray m of a stochastic cluster n,  Ptx,n,mk,p=1Ktx,Rk,p+1Ptx,nk,pM, M is number of rays per cluster

- If the STX-SPST link is not in LOS condition,

  - for a NLOS ray m of a stochastic cluster n, Ptx,n,mk,p=Ptx,nk,pM, M is number of rays per cluster

- If the SPST-SRX link is in LOS condition,

  - for a LOS ray, Prx,0,0k,p=Krx,Rk,pKrx,Rk,p+1, Krx,Rk,p is the Ricean K-factor of the SPST-SRX link.

  - for a NLOS ray m’ of a stochastic cluster n’, Prx,n',m'k,p=1Krx,Rk,p+1Prx,n'k,pM

- If the SPST-SRX link is not in LOS condition,

  - for a NLOS ray m’ of a stochastic cluster n’, Prx,n',m'k,p=Prx,n'k,pM

A path with power less than [-40dB] compared to the maximum power among all paths in set R0 is dropped. The set of remaining paths (k,p,n',m',n,m) after path dropping is denoted as set R. The path that couples the LOS ray in the STX-SPST link and the LOS ray in the SPST-SRX link, if present, is included in set R.

Step 11: Obtain the absolute delay for each path in set R

The absolute delay of a path (k,p,n',m',n,m) is given by

τn',m',n,mk,p=τrx,n',m'k,p+drx,3Dk,pc+∆τrxk,p+τtx,n,mk,p+dtx,3Dk,pc+∆τtxk,p 						(7.9-xx)

where, for the LOS ray in the STX-SPST link, if present, τtx,0,0k,p=0. For the LOS ray in the SPST-SRX link, if present, τrx,0,0k,p=0. dtx,3Dk,p is the 3D distance between STX and SPST p. drx,3Dk,p is the 3D distance between SPST p and SRX. ∆τrxk,p,∆τtxk,p are generated separately for NLOS rays in the SPST-SRX link and the STX-SPST link for bistatic sensing mode, according to Clause 7.6.9. For the two strongest clusters in each of the STX-SPST link and SPST-SRX link, τrx,n',m'k,p and τtx,n,mk,p of sub-clusters are revised according to equation (7.5-26). For monostatic sensing mode, ∆τrxk,p is equal to ∆τtxk,p. ∆τrxk,p,∆τtxk,p are set to 0 for the LOS ray, if present, in the SPST-SRX link and the STX-SPST link.

Step 12: Generate the cross polarization power ratios for paths in set R.

The cross polarization power ratios for each ray m of a cluster n in a STX-SPST link is generated using Step 9 of Clause 7.5, i.e., κtx,n,mk,p=κn,m.

The cross polarization power ratios for each ray m’ of a cluster n’ in a ST-SRX link is generated using Step 9 of Clause 7.5 by replacing subscript n, m with n’, m’, i.e., κrx,n',m'k,p=κn',m'.

For monostatic sensing mode, κtx,n,mk,p is equal to κrx,n',m'k,p if n=n' and m=m'.

Generate the cross polarization power ratios (XPR) κn',m',m,nk,pfor each path (k,p,n',m',n,m) in set R at SPST p. XPR is log-Normal distributed. Draw XPR values as

κn',m',m,nk,p=10Xn',m',m,nk,p/10,													(7.9-xx)

where Xn',m',m,nk,p~N(μXPR,σXPR2) is Gaussian distributed with μσXPRand σμXPR from Table 7.9.2.2. Note: Xn,m is independently drawn for each path in set R.

The outcome of Steps 1-12 shall be identical for all the links from co-sited sectors to a STX/ST/SRX.

Coefficient generation:

Step 13: Draw initial random phases for paths in set R.

The random initial phases for each ray m of a cluster n in a STX-ST link is generated using Step 9 of Clause 7.5, i.e., Φtx,n,mk,p,θθ,Φtx,n,mk,p,θϕ,Φtx,n,mk,p,ϕθ,Φtx,n,mk,p,ϕϕ=Φn,mθθ,Φn,mθϕ,Φn,mϕθ,Φn,mϕϕ

The random initial phases for each ray m’ of a cluster n’ in a ST-SRX link is generated using Step 9 of Clause 7.5 by replacing subscript n, m with n’, m’, i.e., Φrx,n',m'k,p,θθ,Φrx,n',m'k,p,θϕ,Φrx,n',m'k,p,ϕθ,Φrx,n',m'k,p,ϕϕ=Φn',m'θθ,Φn',m'θϕ,Φn',m'ϕθ,Φn',m'ϕϕ.

[For monostatic sensing mode, Φtx,n,mk,p,θθ,Φtx,n,mk,p,θϕ,Φtx,n,mk,p,ϕθ,Φtx,n,mk,p,ϕϕ are respectively equal to Φrx,n',m'k,p,θθ,Φrx,n',m'k,p,θϕ,Φrx,n',m'k,p,ϕθ, Φrx,n',m'k,p,ϕϕ if n=n' and m=m'.]

Draw random initial phases Φn',m',m,nk,p,θθ,Φn',m',m,nk,p,θϕ,Φn',m',m,nk,p,ϕθ,Φn',m',m,nk,p,ϕϕ for each path  (k,p,n',m',n,m) in set R at SPST p and for four different polarisation combinations (θθ, θϕ, ϕθ, ϕϕ). The distribution for initial phases is uniform within (-π, π).

Step 14: Generate channel coefficients for paths in set R for each receiver and transmitter element pair u, s.

The channel coefficient for a path (k,p,n',m',n,m) in set R is generated by

Hu,s,n''e,m',n,mk,pt=Pn',m',n,mk,pFrx,u,θθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pFrx,u,ϕθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pTCPMrx,n',m'k,pCPMn',m',n,mk,pCPMtx,n, mk,pd112+d2222

∙Ftx,s,θθtx,n,m,ZODk,p,ϕtx,n,m,AODk,pFtx,s,ϕθtx,n,m,ZODk,p,ϕtx,n,m,AODk,pexpj2πrrx,k,p,n',m'Tt.drx,u+rtx,k,p,n,mTt.dtx,sλ0expj2πt0tfD,n',m',n,mk,ptdt

(7.9-xx)

with the Doppler frequency fD,n',m',n,mk,pt defined as

fD,n',m',n,mk,pt=rrx,k,p,n',m'Ttvrxt+rk,p,n',m'Ttvk,ptλ0+rtx,k,p,n,mTtvtxt+rk,p,n,mTtvk,ptλ0			(7.9-xx)

where,

- CPMn',m',n,mk,p is the polarization matrix of the SPST p.

- CPMn',m',n,mk,p=expjΦn',m',m,nk,p,θθκn',m',n,mk,p-1expjΦn',m',m,nk,p,θϕκn',m',n,mk,p-1expjΦn',m',m,nk,p,ϕθexpjΦn',m',m,nk,p,ϕϕ		(7.9-xx)

- CPMtx,n, mk,p is the polarization matrix of the ray in the STX-SPST link.

  - for the LOS ray, if present,   CPMtx,n0, m0k,p=100-1

  - for NLOS ray generated by stochastic cluster,

CPMtx,n, mk,p=expjΦtx,n,mk,p,θθκtx,n,mk,p-1expjΦtx,n,mk,p,θϕκtx,n,mk,p-1expjΦtx,n,mk,p,ϕθexpjΦtx,n,mk,p,ϕϕ				(7.9-xx)

- CPMrx,n',m'k,p is the polarization matrix of th ray in the SPST-SRX link.

  - for the LOS ray, if present, CPMrx,n'0,m'0k,p=100-1

  - for NLOS ray generated by stochastic cluster,

CPMrx,n',m'k,p=expjΦrx,n',m'k,p,θθκrx,n',m'k,p-1expjΦrx,n',m'k,p,θϕκrx,n',m'k,p-1expjΦrx,n',m'k,p,ϕθexpjΦrx,n',m'k,p,ϕϕ				(7.9-xx)

- d11,d22 are the two elements in the main diagonal of matrix CPMrx,n',m'k,pCPMn',m',n,mk,pCPMtx,n, mk,p

- rrx,k,p,n',m'T is the spherical unit vector at receiver for the link from SRX to SPST p of ST k, given by

rrx,k,p,n',m'rrx,k,p,n',m'=sinθrx,n',m',ZOAk,pcosϕrx,n',m',AOAk,psinθrx,n',m',ZOAk,psinϕrx,n',m',AOAk,pcosθrx,n',m',ZOAk,p									(7.9-xx)

- rtx,k,p,n,mT  is the spherical unit vector at transmitter for the link from STX to SPST p of ST k, given by

rtx,k,p,n,mrtx,k,p,n,m=sinθtx,n,m,ZODk,pcosϕtx,n,m,AODk,psinθtx,n,m,ZODk,psinϕtx,n,m,AODk,pcosθtx,n,m,ZODk,p									(7.9-xx)

- rk,p,n',m'T is the spherical unit vector at the scattering point for the link from SPST p of ST k to SRX, given by

rk,p,n',m'rk,p,n',m'=sinθrx,n',m',ZODk,pcosϕrx,n',m',AODk,psinθrx,n',m',ZODk,psinϕrx,n',m',AODk,pcosθrx,n',m',ZODk,p								(7.9-xx)

- rk,p,n,mT is the spherical unit vector at the scattering point for the link from SPST p of ST k to STX, given by

rk,p,n,mrk,p,n,m=sinθtx,n,m,ZOAk,pcosϕtx,n,m,AOAk,psinθtx,n,m,ZOAk,psinϕtx,n,m,AOAk,pcosθtx,n,m,ZOAk,p								(7.9-xx)

- vrxt,vtxt are respectively the velocity of SRX, STX

- vk,pt is the velocity of SPST p of ST k, vk,pt=vma,kt+vmi, k, pt, where vma,kt is the velocity of the ST k, vmi, k, pt is velocity due to micro motion of SPST p of ST k

- t0 denotes a reference point in time that defines the initial phase, e.g. t0=0.

The channel impulse response of SPST p of ST k is given by

Hu,sk,pτ,t=k,p,n',m',n,m∈RHu,s,n',m',n,mk,pt δτ-τn',m',n,mk,p						(7.9-xx)

Step 15: Apply pathloss, shadowing, the first component of RCS for the channel coefficients.

The target channel for ST k, Hu,s(k)τ,t is formed by summing the channel coefficients of the P SPST respectively scaled by the pathloss, shadowing and the first component of RCS σM of the P SPST.

Hu,s(k)τ,t=p10-PLtx,k,p+PLrx,k,p+SFtx,k,p+SFrx,k,p20∙4πσMk,pλ02λ0∙Hu,s(k,p)τ,t				(7.9-xx)

Where,

- PLtx,k,p is the pathloss of the STX-SPST link

- PLrx,k,p is the pathloss of the SPST-SRX link

- SFtx,k,p is the shadow fading of the STX-SPST link

- SFrx,k,p is the shadow fading of the SPST-SRX link

- σMk,p is the first component of RCS for the SPST p of ST k, referring to the RCS model defined in Clause 7.9.2.1,

- p=0,1,..P-1, P is the total number of SPST of the ST k.

7.9.4.2	Background channel

For TRP-TRP, TRP-UE, UE-TRP and UE-UE bistatic sensing modes, following Step 1 in Clause 7.9.4.0, the background channel between a pair of STX and SRX is generated using Step 2 to Step12 of Clause 7.5 with parameters derived by Table 7.9.3-3. The absolute time of arrival in clause 7.6.9 is applied. The resulting channel is denoted as Hu,sbkτ,t.

For TRP monostatic and UE monostatic sensing modes, the background channel between a pair of STX and SRX is generated using the following steps after Step 1 in Clause 7.9.4.0.

Step 2: Generate 3 reference points (RPs) for the STX/SRX

Draw the 2D distance between the STX/SRX and each RP, the height of each RP respectively from Gamma distribution Γαd,βd+cd and Γαh,βh+ch defined in Table 7.9.4.2-1/2. With uniform distribution within range -π,π, draw the LOS AOD (ϕLOS,AOD) between the STX/SRX and the first RP. The LOS AOD is further rotated by 23π and 43π to respectively derive the LOS AOD from the STX/SRX to the second and third RPs. Consequently, the 3D location of each RP can be calculated.

- Determine LOS ZOD (θLOS,ZOD), LOS AOA (ϕLOS,AOA), and LOS ZOA (θLOS,ZOA) of the STX/SRX and each RP in the global coordinate system.

- Set each RP the same antenna field patterns Frx in the global coordinate system and array geometries as the STX/SRX

- Set each RP the same array orientations with respect to the global coordinate system, i.e., ΩRP,α (RP bearing angle), ΩRP,β (RP downtilt angle) and ΩRP,γ (RP slant angle) as the STX/SRX.

- Set each RP the same velocity as the STX/SRX.

Note: Distributions of height and distance of RPs are not subject to geographical constraints on TRP and UT given in TR 38.901 for the corresponding deployment scenario.

Step 3: Assign NLOS propagation condition to the channel between the STX/SRX and each of the 3 RPs.

Step 4: Generate the channel between the STX/SRX and each of the 3 RPs using Step 3 to Step12 of Clause 7.5 with parameters derived by Table 7.9.3-3, unless stated otherwise.

- In Step 7 of Clause 7.5, the arrival angles ϕn,m,AOA,θn,m,ZOA are respectively equal to departure angles ϕn,m,AOD, θn,m,ZOD. The rays in a cluster with θn,m,ZOA less than [50, 60 and 90] degrees respectively for scenario UMi, UMa and RMa are dropped. The dropping is not applicable to other sensing scenarios.

- The absolute time of arrival [in clause 7.6.9] is applied [with the following updates]. The same ∆τ is generated and applied to the 3 channels between the STX/SRX and the 3 RPs.

Note: In the UT monostatic sensing in UMa and UMi scenario, the ZOD offset should be set as 0.

Step 5: Generate the background channel for the STX/SRX

The background channel for the STX/SRX Hu,sbkτ,t is formed by summing the 3 channels between STX/SRX and the 3 RPs after applying the respective pathloss and shadowing.

Hu,sbkτ,t=r=0210-PLrx,txbk,r-+SFrx,txbk,r20∙Hu,sbk,rτ,t

Where,

- PLrx,txbk,r is the pathloss between the STX/SRX and the RP with index r

- SFrx,txbk,r is the shadow fading between the STX/SRX and the RP with index r

Table 7.9.4.2-1: Parameters of background channel for TRP monostatic sensing

| Scenarios |  | UMi | UMa / Urban grid / Highway(FR2) / HST(FR2) | RMa / Highway(FR1) / HST(FR1) | Indoor office | Indoor Factory |
| --- | --- | --- | --- | --- | --- | --- |
| Distance | αd | 6.1996 | 10.3370 | 6.2025 | 4.236 | 0.039836 |
|  | βd | 0.1558 | 0.1317 | 0.0391 | 0.19255 | 0.179783 |
|  | cd | 15.2697 | 68.7778 | 1.2940 | 4.99 | 1.130020 |
| Height | αh | 12.0487 | 16.2253 | 0.0007 | 1.3293 | 0.283447 |
|  | βh | 2.3261 | 1.9218 | 5.0146 | 0.1442 | 0.435965 |
|  | ch | 0.0157 | 2.6142 | 0.0522 | -13.19 | -17.043530 |
| Threshold D for ZOA |  | 50 | 80 | 90 | N/A | N/A |

Table 7.9.4.2-2 Part-1: Parameters of background channel for UT monostatic sensing

| Scenarios |  | UMi | UMa / Urban grid / Highway(FR2) / HST(FR2) | RMa / Highway(FR1) / HST(FR1) | Indoor office | Indoor Factory |
| --- | --- | --- | --- | --- | --- | --- |
| Distance | αd | 10.0220 | 2.9072 | 10.2421 | 4.3733 | 0.231418 |
|  | βd | 1.2522 | 0.1031 | 0.0526 | 0.4457 | 0.128133 |
|  | cd | 11.0040 | 3.8471 | 3.3131 | 4.6302 | 2.004903 |
| Height | αh | 3.0487 | 1.6640 | 0.3175 | 0.2974 | 0.462968 |
|  | βh | 1.9128 | 1.6215 | 1.4150 | 0.4103 | 0.281526 |
|  | ch | 0.1785 | -1.4205 | 1.5906 | 2.9711 | -16.921515 |
| Threshold D for ZOA |  | 50 | 80 | 90 | N/A | N/A |

Table 7.9.4.2-2 Part-2: Parameters of background channel for UT monostatic sensing

| Scenarios |  | UMi-AV | UMa-AV | RMa-AV |
| --- | --- | --- | --- | --- |
| Distance | αd | 0.83+0.00015h |  |  |
|  | βd | 1536.305+1.0279h |  |  |
|  | cd | 13.824+0.03085h |  |  |
| Height | αh | 0.9054-0.0001117h |  |  |
|  | βh | 138.672-0.04658h |  |  |
|  | ch | 25.4898-0.02398h |  |  |
| Threshold D for ZOA |  |  |  |  |

7.9.4.3	Combining target channel and background channel

The channel model for ISAC for a pair of STX and STX is the sum of the target channel(s) and the background channel generated in Clause 7.9.4.1 and 7.9.4.2, i.e.,

HISAC=kHu,s(k)τ,t+Hu,sbkτ,t.											(7.9-xx)

7.9.5	Additional modelling components

7.9.5.1	Spatial consistency

The spatial consistency procedure is used to generate the random variables for the STX-SPST links, the SPST-SRX links and the background channels, so that they are spatial consistent. The spatial consistency procedures in Clause 7.6.3 are reused to handle the links between TRPs and STs/UTs.

For sensing scenario UMi-AV, UMa-AV and RMa-AV, the 2D random process (in the horizontal plane) can be extended to 3D random process [with vertical correlation distance]. The random coupling of NLOS rays of Option 2 in Step 9 in Clause 7.9.4.1 shall be kept unchanged per simulation drop even if STX/ST/SRX position(s) changes during simulation.

A new spatial consistency procedure is provided for the large scale parameter, cluster specific parameters and ray specific parameters of the links between UT and UT/SPSTs (denoted as UT-UT/SPST links).

- Link-correlated: parameters of any two links of UT-UT/SPST links are correlated, subjected to correlation distance.

- All-correlated: all UT-UT/SPST links are correlated.

In Table 7.9.5.1-1, correlation type for each parameter of the new spatial consistency procedure is clarified.

Table 7.9.5.1-1: Correlation type among UT-UT/SPST links

| Parameters | Correlation type |
| --- | --- |
| Delays | Link-correlated |
| Cluster powers | Link-correlated |
| AOA/ZOA/AOD/ZOD offset | Link-correlated |
| AOA/ZOA/AOD/ZOD sign | Link-correlated |
| Random coupling | Link-correlated |
| XPR | Link-correlated |
| Initial random phase | Link-correlated |
| LOS/NLOS states | Link-correlated |
| Blockage (Model A) | All-correlated |
| O2I penetration loss | All-correlated |
| Indoor distance | All-correlated |
| Indoor states | All-correlated |

Spatial consistency is not modelled among STX-SPST links, the SPST-SRX links and the background channels in the following situations:

-	Different link types, e.g., outdoor LOS, outdoor NLOS or O2I, as defined in Table 7.6.3.4-2

-	UE locates on different floors, as defined in Table 7.6.3.4-2

-	Background channel for monostatic sensing mode, and any link (STX-SPST link, SPST-SRX link, background channel) for bistatic sensing mode

-	Links associated with different non-co-located TRPs, e.g. TRP1-ST/UT/TRP and TRP2-ST/UT/TRP.

7.9.5.2	Type-2 environment object

A type-2 EO, e.g., wall, is modelled as a surface with finite size. A ray specularly reflected by a type-2 EO can be modelled in the STX-SPST link, SPST-SRX link [and/or the background channel], if a specular reflection point can be found within the surface of the type-2 EO.

A general procedure to generated small scale parameters of a type-2 EO, e.g., a wall, is provided assuming the incident ray comes from Tx and the scattered ray arrives at Rx.

Step A: Check whether a ray specularly reflected by a type-2 EO needs to be modelled or not for the pair of Tx and Rx

The type-2 EO can be described by a plane equation perpendicular to the ground.

Ax+By=1, x∈xleft, xright, y∈yleft, yright,z∈[zleft, zright]					(7.9-xx)

[A, B] is the normal vector of the plane, which points to the same direction as the orientation of the type-2 EO. The location of Tx and Rx is denoted as xtx,ytx,ztx and xrx,yrx,zrx.

If Axtx+Bytx-1≤0 or Axrx+Byrx-1≤0, there is no reflection path in the Tx-Rx link due to the type-2 EO. Otherwise, the location of reflection point xw,yw,zw is determined as follows.

xw=xtx'+xrx-xtx'(1-Axtx'-Bytx')Axrx-xtx'+B(yrx-ytx')											(7.9-xx)
yw=ytx'+yrx-ytx'(1-Axtx'-Bytx')Axrx-xtx'+B(yrx-ytx')											(7.9-xx)
zw=ztx'+zrx-ztx'(1-Axtx'-Bytx')Axrx-xtx'+B(yrx-ytx')  											(7.9-xx)

Where, xtx',ytx',ztx' is the location of the reflection image of Tx relative to the plane. xtx'=xtx-2AAxtx+Bytx-1A2+B2.  ytx'=ytx-2BAxtx+Bytx-1A2+B2. ztx'=ztx.

If  xw∈xleft, xright, yw∈yleft, yright,zw∈[zleft, zright], the reflection point is on the surface of the type-2 EO, otherwise, there is no reflection path in the Tx-Rx link due to the type-2 EO.

Step B: Generate parameters of the ray specularly reflected by a type-2 EO if the reflection point is only the surface of the type-2 EO

The 3D distance from the Tx to the reflection point then to the Rx dEO is

dEO=(xtx'-xrx)2+(ytx'-yrx)2+(ztx'-zrx)2						(7.9-xx)

The ZoD and AoD from Tx to the reflection point are derived based on the location of Tx and reflection point.

θEO, ZOD=arccos⁡(zw-ztx(xw-xtx)2+(yw-ytx)2+(zw-ztx)2)							(7.9-xx)

ϕEO, AOD=arctanyw-ytxxw-xtxmod 180° 									(7.9-xx)

The ZoA and AoA from Rx to the reflection point are derived based on the location of Rx and reflection point.

θEO, ZOA=arccos⁡(zw-zrx(xw-xrx)2+(yw-yrx)2+(zw-zrx)2)							(7.9-xx)

ϕEO, AOA=arctanyw-yrxxw-xrx 										(7.9-xx)

The effective polarization matrix of the type-2 EO reflection path is given by

CPMEO=cosγ2sinγ2-sinγ2cosγ2R∥00-R⊥cosγ1-sinγ1sinγ1cosγ1						(7.9-xx)

Where,

- γ1=actaneθEO, ZODnplaneeϕEO, AODnplane mod 180°. nplane represents the normal vector of the incident plane. nplane=vtx→w×vw→rx, in which vtx→w=xw-xtx,yw-ytx,zw-ztx and vw→rx=xrx-xw,yrx-yw,zrx-zw. eθEO, ZOD represents the spherical basis vector of incident ray in vertical direction. eθEO,ZOD=cosθEO, ZODcosϕEO, AOD,cosθEO, ZODsinϕEO, AOD,-sinθEO, ZODT. eϕEO,AOD represents the spherical basis vector of incident ray in horizontal direction. eϕEO,AOD=-sinφEO, AOD,cosφEO, AOD,0T.

- γ2=actaneθEO, ZOAnplaneeϕEO, AOAnplane mod 180°. eθEO, ZOA represents the polar basis vector of scattering ray in vertical direction. eϕEO, AOA represents the polar basis vector of scattering ray in horizontal direction. eθEO, ZOA=cosθEO, ZOAcosϕEO, AOA,cosθEO, ZOAsinϕEO, AOA,-sinθEO, ZOAT. eϕEO, AOA=-sinϕEO, AOA,cosϕEO, AOAT.

- The formula 7.6-38 and 7.6-39 of reflection coefficients R// and R⊥ in Clause 7.6.8 can be reused with modified incidence angle. The incidence angle θEO is the angle between the incident vector from Tx to the reflected point and the normal vector, which is

θEO=arccosxw-xtxA+(yw-ytx)BA2+B2(xw-xtx)2+(yw-ytx)2+(zw-ztx)2

The reflection coefficients for parallel and perpendicular polarization are given by

R//=εGRε0cosθEO+εGRε0-sin2θEOεGRε0cosθEO-εGRε0-sin2θEO

R⊥=cosθEO+εGRε0-sin2θEOcosθEO-εGRε0-sin2θEO

When a type-2 EO is deployed in the simulation area, the following modification to the ISAC channel generation in Clause 7.9.4, 7.9.4.1 and 7.9.4.2 can be applied.

- In Step 1 in Clause 7.9.4.0,

b)	Give number of type-2 EO, additionally.

c)	Give 3D locations of type-2 EO in the global coordinate system, additionally.

e)	Give the orientation of type-2 EO in the global coordinate system, additionally. The type-2 EO orientation is defined by three angles ΩEO,α (type-2 EO bearing angle), ΩEO,β (type-2 EO downtilt angle) and ΩEO,γ (type-2 EO slant angle).

Note:	In case wrapping is used, each wrapping copy of a type-2 EO should be treated as a separate type-2EO considering channel generation.

- [In Step 2 in Clause 7.9.4.1, ]

[Rapporteur’s note: Further agreement necessary regarding LOS condition when Type-2 EO is present.]

- Between Step 8 and 9 in Clause 7.9.4.1, insert one step to generate NLOS rays specularly reflected by type-2 EO, if present.

In a STX-SPST link, the general procedure to model a type-2 EO as described above is executed by substitute Tx with the STX and Rx with the SPST. A NLOS ray specularly reflected by a EO type-2, if present, is represented by n=0,m>0. dtx,EO,mk,p=dEO,θtx,0,m,ZOAk,p=θEO, ZOA,ϕtx,0,m,AOAk,p=ϕEO, AOA,θtx,0,m,ZODk,p=θEO, ZOD,ϕtx,0,m,AODk,p=ϕEO, AOD,  CPMtx,n, mk,p=CPMEO.

In a SPST-SRX link, the general procedure to model a type-2 EO as described above is executed by substitute Tx with the SPST and Rx with the SRX. A NLOS ray specularly reflected by a EO type-2, if present, is represented by n'=0,m'>0. drx,EO,m'k,p=dEO,θrx,0,m',ZOAk,p=θEO, ZOA,ϕrx,0,m',AOAk,p=ϕEO, AOA,θrx,0,m',ZODk,p=θEO, ZOD,ϕrx,0,m',AODk,p=ϕEO, AOD,  CPMrx,n, m'k,p=CPMEO.

- In Step 9 in Clause 7.9.4.1,

The following paths are generated and added to set R0.

- A NLOS ray specularly reflected by type-2 EO, if present in the SPST-SRX link is coupled with a NLOS ray specularly reflected by type-2 EO, if present in the STX-SPST link

- A NLOS ray specularly reflected by type-2 EO, if present in the SPST-SRX link is coupled with any LOS ray (if present) and NLOS rays of stochastic clusters in the STX-SPST link

- A NLOS ray specularly reflected by type-2 EO, if present in the STX-SPST link is coupled with any LOS ray (if present) and NLOS rays of stochastic clusters in the SPST-SRX link

- In Step 10 in Clause 7.9.4.1,

Prx,0,m'k,p,Ptx,0,mk,p for a NLOS ray specularly reflected by a type-2 EO, if present, in the SPST-SRX link and the STX-SPST link is determined as follows.

- If the STX-SPST link is in LOS condition, Ptx,0,mk,p=dtx,3Dk,pdtx,EO,mk,p2Ktx,Rk,pKtx,Rk,p+1

- If the STX-SPST link is not in LOS condition, [TBD]

- If the SPST-SRX link is in LOS condition, Prx,0,m'k,p=drx,3Dk,pdrx,EO,m'k,p2Krx,Rk,pKrx,Rk,p+1

- If the SPST-SRX link is not in LOS condition, [TBD]

- In Step 11 in Clause 7.9.4.1,

For a NLOS ray specularly reflected by a type-2 EO in the STX-SPST link, if present, τtx,n,mk,p+dtx,3Dk,pc is replaced by dtx,EO,mk,pc, and ∆τtxk,p is set to 0.

For a NLOS ray specularly reflected by a type-2 EO in the SPST-SRX link, if present, τrx,n',m'k,p+drx,3Dk,pc is replaced by drx,EO,m'k,pc, and ∆τrxk,p is set to 0.

- The outcome of Steps 1-12 in Clause 7.9.4.1 shall be identical for all the links from co-sited sectors to a STX/ST/EO/SRX.

[Rapporteur’s note: Further agreement necessary regarding Type-2 EO in background channel.]

- [In Clause 7.9.4.2, if type-2 EO is present, a ray specularly reflected by a type-2 EO is generated if a specular reflection point can be found within a surface of the EO.]

7.9.5.3	Power normalization across target channel and background channel

[Rapporteur’s note: further agreements are necessary on model B with power normalization]

To combine the target channel and the background channel, an alternative scheme of power normalization can be applied to keep the same/similar channel power as the background channel without sensing target.

7.9.5.4	Doppler of mobile scatters

To support scatterer mobility, the Doppler frequency component fD,n',m',n,mk,pt in the channel coefficient generation in step 13 in clause 7.9.4.1 should be updated as follows.

fD,n',m',n,mk,pt=rrx,k,p,n',m'Ttvrxt+rk,p,n',m'Ttvk,pt+2αrx,n',m'k,pDrx,n',m'k,pλ0+rtx,k,p,n,mTtvtxt+rk,p,n,mTtvk,pt+2αtx,n,mk,pDtx,n,mk,pλ0		(7.9-xx)

Where,

- Drx,,n',m'k,p is a random variable from -vscatt to vscatt, and vscatt is the maximum speed of the clutter. αrx,n',m'k,p is a random variable of Bernoulli distribution with mean p' if n'>0, otherwise αrx,n',m'k,p=0. Parameter p' determines the proportion of mobile scatterers and can thus be selected to appropriately model statistically larger number of mobile scatterers (higher p') or statistically smaller number of mobile scatterers (e.g. in case of a completely static environment: p'=0 results in all scatteres having zero speed). [A typical value of p' is 0.2].

- Dtx,n,mk,p is a random variable from -vscatt to vscatt, and vscatt is the maximum speed of the clutter. αtx,n,mk,p is a random variable of Bernoulli distribution with mean p if n>0, otherwise αtx,n,mk,p=0. Parameter p determines the proportion of mobile scatterers and can thus be selected to appropriately model statistically larger number of mobile scatterers (higher p) or statistically smaller number of mobile scatterers (e.g. in case of a completely static environment: p=0 results in all scatteres having zero speed). [A typical value of p is 0.2].

7.9.5.5	Lower power clusters

In addition to the clusters/rays in background channel generated in Clause 7.9.4.2, more clusters/rays can be generated to simulate the weak rays in the background channel which still impacts the sensing performance. The following procedure is applicable for bistatic sensing or for monostatic sensing.

Step 1: Generate a first background channel Hu,sbk1τ,t according to Clause 7.9.4.2.

Step 2: Generate a second background channel Hu,sbk2τ,t according to Clause 7.9.4.2 with the following revised parameters: 1) The number of clusters (N in Clause 7.5) is increased to 360; 2) The number of ray per cluster (M in Clause 7.5) is reduced to 1.

Step 3: The power of the clusters/rays in the second background channel is scaled down by PmaxS110G10, and is added to the first background channel, where, G=-25 dB, PmaxS1 is the maximum power of the NLOS clusters in the first background channel. The final background channel Hu,sbkτ,t is

Hu,sbkτ,t=Hu,sbk1τ,t+P1S110G10∙Hu,sbk2τ,t								(7.9-xx)

7.9.6	Channel models for link-level evaluations

[Rapporteur’s note: this clause is to capture the agreements on LLS channel model for ISAC.]

7.9.7	Channel model calibration

7.9.7.1	Large scale calibration

For the purposes of large scale calibration without fast fading modelling for sensing targets UAV, human, automotive and AGV, the following calibration parameters are respectively provided in Table 7.9.7.1-1/2/3/4. Unspecified parameters in Table 7.9.7.1-2/3/4 are the same as those in Table 7.9.7.1-1. The calibration results based on xxxx can be found in R1-xxxxxx.

Table 7.9.7.1-1: Simulation assumptions for large scale calibration for UAV sensing targets

| Parameters | Values |
| --- | --- |
| Scenario | UMa-AV |
| Sensing mode | TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic |
| Target type | UAV of small size (0.3m x 0.4m x 0.2m) |
| Sectorization | Single 360-degree sector can be assumed |
| Carrier Frequency | FR1: 6 GHz FR2: 30 GHz |
| BS antenna configurations | Single dual-pol isotropic antenna |
| BS Tx power | FR1: 56dBm FR2: 41dBm |
| Bandwidth | FR1: 100MHz FR2: 400MHz |
| BS noise figure | FR1: 5dB FR2: 7dB |
| UT antenna configurations | Single dual-pol isotropic antenna; (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1) |
| UT noise figure | FR1: 9dB FR2: 10dB |
| UT height | 1.5m for terrestrial UTs, |
| UT Tx power | 23dBm |
| UT Distribution | • The overall number of UTs is 30 uniformly distributed in the center cell. • All of the UTs are either terrestrial UTs or aerial UTs, all outdoors. • Vertical distribution of aerial UE: Fixed height value of 200 m. • FR1 is assumed for aerial UE. |
| Sensing target distribution | 1 target uniformly distributed (across multiple drops) within the center cell. Vertical distribution: Fixed height value of 200 m. |
| Component A of the RCS for each scattering point | -12.81 dBsm |
| Minimum 3D distances between pairs of Tx/Rx and sensing target | 10 m |
| Wrapping Method | No wrapping method is used if interference is not modelled, otherwise geographical distance based wrapping |
| Coupling loss for target channel | power scaling factor (pathloss, shadow fading, and RCS component A included): LTX-SPST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2 |
| Sensing Tx/Rx selection | Best N = 4 Tx-Rx pairs to be selected for the target. NOTE1: Based on the Tx-Rx pairs with the smallest power scaling factor of the target channel. |
| Metrics | Coupling loss for target channel Coupling loss for background channel (in case of monostatic sensing, this is the coupling loss between Tx and one reference point) Note: CDFs can be separately generated for target channel, background channel |

Table 7.9.7.1-2. Simulation assumptions for large scale calibration for Human sensing targets

| Parameters | Indoor Values | Outdoor Values |
| --- | --- | --- |
| Scenario | Indoor office 12 sectors per 120m * 50m * 3m ISD = 20m InF-SH Hall size: 300x150 m Room height: 10m 18 BSs on a square lattice with spacing D, located D/2 from the walls. -big hall (L=300m x W=150m): D=50m | UMa (ISD = 500m), UMi (ISD= 200m) |
| Sensing mode | TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic, UE monostatic |  |
| Target type | Adult Pedestrian: 0.5m x 0.5m x 1.75m. Note: Height of scattering point 1.5m |  |
| BS Tx power | FR1: 24dBm FR2: 23dBm | FR1: 56dBm FR2: 41dBm |
| UT height | 1m | 1.5m |
| UT Distribution | Per Table 7.8-1 Indoor-Office Number of UTs: 20 | Per Table 7.8-1. Number of UTs/cell: 10 |
| Sensing target distribution | 100% indoor, 1 target uniformly distributed (across multiple drops) over the convex hull of the TRP deployment | 100% outdoor. 1 target uniformly distributed (across multiple drops) within the center cell. |
| Component A of the RCS for each scattering point | -1.37 dBsm |  |
| Minimum 3D distances between pairs of Tx/Rx and sensing target | Min distances defined in TR 38.901 and TR36.843 and TR38.859 |  |
| Wrapping Method | N/A | No wrapping method is used if interference is not modelled, otherwise geographical distance based wrapping |

Table 7.9.7.1-3. Simulation assumptions for large scale calibration for Automotive sensing targets

| Parameters | Values |
| --- | --- |
| Scenario | For FR1: Urban Grid (ISD=500m, BS height=25m) Highway (ISD=1732m, BS height=35m) For FR2: Urban Grid (ISD=250m, BS height=25m) Highway (ISD=500m, BS height=35m) For Urban Grid ISD =250m - The layout is configured as follows: - Red triangles: BS with 250m ISD, 18 BSs are located. |
| Sensing mode | TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic, UE monostatic |
| Target type | Vehicle type 2 [TR37.885] |
| UT height | 1.5m for pedestrian type UE 5m for RSU type UE 1.6m for vehicle type UE |
| UT Distribution | For Highway: - Vehicle Type UT distribution follows vehicle UE dropping as in Table A.1.2-1 from TR36.885. - RSU-type UTs are uniformly allocated with 100m spacing in the middle of the freeway as per TR36.885. For Urban grid: - Vehicle Type UT distribution follows vehicle UE dropping as in Table A.1.2-1 from TR36.885. - Pedestrian type UT, the dropping using equally spaced along the sidewalk with a fixed inter-pedestrian X m dropped per TR36.885 - Total number of pedestrian UEs is 16 in the centre grid. - Pedestrian UE is in the middle of the sidewalk - The inter-pedestrian UE distance (m) (i.e., X) is calculated by ‘A/16’, where ‘A’ is the total length of sidewalk where the pedestrian UEs are dropped under the assumption of ‘N’ road grids (i.e., ‘{(250m – 17m) + (433m – 17m)} * 2 * N’). For example, if the pedestrian UEs are dropped in ‘14’ road grids, the inter-pedestrian UE distance (m) is ‘36.344’. - N=1; - RSU-type UT: the dropping is at the center of intersection per TR36.885. NOTE: A single UT type is used per calibration, e.g., pedestrian type UT, RSU type UT, or vehicle type UTPer TR37.885 |
| Sensing target distribution | Per TR37.885: - Option A - Vehicle type distribution: 100% vehicle type 2. - Clustered dropping is not used. - Highway: one target uniformly distributed (across multiple drops) within the simulation region. Vehicle speed is 140 km/h in all the lanes as baseline. - Urban Grid: one target is uniformly distributed (across multiple drops) within the center road grid. Vehicle speed is 60 km/h in all the lanes as baseline. NOTE: vehicle is dropped with 5 scattering points (front/left/right/back/roof) and each point has one location, or vehicle is dropped with 1 scattering points. In the case of vehicle with 5 scattering points, spatial consistency is enabled with the following assumptions: - The correlation for LOS/NLOS condition of the 5 points is assumed equal to 1. LOS/NLOS condition can be calculated based on the distance of the Tx/Rx to the centroid of the ST, then apply the LOS/NLOS condition to alleach of the 5 points. The correlation for stochastic cluster paths of the 5 points is assumed equal to 1. The stochastic cluster paths can be calculated between the Tx/Rx and the centroid of the ST, then apply the stochastic cluster paths are added to each ofto all the 5 points. |
| Component A of the RCS for each scattering point | -20dBsm |
| Minimum 3D distances between pairs of Tx/Rx and sensing target | 10 m |
| Wrapping Method | As defined in urban grid/highway scenario |

Table 7.9.7.1-4. Simulation assumptions for large scale calibration for AGV sensing targets

| Parameters | Values |
| --- | --- |
| Scenario | InF: 18 TRPs per the big hall: L = 300 m x W = 150 m - X-axis is pointing down to the floor - The antenna array is mounted in the Y-Z plane with boresight along the X-axis - The X-axis/Y-axis/Z-axis refer to LCS - 8 m for high BS scenario |
| Sensing mode | TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic, UE monostatic |
| Target type | Option 2: 1.5m x 3.0m x 1.5m |
| BS Tx power | FR1: 24dBm FR2: 23dBm |
| UT height | 1.5m |
| UT Distribution | Per Table 7.8-7 Indoor Factory. Number of UTs: 30 |
| Sensing target distribution | 100% indoor, 1 target uniformly distributed (across multiple drops) over the convex hull of the TRP deployment |
| Component A of the RCS for each scattering point | -1.37 dBsm Note: For calibration purposes, other value(s) are not precluded. |

7.9.7.2	Full calibration

For the purpose of full calibration including the fast fading modelling for sensing scenarios UAV, [human], automotive and [AGV], the calibration parameters are respectively provided in Table 7.9.7.2-1/2/3/4. Unspecified parameters in the tables are the same as those in Table 7.9.7.1-1/2/3/4 for the same sensing target. If still not specified, the parameters in Table 7.9.7.2-1 are used. The calibration results based on xxxx can be found in R1-xxxxx.

Table 7.9.7.2-1: Simulation assumptions for full calibration for UAV sensing targets

| Parameters | Values |
| --- | --- |
| Scenario | UMa-AV |
| Sensing mode | TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic |
| Target type | UAV of small size (0.3m x 0.4m x 0.2m) |
| RCS for each scattering point | Component A: -12.81 dBsm Component B1: 0 dB Component B2: 3.74 dB for standard deviation The same values are used for monostatic RCS and bistatic RCS |
| Fast fading model | TR 36.777 Annex B.1.3 |
| (u, std) for XPR of target | Mean 13.75 dB, deviation 7.07 dB |
| The power threshold for path dropping after concatenation for target channel | FFS |
| Coupling loss for target channel | By definition, need to consider all direct and indirect paths. The following parameters are included in the calculation: - power scaling factor (pathloss, shadow fading, and RCS component A included) - for small scale RCS B1/B2 and power of rays in Tx-target/target-Rx links (Pn',m',n,mk,p), Tx/Rx antenna pattern, 3 polarization matrixes, i.e., Pn',m',n,mk,pFrx,u,θθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pFrx,u,ϕθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pTCPMrx,n',m'k,pCPMn',m',n,mk,pCPMtx,n, mk,pd112+d2222 ∙Ftx,s,θθtx,n,m,ZODk,p,ϕtx,n,m,AODk,pFtx,s,ϕθtx,n,m,ZODk,p,ϕtx,n,m,AODk,p |
| Absolute delay | The model of UMa scenario defined in TR 38.901 7-24GHz channel modeling [ref] is reused for UMa-AV for all sensing modes. |
| Metrics | Coupling loss for target channel Coupling loss for background channel (in case of monostatic sensing, this is the linear sum of coupling losses between Tx/Rx and all reference points) Note: CDFs can be separately generated for target channel, background channel CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA). Definition of Delay Spread is similar to the definition of angle spread in Annex A of TR 25.996, Definition of Angle Spread can ref to Annex A of TR 25.996. |

Table 7.9.7.2-2: Simulation assumptions for full calibration for Human sensing targets

| Parameters | Indoor Values | Outdoor Values |
| --- | --- | --- |
| Scenario | As specified in Table 7.9.7.1-2 | As specified in Table 7.9.7.1-2 |
| Sensing mode | TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic, UE monostatic |  |
| Target type | Adult Pedestrian: 0.5m x 0.5m x 1.75m Note: Height of scattering point 1.5m |  |
| UT Distribution | Per Table 7.8-2 Indoor-Office Number of UTs: 20 | Per Table 7.8-2 Number of UTs/cell: 10 |
| RCS for each scattering point | Component A: -1.37 dBsm Component B1: 0 dB Component B2: 3.94 dB The same values are used for monostatic RCS and bistatic RCS | Component A: -1.37 dBsm Component B1: 0 dB Component B2: 3.94 dB The same values are used for monostatic RCS and bistatic RCS |
| Fast fading model | For BS to UE link: Follow the procedure defined in TR38.901 For BS to target link: Follow the procedure defined in TR38.901 for target to UE link: Follow the procedure defined in 38.858 | For BS to UE link: Follow the procedure defined in TR38.901 For BS to target link: Follow the procedure defined in TR38.901 for target to UE link: Follow the procedure defined in 38.858 |
| (u, std) for XPR of target | FFS | FFS |
| Absolute delay | The model of InH, InF-SH scenarios defined in TR 38.901 7-24GHz channel modeling [Table 7.6.9-1]. | The model of UMa/UMi scenario defined in TR 38.901 7-24GHz channel modeling [Table 7.6.9-1]. |

Table 7.9.7.2-3: Simulation assumptions for full calibration for Automotive sensing targets

| Parameters | Values |
| --- | --- |
| Scenario | As specified in Table 7.9.7.1-3 |
| Sensing mode | TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic, UE monostatic |
| Target type | Vehicle type 2 [TR37.885] |
| RCS for each scattering point | Based on the RCS model in Clause 7.9.2.1. |
| Fast fading model (full calibration) | Procedures based on 37.885 |
| (u, std) for XPR of target | (21.12, 6.88) dB. |
| Absolute delay | The model of UMa scenario defined in TR 38.901 7-24GHz channel modelling [Table 7.6.9-1] is reused for Highway (FR2)/Urban Grid for all sensing modes. The model of RMa scenario defined in TR 38.901 7-24GHz channel modelling [Table 7.6.9-1] is reused for Highway (1732m ISD) for all sensing modes. |

Table 7.9.7.2-4: Simulation assumptions for full calibration for AGV sensing targets

| Parameters | Values |
| --- | --- |
| Scenario | As specified in Table 7.9.7.1-4 |
| Sensing mode | TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic, UE monostatic |
| Target type | Option 2: 1.5m x 3.0m x 1.5m |
| RCS for each scattering point | The values/pattern of component A*B1 are generated by Table 7.9.2.1-6 The same values are used for monostatic RCS and bistatic RCS |
| Fast fading model (Full calibration) | For BS to UE link: Follow the procedure defined in TR38.901 For BS to target link: Follow the procedure defined in TR38.901 for target to UE link: Follow the procedure defined in TR 38.858 |
| (u, std) for XPR of target | FFS |
| Absolute delay | The model of InF scenario defined in TR 38.901 7-24GHz channel modeling [Table 7.6.9-1] is reused for all sensing modes. |

7.9.7.3	Calibration of additional features

The calibration parameters for the calibration of spatial consistency and type-2 EO can be respectively found in Table 7.9.7.3-1/2. Unspecified parameters in these tables are the same as those in Tables 7.9.7.1-2/3 and Tables 7.9.7.2-2/3. The calibration results based on xxxx can be found in R1-xxxxx.

Table 7.9.7.3-1: Simulation assumptions for calibration of spatial consistency

| Parameter | Values |
| --- | --- |
| Scenario | Urban grid, Indoor office |
| Cell layout | Urban grid: ISD = 500m, the same layout with 37.885 Indoor office: Office size (WxLxH): 120mx50mx3m, ISD=20m (refer to TR 38.901) |
| Sensing mode | TRP- pedestrian UE bistatic in urban grid and TRP-UE bistatic in indoor office |
| Polarized antenna model | Model-2 in clause 7.3.2 in TR 38.901 |
| Scattering point | Single point |
| Calibration link and calibration method | - Calibration link: same target associated to different UT as following figure - Calibration method Step1: In single drop simulation, drop one Target and multiple UTs. Step2: Generate the channel of each link based on Target level spatial consistency method. Step3: Determine Target-UT groups based on the distance between UTs. Step4: Bin all the link pairs into certain distance groups, e.g., - 0m <= the location distance of link pair < 1m -> 0m group - 1m <= the location distance of link pair < 2m -> 1m group - 2m <= the location distance of link pair < 3m -> 2m group - 3m <= the location distance of link pair < 4m -> 3m group …… - (N)m <= the location distance of pair < (N+1)m -> (N)m group Note: N is at least twice the maximum correlation distance associated with the channel parameters in the scenario Step5: Calculate the correlation coefficient metric of each distance groups. Step6: Draw the x-y cure, where x=0, …, N, y= correlation coefficient metric related to (x)m group |
| ST and UT distribution | - Urban grid one target is uniformly dropped within the center grid in the outside lane with length of 433m, the UT is distributed by a uniform distribution distance of [0,1]m in the walk road where is in the same street with the target. As following figure. Note: The ST-UT link only consider LOS condition discarding NLOSv condition. - Indoor office 1 Target is uniform dropped in office region 10 UTs are dropped uniformly per cell, i.e., totally 120 UTs are uniformly dropped in office region. |
| Metrics | - Cross-correlation coefficient of delay for the third cluster of channel between link pairs. - Cross-correlation coefficient of AOA (for UT) for the third cluster of channel between link pairs. - Cross-correlation coefficient of LOS/NLOS status of channel between link pairs(only for indoor office scenario). |
| Note: Other necessary but unspecified parameters in this table are the same as those in the table for urban grid scenario calibration and indoor office scenario calibration. |  |

Table 7.9.7.3-2: Simulation assumptions for calibration of type-2 EO

| Parameter | Values |
| --- | --- |
| Scenario | Urban grid |
| Cell layout | For FR1: ISD = 500m, the layout is defined as below: The BSs are placed at the top of buildings at one corner. Specifically, the road grids shall be shifted by (ISD3-10,32*ISD3-10) m in horizontal plane, or the BSs are shifted by (-ISD3+10,-32*ISD3+10)m in horizontal plane equivalently. For FR2 ISD=250m, the cell layout is as that specified in Table 7.9.7.1-3 |
| Sensing mode | TRP- pedestrian UE bistatic TRP- RSU type UE bistatic TRP monostatic TRP-TRP bistatic RSU type UE - RSU type UE bistatic (optional) |
| EO deployment | 9 buildings with size 413m x 230m x 20m illustrated by the green blocks as in the figure shown in the row of cell layout. |
| ST distribution | one target is uniformly distributed (across multiple drops) within the center of the outside lane of the grid. |
| UT distribution | - For pedestrian UT - Pedestrian type UE, the dropping using equally spaced along the sidewalk with a fixed inter-pedestrian X m dropped per TR36.885. - Total number of pedestrian UEs is 16 in the centre grid. - Pedestrian UE is in the middle of the sidewalk - The inter-pedestrian UE distance (m) (i.e., X) is calculated by ‘A/16’, where ‘A’ is the total length of sidewalk where the pedestrian UEs are dropped under the assumption of ‘N’ road grids (i.e., ‘{(250m – 17m) + (433m – 17m)} * 2 * N’). For example, if the pedestrian UEs are dropped in ‘14’ road grids, the inter-pedestrian UE distance (m) is ‘36.344’. - N=1; - For RSU type UT - The dropping is at the center of intersection per TR36.885. |
| EM-parameters | Table 7.6.8-1 Material properties [TR38.901] Material class; Relative permittivity; Conductivity; Frequency range in [GHz]. aε; bε; cσ; dσ. Concrete; 5.31; 0; 0.0326; 0.8095; 1-100. |
| Metrics | CDF curves: - Full coupling loss: calculate the coupling loss for each Tx-EO-ST-LOS-Rx and Tx-LOS-ST-EO-Rx rays. - CDF of the Delay. - CDF of the AoA, AoD, ZoA, ZoD. Additional CDF curves: - Coupling loss for target channel - CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA). Definition of Delay Spread is similar to the definition of angle spread [TR 25.996, Annex A]. |
| Note: Other necessary but unspecified parameters in this table are the same as those in the table for urban grid scenario calibration. |  |

< Unchanged text omitted >