3GPP TSG RAN WG1 #121	R1-2503234

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	9.5.2

Source:	Futurewei

Title:	Discussion of on-demand SIB1 for idle/inactive mode UEs

Document for:	Discussion and decision

# Introduction

At RAN#105, companies agreed to support specifications for Case 2 of on-demand SIB1.

[RP-242354]. The updated WID includes the following objective:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3]

  - Specify procedures and signaling method(s) for Case 2 [RAN1/2]

    - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell

    - Triggering method by UL WUS using PRACH

  - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3]

  - Note 1: No modification of SSB will be discussed under this objective

  - Note 2:

    - UL WUS: Uplink wake-up signal

    - Cell A: A cell that is periodically transmitting at least its own SIB1

    - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE

  - Note 3:

    - RAN1 strives to minimize impact to legacy UE

    - RAN1 specification impact to support this feature should be minimized

In this contribution, this objective of on-demand SIB1 for idle/inactive mode UEs will be analyzed, and our views will be provided, based on relevant agreements from the previous meetings.

# Discussion of open issues

## SSB Indication

During RAN1 #120bis, companies agreed that:

Agreement

The UE assumes that, in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in PDCCH monitoring occasions corresponding only to at least the SSB associated with the transmitted PRACH for UL-WUS if this is indicated via UL WUS configuration by a 1-bit indication

- If the 1-bit indication is included in UL WUS config and indicate as TRUE, the UE assumes that, in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in PDCCH monitoring occasions corresponding only to the SSB associated with the PRACH for UL-WUS.

- If the 1-bit indication is not included in UL WUS config or the 1-bit indication is FALSE, the UE assumes the same as legacy, i.e., in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB.

- Further study possible down selection from the following options to indicate additional information related to SSBs associated with PDCCH monitoring occasions

  - Alt1: The information is provided in UL-WUS config

  - Alt2: The information is provided in RAR in response to UL-WUS transmission

  - Alt3: No additional information is provided

One remaining open issue from the above agreement is the indication of information related to SSBs associated with PDCCH monitoring occasions. We note that so far, the agreements related to RAR do not add any additional information specific to NES Rel 19.  The content of RAR is based on the legacy RAR for OSI. Furthermore, we believe that no additional information needs to be provided to the UE related to SSBs associated with PDCCH monitoring occasions. The UE can rely on the legacy SSB monitoring approach, which is consistent with UE behavior when the 1-bit indication is not present in UL WUS configuration. If such bit is included, the UE may identify the PDCCH monitoring occasions corresponding only to the SSB associated with the PRACH for UL-WUS without any additional information.

Proposal 1: Support Alt 3: No additional information is provided to indicate additional information related to SSBs associated with PDCCH monitoring occasions.

## SIB1 duration

Another open issue from the last RAN1 meeting agreement is the value range of od-sib1-duration.

Agreement

RAN1 to discuss the value range for od-sib1-duration. For example,

- Option 1: Use the value-range for si-windowLength (from 5 slots to 5120 slot) as starting point.

- Option 2: The duration of the time window can be a multiple of 160ms (i.e. legacy SIB1 periodicity).

- Option 3: The duration of the time window can be a multiple of the on-demand SIB1 repetition periodicity (if supported)

- Other options are not precluded.

We note that a previous agreement in RAN1 #118bis establishes that:

Agreement

For the repetition periodicity of SIB1 within the time window of on-demand SIB1:

- Up to NW implementation (no change to existing specification)

Based on this agreement we prefer that the maximum legacy SIB1 periodicity of 160ms to be considered as the value range for od-sib1-duration. This solution is preferable over si-WindowLength (up to 5120 slots) to maintain compatibility and simplify implementation.

Proposal 2: Support Option 2: The value range for od-sib1-duration can be a multiple of 160ms.

## NR-U and RedCap NES support

In RAN1 #120bis [R1-2503008], the FL identified two possible issues for further study:

FL Proposal 11-1: RAN1 to further study whether the R19 OD-SIB1 feature can work with legacy RedCap features.

FL Proposal 11-2: RAN1 to further study whether R19 OD-SIB1 feature can work with legacy NR-U features.

First, we note that during the NES study, companies did not provide results for unlicensed bands (NR-U), thus there is no evidence at this time that on-demand SIB1 is beneficial for unlicensed band operation, given that any broadcast in unlicensed band requires an additional listen before talk procedure. Such a study would require additional discussions and analysis, which is not feasible in the remaining time for this WI.

Proposal 3: In the absence of supporting performance evaluations for NR-U, and considering LBT complexity, we recommend excluding OD-SIB1 for NR-U in Rel-19.

On the NES for RedCap, there is no restriction for RedCap UEs for SIB1 monitoring and acquiring. Thus, supporting on-demand SIB1 seems feasible for RedCap UEs. However, it may be tricky signaling that a cell is barred for RedCap UEs.

When a RedCap UE is receiving a SSB from a NES Cell, as per legacy behavior, if a RedCap UE is ultimately unable to acquire SIB1, the UE shall consider the cell as barred. Therefore, if a cell indicates that the SIB1 is not provided as in K_SSB > 23 for FR1 and K_SSB >11 for FR2, a RedCap UE may assume that the cell is barred for its access.

However, as in legacy specs, a RedCap UE may use the K_SSB value to determine where a CD-SSB is broadcast. In such a case the CD-SSB is provided by Cell A.  But a RedCap NES UE has “a double personality”.

Thus, even if a RedCap NES UE camps on Cell A providing a UL WUS configuration for the NES cell, after visiting the NES cell, the RedCap part of the RedCap NES UE may interpret that the NES cell is barred. In this case the two personalities of RedCap NES UE may have a different (contradictory) view of their permission to access NES cell.

Thus, unless the specs for RedCap UE are changed in Rel 19, a RedCap NES UE may not access a NES cell. This would imply a change in the definition of a Rel 19 RedCap UE that could support NES features.

Observation 1: The R19 OD-SIB1 feature requires changes of the legacy RedCap initial access.

However, we estimate that such changes are minor at least in the case when Cell A is not barred for RedCap UEs. Thus, if RAN1 decides to support R19 OD-SIB1 for RedCap, a simple solution is to add to the UL WUS configuration information on RedCap barring access to the NES cell. If a RedCap NES UE, after receiving UL WUS configuration, determines that a RedCap UE is allowed to camp on the NES cell, it may decide to access NES cell and request a SIB1 on demand (as a NES UE) and then behave as a RedCap UE in the NES cell. Otherwise, the RedCap NES UE will not access the NES cell.

Proposal 4: If RAN1 supports R19 OD-SIB1 feature for RedCap UEs, provide in UL WUS configuration information on RedCap barring access to the NES cell.

# Conclusions

In this contribution, we present our views on on-demand SIB1 for idle/inactive mode UEs. We have the following proposals.

Proposal 1: Support Alt 3: No additional information is provided to indicate additional information related to SSBs associated with PDCCH monitoring occasions.

Proposal 2: Support Option 2: The value range for od-sib1-duration can be a multiple of 160ms.

Proposal 3: In the absence of supporting performance evaluations for NR-U, and considering LBT complexity, we recommend excluding OD-SIB1 for NR-U in Rel-19.

Observation 1: The R19 OD-SIB1 feature requires changes of the legacy RedCap initial access.

Proposal 4: If RAN1 supports R19 OD-SIB1 feature for RedCap UEs, provide in UL WUS configuration information on RedCap barring access to the NES cell.

# References

RP-240170, Enhancements of network energy savings for NR, Ericsson (Moderator).

R1-2403870, Discussion of on-demand SSB SCell operation, Futurewei.

RP-242354, NES WID

Chair Notes RAN1 #117 eom4

Chair Notes RAN1 #118 eom4

Chair Notes RAN1 #119 eom1

Chair Notes RAN1 #120 eom1

Chair Notes RAN1 #120bis eom1