3GPP TSG-RAN WG1 Meeting #121	R1-2503261

St Julian’s, Malta, May 19 – 23, 2025

Agenda Item:	9.3.1

Source:	Huawei, HiSilicon

Title:	On subband full duplex design

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss the semi-static indication of time and frequency location of SBFD subbands. Additionally, we discuss the remaining issues of UE transmission, reception and measurement behaviour and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UEs.

- Discussion

  - SBFD subband indication

    - Indication of frequency domain location of SBFD subbands

The following agreement has been made in RAN1#116.

Agreement For RRC connected mode UEs, at least cell-specific configuration on time and frequency(working assumption) location of SBFD subbands is supported within a TDD carrier. - FFS: Additional support of UE-specific configuration on time and/or frequency locations of SBFD subbands.

The motivation to support UE-specific configuration on frequency locations of SBFD subbands, mentioned by companies, is to accommodate UE-specific guardbands and UE requirements to receive in one single DL subband. These two aspects can be discussed separately.

For the first aspect, the proponents provide two main reasons: SBFD-aware UEs may have different filtering capabilities which potentially require different implementation complexity; different SBFD-aware UEs may require different frequency separations to reduce UE-to-UE CLI [1].

- To reduce the impact of inter-UE CLI, different DL/UL usable resources can be considered for different UEs. Note that this can be achieved by avoiding scheduling some RBs at the gNB in a transparent manner by replying on the UE-to-UE CLI measurement and reporting (potentially together with CSI reporting).

- NR has not yet introduced UE-specific filtering capability in specification, even though there are already different UE implementations with different capabilities. This would require more discussion in RAN4 on how to define performance requirement and testing methods.

For the second aspect, while we see the motivation to introduce UEs supporting DL reception only in one DL subband, this does not necessarily mean a UE-specific guardband configuration is required. During initial access, the UE capability has not reported to the gNB, the gNB assumes the basic features for SBFD aware UEs, i.e., DL reception in one DL subband. For UEs in RRC_CONNECTED mode, a UE reports to the gNB whether DL reception in two non-contiguous DL subbands is supported or not, the gNB can configure CSI-RS and schedule the UE accordingly based on cell-specific SBFD subband configuration.

Besides, one may also need to discuss if guard bands for SBFD aware UEs can be always within gNB’s guardband(s). If so, the UE-specific guard bands are not necessary. Besides, for random access operation of SBFD-aware UE in RRC_IDLE/INACTIVE mode, gNB has no UE capability information with respect to guardband size or receiving only one single DL subband, so UE-specific configuration on frequency location of SBFD subbands cannot be assumed.

From operation point of view, it is not preferred that gNB has to deal with UEs with different filtering capabilities in order to operate SBFD. Therefore, cell-specific configuration on frequency location of SBFD subbands should be the baseline for SBFD operation. The necessity of UE-specific guardbands can be further discussed in RAN4 as agreed in RAN4#110bis.

Base on the above discussion, we have the following proposal:

Proposal 1: UE-specific configuration on frequency locations of SBFD subbands is not supported in Rel-19.

    - Indication of time location of SBFD subbands

RAN4 agreed that a transient period at the gNB between SBFD symbols and non-SBFD symbols is required. The existing TDD BS transmitter transient period is reused, i.e., 10us for FR1 and 3us for FR2-1. These agreements are based on the understanding that such a transient period may occur at the gNB side, primarily due to power amplifier ramping up/down. RAN4 send an LS to RAN1 in R1-2503213 and informs RAN1 about the RAN4 agreement on transition period.

Figure 1. Illustration of four cases of transitions to/from SBFD symbols

As discussed in [2], the gNB can reserve the transition period via implementation by avoiding scheduling UL transmissions and/or DL receptions overlapping with the transition period in SBFD symbols or non-SBFD symbols. Therefore, there is no need to indicate the transition period to the UE.

The four cases identified by RAN4 assumes that there is link direction switch between SBFD symbols and non-SBFD symbols. However, there may be transitions even when there is no link direction switch.

- For Case-A, there may be transition within SBFD DL subband(s). In SBFD symbols, depending on gNB implementation, a SBFD DL subband filter may be implemented on gNB Tx RF chains to further reduce the leakage into SBFD UL subband. In DL symbols, for these gNB Tx RF chains, the filter should be switched to carrier/band filter. Thus, a switch delay is needed. The transient period can be located in SBFD symbol(s) or DL symbols.

- For Case-B, there may be transition within SBFD UL subband due to filter switch on gNB Rx RF chains. However, for this case, it has been agreed the transient period can be reserved only in the DL part(s) of SBFD symbol(s) or in the whole channel bandwidth, up to gNB implementation. No further discussion is needed.

- For Case-C, similar to Case-A, there may be transition within SBFD DL subband(s) due to Tx filter switch on gNB Tx RF chains. Thus, the transient period can be reserved in in DL symbol(s), or, only in the SBFD DL subband of SBFD symbol(s) or in the whole channel bandwidth of SBFD symbol(s), up to gNB implementation.

- For Case-D, similar to Case-B, there may also be transition within SBFD UL subband(s) due to filter switch on gNB Rx RF chains. Transient period can be located in SBFD symbol(s) or UL symbols.

Similar to the cases where there is link direction switch, the transition period can also be handled by gNB implementation when there is no link direction switch.

Proposal 2: The guard period between SBFD symbols and non-SBFD symbols is implicitly provided by the gNB.

  - UE transmission, reception and measurement enhancements across SBFD symbols and non-SBFD symbols

    - Enhancement on resource allocation in frequency domain in SBFD symbols

      - PDSCH

PRG determination for PDSCH with non-contiguous PRBs in SBFD symbols

In current specification, when scheduled by DCI format with CRC scrambled by C-RNTI, MCS-C-RNTI or CS-RNTI other than DCI format 1_0, UE determines the PRG size based on RRC configuration if the higher layer parameter prb-BundlingType is set to 'staticBundling', or based on PRB bundling size indicator in DCI if the higher layer parameter prb-BundlingType is set to 'dynamicBundling'. And according to current specification, for a UE, support of semi-static PRB bundling is mandatory while support of dynamic PRB bundling is optional.

Based on the conclusion in RAN1#118, if PRG is determined as wideband, UE does not expect to be scheduled with non-contiguous PRBs in SBFD symbols. This means once prb-BundlingType is set to 'staticBundling' and bundleSize is configured as wideband, the gNB can only schedule the PDSCH in only one DL subband. This poses a limitation to peak data rate and also reduces gNB scheduling flexibility. Considering that the PRG size for PDSCH scheduled by DCI format 1_0 is always 2, the same mechanism can be reused to relax this restriction, i.e., if a UE is scheduled a PDSCH with non-contiguous PRBs in SBFD symbols, the UE shall always assume PRG size is equal to 2 PRBs.

Proposal 3: If a UE is scheduled with a PDSCH across two DL subbands in SBFD symbols, the UE shall assume the PRG size is equal to 2 PRBs.

Restriction for RE(s) not available for PDSCH

Based on the above agreement, assigned PRBs that are outside DL usable PRBs are considered to be invalid and should not be used for PDSCH resource mapping. In current specification, a UE is not expected to handle the case where PDSCH DM-RS REs are overlapping, even partially, with any RE(s) not available for PDSCH. This restriction is to avoid the case that the PDSCH DM-RS is not transmitted in certain RB(s) and the REs in the same RB(s) but different symbol(s) are available for data. For SBFD, this restriction can be relaxed as the assigned PRBs outside DL usable PRBs are considered to be not available for PDSCH, which includes both data symbols and DM-RS symbols of the PDSCH.

Proposal 4: The following existing restriction is relaxed for SBFD-aware UEs in SBFD symbols.

- A UE is not expected to handle the case where PDSCH DM-RS REs are overlapping, even partially, with any RE(s) not available for PDSCH.

      - CSI-RS

CSI-RS resource allocation across two DL subbands

For frequency domain resource allocation for CSI-RS across DL subbands for SBFD-aware UEs, the following working assumption and agreements were made until RAN1#118bis.

Working Assumption For frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, support one contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL usable PRBs. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured) - RAN1 to further study the impact on CSI processing timeline in SBFD symbols to process the CSI-RS across the two DL subbands Agreement For CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands, - Option 3: It is up to UE capability to indicate one of the following: - CSI processing timeline and CPU counting are the same as legacy. - Scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy.

For CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands, it was agreed that a UE can indicate the support of scaling the CSI processing timeline Z/Z’ by factor of 2 or the legacy CSI processing timeline. This agreement needs to be examined if the CSI report is for L1 beam reporting. For this case, the (Z, Z’) is defined as (Z3,Z3') of table 5.4-2, where Xµ is according to UE reported capability beamReportTiming and KBl is according to UE reported capability beamSwitchTiming. Considering the two parts of the CSI-RS falling within two DL subbands share same receiving beam, beam switching time KBl can be reused without scaled by the factor of 2. Therefore, only beam reporting time Xµ needs to be doubled.

Proposal 5: For CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands, if the associated CSI report is for L1 beam reporting, clarify whether both the beam reporting time Xµand beam switching time KBl need to be scaled by the factor of 2 if UE indicates the support of “Scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy.

Partial 4-RB CSI-RS

In current specification, the number of RBs of a CSI-RS resource is integer multiples of 4. The smallest configurable number is the minimum of 24 and the width of the associated BWP. If the configured value is larger than the width of the corresponding BWP, the UE shall assume that the actual CSI-RS bandwidth is equal to the width of the BWP. For a CSI-RS resource in SBFD symbols, the RBs of the CSI-RS resource may be outside SBFD DL subband(s). Reusing the legacy rules of determining actual bandwidth CSI-RS, only DL RBs of the CSI-RS resource within DL usable PRBs are valid. Note that this is only a simple extension of the RAN1#117 agreement which has already addressed the case wherein one CSI-RS is across two DL subbands.

Proposal 6: For a CSI-RS resource which overlaps with SBFD subband boundaries, only CSI-RS resources within DL subband(s) are valid for an SBFD-aware UE.

    - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols

      - Power control

For separate UL power control parameters in SBFD and non-SBFD slots, the following agreement was made in RAN1#119.

Agreement For a single TRP scenario, for separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols based on unified TCI state framework, - Option 2: Same unified TCI state is associated with separate UL power control parameters for SBFD symbols and non-SBFD symbols - New P0AlphaSets are introduced in Uplink-powerControl for SBFD symbols for PUSCH, PUCCH and SRS respectively

Separate Pcmax configurations for SBFD symbols and non-SBFD symbols

In RAN1#120bis, there was a discussion on whether to support separate maximum configured output power PCMAX,c for uplink transmission in SBFD symbols and non-SBFD symbols. The main benefit of putting an upper limit to the maximum output power in SBFD symbols is that the UE-to-UE CLI caused by uplink transmissions in SBFD symbols can be managed, especially for PRACH transmissions triggered by UE itself. Although it may be argued that this may reduce UL coverage gain achieved by SBFD, for the scenario where UE-to-UE CLI is severe, it can reduce the DL performance loss, thereby improve the overall system performance. It should be noted that whether or not to configure an additional PCMAX,c for SBFD symbols should be up to gNB. If the additional PCMAX,c for SBFD symbols is not configured, the legacy PCMAX,c configuration should be applied for both SBFD symbols and non-SBFD symbols. As analysed in [3], a separate Pcmax configuration for SBFD symbols also provides the flexibility to limit the UE-to-UE CLI in adjacent legacy TDD networks.

Based on the above discussion, we have the following proposal.

Proposal 7: Support separate PCMAX,c configurations for SBFD symbols and non-SBFD symbols.

Separate PHR reporting

Separate UL power control has been supported for UL transmissions in SBFD symbols and non-SBFD symbols. In RAN1#120bis, separate PHR reporting associated with UL transmissions in SBFD symbols and non-SBFD symbols were discussed. The main motivation is to provide the gNB with the information about the UE’s available transmit power on SBFD and non-SBFD symbols, facilitating more accurate scheduling. To support separate PHR reporting scheme, one possible solution is to reuse the PHR reporting mechanisms for mTRP, i.e., one PHR in non-SBFD symbols and one PHR in SBFD symbols are reported as illustrated in Figure 2. A new MAC CE needs to be defined by RAN2.

Figure 2. Illustration of separate PHR reporting based on twoPHRmode for mTRP

Proposal 8: Support separate PHR reporting for SBFD symbols and non-SBFD symbols based on the scheme defined for mTRP.

    - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots

      - PUSCH

For PUSCH, the following agreements were made in RAN1#119bis.

Agreement At least in case when PUSCH frequency hopping is not enabled, for a CG PUSCH configuration without repetitions, if the transmission occasions are across SBFD symbols and non-SBFD symbols where each transmission occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), for PUSCH repetition type-A across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PUSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PUSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for TBoMS across SBFD symbols and non-SBFD symbols in different slots, where each transmission within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), for determining starting PRB for PUSCH transmissions in SBFD symbols, - Equation 1-C2: RBstartSBFD=RBstartUL SB+RBstartnon-SBFD+RBoffsetSBFDmodNUL SBsize - If RBoffsetSBFD is not configured, it is zero

UE behaviour for PUSCH with Configuration 2

Based on the agreement of previous meetings, a SBFD-aware UE can be configured with Configuration 2 on a per UL BWP basis. And the number of PRBs are the same for PUSCH transmissions in SBFD symbols and non-SBFD symbols for Configuration 2. When a UE is configured with Configuration 2 on an UL BWP, for all the PUSCH repetition type-A across SBFD symbols and non-SBFD symbols in different slots or multi-PUSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols on this UL BWP, the frequency resource allocation for PUSCH must be located within the UL usable PRBs. However, the starting PRB for PUSCH transmission in SBFD symbols is determined based on Equation 1-C2, which is dependent on starting PRB for PUSCH transmissions in non-SBFD symbols RBstartnon-SBFD. Note that RBstartnon-SBFDcan be any PRB in non-SBFD symbols, there will be cases that RBstartSBFD is close to the edge of UL usable PRBs and the frequency resource allocation for PUSCH in SBFD symbols will be outside the usable PRBs. One may argue this can be avoided by gNB scheduler restrictions but this also reduces the PUSCH transmission opportunities.

To solve this issue, a fall-back mechanism from Configuration 2 to Configuration 1 can be introduced. As Configuration 1 is the basic capability for SBFD-aware UEs, a UE supporting Configuration 2 needs to support Configuration 1. When the UL BWP is configured with Configuration 2, if the frequency resource position of any PUSCH transmission in SBFD symbols is located outside UL usable PRB, UE will fall back to Configuration 1. This applies for PUSCH repetition type-A across SBFD symbols and non-SBFD symbols in different slots and multi-PUSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PUSCH within a slot has either all SBFD or all non-SBFD symbols. Another solution is that UE just drop the PUSCH transmission in SBFD symbols, i.e., no postpone for this repetition in case of PUSCH repetition type-A.

Proposal 9: For PUSCH repetition type-A across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e., Configuration 2), and for multi-PUSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PUSCH within a slot has either all SBFD or all non-SBFD symbols (i.e., Configuration 2), if the frequency resource allocation of any PUSCH transmission in SBFD symbols is outside the UL usable PRBs, UE falls back to Configuration 1 and transmits PUSCH only in non-SBFD symbols.

      - SPS HARQ-ACK transmission

For HARQ-ACK PUCCH transmission, the following agreements and conclusion were made in previous meetings.

Agreement It is clarified that ‘the first transmission/reception’ in the following agreement in RAN1#118 refers to the first transmission/reception occasion indicated by scheduling DCI. - It is not expected that the first transmission/reception occasion indicated by scheduling DCI is mapped to both SBFD and non-SBFD symbols. Agreement For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only, the valid symbol type is determined as following. <Unrelated part omitted> - For dynamically scheduled transmissions/receptions, the valid symbol type is determined based on the symbol type of the first transmission/reception. <Unrelated part omitted> Conclusion For SPS HARQ-ACK transmission when Configuration 1 is configured for the UL BWP, the symbol type can be different for different PUCCH transmission occasions.

When Configuration 1 is configured, for SPS HARQ-ACK transmission, it is concluded that the symbol type can be different for different PUCCH transmission occasions. Based on the conclusion, if the SPS HARQ-ACK is transmitted on PUCCH without repetition, it is clear that for each SPS PDSCH reception, the corresponding SPS HARQ-ACK PUCCH can be transmitted on symbols with different symbol types.

However, the UE behaviour is still not clear when SPS HARQ-ACK PUCCH is configured with repetition, whether the PUCCH repetitions for a SPS PDSCH reception can be across different symbol types. To ensure a unified solution for HARQ-ACK PUCCH repetition, it is preferred that the PUCCH repetitions for a SPS PDSCH reception are restricted to one symbol type when Configuration 1 is configured for the active UL BWP. And the valid symbol type of PUCCH repetitions for different SPS PDSCH receptions can be different.

When the SPS HARQ-ACK is transmitted together with dynamically scheduled HARQ-ACK in the same PUCCH and if the PUCCH resource is configured with repetition, the valid symbol type is determined based on the symbol type of the first transmission according to the above agreement. Therefore, when only the SPS HARQ-ACK is transmitted and if the PUCCH resource is configured with repetition, for each PUCCH repetitions corresponding to each SPS PDSCH reception, the valid symbol type can also be determined by the first transmission occasion of PUCCH repetitions.

For dynamically scheduled transmissions, it is not expected that the first transmission occasion indicated by scheduling DCI is mapped to both SBFD and non-SBFD symbols. But for SPS HARQ-ACK, as the PUCCH repetition occurs periodically, and the periodicity of SPS PDSCH and the periodicity of SBFD are configured separately, it is difficult to guarantee the first transmission occasion of each PUCCH repetition is mapped to the symbols with one symbol type. So for SPS HARQ-ACK PUCCH repetition, the first PUCCH transmission occasion can be postponed to the next slot if it is mapped to both SBFD and non-SBFD symbols, and the valid symbol type is determined by the first postponed transmission occasion.

Proposal 10: For SPS HARQ-ACK transmission with PUCCH repetition when UE is configured with Configuration 1 for the UL BWP and if the PUCCH carries SPS HARQ-ACK only

- The valid symbol type for SPS HARQ-ACK transmission is determined by the first PUCCH transmission occasion of the PUCCH repetitions.

  - The first PUCCH transmission occasion is postponed to the next slot if it is mapped to both SBFD and non-SBFD symbols.

- The valid symbol type can be different for different SPS HARQ-ACK transmission with PUCCH repetitions

      - CSI measurement and reporting

In this section, we discuss CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots. In the previous meetings, the following agreements were made.

Agreement For CSI report associated with periodic/semi-persistent CSI-RS, discuss and decide whether to support the following options. - Option A: For separate CSI reports on SBFD and non-SBFD, one CSI-ReportConfig is associated with CSI-RS(s) restricted to SBFD symbols only and the second CSI-ReportConfig is associated with CSI-RS(s) restricted to non-SBFD symbols only. - gNB configuration may not ensure that the CSI-RS associated with each CSI-ReportConfig is confined to either SBFD symbols or non-SBFD symbols only. - For the CSI-ReportConfig associated with CSI-RS(s) restricted to SBFD symbols only, only CSI-RS transmission occasions within SBFD symbols are used for CSI derivation. For the CSI-ReportConfig associated with CSI-RS(s) restricted to non-SBFD symbols only, only CSI-RS transmission occasions within non-SBFD symbols are used for CSI derivation. - Option B: Enhance Rel-18 NES CSI reporting framework to support one CSI-ReportConfig with one sub-configuration associated with SBFD symbols and the other sub-configuration associated with non-SBFD symbols Agreement For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation.

For a CSI report associated with aperiodic CSI-RS, only one of the aperiodic CSI-RS resource sets from the Resource Setting of the CSI report is associated with the trigger state, and the UE is configured per trigger state per Resource Setting to select the one aperiodic CSI-RS resource set from the Resource Setting. The selected aperiodic CSI-RS resource set consists of one or more aperiodic CSI-RS resources. If UE receives the DCI triggering the CSI report in slot n, UE receives the one or more aperiodic CSI-RS resources in slot n + offset, where the value of offset is given by aperiodicTriggeringOffset configured per CSI-RS resource set.

If the triggered aperiodic CSI-RS resource set occurs in a slot consists only SBFD symbols or only non-SBFD symbols, all the CSI-RS transmission occasions in the set can be used for CSI derivation as legacy. If the triggered aperiodic CSI-RS resource set occurs in a slot consists both SBFD symbols and non-SBFD symbols, it is difficult for UE to derivate CSI based from CSI-RS resources on both SBFD symbols and non-SBFD symbols. The valid symbol type for CSI derivation can also be used for aperiodic CSI-RS resources in above case.

Proposal 11: For a CSI report associated with aperiodic CSI-RS, the valid symbol type for CSI derivation for aperiodic CSI-RS resources for the CSI report is explicitly configured.

- When the aperiodic CSI-RS resources are triggered in a slot consists both SBFD symbols and non-SBFD symbols, only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation.

- When the aperiodic CSI-RS resources are triggered in a slot consists only SBFD symbols or only non-SBFD symbols, the valid symbol type is ignored and all CSI-RS transmission occasions are used for CSI derivation.

  - SBFD operation on one TDD carrier in multi-carrier

In RAN1#120, it was agreed that SBFD-aware UEs can support SBFD operation in one TDD carrier in multi-carrier scenario.

Agreement Support SBFD operation on one TDD carrier in multi-carrier scenario. - Note: If the TDD carrier for SBFD operation is an Scell, support UE dedicated signaling for cell-specific configuration of time and frequency location of SBFD subbands for the Scell. - FFS whether/how to handle half-duplex CA case

The above agreement implies that a SBFD-aware UE can be configured with carrier aggregation and SBFD operation is restricted on one TDD carrier within the configured carriers.

In the Rel-16 half-duplex CA TEI, the link direction collisions among different carriers are addressed. In case of CA and one of the TDD carrier is configured with SBFD operation, a SBFD-aware UE needs to handle both intra-carrier link direction collision and inter-carrier link direction collision. In particular, an SBFD symbols can be a downlink symbol configured by tdd-UL-DL-ConfigurationCommon. Therefore, it is not possible to apply the collision handling rules for Rel-16 half-duplex CA directly.

In Rel-16 half-duplex CA scenario, some collision handling rules were introduced to handle the potential collision between configured serving cells. In general, a UE supporting half-duplex CA should firstly determine a reference cell within the configured severing cells or cells of each band respectively depending on whether or not the UE is not capable of simultaneous transmission and reception as indicated by simultaneousRxTxInterBandCA among the multiple serving cells. Then the UE can handle the potential link direction collision between reference cell(s) and other cells based on the predefines rules summarized in the following table.

Table.1 Collision handling rules for half-duplex CA

| Same or different frequency band | Ref cell | Another cell | action |
| --- | --- | --- | --- |
| regardless of whether the reference cell and another cell operate in same or different frequency bands | Semi-U | DG-D | Error case |
|  | RRC-U | DG-D | Error case |
|  | Semi-D or RRC-D | RRC-U | drop U |
|  | Semi-U or RRC-U | RRC-D | drop D |
|  | RRC-U or RRC-D | Semi-D or Semi-U | Assume as flexible symbol |
|  | DG-U or DG-D | DG-D or DG-U | Error case |
| reference cell and another cell are in same frequency band | Semi-U or Semi-D | Semi-D or Semi-U | Error case |
|  | Semi-D | DG-U | Error case |
|  | RRC-U | DG-D | Error case |
| reference cell and another cell are in different frequency bands | Semi-D or Semi-U | Semi-U or Semi-D | Assume as flexible symbol Not receive RRC-D and transmit RRC-U |
|  | Semi-D | DG-U | Transmit U |
|  | RRC-D | DG-U | Transmit U |

It should be noted that in the half-duplex CA scenario, both intra-carrier (for flexible symbols in particular) and inter-carrier link direction collisions may occur simultaneously. Based on our understanding, the UE should first address potential intra-carrier link direction collisions for the configured serving cells. After resolving these, the UE can proceed to handle the potential inter-carrier link direction collisions, which includes determining the reference cell(s) and managing collisions among the configured serving cells.

Observation 1: In half-duplex CA case, a UE will first handle the potential intra-carrier link direction collisions for the configured severing cells. After resolving this, the UE continues to handle potential inter-carrier link direction collision, including determination of reference cell(s) and collision handling among configured serving cells.

In principle, a SBFD-aware UE should reuse the framework of existing collision handling rules for half-duplex CA as much as possible. For non-SBFD symbols, the current specification can be reused directly without modifications. For SBFD symbols, some necessary modifications and enhancements are needed.

Proposal 12: For non-SBFD symbols, current HD CA collision handling mechanism can be reused directly without modifications.

Definition of SBFD symbol for severing cell configured with SBFD operation

Considering that there is no semi-static link direction indication for SBFD symbols, from the perspective of UE, a SBFD symbol is similar as a flexible symbol regardless of whether or not it is indicated as downlink by tdd-UL-DL-ConfigurationCommon. Thus, in the procedure of collision handling, a SBFD symbol can be regarded as a flexible symbol, not a downlink symbol regardless of whether or not it is indicated as downlink by tdd-UL-DL-ConfigurationCommon.

Proposal 13: In the procedure of HD CA collision handling, a SBFD symbol is regarded as a flexible symbol regardless of whether it is indicated as downlink by tdd-UL-DL-ConfigurationCommon.

Determination of reference cell(s)

In current specification, for a symbol, the UE determine reference cells(s) according to the following specification.

TS 38.213 section 11.1 If a UE - is configured with multiple serving cells and is provided with directionalCollisionHandling-r16 = 'enabled' for a set of serving cell(s) among the configured multiple serving cells, and - indicates support of half-DuplexTDD-CA-SameSCS-r16 capability, and - is not configured to monitor PDCCH for detection of DCI format 2_0 on any of the multiple serving cells, the UE determines a reference cell for a symbol as an active cell with the smallest cell index among - the configured multiple serving cells if the UE is not capable of simultaneous transmission and reception as indicated by simultaneousRxTxInterBandCA among the multiple serving cells, and - the cells of each band respectively if the UE is capable of simultaneous transmission and reception by simultaneousRxTxInterBandCA for the configured multiple serving cells, where the symbol is configured by higher layers as - downlink, or uplink, as indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated - uplink, if the symbol is flexible and the UE is configured by higher layers to transmit SRS, PUCCH, PUSCH, or PRACH on the symbol - downlink, if the symbol is flexible and the UE is configured by higher layers to receive PDCCH, PDSCH or CSI-RS on the symbol.

The basic design principle of HD CA collision handling is that the transmissions or receptions on reference cells have higher priority than that of other cells for almost collision cases. So, if current determination method of reference cells is reused, the SBFD cell may not be determined as the reference cell, then the transmissions or receptions on the SBFD cell may be cancelled or not allowed, that will reduce SBFD performance gain. Thus, we propose to consider some enhancement for the determination of reference cell in SBFD scenario. For a symbol, if there were RRC configured transmissions or receptions in a severing cell configured with SBFD operation, then this severing cell should be determined as the reference cell regardless of the cell index.

Proposal 14: In the procedure of HD CA collision handling, for a SBFD symbol, the active cell configured with SBFD operation is determined as the reference cell if the UE is configured by higher layers to transmit SRS, PUCCH, PUSCH, or PRACH or receive PDCCH, PDSCH or CSI-RS on the symbol in this cell.

- Conclusions

In this contribution, we provide our views on evaluation on NR duplex evolution with following proposals:

Observation 1: In half-duplex CA case, a UE will first handle the potential intra-carrier link direction collisions for the configured severing cells. After resolving this, the UE continues to handle potential inter-carrier link direction collision, including determination of reference cell(s) and collision handling among configured serving cells.

Proposal 1: UE-specific configuration on frequency locations of SBFD subbands is not supported in Rel-19.

Proposal 2: The guard period between SBFD symbols and non-SBFD symbols is implicitly provided by the gNB.

Proposal 3: If a UE is scheduled with a PDSCH across two DL subbands in SBFD symbols, the UE shall assume the PRG size is equal to 2 PRBs.

Proposal 4: The following existing restriction is relaxed for SBFD-aware UEs in SBFD symbols.

- A UE is not expected to handle the case where PDSCH DM-RS REs are overlapping, even partially, with any RE(s) not available for PDSCH.

Proposal 5: For CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands, if the associated CSI report is for L1 beam reporting, clarify whether both the beam reporting time Xµand beam switching time KBl need to be scaled by the factor of 2 if UE indicates the support of “Scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy.

Proposal 6: For a CSI-RS resource which overlaps with SBFD subband boundaries, only CSI-RS resources within DL subband(s) are valid for an SBFD-aware UE.

Proposal 7: Support separate PCMAX,c configurations for SBFD symbols and non-SBFD symbols.

Proposal 8: Support separate PHR reporting for SBFD symbols and non-SBFD symbols based on the scheme defined for mTRP.

Proposal 9: For PUSCH repetition type-A across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e., Configuration 2), and for multi-PUSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PUSCH within a slot has either all SBFD or all non-SBFD symbols (i.e., Configuration 2), if the frequency resource allocation of any PUSCH transmission in SBFD symbols is outside the UL usable PRBs, UE falls back to Configuration 1 and transmit PUSCH only in non-SBFD symbols.

Proposal 10: For SPS HARQ-ACK transmission with PUCCH repetition when UE is configured with Configuration 1 for the UL BWP and if the PUCCH carries SPS HARQ-ACK only

- The valid symbol type for SPS HARQ-ACK transmission is determined by the first PUCCH transmission occasion of the PUCCH repetitions

  - The first PUCCH transmission occasion is postponed to the next slot if it is mapped to both SBFD and non-SBFD symbols.

- The valid symbol type can be different for different SPS HARQ-ACK transmission with PUCCH repetitions

Proposal 11: For a CSI report associated with aperiodic CSI-RS, the valid symbol type for CSI derivation for aperiodic CSI-RS resources for the CSI report is explicitly configured.

- When the aperiodic CSI-RS resources are triggered in a slot consists both SBFD symbols and non-SBFD symbols, only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation.

- When the aperiodic CSI-RS resources are triggered in a slot consists only SBFD symbols or only non-SBFD symbols, the valid symbol type is ignored and all CSI-RS transmission occasions are used for CSI derivation.

Proposal 12: For non-SBFD symbols, current HD CA collision handling mechanism can be reused directly without modifications.

Proposal 13: In the procedure of HD CA collision handling, a SBFD symbol is regarded as a flexible symbol regardless of whether it is indicated as downlink by tdd-UL-DL-ConfigurationCommon.

Proposal 14: In the procedure of HD CA collision handling, for a SBFD symbol, the active cell configured with SBFD operation is determined as the reference cell if the UE is configured by higher layers to transmit SRS, PUCCH, PUSCH, or PRACH or receive PDCCH, PDSCH or CSI-RS on the symbol in this cell.

# References

- R1-2405425, Summary #4 of SBFD TX/RX/measurement procedures, Moderator (CATT)

- R1-2503265, Discussion on LS on the transient period for SBFD operation, Huawei, HiSilicon

- R1-2502652, Discussion on SBFD Tx/Rx/measurement procedures, Charter Communications