3GPP TSG-RAN WG1 Meeting #121	R1-2503285

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	9.15.8

Source:	Huawei, HiSilicon

Title:	UE features for NTN for NR phase 3

Document for:	Discussion and Decision

# Introduction

In this contribution, UE features for NTN DL coverage enhancement and UL capacity enhancement, are discussed respectively.

# UE feature for NR NTN DL coverage enhancement

## UE feature for SSB periodicity extension

In RAN1#118ibs [1] and RAN1#120 [3], following agreements were made with respect to SSB periodicity extension in NR NTN.

Agreement For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is at least 160 ms. - FFS: whether 320ms can be supported as the maximum of the additional default value instead of or in addition to 160ms Agreement For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The additional default value assumed by UE during initial access (apart from the existing 20ms value) is 160 ms.

During last meeting [4], FG 65-1-1 is adopted with some FFS highlights for the support of 160ms SSB periodicity during initial access for Rel-19 NR-NTN as shown following.

| 65-1-1 | 160 ms SSB periodicity assumed during initial access | 1. Support additional default value (apart from 20 ms value) of 160ms periodicity of the half frames with SS/PBCH blocks during initial [cell selection or access] [FFS Other components] | FFS | FFS | N/A | FFS: UE may assume 20 ms periodicity of the half frames with SS/PBCH blocks during initial access | Per band | N/A | N/A | N/A | Note: As described in the WID, this UE feature group is applicable only for bands in Tables 5.2.2-1 [and 5.2.3-1] in TS 38.101-5 | Optional [with/without] capability signaling FFS: A UE that supports Rel-19 NR-NTN must support this FG |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Regarding the agreed FG, the motivation of introducing extended SSB is to enable the network to extend the coverage ratio. To achieve this target, FG 65-1-1 should be mandatory support if the UE supports Rel-19 NR NTN. The coverage ratio on network would not be improved if the Rel-19 NTN UE cannot support it. Therefore, we propose FG 65-1-1 as mandatory without capability signalling and it is a UE that supports Rel-19 NR-NTN must support this FG.

Secondly, for the note in this FG, the agreement does not mention extended SSB periodicity is only applicable to FR1-NTN, therefore, the square brackets should be removed in the note.

For the prerequisite, note that FG26-1 is for support of UL time and frequency pre-compensation and timing relationship enhancements to NTN, it is not necessary for procedure of UE detecting SSB or receiving Type0 PDCCH and SIB1. So the FFS in the prerequisite column should be removed.

For the consequence if the feature is not supported, it is more straight forward to say “UE does not support the extended default SSB periodicity of 160ms”.

In summary, the following proposal is provided:

Proposal 1: Update the FG 65-1-1 for UE supporting of 160ms SSB periodicity, as shown in the Annex:

- Update the UE feature as mandatory or remove the FFS before “A UE that supports Rel-19 NR-NTN must support this FG”;

- Remove the FFS in the prerequisite column;

- Update the column of “Consequence if the feature is not supported by the UE” as “UE does not support the extended default SSB periodicity of 160ms”.

## UE feature for link level coverage enhancement

During RAN1#119 [2] to RAN1#120bis [4], following agreements with respect to PDCCH repetition in CSS types other than Type3, and SIB1 PDSCH repetition, were made in RAN1.

Agreement For PDCCH CSS (except Type-3) link level enhancements, support only PDCCH repetition for NTN. - FFS: intra-slot and/or inter-slot Agreement Support only inter-slot repetition for Type0 PDCCH CSS. Agreement For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1, support repeated PDCCH candidates in the two consecutive slots n0 and n0+1 associated with the same SSB index ( n0 as defined in section 13 of TS 38.213) at least for M=2. - Repeated PDCCH candidates share the same aggregation level (AL), coded bits and same candidate index - Note: if the network repeats the Type 0 PDCCH across two consecutive slots, a legacy UE might decode the PDCCH and associated PDSCH in one slot and skip PDCCH monitoring in the other slot. Note: further discuss the potential solution for M=1 and M=1/2. Agreement For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1: - Enabling/disabling using reserved bit(s) in PBCH payload - No UE behavior is defined for UE in connected mode specifically for the case where the network changes its signaling between enabling and disabling PDCCH repetition for Type0 PDCCH CSS. Working assumption For PDCCH CSS other than Type-0 CSS and other than Type-3 CSS for common search spaces other than SearchSpaceZero, intra-slot PDCCH repetition is supported. RAN1 to down select between option 1 and option 2: Option 1: Use same CORESET and two different SS (SS Set1 and SS Set2) - Linking two PDCCH candidates (adopt the same mechanism for SS linking specified in Release 17) - FFS: Blind decoding limit Option 2: Use same CORESET associated with one SS which is repeated by introducing symbol domain offset X - FFS: Blind decoding limit - FFS: details configuration and signalling Agreement For link level enhancement of PDSCH with SIB1: - Support PDSCH repetitions within 20 ms duration - The number of repetitions is fixed to 2 repetitions - Further discuss the specification impact for at least the following: - Procedure and signaling (enabling repetitions, associated time resource determination, etc.) - Note 1: without the above PDSCH repetitions, the coverage gap is 2.2 dB to 4.6 dB depending on SIB1 size. - Note 2: Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level. Note 3: the above is not related to multiple SIB1 transmissions across 20 ms periodicities of SSB, which may not be available when the SSB periodicity is 160 ms or larger (if supported) depending on the SSB and CORESET multiplexing pattern. Agreement When PDCCH CSS type-0 repetition is performed, for SIB1 link level enhancement, support PDSCH repetition of SIB1 transmitted within the same slot as the type0-CSS PDCCH repetition. - UE supporting SIB1 PDSCH coverage enhancement assumes that the PDCCH and associated PDSCH to be repeated in both slots where the corresponding PDCCHs are transmitted. - Each PDSCH SIB1 repetition is within the same slot of each PDCCH candidate for scheduling DCI - The two associated PDSCHs have the same RV FFS: Whether it is supported that type-0 PDCCH repetition is not performed while the PDSCH-SIB1 repetition is performed, and if so whether/how to handle the slot determination. Agreement For enabling/disabling SIB1 PDSCH repetition, RAN1 to consider the following options: - Option 1: Using reserved bit(s) in PBCH payload. - Option 2: Using scheduling PDCCH. - Option 3: The enabling/disabling of SIB1 PDSCH repetition is implicitly indicating by the enabling/disabling of Type-0 CSS PDCCH repetition.

During last meeting, the following FG 65-1-2 is adopted for PDCCH repetition for Type0 PDCCH CSS.

| 65-1-2 | PDCCH repetition for Type0 PDCCH CSS | 1. Support reception of PDCCH repetition for Type0 PDCCH CSS [of searchSpaceZero configured within MIB pdcch-ConfigSIB1] FFS: whether to merge this FG with FG(s) for PDCCH repetition for other than Type0 PDCCH CSS FFS: whether to merge this FG with FG(s) for SIB1 PDSCH repetition | FFS: | FFS | N/A | PDCCH repetition for Type0 PDCCH CSS is not supported | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable [only] for bands in Tables 5.2.2-1 [and 5.2.3-1] in TS 38.101-5 [FFS: Applicabilitiy of this FG to TN] | Optional [with/without] capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

As discussed in RAN1#120bis email discussion, as guided by Mr. Chairman, RAN1 will discuss and decide whether these NTN features/agreements are applicable for TN. Firstly, it is very clear that common PDCCH repetition is beneficial for TN scenario considering PDCCH repetition is used. Secondly, from UE capability perspective, once the UE implements the combination of PDCCH repetitions for NR NTN band, there is no additional UE complexity to support the band for TN scenarios. Considering this, we propose update the yellow part in the note to make this UE feature applicable to TN.

For the components part in FG65-1-2, if a UE faces coverage issue for Type0-CSS PDCCH for SIB1, the UE may also face the coverage issue for PDCCH of other CSS other than Type3-CSS. In this sense, we think it makes sense to merge FG 65-1-2 with the FG for PDCCH repetition for CSS other than Type0 PDCCH CSS. All these CSS types are related to the scheduling of SIB1, SIB19, Msg2 and Msg4, which are all important for UE reception during the initial access procedure. And since all these PDCCH CSS types are already agreed to be enhanced by repetition transmission in RAN1, it is reasonable to avoid UE capability fragmentation by having all these link level enhancements covered as one single FG.

In addition, SIB1 PDSCH repetition is also agreed in RAN1. And based on RAN1 discussion, both the potential repetition technique and signaling design for SIB1 PDSCH repetition are coupled with that of Type0 PDCCH repetition. Hence, we would also like to merge these two features together in FG65-1-2.

Proposal 2: Merge the FG for PDCCH reception for CSS other than Type0 PDCCH CSS and the FG for SIB1 PDSCH repetition into FG65-1-2.

The bracket in the first component should be removed, since it is agreed in RAN1 that Type0 PDCCH CSS repetition is of SearchSpaceZero configured within MIB pdcch-ConfigSIB1.

For the same reason as given for FG 65-1-1, the FFS in ‘Prerequisite feature groups’ should be removed for FG 65-1-2.

Based on the above discussion, we have the following proposal:

Proposal 3: Update the FG 65-1-2 as in Annex:

- Update the yellow part in the note to make this UE feature applicable to TN;

- Remove the square brackets in the first component;

- Include the component for PDCCH repetition for CSS other than Type0-CSS and other than Type3-CSS;

- Include the component for SIB1 PDSCH repetition within 20 ms duration;

- Remove the FFS in the prerequisite column.

# UE feature for NR UL capacity enhancement

In RAN1#119 [2] and RAN1#120bis [4], agreements have been achieved to support UE capabilities for inter-slot OCC.

Agreement RAN1 to confirm the working assumption of RAN1#118bis with revisions as follows: - Support OCC length 2 with inter-slot OCC to multiplex up to 2 UEs. - Support Option 1: inter-slot OCC with OCC length 4 to multiplex up to 4 UEs using Hadamard sequences - RAN1 does not pursue Option 2: Intra-symbol pre-DFT OCC with OCC length 4 to multiplex up to 4 UEs. - RAN1 does not pursue Option 3: Combination of Inter-slot OCC with OCC length 2 and intra-symbol pre-DFT OCC with OCC length 2 to multiplex up to 4 UEs. Note 1: there will be separate UE capabilities for OCC length 2 and OCC length 4, where UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4. Note 2: gNB can ensure the performance of Option 1 by UE grouping with similar CFO (e.g. maximum differential CFO of 50 or 100 Hz or 200 Hz). Without CFO grouping (e.g. maximum differential CFO of 400 Hz), the performance of option 1 is degraded by at least 1 dB in several cases. For CFO grouping, several companies in RAN1 state that CFO grouping is feasible based on network implementation without any new specification impact. - RAN1 assumes no specification impact for CFO grouping - RAN1 does not pursue closed-loop frequency adjustment commands. - RAN1 assumes that RAN4 does not define new UE requirements for CFO. Agreement RAN1 assumes that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH. - FFS: under which conditions the above applies, e.g. under the same conditions that phase continuity applies for DMRS bundling Agreement For resolving the overlapping PUSCH repetitions with inter-slot OCC and PUCCH with/without repetitions, the legacy timeline conditions for UCI multiplexing or prioritization for dropping PUSCH [/ PUCCH] applies according with the following update: - “the first PUSCH repetition of an OCC group” is used instead of “PUSCH repetition” for the legacy timeline condition, where the legacy PUSCH repetition that overlaps with a PUCCH belongs to the OCC group Note: how to capture the above agreement is up to the spec editor. Agreement If PUCCH without repetitions overlaps with inter-slot OCC with any PUSCH repetitions in an OCC group with a same priority index for PUCCH/PUSCH, the legacy conditions and rules for UCI multiplexing or prioritization for dropping applies with the following updates: - If the UCI is multiplexed on the PUSCH repetition according to legacy rules and the updated timeline conditions for UCI multiplexing are satisfied, UCI is multiplexed on all PUSCH repetitions without A-CSI reports within an OCC group with inter-slot OCC overlaps with the PUCCH. (Option 3-a) - FFS: PUSCH repetition with A-CSI reports - If the PUSCH repetition is dropped according to legacy rules and the updated timeline conditions for PUSCH dropping are satisfied, UCI is transmitted on PUCCH and all PUSCH repetitions within the OCC group that overlaps with the PUCCH are dropped (Option 2) - FFS: if PUCCH is overlap with PUSCH repetition in both time and frequency domain. - UE does not expect there are multiple PUCCHs without repetitions in different PUCCH slots with a same or different UCI types other than SR overlapping with multiple PUSCH repetitions in the same OCC group. - FFS: whether the above applies only when at least one of the overlapping PUCCHs result in a UCI being multiplexed on the PUSCH Note 1: If the UCI on the PUCCH is dropped according to legacy rules and [updated] timeline conditions for UCI dropping are satisfied, there is no [additional] spec impact. (Option 1) Note 2: There can be multiple PUCCHs with same or different UCI types in the same slot (i.e. CSI report and HARQ-ACK) as in the legacy specifications Working assumption 1: The above agreement applies to different priority indexes for PUCCH/PUSCH if no additional specification impact is identified. Working assumption 2: The above agreement applies to PUCCH with repetitions if no additional specification impact is identified.

And during RAN1#120bis, new FG 65-3-1 and FG 65-3-2 have been agreed to be introduced for the support of PUSCH with inter-slot slot OCC of length 2 and length 4, respectively. For resolving the overlapping PUSCH repetitions with inter-slot OCC and PUCCH with/without repetitions, UE should support multiplexing UCI on all PUSCH repetitions without A-CSI report or dropping PUSCH repetitions within the OCC group if the UCI is multiplexed on the PUSCH repetition or the PUSCH repetition is dropped according to legacy rules and updated timeline conditions. And UE should also have the ability to keep phase continuity and power consistency for the duration of one OCC group. Furthermore, since inter-slot OCC can also be applied without DMRS bundling, inter-slot OCC and DMRS bundling should be decoupled and feature 30-4a should not be a prerequisite for inter-slot OCC. As the inter-slot OCC is only evaluated in NTN, further discussion is needed to evaluate whether it can be extended to TN with a different channel model.

Proposal 4: Adopt the FG 65-3-1 and FG 65-3-2 in Annex for UE supporting inter-slot OCC 2 with PUSCH repetition type A and inter-slot OCC 4 with PUSCH repetition type A, respectively.

# UE feature for NR HD-FDD operation of (e)RedCap in FR1-NTN bands

## Support of (e)RedCap HD-FDD collision handling

It was agreed in RAN1#119 [2] that for a (e)RedCap UE in RRC-connected mode, a default priority rule is adopted for the UE to handle the HD collisions in Case 3 wherein DL reception is prioritized to UL transmission, with the exception of the collision with Type-0/0A/1/2-PDCCH CSS  which is left to UE implementation. It was also agreed that the default rule can be overridden by an RRC network indication.

Agreement For Rel-19 HD-FDD (e)Redcap UE in RRC connected mode, for collision case 3, - Handling of collision with Type-0/0A/1/2-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note. - For other use cases, default priority rule for collision case 3 in RRC-Connected mode is that DL is prioritized. - Network is allowed to indicate UL overriding DL for all the other use cases above - This is signaled by RRC configuration Note: UE shall comply to the following existing procedure in 38.331: UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

Although gNB would attempt to avoid collisions between dynamic DL reception and dynamic UL transmission for a given HD-FDD UE by proper scheduling, such scheduling would be based on the latest TA assumed for the UE or latest reported by the UE. As such, Case 4 collisions may still occur due to the TA mismatch with the actual TA used by that UE. Since this situation is in fact not different from Case 3 collisions, the following it was agreed last meeting to pdate the Working Assumption from RAN1#120 [3] for a HD-FDD UE in RRC-connected state as follows:

Agreement Update the working assumption in RAN1 #120 with the following updates: For Rel-19 NTN HD-FDD (e)Redcap UE in RRC connected mode, the following handling rule for collision case 4 is supported: - Handling of collision with PDSCH (at least for system information) scheduled by Type-0/0A-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the note2. - Handling of collision with PDSCH scheduled by Type-1/2-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the note2. - FFS: handling of PDCCH ordered PRACH transmission - For other use cases, default priority rule for collision case 4 in RRC-Connected mode is that DL is prioritized. - Network is allowed to indicate UL overriding DL for all cases - This is signaled by one UE specific RRC parameter - Note1: if DL is prioritized, the DL prioritization applies only if the UL cancellation timeline can be satisfied, otherwise UL is prioritized. Note2: UE shall comply to the following existing procedure in 38.331: - UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

Since the majority view in RAN1 is to devise a solution for Case 4 that is similar in principle to the solution agreed for Case 3, we assume that the WA can be confirmed in this meeting with the following modifications:

•	Removing the restriction of PDSCH scheduled by Type0/0A-PDCCH in CSS to system information

•	No exceptional use case for PDCCH-ordered PRACH

In the last meeting RAN1#120bis [6] it was agreed to support a FG for handling the collision cases 3 and 4 but with following issues left for further study:

  - Whether to divide this FG into two separate FGs between case 3 and case 4

  - Whether to divide this FG into two separate FGs between RRC CONNECTED and INACTIVE state

  - Which NTN and/or (e)RedCap FGs  should be set as pre-requisite

In our view, in order for the Rel-19 HD-FDD (e)RedCap UE to handle the collision cases other than cases 3 and 4, a pre-requisite for supporting the new FG should be to support FG 28-3 (Half-duplex FDD operation type A for RedCap UE) which in turn requires the UE to support either FG 28-1 (RedCap UE) or FG 48-1 (eRedCap UE) since the specifications for a UE supporting FG 28-1 also applies for a UE supporting FG 48-1 unless stated otherwise. Furthermore, given that the issue addressed by the newly introduced collision handling Cases 3 and 4 is the same (TA mismatch between UE and gNB) and the agreed solution is the same (default DL priority rule with possible RRC override), we do not see a plausible reason for a HD-FDD (e)RedCap UE to support handling of one case and not the other, and thus we do not see a clear motivation to split these two cases into two different FGs. Similar method had been adopted in R17 in which handling rules for all collision cases are included in the same FG. Introducing separate FG for case 3 and case 4 may further complicate specification design for UE only support either case 3 or case 4.  Also, since the consensus in RAN1 was to leave all collision handling for an UE in INACTIVE state to UE implementation, there is no need to address such a state in a separate FG.

As to whether the support of this FG should be mandatory or optional, we believe it should be ‘Optional with Capability Signaling’ since the network can still handle the issue for some very low cost (e)RedCap UEs that incapable of performing the Rel-19 handling procedures via conservative scheduling though on the expense of spectral efficiency of those UEs.

Proposal 5: Adopt the FG 65-2-1 in Annex for supporting Rel-19 NR HD-FDD operation of (e)RedCap in FR1-NTN bands with the following update.

# Conclusions

In this contribution, the following are proposed:

Proposal 1: Update the FG 65-1-1 for UE supporting of 160ms SSB periodicity, as shown in the Annex:

- Update the UE feature as mandatory or remove the FFS before “A UE that supports Rel-19 NR-NTN must support this FG”;

- Remove the FFS in the prerequisite column;

- Update the column of “Consequence if the feature is not supported by the UE” as “UE does not support the extended default SSB periodicity of 160ms”.

Proposal 2: Merge the FG for PDCCH reception for CSS other than Type0 PDCCH CSS and the FG for SIB1 PDSCH repetition into FG65-1-2.

Proposal 3: Update the FG 65-1-2 as in Annex:

- Update the yellow part in the note to make this UE feature applicable to TN;

- Remove the square brackets in the first component;

- Include the component for PDCCH repetition for CSS other than Type0-CSS and other than Type3-CSS;

- Include the component for SIB1 PDSCH repetition within 20 ms duration;

- Remove the FFS in the prerequisite column.

Proposal 4: Adopt the FG 65-3-1 and FG 65-3-2 in Annex for UE supporting inter-slot OCC 2 with PUSCH repetition type A and inter-slot OCC 4 with PUSCH repetition type A, respectively.

Proposal 5: Adopt the FG 65-2-1 in Annex for supporting Rel-19 NR HD-FDD operation of (e)RedCap in FR1-NTN bands with the following update.

# References

- Chair’s notes RAN1#118bis.

- Chair’s notes RAN1#119.

- Chair’s notes RAN1#120.

- Chair’s notes RAN1#120bis.

- R1-2502219, Discussion on downlink coverage enhancements for NR NTN, Huawei, HiSilicon

- R1-2502697, Session Notes of AI 9.15.8, NTT DoCoMo

# Annex: UE features for Rel-19 NTN

| Index | Feature group | Components |  | Prerequisite feature groups | Need for the gNB to know if the feature is supported |  | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. |  | Consequence if the feature is not supported by the UE |  | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) |  | Need of FDD/TDD differentiation |  | Need of FR1/FR2 differentiation |  | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 |  | Note |  | Mandatory/Optional |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 65-1-1 | 160 ms SSB periodicity assumed during initial access | 1. Support additional default value (apart from 20 ms value) of 160ms periodicity of the half frames with SS/PBCH blocks during initial cell selection [FFS Other components] |  |  | FFS |  | N/A |  | UE does not support the extended default SSB periodicity of 160ms |  | Per band |  | N/A |  | N/A |  | N/A |  | Note: As described in the WID, this UE feature group is applicable only for bands in Tables 5.2.2-1 and 5.2.3-1 in TS 38.101-5 |  | Mandatory A UE that supports Rel-19 NR-NTN must support this FG |  |
| 65-1-2 | PDCCH repetition for Type0 PDCCH CSS | 1. Support reception of PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1 2. Support reception of SIB1 PDSCH repetition within 20ms duration 3. Support reception of PDCCH repetition for Type [0A/0B/1/1A/2/2A] PDCCH CSS |  |  | FFS |  | N/A |  | 1. PDCCH repetition for Type0 PDCCH CSS is not supported 2. SIB1 PDSCH repetition within 20ms duration is not supported 3. PDCCH repetition for Type [0A/0B/1/1A/2/2A] CSS is not supported |  | Per band |  | N/A |  | N/A |  | N/A |  | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 and 5.2.3-1 in TS 38.101-5 This FG is applicable to TN |  | Optional [with/without] capability signaling |  |
| 65-3-1 | PUSCH with inter-slot OCC of length 2 | 1. Support inter-slot OCC with OCC length 2 for DFT-s-OFDM PUSCH transmissions with repetition Type A. 2. Support RV cycling across OCC groups for DG-PUSCH and CG-PUSCH. 3. Support maintaining phase continuity and power consistency for the duration of one OCC group. 4. Support multiplexing UCI on all PUSCH repetitions without A-CSI report or dropping PUSCH repetitions within the OCC group if the UCI is multiplexed on the PUSCH repetition or the PUSCH repetition is dropped according to legacy rules and updated timeline conditions. 5. FFS: Support frequency hopping across OCC groups. |  | 5-14, 5-16, 5-17, 11-6, 26-1, 30-1, 30-2 | YES |  |  |  | UE does not support inter-slot OCC for DFT-s-OFDM PUSCH with repetition Type A |  | Per band |  | N/A |  | FR1 only |  | N/A |  | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5. |  | Optional with capability signaling |  |
| 65-3-2 | PUSCH with inter-slot OCC of length 4 | 1. Support inter-slot OCC with OCC length 4 for DFT-s-OFDM PUSCH transmissions with repetition Type A. |  | 65-3-1 | YES |  |  |  | UE does not support OCC length 4 for DFT-s-OFDM PUSCH with repetition Type A |  | Per band |  | N/A |  | FR1 only |  | N/A |  | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 |  | Optional with capability signaling |  |
| 65-2-1 | Handling collision Case 3 and collision Case 4 for half-duplex FDD operation for (e)RedCap UE |  | 1. Support handling collision Case 3 2. Support handling collision Case 4 | 26-1, 26-4, 28-1, 28-3, 48-1, 48-2 | YES | N/A |  | Handling collision Case 3 and collision Case 4 for half-duplex FDD operation for (e)RedCap UE is not supported |  | Per band |  | FDD only |  | FR1 only |  | N/A |  | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 Note: collision Case 3: Semi-statically configured DL reception collides with semi-statically configured UL transmission Note: collision Case 4: Dynamically scheduled DL reception collides with dynamic scheduled UL transmission Note: this FG is for FDD only Note: Case 3 handling is for RRC CONNECTED and INACTIVE states, and Case 4 is for RRC CONNECTED state only |  | Optional with capability signaling |  |