3GPP TSG-RAN WG1 Meeting #121	R1-2503286

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	9.15.9

Source:	Huawei, HiSilicon

Title:	UE features for IoT-NTN Phase 3

Document for:	Discussion and Decision

Introduction

In RAN1#120bis [1], two FGs have been agreed on OCC for NPUSCH format 1 with 3.75kHz SCS and 15kHz SCS respectively. In this contribution, we further discuss the remaining issues for these two FGs.

UE feature for IoT-NTN UL capacity enhancement

In RAN1# #120bis meeting [2], the following agreements have been made with respect to NPUSCH format 1 with OCC in IoT-NTN. In addition, based on the LS reply from R4-2504716, no feasibility issue is identified for TDM DMRS for 3.75kHz SCS OCC. Thus, the bracket on component 2 and component 3 of FG1-1 and FG1-2 can be removed.

Agreement Dynamic activation / deactivation of OCC is supported by DCI. - FFS: details of signalling by DCI Agreement For NPUSCH Format 1 single-tone 15kHz SCS, for CDM DMRS with legacy pattern: - DMRS symbols are spread before the OCC is applied - Option 1_1: according to the formula: ru,OCCMn+m=runqm, 0≤n<X/M, m=0, …, M-1 Where: M is the OCC length, q is the assigned OCC codeword for the UE, run is the reference signal sequence defined in TS36.211 section 10.1.4.1.1 and X is the total number of slots in the NPUSCH transmission after OCC is applied

Considering the OCC for NPUSCH format 1 is added on top of R17 IoT NTN basic FG, Rel-17 FG2-1b should be the prerequisite for FG1-1 and FG1-2. Due to limited capability of NB IoT UE, the overhead of UE capability reporting should be kept as small as possible. Thus, we propose the report granularity is per UE, which is aligned with other IoT NTN FGs in previous releases.  We support this UE feature group is applicable only for IoT-NTN cell for communication via satellite, for any other cell this feature is not supported.

Proposal 1: Adopt the changes on the IoT NTN FGs identified in the Table in Annex.

Conclusion

According to the above discussions, we have the following proposals:

Proposal 1: Adopt the changes on the IoT NTN FGs identified in the Table in Annex.

Reference

[1] R1-2502968, “Session Notes of AI 9.15.9”

[2] R1-2502333, “Final FL summary for IoT-NTN”

# Annex 1: UE features for Rel-19 IoT-NTN

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. IoT_NTN_Ph3 | 1-1 | OCC for single-tone NPUSCH format 1 with 3.75 kHz SCS in RRC_CONNECTED | 1. Support of symbol-level length-2 OCC for single-tone NPUSCH format 1 with 3.75 kHz SCS in RRC_CONNECTED 2. Support of TDM DMRS over 4 slots where DMRS are transmitted in the first 2 slots and DMRS REs are blanked in the next 2 slots, or vice-versa 3. Dynamic activation/deactivation of OCC feature 4. FFS: Support same RV value within an OCC group for NoccNRUNslotsUL slots [FFS: Whether to have other component(s)] | 2-1b | Yes | N/A | UE does not support OCC for single-tone NPUSCH format 1 with 3.75 kHz SCS in RRC_CONNECTED | Per UE | N/A | N/A | [FFS: Whether to separate this FG for GSO and NGSO] Note: This FG is applicable only to [IoT-NTN band] | Optional with capability signalling |
| 1. IoT_NTN_Ph3 | 1-2 | OCC for single-tone NPUSCH format 1 with 15 kHz SCS in RRC_CONNECTED | 1. Support of slot-level length-2 OCC for single-tone NPUSCH format 1 with 15 kHz SCS in RRC_CONNECTED 2. Support of CDM DMRS with the pattern supported for NPUSCH format 1 with 15 kHz SCS 3. Dynamic activation/deactivation of OCC feature 4. FFS: Support same RV value within an OCC group for NoccNRUNslotsUL slots [FFS: Whether to have other component(s)] | 2-1b | Yes | N/A | UE does not support OCC for single-tone NPUSCH format 1 with 15 kHz SCS in RRC_CONNECTED | Per UE | N/A | N/A | [FFS: Whether to separate this FG for GSO and NGSO] Note: This FG is applicable only to [IoT-NTN band] | Optional with capability signalling |