3GPP TSG RAN WG1 #121	R1-2503301

St Julian's, Malta, May 19th - 23rd, 2025

Source:	Nokia

Title:	AIoT Timing acquisition and synchronization

Agenda item:	9.4.3

Document for:	Discussion and Decision

# Introduction

At RAN#106, a new work item “Solutions for Ambient IoT (Internet of Things) in NR” (Ambient_IoT_Solutions) was approved; the WID was most recently revised in [1].

The following objective(s) is/are relevant for the AIoT while considering the topic specific to each agenda item:

General Scope The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope: - The overall objective shall be to standardize the following Ambient IoT device: Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Deployment scenario 1 with Topology 1, according to D1T1-B. - FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum. - Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor. - Traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command). - Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769: - Case 1-4 for D1T1-B - Proximity determination via Solution 1 in TR 38.769 only. - Device (un)availability via Direction 1 in TR 38.769 only. WGs begin their discussions from the decisions already made in TR 38.769, with the following refinements for the scope:

From RAN1 perspective, following topics are considered for further evaluation. The topic specific to this agenda item is highlighted

RAN1 scope: - PRDCH and PDRCH, which are the only physical channels in R2D and D2R, respectively. - R2D and D2R signal(s) - Multiplexing/multiple access in R2D is by only TDMA, and in D2R is by only TDMA and FDMA. - R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations. - R2D transmission supports only the Manchester line code in TR 38.769 - D2R transmission supports: - Either the Manchester line code in TR 38.769 or no line code (one to be down-selected); and - A corresponding small frequency shift method according to the options in TR 38.769. - R2D does not support FEC. D2R supports only convolutional code with generator polynomials as per TS 36.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis). - PRDCH and PDRCH both support transmission without CRC, and with CRC as per the generator polynomials in TS 38.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis) for 6-bit CRC and 16-bit CRC. Cases to use which length of CRC, or no CRC, to be decided in RAN1. - D2R supports physical layer repetition transmission. R2D does not support physical-layer repetition transmission.

# Discussion

In this agenda, we discuss the design of timing acquisition and clock synchronization signal used before PDRCH and PRDCH transmissions. Unlike the data channel, the timing acquisition and clock synchronization signal provides frame boundary and sampling accuracy for AIoT device before receiving PRDCH transmission. Similarly, in the PDRCH transmission, the AIoT device transmits synchronization signals to provide timing and sampling accuracy for the reader to detect the AIoT transmission. To distinguish the synchronization signal design specific to PRDCH and PDRCH, we present them depending on the transmission type.

## R2D Transmission

### Preamble

The R-TAS start-indicator part is not included in TD2R_min, and an ON/OFF pattern i.e., high/low voltage transmission, is applied. The following options have been studied for the start-indicator part: - Option 1: ON-OFF transmission is considered based on energy/edge detection, and multiple alternatives have been studied including - Alt 1: A single ON-OFF transmission, i.e. one high-voltage transmission followed by one low-voltage transmission, where ON and OFF may have same or different durations - Alt 2: A multi-ON-OFF transmission, where different ON and different OFF may have same or different durations and different parts may have same or different duration - Option 2: ON-OFF sequence-based design is considered which consists of a pre-defined sequence for detection of start-indicator part based on digital correlation - For both the options, it is observed that a fixed duration for the start-indicator part can be considered, regardless of the value of M used for PRDCH transmissions.

In this section, we will discuss the design and need of various synchronization signals intended to provide timing, frequency, or sampling corrections. Fig. 1 highlights PRDCH transmission with different segments/parts of synchronization signal, which are discussed below.

Fig. 1 R2D timing acquisition signal (R=TAS)

In RAN1#118bis, FL provided the way forward for next meeting discussion topics.

R2D - Remaining design details for R2D signals including clock-acquisition part and other signals (if any, potentially for frequency synchronization) - Remaining R2D control information for D2R scheduling and/or R2D reception

#### Start delimiter

To avoid any unwanted AIoT tag triggering preamble correlation, a start delimiter is used only to initiate the AIoT tag to get into active state to initiate preamble detection followed by PRDCH reception. Thus, the start delimiter is used only to initiate the AIoT tag, during which the tag has no timing accuracy. Thus, to ensure reliable start delimiter detection, the silence length should be longer with two ON segments to avoid any unwanted glitch getting detected as a start delimiter. Furthermore, the width of two ON pulses should be wide enough to provide sufficient energy to trigger the AIoT tag.

Agreement For the SIP of R-TAS, for providing the start of the R2D transmission, one single design based on Option 1 is supported and further down-selection to be done among Alt 1 and Alt 2: - Option 1: ON-OFF transmission with following alternatives: - Alt 1: A single ON-OFF transmission with pre-defined duration for each of the ON-OFF, where ON and OFF may have same or different durations - Continue discussion to clarify the duration of each of the ON and OFF - Continue discussion to list the different candidate proposals under Alt1 - Alt 2: A multi-ON-OFF transmission with pre-defined duration for each of the ON(s)-OFF(s), where different ON and different OFF may have same or different durations and different parts may have same or different duration - Continue discussion to clarify the duration of each of the ON and OFF - Continue discussion to list the different candidate proposals under Alt2 - Only a single fixed value for entire duration of SIP of R-TAS is supported, which is independent of the value of “M” used in CAP and PRDCH - Note: Specific design and duration for SIP of R-TAS are further discussed, and companies are encouraged to evaluate the designs in terms of target MDR of [10%] for a FAR up to [1%] and at least following assumptions are used: - MDR refers to the probability that SIP is not detected when it was actually transmitted - FAR probability that the receiver incorrectly detects SIP when SIP was not transmitted - Energy/edge detection-based method is the baseline assumption for evaluation purpose - Continue discussion on necessary details for simulation assumptions Agreement For the SIP of R-TAS, down-select among the following candidates: - Alt 1 (Single ON-OFF transmission) - Alt 1-1: ON followed by OFF with same duration for both - Alt 1-2: ON followed by OFF with a duration ratio of 1:[2,3] - Alt 1-3: ON followed by OFF with a duration ratio of [2,3]:1 - Alt 2 (Multi-ON-OFF transmission) - Alt 2-1: A number of repetition instances of Alt 1-1 or Alt 1-2 or Alt 1-3 - Alt 2-2: ON-OFF-ON (duration of ON and OFF can be different) - Alt 2-3: OFF-ON-OFF (duration of ON and OFF can be different) - Alt 2-4: Combination of single instance of Alt 1-1 and single instance of Alt 1-2 - For the evaluation purpose, for both options, candidate values related to duration are considered: - Entire duration of SIP: 1/2 OFDM symbol duration or 1 OFDM symbol duration (including clarifying whether OFDM symbol duration includes CP); additional durations can be considered and reported by companies with justification - Companies to report the exact duration(s) for ON or OFF - Companies are encouraged to report at least the following details for the evaluations: - Baseline assumption is that RF transmission is not present; companies can report other consideration - For FAR calculation, whether noise and/or PRDCH transmission is considered - Details on threshold detection method including whether/how threshold detection training is used based on the proposed design alternative or not - BW assumptions for RF-ED and BB-LPF - Target MDR of up to 1% for FAR of up to [1%, 10%] Agreement For the pattern of SIP of R-TAS, only the following 2 alternatives are considered for further down-selection: - Alt 1-2: ON-OFF with a ratio of 1:3 and with following total SIP duration to be further down-selected: - Option 1: 0.5 OFDM symbol duration - Option 2: 1 OFDM symbol duration - Alt 2-4: ON-OFF-ON-OFF with a ratio of 1:1:1:3 and with following total SIP duration to be further down-selected: - Option 1: 0.5 OFDM symbol duration - Option 2: 1 OFDM symbol duration Agreement For R-TAS, SIP duration of 1 OFDM symbol is adopted with CAP pattern ON-OFF-ON-OFF for all values of M corresponding to PRDCH - Note: device cannot assume the presence/absence of RF transmission prior to the SIP.

The start delimiter signal must be robust enough to be detected even in the presence of interference. As the device is assumed to perform preamble detection only after the start delimiter detection, any failure in detecting start delimiter may cause the complete failure of AIoT transmission destined to the tag. Thus, the start delimiter should be designed by considering both interference and interference-free scenarios. However, using sequence for SIP may not be optimal as it may require correlation to be performed by the AIoT device continuously, which incurs more power. Thus, a simple signal that can ensure the reliable detection of edges and then count the duration of the signal level before the next transition would be ideal.

Fig. 2 SIP signal detection using edge detection/energy level detection.

Thus, to ensure proper detection of start delimiter, having a ON/OFF pattern with high/low transition enables the device to identify the presence of start delimiter even in the presence or absence of interference as long as the interference stays constant for a duration of start delimiter as illustrated in Fig. 2. However, having OFF only pattern for start delimiter may not solve the purpose of reliable detection as the device cannot ensure the constant voltage with variations is due to interference or silence. Therefore, having a constant low voltage may not guarantee proper detection when there is interference. During silence period, having a low voltage as start delimiter will force the device to start preamble detection unnecessarily.

Start delimiter should be long enough to ensure reliable detection due to large sampling offset observed in the AIoT tag before estimating/correcting from preamble.

SIP design should consider level detection followed by edge detection to ensure reliability while spending lower power.

In Fig. 3, we compare the performance of SIP detection using different patterns agreed in the previous RAN1#120 meeting. The objective of detecting SIP is under huge SFO at the device as the AIoT device may not have reliable clock calibration while detecting the SIP. Under this assumption, we consider the SIP detection under SFO of 103-105ppm error. From Fig. 3, we can see that the need for repetition in SIP may not be needed as it may reduce the detection rate due to more pulses to determine before start detecting the preamble for certainty. Thus, the SIP shall be a simple signal with few 1’s and 0’s. On the contrary, having [0,1,0,1] as SIP pattern is not ideal as well as it is very common to make a mistake in the detection procedure.

Fig. 3 SIP detection performance of various patterns under SFO offset in TDL-A 30ns channel.

Fig. 3 is designed with M=1 as the baseline modulation and the performance of repetition is not that different than the one without repetition of the same underlying pattern. Even though there is no difference in the detection performance between Alt-1 and Alt-2, the reliability of detection in the presence of NR signal that spans over a symbol may cause misdetection, which may lead to increased power consumption. Thus, we prefer to use Alt-2 having [0,1,1,0,1,1,0] as the SIP pattern to avoid false alarm due to reference signal transmission that are separated by 4 symbols (TRS).

For SIP of R-TAS, for down-selection between SIP Alt 1-2 and Alt 2-4, companies are encouraged to provide their evaluations considering the assumptions and performance metrics that have been agreed during RAN1#120 and RAN1#120bis

SIP with more than single ON and OFF pattern increases the FAR due to any adjacent NR transmissions, such as LP-WUS or LP-SS, which also uses OOK type of signaling. Thus, to ensure robustness against such NR transmissions, having Alt-2-4 design spanning ONE OFDM symbols is beneficial.

RAN1 shall consider SIP pattern spanning 1 OFDM symbol using Alt-2-4 pattern over Alt-1-2.

#### Clock acquisition signal

Given the nature of the devices to be read and the agreed topologies, there is no need for periodic synchronization signal transmission. Instead, a preamble-based synchronization is the preferred option, where the preamble is attached to the R2D signal. The preamble serves at least two purposes, i.e., that of synchronization and providing some knowledge about the channel condition between the reader and the AIoT device. The following agreement was agreed in the previous RAN1 meeting 118, where the start delimiter is not a part of the preamble but will be used before the transmission.

Agreement For each D2R transmission, no separate part for start-indicator is considered for the preamble preceding the PDRCH. Agreement For D2R transmission, preamble preceding the PDRCH is studied also for the potential additional functionalities: - SFO estimation - CFO estimation - Channel estimation - Interference estimation Note: this does not preclude studying the above functionalities by using a midamble and/or postamble, if supported FFS: Other functionalities, if any Agreement For the clock-acquisition part of the R2D time acquisition signal for OOK chip duration determination, following options are studied: - Option 1: Duration of the clock-acquisition part is variable for different M values, i.e. the duration becomes shorter with increasing value of M - Option 2: Duration of the clock-acquisition part is constant for different M values based on repetition, i.e. repetition factor is increased with increasing value of M to keep the duration constant - FFS: Whether/what restriction on M values for the clock-acquisition part Note: Other functionalities of clock-acquisition part is a separate discussion Agreement For the CAP of R-TAS, the starting chip has a different voltage level compared to the end of the SIP of R-TAS. Agreement For the design of the CAP of R-TAS, at least 2 transition edges in same direction are included, i.e. at least two transitions from “OFF” chip to “ON” chip or two transitions from “ON” chip to “OFF” chip. Agreement For CAP of R-TAS, the following is adopted: - Option 1 for CAP of R-TAS from TR 38.769 is adopted where the CAP duration becomes proportionally shorter with increasing value of M, i.e. if for M = X, duration is Y OFDM symbol long, then for M = aX, duration is Y/a OFDM symbol long - Note: Duration without CP insertion is considered above, with CP insertion, the total duration may not be exactly proportional - Only the following two alternatives for CAP pattern are considered for further down-selection to one alternative: - Alt 1: ON-OFF-ON-OFF - Alt 2: ON-OFF-ON

The preamble is a known sequence that is used by the AIoT tag upon activation to obtain the synchronization by determining the start of a transmission, i.e., identifying the frame boundary. The AIoT tag must perform a sequence correlation with a known preamble sequence and determine the peak to obtain the timing of PRDCH receptions. The use Manchester coding is subjected to the synchronization accuracy and the quality of autocorrelation from the resulting sequence correlation.

Fig. 4 Clock acquisition signal design with or without Manchester encoding

Once the start delimiter is detected and the device is in active mode to detect the preamble or clock acquisition signal, the need for Manchester encoding on preamble sequence is not imperative unlike the payload, which is detected bit by bit.

Unlike RFID, which use a sequence of alternating 1’s and 0’s of length 16, the preamble detection is typically performed as a sequence correlation, it is essential to design the sequence robust against all receiver impairments, such as, frequency and sampling frequency offsets. The timing estimation is the primary target of preamble sequence, and thus the preamble sequence must demonstrate better autocorrelation properties. Thus, to satisfy the above properties, such as, balanced number of ones and zeros to aid AGC tuning and threshold determination together with better autocorrelation properties, “m-sequence” family of sequences can be considered.

Preamble sequences must have better autocorrelation properties to obtain better timing accuracy even with large sampling offset.

Increasing the value of M, while retaining the same transmission duration, improves the auto-/cross- correlation properties of the sequence due to increase in sequence length.

Even though Manchester encoding is not needed for preamble sequence, the sequence design must ensure that equal number of 1’s and 0’s should be available within an OFDM symbol as the generation of preamble from reader (e.g., gNB) uses OOK4 type of generation. Due to DFT based generation, the power within each symbol will be constant, and therefore if there is unbalanced number of 0’s and 1’s within OFDM symbol, few ON duration may carry different power than the remaining due to per symbol power constraint. Fig. 5 demonstrates the FAR of preamble sequence in the presence of noise. Note that for a fixed length of 16, the FAR of M=8 is smaller than M=24, due to the overall length of the signal over which the evaluation is performed.

Having a shorter CAP sequence may trigger higher FAR for higher M values than the one with lower M values.

| (a) SNR 10dB | (b) SNR 20dB |
| --- | --- |

Fig. 5 Detecting alternating 1/0’s from noise (FAR of preamble detection).

Fig. 6 illustrates the preamble detection with different M values. The choice of M up to 24 is possible to ensure reliable detection with different CAP length. However, considering the outcome of both Fig. 5 and Fig. 6, if we check the performance of CAP sequence as a function of length, higher CAP length is beneficial for higher M values to reduce the FAR. Even though it benefits lower M values, it may not be needed to use that.

| (a) SNR 10dB | (b) SNR 20dB |
| --- | --- |

Fig. 6 Preamble detection @SNR levels [10,20]dB

RAN1 to consider designing PRDCH preamble by ensuring uniform distribution of 1’s and 0’s, i.e., by simply repeating 1,0 pattern to the desired length.

RAN1 to consider designing PRDCH preamble using alternate 1’s and 0’s.

To evaluate the impact of preamble length on the timing estimation accuracy, detection performance and FAR, we considered preamble sequence design with L={16,24,32,48} using simple repetition of alternating 1’s and 0’s. The channel used is “TDLA30ns” with 1.92MHz transmission BW and 2.08MHz occupied BW and sampling rate of the AIoT device. The comparison is drawn with sampling offsets {103,104,105} ppm and at SNR of {10,20}dB.

| (a) Residual timing error after CAP detection @10dB SNR | (b) Residual timing error after CAP detection @20dB SNR |
| --- | --- |

Fig. 7 Residual timing estimation under varying SFO, M, and preamble length in TDL-A 30ns.

Fig. 7 illustrates 90 percentile timing estimation accuracy against the preamble length, OOK chip rate M∈{8,12,18,24}, and with sampling offsets of 103ppm and 104ppm, respectively. As the detection is only based on the edge transition and identifying the pulse width from the

On the contrary, when the SFO is high, i.e., ≈100000ppm, the accuracy of estimating timing offset degrades with the increase in the preamble sequence length. This is due to the drift experienced over time, which is quite evident with lower M={1,2} than with higher M values. Thus, the preamble sequence length should be designed by considering both OOK M value and the minimum SFO that must be supported by the sequence design.

| (a) Impact of transmission BW on the pulse shape | (b) Impact of SFO on the pulse width and edges |
| --- | --- |

Fig. 8 Factors impacting CAP detection accuracy.

Use of M≥4 provides better timing estimation accuracy even in the presence of SFO as the sequence length spans only over a shorter duration.

The length of preamble sequence must be decided considering coverage, timing estimation accuracy, and the robustness against SFO.

RAN1 should consider a R2D preamble sequence of length at least 32 to ensure reliable detection and to lower FAR.

The use of preamble to estimate sampling frequency requires multiple preamble correlations and calculating the timing drift at each correlation attempts. However, using a single preamble to determine the sampling offset may not be efficient as the drift observed over a smaller duration cannot provide robust estimate of the sampling offset.

### Midamble

RAN1 also discussed the need for a midamble in R2D signal to ensure that the device remains synchronized with the reader (activator). It is worth noting that the AIoT transmission uses a low data rate and may last only for a shorter period. Therefore, the impact of sampling frequency offset, which is in the order of 10Xppm, may not severely degrade the detectability of AIoT device.

The midamble may additionally help the reader to estimate the tag channel and CW interference (only if the target signal is in each SINR range) and ease the reader in performing the CW interference prior to detection.

As the Manchester encoding is assumed for PRDCH payload, midamble may not be needed as Manchester coding itself is a self-synchronizing sequence.

Lastly, a postamble to end the R2D signal reception, was also discussed by many companies in the previous meeting. The use of postamble may not be needed if the length of each R2D transmission uses a fixed length or the size of the payload is indicated by other means, namely, a control signal or by using a unique preamble sequence. This has been captured by the FL summary in one of the proposals presented [1], which is as follows.

Agreement R2D transmission does not include a midamble.

### Postamble

The postamble can be attached to the PRDCH transmission to improve the reliability of the estimation in case the sampling offset experienced by the device is high. In such cases, the device can utilize the additional synchronization signal that is coming at the end of transmission to estimate the sampling offset/drift experienced between the preamble and the postamble together with the message size to correct the sampling clock. However, if Manchester encoding is used as line code for PRDCH transmission, this becomes redundant and adds only to an overhead with marginal performance improvement.

Using postamble and preamble together with the length of PRDCH transmission, an AIoT device can determine the sampling offset/drift experienced.

On the contrary, if the postamble is not used, the device may not be able to detect the end of PRDCH transmission reliably even though the length of the payload is signaled before the transmission as a control message due to the sampling drift observed by the device. Thus, to provide a reliable form of communication, postamble must be included in each message, that can provide additional reliability for the proper functioning of AIoT scheme.

- For R2D postamble related aspects, following FL proposal and corresponding potential specification impact, if any, can be considered for further discussions and decision on whether R2D postamble is specified or not. And if it is agreed to be specified, preferred patterns with justifications are encouraged Proposal 4-1d The Manchester coding rule violation corresponding to M value for PRDCH is applied by reader’s implementation that can be used by the device to determine the end of PRDCH transmission There is no consensus to specify R2D postamble

Detection of violating Manchester coding rule based on reader’s implementation may not be a better solution as the AIoT device can use the Manchester encoding failure to readjust the clock recovery/sampling or to ensure detection error.

RAN1 shall not consider detection of violating Manchester coding rule based on reader’s implementation as the reader is controlling both transmission size and power level.

- It would be the better design for the device to be able to know the end of the codeword based on transmission payload indications. The detection of violating Manchester coding rule based on reader’s implementation may lead to ambiguous device behavior.

## D2R Transmission

### Preamble

Agreement For each D2R transmission, no separate part for start-indicator is considered for the preamble preceding the PDRCH. Agreement For D2R transmission, preamble preceding the PDRCH is studied also for the potential additional functionalities: - SFO estimation - CFO estimation - Channel estimation - Interference estimation Note: this does not preclude studying the above functionalities by using a midamble and/or postamble, if supported FFS: Other functionalities, if any Agreement - For D2R preamble design, the functionalities of timing acquisition, SFO estimation/time tracking and channel estimation should be supported - For D2R midamble design, the functionalities of SFO estimation/time tracking and channel estimation should be supported - D2R midamble can be transmitted at the end of the PDRCH transmission. If it is at the end, it is not designed for being used for indicating the end of PDRCH transmission - FFS: condition(s) and/or indication where the D2R midamble is present or not Agreement - For D2R preamble/midamble, base sequence is generated from m-sequence, where the length of the sequence is 2n-1 - Value(s) of n - Long preamble/midamble is generated based on n = 5 - Working assumption: Short preamble/midamble is generated based on n=3 - Only 1-part preamble/midamble are supported for D2R - Preamble immediately precedes the PDRCH without any gap - Both long and short preamble and midamble are supported based on the working assumption on n - when midamble is present at least the following cases are supported and reader explicitly indicates one of the following cases for PDRCH: - Short preamble and short midamble - Long preamble and long midamble Note: the case of short preamble and long midamble will not be supported - When midamble is not present the reader explicitly indicates short or long preamble for PDRCH

The D2R signal transmission between the AIoT device to the reader contains preamble without the start delimiter as the reader can detect the preamble sequence once the R2D transmission ends. This was agreed in previous RAN1 meeting. Thus, following our earlier discussions on the preamble length and the sequence type, it can reuse the same content provide for R2D, i.e., preamble sequence can be based on “m-sequence” of length [16,32] without Manchester encoding to obtain better autocorrelation peak, which helps in determining the timing estimation.

| (a) Initial SFO of 1000ppm | (b) Initial SFO of 10000 ppm |
| --- | --- |

Fig. 9 Residual timing estimation error using PDRCH preamble of different lengths.

Fig. 9 demonstrates the residual timing estimation observed at the reader upon receiving the PDRCH preamble of lengths {8,16,24,32,48,64} bits. In this simulation, a single carrier tone is generated and modulated using ‘m-sequence’ with OOK scheme. At the reader side, the DSB signal is filtered and detected using simple correlator with multiple hypothesis for SFO error. In this evaluation, we assumed 51 hypothesis that are spaced around 400ppm interval to span the SFO offset of ±104ppm error. To determine the timing hypothesis, we used ±20μsec uncertainty interval to determine the timing offset of actual preamble correlation peak. Fig. 9 illustrates the performance of PDRCH preamble detection accuracy in estimating the timing and the plots demonstrate the residual estimation error after correcting the timing from preamble.

For D2R preamble/midamble, companies are encouraged to provide their preferred sequence for length corresponding to n = 5 and n =3 and potentially including evaluations considering the assumptions and performance metrics that have been agreed during RAN1#120 - Also, companies are encouraged to provide the midamble interval values accordingly Corresponding signaling details related to D2R preamble/midamble will also be handled under this agenda

The choice of sequences can be derived from the primitive polynomials with initial seed that can be configured based on the transaction ID to identify and separate the transmissions from the reader perspective. Furthermore, this provides additional checks at the reader to ensure both AIoT device and the reader are well aligned with the respective transmissions as the reader may handle multiple parallel D2R transmissions at the same time by multiplexing them in the frequency domain. The choice of m-sequence polynomial for 3 and 5 can be derived from the basic form, i.e., D3+D1+1 and 	D5+D3+1, respectively. Alternatively, we can choose D3+D2+1 and D5+D2+1 for polynomial with root 3 and 5 respectively.

The m-sequence for root n=3 can use either D3+D1+1 or D3+D2+1 with a certain initialization sequence that can be configured by the reader.

The m-sequence for root n=5 can use either D5+D3+1 or D5+D2+1 with a certain initialization sequence that can be configured by the reader.

### Midamble

Agreement For D2R midamble, for determining the presence and location of midamble(s) at the device: - Reader explicitly indicates the same interval between consecutive midambles, and between the preamble and the first midamble, via R2D control information - FFS: details of signalling - FFS: whether the reader can explicitly indicate with one bit whether a midamble is additionally present at the end - Note: This does not preclude the support of having no midamble present in the D2R transmission Agreement For indicating the interval between consecutive midambles, and between the preamble and the first midamble, via R2D control information, following is adopted: - Unit of interval is number of bits after FEC (if FEC is applied) and repetition (if repetition is applied) - FFS: the candidate values in terms of the unit of interval

As discussed in our companion paper, the need for midamble is subjected to the type of modulation used for D2R transmissions. As the waveform design is based on MC encoded binary sequence, the need for midamble is not demanding as the modulated data sequence itself provides synchronization to a certain extent. On the contrary, if BPSK/OOK based transmission is used with simple square wave modulation, then the need for midamble assists the reader in detecting the D2R transmission with improved reliability. Furthermore, the reader can utilize the midamble that are inserted within the D2R transmission at regular intervals to correct the timing drift without compensating on the sampling drift by simple correlation technique.

The need for midamble depends on the type of modulation used for D2R transmission. If the D2R transmission uses self-synchronizing modulation schemes like FM0, Miller, or MC, then the need for midamble is not essential.

The use of midamble in the D2R transmission may assist the reader to perform resynchronization using a specific pattern and to adjust the sampling rate accordingly.

The size and the interval of midamble should be considered both from the overhead and the detection performance.

Assuming the SFO experienced by the AIoT device is ≈103ppm, the number of bits that can elapse after the initial preamble detection by the reader determines the detection reliability. If the reader adjusts the timing after preamble detection, the AIoT device transmission will experience the drift that can vary up to 103ppm as agreed in the WID. To ensure reliable detection of OOK signal by the reader, irrespective of the bit duration, 96 bits with Manchester encoding like alternate ON/OFF transitions will experience 20% loss in the chip energy after 96 bits and 25% loss in chip energy after 128 bits.

The choice of OOK chip duration does not impact the midamble insertion if the number of bits is restricted to 96 or 128bits for 20% or 25% loss in chip level synchronization, respectively.

The midamble of length 7 shall be inserted for every 96/128 bits of payload.

RAN1 agreed to support D2R midamble from the above agreement. The reader explicitly indicates the interval between the preamble and the first midamble, and it also indicates the interval between midamble. The midamble indication may be optional, so if the reader does not indicate it, the default setting would be no presence of midamble. One FFS is whether the reader can explicitly indicate with one bit whether a midamble is additionally present at the end. In our understanding, it is equivalent to the indication on the presence of the postamble. The simple approach would be to indicate the number of midamble and their interval.

The base sequences of preamble/midamble is generated from m-sequence. The same or different sequences can be used across preamble and midamble based on indications. We could consider that the network provides a set of sequences for preamble/midamble so that the device can select specific sequences for preamble/midamble to inform a certain control information.

### Postamble

Unlike PRDCH, PDRCH is designed to be detected by the reader with higher processing capability. Furthermore, due to the use of FEC by the device in D2R transmission, the size of the information bits and the length of the transmission are to be known at both encoder and decoder. Thus, in this case, the reader provides the information about the information size and the length of transmission in bits, which fixes the code rate of FEC. Thus, the device cannot autonomously send a postamble to terminate the transmission until the whole codeword is transmitted.

As FEC is adopted in D2R transmission, device cannot autonomously terminate the transmission by sending postamble for early termination, since the coding rate is fixed by the size of the information bits and the codeword length, both are signaled in the previous PRDCH control.

The postamble may improve detection if it is used as stop bits to ensure reception of all payload bits in PDRCH before FEC decoding when there is larger SFO at the AIoT device.

The use of postamble for PDRCH transmission shall be signaled by the reader depending on the payload size and the SFO accuracy of the AIoT device.

# Conclusion

In this contribution we have made the following observations and proposals:

## List of Observations

Observation 1:	Start delimiter should be long enough to ensure reliable detection due to large sampling offset observed in the AIoT tag before estimating/correcting from preamble.

Observation 2:	SIP design should consider level detection followed by edge detection to ensure reliability while spending lower power.

Observation 3:	SIP with more than single ON and OFF pattern increases the FAR due to any adjacent NR transmissions, such as LP-WUS or LP-SS, which also uses OOK type of signaling. Thus, to ensure robustness against such NR transmissions, having Alt-2-4 design spanning ONE OFDM symbols is beneficial.

Observation 4:	Preamble sequences must have better autocorrelation properties to obtain better timing accuracy even with large sampling offset.

Observation 5:	Increasing the value of M, while retaining the same transmission duration, improves the auto-/cross- correlation properties of the sequence due to increase in sequence length.

Observation 6:	Having a shorter CAP sequence may trigger higher FAR for higher M values than the one with lower M values.

Observation 7:	RAN1 to consider designing PRDCH preamble by ensuring uniform distribution of 1’s and 0’s, i.e., by simply repeating 1,0 pattern to the desired length.

Observation 8:	Use of M≥4 provides better timing estimation accuracy even in the presence of SFO as the sequence length spans only over a shorter duration.

Observation 9:	The length of preamble sequence must be decided considering coverage, timing estimation accuracy, and the robustness against SFO.

Observation 10:	As the Manchester encoding is assumed for PRDCH payload, midamble may not be needed as Manchester coding itself is a self-synchronizing sequence.

Observation 11:	Using postamble and preamble together with the length of PRDCH transmission, an AIoT device can determine the sampling offset/drift experienced.

Observation 12:	Detection of violating Manchester coding rule based on reader’s implementation may not be a better solution as the AIoT device can use the Manchester encoding failure to readjust the clock recovery/sampling or to ensure detection error.

Observation 13:	The need for midamble depends on the type of modulation used for D2R transmission. If the D2R transmission uses self-synchronizing modulation schemes like FM0, Miller, or MC, then the need for midamble is not essential.

Observation 14:	The use of midamble in the D2R transmission may assist the reader to perform resynchronization using a specific pattern and to adjust the sampling rate accordingly.

Observation 15:	The size and the interval of midamble should be considered both from the overhead and the detection performance.

Observation 16:	The choice of OOK chip duration does not impact the midamble insertion if the number of bits is restricted to 96 or 128bits for 20% or 25% loss in chip level synchronization, respectively.

Observation 17:	As FEC is adopted in D2R transmission, device cannot autonomously terminate the transmission by sending postamble for early termination, since the coding rate is fixed by the size of the information bits and the codeword length, both are signaled in the previous PRDCH control.

Observation 18:	The postamble may improve detection if it is used as stop bits to ensure reception of all payload bits in PDRCH before FEC decoding when there is larger SFO at the AIoT device.

## List of Proposals

Proposal 1:	RAN1 shall consider SIP pattern spanning 1 OFDM symbol using Alt-2-4 pattern over Alt-1-2.

Proposal 2:	RAN1 to consider designing PRDCH preamble using alternate 1’s and 0’s.

Proposal 3:	RAN1 should consider a R2D preamble sequence of length at least 32 to ensure reliable detection and to lower FAR.

Proposal 4:	RAN1 shall not consider detection of violating Manchester coding rule based on reader’s implementation as the reader is controlling both transmission size and power level.

It would be the better design for the device to be able to know the end of the codeword based on transmission payload indications. The detection of violating Manchester coding rule based on reader’s implementation may lead to ambiguous device behavior.

Proposal 5:	The m-sequence for root n=3 can use either D3+D1+1 or D3+D2+1 with a certain initialization sequence that can be configured by the reader.

Proposal 6:	The m-sequence for root n=5 can use either D5+D3+1 or D5+D2+1 with a certain initialization sequence that can be configured by the reader.

Proposal 7:	The midamble of length 7 shall be inserted for every 96/128 bits of payload.

Proposal 8:	The use of postamble for PDRCH transmission shall be signaled by the reader depending on the payload size and the SFO accuracy of the AIoT device.

# References

- RP-250796, ” New WID on Rel-19 Ambient IoT”, RAN#107

- TR 38.769, “Study on solutions for Ambient IoT (Internet of Things) in NR”