3GPP TSG RAN WG1 #121			R1-2503333

St Julian’s, Malta, May 19th – 23th, 2025

Source:	ZTE

Title:	Discussion on UE features for Rel-19 SBFD

Agenda item:	9.15.3

Document for:	Discussion and Decision

Introduction

Based on the progress in RAN1#120bis [1][2], we provide our views on UE features for Rel-19 SBFD including basic transmission/reception behaviour procedures, RACH in SBFD symbols and L1 UE-UE CLI handling.

UE features for transmission/reception behaviour and procedures

Basic UE feature for SBFD operation

In RAN1#120bis, the following agreements were reached on the basic UE feature for SBFD.

| 60-1 | Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures | 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one[/two] DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling 4. Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1) 5. Support of separate CSI reports for SBFD and non-SBFD symbols 6. Support of separate PUCCH frequency resource configuration in SBFD symbols and non-SBFD symbols [FFS: 7. Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols] 8. Support of separate PUSCH frequency hopping offset in SBFD symbols and non-SBFD symbols [FFS: 9. Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols] FFS: Whether to include two DL subband support in component 2 FFS: other component (which may be separate FG) |  | YES | n/a | Basic feature for SBFD-aware UE TX/RX/ measurement behaviour and procedures is not supported | Per band | TDD only | n/a | n/a | [Note: Only UL transmissions are allowed within UL subband and only DL receptions (except for CLI measurement) are allowed within DL subband(s)] [Note: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one.] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For the basic FG, our views are summarized as below.

- Add the following components in the basic FG.

  - Support of DL reception in two non-contiguous DL subbands. Most of the procedure for PDSCH reception in two non-contiguous DL subbands are not changed compared to legacy, .e.g, VRB-to-PRB mapping and DMRS sequence etc. The main change is TBS determination, which in our view is a basic change for SBFD operation and does not require one additional FG.

  - Support of separate UL configurations for SBFD and non-SBFD symbols, including component 7 and 9.

  - Unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG

  - Support of CSI-RS reception in two non-contiguous DL subbands as part of basic FG for SBFD operation. Supported CSI processing timeline for CSI-RS reception in two non-contiguous DL subbands should be reported as component of the basic FG for SBFD.

    - Two candidate values may be reported: {CSI processing timeline and CPU counting are the same as legacy, scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy}.

  - Supported maximum number of DL partial PRGs for PDSCH in SBFD symbols should be reported as part of basic FG for SBFD operation

    - Candidate values for supported maximum number of DL partial PRGs is {2, 4}. If two DL subbands are configured and value 2 is reported by the UE, a PDSCH transmission is valid as long as it only contains up to 2 partial PRGs, no matter the partial PRG are caused by the boundary of BWP as legacy or it is caused by the boundary of DL subband. Otherwise, a PDSCH transmission is valid no matter which and how many partial PRGs the PDSCH contains.

  - For PUSCH repetition type B, a nominal repetition is segmented into actual repetitions around boundary of SBFD symbols and non-SBFD symbols.

- No need of FR1/FR2 differentiation since per band reporting is supported.

- To keep the two bullets in the note column.

With above, we propose to revise the basic FG per following proposal.

Proposal 1: Support the following revision on the basic FG for SBFD operation.

| 60-1 | Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures | 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one/two DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling 4. Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1) 5. Support of separate CSI reports for SBFD and non-SBFD symbols 6. Support of separate PUCCH frequency resource configuration in SBFD symbols and non-SBFD symbols 7. Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols 8. Support of separate PUSCH frequency hopping offset in SBFD symbols and non-SBFD symbols 9. Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols 10. Unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG 11. Support of CSI-RS reception in two non-contiguous DL subbands as part of basic FG for SBFD operation 12. Supported maximum number of DL partial PRGs for PDSCH in SBFD symbols 13. For PUSCH repetition type B, a nominal repetition is segmented into actual repetitions around boundary of SBFD symbols and non-SBFD symbols. |  | YES | n/a | Basic feature for SBFD-aware UE TX/RX/ measurement behaviour and procedures is not supported | Per band | TDD only | n/a | n/a | For component 11, candidate values: {CSI processing timeline and CPU counting are the same as legacy, scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy} For component 12, candidate values: {2, 4} Note: Only UL transmissions are allowed within UL subband and only DL receptions (except for CLI measurement) are allowed within DL subband(s)] [Note: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Configuration 2 for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots.

For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots, it was agreed that Configuration 1 is the default capability and support of Configuration 2 is subject to UE capability. So, support of Configuration 1 is included in the basic FG 60-1 as shown above. A separate UE capability is needed for Configuration 2.

Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, - Support of configuration 2 is subject to UE capability. Configuration 1 is the default capability. - SBFD-aware UE can be configured with Configuration 2 on a per UL/DL BWP basis. - The configuration for DL BWP at least applies to PDSCH receptions within the DL BWP. - The configuration for UL BWP applies to PUCCH and PUSCH transmissions within the UL BWP. - For SRS, only Configuration 1 is applicable.

In RAN1#120bis, FG 60-2 was agreed at least for DL reception and it is still pending whether to have separate FG for UL transmission. In our view, the main difference between transmission and reception is that different RRC configurations are applied for some aspects between SBFD and non-SBFD symbols. However, these aspects have already been included in the basic FG. In this context, we have the following proposal.

Proposal 2: Support the following revision on FG 60-2.

| 60-2 | Transmission and reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) | 1. Support of PDSCH reception across SBFD symbols and non-SBFD symbols (Configuration 2) 2. Support of UL transmission across SBFD symbols and non-SBFD symbols (Configuration 2) | FG 60-1 | YES | n/a | Transmission and reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) are not supported | per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

UE features for RACH procedure

In RAN1#120bis, the following agreements were reached on SBFD random access operation.

| 60-3 | SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode | Support of SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 1) FFS: Other component(s) which may be separate FG | 60-1 | YES | n/a | SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60-4 | SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode | Support of SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 2) FFS: Other component(s) which may be separate FG | [60-1, 60-3] | YES | n/a | SBFD random access operation based on two separate RACH configurations in RRC_IDLE/INACTIVE/CONNECTED mode is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |

For FG 60-3 and 60-4, our views are summarized as below.

- Regarding whether to add additional components, down-select from the following two alternatives.

  - Alt 1:

    - Add the following components in FG 60-3.

      - Support of reinterpretation of frequency offset of lowest RO

      - Support of separate power control parameters in additional-RO

      - Support of separate mapping of SSB and additional ROs, and separate determination of the association period and association pattern period.

    - Add the following components in FG 60-4.

      - Support of valid RO across SBFD and non-SBFD symbols

  - Alt 2: Keep FG 60-3 and FG 60-4 concise as it is.  Neither add any additional components nor any additional separate FGs.

    - Note: If legacy FGs, e.g., Msg1 repetition, is reported together with FG 60-3/60-4, the conjunction of Msg1 repetition and SBFD is supported.

- FG 60-3 is the prerequisite FG of FG 60-4.

With above, we have the following proposal.

Proposal 3: For FG 60-3 and FG 60-4, FG 60-3 is the prerequisite FG of FG 60-4, and down-select from the following two alternatives.

- Regarding whether to add additional components, down-select from the following two alternatives.

  - Alt 1:

    - Add the following components in FG 60-3.

      - Support of reinterpretation of frequency offset of lowest RO

      - Support of separate power control parameters in additional-RO

      - Support of separate mapping of SSB and additional ROs, and separate determination of the association period and association pattern period.

    - Add the following components in FG 60-4.

      - Support of valid RO across SBFD and non-SBFD symbols

  - Alt 2: Keep FG 60-3 and FG 60-4 concise as it is.  Neither add any additional components nor any additional separate FGs.

    - Note: If legacy FGs, e.g., Msg1 repetition, is reported together with FG 60-3/60-4, the conjunction of Msg1 repetition and SBFD is supported.

UE features for L1 UE-UE CLI handling

UL resource muting

In RAN1#120bis, the following FGs were agreed for UL resource muting.

| 60-7 | UL resource muting for CP-OFDM waveform | 1. Support semi-static configuration of time location and frequency location of UL resource muting for CP-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol [- FFS: Whether this FG only support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH] 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH FFS: other potential component/FG/additional restriction | [FFS] | YES | n/a | UL resource muting for CP-OFDM waveform is not supported | [Per band] | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60-7a | UL resource muting for DFTS-OFDM waveform | 1. Support semi-static configuration of time location and frequency location of UL resource muting for DFTS-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol [- FFS: Whether this FG only support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH] 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH FFS: other potential component/FG/additional restriction | [FFS] | YES | n/a | UL resource muting for DFTS-OFDM waveform is not supported | [Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |

Regarding the yellow highlighted pending parts, our views are summarized as below.

- Support different UL muting patterns between DG/Type 2 CG PUSCH and Type 1 CG PUSCH

- Prefer NOT to add FG 60-1 as the prerequisite

- OK with per band reporting

Proposal 4: For UL resource muting, support the following for FG 60-7 and 60-7a.

- Support different UL muting patterns between DG/Type 2 CG PUSCH and Type 1 CG PUSCH

- NOT to add FG 60-1 as the prerequisite

- Reporting granularity: per band

L1-based CLI measurement and reporting

In RAN1#120bis, the following FGs were agreed for L1-based UE-UE CLI handling.

| 60-8 | L1 CLI-RSSI measurement and [aperiodic] reporting | 1. Aperiodic L1 CLI-RSSI reporting on PUSCH [and periodic L1 CLI-RSSI reporting on PUCCH] 2. Support of CLI-RSSI measurement resource configured in one UL subband only, in one DL subband only, or across two DL subbands only. 3. [Periodic and] aperiodic CLI-RSSI measurement resource FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG | FFS | YES | n/a | [Aperiodic] L1 CLI-RSSI measurement and reporting is not supported | [Per band] | TDD only | [n/a] | [n/a] |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60-9 | L1 SRS-RSRP measurement and aperiodic reporting | 1. Aperiodic L1 SRS-RSRP reporting on PUSCH, and [periodic and] aperiodic SRS-RSRP measurement resource FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG | FFS | YES | n/a | [Aperiodic] L1 SRS-RSRP measurement and reporting is not supported | [Per band] | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60-9a | FDM-ed reception of DL signals/channels and L1 SRS-RSRP measurement resource | 1. Support of FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource | 60-9 | YES | n/a | FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource is not supported | [Per band] | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60-9b | Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting | 1. Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting | 60-9 | YES | n/a | Semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting is not supported | [Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |

Regarding the yellow highlighted pending parts, our views are summarized as below.

- Prefer to include periodic and aperiodic measurement and reporting in one FG

- Similar to FG 60-9b, we are ok to add one separate FG for semi-persistent L1 CLI-RSSI measurement resource and aperiodic reporting

- Prefer NOT to add FG 60-1 as the prerequisite for FG 60-8 and 60-9

- OK with per band reporting

Proposal 5: Support the following revisions on L1-based UE-UE measurement and reporting related FGs.

| 60-8 | L1 CLI-RSSI measurement and aperiodic reporting | 1. Aperiodic L1 CLI-RSSI reporting on PUSCH and periodic L1 CLI-RSSI reporting on PUCCH 2. Support of CLI-RSSI measurement resource configured in one UL subband only, in one DL subband only, or across two DL subbands only. 3. Periodic and aperiodic CLI-RSSI measurement resource |  | YES | n/a | Aperiodic L1 CLI-RSSI measurement and reporting is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60-8a | Support of semi-persistent L1 CLI-RSSI measurement resource and aperiodic reporting | 1. Support of semi-persistent L1 CLI-RSSI measurement resource and aperiodic reporting | 60-8 | YES | n/a | Semi-persistent L1 S CLI-RSSI measurement resource and aperiodic reporting is not supported | Per Band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60-9 | L1 SRS-RSRP measurement and aperiodic reporting | 1. Aperiodic L1 SRS-RSRP reporting on PUSCH, and periodic and aperiodic SRS-RSRP measurement resource |  | YES | n/a | Aperiodic L1 SRS-RSRP measurement and reporting is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60-9a | FDM-ed reception of DL signals/channels and L1 SRS-RSRP measurement resource | 1. Support of FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource | 60-9 | YES | n/a | FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60-9b | Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting | 1. Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting | 60-9 | YES | n/a | Semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting is not supported | Per Band | TDD only | n/a | n/a |  | Optional with capability signalling |

Conclusion

In this section, our proposals on UE features for SBFD operation in Rel-19 are summarized.

Proposal 1: Support the following revision on the basic FG for SBFD operation.

| 60-1 | Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures | 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one/two DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling 4. Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1) 5. Support of separate CSI reports for SBFD and non-SBFD symbols 6. Support of separate PUCCH frequency resource configuration in SBFD symbols and non-SBFD symbols 7. Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols 8. Support of separate PUSCH frequency hopping offset in SBFD symbols and non-SBFD symbols 9. Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols 10. Unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG 11. Support of CSI-RS reception in two non-contiguous DL subbands as part of basic FG for SBFD operation 12. Supported maximum number of DL partial PRGs for PDSCH in SBFD symbols 13. For PUSCH repetition type B, a nominal repetition is segmented into actual repetitions around boundary of SBFD symbols and non-SBFD symbols. |  | YES | n/a | Basic feature for SBFD-aware UE TX/RX/ measurement behaviour and procedures is not supported | Per band | TDD only | n/a | n/a | For component 11, candidate values: {CSI processing timeline and CPU counting are the same as legacy, scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy} For component 12, candidate values: {2, 4} Note: Only UL transmissions are allowed within UL subband and only DL receptions (except for CLI measurement) are allowed within DL subband(s)] [Note: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 2: Support the following revision on FG 60-2.

| 60-2 | Transmission and reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) | 1. Support of PDSCH reception across SBFD symbols and non-SBFD symbols (Configuration 2) 2. Support of UL transmission across SBFD symbols and non-SBFD symbols (Configuration 2) | FG 60-1 | YES | n/a | Transmission and reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) are not supported | per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 3: For FG 60-3 and FG 60-4, FG 60-3 is the prerequisite FG of FG 60-4, and down-select from the following two alternatives.

- Regarding whether to add additional components, down-select from the following two alternatives.

  - Alt 1:

    - Add the following components in FG 60-3.

      - Support of reinterpretation of frequency offset of lowest RO

      - Support of separate power control parameters in additional-RO

      - Support of separate mapping of SSB and additional ROs, and separate determination of the association period and association pattern period.

    - Add the following components in FG 60-4.

      - Support of valid RO across SBFD and non-SBFD symbols

  - Alt 2: Keep FG 60-3 and FG 60-4 concise as it is.  Neither add any additional components nor any additional separate FGs.

    - Note: If legacy FGs, e.g., Msg1 repetition, is reported together with FG 60-3/60-4, the conjunction of Msg1 repetition and SBFD is supported.

Proposal 4: For UL resource muting, support the following for FG 60-7 and 60-7a.

- Support different UL muting patterns between DG/Type 2 CG PUSCH and Type 1 CG PUSCH

- NOT to add FG 60-1 as the prerequisite

- Reporting granularity: per band

Proposal 5: Support the following revisions on L1-based UE-UE measurement and reporting related FGs.

| 60-8 | L1 CLI-RSSI measurement and aperiodic reporting | 1. Aperiodic L1 CLI-RSSI reporting on PUSCH and periodic L1 CLI-RSSI reporting on PUCCH 2. Support of CLI-RSSI measurement resource configured in one UL subband only, in one DL subband only, or across two DL subbands only. 3. Periodic and aperiodic CLI-RSSI measurement resource |  | YES | n/a | Aperiodic L1 CLI-RSSI measurement and reporting is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60-8a | Support of semi-persistent L1 CLI-RSSI measurement resource and aperiodic reporting | 1. Support of semi-persistent L1 CLI-RSSI measurement resource and aperiodic reporting | 60-8 | YES | n/a | Semi-persistent L1 S CLI-RSSI measurement resource and aperiodic reporting is not supported | Per Band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60-9 | L1 SRS-RSRP measurement and aperiodic reporting | 1. Aperiodic L1 SRS-RSRP reporting on PUSCH, and periodic and aperiodic SRS-RSRP measurement resource |  | YES | n/a | Aperiodic L1 SRS-RSRP measurement and reporting is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60-9a | FDM-ed reception of DL signals/channels and L1 SRS-RSRP measurement resource | 1. Support of FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource | 60-9 | YES | n/a | FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60-9b | Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting | 1. Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting | 60-9 | YES | n/a | Semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting is not supported | Per Band | TDD only | n/a | n/a |  | Optional with capability signalling |

Reference

[1] RAN1#120bis, R1-2502971, Summary of discussion on Rel-19 NR UE features for SBFD, Moderator (NTT DOCOMO, INC.)

[2] RAN1#120bis, R1-2502977, Updated RAN1 UE features list for Rel-19 NR after RAN1 #120bis, Moderators (AT&T, NTT DOCOMO, INC.)