3GPP TSG RAN WG1 #121                                                                       R1-2503365

St Julian’s, Malta, May 19th – 23th, 2025

Source: 	vivo

Title:	Remaining issues on on-demand SSB Scell operation

Agenda Item:	9.5.1

Document for:	Discussion and Decision

- Introduction

In RAN#102, it has been approved in the Rel-19 Network Energy Saving WID [1] to specify the procedures and signaling method(s) to support on-demand SSB SCell operation for UEs.

- Specify procedures and signalling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signalling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In this contribution, we provide our views on the remaining issues about on-demand SSB SCell operation, as well as some possible solutions.

- Limitation of on-demand SSB

Agreement For a cell supporting on-demand SSB SCell operation, at least the following is supported - On-demand SSB on the cell is not located on synchronization raster. - On-demand SSB on the cell is non-cell-defining SSB FFS: Additional support of OD-SSB for CD-SSB located on sync-raster Agreement For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), study at least the following Mux-Cases. - Mux-Case #1: No time-domain overlap between always-on SSB and on-demand SSB - Mux-Case #2: Always-on SSB and on-demand SSB overlap at least in time or frequency domain Agreement Down-select at least one of the following alternatives. - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - Alt 2: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB - Alt 3: Do not support the case where always-on SSB is CD-SSB on a synchronization raster. Down-select at least one of the following alternatives. - Alt A: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB can be same or different from the frequency location of always-on SSB, subject to its configuration. - Alt B: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB - Alt C: Do not support the case where always-on SSB is CD-SSB and not on a synchronization raster. Agreement Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability - Note: UE is not required to measure both AO-SSB and OD-SSB Agreement Regarding the relation in terms of time location between the always-on SSB and on-demand SSB, - For the case when the center frequency locations of always-on SSB and on-demand SSB are same, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - From RAN1 perspective, - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification). - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported. - It is up to RAN4 to define requirements, if any, corresponding to both or either of Alt Time-C1 or Alt Time-C2 - At least the following is supported: PBCH payload for the same SSB index (other than SFN index, half frame index) is the same for AO-SSB and OD-SSB - FFS: Whether half frame index is the same or different for AO-SSB and OD-SSB - For the case when the center frequency locations of always-on SSB and on-demand SSB are different, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - UE assumes that frequency resources of always-on SSB are not overlapped with those of on-demand SSB in frequency domain. - AO-SSB and OD-SSB are located in the same BWP - FFS: PBCH payload for the same SSB index (other than SFN index, half frame index) should be the same for AO-SSB and OD-SSB - NOTE: AO-SSB periodicity is not adapted Send an LS to RAN4 to inform them of the above agreement. Final LS in R1-2501633. Agreement For a cell supporting on-demand SSB SCell operation, the following combinations of scenarios and cases are supported for indicating OD-SSB using a MAC-CE. - Scenario #3B and Case #1 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication. - Scenario #3B and Case #2 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

In the last meeting, it has been agreed that MAC CE could be used for updating the transmission parameter of a transmitted OD-SSB for the cell when SCell is activated. Then, which transmission parameter could be updated should be decided. The motivation of updating the transmission parameter of a transmitted OD-SSB mainly lies in adapting the periodicity to a longer periodicity after SCell activation is completed. Because the transmission of OD-SSB is no more needed to be as frequent as that in SCell activation procedure. Otherwise, the power consumption of the network would be wasted. There is no motivation to update other transmission parameters of a transmitted OD-SSB. Otherwise, the UE complexity would be increased.

Proposal 1: When MAC CE is used for updating the transmission parameter of a transmitted OD-SSB for the cell, only the parameter of periodicity could be updated.

When the periodicity is updated by MAC CE, whether there is a limitation that SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity should be discussed. In our opinion, the limitation could decrease the UE complexity after the periodicity update. Because UE does not need to search the SSB again, the measurement behaviour could also be simplified.

Proposal 2: For updating the transmission parameter of a transmitted OD-SSB for the cell, SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity.

Moreover, in addition to updating the transmission periodicity in Scenario #3B, it should also be allowed to update the periodicity in Scenario #2 and Scenario #2A. Because the transmission periodicity could be different between different scenarios and different stages in one scenario.

Proposal 3: MAC CE could be used for updating the transmission parameter of a transmitted OD-SSB in Scenario #2 and Scenario #2A.

There is a remaining issue whether to support the case that always-on SSB is CD-SSB and not on a synchronization raster. This issue has an impact on the support of OD-SSB for CD-SSB located on sync-raster. In our opinion, there is no clear benefit to support this case since always-on SSB could be CD-SSB and on a synchronization raster. Therefore, we prefer Alt C, i.e. do not support the case where always-on SSB is CD-SSB and not on a synchronization raster.

Proposal 4: Do not support the case where always-on SSB is CD-SSB and not on a synchronization raster.

When always-on SSB operates as CD-SSB on a synchronization raster, there was an agreement reached in the previous meeting that both always-on SSB and on-demand SSB must be located within the same BWP.  However, the size of a BWP can vary and reach up to 100 MHz, then the frequency gap between always-on SSB and on-demand SSB could still be large. Then UE may still need to perform RF retuning or to search SSB with a large RF range to avoid RF retuning, which would cause the same BWP restriction meaningless. Therefore, the frequency gap between always-on SSB and on-demand SSB should be further limited. Given that up to four BWPs can be configured in SCell, it’s preferred that always-on SSB and on-demand SSB are located in the same BWP with the smallest bandwidth. In addition, the center frequency gap between always-on SSB and on-demand SSB should be smaller than a pre-defined threshold.

Proposal 5: Always-on SSB and on-demand SSB are located in the same BWP with the smallest bandwidth, and the center frequency gap between always-on SSB and on-demand SSB are smaller than a pre-defined threshold.

When always-on SSB operates as CD-SSB, kSSB represents the subcarrier offset from subcarrier 0 in common resource block to subcarrier 0 of the SSB. However, as on-demand SSB couldn’t be CD-SSB,  kSSB may indicate the nearest GSCN of another SSB having an associated Type0-PDCCH CSS set, or may indicate there is no SSB having an associated Type0-PDCCH CSS set within a GSCN range. When indicating the nearest GSCN, the field pdcch-ConfigSIB1 would be used to indicate the frequency positions. Hence, other than SFN index and half frame index, the PBCH payload related to kSSB and pdcch-ConfigSIB1 could also be different between always-on SSB and on-demand SSB.

Proposal 6: For the case when the center frequency locations of always-on SSB and on-demand SSB are different, other than SFN index, half frame index, kSSB and pdcch-ConfigSIB1 could also be different between always-on SSB and on-demand SSB in PBCH payload for the same SSB index.

For the case when the center frequency locations of always-on SSB and on-demand SSB are same, a remaining issue is whether half frame index is the same or different for AO-SSB and OD-SSB.  It is noticed that no restriction about the time domain overlapping between two types of SSB has been agreed. Moreover, the Alt Time-C2 supports that the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern. Therefore, the half frame location could be different for AO-SSB and OD-SSB in Alt Time-C2. However, in Alt Time-C1, to support a union of periodic time domain pattern, the half frame index of AO-SSB and OD-SSB should be same.

Proposal 7：For the case when the center frequency locations of always-on SSB and on-demand SSB are same，the half frame index of AO-SSB and OD-SSB should be same in Alt Time C1 and could be different in Alt Time-C2.

- Timing of On-demand SSB operation

  - Starting timing for on demand SSB transmission

Agreement For RRC based OD-SSB indication, UE expects on-demand SSB is transmitted from the first on-demand SSB burst after receiving RRC carrying indication of OD-SSB transmission

For SSB bursts indicated by On-demand SSB SCell operation via RRC signalling, it was agreed that UE should expect on-demand SSB to be transmitted from the first on-demand SSB burst after receiving RRC carrying indication of OD-SSB transmission. However, if the RRC signalling specifies the number N of on-demand SSB bursts to be transmitted, it is unclear for UE how to determine the location of the first on-demand SSB burst without a clear defined time instance A related to RRC signalling. This could lead to a misalignment between the network and UE regarding the deactivation location of on-demand SSB transmission.

To address this issue, one method is to further define time instance A for SSB bursts indicated by On-demand SSB SCell operation via RRC signalling. Similar to the MAC CE signalling procedure, the on-demand SSB deactivation location could be determined within the RRC signalling procedure once time instance A is certain to UE. Another method is to restrict the deactivation method about the on-demand burst transmission number of N can only be indicated via MAC CE. Thus the misalignment about the deactivation location between the network and UE could be avoided. To minimize additional spec efforts, it is preferred to adopt the second method.

Proposal 8: The number N of on-demand SSB bursts to be transmitted could only be indicated via MAC-CE.

  - End timing for on demand SSB transmission

Agreement For a cell supporting on-demand SSB SCell operation, deactivation of on-demand SSB transmission is supported. In order to deactivate on-demand SSB transmission from a UE perspective, support at least one of the following options. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - Option 3: Configuration/indication of the duration of on-demand SSB transmission window - Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell - Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired - Option 5: On-demand SSB transmission, if any, is deactivated when SCell activation is completed - Option 6: Explicit indication of deactivation for on-demand SSB via [group-common] DCI - FFS: Each option is applicable to which Cases or Scenarios - FFS: Details related to each of the above options Agreement For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 - FFS: Whether the value of N can be implicitly determined using a timer Agreement For a cell supporting on-demand SSB SCell operation, the following combinations of scenarios and cases are supported for indicating OD-SSB using a MAC-CE. - Scenario #3B and Case #1 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication. - Scenario #3B and Case #2 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication. Agreement For a cell supporting on-demand SSB SCell operation, for Case #1 (i.e., No always-on SSB on the cell) - UE does not expect the OD-SSB transmission indicated by RRC/MAC-CE to be deactivated while the SCell is activated. Agreement For a cell supporting on-demand SSB SCell operation, for configuring the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofBurst), - Alt 2: The value range of od-ssb-nrofBurst is {N2 integer values} - N2= [8] - If od-ssb-nrofBurst for an on-demand SSB is NOT configured, the on-demand SSB is deactivated via MAC CE. - If od-ssb-nrofBurst for an on-demand SSB is configured, the on-demand SSB is deactivated based on the configured value for od-ssb-nrofBurst [or via MAC CE].

In Case #1, where there is no always-on SSB on the SCell, UE would perform synchronization and AGC periodically on this cell for data transmission and reception after the completion of SCell activation. Thus, on-demand SSB should not terminate before the SCell deactivation, ensuring continuous periodic SSB transmission for synchronization and AGC procedure once activated alongside the SCell activation command. Compared to Option 1, the overhead of an explicit MAC CE deactivation signalling could be saved in Option 4 and Option 4a. Moreover, the ending time of OD-SSB transmission defined in the last meeting of Option 1 does not consider the timeline of SCell deactivation. Therefore, Option 4 and 4a should be supported.

Proposal 9: Support Option 4 and 4a for Case #1 in addition to Option 2.

To achieve Option 4 and 4a, a nonnumerical indication of N could indicate that on-demand SSB transmission is deactivated along with the deactivation of SCell.

Proposal 10: A nonnumerical indication of N indicates that on-demand SSB transmission is deactivated along with the deactivation of SCell.

A remaining FFS states that whether the value of N can be implicitly determined using a timer. In our opinion, the time of on-demand SSB transmission depends on different Scenario and different period. Thus, it is difficult to configure only several values of transmission timer. In addition, how to determine the start of this timer also needs to be decided. Given Option 2 has been agreed, there is no need to introduce another similar method requiring more efforts.

Proposal 11: Do not support that the value of N can be implicitly determined using a timer.

In addition, it should be decided that whether MAC CE could be used to deactivate the on-demand SSB if od-ssb-nrofBurst for an on-demand SSB is configured. Considering the network may indicate a large transmitted number of on-demand SSB before SCell activation, expecting the on-demand SSB could serve during the SCell activation procedure and the following procedure, but the SCell may be not selected to be activated in the end. Obviously, deactivate the on-demand SSB transmission via MAC CE signaling in advance is beneficial to the power saving of the network. Therefore, it could be supported that MAC CE could be used to deactivate the on-demand SSB if od-ssb-nrofBurst for an on-demand SSB is configured.

Proposal 12: MAC CE could be used to deactivate the on-demand SSB if od-ssb-nrofBurst for an on-demand SSB is configured.

- Configuration of On-demand SSB operation

Agreement - For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported. - Frequency of the on-demand SSB - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst - Periodicity of the on-demand SSB - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE - FFS: Whether the RRC is newly introduced or existing RRC is reused Agreement - For a cell supporting on-demand SSB SCell operation, at least the followings for on-demand SSB are known to UE. - Sub-carrier spacing of the on-demand SSB - Physical Cell ID of the on-demand SSB - Location of on-demand SSB burst - Downlink transmit power of on-demand SSB - FFS: Other parameters - FFS: Whether each of above parameters is configured/indicated explicitly or not Agreement For a cell supporting on-demand SSB SCell operation, support to provide at least the following parameters for on-demand SSB configuration by RRC at least for Case #1. - Sub-carrier spacing of the on-demand SSB - FFS if this can be absent - Physical Cell ID of the on-demand SSB - FFS: Time domain location of on-demand SSB burst such as SFN offset and half frame index - Downlink transmit power of on-demand SSB - FFS: The number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS whether the above parameters are configured by reusing legacy RRC parameters or new RRC parameters Agreement For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB - FFS: Any other relevant parameters Agreement - For a cell supporting on-demand SSB SCell operation, support to configure time domain location of on-demand SSB per on-demand SSB periodicity by RRC for both Case #1 and Case #2. - For Case #1 (i.e., No always-on SSB on the cell), - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0 - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0. - If half frame index parameter is NOT configured, UE assumes half frame index set to 0. - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced. - The value range of half frame index is 0 or 1. - For Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), down-select one of the following alternatives. - Alt A: Same as for Case #1 - Alt B: Based on a single parameter which is to indicate the time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset) Agreement - For a cell supporting on-demand SSB SCell operation, to provide time domain location of on-demand SSB by RRC, - For Case #2, adopt Alt A in the previous RAN1 agreement, i.e., - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0 - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0. - If half frame index parameter is NOT configured, UE assumes half frame index set to 0. - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced. - The value range of half frame index is 0 or 1. - FFS: Whether/how to indicate time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset) - By defining reference point as SFN where AO-SSB burst exists Agreement - For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameter (i.e., od-ssb-config) for on-demand SSB configuration, for both Case #1 and Case #2. - For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameters to configure each of the followings, for both Case #1 and Case #2. - Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency) - FFS: Whether this parameter can be absent for Case #2 - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) - FFS: Whether this parameter can be absent for Case #2 - Periodicity of the on-demand SSB (i.e., od-ssb-Periodicity) - Sub-carrier spacing of the on-demand SSB (i.e., od-ssbSubcarrierSpacing) - For Case #2, this parameter is absent and sub-carrier spagcing of on-demand SSB is the same as that of always-on SSB. - Physical Cell ID of the on-demand SSB (i.e., od-ssb-physCellId) - For Case #2, this parameter is absent and physical cell ID of on-demand SSB is the same as that of always-on SSB. - Time location of on-demand SSB burst (i.e., od-ssb-sfn-Offset and od-ssb-halfFrameIndex) - Downlink transmit power of on-demand SSB (i.e., od-ss-PBCH-BlockPower) - For Case #2, this parameter is absent and downlink transmit power of on-demand SSB is the same as that of always-on SSB. - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofTx) - FFS: Which of the above parameters are a list

To provide time domain location of on-demand SSB by RRC, a remaining issue is whether to indicate the time offset between always-on SSB and on-demand SSB. However, since it has already been agreed to indicate the time domain location of on-demand SSB through the SFN offset and half frame index parameters, there is no need to introduce additional parameter to increase the UE complexity.

Proposal 13: The time offset between always-on SSB and on-demand SSB is not introduced.

In addition, to reduce signalling overhead, some of the on-demand SSB parameters can be consolidated into a parameter list. This allows the network to indicate multiple on-demand SSB parameters to UE using a single parameter list index. To save the signalling overhead effectively, the parameters with a small variable range are more suitable for composing a parameter list. Therefore, in addition to periodicity, SSB positions within an on-demand SSB burst, and the number N of on-demand SSB bursts, SFN offset and Half frame index could also be included in the parameter list.

Proposal 14: The parameter list includes the following parameters:

- SFN offset

- Half frame index

- L1 measurement and report

Agreement - Update the previous RAN1 agreement as follows. - At least support L1 measurement based on on-demand SSB - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported. - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB - The support of LTM is a separate discussion point Agreement For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), consider only one or both of the following options for UE to perform L1 measurement based on on-demand SSB. - Option 1: A CSI report configuration is associated with both of on-demand SSB and always-on SSB - Option 2: A CSI report configuration is associated with one of always-on SSB and on-demand SSB - FFS: Whether OD-SSB and always on SSB have same beam or not Agreement For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), - A CSI report configuration can be associated with - both of AO-SSB and OD-SSB - Which SSB to measure is up to UE implementation as long as RAN4 requirements are met - Applies at least for the case where AO-SSB and OD-SSB have the same center frequency and the same positions in the same SSB burst - FFS: It is up to UE to measure both SSBs or one of them if the higher layer timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “Configured”. - OD-SSB only - FFS: Applies at least for the case where AO-SSB and OD-SSB have the different center frequencies

  - On-demand SSB for CSI report

According to current design, if the higher layer timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “Configured”, UE should drive the channel measurements based on only the most recent, no later than the CSI reference resource, occasion of SSB associated with the CSI resource setting. Therefore, when a CSI report configuration is associated with both of AO-SSB and OD-SSB, it should be clarified that which SSB occasion is the most recent occasion given there are two types of SSB. Considering the time restriction aims to decrease the latency between channel measurement and CSI report, UE should measure only the most recent, no later than the CSI reference resource, occasion between OD-SSB occasion and AO-SSB occasion. If up to UE to measure both SSBs or one of them, the most recent AO-SSB occasion or OD-SSB occasion may be far away from the CSI reference resource, and the time restriction would be useless.

Proposal 15: If the higher layer timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “Configured”, UE should measure only the most recent, no later than the CSI reference resource, occasion between OD-SSB occasion and AO-SSB occasion.

When CSI report configuration is associated with one of on-demand SSB and always-on SSB, the existing CSI report configuration and mechanism can be mostly reused. In legacy, the associate SSB resource to a CSI report configuration is the SSB-index refers to the SSB indicated in servingcellconfig and servingcellconfigcommon. Therefore, given the SSB type cannot be distinguished from the SSB-index, how to distinguish the CSI resource associated with a CSI report configuration is always-on SSB or on-demand SSB needs to be solved. One possible method is to indicate the SSB type in the resource config associated with the CSI report configuration. Then UE could determine directly which SSB is associated with the report configuration.

Proposal 16: To distinguish whether AO-SSB or OD-SSB is associated with a CSI report configuration, indicate the SSB type in the resource config associated with the CSI report configuration.

In legacy, semi-persistent CSI report could be triggered by MAC CE or DCI, and aperiodic CSI report is triggered by DCI. A associated CSI-reportconfig is indicated in trigger signalling, then UE could determine the resource location and the resource type to perform measurement. Similarly, for a cell configured with on-demand SSB, UE could distinguish whether to measure on-demand SSB or always-on SSB through the associated report configuration. Thus, reusing the triggering/activation mechanisms for semi-persistent and aperiodic CSI report based on on-demand SSB is enough.

Proposal 17: Semi-persistent and aperiodic CSI report configuration can be associated with any kind of SSB.

However, how to support triggering periodic CSI report based on on-demand SSB should be further studied. In legacy, periodic CSI report should be associated with a periodic measurement resource. But on-demand SSB would be deactivated after a short duration in most scenarios, thus it is more like a semi-persistent resource. Hence, to support periodic CSI report, one possible method is to allow triggering periodic CSI report based on on-demand SSB only when on-demand SSB is activated at the time SCell is configured to UE through RRC signalling and is deactivated along with the deactivation of SCell. In this case, on-demand SSB could be considered a periodic resource.

Proposal 18: Periodic CSI report configuration can only be associated with on-demand SSB that is activated through RRC signalling and will be deactivated along with the deactivation of SCell.

  - On-demand SSB for LTM

LTM is a procedure in which a gNB receives L1 measurement report(s) from a UE, and on their basis the gNB may change UE serving cell by using a cell switch command signalled via a MAC CE. The cell switch command indicates an LTM candidate configuration that the gNB previously prepared and provided to the UE through RRC signalling. Then the UE switches to the target configuration according to the cell switch command. The LTM procedure can be used to reduce the mobility latency.

A periodical SSB transmission is necessary for UE to perform the LTM procedure in legacy. If there is only on-demand SSB transmission in a cell, this cell should transfer the SSB transmission pattern after configured as a LTM candidate cell. Then the cell cannot further obtain the network power saving gain. Thus, if on-demand SSB is supported for LTM, the network could still maintain the power saving state even if being configured as a LTM candidate cell. In addition, the on-demand SSB transmission could shorten the delay of the synchronization and measurement procedure to further reduce the mobility latency as the on-demand SSB could adopt a shorter periodicity.

However, the on-demand SSB triggered in Scenario #2 and Scenario #2A would likely be deactivated after a short- term transmission. Then if the SSB resource configured in the LTM candidate cell is the on-demand SSB indicated in Scenario #2 and Scenario #2A, there may be no transmitted SSB for UE to measure during the LTM procedure after the on-demand SSB deactivates. To solve this problem, it is better to allow that on-demand SSB could be triggered by the related LTM signaling, e.g. LTM configuration message, or triggered during the LTM procedure. But obviously, this is an absolutely separate issue from the on-demand SSB SCell operation we discussed in the last few meetings. Hence, we prefer not to support LTM for now.

Proposal 19: Do not support on-demand SSB for LTM.

- Collision between on-demand SSB transmission and other transmissions

In legacy, there might be collision between SSB reception and other UL transmissions, e.g. PUSCH, PUCCH or PRACH. UE would not transmit the UL transmission in the slot overlap with SSB reception as shown in below:

TS 38.213 Clause 11.1 If a UE - is configured with multiple serving cells and is provided with directionalCollisionHandling-r16 = 'enabled' for a set of serving cell(s) among the multiple serving cells, and - indicates support of half-DuplexTDD-CA-SameSCS-r16 capability, and - is not configured to monitor PDCCH for detection of DCI format 2_0 on any of the multiple serving cells, for a set of symbols of a slot that are indicated to the UE for reception of SS/PBCH blocks in a first cell of the multiple serving cells by ssb-PositionsInBurst in SystemInformationBlockType1 or by ssb-PositionsInBurst in ServingCellConfigCommon or, if the UE is not provided dl-OrJointTCI-StateList, by ssb-PositionsInBurst in SSB-MTCAdditionalPCI associated to physical cell ID with active TCI states for PDCCH or PDSCH, or for a set of symbols of a slot corresponding to SS/PBCH blocks configured for L1 beam measurement/reporting, the UE does not transmit PUSCH, PUCCH, or PRACH in the slot if a transmission would overlap with any symbol from the set of symbols, and the UE does not transmit SRS in the set of symbols of the slot in - any of the multiple serving cells if the UE is not capable of simultaneous transmission and reception as indicated by simultaneousRxTxInterBandCA among the multiple serving cells, and - any one of the cells corresponding to the same band as the first cell, irrespective of any capability indicated by simultaneousRxTxInterBandCA.

Similarly, the collision between on-demand SSB and other transmission would also exists. Then how to solve the collision in on-demand SSB operation should be studied. One possible method is to perform a prioritization procedure between on-demand SSB transmission and other transmissions. In legacy, SSB transmission would prioritize other transmission. Similarly, when on-demand SSB collides with other transmissions, on-demand SSB could prioritize over those transmissions.

Proposal 20: To support on-demand SSB operation, prioritize the on-demand SSB transmission if there is collision between on-demand SSB and other transmission.

- On-demand SSB for BFD

In the previous meetings, it was proposed by some companies to consider supporting on-demand SSB for BFD. However, in legacy, the RS resource configuration in the failureDetectionResourcesToAddModList set on SCell could only be periodic CSI-RS as shown below, which means SSB could not be used for BFD on SCell.

TS 38.133 Clause 8.5.1 The UE shall assess the downlink radio link quality of a serving cell based on the reference signal in the set as specified in TS 38.213 [3] in order to detect beam failure on: - PCell in SA, NR-DC, or NE-DC operation mode, - PSCell in NR-DC and EN-DC operation mode, - SCell in SA, NR-DC, NE-DC or EN-DC operation mode, - Deactivated PSCell in NR-DC and EN-DC operation mode The RS resource configurations in the set on PCell, PSCell or deactivated PSCell (if configured with bfd-and-RLM with value true) can be periodic CSI-RS resources and/or SSBs. RS resource configuration in the set on SCell shall be periodic CSI-RS. UE is not required to perform beam failure detection outside the active DL BWP unless the UE supports bwpOperationMeasWithoutInterrupt-r18, provided that the SSB is within the configured UE-specific CBW.

It is because that the QCL source in the TCI state of PDCCH could only be CSI-RS, not an SSB, this limitation arises because the bandwidth of the SSB is likely different from that of the PDCCH, and the beam configuration of the SSB may differ from that of the PDCCH. Given the basic design of on-demand SSB wouldn’t be different from the existing SSB, the same drawbacks persist when using on-demand SSBs as a reference resource for BFD. Hence, on-demand SSB should not be used for BFD on SCell either.

Observation 1: SSB is not used for BFD on Scell in legacy.

Proposal 21: Do not support on-demand SSB for BFD on Scell.

- Conclusion

This contribution focuses on the discussion of on-demand SSB for SCell with the following observations and proposals:

Observation 1: SSB is not used for BFD on Scell in legacy.

Proposal 1: When MAC CE is used for updating the transmission parameter of a transmitted OD-SSB for the cell, only the parameter of periodicity could be updated.

Proposal 2: For updating the transmission parameter of a transmitted OD-SSB for the cell, SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity.

Proposal 3: MAC CE could be used for updating the transmission parameter of a transmitted OD-SSB in Scenario #2 and Scenario #2A.

Proposal 4: Do not support the case where always-on SSB is CD-SSB and not on a synchronization raster.

Proposal 5: Always-on SSB and on-demand SSB are located in the same BWP with the smallest bandwidth, and the center frequency gap between always-on SSB and on-demand SSB are smaller than a pre-defined threshold.

Proposal 6: For the case when the center frequency locations of always-on SSB and on-demand SSB are different, other than SFN index, half frame index, kSSB and pdcch-ConfigSIB1 could also be different between always-on SSB and on-demand SSB in PBCH payload for the same SSB index.

Proposal 7：For the case when the center frequency locations of always-on SSB and on-demand SSB are same，the half frame index of AO-SSB and OD-SSB should be same in Alt Time C1 and could be different in Alt Time-C2.

Proposal 9: Support Option 4 and 4a for Case #1 in addition to Option 2.

Proposal 10: A nonnumerical indication of N indicates that on-demand SSB transmission is deactivated along with the deactivation of SCell.

Proposal 11: Do not support that the value of N can be implicitly determined using a timer.

Proposal 12: MAC CE could be used to deactivate the on-demand SSB if od-ssb-nrofBurst for an on-demand SSB is configured.

Proposal 13: The time offset between always-on SSB and on-demand SSB is not introduced.

Proposal 14: The parameter list includes the following parameters:

- SFN offset

- Half frame index

Proposal 15: If the higher layer timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “Configured”, UE should measure only the most recent, no later than the CSI reference resource, occasion between OD-SSB occasion and AO-SSB occasion.

Proposal 16: To distinguish whether AO-SSB or OD-SSB is associated with a CSI report configuration, indicate the SSB type in the resource config associated with the CSI report configuration.

Proposal 17: Semi-persistent and aperiodic CSI report configuration can be associated with any kind of SSB.

Proposal 18: Periodic CSI report configuration can only be associated with on-demand SSB that is activated through RRC signalling and will be deactivated along with the deactivation of SCell.

Proposal 19: Do not support on-demand SSB for LTM.

Proposal 20: To support on-demand SSB operation, prioritize the on-demand SSB transmission if there is collision between on-demand SSB and other transmission.

Proposal 21: Do not support on-demand SSB for BFD on Scell.

# Reference

- 3GPP TSG RAN Meeting #102, Edinburgh, Scotland, December 11th-15th, 2023, New WID: Enhancements of network energy savings for NR, RP-234065

- 3GPP TS 38.331, Radio Resource Control (RRC) protocol specification.

- 3GPP TR 37.910, Study on self evaluation towards IMT-2020 submission Chapter 5.7.2.1.