3GPP TSG RAN WG1 #121	                                           R1-2503377

St Julian’s, Malta, May 19th – 23th, 2025

Source:	vivo

Title:	Remaining issues on enabling data transmissions for XR during RRM measurements

Agenda Item:	9.10.1

Document for:	Discussion and Decision

- Introduction

In RAN1#120 meeting, the following agreements were made regarding enabling transmission/reception in gaps/restrictions that are caused by RRM measurements [1].

Agreement Confirm the Working assumption in RAN1#118 for supporting Alt. 1-1: Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI.. Conclusion It is RAN1 understanding that if measurement gap/restriction is indicated to be skipped by explicit indication by DCI, the TX/RX is enabled on all the serving cells in the applicable range of the skipped measurement gap/restriction. - Note: Applicable range of measurement gap/restriction is based on RAN4 specifications. Agreement Agreement from RAN1#119 is updated (red part) For explicit indication by DCI to skip a particular gap/restriction, in addition to DCI formats 0_1/1_1 and 0_2/1_2, one bit indication can be included as a part of DCI formats 0_3/1_3. - Explicit indication can be configured for each DCI format individually by higher layer. - The skipping indication corresponds to the first gap/restriction occasion among co-scheduled cells by the DCI (Type-1C field) Above applies only for intra-band CA cases (same SCS for cells in the set of co-scheduled cells). - Alt1: If skipping field is configured to DCI format 0_3/1_3, only cells that are intra-band (and have same SCS) can be co-scheduled. In other words, if co-scheduled cells could be inter-band, skipping indication field cannot be configured. Agreement When measurement gap/restriction is indicated to be skipped by explicit indication by DCI format 1_1/1_2 that does not schedule PDSCH and - if CIF field is present, the scheduled cell for the skipping indication is determined by CIF field. - if CIF field is not present, the scheduled cell for the skipping indication is the scheduling cell.

In this contribution, we will discuss some remaining issues about the enhancements to enable transmissions/receptions in gaps/restrictions that are caused by RRM measurements.

- Discussion

  - Remaining issues about enhancements to enable transmissions/receptions in gaps/restrictions

  - Determination of last DCI format for a gap/restriction occasion

In the previous meeting, two options were agreed for a UE to interpret the indication of bit value “0”. For Option 2, bit value equal to “1” indicates the corresponding gap/restriction occasion is to be “skipped”, and bit value equal to “0” means UE behavior for the corresponding gap/restriction occasion is as per legacy behavior. In other words, it means overriding operation is allowed for this option. From UE’s perspective, there can be multiple DCI formats corresponding the same gap/restriction occasion, and the multiple DCI formats may indicate different values. Thus, the UE needs to figure out a last DCI format among the multiple DCI formats for the same gap/restriction occasion. Firstly, the UE can figure out all the DCI formats corresponding the same gap/restriction occasion based on the gap skipping indicator field and the time offset, then these identified DCI formats can be ordered based on a certain rule. For example, these DCI formats are ordered based on an ascending order across indexes of PDCCH monitoring occasions where they are detected. For a same PDCCH monitoring occasion where multiple DCI formats are detected, either it is expected that all the DCI formats indicate the same value, or the DCI formats are ordered in ascending order across indexes of respective scheduled cells. For a PDCCH monitoring occasion and a scheduled cell, if there are multiple DCI formats from different TRPs corresponding to the same gap/restriction occasion, this is similar to the case of determining a PUCCH resource for HARQ-ACK reporting when joint mode for HARQ-ACK feedback is configured for mDCI-based mTRP scenario and same rule can be considered. That is, in this case, if a UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs on an active DL BWP of a serving cell, detected DCI formats from PDCCH receptions in the first CORESETs are indexed prior to detected DCI formats from PDCCH receptions in the second CORESETs.

Proposal 1: For Option 2, if there are multiple DCI formats corresponding to the same gap/restriction occasion, a UE follows the indication in a last DCI format, where the last DCI format is determined in a similar way as that of determining a last DCI format for HARQ-ACK PUCCH resource determination. Specifically, among the detected DCI formats that have a value of gap skipping indicator field indicating a same gap/restriction occasion, to determine a last DCI format for determining skipping status for the gap/restriction occasion,

- The detected DCI formats are first indexed in ascending order across indexes of respective scheduled cells for a same PDCCH monitoring occasion, and are then indexed in ascending order across indexes of PDCCH monitoring occasions.

  - For indexing a detected DCI format associated with two or more scheduled cells, a respective scheduled cell is the one with the smallest index among the two or more scheduled cells.

- If a UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs on an active DL BWP of a serving cell, detected DCI formats from PDCCH receptions in the first CORESETs are indexed prior to detected DCI formats from PDCCH receptions in the second CORESETs.

- Conclusion

In this contribution, some remaining issues about enhancements to enable transmissions/receptions in gaps/restrictions are discussed, with the following proposals:

Proposal 1: For Option 2, if there are multiple DCI formats corresponding to the same gap/restriction occasion, a UE follows the indication in a last DCI format, where the last DCI format is determined in a similar way as that of determining a last DCI format for HARQ-ACK PUCCH resource determination. Specifically, among the detected DCI formats that have a value of gap skipping indicator field indicating a same gap/restriction occasion, to determine a last DCI format for determining skipping status for the gap/restriction occasion,

- The detected DCI formats are first indexed in ascending order across indexes of respective scheduled cells for a same PDCCH monitoring occasion, and are then indexed in ascending order across indexes of PDCCH monitoring occasions.

  - For indexing a detected DCI format associated with two or more scheduled cells, a respective scheduled cell is the one with the smallest index among the two or more scheduled cells.

- If a UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs on an active DL BWP of a serving cell, detected DCI formats from PDCCH receptions in the first CORESETs are indexed prior to detected DCI formats from PDCCH receptions in the second CORESETs.

# References

- Chair’s notes RAN1#120 meeting, Athens, Greece, February 17th – 21st, 2025