3GPP TSG RAN WG1 #121	R1- 2503402

St Julian, Malta, 19 – 23 May 2025

Agenda item:		9.15.4

Source:	Nokia

Title:	Rel-19 Network Energy Saving UE features

Document for:		Discussion and Decision

# Introduction

This contribution presents our views on the remaining issues of the Rel-19 NR-NES UE features building on the outcome of RAN1#120bis [1].

# Discussion and proposals

## On-demand SSB SCell operation

RAN1#120bis agreed that on-demand SSB can be deactivated via MAC-CE at least when od-ssb-nrofBurst is not configured:

Agreement For a cell supporting on-demand SSB SCell operation, for configuring the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofBurst), - Alt 2: The value range of od-ssb-nrofBurst is {N2 integer values} - N2= [8] - If od-ssb-nrofBurst for an on-demand SSB is NOT configured, the on-demand SSB is deactivated via MAC CE. - If od-ssb-nrofBurst for an on-demand SSB is configured, the on-demand SSB is deactivated based on the configured value for od-ssb-nrofBurst [or via MAC CE].

Proposal 1: Confirm FG 61-3 component 2 and FG61-4 component 3 by removing the square brackets and yellow highlighting.

| # | Feature group | Components |
| --- | --- | --- |
| 61-3 | On-demand SSB SCell operation indicated via MAC CE in Case #1 | 1. Support MAC CE based signalling to indicate on-demand SSB transmission on the SCell in Case #1 (No always-on SSB on the cell) [2. Supported on-demand SSB deactivation mechanisms: - Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication Deactivation via Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated] |
| 61-4 | On-demand SSB SCell operation indicated via MAC CE in Case #2 for same center frequency | 1. Support MAC CE based signalling to indicate on-demand SSB transmission on the SCell in Case #2 (Always-on SSB is periodically transmitted on the cell) for same center frequency 2.Supported time domain relation between on-demand SSB and always-on SSB [3. Supported on-demand SSB deactivation mechanisms: - Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation via Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated] |

## Adaptation of common signal/channel transmissions

The remaining open issues relate to the note and component 5 of FG 61-7. With regard to the component 5, we don’t see sufficient motivation to spin it off as a separate FG, and hence suggest that component 5 is confirmed as part of FG61-7.

The NOTE of FG 61-7 seems to try to emphasize that the UE is supposed to be also able to transmit PRACH in the additional ROs, not merely support the configuration. In our view this is obvious and thus the note is redundant and could be removed.

Proposal 2: Component 5 is confirmed as part of the FG 61-7 and the related FFS point is deleted

Proposal 3: Delete the Note in FG 61-7 as redundant.

| # | Feature group | Components | Note |
| --- | --- | --- | --- |
| 61-7 | Adaptation of RACH in time domain based on additional RACH resources | 1. Support of adaptation of RACH in time domain based on additional RACH resources in RRC idle/inactive/connected mode 2. Configuration of additional PRACH resources via higher layer signaling 3. DCI-based indication of additional PRACH resources by DCI format 1_0 with P-RNTI 4. DCI-based indication of additional PRACH resources by DCI format 1_0 with C-RNTI for PDCCH-ordered PRACH [5. Support semi-static PRACH mask to identify the subset of additional resources] FFS: component 5 is kept in this FG or is separated to new FG | [A UE that supports transmission of PRACH in additional RO based on configuration of additional PRACH resources via higher layer signaling supports this FG] |

# References

- R1-2502977	Updated RAN1 UE features list for Rel-19 NR after RAN1 #120bis, Moderators (AT&T, NTT DOCOMO, INC.)

# Annex: Netw_Energy_NR_enh UE features after RAN1#120bis [1]

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 61. Netw_Energy_NR_enh | 61-1 | On-demand SSB SCell operation indicated by RRC based signaling in Case #1 | 1. Support RRC based signalling to indicate on-demand SSB transmission on the SCell in Case #1 (No always-on SSB on the cell) | FFS | Yes |  | UE does not support on-demand SSB transmission indicated by RRC based signaling in Case #1 | Per band | n/a | n/a | n/a | FFS: supported deactivation mechanisms Note: RRC based deactivation mechanism is up to RAN2 | Optional with capability signaling |
| 61. Netw_Energy_NR_enh | 61-2 | On-demand SSB SCell operation indicated by RRC based signaling in Case #2 for same center frequency | 1. Support RRC based signalling to indicate on-demand SSB transmission on the SCell in Case #2 (Always-on SSB is periodically transmitted on the cell) for same center frequency 2. Supported time domain relation between on-demand SSB and always-on SSB | FFS | Yes |  | UE does not support on-demand SSB transmission on the SCell indicated by RRC based signaling in Case #2 for same center frequency | Per band | n/a | n/a | n/a | Candidate value of component 2 = {Time-C1, Time-C1nC2} Note: - Time-C1: During OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern (the interval between SSB bursts is even and supported in legacy specification) - Time-C1nC2 includes both Time-C1 and Time-C2 (Time-C2: During OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern) Note: RRC based deactivation mechanism is up to RAN2 FFS: supported deactivation mechanisms | Optional with capability signaling |
| 61. Netw_Energy_NR_enh | 61-2a | On-demand SSB SCell operation indicated by RRC based signaling in Case #2 for different center frequency | 1. Support RRC based signalling to indicate on-demand SSB transmission on the SCell in Case #2 (Always-on SSB is periodically transmitted on the cell) for different center frequency between always-on SSB and on-demand SSB | 61-2 | Yes |  | UE does not support on-demand SSB transmission on the SCell indicated by RRC based signaling in Case #2 for different center frequency | Per band | n/a | n/a | n/a | Note: RRC based deactivation mechanism is up to RAN2 FFS: supported deactivation mechanisms | Optional with capability signaling |
| 61. Netw_Energy_NR_enh | 61-3 | On-demand SSB SCell operation indicated via MAC CE in Case #1 | 1. Support MAC CE based signalling to indicate on-demand SSB transmission on the SCell in Case #1 (No always-on SSB on the cell) [2. Supported on-demand SSB deactivation mechanisms: - Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication Deactivation via Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated] | FFS | Yes |  | UE does not support on-demand SSB transmission on the SCell indicated via MAC CE in Case #1 | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 61. Netw_Energy_NR_enh | 61-4 | On-demand SSB SCell operation indicated via MAC CE in Case #2 for same center frequency | 1. Support MAC CE based signalling to indicate on-demand SSB transmission on the SCell in Case #2 (Always-on SSB is periodically transmitted on the cell) for same center frequency 2.Supported time domain relation between on-demand SSB and always-on SSB [3. Supported on-demand SSB deactivation mechanisms: - Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation via Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated] | FFS | Yes |  | UE does not support on-demand SSB transmission on the SCell indicated via MAC CE in Case #2 for same center frequency | Per band | n/a | n/a | n/a | Candidate value of component 2 = {Time-C1, Time-C1nC2} Note: - Time-C1: During OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern (the interval between SSB bursts is even and supported in legacy specification) - Time-C1nC2 includes both Time-C1 and Time-C2 (Time-C2: During OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern) | Optional with capability signaling |
| 61. Netw_Energy_NR_enh | 61-4a | On-demand SSB SCell operation indicated via MAC CE in Case #2 for different center frequency | 1. Support MAC CE based signalling to indicate on-demand SSB transmission on the SCell in Case #2 (Always-on SSB is periodically transmitted on the cell) for different center frequency | 61-4 | Yes |  | UE does not support on-demand SSB transmission on the SCell indicated via MAC CE in Case #2 for different center frequency | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 61. Netw_Energy_NR_enh | 61-5 | SIB1 request for idle/inactive UEs | 1. Reception of SIB1 request configuration associated with SIB1 request for a cell 2. Transmission of PRACH on the uplink to request SIB1 of the cell 3. Reception of SIB1 [in a window] [at least] upon SIB1 request |  | FFS |  |  | FFS | FFS | FFS | FFS | A UE indicates support of this FG if it transmits a SIB1 request | Optional [with/without] capability signaling |
| 61. Netw_Energy_NR_enh | 61-6 | SSB burst periodicity adaptation for SCell operation | Support of adaptation of SSB burst periodicity for SCell by DCI format 2_9 |  | Yes |  | UE does not support adaptation of SSB burst periodicity for SCell | Per band | n/a | n/a | n/a | Note: the SSB for this FG is not cell defining SSB | Optional with capability signaling |
| 61. Netw_Energy_NR_enh | 61-7 | Adaptation of RACH in time domain based on additional RACH resources | 1. Support of adaptation of RACH in time domain based on additional RACH resources in RRC idle/inactive/connected mode 2. Configuration of additional PRACH resources via higher layer signaling 3. DCI-based indication of additional PRACH resources by DCI format 1_0 with P-RNTI 4. DCI-based indication of additional PRACH resources by DCI format 1_0 with C-RNTI for PDCCH-ordered PRACH [5. Support semi-static PRACH mask to identify the subset of additional resources] FFS: component 5 is kept in this FG or is separated to new FG |  | Yes |  | UE does not support adaptation of RACH in time domain based on additional RACH resources | Per band | n/a | n/a | n/a | [A UE that supports transmission of PRACH in additional RO based on configuration of additional PRACH resources via higher layer signaling supports this FG] | Optional with capability signaling |