3GPP TSG RAN WG1 #121	R1- 2503403

St Julian, Malta, 19 – 23 May 2025

Agenda item:		9.15.5

Source:	Nokia

Title:	Rel-19 LP-WUS/WUR UE features

Document for:		Discussion and Decision

# Introduction

This contribution presents our views on the remaining issues of the Rel-19 LP-WUS/WUR UE features building on the outcome of RAN1#120bis [1].

# Discussion and proposals

## IDLE/INACTIVE mode FGs 62-1 and 62-1a

The outcome of RAN1#120bis resulted with two FGs for idle/inactive mode [1]:

We propose the following modifications:

Proposals on FG62-1:

- Component 2: Remove square brackets

- Component 3: Modify the description to reflect that the UE is configurable with the relaxed requirements:

- [3. The support for configuration of relaxed RRM requirements of further RRM relaxation of UE MR]

- Component 4: Remove the square brackets and modify the description to reflect that the UE is configurable with the RRM measurement offloading:
[4. The support for configuration of serving cell RRM measurement offloading]

- Component 5: Remove square brackets

- Component 6 (new): Add a component for supported M values and corresponding L-values and remove the corresponding FFS:
6. The supported M and L values for OOK

- M=1, L = {6,8}

- M=2, L = {12,16}

- M=4, L = {16,32}

- FFS on other components: Remove the FFS line

Proposals on FG62-1a:

- FG name and Component 1: Remove the word ‘overlaid’, as the sequence is a sequence, there is no different sequence or different FG for support of OFDM sequence that is not overlaid.

- Component 2: Remove square brackets and FFS

- Component X: Remove square brackets and FFS, renumber as component 2a

- Component 3: Remove square brackets and clarify the description and confirm the component, and avoid specific references to a receiver type: 
[3. The support for configuration of relaxed RRM requirements of further RRM relaxation of UE MR]

- Component 4: Remove the square brackets and modify the description to reflect that the UE is configurable with the RRM measurement offloading:
[4. The support for configuration of serving cell RRM measurement offloading]

- Component 5: Remove square brackets. Add component values in []

Component 5 candidate values: FFS [{Capability 1, Capability 2, Capability 3}]

- Component 6 (new): Add a component for supported M values and corresponding L-values and remove the corresponding FFS: 
6. The supported M and L values for OOK

- M=1, L = {6,8}

- M=2, L = {12,16}

- M=4, L = {16,32}

- FFS on other components: Remove the FFS line.

Resulting with the following updates to FG62-1 and 62-1a:

| Index | Feature group | Components | Prerequisite feature groups | Type | Note |
| --- | --- | --- | --- | --- | --- |
| 62-1 | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OOK [2. The support of LP-SS based RRM measurement and/or SSB-based measurement, corresponding to component 1] [3. The support for configuration of relaxed RRM requirements of further RRM relaxation of UE MR] [4. The support for configuration of serving cell RRM measurement offloading] [5. The supported wake-up delay] 6. The supported M and L values for OOK - M=1, L = {6,8} - M=2, L = {12,16} - M=4, L = {16,32} FFS: M values FFS: Other components and/or further FG separation |  | Per Band | Component 5 candidate values: FFS [{Capability 1, Capability 2, Capability 3}] |
| 62-1a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OFDM overlaid sequence [2. FFS: The support of LP-SS based RRM measurement and/or [X. FFS: 2a. The support of SSB-based RRM measurement] [3. The support for configuration of relaxed RRM requirements of further RRM relaxation of UE MR] [4. The support for configuration of serving cell RRM measurement offloading] [5. The supported wake-up delay] 6. The supported M and L values for OOK - M=1, L = {6,8} - M=2, L = {12,16} - M=4, L = {16,32} FFS: M values FFS: Other components and/or further FG separation |  | Per band | Component 5 candidate values: FFS [{Capability 1, Capability 2, Capability 3}] |

## CONNECTED mode FGs 62-2, 62-2a and 62-3

The outcome of RAN1#120bis resulted with three FGs for CONNECTED mode [1]:

We propose the following modifications:

Proposals on FG62-2 and 62-2a:

- FG 62-2a name: Remove the word ‘overlaid’, as the sequence is a sequence, there is no different sequence or different FG for support of OFDM sequence that is not overlaid.

- FG 62-2a component 1: Remove the word ‘overlaid’, as the sequence is a sequence, there is no different sequence or different FG for support of OFDM sequence that is not overlaid.

- FG 62-2a consequence if not supported: Remove the word ‘overlaid’, as the sequence is a sequence, there is no different sequence or different FG for support of OFDM sequence that is not overlaid.

- Component 2: Confirm the component, add candidate values {1-1, 1-1 and 1-2} and clarify to what the procedures refer to.

1-1: LP-WUS monitoring before the drx-onDurationTimer

1-2: P-WUS monitoring outside C-DRX active time

Component 2 candidate values: FFS {1-1, 1-1 and 1-2}

- Component 3: Confirm the component. Update the candidate values to reflect the current situation in the WI, final values to be defined within the work item agenda.

Component 3 candidate values: FFS {[3,5,6], [10,13,20], [33,42]} ms.

- FFS, M values: Make this a component 5, with M values 1, 2 and 4 supported with the corresponding L-values agreed in RAN1#120bis.

5. The supported M and L values for OOK

- M=1, L = {6,8}

- M=2, L = {12,16}

- M=4, L = {16,32}

- FFS, SCS: Remove the FFS

- FFS, Other components: Remove the FFS

- FG type: Make the FGs ‘Per Band’

Proposals on FG62-3:

- FG type: Make the FG ‘Per Band’

| Index | Feature group | Components | Consequence if the feature is not supported by the UE | Prerequisite feature groups | Type | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 62-2 | LP-WUS operation in CONNECTED mode based on OOK signal | 1. LP-WUS operation in CONNECTED mode based on OOK signal 2. The supported procedure(s) 1-1: LP-WUS monitoring before the drx-onDurationTimer 1-2: P-WUS monitoring outside C-DRX active time 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. Support of LP-WUS frequency resource within active DL BWP 5. The supported M and L values for OOK - M=1, L = {6,8} - M=2, L = {12,16} - M=4, L = {16,32} FFS: M values FFS: SCS FFS: Other components and/or further FG separation | LP-WUS operation in CONNECTED mode based on OOK signal is not supported |  | [Per band] | Component 2 candidate values: FFS {1-1, 1-1 and 1-2} Component 3 candidate values: FFS |
| 62-2a | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence | 1. LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) 2. The supported procedure(s) 1-1: LP-WUS monitoring before the drx-onDurationTimer 1-2: P-WUS monitoring outside C-DRX active time 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. Support of LP-WUS frequency resource within active DL BWP 5. The supported M and L values for OOK - M=1, L = {6,8} - M=2, L = {12,16} - M=4, L = {16,32} FFS: M values FFS: SCS FFS: Other components and/or further FG separation | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence is not supported |  | [Per band] | Component 2 candidate values: FFS {1-1, 1-1 and 1-2} Component 3 candidate values: FFS {[3,5,6], [10,13,20], [33,42]} ms. |
| 62-3 | LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode | 1. Support of LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode | LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode is not supported | FG 62-2 or 62-2a | [Per band] |  |

# References

- R1-2502977	Updated RAN1 UE features list for Rel-19 NR after RAN1 #120bis, Moderators (AT&T, NTT DOCOMO, INC.)

# Annex: NW_LPWUS UE features after RAN1#120bis [1]

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 62. NR_LPWUS | 62-1 | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OOK [2. The support of LP-SS based RRM measurement and/or SSB-based measurement, corresponding to component 1] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [5. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal is not supported | Per Band | n/a | n/a | n/a | Component 5 candidate values: FFS | Optional with capability signalling |
| 62. NR_LPWUS | 62-1a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OFDM overlaid sequence [2. FFS: The support of LP-SS based RRM measurement and/or [X. FFS: The support of SSB-based RRM measurement] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [5. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence is not supported | Per band | n/a | n/a | n/a | Component 5 candidate values: FFS | Optional with capability signalling |
| 62. NR_LPWUS | 62-2 | LP-WUS operation in CONNECTED mode based on OOK signal | 1. LP-WUS operation in CONNECTED mode based on OOK signal 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. Support of LP-WUS frequency resource within active DL BWP FFS: M values FFS: SCS FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in CONNECTED mode based on OOK signal is not supported | [Per band] | n/a | n/a | n/a | Component 2 candidate values: FFS Component 3 candidate values: FFS | Optional with capability signalling |
| 62. NR_LPWUS | 62-2a | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence | 1. LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. Support of LP-WUS frequency resource within active DL BWP FFS: M values FFS: SCS FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence is not supported | [Per band] | n/a | n/a | n/a | Component 2 candidate values: FFS Component 3 candidate values: FFS | Optional with capability signalling |
| 62. NR_LPWUS | 62-3 | LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode | 1. Support of LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode | FG 62-2 or 62-2a | YES | n/a | LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode is not supported | [Per band] | n/a | n/a | n/a |  | Optional with capability signalling |