3GPP TSG RAN WG1 Meeting #120-bis	R1- 2503451

Wuhan, China, April 7th – 11st, 2025

Source: 	Moderator (Intel Corporation)

Title:	Editor’s summary on draft CR 38.215 for NR_AIML_air

Agenda item:	9

Document for:	Discussion

# Introduction

In this contribution, editor of TS38.215 summarizes email discussions for introduction of Rel-19 AI/ML air-interface features for TS38.215.

# Summary of Discussion

### Round#1 Discussion

Editor has provided initial version of the draft CR for TS38.215 containing introduction of Rel-19 AI/ML air-interface features in

https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_120b/Inbox/drafts/9(NR_R19)/38.215%20draft%20CRs/NR_AIML_air/R1-250xxxx%20TS38215_Rel19_CRdraft_AIML_v00.docx

The following RAN1 agreements and conclusions were used to develop the draft CR.

[R1#116]Agreement For Rel-19 AI/ML based positioning, the measurements for determining model input are based on the DL PRS and UL SRS defined in TS38.211. - Note: The use of SRS for MIMO resource is transparent to UE. [R1#116]Agreement - For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting: - timing information; - paired timing information and power information. [R1#116bis]Agreement For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to the existing UL RTOA reference time T0+tSRS as defined in TS 38.215. FFS: whether it is applicable when Case 3b is used to support multi-RTT [R1#117]Agreement Sample-based measurement is defined as: - The measurement is composed of Nt' samples of the estimated channel response in time domain. The timing information for the Nt' samples are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The corresponding measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' samples. - Nt' and k can be signalled - FFS: the value range of Nt'; the value range of integer k for the timing granularity T. - The timing information is defined relative to a reference time Further discussion is expected on the determination of Nt' and k (including signaling) , and a rule to be introduced for selecting Nt' samples. Note: It doesn’t imply the definition of Sample-based measurement will be captured into the spec. [R1#118]Agreement For the definition of sample-based measurement, select Nt’ samples out of a list of Nt consecutive samples - The Nt samples have timing granularity T. - FFS: the starting time of the list of Nt samples - FFS: the value range of Nt For the sample-based measurement (if accepted in Rel-19), - For measurement by TRP/gNB, the Nt’ selected samples are expected to be those with the highest power. - Note: Choice of the maximum value of Nt, Nt’ should take into account the need to preserve proprietary implementation. [R1#118]Agreement For AI/ML positioning Case 2b and 3b, regarding the power information for determining the model input, - For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of DL PRS-RSRPP, use the existing measurement report mapping table for PRS-RSRPP in 38.133 as a starting point. - For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of UL SRS-RSRPP, use the existing measurement report mapping table for SRS-RSRPP in 38.133 as a starting point. [R1#118]Agreement For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective, - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. [R1#118bis]Agreement From RAN1 perspective, for model inference of AI/ML positioning Case 3b, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) Channel measurement; - (Optional) Quality of the channel measurement; - FFS: details of the quality - (Mandatory) Time stamp of the channel measurement. [R1#118bis]Agreement From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) timing information; - (Optional) Quality of the timing information; - Existing IE “Timing Measurement Quality” can be reused. - (Mandatory) Time stamp. FFS: LOS/NLOS indicator. Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3. [R1#119]Agreement For the definition of sample-based measurement, for gNB/TRP measurement of an estimated channel response between a pair of UE and TRP, the starting time of the list of Nt consecutive samples is determined as follows. - starting time = first detected path rounded down with timing granularity T. Note: UE-side measurement is a separate discussion [R1#119]Agreement For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement, - The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T. - The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values. - The timing information is defined relative to a reference time, same as the path-based measurement. - The Nt’ selected time domain channel measurement values are expected to be those with the highest power. - The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T. - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values: - Nt'<=24. FFS: Nt' values. - Nt = {32, 64, 128} - FFS: k - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement - FFS: whether transmit offset from gNB to LMF Note: measurement by UE is a separate discussion. Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified [R1#120]Agreement For AI/ML based positioning Case 3b, with the existing definition of two channel measurement types (A) or type (B): (A) path-based measurement, i.e., measurement in the existing specifications (up to Rel-18), (B) Rel-19 enhanced measurement (see definition in RAN1#119 agreement for Case 3b). - Descriptive info of the channel measurement is included together with the channel measurement values, either explicitly or implicitly. - The descriptive info of the channel measurement includes at least: - measurement type (A) or type (B) and - measurement parameters. - Regarding measurement parameters, - For (A) path-based measurement, the measurement parameters include: number of additional paths, k. - For (B) Rel-19 enhanced measurement, the measurement parameters include at least: Nt', k. - FFS: Nt - Note: Type (A) and Type (B) are used for discuss purpose only. [R1#120bis]Conclusion - The offset (used in the agreement made in RAN1#119) refers to the duration (in samples) between the reference time and the starting time of the list of Nt consecutive values. [R1#120bis]Agreement For Rel-19 AI/ML based positioning, for Case 3b, “FFS: whether transmit offset from gNB to LMF” in RAN1#119 agreement is resolved by: - No offset is transmitted from gNB to LMF [R1#120bis]Conclusion For channel measurement type (A) (i.e., path-based measurement), it is not necessary to introduce enhancements on the number of reported paths in Rel-19. [R1#120bis]Conclusion For training data collection of Part B in AI/ML based positioning Case 3a, for the case when Part B label includes the LOS/NLOS indicator, - The corresponding label is provided from LMF to the gNB by using the existing IE LOS-NLOS-Indicator-r17. There is no need of a separate field for quality indicator of LOS/NLOS indicator.

Editor has asked companies to provide inputs and comments on initial version.

##### Company Comments for Initial version of draft CR

| Company | Comments |
| --- | --- |
| Google | We suggest adding the definition of predicted RSRP. Current SS-RSRP/CSI-RSRP cannot be reused for predicted RSRP. For example, the SS-RSRP is defined as follows: “SS reference signal received power (SS-RSRP) is defined as the linear average over the power contributions (in [W]) of the resource elements that carry secondary synchronization signals. The measurement time resource(s) for SS-RSRP are confined within SS/PBCH Block Measurement Time Configuration (SMTC) window duration. If SS-RSRP is used for L1-RSRP as configured by reporting configurations as defined in TS 38.214 [6], the measurement time resources(s) restriction by SMTC window duration is not applicable.” But according to the RAN1 agreement below, UE is not required to measure the Set A RS, and the SMTC should not be applicable for predicted RSRP, since UE does not need to measure the RS for predicted RSRP. Agreement For UE-sided model at least for BM Case-1, for inference results report - Two resource sets can be configured for Set A and Set B separately in the CSI report configuration for the report - FFS whether support only resource set for Set B is configured - UE performs measurement on the resource set for Set B for inference, and UE is not expected to measure resource set for Set A for inference, - The beam information in the inference report refers to the resource set for Set A One possible way is that we can add some changes for current SS-RSRP/CSI-RS-RSRP definition. We would like to propose the following changes. Probably we can also consider adding a dedicated section to define the predicted RSRP, which could be cleaner. ----------------------------- start of TP -------------------------- ### 5.1.1 SS reference signal received power (SS-RSRP) Definition; SS reference signal received power (SS-RSRP) is defined as the linear average over the power contributions (in [W]) of the resource elements that carry secondary synchronization signals. The measurement time resource(s) for SS-RSRP are confined within SS/PBCH Block Measurement Time Configuration (SMTC) window duration. If SS-RSRP is used for L1-RSRP or predicted RSRP as configured by reporting configurations as defined in TS 38.214 [6], the measurement time resources(s) restriction by SMTC window duration is not applicable. For SS-RSRP determination other than SS-RSRP for predicted RSRP demodulation reference signals for physical broadcast channel (PBCH) and, if indicated by higher layers, CSI reference signals in addition to secondary synchronization signals may be used. SS-RSRP using demodulation reference signal for PBCH or CSI reference signal shall be measured by linear averaging over the power contributions of the resource elements that carry corresponding reference signals taking into account power scaling for the reference signals as defined in TS 38.213 [5]. If SS-RSRP is not used for L1-RSRP, the additional use of CSI reference signals for SS-RSRP determination is not applicable. If the SS-RSRP is not used for predicted RSRP, SS-RSRP shall be measured only among the reference signals corresponding to SS/PBCH blocks with the same SS/PBCH block index and the same physical-layer cell identity. If SS-RSRP is not used for L1-RSRP or predicted RSRP and higher-layers indicate certain SS/PBCH blocks for performing SS-RSRP measurements, then SS-RSRP is measured only from the indicated set of SS/PBCH block(s). For frequency range 1, the reference point for the SS-RSRP shall be the antenna connector of the UE. For frequency range 2, SS-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported SS-RSRP value shall not be lower than the corresponding SS-RSRP of any of the individual receiver branches.. Applicable for; If SS-RSRP is used for L1-RSRP or predicted RSRP, RRC_CONNECTED intra-frequency. Otherwise, RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_INACTIVE intra-frequency, RRC_INACTIVE inter-frequency, RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency. NOTE 1: The number of resource elements within the measurement period that are used by the UE to determine SS-RSRP is left up to the UE implementation with the limitation that corresponding measurement accuracy requirements have to be fulfilled. NOTE 2: The power per resource element is determined from the energy received during the useful part of the symbol, excluding the CP. ### 5.1.2 CSI reference signal received power (CSI-RSRP) Definition; CSI reference signal received power (CSI-RSRP), is defined as the linear average over the power contributions (in [W]) of the resource elements of the antenna port(s) that carry CSI reference signals configured for RSRP measurements within the considered measurement frequency bandwidth in the configured CSI-RS occasions. For CSI-RSRP determination other than CSI-RSRP for predicted RSRP CSI reference signals transmitted on antenna port 3000 according to TS 38.211 [3] shall be used. If CSI-RSRP is used for L1-RSRP, CSI reference signals transmitted on antenna ports 3000, 3001 can be used for CSI-RSRP determination. For intra-frequency CSI-RSRP measurements, if the measurement gap is not configured, UE is not expected to measure the CSI-RS resource(s) outside of the active downlink bandwidth part. For frequency range 1, the reference point for the CSI-RSRP shall be the antenna connector of the UE. For frequency range 2, CSI-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported CSI-RSRP value shall not be lower than the corresponding CSI-RSRP of any of the individual receiver branches.. Applicable for; If CSI-RSRP is used for L1-RSRP or predicted RSRP, RRC_CONNECTED intra-frequency. Otherwise, RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency. NOTE 1: The number of resource elements within the considered measurement frequency bandwidth and within the measurement period that are used by the UE to determine CSI-RSRP is left up to the UE implementation with the limitation that corresponding measurement accuracy requirements have to be fulfilled. NOTE 2: The power per resource element is determined from the energy received during the useful part of the symbol, excluding the CP. ----------------------------- end of TP -------------------------- |
| NEC | Thank you for editor's great efforts. Regarding the updates on 5.2.9 and 5.2.10, actually RAN1#120bis had some discussion (R1-2503144, section 2.3) on how to update the definition of existing UL SRS-RSRPP in TS 38.215 to capture the Rel-19 positioning measurements. We think the outcome (if any) may largely impact the current texts provided by editor. To this end, could you kindly add following notes for the two measurement definitions? Thank you. Editor's Note: The texts above could be further refined based on further RAN1 agreements Regarding the draft CR contents itself, we have following suggested modifications: "…The Nt values of timing information are selected such that they correspond to the time instances which are expected to be those with the highest power values among Nt consecutive time instances…" to better align the agreement. |
| Nokia | Thanks to Editor for the great effort. In RAN1#120bis there was a similar proposal rejected by few companies directly related to this CR. The main reason is that few companies understanding is that in the “R1#119 Agreement”, the main sentence is indicated the following (check the highlighted yellow text): [R1#119]Agreement For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement, … So, the definition related to sample based representation is only an enhancement to the current path-based measurement (the one that is today specified) and not a new measurement. So, we do not agree to introduce a new definition for Case 3b measurement in TS 38.215. If it was not the mutual understanding between companies, we may further discuss it in RAN1#121 (Malta). |
| HW/HiSi | There is no need to introduce the new definitions 5.2.9 and 5.2.10 in 38.215, as there is no RAN1 agreement indicating to introduce new measurements for Case 3b. RAN1 has agreed to additionally support for Case 3b an enhancement of the path-based measurement from legacy, where in contrast to legacy, a rule for the selection of the paths/values has been defined and in addition, the number of reported paths/values can be up to 24. Although no rule for the path selection has been specified in legacy, the legacy framework for the definition of UL RTOA and UL SRS-RSRPP measurement in legacy 38.215 is generic and also applicable for the R19 enhancement of the path-based measurement. As the timing information of the paths/values in the enhancement of the path-based measurement is relative to the UL RTOA reference time and based on the first detected path, the timing information for the enhancement of the path-based measurement can be reported in the same way that the additional path delays are reported for an UL RTOA measurement in NRPPa. Likewise, the power information of the paths/values in the enhancement of the path-based measurement can be reported in the same way that the power information of the additional path delays is reported in legacy, i.e., by using UL SRS-RSRPP. Furthermore, RAN3 has prepared a CR for 38.455 which already captures the number of reported paths/values (up to 24), and it is our RAN3 colleagues’ understanding that the rule for the selection of the paths/values can also be captured in the procedural description within NRPPa. Thus, to avoid duplicated specification, the R19 enhancement of the path-based measurement should not be introduced in TS38.215, in which procedures are not specified. |
| vivo | Thanks editor for your great efforts. We fully support this draft CR. To Nokia: About Nokia’s statement, “In RAN1#120bis there was a similar proposal rejected by few companies directly related to this CR”. First of all, I checked the latest Chairman’s notes and there’s no proposal related to this CR treated online in RAN1#120bis. If you were referring to proposals in FL summary during offline discussion, please be precise as otherwise it may be interpreted that there was RAN1 conclusion/agreement not to introduce new measurement in 38.215. To Nokia, HW/HiSi: We don’t understand why enhanced measurement cannot be defined in 38.215. Although similar in concepts, but current definitions of UL RTOA and UL SRS-RSRPP cannot be used directly for sample-based measurement. That is why FL organized discussion in RAN1#120bis where modification to the definitions of UL RTOA and UL SRS-RSRPP were proposed to support sample-based measurement. That discussion was not conclude as some companies saying editor can do it directly. Recall that we made this agreement. [R1#120]Agreement For AI/ML based positioning Case 3b, with the existing definition of two channel measurement types (A) or type (B): (A) path-based measurement, i.e., measurement in the existing specifications (up to Rel-18), (B) Rel-19 enhanced measurement (see definition in RAN1#119 agreement for Case 3b). - Descriptive info of the channel measurement is included together with the channel measurement values, either explicitly or implicitly. - The descriptive info of the channel measurement includes at least: - measurement type (A) or type (B) and - measurement parameters. - Regarding measurement parameters, - For (A) path-based measurement, the measurement parameters include: number of additional paths, k. - For (B) Rel-19 enhanced measurement, the measurement parameters include at least: Nt', k. - FFS: Nt - Note: Type (A) and Type (B) are used for discuss purpose only. Clearly, path-based and sample-based measurement are not the same thing as otherwise we would not having the above agreement. If no new measurement definitions were introduced in 38.215, how to tell them apart in the measurement request/report where it’s a type (A) or type (B)? In our understanding, RAN3’s CR itself is not sufficient and thus definitions for sample-based measurement in 38.215 is not duplicate but rather necessary. |
| Qualcomm | Added by Moderator/Editor which is copied form the RAN1 reflector: We do not agree to introduce new definitions for Case 3b measurement in TS 38.215. As you can see from RAN1 agreement, these are enhanced measurements which their relation to existing measurements can be clarified and captured in procedure-related specifications. Agreement (RAN1 #119) For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement, - The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T. - The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values. - The timing information is defined relative to a reference time, same as the path-based measurement. - The Nt’ selected time domain channel measurement values are expected to be those with the highest power. - The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T. - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values: - Nt'<=24. FFS: Nt' values. - Nt = {32, 64, 128} - FFS: k - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement - FFS: whether transmit offset from gNB to LMF Note: measurement by UE is a separate discussion. Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified Agreement (RAN1#118) For AI/ML positioning Case 2b and 3b, regarding the power information for determining the model input, - For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of DL PRS-RSRPP, use the existing measurement report mapping table for PRS-RSRPP in 38.133 as a starting point. - For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of UL SRS-RSRPP, use the existing measurement report mapping table for SRS-RSRPP in 38.133 as a starting point.. |
| Vivo2 | In RAN1#120b, we have agreed to have SGCS as a new UE report for monitoring of AI based CSI prediction. The relevant agreements are listed below. Agreement For CSI prediction using UE-side model, for performance monitoring, support UE assisted performance monitoring subject to an additional UE capability, and UE assisted performance monitoring is based on Type 3 performance monitoring Agreement For CSI prediction using UE-side model, for performance monitoring type 3, support SGCS as a performance metric. Agreement For the definition of SGCS, Note: How to handle layer mapping mismatch, if any, is up to UE implementation. Hence, we think SGCS needs to be defined accordingly in 215 as a new measurement quantity. |
| Xiaomi | For the new measurement definition of AI positioning case 3b, we share similar view with vivo that a new measurement definition should be considered. Firstly, the sample-based measurement is not the same with path-based measurement although there is certain commonality. As shown in the achieved agreement, the generation manner, the configuration is different and the existing definition can’t reflect such difference, so we consider new measurement definition is needed. For the discussion in RAN3, the agreed draft CR captured the sample-based as a new channel measurement type although currently it is set as FFS currently. In this case, a new measurement definition in 38.215 is necessary to support the measurement type configuration in 38.455. >>TRP Measurement Type; M; ENUMERATED (gNB-RxTxTimeDiff, UL-SRS-RSRP, UL-AoA, UL-RTOA,…, Multiple UL-AoA, UL SRS-RSRPP, UL-RSCP, Channel Response(FFS)); The UL-RSCP measurement is applicable only when the UL-RTOA and/or gNB-RxTxTimeDiff measurement(s) is also requested.; -. |
| HW/HiSi2 | We thank again the editor for the great efforts. Regarding the reporting of measurements for positioning, the measurement report in legacy is captured in TS 38.455 and thus, the same principle can be followed to distinguish the enhancement of the path-based measurement in NRPPa. Nothing needs to be done in TS 38.215, as no new form of measurement has been introduced, but rather an enhancement of a path-based measurement with a defined rule for path/value selection of up to 24 paths/values. |
| Google2 | Regarding the comment for adding SGCS from vivo. We suggest not adding it at current stage, since a lot of details are still FFS in the agreement below. Agreement For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, down-select one alternative by RAN1#121. - Alt 1: one SGCS is calculated based on predicted CSI for one inference reporting, ground truth CSI - Alt 2: one SGCS is calculated based on predicted CSI for one inference reporting, and ground truth CSI, another SGCS is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance. - Alt 3: statistic of reporting contents (e.g., mean, x% CDF of SGCS values) of inference reporting instances within configured monitoring window - Monitoring window can be configured by NW - FFS on signalling details - FFS on whether to report per prediction instance, selected prediction instance(s), averaged over prediction instances - FFS on report frequency granularity (e.g., per wideband or per subband or averaged over subband or selected subband) - FFS on whether to report per layer, the first layer, averaged over layer - FFS on how to quantize and report mechanism |
| Editor | @Google: For adding Predicted RSRP. It is not immediately clear that just because measurements across set A and B should not be combined, this is equivalent to not being able to make measurements within SMTC. The measurement distinction for set A and set B could be performed within SMTC or not. This is not clearly addressed by any agreements. Therefore, Editor is hesitant to start adding exclusion rules for predicted RSRP for SMTC. It would be better if RAN1 make a clear agreement on the matter. As for applicability of Predicted RSRP to connected mode intra-frequency, Editor thinks this should be reasonable but unable to find any agreements on the issue. Editor will add the suggestion but with an editor note that this is not explicitly agreed but seems to be implicitly derived based on set of agreements on predicted RSRP operation. As for limiting RS for measurements for Predicted RSRP, Editor is hesitant to make changes to start limiting RS for measurement for predicted RSRP. The predicted RSRP itself is not an measurement but a hypothetical value derived from RSRP or other measurements the UE makes. There doesn’t seem to be any formal agreement that predicted RSRP cannot be derived from measured from regular RSRP, and if so, the regular RSRP will have the RS restriction set forth for RSRP. Therefore, it is difficult to conclude that prediction RSRP should have a exception on this. If this is something that needs to be clarified, then it might be better to have an explicit RAN1 agreement on the matter. @NEC: Added editor note to UL SRS-RSRPP as requested. For the addition of “expected to be”. Editor has thought about this for some time and do not think it is ideal for specification. It is understood the language was deliberately chosen to give some degree of freedom for implementation as RAN1 was not able to conclude on reporting criteria. However, it is not immediately clear if the current language without “expected to be” necessarily restricts implementation. Whether gNB is reporting expecting peaks or actually detect peaks would not be visible to outside the gNB implementation. What is important is how entity that processes the reported information perceives the information. From that perspective, the entity would treat the reported information the same way assuming they correspond to highest peaks, irrespective of it was some imaginary expected peaks or actual detected peaks. Therefore, Editor suggests to keep the description simple as possible without the qualitive language. @Nokia: Editor has consulted with feature lead of AI/ML positioning on the matter and Editor is with the understanding concluded path based measurements and sample based measurement are NOT identical and non-interchangeable and hence the need for “enhancements”. This seems to be supported by Agreement in RAN#120 (as pointed out by vivo) Agreement (RAN1#120) For AI/ML based positioning Case 3b, with the existing definition of two channel measurement types (A) or type (B): (A) path-based measurement, i.e., measurement in the existing specifications (up to Rel-18), (B) Rel-19 enhanced measurement (see definition in RAN1#119 agreement for Case 3b). - Descriptive info of the channel measurement is included together with the channel measurement values, either explicitly or implicitly. - The descriptive info of the channel measurement includes at least: - measurement type (A) or type (B) and - measurement parameters. - Regarding measurement parameters, - For (A) path-based measurement, the measurement parameters include: number of additional paths, k. - For (B) Rel-19 enhanced measurement, the measurement parameters include at least: Nt', k. - FFS: Nt - Note: Type (A) and Type (B) are used for discuss purpose only.. It is not clear to the editor if there was a hidden intent behind phrasing of a feature as “enhancement” or something else. The goal of the specification should be to provide a clear and concise method to describe the feature agreed. The conclusion from RAN1#120bis that states that reported path enhancements are not made (see below) seems to strongly imply the sample based measurement should be defined. It is unclear to the editor how to address the conclusion from RAN1 #120bis while not defining the measurement definition in a clear and concise way. RAN1#120bis: Conclusion For channel measurement type (A) (i.e., path-based measurement), it is not necessary to introduce enhancements on the number of reported paths in Rel-19.. Additionally, based on following agreements from RAN1 #119, sample based measurement starts the quantization of the samples at the first detected path rounded down with timing granularity T and reports power values of different sample instances. However, the TOA report provides information whether certain peaks exists within a range of boundary values that correspond to quantized sample times, and the reported power will correspond to these peaks within the range of boundary values. Therefore, they do not seem identical and difficult to imagine how this could be defined by using “enhancement” procedural text (that should not be introduced according to RAN1#120bis conclusion). Agreement For the definition of sample-based measurement, for gNB/TRP measurement of an estimated channel response between a pair of UE and TRP, the starting time of the list of Nt consecutive samples is determined as follows. - starting time = first detected path rounded down with timing granularity T. Note: UE-side measurement is a separate discussion Agreement For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement, - The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T. - The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values. - The timing information is defined relative to a reference time, same as the path-based measurement. - The Nt’ selected time domain channel measurement values are expected to be those with the highest power. - The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T. - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values: - Nt'<=24. FFS: Nt' values. - Nt = {32, 64, 128} - FFS: k - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement - FFS: whether transmit offset from gNB to LMF Note: measurement by UE is a separate discussion. Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified. From editor understanding, the concern seems to on questioning the best method to capture a feature in the specification rather than the technical contents of the draft CR. If so, editor suggests moving on with endorsing the draft CR as is. Companies can have further discussion in Malta, if there could be consensus on a better method to capture the agreements. Once agreements are made, Editor can work out the details to draft the specification based on updated agreements. Also on a note, if RAN1 has concluded to not introduce enhancements to path based reporting, the given that there seems to be a CR being discussed in RAN3 which exactly does the opposite, Editor would recommend RAN1 to discuss potentially sending an LS to RAN3 or revise the conclusion made in RAN1. It doesn’t look good if different WG are making different agreements for the same feature. Given that RAN1 is the lead on the measurement definitions, sending an LS to RAN3 may be more reasonable approach. @Huawei: Please see the response to Nokia that address the same comments. @Qualcomm: Pleas see the response to Nokia that address the same comments. @vivo: For the SGCS, I’ve check the Mihai’s TS38.214 CR and it doesn’t seem to be captured there. However, the procedural aspects for the SGCS is also not captured in 38.214 CR. The performance metric reporting using SCGS, doesn’t seem like a direct measurement but rather a derived metric that is partly from measurements and partly from processed information of measurements. The nature of the definition, seems to hint that this is better defined in TS38.214. With that said, if the definition is not going to be captured in TS38.214, then Editor agrees that it should be captured in TS38.15. Given that the procedural aspects of SCGS report is not included in TS38.214 yet, Editor would like to suggest to revisit where to capture the report definition in the next time. Meanwhile, Editor of TS38.215 can have further discussion with Editor of TS38.214 to discuss how to get alignment between the two specifications. If Editors can get alignment before the endorsement deadline, then the draft CR for both TS38.214 and TS38.215 will be updated for SCGS. Editor apologizes for the delay in getting features captured correctly. @Google (on SCGS): See response to vivo above. |

### Round#2 Discussion

Editor has provided initial version of the draft CR for TS38.215 containing introduction of Rel-19 AI/ML air-interface features in

https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_120b/Inbox/drafts/9(NR_R19)/38.215%20draft%20CRs/NR_AIML_air/R1-250xxxx%20TS38215_Rel19_CRdraft_AIML_v01.docx

Please provide comments on the revision. The following is a summary of changes made based on comments received:

- Limiting Predicted RSRP for connected mode intra-frequency with editor note.

- Added editor note for UL-SRS-RSRPP as suggested by NEC. No change in actual specification text for UL-SRS-RSRPP.

##### Company Comments for Initial version of draft CR

| Company | Comments |
| --- | --- |
| HW/HiSi | We thank the editor for the reply and the great efforts. In our understanding, the issue is not about distinguishing the path-based measurement and the R19 enhancement of the path-based measurement (for which RAN1 has made the RAN1#120 agreement), but rather whether the new definitions 5.2.9 and 5.2.10 in 38.215 are needed for reporting the R19 enhancement of the path-based measurement. In 38.215, the definitions of UL RTOA and UL SRS-RSRPP are defined for a single path, independent of how the path has been selected. Thus, these definitions are applicable for reporting the timing information and the power information of a single path/value for both the path-based measurement and the R19 enhancement of the path-based measurement. The R19 enhancement differs from the path-based measurement in how the paths/values are selected, which is however not related to how the timing and power information of a single path/value is represented. With regards to the new definition 5.2.9, note that in legacy, the timing information of additional paths is not defined in 38.215 and hence, it is unclear why the timing information of additional paths/values of the R19 enhancement needs to be captured in 38.215. Furthermore, it is not appropriate to capture the procedure (selection rule of paths/vales of the R19 enhancement) in 38.215. Moreover, RAN1 has no agreements to introduce new definitions of measurements to represent per path/value level timing information and power information, and thus, we kindly ask the Editor to refrain from adding anything to 38.215 until further RAN1 agreement is made in Malta. We would also like to point out that the measurement type “Channel Response (FFS)” in the RAN3 CR in the measurement request of 38.455 does not imply that a new physical layer measurement should be introduced, e.g., as how the measurement type “Multiple UL-AoA” does not led to the introduction of a new definition in 38.215. |
| Qualcomm | We would like to thank the Editor for efforts brining the new definitions 5.2.9/.10 for Case 3b. Just to clarify a few related issues: - There was a long debate among RAN1 companies in Rel-19 on the necessity of introducing rule for measurement selection in Case 2b/Case 3b. The compromise was to avoid new measurements and focus on enhancing the path-based measurements (i.e., Type A) by introducing a rule for selection of additional path measurements (a.k.a., enhanced measurement – Type B). -As you can notice, there is no definition in 38.215 for timing info related to additional path measurements. The same principle needs to apply for the enhanced “path” measurements. The agreed enhancements (i.e., agreed selection rule) can be clarified in other specifications. In addition, for power related to enhanced measurements, it was agreed to use UL-SRS-RSRPP (similar to the legacy). Any additional clarifications can be clarified in the same definition or in other specifications. -While we understand there is need to clarify aspects related to the enhanced measurements, we think 38.215 is not the intended specifications. Due to divergence among companies, we suggest dropping the definitions for now and we can discuss this again in the May meeting. For technical content, it is difficult to endorse this as many details may end up unnecessary. Some of the current technical content can be a starting point for revision in other specifications. |
| Nokia | Dear Editor, first, let us thank you for the effort on moderating the measurement aspects related to Case 3b. Based on your reply to our first comment, we understand that other companies may have a different interpretation of the agreements. So, for this reason our suggestion is to align companies’ common understanding in May meeting before triggering definitions that may impact 38.215. |
| Ericsson | Editor/Moderator: Ericsson (feature lead) comments copied from Reflector below. Many thanks to the editor for the great effort, and many thanks to companies for the comments. The editor’s assessment below is very fair and reasonable in my view. RAN1 agreements are already made, and they should be captured. The dispute is on which spec is the best one to capture it. The only possibilities are 38.215 (RAN1 spec) or 38.455 (RAN3 spec). RAN1 can discuss in next meeting whether to (a) keep the draft CR texts in 38.215, or (b) send the texts in an LS to RAN3 and ask 38.455 editor to capture it. As I understand it, 38.455 is the only alternative if removing from 38.215. Having the draft CR texts captured is very useful for follow-up discussion, regardless of (a) or (b). As suggested by editor, companies can focus on the accuracy of the CR text, if any revision is needed on top of editor’s v01. |
| HW/HiSi2 | We thank again the editor for the great efforts. Indeed, the RAN1 agreements need to be captured, but as mentioned above, RAN1 has no agreements to introduce new definitions of measurements. RAN1 has agreed on a rule to select paths/values as an enhancement to the path-based measurement. Note that in legacy, no new definitions are introduced in 38.215 when the measurement is enhanced to include information of additional paths. Regarding the CR text, as 5.2.9 states that Nt, Nt and k are configured, we wonder whether this is aligned with the fact that the gNB/TRP may use different Nt, Nt and/or k values other than the signalled parameter, as per the RAN1#119 agreement. In addition, the purpose of 5.2.10 is not clear, as 5.2.10 basically describes Nt values of UL SRS-RSRPP. |
| Editor | @Huawei, Qualcomm, Nokia: Editor is unsure what is meant by RAN1 has no agreements to introduce new definition or not. There are agreements in RAN1 and Editor’s role is to determine the best way to capture the agreements in a technical way. If there is explicit agreements to not to capture something that is something that Editor should factor. However, it is unclear if it is forbidden to define definitions and parameters supported by the agreements, in the lack of presence of agreements to create something new. This logic doesn’t seem right. As for whether no changes being made for additional paths previous release, Editor is unsure why this is critical. Features are captured in the best possible way given the agreements and circumstances. Editor is unsure what circumstances that resulted in updates of prior release specification and how it relates to this particular case. Maybe there is some connection, but no one has been able to provide a “technical” reason other than it was done this way. The main technical problem that Editor see is that there is a clear conclusion in RAN1 NOT to enhance path based measurements. No companies has provided information on how to capture the agreed feature while not changing the path base measurements. Even if we set aside the technical difference between path versus samples, the existing reported path is limited to be smaller than what Nt’ can be. It is unclear to the Editor what companies are proposing when feature can be implemented with enhancements of additional paths, when RAN1 agreed not to change this. RAN1#120bis: Conclusion For channel measurement type (A) (i.e., path-based measurement), it is not necessary to introduce enhancements on the number of reported paths in Rel-19.. On the configuration of Nt, Nt’, and k, Editor will update this to given by higher layers. This should be a neutral term used throughout the specification that does not imply gNB must follow information provided by the LMF. gNB can choose to provide this to lower layers that are different from what was provided. All the comments so far do not seem to be concerns on the technical content and nature of the description, but rather an opinion of preference on where and how to best capture the agreed feature. Editor apologizes if the description is unsatisfactory for some companies, but there doesn’t seem to be real concern other than companies think there are alternative ways to describe the feature. If needed, maybe this is something that should be addressed by the Chair. |

### Round#3 Discussion

Editor has provided updated version of the draft CR for TS38.215 containing introduction of Rel-19 AI/ML air-interface features in

https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_120b/Inbox/drafts/9(NR_R19)/38.215%20draft%20CRs/NR_AIML_air/R1-250xxxx%20TS38215_Rel19_CRdraft_AIML_v02.docx

The following is a summary of changes made based on comments received:

- Changing ‘configured’ to ‘given to higher layers’

From editor’s understanding the draft CR seems to be stable from technical content perspective. However, it is noted that at least three companies seems to think there are alternative methods to capture RAN1 agreements in different specifications (possibly in 38.455 maintained by RAN3). After consultation with feature lead, Editor is unclear as to how to capture the RAN1 agreements in 38.455, not just because of technical differences but also due to prior RAN1 conclusion (to not enhance report paths). From editor’s understanding the concerns seem to be opinions on a better method to capture the content, and not related to technical correctness of the description. Therefore, Editor has put together the draft CR that Editor believes best captures the technical content of the agreements so far. In fact, when Editor was first preparing the draft CR, Editor has no consultation with (internally) Intel’s AI/ML delegates nor with any other delegates from other companies. Editor mechanically drafted CR based on agreements available.

With this all said, Editor notes that the opinion of the three companies seem to be strong. Based on this there really seems to few possibilities.

(1) Endorse the draft CR, and have RAN1 agree to move on feature description in the draft CR. (Basically make an executive decision that draft CR of 38.215 is way to capture the RAN1 agreements). This will potentially avoid future discussions in RAN1 about where and how to capture the feature and save time for more technical discussion necessary.

(2) Conclude the other way around to NOT endorse the draft CR and have RAN1 agree to capture the timing and power measurements in other specifications. This will also potentially avoid future discussion in RAN1 about where and how to capture the feature and save time for more technical discussion.

(3) Technical endorse the draft CR in principle, RAN1 to work out if changes are needed (including removing of the measurement definition and defining them somewhere else) in the next meeting as needed. Since there is still time for the final CR approval in next meeting, having the technical content available for companies to review will likely help companies with further discussions needed.

(4) Take the draft CR as information in this meeting. Further discuss how to proceed in RAN1.

Editor askes to take one of the four options listed above. Among all options Editor thinks Option 4 might be the least productive given that it guarantees that companies will debate over the best method for capturing agreements. A work that is typically tasked to the Editor, but companies wanting to debate in RAN1 sessions.

Editor asks Chairman guidance on the matter.

##### Company Comments for updated version of draft CR & proposal from Editor

| Company | Comments |
| --- | --- |

### Summary of Discussion

Chairman has noted the draft CR. It was suggested to further discuss the draft CR in the next RAN1 meeting.