3GPP TSG RAN WG1 Meeting #120bis		R1-2503491

# Wuhan, China, April 7th – 11th, 2025

Agenda item:       9

Source: 	Nokia

Title: 	Summary of the discussion concerning the UEIBM draft CR for Release 19

Document for:     Discussion

# 1	Discussion – first round

In this document we summarize initial views on the Rel-19 UEIBM draft CR.

Comments in this section are based on version 4 and updated versions of the draft CR!

According to the schedule, this draft CR needs to be endorsed by April 30!

| Company | Comments | Editor reply/Notes |
| --- | --- | --- |
| Samsung | Thank you Mihai for your great efforts in putting up a nice draft CR! We have the following comments for your consideration (again, we prefer to first stabilize event-2): Comment #1 on time window determination We think that the descriptions of the time window may need to be further clarified. First of all, we suggest to clarify that starting of a time window (with a fixed length T_window) specific to a new beam RS of a CSI report configuration should correspond to start of a transmission occasion of the new beam RS that satisfies the event. Furthermore, multiple time windows (if exist) for the same new beam RS are not allowed to be overlapped in time domain – this is according to the restriction of “the timer for the new beam is not running” in the agreement. We provide one conceptual example in the figure below (Window #1 and Window #2 are two non-overlapping time windows specific to the same new beam RS #1 before a corresponding first PUCCH TO). However, the current text “if eventDetectionTimeWindowLength-r19 is provided and the time window for such reference signal has not started yet, the UE starts the time window for such reference signal.” may not accurately reflect the agreement. According to the yellow highlighted part in the current text, the UE cannot start Window #2 in the above figure after Window #1, which should not be the intention. We suggest the following updates according to the above. “For a UE configured with a CSI-ReportConfig with eventType-r19, and with dl-OrJointTCI-StateList or ul-TCI-StateList for periodic reference signals [with the same periodicity], when an event instance is determined for the UE determines a time window of a length eventDetectionTimeWindowLength-r19, if provided, where the time window starts at the beginning of a transmission occasion of a reference signal configured by the newBeamResourceSet-r19 (as described in the following clauses) for which an event instance is determined, wherein the time window does not overlap in time domain with another time window for the same reference signal if eventDetectionTimeWindowLength-r19 is provided and the time window for such reference signal has not started yet, the UE starts the time window for such reference signal.” Comment #2 on conditions for first PUCCH transmission First of all, it should clarify that the number of event instances should be within a window if determined. Secondly, there can be multiple first PUCCHs after the time window and the UE should transmit the earliest one for latency reduction. In addition, “format 0/1” can be removed for simiplicity which is already clarified in TS 38.213. We suggest the following updates according to the above. “When the number of event instances determined for such the same reference signal within the time window, if any, is greater than or equal to 1 or to eventInstanceCount-r19, if provided, or 1 otherwise, the UE transmits a PUCCH format 0 or format 1 in the an earliest PUCCH resource configured by firstPUCCHResourceConfig-UEIBR-r19 in the CSI-ReportConfig starting after the time window.” Comment #3 on event instance counting The details of counting may not need to be explicity specified. Instead of specifying that the counter is counted from 0 to N which is not captured in the CR yet, we can only focus on the total number of instances as described in “When the number of event instances determined for such reference signal” Details related to TCI state update need further clarification at least for the following. - When resetting would occur, e.g., the time when the UE detects the DCI or the time when the new beam is applied. - Whether the time window should be redetermined - Whether the event cannot be considered as triggered when detecting TCI state update in a time window. Given the above, we suggest to remove the texts below for now (can revisit it after the above missing details are clarified): The counting of the event instances for such reference signal is restarted: - at the end of the time window associated to such reference signal, or - if the reference signal in the indicated TCI state or the SSB which is QCLed with the reference signal in the indicated TCI state is updated. Comment #4 editorial suggestions on the descriptions for reference signals The procedure is related to multiple reference signals, we suggest to use “first reference signal” for new beam and “second reference signal” for the current beam to improve the readability as well as simplify the descriptions by removing duplicated restrictions. For example, the removed text below is already clarified in clause 5.2.1.5.4.1 UE Initiated CSI reporting. “For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType-r19 set to ‘event2’, an event instance is determined for a first reference signal configured by the newBeamResourceSet-r19 (in a same CC of CSI-ReportConfig, or provided by carrier in the CSI-ReportConfig) if the L1-RSRP value determined for a first transmission occasion of the first reference signal is an eventThreshold-r19 greater than the L1-RSRP of a second transmission occasion of a second reference signal defined as - the reference signal in the indicated TCI state [in a same CC of CSI-ReportConfig, or the CC indicated by ServCellIndex in the CSI-ReportConfig, if newBeamResourceSet-r19 is a NZP-CSI-RS-ResourceSet configured with repetition, else - the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state (in a same CC of CSI-ReportConfig or in the CC indicated by ServCellIndex in the CSI-ReportConfig), if newBeamResourceSet-r19 is a CSI-SSB-ResourceSet, ” | Comment#1: Some changes are done on this issue based also on Vivo’s Comment#1. On the other hand, regarding your observation on the starting time of the window, I prefer to keep the current text, as the agreement says “an Event-2 instance for a new beam is obtained at the time t”, without explicitly detailing if time t is the time of a transmission occasion of that new beam or something else, e.g., the time at which such beam is actually measured or must be measured (as a requirement) at the UE. Comment#2: Captured explicitly that the counting is within the time window. Regarding the first PUCCH usage however, we do not have agreement saying that the first PUCCH should be after the end of the window: if eventInstanceCount-r19 are counted within a time window, and there is a first PUCCH occasion before the end of the window, a UE can use that to initiate the reporting. Comment#3: Indeed, we do not need to provide too many details on how a counter should be actually implemented, therefore I removed the first bullet point, it is enough to say that the counting is within the window. However, we need to keep the second bullet that refers to the agreement that counting must be reset when the beam associated to the indicated TCI state is updated. Comment#4: Thank you, however, I prefer to keep the current wording to describe each reference signal that is a bit lengthier but clearer: writing it in a more explicit way (e.g., reference signal in the indicated TCI state) is clearer than introducing first, second, third, and so on. |
| vivo | Thanks Mihai for the draft, please find few comments below mainly on Event-2. Comment#1 on condition of the start of time window As mentioned by Samsung, the description of “the time window for such reference signal” in the draft CR is not equal to “corresponding timer is not running” in the agreement. Besides the revision from Samsung, another suggestion is introducing an additional condition as in red text below. For a UE configured with a CSI-ReportConfig with eventType-r19, and with dl-OrJointTCI-StateList or ul-TCI-StateList for periodic reference signals [with the same periodicity], when an event instance is determined for a reference signal configured by the newBeamResourceSet-r19 (as described in the following clauses), if eventDetectionTimeWindowLength-r19 is provided and the time window for such reference signal has not started yet or has ended, the UE starts the time window for such reference signal.. Comment#2 on resetting counting of event instances No agreement has been achieved for the first condition of resetting of counting of event instances, i.e., “at the end of the time window associated to such reference signal”. Besides, we think it is not necessary if the initial number of event instances is set to 1 at the start of the time window. Therefore, we suggest to remove the first condition. The counting of the event instances for such reference signal is restarted: - at the end of the time window associated to such reference signal, or - if the reference signal in the indicated TCI state or the SSB which is QCLed with the reference signal in the indicated TCI state is updated.. Comment#3 on candidate beam RS and new beam RS in the same CC Agreement On cross-CC beam report measurement for UE-initiated/event-driven beam reporting, regarding Event-2, the following is supported - The current beam RS and new beam RS(s) are in the same CC - The above does NOT imply to preclude the cross-CC TCI indication. - The CC on which the indicated TCI state is applied, and the CC in which new beam RS(s) are can be the same or different. - FFS: Whether/how to introduce an RRC parameter to indicate the CC on which the indicated TCI state is applied.. According to the above agreement, current beam RS and new beam RS need to be configured in the same CC for cross-CC UEIBR and the current beam RS can be CSI-RS or SSB. However, only the reference signal in the indicated TCI state, i.e., CSI-RS is captured in the draft CR. To align with the agreement, SSB should be added as red text. The UE does not expect that the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state is in a different CC from the reference signals configured by newBeamResourceSet-r19.. Comment#4 on the CC of the indicated TCI state Agreement On cross-CC beam report measurement for UE-initiated/event-driven beam reporting, regarding Event-2, the following is supported - The current beam RS and new beam RS(s) are in the same CC - The above does NOT imply to preclude the cross-CC TCI indication. - The CC on which the indicated TCI state is applied, and the CC in which new beam RS(s) are can be the same or different. FFS: Whether/how to introduce an RRC parameter to indicate the CC on which the indicated TCI state is applied.. According to the above agreement, the CC on which the indicated TCI state is applied can be the same as the CC in which new beam RS(s). The CC of the new beam RS(s) can be the same as the CC of the CSI-ReportConfig or indicated by the carrier. However, only the CC of the CSI-ReportConfig is captured in the draft CR. To capture the agreement completely, the following revision in red text is suggested. For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType-r19 set to ‘event2’, an event instance is determined for a reference signal configured by the newBeamResourceSet-r19 (in a same CC of CSI-ReportConfig, or provided by carrier in the CSI-ReportConfig) if the L1-RSRP value determined for a transmission occasion of the reference signal is an eventThreshold-r19 greater than the L1-RSRP of - the reference signal in the indicated TCI state in a same CC of the reference signal configured by newBeamResourceSet-r19 CSI-ReportConfig, or the CC indicated by ServCellIndex in the CSI-ReportConfig, if newBeamResourceSet-r19 is a NZP-CSI-RS-ResourceSet configured with repetition, else - the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state (in a same CC of the reference signal configured by newBeamResourceSet-r19 CSI-ReportConfig or in the CC indicated by ServCellIndex in the CSI-ReportConfig), if newBeamResourceSet-r19 is a CSI-SSB-ResourceSet,. Comment#5 on editorial issues of report content For Event-2, only one absolute L1-RSRP measurement value would be reported. Therefore, the following revision should be adopted. The UE, after transmitting PUCCH format 0 or 1, reports in a single reporting instance nrofReportedRS-UEIBR-r19 CRIs or SSBRIs, their absolute L1-RSRPs or differential L1-RSRPs and their [indications whether they satisfy the event] when [RRC parameter], eventDetectionTimeWindowLength-r19, and eventInstanceCount-r19 are configured and nrofReportedRS-UEIBR-r19 is set to a value greater than 1, corresponding to reference signals provided by the newBeamResourceSet-r19 that comprise at least one reference signal that satisfies the event, and, when enabledCurrentBeamReport-r19 is configured, the differential L1-RSRP correponding to the reference signal in the indicated TCI state, or to the SSB which is QCLed with the reference signal in the indicated TCI state:. | Comment#1: Thank you, captured. Comment#2: Thank you, captured, I agree, and commented on the same issue also in Samsung Comment#3. Comment#3: Thank you, captured. Comment#4: Good observation, captured. Comment#5: Thank you, captured (btw, in your text I also observed a typo in “corresponding” that I fixed) |
| ZTE | Thanks Mihai so much for your great effort on the draft CR. Please find our comments of Rel-19 UEIBM as follows. Comment#1 Based on the agreement reached in RAN1#120bis, Option-4 is adopted as follows, where the time window is agreed to be a timer. Agreement Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, support Option-4. - Option-4: If an Event-2 instance for a new beam is obtained at the time t and the timer for the new beam is not running, UE starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window) - Note: Timer is new beam specific. - Note: T_window is the agreed time window parameter for measurement. - Introduce separate UE capability to limit the number of timers. There is only 1 timer per new beam. Above agreement is captured in RAN1 specifications.. Therefore, we suggest the following revision regarding time window for alignment. In addition, UE behaviour of counting the number of event instances are unclear. Either the timer is running or not running, the counter should be incremented by one, hence we suggest to add the counting behaviour accordingly. Proposed Change 5.2.1.5.4 UE Initiated reporting 5.2.1.5.4.1 UE Initiated CSI reporting For a UE configured with a CSI-ReportConfig with eventType-r19, and with dl-OrJointTCI-StateList or ul-TCI-StateList for periodic reference signals [with the same periodicity], when an event instance is determined for a reference signal configured by the newBeamResourceSet-r19 (as described in the following clauses), if eventDetectionTimeWindowLength-r19 is provided, the UE incerments the value of the counter by one for such reference signal, and if the timer for such reference signal is not running, the UE starts the timer for such reference signal.. Comment#2 UE behaviour of restarting the counting is unclear, we suggest the following revision to align with the agreement. Furthermore, the first condition to reset the counting is still to be discussed so far, we suggest to remove it for now. Proposed Change The counting of the event instances for such reference signal is set to be zero: - if the reference signal in the indicated TCI state or the SSB which is QCLed with the reference signal in the indicated TCI state is updated.. Comment#3 We suggest the following revision to clarify that the PUCCH format 0/1 corresponds to the UEI-CSI report. Proposed Change The UE, after transmitting PUCCH format 0 or 1 corresponding to a CSI-ReportConfig with eventType-r19, reports in a single reporting instance nrofReportedRS-UEIBR-r19 CRIs or SSBRIs, their absolute L1-RSRPs or differential L1-RSRPs and their [indications whether they satisfy the event] when [RRC parameter], eventDetectionTimeWindowLength-r19, and eventInstanceCount-r19 are configured and nrofReportedRS-UEIBR-r19 is set to a value greater than 1, corresponding to reference signals provided by the newBeamResourceSet-r19 that comprise at least one reference signal that satisfies the event, and, when enabledCurrentBeamReport-r19 is configured, the differential L1-RSRP correponding to the reference signal in the indicated TCI state, or to the SSB which is QCLed with the reference signal in the indicated TCI state:. Comment #4 Given that it has not been agreed/discussed that multiple UEI-CSI report configurations associated with the same first PUCCH resource should be configured with the same event type, we suggest to remove the corresponding restriction as of now. Proposed Change 5.2.1.5.4.1d UE Initiated CSI reporting for multiple CSI configurations For a UE is configured with multiple CSI-ReportConfig all with same higher layer parameters firstPUCCHResourceConfig-UEIBR-r19 and reportTransmissionMode-r19, the UE expects - all multiple CSI-ReportConfig in a same CC, and - the same CSI trigger state if reportTransmissionMode-r19 is configured as ‘ModeA’, else - the same configuredResourceForSecondChannelOfModeB-r19 if reportTransmissionMode-r19 is configured as ‘ModeB’, and when reporting in a single reporting instance nrofReportedRS-UEIBR-r19 CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet-r19 in a CSI-ReportConfig that satisfies the event, the CSI report contains the corresponding CSI-ReportConfigId and is zero padded to a fixed payload size (when needed), with the fixed payload size given by the maximum payload size among all the multiple CSI-ReportConfig.. | Comment#1: Thank you, but I think “timer” and “time window” can be seen as synonyms here, so I would prefer the use of time-window for now for now… Regarding “the increasing by one”, I prefer to avoid including such details in the spec, we do not have agreement saying that the counting is from 0 to N (increasing by 1) or from N to 0 (decreasing by 1) or even from 0 to 2N (increasing by 2). Comment#2: Thank you, captured. Comment#C3: Thank you, captured. Comment#4: Thank you, added now square brackets there. However, this issue of cross-event operations should be discussed in the upcoming meeting, because the agreement for the case of multiple CSI report configurations associated to a same first PUCCH explicitly say that those CSI report configurations should all be associated to the same Event-2. |
| MediaTek | Thank editor for your great effort on CR drafting. We have the following comments: Comment 1 (starting of time window): We have different view from Samsung. We think that current version is clear that a time window for a RS will start only if there is no running time window for the RS, such that there never have overlapping time windows for the same RS in time series. Comment 2 (triggering evaluation): On the following paragraph: For a UE configured with a CSI-ReportConfig with eventType-r19, and with dl-OrJointTCI-StateList or ul-TCI-StateList for periodic reference signals [with the same periodicity], when an event instance is determined for a reference signal configured by the newBeamResourceSet-r19 (as described in the following clauses), if eventDetectionTimeWindowLength-r19 is provided and the time window for such reference signal has not started yet, the UE starts the time window for such reference signal.. Firstly, we think there is no need to be conditioned on the present configuration of ul-TCI-StateList, as using dl-OrJointTCI-StateList configuration is sufficient for defining applicable case. Secondly, periodic reference signal(s) is required for new beam RS(s) and current beam RS derived from the indicated/activated TCI state, we prefer to have more specific description instead of mentioning “for periodic reference signals” in this paragraph. Thirdly, this paragraph is not applicable for Event-1. On event instance determination for Event-1, whether to determine an event instance is based on the quality of the current beam instead of the new beam(s). From our view, there is only one beam-specific window if window evaluation is configured, and that beam-specific window corresponds to the current beam only instead of new beam(s). Proposed change: For a UE configured with a CSI-ReportConfig with eventType-r19 and with dl-OrJointTCI-StateList or ul-TCI-StateList for periodic reference signals [with the same periodicity], when an event instance is determined for a reference signal configured by the newBeamResourceSet-r19 (as described in the following clauses), if eventDetectionTimeWindowLength-r19 is provided and the time window for such reference signal has not started yet, the UE starts the time window for the reference signal, where - the reference signal is a SS/PBCH block or a periodic CSI-RS resource configured by newBeamResourceSet-r19, if the higher layer parameter eventType-r19 for the CSI-ReportConfig set to ‘event2’ or ‘event7’; or - the reference signal is the periodic CSI-RS resource in the indicated TCI state or the SS/PBCH block which is QCLed with the periodic CSI-RS resource in the indicated TCI state, if the higher layer parameter eventType-r19 for the CSI-ReportConfig set to ‘event1’.. Comment 3 (triggering evaluation): For consistent description, we prefer to change the wording of “the nunmber of event instances” to “the counting of event instances” in the following paragraph. Meanwhile, current draft doesn’t reflect that only event instances within the time window are counted. Proposed change: When the counting of event instances determined for such reference signal is greater than or equal to 1 or to eventInstanceCount-r19 within the time window for such reference signal, if provided, the UE transmits PUCCH format 0 or format 1 in the PUCCH resource configured by firstPUCCHResourceConfig-UEIBR-r19 in the CSI-ReportConfig.. Comment 4 (counting resetting): The first condition for the case of window expiry is applicable only if eventDetectionTimeWindowLength-r19 is provided. Proposed change: The counting of the event instances for such reference signal is restarted: - at the end of the time window, if provided, for such reference signal, or - if the reference signal in the indicated TCI state or the SSB which is QCLed with the reference signal in the indicated TCI state is updated.. Command 5 (evaluation periodicity and restriction on RS periodicity): In RAN1#119, the following agreement was made to define the periodicity when the UE shall perform the triggering evaluation for determining the event instance. In addition, RAN1 has the consensus on that the same RS periodicity is expected for new beam RS(s) and the current beam RS. The agreement seems missing in the current draft CR. [119] Agreement Regarding for the evaluation periodicity for determining Event-2 instance [at least when DRX is not configured], at least Alt-1 is supported for the single-CC beam reporting (for case that the CSI report configuration and RS for the current beam and new beam(s) are in the same CC). - Alt-1: The periodicity of the current beam RS should be the same as that of the new beam RS(s). - The evaluation periodicity is the same as the periodicity of the current and new beam RS(s). - Alt-2: The periodicity of the current beam RS can be different from that of the new beam RS(s) - Alt-2_1: The evaluation periodicity is the same as the periodicity of the current beam RS. - Alt-2_2: The evaluation periodicity is the same as periodicity of the new beam RS. - Alt-2_3: The evaluation periodicity is the same as shortest periodicity of the current beam RS and new beam RS(s). - Alt-2_4: The evaluation periodicity is the maximum of {X ms, shortest periodicity of the current beam RS and new beam RS(s)}. - Alt-2_5: The evaluation periodicity is the same as largest periodicity of the current beam RS and new beam RS(s). Note: There is the same periodicity for the new beam RS(s). FFS: Down-selection among above Alt(s) for cross-CC beam reporting. Strong concerns were expressed by Huawei and CATT that if only Alt-1 is supported in the end, the feature will not be practical. Send an LS to RAN4. Final LS in R1-2410914.. | Comment#1: OK! Comment#2: Thank you, but as also observed by Huawei in Comment#9, that ul-TCI-StateList is there to clarify that the scenario assume the unified TCI framework. Regarding the applicability to Event-1, I fully share your observation that for Event-1 it is the current beam eventually that triggers the event, and I thought about a change of the CR similar to your proposal. However, such change cannot be captured based on the current agreements we have because: a) We agreed for Event-2 that the counting is per new beam; b) We agreed to extend to Event-1 and Event-7 the design for Event-2 unless otherwise agreed, and we have no agreement stating that Event-1 counting is not per new beam. The current text captures correctly mechanism and agreements, because we have a counter per new beam, but they essentially all count the same event-1 instances, i.e., when the current beam goes below the configured threshold. Comment#3: Thank you, modified that part to make sure the counting is within the window. Comment#4: Thank you, as commented in other places, removed that first condition, because eventually not needed when explicitly saying that the counting is within the window. Comment#5: Thank you, I think this is captured by the text in the following square brackets (which have not been removed yet because some companies mentioned that different periodicity is still to be discussed for cross-CC) : For a UE configured with a CSI-ReportConfig with eventType-r19, and with dl-OrJointTCI-StateList or ul-TCI-StateList for periodic reference signals [with the same periodicity] |
| Xiaomi | Comment# 1: propose the following update For a UE configured with a CSI-ReportConfig with eventType-r19, and with dl-OrJointTCI-StateList for periodic reference signals [with the same periodicity], when an event instance is determined for a reference signal configured by the newBeamResourceSet-r19 (as described in the following clauses), if eventDetectionTimeWindowLength-r19 is provided and the time window for such reference signal is not running, the UE starts the time window for such reference signal. Comment# 2: There is no such agreement, so propose the following update. The counting of the event instances for such reference signal is restarted: - if the reference signal in the indicated TCI state or the SSB which is QCLed with the reference signal in the indicated TCI state is updated. Comment# 3: based on previous agreement, propose the following update The reference signal in the indicated TCI state is the reference signal w.r.t. QCL-TypeD, if there are two QCL RSs in the indicated TCI state. And UE assumes that the QCL-Type D reference signal resource in the indicated TCI state is configured in a CSI-RS resource set configured with repetition. Comment# 4: based on previous agreement, propose the following update The reference signal with the valueOfQ-r19 highest L1-RSRP out of the activated TCI state reference signals is the reference signal w.r.t. QCL-TypeD, if there are two QCL RSs in such activated TCI state. And UE assumes that the reference signal with the valueOfQ-r19 highest L1-RSRP out of the activated TCI state reference signals is configured in a CSI-RS resource set configured with repetition. | C1: Thank you, but as also observed by Huawei in Comment#9, that ul-TCI-StateList is there to clarify that the scenario assume the unified TCI framework. But modified that text about the window as some other companies suggested that. Comment#2: Thank you, as commented in other places, removed that first condition, because eventually not needed when explicitly saying that the counting is within the window. Comment#3: Thank you, I think that there is not an agreement supporting such modification, maybe if you could point the right part of the agreement, it would be great. Comment#4: Thank you, but as mentioned above, there is no an agreement supporting such modification. |
| Ericsson | Thanks for your efforts on the draft CR. It is a difficult topic. And forgive me if I bring up comments that were part of the March version In general, I find it counter-intuitive to start with the counter/timer, and then describe the event instance definitions, since the measurement comparison is done before the timer/counter actions. But maybe you already tried the other order. A few detailed comments: Comment #1: For the RRC parameter, we skip the suffix “-rxx” if there is no risk for misunderstanding. Note that we write dl-OrJointTCI-StateList, not dl-OrJointTCI-StateList-r17. In the draft CR, the suffix -r19 occurs on almost all parameters. Comment #2: The section number is 5.2.1.5.4. Does this really fit under “Triggering/activation of CSI Reports and CSI-RS”? I feel it deserves it own section under 5.2.1. Comment #3: For a UE configured with a CSI-ReportConfig with eventType-r19, and with dl-OrJointTCI-StateList or ul-TCI-StateList for periodic reference signals [with the same periodicity], when an event instance is determined for a reference signal configured by the newBeamResourceSet-r19 (as described in the following clauses), if eventDetectionTimeWindowLength-r19 is provided and the time window for such reference signal has not started yet, the UE starts the time window for such reference signal. - What does “for periodic reference signals” mean? Is this an implicit configuration restriction of the RSs in newBeamResourceSet? It’s very difficult to realize that when performing the RRC configuration - A time window cannot be started. The agreement says that a timer is started - please capture according to the agreement. - There is no description when the counter is incremented. Please add that Comment #4: When the number of event instances determined for such reference signal is greater than or equal to 1 or to eventInstanceCount-r19, if provided, the UE transmits PUCCH format 0 or format 1 in the PUCCH resource configured by firstPUCCHResourceConfig-UEIBR-r19 in the CSI-ReportConfig. This was renamed PUCCHResource in the latest RRC parameter list. Comment #5: For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType-r19 set to ‘event2’, an event instance is determined for a reference signal configured by the newBeamResourceSet-r19 (in a same CC of CSI-ReportConfig, or provided by carrier in the CSI-ReportConfig) if the L1-RSRP value determined for a transmission occasion of the reference signal is an eventThreshold-r19 greater than the L1-RSRP of - the reference signal in the indicated TCI state [in a same CC of CSI-ReportConfig, or the CC indicated by ServCellIndex in the CSI-ReportConfig, if newBeamResourceSet-r19 is a NZP-CSI-RS-ResourceSet configured with repetition, else - the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state (in a same CC of CSI-ReportConfig or in the CC indicated by ServCellIndex in the CSI-ReportConfig), if newBeamResourceSet-r19 is a CSI-SSB-ResourceSet, - “in a same CC of CSI-ReportConfig, or provided by carrier in the CSI-ReportConfig” – this is not needed, it is like legacy - ServCellIndex is the “type”, the field name is “carrier”. The same text appears for event-1 and event-7. Comment #6: For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType-r19 set to ‘event2’, an event instance is determined for a reference signal configured by the newBeamResourceSet-r19 (in a same CC of CSI-ReportConfig, or provided by carrier in the CSI-ReportConfig) if the L1-RSRP value determined for a transmission occasion of the reference signal is an eventThreshold-r19 greater than the L1-RSRP of - the reference signal in the indicated TCI state [in a same CC of CSI-ReportConfig, or the CC indicated by ServCellIndex in the CSI-ReportConfig, if newBeamResourceSet-r19 is a NZP-CSI-RS-ResourceSet configured with repetition, else - the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state (in a same CC of CSI-ReportConfig or in the CC indicated by ServCellIndex in the CSI-ReportConfig), if newBeamResourceSet-r19 is a CSI-SSB-ResourceSet, We have used the term “current beam” for this during the work, and we should not use that. However, this text will occur in several places, so it would make the text easier to read if we actually had a name for it. How about “serving reference signal”? This could be defined early in the section, and in all the other places, we could simply write “the UE includes the RSRP of the serving reference signal”. Comment #7: The UE, after transmitting PUCCH format 0 or 1, reports in a single reporting instance nrofReportedRS-UEIBR-r19 CRIs or SSBRIs, their absolute L1-RSRPs or differential L1-RSRPs and their [indications whether they satisfy the event] when [RRC parameter], eventDetectionTimeWindowLength-r19, and eventInstanceCount-r19 are configured and nrofReportedRS-UEIBR-r19 is set to a value greater than 1, corresponding to reference signals provided by the newBeamResourceSet-r19 that comprise at least one reference signal that satisfies the event, and, when enabledCurrentBeamReport-r19 is configured, the differential L1-RSRP correponding to the reference signal in the indicated TCI state, or to the SSB which is QCLed with the reference signal in the indicated TCI state: The first highlighted text makes the text more difficult to read. It is also unnecessary, the exact report content and the related is (in this case should be) described in 38.212. The current text actually suggests that RSRP is only reported when “[RRC parameter], eventDetectionTimeWindowLength-r19, and eventInstanceCount-r19 are configured“, and that is incorrect The same text appears for event-7. Comment #8: The UE, after transmitting PUCCH format 0 or 1, reports in a single reporting instance nrofReportedRS-UEIBR-r19 CRIs or SSBRIs, their absolute L1-RSRPs or differential L1-RSRPs and their [indications whether they satisfy the event] when [RRC parameter], eventDetectionTimeWindowLength-r19, and eventInstanceCount-r19 are configured and nrofReportedRS-UEIBR-r19 is set to a value greater than 1, corresponding to reference signals provided by the newBeamResourceSet-r19 that comprise at least one reference signal that satisfies the event, and, when enabledCurrentBeamReport-r19 is configured, the differential L1-RSRP correponding to the reference signal in the indicated TCI state, or to the SSB which is QCLed with the reference signal in the indicated TCI state: For the legacy reporting, there is no corresponding reference to the list of reference signals. Do we need it here? If we do, the shorter statement “from newBeamResourceSet” is easier to understand. The same text appears for event-1 and event-7. Comment #9: The UE, after transmitting PUCCH format 0 or 1, reports in a single reporting instance nrofReportedRS-UEIBR-r19 CRIs or SSBRIs, their absolute L1-RSRPs or differential L1-RSRPs and their [indications whether they satisfy the event] when [RRC parameter], eventDetectionTimeWindowLength-r19, and eventInstanceCount-r19 are configured and nrofReportedRS-UEIBR-r19 is set to a value greater than 1, corresponding to reference signals provided by the newBeamResourceSet-r19 that comprise at least one reference signal that satisfies the event, and, when enabledCurrentBeamReport-r19 is configured, the differential L1-RSRP correponding to the reference signal in the indicated TCI state, or to the SSB which is QCLed with the reference signal in the indicated TCI state: What does “that” refer to? If we streamline the text, we get this: The UE reports in a single reporting instance nrofReportedRS-UEIBR-r19 CRIs or SSBRIs, their absolute L1-RSRPs or differential L1-RSRPs and their indications corresponding to reference signals provided by the newBeamResourceSet-r19 that comprise at least one reference signal that satisfies the event. The report contains a list of CRIs, RSRPs and indications. How can that comprise of a reference signal? I know that it should mean that at least one of the reference signals in the report satisfies the event, but it does not say that now. The same text appears for event-7. Comment #10: The UE, after transmitting PUCCH format 0 or 1, reports in a single reporting instance nrofReportedRS-UEIBR-r19 CRIs or SSBRIs, their absolute L1-RSRPs or differential L1-RSRPs and their [indications whether they satisfy the event] when [RRC parameter], eventDetectionTimeWindowLength-r19, and eventInstanceCount-r19 are configured and nrofReportedRS-UEIBR-r19 is set to a value greater than 1, corresponding to reference signals provided by the newBeamResourceSet-r19 that comprise at least one reference signal that satisfies the event, and, when enabledCurrentBeamReport-r19 is configured, the differential L1-RSRP correponding to the reference signal in the indicated TCI state, or to the SSB which is QCLed with the reference signal in the indicated TCI state: - on a PUSCH indicated by the DCI format 0_1/0_2 in a PDCCH reception after the end of the PUCCH if reportTransmissionMode-r19 is configured as ‘ModeA’ and the CSI trigger state associated with the CSI-ReportConfig is indicated in the CSI request field in the DCI format 0_1/0_2, or - on a type 1 CG-PUSCH configured by configuredResourceForSecondChannelOfModeB-r19 in the same CC as the corresponding CSI-ReportConfig which is the first available transmission occasion numOfSymbols-ModeB-r19 symbols after the end of the PUCCH if reportTransmissionMode-r19 is configured as ‘ModeB’, where the periodicity of the PUCCH resource and type 1 CG-PUSCH resource is the same, and the CG-PUSCH does not carry UL-SCH. Here it is unclear to which PUCCH resource this refers. Is it any PUCCH? The same text appears for event-1 and event-7. Comment #11: The UE, after transmitting PUCCH format 0 or 1, reports in a single reporting instance nrofReportedRS-UEIBR-r19 CRIs or SSBRIs, their absolute L1-RSRPs or differential L1-RSRPs and their [indications whether they satisfy the event] when [RRC parameter], eventDetectionTimeWindowLength-r19, and eventInstanceCount-r19 are configured and nrofReportedRS-UEIBR-r19 is set to a value greater than 1, corresponding to reference signals provided by the newBeamResourceSet-r19 that comprise at least one reference signal that satisfies the event, and, when enabledCurrentBeamReport-r19 is configured, the differential L1-RSRP correponding to the reference signal in the indicated TCI state, or to the SSB which is QCLed with the reference signal in the indicated TCI state: - on a PUSCH indicated by the DCI format 0_1/0_2 in a PDCCH reception after the end of the PUCCH if reportTransmissionMode-r19 is configured as ‘ModeA’ and the CSI trigger state associated with the CSI-ReportConfig is indicated in the CSI request field in the DCI format 0_1/0_2, or - on a type 1 CG-PUSCH configured by configuredResourceForSecondChannelOfModeB-r19 in the same CC as the corresponding CSI-ReportConfig which is the first available transmission occasion numOfSymbols-ModeB-r19 symbols after the end of the PUCCH if reportTransmissionMode-r19 is configured as ‘ModeB’, where the periodicity of the PUCCH resource and type 1 CG-PUSCH resource is the same, and the CG-PUSCH does not carry UL-SCH. This was renamed to configuredPUSCHResourceOfModeB-r19 in the RRC parameter list The same text appears for event-1 and event-7. Comment #12: The UE, after transmitting PUCCH format 0 or 1, reports in a single reporting instance nrofReportedRS-UEIBR-r19 CRIs or SSBRIs, their absolute L1-RSRPs or differential L1-RSRPs and their [indications whether they satisfy the event] when [RRC parameter], eventDetectionTimeWindowLength-r19, and eventInstanceCount-r19 are configured and nrofReportedRS-UEIBR-r19 is set to a value greater than 1, corresponding to reference signals provided by the newBeamResourceSet-r19 that comprise at least one reference signal that satisfies the event, and, when enabledCurrentBeamReport-r19 is configured, the differential L1-RSRP correponding to the reference signal in the indicated TCI state, or to the SSB which is QCLed with the reference signal in the indicated TCI state: - on a PUSCH indicated by the DCI format 0_1/0_2 in a PDCCH reception after the end of the PUCCH if reportTransmissionMode-r19 is configured as ‘ModeA’ and the CSI trigger state associated with the CSI-ReportConfig is indicated in the CSI request field in the DCI format 0_1/0_2, or - on a type 1 CG-PUSCH configured by configuredResourceForSecondChannelOfModeB-r19 in the same CC as the corresponding CSI-ReportConfig which is the first available transmission occasion numOfSymbols-ModeB-r19 symbols after the end of the PUCCH if reportTransmissionMode-r19 is configured as ‘ModeB’, where the periodicity of the PUCCH resource and type 1 CG-PUSCH resource is the same, and the CG-PUSCH does not carry UL-SCH. What does “which” refer to? If we clean the text a bit we end up with “on a type 1 CG-PUSCH which is the first available transmission occasion” which I don’t think makes sense. The same text appears for event-1 and event-7. Comment #13: For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType-r19 set to ‘event1’, an event instance is determined for a reference signal configured by the newBeamResourceSet-r19 (in a same CC of CSI-ReportConfig, or provided by carrier in the CSI-ReportConfig) if the L1-RSRP value determined for a transmission occasion of - the reference signal in the indicated TCI state (in a same CC of CSI-ReportConfig, or in the CC indicated by ServCellIndex in the CSI-ReportConfig), if newBeamResourceSet-r19 is a NZP-CSI-RS-ResourceSet configured with repetition, else - the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state (in a same CC of CSI-ReportConfig, or in the CC indicated by ServCellIndex in the CSI-ReportConfig), if newBeamResourceSet-r19 is a CSI-SSB-ResourceSet, is lower than eventThresholdEvent1-r19. The RRC parameter name is eventThreshold-Event1-r19 Comment #14: For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType-r19 set to ‘event7’, an event instance is determined for a reference signal configured by the newBeamResourceSet-r19 (in a same CC of CSI-ReportConfig, or provided by carrier in the CSI-ReportConfig) if the L1-RSRP value determined for a transmission occasion of the reference signal is an eventThresholdEvent7-r19 greater than the L1-RSRP of - the reference signal with the valueOfQ-r19 highest L1-RSRP out of the activated TCI state reference signals (in a same CC of CSI-ReportConfig, or in the CC indicated by ServCellIndex in the CSI-ReportConfig), if newBeamResourceSet-r19 is a NZP-CSI-RS-ResourceSet configured with repetition, else - the SS/PBCH block which is QCLed with the reference signal with the valueOfQ-r19 highest L1-RSRP out of the activated TCI state reference signals (in a same CC of CSI-ReportConfig, or in the CC indicated by ServCellIndex in the CSI-ReportConfig), if newBeamResourceSet-r19 is a CSI-SSB-ResourceSet. The RRC parameter name is eventThreshold (same as for event-2) Comment #15: For a UE is configured with multiple CSI-ReportConfig all with same higher layer parameters eventType-r19, firstPUCCHResourceConfig-UEIBR-r19, and reportTransmissionMode-r19, the UE expects - all multiple CSI-ReportConfig in a same CC, and - the same CSI trigger state if reportTransmissionMode-r19 is configured as ‘ModeA’, else - the same configuredResourceForSecondChannelOfModeB-r19 if reportTransmissionMode-r19 is configured as ‘ModeB’, and I don’t recognize this: we can configure CSI-ReportConfigs on different CCs. I assume that we try to capture the configuration restrictions. Maybe something like this: For a UE is configured with multiple CSI-ReportConfig on one serving cell with same higher layer parameters eventType, PUCCHResource, and reportTransmissionMode, the UE expects - the CSI-ReportConfig are associated with same CSI trigger state if reportTransmissionMode-r19 is configured as ‘ModeA’, - the same configuredResourceOfModeB-r19 if reportTransmissionMode-r19 is configured as ‘ModeB’. Comment #16: For a UE is configured with multiple CSI-ReportConfig all with same higher layer parameters eventType-r19, firstPUCCHResourceConfig-UEIBR-r19, and reportTransmissionMode-r19, the UE expects - all multiple CSI-ReportConfig in a same CC, and - the same CSI trigger state if reportTransmissionMode-r19 is configured as ‘ModeA’, else - the same configuredResourceForSecondChannelOfModeB-r19 if reportTransmissionMode-r19 is configured as ‘ModeB’, and when reporting in a single reporting instance nrofReportedRS-UEIBR-r19 CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet-r19 in a CSI-ReportConfig that satisfies the event, the CSI report contains the corresponding CSI-ReportConfigId and is zero padded to a fixed payload size (when needed), with the fixed payload size given by the maximum payload size among all the multiple CSI-ReportConfig. The report content is (in this case should be) captured in 38.212. Suggest to remove. | Comment#1: Thank you, we have updated the RRC parameters with the most recent names available, that still however include in our understanding many suffix -r19. We can work on this later… Comment#2: Thank you, I think current section seems ok, as this UE-initiated reporting can be seen as an additional way to aperiodic, semi-persistent and periodic reporting, that was the intention! Comment#3: That “for periodic reference signals [with the same periodicity]” refer to the current agreement from RAN1#119 stating that “The periodicity of the current beam RS should be the same as that of the new beam RS(s)”. In my view “timer” and “time window” can be seen as synonyms here, we can keep time-window for now. Regarding increment of the counter, I prefer to avoid including such details, also because we do not have agreements saying that the counting is from 0 to N (increasing by 1) or from N to 0 (decreasing by 1) or even from 0 to 2N (increasing by 2). Comment#4: Thank you, captured, although in my understanding the last agreed name is “PUCCHResource-r19” Comment#5: Thank you. On the first point, I prefer to keep it for clarity, as we have explicit agreement for that. On the second bullet point, good observation, although the right parameter in my understanding is “CCofIndicatedTCI-r19”. Comment#6: Thank you, that is a good observation, I also thought about that. Did not implement it however because it would be better to introduce a new term that applies for all events (and in Event-7 the current beam is not associated to the indicated TCI state). Have not implemented any change yet, but I keep this comment in mind for the next iteration, any suggestion would be welcome. Comment#7: Thank you. It is true that the details of the reporting content are in 38.212, but think for better clarity also here we need to capture some information. Slightly changed the order to avoid ambiguity. Comment#8: Thank you, but as explained in Comment#9 I think the current formulation is clearer. Comment#9: Thank you, I think the text is clear, the list of reported RSs (which is a list or CRI or SSBRI) corresponds to new beam RSs that include/comprise one that satisfies the event. Comment#10: Thank you, updated to “transmitted PUCCH” for improving clarity. Comment#11: Thank you, captured. Comment#12: Thank you, updated the text here to improve clarity. Comment#13: Thank you, captured. Comment#14: Thank you, captured. Comment#15: Thank you, captured. Comment#16: Thank you, but as commented above, although all details are indeed in 38.212, I think it is ok to cover also here some information about the report content. |
| Google | Thank you Mihai for the nice draft. Please find our comments/proposals below. Comment 1 Regarding below quotation, for the yellow highlight part, we suggest replacing “time window” with “timer”, which better aligns the latest agreement on implementation of the time window. We can use wordings like “if a timer for such reference signal has not started yet, the UE starts the timer for such reference signal, where the length of the timer is provided by eventDetectionTimeWindowLength-r19” In addition, the cyan highlight part could mislead the reader that dl-OrJointTCI-StateList or ul-TCI-StateList is provided for periodic reference signals, which is not the intention. Some reordering of this sentence is preferable. For a UE configured with a CSI-ReportConfig with eventType-r19, and with dl-OrJointTCI-StateList or ul-TCI-StateList for periodic reference signals [with the same periodicity], when an event instance is determined for a reference signal configured by the newBeamResourceSet-r19 (as described in the following clauses), if eventDetectionTimeWindowLength-r19 is provided and the time window for such reference signal has not started yet, the UE starts the time window for such reference signal.. Proposal 1 We thus suggest the following revisions. For a UE configured with a CSI-ReportConfig with eventType-r19, and with dl-OrJointTCI-StateList or ul-TCI-StateList for periodic reference signals [with the same periodicity] and configured with dl-OrJointTCI-StateList or ul-TCI-StateList, when an event instance is determined for a reference signal configured by the newBeamResourceSet-r19 (as described in the following clauses), if eventDetectionTimeWindowLength-r19 is provided and the time window for such reference signal has not started yet, the UE starts the time window for such reference signal if a timer for such reference signal has not started yet, the UE starts the timer for such reference signal, where the length of the timer is provided by eventDetectionTimeWindowLength-r19.. Comment 2 For below quotation, we have some editorial suggestions. First, there is only one absolute RSRP for new beam RS to be reported, hence no need to add “s” (Similar issue appear in corresponding part in Event 1 and Event 7). Second, regarding the sentence “[indications whether they satisfy the event]”, we suggest some more wordings added to make them clearer, since “they” would be too general. For example, “[indications on whether referecnes signal(s) indicated by reported CRIs or SSBRIs satisfy the event]”. This change can also apply for corresponding part in Event 7. 5.2.1.5.4.1a UE Initiated CSI reporting for event 2 … The UE, after transmitting PUCCH format 0 or 1, reports in a single reporting instance nrofReportedRS-UEIBR-r19 CRIs or SSBRIs, their absolute L1-RSRPs or differential L1-RSRPs and their [indications whether they satisfy the event] when [RRC parameter], eventDetectionTimeWindowLength-r19, and eventInstanceCount-r19 are configured and nrofReportedRS-UEIBR-r19 is set to a value greater than 1, corresponding to reference signals provided by the newBeamResourceSet-r19 that comprise at least one reference signal that satisfies the event, and, when enabledCurrentBeamReport-r19 is configured, the differential L1-RSRP correponding to the reference signal in the indicated TCI state, or to the SSB which is QCLed with the reference signal in the indicated TCI state:. Proposal 2 We propose the following revisions. 5.2.1.5.4.1a UE Initiated CSI reporting for event 2 … The UE, after transmitting PUCCH format 0 or 1, reports in a single reporting instance nrofReportedRS-UEIBR-r19 CRIs or SSBRIs, their absolute L1-RSRPs or differential L1-RSRPs and their [indications on whether they referecnes signal(s) indicated by reported CRIs or SSBRIs satisfy the event] when [RRC parameter], eventDetectionTimeWindowLength-r19, and eventInstanceCount-r19 are configured and nrofReportedRS-UEIBR-r19 is set to a value greater than 1, corresponding to reference signals provided by the newBeamResourceSet-r19 that comprise at least one reference signal that satisfies the event, and, when enabledCurrentBeamReport-r19 is configured, the differential L1-RSRP correponding to the reference signal in the indicated TCI state, or to the SSB which is QCLed with the reference signal in the indicated TCI state:. Comment 3 For below quotation for Event 1, we believe the report instance should be based on current beam RS, instead of new beam RS. 5.2.1.5.4.1b UE Initiated CSI reporting for event 1 For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType-r19 set to ‘event1’, an event instance is determined for a reference signal configured by the newBeamResourceSet-r19 (in a same CC of CSI-ReportConfig, or provided by carrier in the CSI-ReportConfig) if the L1-RSRP value determined for a transmission occasion of - the reference signal in the indicated TCI state (in a same CC of CSI-ReportConfig, or in the CC indicated by ServCellIndex in the CSI-ReportConfig), if newBeamResourceSet-r19 is a NZP-CSI-RS-ResourceSet configured with repetition, else - the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state (in a same CC of CSI-ReportConfig, or in the CC indicated by ServCellIndex in the CSI-ReportConfig), if newBeamResourceSet-r19 is a CSI-SSB-ResourceSet, is lower than eventThresholdEvent1-r19.. Proposal 3 We thus propose the following revisions. 5.2.1.5.4.1b UE Initiated CSI reporting for event 1 For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType-r19 set to ‘event1’, an event instance is determined for a reference signal configured by the newBeamResourceSet-r19 (in a same CC of CSI-ReportConfig, or provided by carrier in the CSI-ReportConfig) if the L1-RSRP value determined for a transmission occasion of - the reference signal in the indicated TCI state (in a same CC of CSI-ReportConfig, or in the CC indicated by ServCellIndex in the CSI-ReportConfig), if newBeamResourceSet-r19 is a NZP-CSI-RS-ResourceSet configured with repetition, else - the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state (in a same CC of CSI-ReportConfig, or in the CC indicated by ServCellIndex in the CSI-ReportConfig), if newBeamResourceSet-r19 is a CSI-SSB-ResourceSet, is lower than eventThresholdEvent1-r19.. | Comment#1: Thank you, as commented in other parts in my view “timer” and “time window” can be seen as synonyms here… Captured the suggestion on the “cyan” part though. Comment#2: Thank you, captured. Comment#3: Thank you, please on this see also the answer to Comment#2 of MediaTek. In fact, I share your understanding, but I also think current agreements do now allow such change. |
| Huawei, HiSilicon | Thanks a lot Mihai for updating the Editor’s CR. We have a few suggestions as follows: Comment 1 (multiple clauses): In 38.213 Editor’s CR, “UE initiated reporting” is used for UEIBM. Also, in both 38.212 and 38.213 Editor’s CRs UEIRI (“UE initiated report indicator”) is used to denote the first PUCCH. It seems easiest to unify terminology in all RAN1 specs to “UE initiated reporting”. Therefore, we suggest to remove “CSI” from all sub-clauses’ titles in 38.214. Of course, this would result in the unwanted situation that the title of 5.2.1.5.4 (which is currently empty, but, understandably is added there as a placeholder) and the title of 5.2.1.5.4.1 to become identical. To avoid this, since 5.2.1.5.4.1 mainly talks about high level procedure that is common among all three events, we suggest to modify its name to “Event-triggering procedure for UE initiated reporting” or, alternatively, simply “Procedure for UE initiated reporting”. Based on above, we suggest the following modification: 5.2.1.5.4 UE Initiated reporting 5.2.1.5.4.1 Event-triggering procedure for UE Initiated CSI reporting […] 5.2.1.5.4.1a UE Initiated CSI reporting for event 2 […] 5.2.1.5.4.1b UE Initiated CSI reporting for event 1 […] 5.2.1.5.4.1c UE Initiated CSI reporting for event 7 […] 5.2.1.5.4.1d UE Initiated CSI reporting for multiple CSI configurations. Comment 2 (Clause 5.2.1.5.4.1): We suggest the following modification for the sake of better clarity: The counting of the event instances for such reference signal is restarteds from zero:. “Restarting the counting”, on its own, may be misinterpreted as restarting the counting after a pause and not necessarily restarting the counting from zero. Comment 3 (multiple clauses): For clause 5.2.1.5.4.1.a We suggest the following modification for the sake of better clarity. Also, we don’t see the need for parentheses. Similar modification is applicable in clauses 5.2.1.5.4.1.b and 5.2.1.5.4.1.c. For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType-r19 set to ‘event2’, an event instance is determined for a reference signal configured by the newBeamResourceSet-r19 (in a same CC of CSI-ReportConfig, or in the CC provided by carrier in the CSI-ReportConfig), if the L1-RSRP value determined for a transmission occasion of the reference signal is an eventThreshold-r19 greater than the L1-RSRP of. Comment 4 (multiple clauses): Based on Agreement A at the bottom of this comment, ServCellIndex is used to indicate the cell on which the indicated TCI state is used while, based on Agreement B at the bottom of this comment, the current beam RS in the indicated TCI state is always located in the same CC as the new beam RS(s) (which, itself, may be determined by the RRC parameter carrier) and, hence, may not be on the cell indicated by ServCellIndex. Given above discussion, the following text (and similar texts elsewhere in the CR) may be confusing as it is not clear from the text whether I) “the reference signal in the indicated TCI state” or II) “the indicated TCI state” itself is in the same CC that is indicated by ServCellIndex. (Based on Agreement A, “II” is correct). the reference signal in the indicated TCI state [in a same CC of CSI-ReportConfig, or the CC indicated by ServCellIndex in the CSI-ReportConfig, if newBeamResourceSet-r19 is a NZP-CSI-RS-ResourceSet configured with repetition, else. To avoid the possible confusion, we suggest the following modifications. Note that by using “applied to”, the reader can clearly understand that text talks about “indicated TCI state” (or “activated TCI state” in the case of Event 7) and not a reference signal, i.e., “the reference signal in the indicated TCI state” (or “the reference signal in the activated TCI state” in the case of Event 7). 5.2.1.5.4.1a UE Initiated CSI reporting for event 2 […] - the reference signal in the indicated TCI state [in a applied to the same CC of CSI-ReportConfig, or to the CC indicated by ServCellIndex in the CSI-ReportConfig, if newBeamResourceSet-r19 is a NZP-CSI-RS-ResourceSet configured with repetition, else - the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state (in a applied to the same CC of CSI-ReportConfig or in to the CC indicated by ServCellIndex in the CSI-ReportConfig), if newBeamResourceSet-r19 is a CSI-SSB-ResourceSet, […] 5.2.1.5.4.1b UE Initiated CSI reporting for event 1 […] - the reference signal in the indicated TCI state (in a applied to the same CC of CSI-ReportConfig, or in to the CC indicated by ServCellIndex in the CSI-ReportConfig), if newBeamResourceSet-r19 is a NZP-CSI-RS-ResourceSet configured with repetition, else - the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state (in a applied to the same CC of CSI-ReportConfig, or in to the CC indicated by ServCellIndex in the CSI-ReportConfig), if newBeamResourceSet-r19 is a CSI-SSB-ResourceSet, […] 5.2.1.5.4.1c UE Initiated CSI reporting for event 7 […] - the reference signal with the valueOfQ-r19 highest L1-RSRP out of the activated TCI state reference signals (in a applied to the same CC of CSI-ReportConfig, or in to the CC indicated by ServCellIndex in the CSI-ReportConfig), if newBeamResourceSet-r19 is a NZP-CSI-RS-ResourceSet configured with repetition, else - the SS/PBCH block which is QCLed with the reference signal with the valueOfQ-r19 highest L1-RSRP out of the activated TCI state reference signals (in a applied to the same CC of CSI-ReportConfig, or in to the CC indicated by ServCellIndex in the CSI-ReportConfig), if newBeamResourceSet-r19 is a CSI-SSB-ResourceSet.. Agreement A (RAN1 120-bis) On cross-CC beam report measurement for UE-initiated/event-driven beam reporting, regarding Event-2, introduce an RRC parameter to indicate ServCellIndex on which the indicated TCI state used to determine the current beam RS is applied - FFS: Indication of the cell that corresponds to the configured first PUCCH resource in CSI report config including whether it is necessary Agreement B (RAN1 120) On cross-CC beam report measurement for UE-initiated/event-driven beam reporting, regarding Event-2, the following is supported - The current beam RS and new beam RS(s) are in the same CC - The above does NOT imply to preclude the cross-CC TCI indication. - The CC on which the indicated TCI state is applied, and the CC in which new beam RS(s) are can be the same or different. - FFS: Whether/how to introduce an RRC parameter to indicate the CC on which the indicated TCI state is applied.. Comment#5 (Clause 5.2.1.5.4.1a): We suggest some reordering of the text in the following paragraph. These conditions are only applicable to [indications whether they satisfy the event], but, since they are brought after all the reporting content (CRI, SSBRI, RSRPs, and the [indications…], the reader may think they are applicable to all reproting content. Note that, lower in the paragraph, the condition for reporting RSRP of the current beam is brought before the RSRP of the current beam: “when enabledCurrentBeamReport-r19 is configured, the differential L1-RSRP correponding to the reference signal in the indicated TCI state”. We suggest to make the following modifications for the sake of more clarity and consistency in the text. The UE, after transmitting PUCCH format 0 or 1, reports in a single reporting instance nrofReportedRS-UEIBR-r19 CRIs or SSBRIs, their absolute L1-RSRPs or differential L1-RSRPs and their [indications whether they satisfy the event] when [RRC parameter], eventDetectionTimeWindowLength-r19, and eventInstanceCount-r19 are configured and nrofReportedRS-UEIBR-r19 is set to a value greater than 1, corresponding to reference signals provided by the newBeamResourceSet-r19 that comprise at least one reference signal that satisfies the event, and, when enabledCurrentBeamReport-r19 is configured, the differential L1-RSRP correponding to the reference signal in the indicated TCI state, or to the SSB which is QCLed with the reference signal in the indicated TCI state:. The UE, after transmitting PUCCH format 0 or 1, reports in a single reporting instance nrofReportedRS-UEIBR-r19 CRIs or SSBRIs, their absolute L1-RSRPs or differential L1-RSRPs and, when [RRC parameter], eventDetectionTimeWindowLength-r19, and eventInstanceCount-r19 are configured and nrofReportedRS-UEIBR-r19 is set to a value greater than 1, their [indications whether they satisfy the event] when [RRC parameter], eventDetectionTimeWindowLength-r19, and eventInstanceCount-r19 are configured and nrofReportedRS-UEIBR-r19 is set to a value greater than 1, corresponding to reference signals provided by the newBeamResourceSet-r19 that comprise at least one reference signal that satisfies the event, and, when enabledCurrentBeamReport-r19 is configured, the differential L1-RSRP correponding to the reference signal in the indicated TCI state, or to the SSB which is QCLed with the reference signal in the indicated TCI state:. Comment 6 (Clause 5.2.1.5.4.1b): A difference between Event-1 and Events 2/7 is that, in contrary to Events 2/7, Event 1 is not “determined” for the new beams; it is “determined” for the current beam (Event-1: Quality of the current beam is worse than a certain threshold). For event-1, the new beams are configured only to let the UE know whether the RS in the indicated TCI state or the SSB QCLed with that RS should be taken as the current beam. Of course, when the event is “determined”, UE will report some of the new beams but the “determination of event 1” has nothing to do with the new beams. Therefore, we suggest the following modification. For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType-r19 set to ‘event1’, an event instance is determined for a reference signal configured by the newBeamResourceSet-r19 (in a same CC of CSI-ReportConfig, or provided by carrier in the CSI-ReportConfig) if the L1-RSRP value determined for a transmission occasion of - the reference signal in the indicated TCI state (in a same CC of CSI-ReportConfig, or in the CC indicated by ServCellIndex in the CSI-ReportConfig), if newBeamResourceSet-r19 is a NZP-CSI-RS-ResourceSet configured with repetition, else - the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state (in a same CC of CSI-ReportConfig, or in the CC indicated by ServCellIndex in the CSI-ReportConfig), if newBeamResourceSet-r19 is a CSI-SSB-ResourceSet, is lower than eventThresholdEvent1-r19 where newBeamResourceSet-r19 is in a same CC of CSI-ReportConfig, or in the CC provided by carrier in the CSI-ReportConfig.. Comment#7 (Clause 5.2.1.5.4.1c): According to the latest RRC parameter list, the same RRC parameter eventThreshold-r19 indicate the event threshold for both Event 7 and Event 2. Therefore, we suggest the following modification: For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType-r19 set to ‘event7’, an event instance is determined for a reference signal configured by the newBeamResourceSet-r19 (in a same CC of CSI-ReportConfig, or provided by carrier in the CSI-ReportConfig) if the L1-RSRP value determined for a transmission occasion of the reference signal is an eventThresholdEvent7-r19 greater than the L1-RSRP of. Comment#8 (Clause 5.2.1.5.4.1d): From the agreement brought at the bottom of this comment, we don’t infer that the multiple CSI-ReportConfigs for UEIBM that are associated with the same PUCCH must correspond to the same event type. In other words, the agreement does not preclude the case that, some of eventType-r19 for the multiple CSI-ReportConfigs are set to Event2 while others set to, for instance, Event7. We understand that whether or not to preclude the mixed-event case for the report configurations that are associated with the same PUCCH resource may need to be further discussed in RAN1, however, for now, the agreement does not seem to preclude it and, therefore, we suggest the following modification for clause 5.2.1.5.4.1d. Also, for the sake of better clarity, we suggest the following further explanation. For a UE is configured with multiple CSI-ReportConfig all with the same higher layer parameters eventType-r19, and with the same higher layer parameters firstPUCCHResourceConfig-UEIBR-r19, and reportTransmissionMode-r19, the UE expects - all multiple CSI-ReportConfig in a same CC, and - the same CSI trigger state if reportTransmissionMode-r19 is configured as ‘ModeA’, else - the same configuredResourceForSecondChannelOfModeB-r19 if reportTransmissionMode-r19 is configured as ‘ModeB’, and when reporting in a single reporting instance nrofReportedRS-UEIBR-r19 CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet-r19 in a CSI-ReportConfig that satisfies the event, the CSI report contains the corresponding CSI-ReportConfigId and is zero padded to a fixed payload size (when needed), with the fixed payload size given by the maximum payload size among all the multiple CSI-ReportConfig associated with the same PUCCH given by firstPUCCHResourceConfig-UEIBR-r19.. Agreement (RAN1 120) On beam report transmission procedure for UE-initiated/event-driven beam reporting, one PUCCH resource of first PUCCH can be associated with one or multiple CSI report configurations, regarding Event-2. - Only a single UEI beam report is carried in the second PUSCH. - Additional indication of one ‘CSI report configuration’ is provided in the report format. - The CSI report configurations associated with the same PUCCH resource are ordered in ascending order of corresponding CSI-ReportConfigId, the number of bits of the additional indication field is ceil(log2(N_CSIconfig)), where the N_CSIconfig denotes the number of CSI report configurations associated with the same PUCCH resource. - The payload size of the single UEI beam report is determined according to the max payload size among the associated CSI report configurations. - Zero-padding can be appended if the payload size of the UEI beam report is less than the max report payload. - The reported UEI beam report should satisfy the triggering condition. - If multiple UE initiated beam report procedures occur, down-select one of the following options: - Option-1: It is up to UE implementation to select one of configuration. - Option-2: The UEI beam report with highest priority is reported - Option-3: The report triggered in the latest measurement is reported in PUSCH - The multiple CSI report configurations associated with the one first PUCCH resource should be configured in the same CC. - Working Assumption: For Mode-A, the multiple CSI report configurations associated with the same PUCCH resource should be associated with a same CSI-AperiodicTriggerState. - For Mode-B, the multiple CSI report configurations associated with the same PUCCH resource should be associated with the same second configured PUSCH FFS: Alt-2 Multiple UEI beam reports associated with the same PUCCH resource for first PUCCH can be transmitted in the second PUSCH. - If RAN1 cannot converge on the support of Alt-2 in RAN1#120bis, this alternative will be dropped from Rel-19. Comment#9 (Sharing our views regarding some of other companies’ comments): -Comment#1 Samsung: We don’t see such modification to be necessary. The original wording by the Editor is closer to the agreement. -Comment#2 Samsung: RAN1 does not have such an agreement that the UCI notification/request should be sent in the earliest configured PUCCH resource after the time window and we don’t think such modification is necessary. Also, such modification triggers other discussions, such as, what happens if the earliest configured PUCCH is not available due to multiplexing/dropping rules. -Comment#3 Samsung: We don’t think that the agreed conditions for restarting the counting should be removed and do not support the suggested modifications. RAN1 agreed that issues related to time window and “the procedure of triggering event determination” to be captured in RAN1 spec and if the conditions for restarting the counting are removed from RAN1 spec, there is no other spec to capture them. -Comment#2 MediaTek: The part starting with “Firstly”: We think the current text in Editor CR is correct. ul-TCI-StateList is brought here along with dl-OrJointTCI-StateList to clarify that, aligned with the WID text, UEIBM is applicable under unified TCI state regime (both joint and separate). The part starting with “Thirdly”: Agree with MTK! | Comment#1: Thank you, good suggestions, let me think a bit on these, we will get something good in the end and indeed, I had a sync with other editors on this issue, we all want to get a good terminology alignment across specs! Comment#2: Thank you, I think such clarification may not be needed as the counter is not paused here. Comment#3: Thank you, captured the text correction, but I think parenthesis improves readability. Comment#4: Thank you, captured. Comment#5: Thank you, captured. Comment#6: Thank you, although I share your view on this issue, I also think that the current agreements states the counter is for the new beams for all events, so such change cannot be implemented, see also the answer to Comment#2 from Mediatek. Comment#7: Thank you, captured. Comment#8: Thank you, added square brackets now on eventType-r19 there. However, the agreement you refer state at the beginning “regarding Event-2”, which means that the multiple CSI report configurations are associated to the same Event-2. Then, because such agreement extends to Event-1 and Event-7, I think that based on current agreements, it is: - possible to associate multiple CSI report configurations to a same first PUCCH when these CSI report configurations are ALL associated to either Event-1 or Event-7; - not possible to associate multiple CSI report configurations to a same first PUCCH when these CSI report configurations are associated to different events. In our view, we should clarify in the next meeting if cross-event operations are supported. Comment#9: Thank you. |
| Huawei, HiSilicon2 | Clause 5.2.1.5.4.1c: It came to our attention that “condition met indicator” is only an FFS for Event-1: Agreement (RAN1 120-bis) On UE-initiated/event-driven beam reporting, support the following interpretation on each of the codepoints of the ‘1-bit’ condition met indicator as follows: - ‘0’ – indicating that the Event-2 instances for corresponding CRI/SSBRI within the time window doesn’t reach the configured number M. - ‘1’ – indicating that the Event-2 instances for corresponding CRI/SSBRI within the time window reach the configured number M. FFS: whether/how to introduce the ‘1-bit’ condition met indicator for Event 7.. Therefore, for now, we suggest to add brackets around the relevant part as follows: The UE, after transmitting PUCCH format 0 or 1, reports in a single reporting instance nrofReportedRS-UEIBR-r19 CRIs or SSBRIs, their absolute L1-RSRPs or differential L1-RSRPs, [and their [indications whether they satisfy the event] when [RRC parameter], eventDetectionTimeWindowLength-r19, and eventInstanceCount-r19 are configured and nrofReportedRS-UEIBR-r19 is set to a value greater than 1,] corresponding to reference signals provided by the newBeamResourceSet-r19 that comprise at least one reference signal that satisfies the event, and, when enabledCurrentBeamReport-r19 is configured, the differential L1-RSRP corresponding to the reference signal with the valueOfQ-r19 highest L1-RSRP out of the activated TCI state reference signals, or to the SSB which is QCLed with the reference signal with the valueOfQ-r19 highest L1-RSRP out of the activated TCI state reference signals:. | Thank you, captured, although the FFS refers to Event-7 (and not Event-1). |
| Editor, 27.04.2025 | Took into account the above comments and suggestions! Version 5 of the draft CR is currently available! |  |
| ZTE2 | Thanks for your effort. Comment#1: After quickly reviewing, the following update is incorrect. The indicated TCI state is determined according to the CC of CSI-reportConfig or the newly added RRC parameter CCofIndicatedTCI-r19. Please find the following suggestion in red: - the reference signal in the indicated TCI state (applied to the same CC of the reference signal configured by the newBeamResourceSet-r19CSI-ReportConfig, or into the CC indicated by CCofIndicatedTCI-r19ServCellIndex in the CSI-ReportConfig), if newBeamResourceSet-r19 is a NZP-CSI-RS-ResourceSet configured with repetition, else - the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state (in aapplied to the same CC of the reference signal configured by the newBeamResourceSet-r19CSI-ReportConfig, or into the CC indicated by CCofIndicatedTCI-r19ServCellIndex in the CSI-ReportConfig), if newBeamResourceSet-r19 is a CSI-SSB-ResourceSet,. It seems that the above is relevant to the configuration rule of ‘new beam and current beam should be from the same CC’. Clearly that has been specified already, and which does not reply that the indicated TCI should be based on the CC of new beam RSs. The UE does not expect that the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state is in a different CC from the reference signals configured by newBeamResourceSet-r19.. Comment#2: We would like to echo Huawei’s suggestion that in the time of each clause, ‘CSI’ should be removed. Clearly, we herein only focus on L1-RSRP report under UE initiated procedure. | C1: Thank you, captured. That update was based on Vivo Comment#4, but after your feedback, I think that your proposal captures better the current agreements. C2: Thank you, but as also commented to Huawei Comment#1, we can think about this editorial change at the end after Malta meeting, further aligning with other editors. |
| Samsung2 | Thank you Mihai for the updates and the replies. Please find our additional comments below. Comment #1 on time window determination Based on the current version, the starting of a time window is still unclear to us: based on the current wording/description in the draft CR, it can be any time after determining an instance. This seems not the intention to our understanding, at least the time window should include the first RS transmission occasion that satisfies the event. “if eventDetectionTimeWindowLength-r19 is provided and the time window for such reference signal has not started or has ended, the UE starts the time window for such reference signal.” Furthermore, the red texts above seem to address the case that the UE can start multiple non-overlapping time windows for a same new beam RS. However, “the” time window in the current wording seems ambiguous. Using the above conceptual example/figure, can the 1st “the time window” in the above texts refer to Window #1 and the 2nd “the time window” in the above texts refer to Window #2? At least according to the current texts, “the time window” seems to refer to a same time window but actually this should not be the intention (i.e., they should refer to two separate non-overlapping time windows – e.g., Window #1 and Window #2 in the example, so we prefer to use “another” time window to differentiate between the two). Given the above, we still suggest the following updates. “For a UE configured with a CSI-ReportConfig with eventType-r19 for periodic reference signals [with the same periodicity] and with dl-OrJointTCI-StateList or ul-TCI-StateList, when an event instance is determined for the UE determines a time window of a length eventDetectionTimeWindowLength-r19, if provided, where the time window starts at the beginning of a transmission occasion of a reference signal configured by the newBeamResourceSet-r19 (as described in the following clauses) for which an event instance is determined (as described in the following clauses), wherein the time window does not overlap in time domain with another time window for the same reference signal if eventDetectionTimeWindowLength-r19 is provided and the time window for such reference signal has not started yet or has ended, the UE starts the time window for such reference signal.” Comment #2 on event instance counting As we clarified in the first round that the details of counting may not need to be explicitly specified and we can only focus on the total number of instances as described in “When the number of event instances determined for such reference signal”. In addition, the current text of resetting the counting may result in ambiguities in interpreting “the number of event instances”, and it does not say that the counted instansce before the TCI state update can trigger the PUCCH or not. Details related to TCI state update need further clarification at least for the following. - When resetting would occur, e.g., the time when the UE detects the DCI or the time when the new beam is applied. - Whether the time window should be redetermined - Whether the event cannot be considered as triggered when detecting TCI state update in a time window. Given the above, we still suggest to remove the texts below for now (can revisit it after the above missing details are clarified): The counting of the event instances for such reference signal is restarted: - - if the reference signal in the indicated TCI state or the SSB which is QCLed with the reference signal in the indicated TCI state is updated. Comment #3 We suggest the following update in red – this is to clarify that the measurements for a new beam RS and the current beam RS should be for their respective transmission occasions in/under the same periodicity (otherwise, the event evaluation/RSRP comparison would not be meaningful). “ For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType-r19 set to ‘event2’, an event instance is determined for a reference signal configured by the newBeamResourceSet-r19 (in a same CC of CSI-ReportConfig, or provided by carrier in the CSI-ReportConfig) if the L1-RSRP value determined for a transmission occasion of the reference signal is an eventThreshold-r19 greater than the L1-RSRP value determined for a transmission occasion of - the reference signal in the indicated TCI state (applied to the same CC of the reference signal configured by the newBeamResourceSet-r19, or to the CC indicated by CCofIndicatedTCI-r19 in the CSI-ReportConfig), if newBeamResourceSet-r19 is a NZP-CSI-RS-ResourceSet configured with repetition, else - the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state (applied to the same CC of the reference signal configured by the newBeamResourceSet-r19 or to the CC indicated by CCofIndicatedTCI-r19 in the CSI-ReportConfig), if newBeamResourceSet-r19 is a CSI-SSB-ResourceSet, where the transmission occasions are in a same periodicity. ” Comment #4: editorial to be aligned with prior descriptions For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType-r19 set to ‘event2’, an event instance is determined for a reference signal configured by the newBeamResourceSet-r19 (in a same CC of CSI-ReportConfig, or provided by carrier in the CSI-ReportConfig) if the L1-RSRP value determined for a transmission occasion of the such reference signal is an eventThreshold-r19 greater than the L1-RSRP of | C1: Thank you for your detailed observation. However, regarding the aspect of avoiding overlapping time windows, made the last change based on Vivo Comment#1, and I think that exactly by using “the time window” we make sure that there is only one time window started/active/running at a time for a new beam. Then, regarding the exact starting time of the time window, I share your view that in the current version it is not fully written exactly when a time window starts after an event instance is determined. However, I also think the current version is the best way to capture the current agreement, that says “an Event-2 instance for a new beam is obtained at the time t”, without explicitly detailing what time t is. It could be the time of a transmission occasion of that new beam as you mention, but it could also be the time at which such beam is actually measured by the UE or the time at which such beam must be measured (as a UE requirement) at the UE. C2: Considering that the conditions for resetting the counter beyond Candidate#2 (updated of the current beam based on the indicated TCI state) are still under discussion and FFS, I have put square brackets for that sentence. C3: I have partially captured your update. Note that the same periodicity of the involved beams/RSs is already in the very first sentence “For a UE configured with a CSI-ReportConfig with eventType-r19 for periodic reference signals [with the same periodicity]” It does not need to be repeated for any event. C4: Captured. |
| Editor, 28.04.2025 | Took into account the above comments and suggestions, plus an update on the counter for Event-1 based on the multiple suggestions in the first round! Version 6 of the draft CR is currently available! |  |
| Huawei, HiSilicon3 | Thank you Mihai for considering our first-round comments. Some further comments for your consideration. Comment#1: After further thoughts, it seems that the inclusion of ul-TCI-StateList is actually unnecessary: If UE is in unified TCI regime with joint UL and DL TCI, dl-OrJointTCI-StateList is configured and if UE is in unified TCI regime with separate UL and DL TCI, both dl-OrJointTCI-StateList and ul-TCI-StateList are configured. So, mentioning dl-OrJointTCI-StateList seems enough to clarify that UEIBR works only in unified TCI regime. Therefore, we suggest to remove ul-TCI-StateList at the beginning of 5.2.1.5.4.1. Sorry for the earlier confusion. Comment#2: The following two conditions brought in 5.2.1.5.4.1 only apply to Event-1 and Event-2 but not Event-7 since, in contrary to Event-1 and Event-2, the current beam for Event-1 is the RS derived from the activated TCI state with the Qth best quality. Text A: The reference signal in the indicated TCI state is the reference signal w.r.t. QCL-TypeD, if there are two QCL RSs in the indicated TCI state. The UE does not expect that the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state is in a different CC from the reference signals configured by newBeamResourceSet-r19.. In fact, the conditions relevant to Event-7 are brought in Clause 5.2.1.5.4.1c: Text B: The reference signal with the valueOfQ-r19 highest L1-RSRP out of the activated TCI state reference signals is the reference signal w.r.t. QCL-TypeD, if there are two QCL RSs in such activated TCI state. The UE does not expect that the reference signal with the valueOfQ-r19 highest L1-RSRP out of the activated TCI state reference signals is in a different CC from the reference signals configured by newBeamResourceSet-r19.. Having Text A in 5.2.1.5.4.1 instead of 5.2.1.5.4.1a (for Event-2) and 5.2.1.5.4.1b (for Event-1) causes the misunderstanding that, for Event-7, not only the conditions in Text B shoul be met but also the conditions in Text A should be met. This is certainly incorrect since Event-7 does not necessarily make any measurement on the RS derived from the indicated TCI state altogether. To avoid such misunderstanding, we suggest to remove Text A from 5.2.1.5.4.1 and put it in 5.2.1.5.4.1a and 5.2.1.5.4.1b. alternatively, we can keep Text A in 5.2.1.5.4.1 but add the condition that it is applicable only to the cases that eventType-r19 set to ‘event1’ or ‘event2’. Comment#3: Regarding Comment#6 in our first round of comments, we do appreciate our Editor’s reply and understand the restriction imposed to our Editor by that fact the there is only one agreement about the counting mechanism and it is per new beam for event-2. However, we think that if the draft CR is endorsed as is, it causes serious implementation issues for Event-1 as follows: Since based on the draft CR (and RAN1 agreements) “when an event instance is determined for a reference signal configured by the newBeamResourceSet-r19 (as described in the following clauses), if eventDetectionTimeWindowLength-r19 is provided and the time window for such reference signal has not started yet or has ended, the UE starts the time window for such reference signal”, for Event-2 and Event-7, UE starts the time window (and the subsequent event instance counting) only for the new beams that satisfy the event-determination condition (e.g, for Event-2, only for the RSs in the newBeamResourceSet-r19 whose RSRP is eventThreshold-r19 greater than the L1-RSRP of the current beam). The number of RSs in newBeamResourceSet-r19 that satisfy such a condition is quite low (unlikely to be more than a couple). This means that for Event-2 and Event-7, UE typically only needs to maintain a couple of time windows/counting at a time. However, for Event-1, based on the text in 5.2.1.5.4.1b, every time the current beam drops below the threshold, an event instance is determined for a reference signal cconfigured by the newBeamResourceSet-r19. This means that if, for instance, if newBeamResourceSet-r19 has 32 resources, 32 parallel windows should be maintained by the UE and, everytime the current beam drops below the threshold, the counters associated with each of these windows increment by one. This makes implementing Event-1 almost impossible in practice. Since this problem in the CR is only due to the oversight in RAN1 that did not make an agreement that the event instance(s) counting is per current beam for Event-1, we are just wondering if we could at least put the related text in brackets to ensure that this issue is discussed and rectified in the next meeting. We suggest the following: 5.2.1.5.4.1b UE Initiated CSI reporting for event 1 For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType-r19 set to ‘event1’, an event instance is determined [for a reference signal configured by the newBeamResourceSet-r19 (in a same CC of CSI-ReportConfig, or in the CC provided by carrier in the CSI-ReportConfig)] if the L1-RSRP value determined for a transmission occasion of - the reference signal in the indicated TCI state (in aapplied to the same CC of the reference signal configured by the newBeamResourceSet-r19CSI-ReportConfig, or into the CC indicated by CCofIndicatedTCI-r19ServCellIndex in the CSI-ReportConfig), if newBeamResourceSet-r19 is a NZP-CSI-RS-ResourceSet configured with repetition, else - the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state (in aapplied to the same CC of the reference signal configured by the newBeamResourceSet-r19CSI-ReportConfig, or into the CC indicated by CCofIndicatedTCI-r19ServCellIndex in the CSI-ReportConfig), if newBeamResourceSet-r19 is a CSI-SSB-ResourceSet, is lower than eventThreshold-Event1-r19.. Comment#4: It is not clear for us why this is in brackets for Event-2. This should be in brackets for Event-7 only due to the following two agreements at the end of this comment: “their [indications on whether reference signals indicated by reported CRIs or SSBRIs satisfy the event],”. Agreement (120) On UE-initiated/event-driven beam reporting, Option-1 is supported as an extension on L1-RSRP report format depending on Event-2. - Option-1: For each of reported CRI/SSBRI, to introduce additional indication of whether the CRI/SSBRI satisfies the condition of Event-2. - The presence of this field is enabled by RRC with subjective to UE capability. - Note: The presence of this field is only for the case that N > 1 and the time window and M are configured Above is NOT applicable for event 1. Agreement (120-b) On UE-initiated/event-driven beam reporting, support the following interpretation on each of the codepoints of the ‘1-bit’ condition met indicator as follows: - ‘0’ – indicating that the Event-2 instances for corresponding CRI/SSBRI within the time window doesn’t reach the configured number M. - ‘1’ – indicating that the Event-2 instances for corresponding CRI/SSBRI within the time window reach the configured number M. FFS: whether/how to introduce the ‘1-bit’ condition met indicator for Event 7.. Comment#5: Based on the agreement at the end of this comment, time window and counting are not used for the basic feature for UE initiated beam report. However, in the current Editor’s CR, even if UE transmit PUCCH when the number of event instances is greater than or equal to 1 (basic feature), such an event instace should happen within the time window if eventDetectionTimeWindowLength-r19 is provided: When the number of event instances determined for such reference signal within the time window, if eventDetectionTimeWindowLength-r19 is provided, is greater than or equal to 1 or to eventInstanceCount-r19, if provided, the UE transmits PUCCH format 0 or format 1 in the PUCCH resource configured by PUCCHResource-r19 in the CSI-ReportConfig.. To clarify that basic feature does not need time window configuration, we suggest the following modification: When the number of event instances determined for such reference signal is greater than or equal to 1 or, if eventDetectionTimeWindowLength-r19 is provided, greater than or equal to eventInstanceCount-r19 within the time window, if eventDetectionTimeWindowLength-r19 is provided, is greater than or equal to 1 or to eventInstanceCount-r19, if provided, the UE transmits PUCCH format 0 or format 1 in the PUCCH resource configured by PUCCHResource-r19 in the CSI-ReportConfig.. Agreement (117) Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification.. Comment#6: Our comment#3 is the first round (in the CC is not added before “provided by carrier”) is captured for Event-1 and Event-7 but not Event-2. | C1: Thank you, captured. C2: Thank you, captured, moved those two sentences into Event-2 and Event-1 paragraphs. C3: Thank you for your comment, after some more thoughts, I agree with you and also the other companies that suggested to capture what you propose in the first round, and now we have revised the text accordingly, i.e., the counter for Event-1 is just for the current beam, as indeed it should technically be, although in RAN1 we overlooked this detail. C4: Introduced square brackets there because I think the details of such 1-bit indication are not fully clear, for example there is an RRC parameter to be introduced which is not yet defined. After Malta, we will polish a bit better the text after the details are fixed. For Event-7, note that the whole text is in square brackets, exactly because that 1-bit indication seems still FFS. C5: Actually, I think the current text is ok, and prefer not to change. I agree with you that the basic feature does not require the time window, and indeed we say “within the time window, if eventDetectionTimeWindowLength-r19 is provided”, meaning exactly that the counting is within the window only if such window is configured, whereas if such window is not configured, then the counting is single-shot, or at least it does not care about that “within the time window”. C6: Thank you, captured it now. |
| ZTE | Thank you so much for your effort and considering our comments. Please find our one more comment: Comment#1: For Event-7, the description of ‘activate TCI state’ should be refined and aligned with the general description in Clause 5.2.1.5.4.1. Then, we have the following two suggestion in red. - The CCofIndicatedTCI-r19 is applied to TCI state, rather than reference signals as we commented before. - ‘Q-best’ is per channel quality of measured RS(s) among activated TCI state(s) (especially for the case of being based on SSB). Clause 5.2.1.5.4.1 The reference signal in the indicated or activated TCI state is the reference signal w.r.t. QCL-TypeD, if there are two QCL RSs in the indicated or activated TCI state.. 5.2.1.5.4.1c UE Initiated CSI reporting for event 7 For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType-r19 set to ‘event7’, an event instance is determined for a reference signal configured by the newBeamResourceSet-r19 (in a same CC of CSI-ReportConfig, or provided by carrier in the CSI-ReportConfig) if the L1-RSRP value determined for a transmission occasion of such reference signal is an eventThreshold-r19 greater than the L1-RSRP value determined for a transmission occasion of - the reference signal with the valueOfQ-r19 highest L1-RSRP out of the activated TCI state reference signals among activated TCI states (applied to the same CC of CSI-ReportConfig, or to the CC indicated by CCofIndicatedTCI-r19 in the CSI-ReportConfig), if newBeamResourceSet-r19 is a NZP-CSI-RS-ResourceSet configured with repetition, else - the SS/PBCH block with the valueOfQ-r19 highest L1-RSRP out of the SS/PBCH blocks which is QCLed with the reference signals with the valueOfQ-r19 highest L1-RSRP out of among the activated TCI states reference signals (applied to the same CC of CSI-ReportConfig, or to the CC indicated by CCofIndicatedTCI-r19 in the CSI-ReportConfig), if newBeamResourceSet-r19 is a CSI-SSB-ResourceSet.. | C1: Thank you, captured your second suggested changes. For the first one though, I think we already clarified that (see also our implementation for Huawei Comment#2 in this round). |
| MediaTek2 | Thanks for updating the draft CR accordingly. On the updated version, we still have some comments and suggestions: Comment #1 (counting evaluation) We’d like to echo Ericsson’s and Google’s view on re-wording the window as the timer. Strictly speaking, the window is the time duration with a certain/fixed length, while the timer is exactly how UE implement the window in real time system. From our view, except for the timer operation, we don’t see any other potential way to implement the window. The above interpretation better matches agreements in R1#117 and R1#120b for counting evaluation. We prefer to replace “window” with “timer”. Comment #2 (unified TCI framework) We appreciate Huawei’s further checking on RRC configuration for unified TCI framework. Present dl-OrJointTCI-StateList RRC configuration do imply that this is feature should be on top of unified TCI framework regardless of ul-TCI-StateList is configured or not. So, we still prefer to remove the wording of “ul-TCI-StateList”. Comment #3 (counting evaluation) We still think that it is better to revise “the number of event instances determined …” as “the counting of event instances determined …” for wording consistency. Comment #4 (multiple CSI report configurations sharing the same PUCCH resource) We think that is also appliable to all the events and there is no restriction on the multiple CSI report configuration should be configured in the same event type. Hence, we suggest removing the wording of “with the same higher layer parameters [eventType-r19]”. Comment#5 (evaluation periodicity) Thank you for clarifying that the wording of “for periodic reference signal [with the same periodicity]” reflect this periodicity restriction between new beam RS and the current beam RS. However, there is a key point regarding “evaluation periodicity” missing in draft CR in the agreement: [119] Agreement Regarding for the evaluation periodicity for determining Event-2 instance [at least when DRX is not configured], at least Alt-1 is supported for the single-CC beam reporting (for case that the CSI report configuration and RS for the current beam and new beam(s) are in the same CC). - Alt-1: The periodicity of the current beam RS should be the same as that of the new beam RS(s). - The evaluation periodicity is the same as the periodicity of the current and new beam RS(s). - … Note: There is the same periodicity for the new beam RS(s). FFS: Down-selection among above Alt(s) for cross-CC beam reporting. Strong concerns were expressed by Huawei and CATT that if only Alt-1 is supported in the end, the feature will not be practical. Send an LS to RAN4. Final LS in R1-2410914.. Beside of RS periodicity restriction, this agreement clearly specify how frequent UE shall perform the event evaluation. Referring to evaluation for legacy BFD, TS38.213 do specify the evaluation periodicity for indicating a beam failure indication as follows: The physical layer informs the higher layers when the radio link quality is worse than the threshold Qout,LR with a periodicity determined by ….. Proposed change in 5.2.1.5.4.1: For a UE configured with a CSI-ReportConfig with eventType-r19 for periodic reference signals [with the same periodicity] and with dl-OrJointTCI-StateList or ul-TCI-StateLis, when an event instance is determined, with a periodicity of reference signal(s) configured by the newBeamResourceSet-r19, - for a reference signal configured by the newBeamResourceSet-r19, if eventType-r19 is set to ‘event2’ or ‘event7’, or - for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI, if eventType-r19 is set to ‘event1’,. | C1: Thank you for your observation, but I think “timer” and “time window” can be seen as synonyms here, so would prefer the use of time-window for now. C2: Thank you, captured. C3: Thank you, but I think that using “number” there fits better, as that “number” is what is actually compared to either 1 or eventInstanceCount-r19. C4: Thank you for your comment. I share your view that this use case of multiple events configured in parallel at the UE is a very relevant one, but, as observed also to Huawei in Comment#8 in the first round, the agreement you refer explicitly states at the beginning “regarding Event-2”, which means that the multiple CSI report configurations associated to the same first PUCCH are also associated to the same Event-2. I think in Malta we should clarify this scenario in case the UE is configured with multiple CSI report configurations associated to different events. C5: Thank you, captured your suggestion, but in a slightly different way, hope it is ok. |
| Editor, 29.04.2025 | Took into account the above comments and suggestions! Version 7 of the draft CR is currently available! |  |
| Samsung3 | Thank you editor Mihai for your great efforts! Regarding allowing multiple non-overlapping time windows, the current wording seems to allow (using the above figure/example to illustrate) the UE to start another time window (say Window #3) from the 6th TO of RS #1 (satisfying the event) which would be overlapping with Window #2 for the same RS #1 – this should not be the intention. To avoid this case, it should be that “Window #1 has ended AND Window #2 has not started”, but the current texts may not accurately reflect this. A simple way would be to just clarify that different time windows would not overlap in time (as suggested by us in previous rounds), but was not adopted. Given the limited time and also RAN1 may need to further clarify whether the time window should start from the first TO of a RS that satisfies the event, we suggest to put a bracket on the following and come back after RAN1#121. “ [if eventDetectionTimeWindowLength-r19 is provided and the time window for such reference signal has not started yet or has ended, the UE starts the time window for such reference signal.] ” | C1: Thank you for your comment. After some more thoughts, I have decided to capture your suggestion in the second round, without however considering the aspect of the starting time of the window because, as commented before, that has not been fully discussed/agreed in RAN1, hope that is ok. |
| Editor, 30.04.2025 | Addressed above comment from Samsung! Version 8 of the draft CR is currently available! |  |