3GPP TSG RAN WG1 #121		R1-2503511

St Julian’s, Malta, May 19th – 23th, 2025

Agenda Item:     9.2.4

Source:	Spreadtrum, UNISOC

Title:	             Enhancement for asymmetric DL sTRP/UL mTRP scenarios

Document for:	Discussion and decision

Introduction

In light of the updated R19 FeMIMO WID [1], the following objective is included in the scope:

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

In this paper, we will present our opinions on enhancements for asymmetric DL sTRP/UL mTRP scenarios.

Discussion

During the last meeting [2], RAN1 discussed whether/how to reset the CLPC of PUSCH/PUCCH/SRS for a UE is configured with two TAGs and when the UE receives a RAR of a PDCCH-order PRACH. As stated in the draft report, companies are encouraged to have further discussion.

Companies are encouraged to consider the following for RAN1#121: Regarding whether/how to reset the CLPC of PUSCH/PUCCH/SRS, further study and down-select one from the following alternatives: - Updated Alt1: For a UE configured with 2 TAGs, when the UE receives a RAR of a PDCCH-order PRACH: - If PL offset is configured in joint/UL TCI state(s), according to the new 1-bit DCI field PL offset indication in DCI format 1_0: - When there is one indicated joint/UL TCI state in Rel-17 Unified TCI: - If the 1-bit DCI field is 0, the UE resets the CLPC that is different from the CLPC associated with the indicated joint/UL TCI state. - If the 1-bit DCI field is 1, the UE resets the CLPC associated with the indicated joint/UL TCI state. - When there are two indicated joint/UL TCI states in Rel-18 Unified TCI: - If the 1-bit DCI field is 0, the UE resets the CLPC associated with the first indicated joint/UL TCI state; - If the 1-bit DCI field is 1, the UE resets the CLPC associated with the second indicated joint/UL TCI state - If PL offset is not configured in joint/UL TCI state(s) and the UE is configured with SSB-MTC-AddtionalPCI, according to the PRACH association indicator in DCI format 1_0: - If the indicated joint/UL TCI state is associated with the same PCI as indicated by the PRACH association indicator, the UE resets the closed loop power control adjustment state associated with the indicated joint/UL TCI state. - If the indicated joint/UL TCI state is associated with a different PCI from the one indicated by the PRACH association indicator, the UE resets the closed loop power control adjustment state that is different from the closed loop power control adjustment state associated with the indicated joint/UL TCI state. - Alt2: For a UE configured with two TAGs, one TAG ID is indicated in the RAR - The UE resets the closed loop power control adjustment state associated with a joint/UL TCI state associated with the indicated TAG. - Alt3: Not introduce any enhancement.

First of all, as mentioned by other companies, this issue has been discussed in Rel-18 two TA without any outcome. We don’t see the necessity to reconsider the issue. Technically, based on current specification, TPC command is commonly used for SRS, PUCCH and PUSCH to dynamically adjust the Tx power. With this understanding, UE will transmit PUSCH/PUCCH/SRS to UL/DL TRP with proper Tx power anyway. Thus we don’t think the updated Alt1 and Alt2 are useful.

Therefore, we support no further enhancement on this issue.

Proposal 1: Regarding whether/how to reset the CLPC of PUSCH/PUCCH/SRS, no further enhancement is needed.

Conclusion

In this contribution, we provide our opinions on enhancements for asymmetric DL sTRP/UL mTRP scenarios:

Proposal 1: Regarding whether/how to reset the CLPC of PUSCH/PUCCH/SRS, no further enhancement is needed.

Reference

- RP-242394, WID revision: NR MIMO Phase 5

- Draft Report of 3GPP TSG RAN WG1 #120b