3GPP TSG RAN WG1 #121                           R1-2503514

St Julian’s, Malta, May 19th – 23th, 2025

Agenda Item:	9.3.3

Source:	Spreadtrum, UNISOC

Title:	Discussion on CLI handling

Document for:	Discussion and decision

Introduction

In Rel-19 WID[1,3] of Evolution of NR duplex operation, CLI handing schemes is specified. The RAN1 work focuses on two schemes, namely UL resource muting for PUSCH and L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework.

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD.

In this contribution, we discuss remaining issues in UL resource muting and L1 based UE-to-UE CLI measurement and reporting [4,5] .

UL resource muting for PUSCH

Indication/determination of UL resource muting

Time location

Whether UL resource muting is applied in non-SBFD symbols

In RAN1#120bis, the following FL proposals were discussed.

#### Proposal 1-1 UL resource muting can be applied in non-SBFD symbols. Note: No additional specification impact is expected to support above. #### Proposal 1-5 Observation The following agreement does not impose any restriction to apply UL resource muting in SBFD symbols or non-SBFD symbols, i.e., UL resource muting can be applied on both type of symbols. Agreement To determine the time location of UL muting symbol(s) in a slot for a PUSCH, the following option is selected for DG PUSCH and Type 2 CG PUSCH Option 2: The time location of each of one or two UL muting symbols is semi-statically configured, and muting all of the semi-statically configured time location of UL muting symbol(s) can be dynamically turned ON/OFF by TDRA field in DCI

Based on previous meeting, it seems that UL muting symbol can be configured in SBFD symbols or non-SBFD symbols. From our perspective, UL resource muting is used to estimate the gNB-to-gNB CLI covariance matrix in SBFD symbol for better PUSCH performance. So some restriction on time location of UL muting symbol is needed, e.g, only the UL muting symbol located in the SBFD symbol is valid.

In previous meeting, some companies mentioned that UL resource muting is beneficial for gNB-gNB CLI mitigation in dynamic/unaligned TDD case, gNB with different SBFD subband configurations case, and co-existence case between TDD and SBFD. It was argued that there is no additional spec impact to support UL resource muting in non-SBFD symbols.

According to R19 WID, only case 1 (Non-coexistence case with single SBFD subband configuration) is prioritized. In deployment case 1, there is no CLI in non-SBFD symbol. In addition, a typical network deployment would have synchronized TDD patterns across different gNBs. So it would lead to resource waste if no CLI exist. Since whether apply muting to non-SBFD symbol depends on deployment case, some companies think it depends on gNB configuration, i.e., gNB can configure whether UL resource muting can be applied the non-SBFD symbols. From our perspective, it is not necessary to introduce additional RRC signaling. We prefer to apply UL resource muting in non-SBFD symbols only.

Proposal 1: UL resource muting cannot be applied in non-SBFD symbols.

Handling of collision between UL muting resources and PTRS

For Collison of UL muting symbols with UL PT-RS, when transform precoding is not enabled, following agreement was made in RAN1#119 meeting. The UE does not expect the muted REs overlaps the REs occupied by PT-RS. It is because for CP-OFDM, PT-RS pattern is not changed between muting symbol and non-muting symbol. Thus, there is no implementation impacts for transmitting/receiving the PT-RS on UL muting symbol.

Agreement If an UL muting symbol overlaps with a symbol containing PT-RS for a PUSCH and transform precoding is not enabled for the PUSCH, the UE does not expect the muted REs overlaps the REs occupied by PT-RS.

When transform precoding is enabled for the PUSCH, whether UL resource muting on a symbol containing PT-RS for a PUSCH was discussed and no consensus was reached. In RAN1#120, the size of DFT for symbols with UL resource muting was discussed. Following two Options were agreed.

Agreement The following agreement replaces the earlier agreement from RAN1#119 If transform precoding is enabled for a PUSCH, the following options are provided for the purpose of specification drafting (how UE implementation is done is left to each company). - Option 1: The DFT size is modified as MscPUSCH2 for symbol(s) with UL resource muting - The MscPUSCH2 modulated symbols are transformed to the frequency domain with a DFT of size MscPUSCH2, and the result is mapped to the MscPUSCH2 unmuted scheduled subcarriers. - Option 2: The DFT size is MscPUSCH for symbol(s) with UL resource muting - Option 2A: The block of MscPUSCH2 modulated symbols is repeated before DFT. After DFT, the MscPUSCH2 non-zero even-indexed subcarriers are extracted and then mapped to the unmuted scheduled subcarriers. - Option 2B: The block of MscPUSCH2 modulated symbols is extended to MscPUSCH modulated symbols by inserting ‘0’ after every modulated symbol. After DFT, the first MscPUSCH2 subcarriers are extracted and then mapped to the unmuted scheduled subcarriers. Note: MscPUSCH is the total number of subcarriers of the PUSCH. Note: Above options are mathematically equivalent. Note: It is up to the spec editor how to capture the above as long as the specification captures the above in a mathematically identical manner.

In RAN1#120bis, when transform precoding is enabled and an UL muting symbol overlaps a symbol containing PT-RS, following alternatives were discussed for down-selection.

Agreement When transform precoding is enabled and an UL muting symbol overlaps a symbol containing PT-RS, down-select from the following alternatives in RAN1#121 - Alt-3 - For PT-RS insertion in a block of MscPUSCH2 samples prior to transform precoding - NgroupPT-RS and Nsampgroup are determined as legacy from 38.214 Table 6.2.3.2-1 - PT-RS sample positions are determined as legacy, but only for the first NgroupPT-RS2 groups - Note: After PT-RS insertion, all of the mathematically equivalent DFT Options 1, 2A, and 2B from the RAN1#120 agreement are applicable, where the choice is up to UE implementation - Alt-4 - UL resource muting is not applied in the PUSCH symbol containing PT-RS

When transform precoding is enabled, PTRS is chunked inserted before DFT, and it reflects to every subcarrier in frequency domain. If UL resource muting is applied to symbol with PTRS, PTRS can only be inserted in PUSCH with the size of MscPUSCH/2, where MscPUSCH is the number of subcarriers for the PUSCH.

For Alt-3, PT-RS sample positions are determined as legacy, but only for the first NgroupPT-RS2 groups. Based on previous agreement, PT-RS samples located in NgroupPT-RS2 groups would be repeated to maintain unchanged number of PT-RS samples. For (NgroupPT-RS,NsampPT-RS)=2,4 or 4,4 or 8,4, the position of PT-RS samples is different with legacy pattern. It would need additional handling in receiver side and the phase tracking performance would be affected. Thus, we prefer Alt-4, i.e., UL resource muting is not applied in the PUSCH symbol containing PT-RS.

Proposal 2: When transform precoding is enabled and an UL muting symbol overlaps a symbol containing PT-RS, UL resource muting is not applied in the PUSCH symbol containing PT-RS.

L1 based UE-to-UE CLI measurement and reporting

Measurement resources

If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI for aperiodic CLI reporting and the first symbol of the aperiodic SRS-RSRP measurement resource or CLI-RSSI measurement resources is smaller than the UE reported threshold beamSwitchTiming, the following agreement was achieved. There are two options for down selection.

Agreement If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI for aperiodic CLI reporting and the first symbol of the aperiodic SRS-RSRP measurement resource or CLI-RSSI measurement resources is smaller than the UE reported threshold beamSwitchTiming, down-select from the following alternatives: - Alt.1: UE does not expect to be configured with a scheduling offset between the last symbol of the PDCCH carrying the triggering DCI for AP CLI and the first symbol of the aperiodic CLI SRS-RSRP or CLI-RSSI resources is smaller than the UE reported threshold beamSwitchTiming. - Alt.2: Same default beam rules as AP CSI-RS resource defined in 38.214 can be applied to AP CLI SRS-RSRP or CLI-RSSI resource. FFS: whether Alt.1 or Alt.2 has any specification impact.

From our perspective, default beam rules defined in spec in 38.214 for AP CSI-RS can be reused. We support Alt.2 to keep same rules as legacy.

Proposal 3: Support Alt.2: Same default beam rules as AP CSI-RS resource defined in 38.214 can be applied to AP CLI SRS-RSRP or CLI-RSSI resource.

Measurement reporting

In last meeting, following proposal was discussed.

- Proposal 2-5 Alt.1 The valid symbol type for CSI derivation configured in CSI-ReportConfig is not applicable for L1 UE-to-UE CLI measurement and reporting. - UE shall perform L1 UE-to-UE CLI measurement only in SBFD symbols Alt.2 The valid symbol type for CSI derivation configured in CSI-ReportConfig is not applicable for L1 UE-to-UE CLI measurement and reporting. - UE shall perform L1 UE-to-UE CLI measurement based on the valid symbol type configured in CSI-ReportConfig - Note: L1 CLI measurement is supported in both SBFD symbols and non-SBFD symbols

From our perspective, the valid symbol type for CSI derivation configured in CSI-ReportConfig is not applicable is not applied to for L1 UE-to-UE CLI measurement and reporting. Although L1 based UE-to-UE CLI measurement and reporting is based on existing CSI framework. The motivation to introduce L1 UE-to-UE CLI measurement and reporting is to obtain the interference of uplink transmission within UL subband of one UE in SBFD symbol to downlink reception within DL subband of another UE. So UE shall perform L1 UE-to-UE CLI measurement only in SBFD symbols. There is no need to perform L1 UE-to-UE CLI measurement on non-SBFD symbols. We prefer unified principle for UL resource muting and UE-to-UE CLI measurement and reporting. Thus, we prefer Option 1.

Proposal 4: The valid symbol type for CSI derivation configured in CSI-ReportConfig is not applicable for L1 UE-to-UE CLI measurement and reporting.

- UE shall perform L1 UE-to-UE CLI measurement only in SBFD symbols

In RAN1#120 meeting, it was agreed that the mapping order of CSI fields for cli-SRS-RSRP and cli-RSSI is provided in Table 1 with following modifications based on Table 6.3.1.1.2-8 defined in TS38.212.

Agreement For discussion purpose, SRS-RSRP measurement resource index is referred to as SRS-RSRP-MRI and CLI-RSSI measurement resource index is referred to as CLI-RSSI-MRI. The mapping order of CSI fields of one report for SRS-RSRP-MRI/SRS-RSRP or CLI-RSSI-MRI/CLI-RSSI reporting is provided in Table 1 with following modifications based on Table 6.3.1.1.2-8 defined in TS38.212 - CRI and SSBRI is replaced with SRS-RSRP-MRI or CLI-RSSI-MRI. - Capability index is crossed out in the table. - RSRP is replaced with L1-SRS-RSRP - L1-CLI-RSSI is added in the table if the CSI report is associated with CLI-RSSI measurement resource. Table 1: Mapping order of CSI fields of one report for SRS-RSRP-MRI/SRS-RSRP or CLI-RSSI-MRI/CLI-RSSI CSI report number; CSI fields. CSI report #n; SRS-RSRP-MRI or CLI-RSSI-MRI #1 as in Table 2, if reported. SRS-RSRP-MRI or CLI-RSSI-MRI #2 as in Table 2, if reported. …. SRS-RSRP-MRI or CLI-RSSI-MRI #4 as in Table 2, if reported. L1-SRS-RSRP or L1-CLI-RSSI #1 as in Table 2, if reported. Differential L1-SRS-RSRP or L1-CLI-RSSI #2 as in Table 2, if reported. …. Differential L1-SRS-RSRP or L1-CLI-RSSI #4 as in Table 2, if reported. Table 2. Bitwidth for SRS-RSRP-MRI, SRS-RSRP, CLI-RSSI-MRI and CLI-RSSI Field; Bitwidth. SRS-RSRP-MRI; ⌈log2KsSRS-RSRP⌉. CLI-RSSI-MRI; ⌈log2KsCLI-RSSI⌉. L1-SRS-RSRP; 7. L1-CLI-RSSI; 7. Differential RSRP; 4. Differential RSSI; 4. KsSRS-RSRP: Number of SRS-RSRP measurement resources in the corresponding resource set KsCLI-RSSI: Number of CLI-RSSI measurement resources in the corresponding resource set

The remaining issue is whether/how to report differential value in case the largest measurement is out of range. The following proposal 2-7 was discussed in last meeting but no consensus was reached.

#### Proposal 2-9 If the number of reported L1-SRS-RSRP measurement resources is M, and the number of L1-SRS-RSRP measurements which are either >=-44dBm or infinity (infinity means UE cannot detect SRS due to too strong signal to measure) is X (X<=M), down-select from the following alternatives Alt.1: The following are reported within one report - An indicator corresponding to a codepoint mapping to out-of-range (L1-SRS-RSRP >= -44 dBm) or infinity: 7 bits - A list of “out-of-range” indications based on differential measurement reporting table are reported for each of the X-1 L1-SRS-RSRP measurement: (X-1)*4bits - A codepoint defined as ‘out-of-range’ is introduced in the differential measurement reporting table - For the M-X L1-SRS-RSRP measurements which are within the reporting range (L1-SRS-RSRP <-44Bm), a differential reporting method is adopted using -44dbm as the reference: (M-X)*4bits Alt.2: The following are reported within one report - An indicator corresponding to a codepoint mapping to out-of-range (L1-SRS-RSRP >= -44) or infinity: 7 bits - For the M-1 L1-SRS-RSRP measurement, a differential reporting method is adopted using -44dbm as the reference: (M-1)*4bits

The following Table 10.1.22.1.2-1 defined in TS38.133 can be used for L1 SRS-RSRP measurement report mapping. If the measured SRS-RSRP corresponding to codepoint (98) is ‘Infinity’, it means UE cannot detect SRS due to too strong signal to measure. In this case, UE does not need to report differential L1-SRS-RSRP in Table 1. gNB would not need to demodulate the differential value if the codepoint (98) is received. So there is no need to define new differential indicator for the other measurement that are out of range. It would lead additional specification work and UE implementation complexity. Thus, there is not necessary to specify new UE behaviors to specifically handle the differential value report in case the largest measured L1-SRS-RSRP is out of range.

Table 10.1.22.1.2-1: SRS-RSRP measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| SRS-RSRP_0 | SRS-RSRP<-140 | dBm |
| SRS-RSRP_1 | -140≤ SRS-RSRP<-139 | dBm |
| SRS-RSRP_2 | -139≤ SRS-RSRP<-138 | dBm |
| SRS-RSRP_3 | -138≤ SRS-RSRP<-137 | dBm |
| SRS-RSRP_4 | -137≤ SRS-RSRP<-136 | dBm |
| .. | .. | … |
| SRS-RSRP_95 | -46≤ SRS-RSRP<-45 | dBm |
| SRS-RSRP_96 | -45≤ SRS-RSRP<-44 | dBm |
| SRS-RSRP_97 | -44≤ SRS-RSRP | dBm |
| SRS-RSRP_98 | Infinity |  |
| Note: ‘Infinity’ means that UE cannot detect SRS due to too strong signal to measure. |  |  |

Proposal 5: No special handling is needed when the largest measurement is out of range.

Conclusion

In this contribution, we discussed the potential normative work for CLI handling. The conclusion and proposals are given below:

Proposal 1: UL resource muting cannot be applied in non-SBFD symbols.

Proposal 2: When transform precoding is enabled and an UL muting symbol overlaps a symbol containing PT-RS, UL resource muting is not applied in the PUSCH symbol containing PT-RS.

Proposal 3: Support Alt.2: Same default beam rules as AP CSI-RS resource defined in 38.214 can be applied to AP CLI SRS-RSRP or CLI-RSSI resource.

Proposal 4: The valid symbol type for CSI derivation configured in CSI-ReportConfig is not applicable for L1 UE-to-UE CLI measurement and reporting.

- UE shall perform L1 UE-to-UE CLI measurement only in SBFD symbols

Proposal 5: No special handling is needed when the largest measurement is out of range.

References

- RP-241614, WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei

- TR 38.858v2.0, Study on Evolution of NR Duplex Operation

- RP-241614, WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, RAN#104, June 17-20, 2024

- 3GPP, Chairman’s Notes, RAN1 #120bis

- R1-2503036, Summary #2 of CLI handling, Moderator (Huawei)