3GPP TSG RAN WG1 #121                                                       R1-2503522

St Julian’s, Malta, May 19th – 23th, 2025

Agenda Item:	9.6.1

Source:	Spreadtrum, UNISOC

Title: 	Discussion on LP-WUS and LP-SS design

Document for:	Discussion and decision

Introduction

It was agreed in WID [1] to specify LP-WUS and LP-SS design.

- To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK bit - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

In the contribution, we show our views on LP-WUS/LP-SS design.

LP-WUS design

In RAN1 previous meeting, the following agreements and working assumption had been achieved.

Agreement For idle mode, regarding the maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell: - support maximum 16 candidates overlaid sequences for M=1 - support maximum 8 candidates overlaid sequences for M=2 - For candidate overlaid sequences across all OOK ON chips of LP-WUS, the number of roots (in specification) is up to [FFS: X], FFS whether the number of roots can be different for different M value. - FFS: The number of overlaid sequences applicable for a UE is no more than 2 per OOK ON chip. Agreement For RRC connected mode, maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell, - support maximum 16 candidates overlaid sequences for M=1 - support maximum 8 candidates overlaid sequences for M=2 - support maximum 4 candidates overlaid sequences for M=4 (agreed in RAN1#119) - For candidate overlaid sequences across all OOK ON chips of LP-WUS, the number of roots (in specification) is up to [FFS: X], FFS whether the number of roots can be different for different M value. - FFS: The number of overlaid sequences applicable for a UE is no more than 2 per OOK ON chip. - FFS: Whether or not the candidate set of overlaid sequences change across OOK ON chips Working Assumption At least for FR1, support maximum 2 roots to be used for overlaid OFDM sequences for M=1/2/4 (max 2 for each M value). - Different roots can be picked for different M values. Agreement For the root(s) used for overlaid OFDM sequences, gNB can configure any value from 1 ~ (Bzc -1), where Bzc is given by the largest prime number such that BZC<LZC, where LZC is the overlaid OFDM sequence length.

Regarding how to configure multiple overlaid OFDM sequence, we prefer that the number of roots N2 and root index(s) can be configurable by the network to have more flexibility. The number of CS and corresponding cyclic shift value is derived by the number of sequences and number of roots.

Proposal 1: Number of roots N2 and root index(s) can be configurable by the network.

Proposal 2: The number of CS and corresponding cyclic shift value is derived by the number of sequences and number of roots.

LP-SS design

In RAN1#120b, the following agreements on LP-SS had been achieved [2].

Agreement Regarding whether to support additional sync signal to LP-SS, support the following for LP-WUS with M=1, M=2: - Option 1A: No additional sync signal, LP-SS periodicity =320ms. - Additionally support LP-SS periodicity of 160ms. Agreement For the M value for LP-WUS and LP-SS, supports: - Alt2: The M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS. - At least for M values larger than 1 (applies to both LP-WUS and LP-SS). - FFS: If M=1 is configured for either of LP-WUS or LP-SS, M=1 is configured for both.

For LP-WUS with M=1, M=2, it was agreed that no additional sync signal is needed to be introduced in last meeting. For M=4, in our views, with LP-SS periodicity of 160ms, there is no need to introduce additional sync signal.

Proposal 3: There is no need to introduce additional sync signal for LP-WUS with M=4.

For M values larger than 1, it was agreed that the M values for LP-WUS and LP-SS can be configured to be same or different and M value for LP-WUS cannot be larger than that of LP-SS in last meeting. For M=4, we support that if M=1 is configured for either of LP-WUS or LP-SS, M=1 is configured for both.

Proposal 4: If M=1 is configured for either of LP-WUS or LP-SS, M=1 is configured for both.

Frequency resource location of LP-WUS/LP-SS

In previous RAN1 meeting, the following agreements on frequency resource location of LP-WUS/LP-SS in RRC connected mode had been achieved.

Agreement For RRC connected, LP-WUS frequency resource can be outside of active DL BWP but has to be within the same carrier as the active DL BWP - Basic capability is LP-WUS, if present, frequency resource within active DL BWP. LP-WUS frequency resource outside of active DL BWP is subject to separate UE capability - No RAN1 optimization specific to the case where LP-WUS frequency resource is outside of active DL BWP Agreement For RRC idle/inactive, support Alt 1 for both Redcap and non-redcap UEs: - Alt 2: LP-WUS/LP-SS frequency resource can be out of initial DL BWP where UE receives paging but within the same carrier. - FFS whether to define the maximum frequency gap between LP-WUS/LP-SS and SSB.

The remaining issue of frequency resource location for LP-WUS/LP-SS is how to configure the frequency resource for LP-WUS/LP-SS. LP-WUS/LP-SS bandwidth is fixed as 11 PRBs at least for channel BW no smaller than 5MHz per RAN1 agreement, so the offset to determine LP-WUS/LP-SS starting PRB needs to be configured. Considering LP-WUS can be out of active BWP for RRC connected mode and LP-WUS/LP-SS frequency resource can be out of initial DL BWP where UE receives paging but within the same carrier, it is better to have unified configuration for all RRC states. Therefore, for LP-WUS/LP-SS frequency resource configuration, PRB offset with reference to carrier boundary determined by offsetToCarrier corresponding to SCS of the active BWP or legacy DL initial BWP is configured by gNB.

Proposal 5: for LP-WUS/LP-SS frequency resource configuration, PRB offset with reference to carrier boundary determined by offsetToCarrier corresponding to SCS of the active BWP or legacy DL initial BWP is configured by gNB.

Conclusion

We have the following proposals.

Proposal 1: Number of roots N2 and root index(s) can be configurable by the network.

Proposal 2: The number of CS and corresponding cyclic shift value is derived by the number of sequences and number of roots.

Proposal 3: There is no need to introduce additional sync signal for LP-WUS with M=4.

Proposal 4: If M=1 is configured for either of LP-WUS or LP-SS, M=1 is configured for both.

Proposal 5: for LP-WUS/LP-SS frequency resource configuration, PRB offset with reference to carrier boundary determined by offsetToCarrier corresponding to SCS of the active BWP or legacy DL initial BWP is configured by gNB.

Reference

- RP-234056, “New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, CMCC, RAN#102, Dec. 11th - 15th, 2023.

- 3GPP RAN1#120b Chairman’s Notes, April 07th – 11th, 2025.