3GPP TSG RAN WG1 #121      	                                    	R1-2503531

Saint Julian, Malta, May 19th – 23th, 2025

Agenda Item:	9.11.5

Source:	Spreadtrum, UNISOC

Title:	Discussion on IoT-NTN TDD mode

Document for:	Discussion and decision

Introduction

The WID on IoT-NTN TDD mode is approved in [1]. The objectives of the work item on IoT-NTN TDD mode are the following.

The work item aims to specify enhancements for NB-IoT NTN to enable NTN operation with a NB-IoT TDD mode leveraging commonalities with half-duplex NB-IoT FDD NTN, by defining a new NB-IoT TDD mode for NTN based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures for the NB-IoT operation in the targeted unpaired MSS allocated band (TN deployment is not expected in this band). The feature is not intended to be applicable to existing 3GPP bands. The work objectives assume the following: - LEO @600 km and @1200 km orbit respectively, with set-1 satellite parameters as reference scenarios (See 3GPP TR 36.763) - Target the 1616-1626.5 MHz MSS allocated band - Standalone deployment with anchor and non-anchor carriers (i.e. operating in carrier(s) used only for NB-IoT) - Operate with Earth fixed Tracking area, with either Earth fixed cells or Earth moving cells for NGSO - The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes (U) and set of usable contiguous DL subframes (D), and guard periods, which is periodic every N radio frames, with N=9 for the target MSS allocated band. This work item includes the following objectives: - Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2] - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period.RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band. - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4] - Specify a new NB-IoT TDD operating NTN band for the MSS allocation spanning 1616-1626.5 MHz for DL and UL, based on the outcome of the study, to be used as example band for this WI [RAN4]. - Specify band numbering - Specify SAN and UE RF characteristics - Specify DL and UL channelization. - Specify channel bandwidth as 200 kHz - Note1: No NTN-NTN coexistence study needed. - Note2: Leverage existing work as much as possible for TN-NTN coexistence of adjacent bands Note3: Reuse the existing RAN4 requirements as much as possible (no intention to have any relaxation of the RF emissions requirements)

In this contribution, we will share our views on the specification impacts of IoT-NTN TDD mode.

DL gap

In RAN1 #120bis, there are three candidate options for how to handle DL gap [2].

Agreement For the issue of handling DL gaps in NB-IoT NTN TDD mode, down-select among the following options: - Option 1: DL gaps do not apply for NB-IoT NTN TDD mode. - Option 2: DL gaps apply based on current specifications. - Option 3: DL gaps apply, where new periodicity(ies) are introduced (either implicity e.g. the periodicity is scaled by a fixed factor F, or explicitly)

DL gaps is introduced to avoid single UE occupying large DL resource. If DL gaps are configured, when PDSCH/PDCCH repetition number (Rmax) >= dl-GapThreshold-r13 {32, 64, 128, 256}, UE stop DL reception in DL gap and postpone reception. When Rmax< dl-GapThreshold-r13, there is no DL gap. With the TDD pattern which leads to postponement for most DL transmissions, the concern about a single UE being served for too long is not addressed by introducing DL gap. There is no significant gain. In our view, DL gaps don’t apply for TDD mode.

If DL gaps are configured, legacy mechanism can be reused. We don’t see the strong motivation to enhance DL gap.

TS 38.211 The NPDSCH transmission can be configured by higher layers with transmission gaps where the NPDSCH transmission is postponed. There are no gaps in the NPDSCH transmission if where is given by the higher layer parameter dl-GapThreshold and is given by [4]. The gap starting frame and subframe is given by where the gap periodicity,, is given by the higher layer parameter dl-GapPeriodicity. The gap duration in number of subframes is given by , where is given by the higher layer parameter dl-GapDurationCoeff. For NPDSCH carrying the BCCH there are no gaps in the transmission.

- Support option 1 or option 2 for DL gaps.

Option 1: DL gaps do not apply for NB-IoT NTN TDD mode.

Option 2: DL gaps apply based on current specifications.

PDCCH monitoring

It was agreed down-select between 2 options for PDCCH postponing [2].

Agreement For the issue of handling NPDCCH postponing, down-select among the following options: - Option 1: No new periodicities are introduced for NPDCCH search space monitoring - FFS: Changes in postponing / overlapping rules, if any - FFS: Introduction of constraints on the configuration of search space parameters (e.g. Rmax, G…), if any - Option 2: New periodicities are introduced for NPDCCH search space monitoring - FFS: Details of the new periodicity, e.g. the existing periodicity is scaled by a fixed factor F, explicit signalling of the new periodicity, reusing the NB-IoT TN TDD periodicities, etc.

For option 1, not introduce new periodicities for NPDCCH monitoring. Following current spec, there maybe an issue that two NPDCCH monitoring occasion are overlapped due to TDD pattern. In our view, UE can skip the monitoring occasions of a search space if they overlap with ongoing monitoring occasion of the same search space.

For option 2, introduce new periodicities for NPDCCH monitoring can’t guaranteed search space always in NB-IoT DL subframes.

- Support option 1 for PDCCH monitoring.

Option 1: No new periodicities are introduced for NPDCCH search space monitoring.

GNSS measurement

In RAN1#120bis, there are 3 candidate options for GNSS measurement.

Agreement For GNSS measurement gap, RAN1 to further discuss at least the following options: - Option 1: No enhancement on the Rel-18 timeline for the start of GNSS gap - Option 2: The start of the GNSS measurement gap starts at a non-D/non-U subframe - Option 3: GNSS measurement gap does not apply for NB-IoT NTN TDD

If a connected mode UE communicates with the network for a long time, it may lose UL synchronization due to GNSS position is invalid, so GNSS measurement gap is introduced. During GNSS measurement gap, UE stop transmission/reception and perform GNSS measurement.

In TDD mode, there are only 8 subframes for UL and DL respectively during 90ms period and a 50ms guard time is introduced. We think UE can perform GNSS measurement in non-D and non-U subframes. it is unnecessary to apply GNSS measurement gap in TDD mode and it is up to UE implementation to perform GNSS measurement.

- Support option 3 for GNSS measurement.

Option 3: GNSS measurement gap does not apply for NB-IoT NTN TDD

Conclusion

In this contribution, we provided views on IoT-NTN TDD mode. In summary, we have following observations and proposals:

- Support option 1 or option 2 for DL gaps.

Option 1: DL gaps do not apply for NB-IoT NTN TDD mode.

Option 2: DL gaps apply based on current specifications.

- Support option 1 for PDCCH monitoring.

Option 1: No new periodicities are introduced for NPDCCH search space monitoring.

- Support option 3 for GNSS measurement.

Option 3: GNSS measurement gap does not apply for NB-IoT NTN TDD

Reference

- RP- 243293, “New WID on introduction of IoT-NTN TDD mode”, Moderator (RAN1 Vice-Chair).

- 3GPP, Chairman’s Notes, RAN1 #120bis.