3GPP TSG RAN WG1 #121			R1-2503535

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:	9.15.11

Source: 	Spreadtrum, UNISOC

Title: 	Discussion on UE features for multi-carrier enhancement

Document for:	Discussion and decision

Introduction

In this contribution, we provide our views on open issues of UE features for multi-carrier enhancement.

FGs for different SCS/carrier type among co-scheduled cells

There were two alternatives for FG 66-1/2, Alt-1 and Alt-2. We support the direction of Alt-2. It is concise and more aligned with Rel-18 FG structure.

### Proposal 1 – Alt-1 (i.e., combination is always “different SCS AND different carrier type”): Introduce the following FG 66-1b/66-2b for multi-cell PDSCH/PUSCH scheduling by DCI format 1_3/0_3 with different carrier type AND different SCS - FFS whether to define FG(s) for same SCS and different carrier type, and for different SCS and same carrier type ### Proposal 1 – Alt-2 (i.e., combination is “different SCS AND different carrier type” or “different SCS AND same carrier type”): - FFS Whether to define FG(s) for “same SCS AND different carrier type” 66-1b; Multi-cell PDSCH scheduling by DCI format 1_3 with different SCS and/or different carrier type for the cells in the set; 1. UE supports monitoring DCI format 1_3 for DL scheduling with different SCS and/or different carrier type for the cells in the set 2. Supported applicable combinations of [two] sets of (carrier types, SCS) for the cells in the set: {(FR1 FDD, 15kHz), (FR1 TDD, 30kHz), (FR2-1, 60kHz), (FR2-1, 120kHz)} and one or more set(s) of (carrier type, SCS) for scheduling cell: {(FR1 FDD, 15kHz), (FR1 TDD, 30kHz)} from the band combination FFS: Other combination(s) of (carrier type, SCS) FFS: Whether to limit the number of cells supported in the set of cells FFS: Whether to limit FR1 TDD and/or FR2-1 to be licensed band only FFS: other component(s); Yes; N/A; Multi-cell PDSCH scheduling by DCI format 1_3 with different SCS and different carrier type for the cells in the set is not supported; [Per BC]; [n/a]; [n/a]; [n/a]; Optional with capability signalling. 66-2b; Multi-cell PUSCH scheduling by DCI format 0_3 with different SCS and/or different carrier type for the cells in the set; 1. UE supports monitoring DCI format 0_3 for UL scheduling with different SCS and/or different carrier type for the cells in the set 2. Supported applicable combinations of [two] sets of (carrier types, SCS) for the cells in the set: {(FR1 FDD, 15kHz), (FR1 TDD, 30kHz), (FR2-1, 60kHz), (FR2-1, 120kHz)} and one or more set(s) of (carrier type, SCS) for scheduling cell: {(FR1 FDD, 15kHz), (FR1 TDD, 30kHz)} from the band combination FFS: Other combination(s) of (carrier type, SCS) FFS: Whether to limit the number of cells supported in the set of cells FFS: Whether to limit FR1 TDD and/or FR2-1 to be licensed band only FFS: other component(s); Yes; N/A; Multi-cell PUSCH scheduling by DCI format 0_3 with different SCS and different carrier type for the cells in the set is not supported; [Per BC]; [n/a]; [n/a]; [n/a]; Optional with capability signalling.

For 66-1b and 2b proposed by Moderator, they seems like scheduling cell is out of set. We suggest having basic feature as scheduling cell in the set, then scheduling cell and co-scheduled cells share two set of SCS and/or carrier type. Another FG can be for scheduling cell out of set, SCS and/or carrier types can be different for scheduling cell and scheduled cells.

FG 66-1

FG 66-1: Multi-cell PDSCH scheduling by DCI format 1_3 with different SCS for the cells in the set with scheduling cell in the set of cells

Components:

- UE supports monitoring DCI format 1_3 for DL scheduling for the cells in the set, where scheduling cell is included in a set of cells in same PUCCH group

- Scheduling cell in the set of cells, it is PCell if set of cells includes PCell, and SCell if set of cells includes only SCells.

- Supported applicable combinations of two sets of (carrier types, SCS) for the cells in the set: {(FR1 FDD, 15kHz), (FR1 TDD, 30kHz), (FR2-1, 60kHz), (FR2-1, 120kHz)}

- Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4}

- Max number of sets of cells supported by UE across PUCCH groups: Candidate value set of {1, 2, 3, 4, 5, 6, 7, 8}

- Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {1, 2, 3, 4}

- The number of unicast DL DCIs to process per N consecutive slots of scheduling cell for a set of cells configured for multi-cell PDSCH scheduling by DCI format 1_3

  - One DCI format 1_3 for the set of cells and,

  - One unicast DL DCI formats 1_0/1_1/1_2 (if supported) for each of the cells that are not scheduled by DCI 1_3

  - For SCS of Scheduling cell equals to lower SCS among the scheduled cells, N = 1.

  - For SCS of Scheduling cell equals to lower SCS among the scheduled cells, N is based on pair of (SCS1, SCS2): N=2 for (30,15), (60,30), (120,60) and N=4 for (60,15), (120,30), N = 8 for (120,15)

- Monitoring SS set(s) for DCI format 1_3 for a set of cells for the following cases

  - Search space set configuration for DCI format 1_3 for the set of cells is provided only on the scheduling cell, or;

  - Search space set configurations for DCI format 1_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being NOT in the set of cells

Type of FG: per BC

Prerequisite: FG 49-1

FG 66-1a

FG 66-1: Multi-cell PDSCH scheduling by DCI format 1_3 with different SCS for the cells in the set with scheduling cell out of the set of cells

Components:

- UE supports monitoring DCI format 1_3 for DL scheduling for the cells in the set, where scheduling cell is not included in a set of cells in same PUCCH group

- Scheduling cell is PCell or SCell, and a set of cells includes only SCellsc

- For the set of co-scheduled cells, two sets of (carrier types, SCS) for the cells in the set: {(FR1 FDD, 15kHz), (FR1 TDD, 30kHz), (FR2-1, 60kHz), (FR2-1, 120kHz)}; Scheduling cell one or more set(s) of (carrier type, SCS). For scheduling cell, {(FR1 FDD, 15kHz), (FR1 TDD, 30kHz)} from the band combination

- Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4}

- Max number of sets of cells supported by UE across PUCCH groups: Candidate value set of {1, 2, 3, 4, 5, 6, 7, 8}

- Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {1, 2, 3, 4}

- The number of unicast DL DCIs to process per N consecutive slots of scheduling cell for a set of cells configured for multi-cell PDSCH scheduling by DCI format 1_3

  - One DCI format 1_3 for the set of cells and,

  - One unicast DL DCI formats 1_0/1_1/1_2 (if supported) for each of the cells that are not scheduled by DCI 1_3

  - For SCS of Scheduling cell equals to lower SCS among the scheduled cells, N = 1.

  - For SCS of Scheduling cell equals to lower SCS among the scheduled cells, N is based on pair of (SCS1, SCS2): N=2 for (30,15), (60,30), (120,60) and N=4 for (60,15), (120,30), N = 8 for (120,15)

Type of FG: per BC

Prerequisite: FG 49-1b

FG 66-2/FG 66-2a

FG structure, component, Type, and prerequisite are same as DL.

FGs for one or multiple PUSCHs/PDSCHs per scheduled cell

During the last RAN1 meeting, the following agreement was achieved. We provide our further consideration on top of it.

Agreement

Introduce the following 2 FGs for Rel-19 multi-cell multi-PDSCH/PUSCH scheduling:

| 66-3 | Multi-cell multi-PDSCH scheduling by DCI format 1_3 | 1. UE supports DCI format 1_3 for DL scheduling of [one or] more PDSCH(s) per cell in the set of cells FFS: 2. supported HARQ enhancement. details for component 2 FFS: Whether to limit this FG to particular FR and/or SCS FFS: Whether to separate this FG per FR and/or SCS FFS: Whether/how to define component to report the supported number of maximum schedulable PDSCHs per cell FFS: Whether/how to define component to report the supported maximum total number of schedulable PDSCHs per DCI FFS: other component(s) |  | Yes | N/A | Multi-cell multi-PDSCH scheduling by DCI format 1_3 is not supported |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 66-4 | Multi-cell multi-PUSCH scheduling by DCI format 0_3 | 1. UE supports DCI format 0_3 for UL scheduling of [one or] more PUSCH(s) per cell in the set of cells FFS: Whether to limit this FG to particular FR and/or SCS FFS: Whether to separate this FG per FR and/or SCS FFS: Whether/how to define component to report the supported number of maximum schedulable PUSCHs per cell FFS: Whether/how to define component to report the supported maximum total number of schedulable PUSCHs per DCI FFS: other component(s) |  | Yes | N/A | Multi-cell multi-PUSCH scheduling by DCI format 0_3 is not supported |  |  |  |  |  | Optional with capability signalling |

FG 66-3

In component 1, one PDSCH can be removed, due to it is capability which can only include the new features, and legacy feature FG 49-1 has already cover one PDSCH scheduling.

In component 2, FG 49-1 can report {type 1, type2, type 1 and type 2}. Type1 is same as Rel-18, and only Type 2 is enhanced. So UE only need to supporte Type 2 HARQ-ACK codebook enhancement, if Type2 is reported in FG 49-1.

In component 3, SCS is 120kHz, and FR2-1 and 2-2 are the target FR.

Component 6 is legacy component in FG 49-1, It can be with different value if with multi-PDSCH scheduling.

Components:

- UE supports DCI format 1_3 for DL scheduling of [one or] more PDSCH(s) per cell in the set of cells. Due to it is capability which can only include the new features, and legacy feature has already cover one PDSCH scheduling.

- Supporte Type 2 HARQ-ACK codebook if Type2 is reported in FG 49-1

- Co-scheduled cells have same SCS 120kHz, and same carrier type: value set: {FR2-1, FR2-2}, UE reports one or multiple of values from the value set

- Max number of schedulable PDSCHs per cell supported by UE is reported with candidate value set of {2, 4, 8}

- Max number of schedulable PDSCHs per DCI supported by UE is reported with candidate value set of {4, 8, 16, 32}

- Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4}

Type of FG: per BC

Prerequisite: FG 49-1/1b

FG 66-4

Component, Type, and prerequisite are same as DL

References

- RP-242904, WI for Multi-Carrier Enhancements for NR Phase 3.

- R1-2502970	Session Notes of AI 9.15.11	Ad-Hoc Chair (NTT DOCOMO, INC.)

- R1-2503138	Summary#3 of discussion on Rel-19 NR UE features for MCE for NR Phase 3