3GPP TSG RAN WG1 #121                                            R1-2503537

St Julian's, Malta, May 19th-23rd, 2025

Source:	TCL

Title:	Discussion on line coding/FEC/CRC/repetition aspects for Ambient IoT

Agenda Item:	9.4.2

Document for:	Discussion and Decision

- Introduction

n RAN #106, detailed scope of newWID has been given for Rel-19 further study. Here, we conclude here general scope and RAN 1 scope [1].

- RAN1 scope - PRDCH and PDRCH, which are the only physical channels in R2D and D2R, respectively. - R2D and D2R signal(s) - Multiplexing/multiple access in R2D is by only TDMA, and in D2R is by only TDMA and FDMA. - R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations. - R2D transmission supports only the Manchester line code in TR 38.769 - D2R transmission supports: - Either the Manchester line code in TR 38.769 or no line code (one to be down-selected); and - A corresponding small frequency shift method according to the options in TR 38.769. - R2D does not support FEC. D2R supports only convolutional code with generator polynomials as per TS 36.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis). - PRDCH and PDRCH both support transmission without CRC, and with CRC as per the generator polynomials in TS 38.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis) for 6-bit CRC and 16-bit CRC. Cases to use which length of CRC, or no CRC, to be decided in RAN1. - D2R supports physical layer repetition transmission. R2D does not support physical-layer repetition transmission.

Actually, the deployment scenario focus on below from RAN #106 discussion, and it can be seen gNB sends R2D signaling and receives corresponding D2R signal.

In this contribution, based on these agreements and FL’s suggestion, we will discuss R2D and D2R modulation related topics, including CP handling.

- SFS discussion

RAN1#120 has some conclusion for D2R line coding and SFS[2]:

Proposal 3.1-1-v4 The potential values of D2R chip duration Tchip, bit duration Tb, and SFS factor R, are shown in the following table: - FFS further down-selections of Tchip, Tb, and R. - Note: Detailed indication signaling of D2R scheduling information for MCS-like information and frequency domain resources are discussed in AI 9.4.4. Tchip (μs). Tb (μs); 133.33; 66.67; 33.33; 16.67; 11.11; 8.33; 5.56; 4.17; 2.78; 2.08; 1.39; 1.04; 0.69; 0.52. 266.67; R=1; R=2; R=4; R=8; R=12; R=16; R=24; R=32; R=48; R=64; R=96; R=128. 133.33; R=1; R=2; R=4; R=6; R=8; R=12; R=16; R=24; R=32; R=48; R=64; R=96; R=128. 66.67; R=1; R=2; R=4; R=6; R=8; R=12; R=16; R=24; R=32; R=48; R=64. 33.33; R=1; R=2; R=4; R=6; R=8; R=12; R=16; R=24; R=32. 22.22; R=1; R=2; R=4; R=8; R=16. 16.67; R=1; R=2; R=4; R=6; R=8; R=12; R=16. 11.11; R=1; R=2; R=4; R=8. 8.33; R=1; R=2; R=4; R=6; R=8. 5.56; R=1; R=2; R=4. 4.17; R=1; R=2; R=4. 2.78; R=1; R=2. 2.08; R=1; R=2. 1.39; R=1. 1.04; R=1. Note: For further down-selection regarding the bit duration, - The first 3 rows are suggested to be kept, which provides better multiplexing capability in the frequency domain. - The last 2 rows are suggested to down-select to 1, which achieves the competitive peak data rate larger than 640 kbps. The remaining rows are suggested to down-select to 4..

RFID has defined the duration of one basic time unit is in the range of 6.25 us to 25us [3]. The user experienced data rate target is in the range of 0.1kbps~ 5kbps for R2D and D2R transmission [4]. To meet the requirement of data rate, assuming 1.92MHz sampling clock and we think 133.33μs could be considered to transmit D2R signal with minimal data rate (e.g., ~0.1kbps). Except for 133.33us as D2R chip duration, other values could be further considered. Like FL summary’s proposal in last meeting, {66.67, 33.33, 16.67, 11.11, 8.33, 5.56, 4.17, 2.78, 2.08, 1.39}μs could be further considered for the requirement of data rate. For the smaller ones, it is not necessary to consider due to it may not meet the requirement of data rate regulated in TR38.848.

Observation 1: The user experienced data rate target is in the range of 0.1kbps~ 5kbps for R2D and D2R transmission.

Proposal 1: The maximum D2R chip duration is 133.33 μs (assuming 1.92MHz sampling clock).

Proposal 2: The minimum D2R chip duration is 1.39 μs.

Proposal 3: The set of D2R chip duration is {133.33, 66.67, 33.33, 16.67, 11.11, 8.33, 5.56, 4.17, 2.78, 2.08, 1.39}μs and further down-selection could be considered if possible.

It can be observation from RAN 1 #118b that there are different Btx,D2R and transmission power of the D2R transmissions due to XOR operation. In our understanding, to reduce detection complexity at reader side and alleviate specification efforts, option 2 of XOR operation and 2SB should not considered in Rel-19 WI phase.

Observation 2: It can be observation from RAN 1 #118b that there are different Btx,D2R and transmission power of the D2R transmissions due to XOR operation.

Proposal 4: Support option 1 or no D2R line code (e.g., by using a square-wave corresponding to the small frequency-shift)

In our understanding, R=2n (including R=1) all is more reasonable than R=2n in Rel-19 due to more large range of SFS factor could be consider and save frequency resource. However, SFS factor setting is associated with SCS/harmonic components/guard band defined in RAN4. If R=2n could avoid the impact of harmonic components (like R=6, R=10, etc.), we prefer to support R=2n as common SFS factor. Companies report R=2 should not considered as SFS factor due to the impact of non-ideal factor, but we are open for this issue cause it is also associated with SCS/guard band.

Observation 3: SFS factor setting is associated with SCS/harmonic components/guard band defined in RAN4.

Proposal 5: Support R=2n as common SFS factor and no special R should be down-selected in Rel-19.

- D2R repetition

- 3.1 Block-level repetition

Last meeting gives two proposals in FL summary, here we copy it as following

[H] Proposal 5.1-v2 For the number of block-level repetitions, down-select from the following options: - Option 1: {1, 2}. - Option 2: {1, 2, 4}. - Note: When the number of block-level repetition is 1, it indicates no repetition. [M] Proposal 5.2-v1 For block-level repetition of a D2R transmission, the repeated blocks are transmitted in the single PDRCH of the D2R transmission.

Rel-19 focus on indoor scenarios and device 1 has quiet limited capability. To enhance transmission reliability, we prefer option 2 (maximum number is 4) as the number of block-level repetitions.

Proposal 6: Support option 2 as the number of block-level repetitions.

- Option 2: {1, 2, 4}

- Note: When the number of block-level repetition is 1, it indicates no repetition.

- Block-level repetition focus on Msg 1/Msg 3 block transmission. As shown figure above, Msg 1/Msg 3 may be transmitted by block-level repetition. As the impact of SFO of device 1, each repetition block needs to consider the potential time offset (△t) to avoid the impact of SFO, which is up to the device’s capability and each block length. More importantly, frequency hopping for Msg 1/Msg 3 with small frequency shift could be considered for improving the detection performance at reader side. In addition, repetition number and resources of block-level repetition needs to be carefully considered. When TBS of MSG 1/MSG 3 is larger than one predefined value, maybe block-level repetition is not necessary.

- Observation 4: As the impact of SFO of device 1, each repetition block needs to consider the potential time offset (△t) to avoid the impact of SFO, which is up to the device’s capability and each block length.

- Proposal 7: Consider D2R block-level repetition based on following aspects,

- Frequency resource for each repetition block, e.g., if hopping frequency could be considered for each block

- How to indicate frequency resource and repetition number (R)

- 3.2 Bit-level repetition

As we know, one bit may cover one or multiple chip. For example, one bit includes one chip if no line coding. However, one bit includes multiple chip if Manchester coding is used (including adding repetition number of Manchester). Similar with time and frequency resources configuration of block-level repetition, we think the impact of SFO and frequency hopping with small frequency shift could also be considered. Specially, limited by one bit length, the impact of SFO during one or multiple bit may be ignored, which is up to repetition number of bit-level repetition.

For the number of bit-level repetition, in our understanding, it is up to coverage and access performance and it will impact on D2R frame structure selection. For example, D2R Midamble or Postamble may be necessary as the increase of number of bit-level repetition. One special way is to predefined the repetition number based on TBS and MCK-like, e.g., N={1, 5, 10, 20....}

Observation 5: Limited by one bit length, the impact of SFO during one or multiple bit may be ignored, which is up to repetition number of bit-level repetition.

- Observation 6：For the number of bit-level repetition, it is up to coverage and access performance and it will impact on D2R frame structure selection.

- Proposal 8: Consider D2R bit-level repetition based on following aspects,

- Repetition number (R)

- How to indicate time/frequency resource and repetition number (R)

- Conclusion

In this contribution, we provide our views on R2D and D2R coding/FEC/repetition related topics for AIoT. The observations and proposals are listed as below:

Observation 1: The user experienced data rate target is in the range of 0.1kbps~ 5kbps for R2D and D2R transmission.

Observation 2: It can be observation from RAN 1 #118b that there are different Btx,D2R and transmission power of the D2R transmissions due to XOR operation.

Observation 3: SFS factor setting is associated with SCS/harmonic components/guard band defined in RAN4.

- Observation 4: As the impact of SFO of device 1, each repetition block needs to consider the potential time offset (△t) to avoid the impact of SFO, which is up to the device’s capability and each block length.

Observation 5: Limited by one bit length, the impact of SFO during one or multiple bit may be ignored, which is up to repetition number of bit-level repetition.

- Observation 6：For the number of bit-level repetition, it is up to coverage and access performance and it will impact on D2R frame structure selection.

Proposal 1: The maximum D2R chip duration is 133.33 μs (assuming 1.92MHz sampling clock).

Proposal 2: The minimum D2R chip duration is 1.39 μs.

Proposal 3: The set of D2R chip duration is {133.33, 66.67, 33.33, 16.67, 11.11, 8.33, 5.56, 4.17, 2.78, 2.08, 1.39}μs and further down-selection could be considered if possible.

Proposal 4: Support option 1 or no D2R line code (e.g., by using a square-wave corresponding to the small frequency-shift)

Proposal 5: Support R=2n as common SFS factor and no special R should be down-selected in Rel-19.

Proposal 6: Support option 2 as the number of block-level repetitions.

- Option 2: {1, 2, 4}

- Note: When the number of block-level repetition is 1, it indicates no repetition.

- Proposal 7: Consider D2R block-level repetition based on following aspects,

- Frequency resource for each repetition block, e.g., if hopping frequency could be considered for each block

- How to indicate frequency resource and repetition number (R)

- Proposal 8: Consider D2R bit-level repetition based on following aspects,

- Repetition number (R)

- How to indicate time/frequency resource and repetition number (R)

# References

- 3GPP RAN#106. RP-243312, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR. Madrid, Spain, Dec 9-12, 2024.

- R1-2503107, Summary #6 for coding aspects of physical channel design, RAN 1#120b, Wuhan.

- Class 1 Generation 2 UHF Air Interface Protocol Standard Release 3.0, Jan 2024.

- 3GPP TR 38.848 Technical Specification Group Radio Access Network, V1.0.0, 2023, 9.