3GPP TSG RAN WG1 #121			R1-2503543

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	5

Source:	Samsung

Title:	Discussion on RAN4 LS on the transient period for SBFD operation

Document for:	Discussion

# 1. Introduction

RAN4 sent an LS [1] to RAN1 to inform RAN4’s agreements on transient period between SBFD and non-SBFD. In this contribution, we present our views on the RAN4 agreements and potential problems to be considered by RAN1.

# 2. Discussion

Transient period length between SBFD and non-SBFD

Agreement: - Transient period length between SBFD and non-SBFD: - The existing TDD BS transmitter transient period reused, i.e., 10us for FR1 and 3us for FR2-1.

RAN4 agreed to reuse the existing TDD BS transmitter transient period for transient period length between SBFD symbol and non-SBFD symbol. The transient period length is 10us for FR1 and 3us for FR2-1. The CP duration of an OFDM symbol with 15/30/60/120kHz is around 4.7us/2.3us/1.17us/0.59us, respectively, which is smaller than 10us for FR1 and 3us for FR1. Thus, the transient period may impact the signal quality (and result in a BLER degradation) when an OFDM symbol overlaps with the transient period.

Figure 1. Illustration of two cases of transitions for BS ON/OFF

In legacy TDD systems, in order to ensure the transient period for Case-b (from DL to UL) at gNB, guard symbols right after DL symbols or right before UL symbols are introduced. In order to ensure the transient period for Case-a (from UL to DL) at gNB, a timing advance is applied. The timing advance value consists of two parts. One is cell-specific and another is UE-specific. NTA,offset is the cell-specific TA_offset value. From Table 7.1.2-2 of TS38.133, NTA,offset=25600 (no coexistence with LTE) or 39936 (no coexistence with LTE). This corresponds to 13us or 20us.

For example, in a commercial deployment of the n78 band, 2 OFDM symbols @30kHz SCS are often used as guard symbols. The duration of 2 OFDM symbols @30 kHz SCS is 71us. So, guard symbols right after DL symbols or right before UL symbols allow for around 71us – 13us = 58us. Also, at least 13us or 20us can be ensured for the guard symbols right after UL symbols or right before the DL symbols, which is larger than the transient period. In short, the transient period is located in explicit guard symbol(s) for case-b or absorbed by TA_offset for case-a.

Observation 1. In legacy TDD systems, the TDD BS transmitter transient period is ensured by the explicit guard symbols between DL symbols and UL symbols and the timing advance.

For gNB supporting SBFD, RAN4 has discussed the transient period location. As legacy TDD systems, RAN4 tried to ensure no overlap with the transient period and DL symbol/UL symbol. The following are the relevant agreements.

Figure 2: Illustration of four cases of transitions to/from SBFD symbols Agreement: - RAN4 shall not define an SBFD-architecture dependent requirement. - Transient period between SBFD and non-SBFD is applicable to all four cases: Frequency regions for transition; BS transmitter behavior during transition. Case-A (SBFD symbol to DL symbol); SBFD UL subband and guardband(s); BS transmitter OFF-to-ON. Case-B (SBFD symbol to UL symbol); SBFD DL subband; BS transmitter ON-to-OFF. Case-C (DL symbol to SBFD symbol); SBFD UL subband and guardband(s); BS transmitter ON-to-OFF. Case-D (UL symbol to SBFD symbol); SBFD DL subband; BS transmitter OFF-to-ON. Agreement: - RAN4 reach the conclusion of the location of transient periods: - Case-A: in TA_offset in SBFD symbol(s); - Case-D: in TA_offset in non-SBFD UL symbol(s). Agreement: - For Case-B, guard symbol(s) should be reserved before UL symbols - The transient period can be reserved only in the DL part(s) of SBFD symbol(s) or in the whole channel bandwidth, up to gNB implementation. Agreement: - For the location of transient periods Case-C, - Both Option 1 (guard symbols reserved in non-SBFD DL symbol(s)) and Option 2 (blanked SBFD symbols in the beginning of SBFD UL transmission) shall be allowed, up to gNB implementation.

- Case-A: The transient period is located in TA_offset in SBFD symbol(s). This is exactly the same as Case-a of TDD in Figure 1. So, the transient period does not overlap with DL symbol and UL symbol, as in legacy TDD.

- Case-B: The transient period is located in guard symbol(s) before UL symbols. This is exactly the same as Case-b of TDD in Figure 1. So, the transient period does not overlap with DL symbol and UL symbol, as in legacy TDD.

- Case-C: the following two options are supported by gNB implementation

  - Option 1 (guard symbols reserved in non-SBFD DL symbol(s)). In this case, guard symbols are needed to configured in the DL symbol(s). However, there is no signaling to define the guard symbols between the DL symbols and SBFD symbols.

  - Option 2 (blanked SBFD symbols in the beginning of SBFD UL transmission). In this case, the first SBFD symbols cannot be used for UL transmission. Again, there is no signaling which the SBFD symbol should be blanked.

- Case-D: The transient period is located in TA_offset in non-SBFD UL symbol. This is exactly the same as Case-a of TDD in Figure 1. So, the transient period does not overlap with the DL symbol and the UL symbol, as in legacy TDD.

For Case-A, -B, and -C, our observation is that there is no overlapping by TA_offset or by guard symbol(s). For Case-C, by gNB implementation, one of two options can be chosen. Based on the choice, the impacted OFDM symbols would be different, i.e., the last DL symbols for option 1 and the first SBFD symbol for option 2. It may result in incorrect reception and HARQ combining by the UE (when option 1 is implemented by the gNB) or unnecessary transmission from the UE (when option 2 is implemented by the gNB).

To avoid such a problem, RAN1 should consider one of the two possible alternatives:

- Alt. 1: gNB ensures no overlapping occurs by scheduling

- Alt. 2: gNB configures which option is implemented and which symbols are impacted.

Observation 2. For Case-A, B, and D, the transient period does not overlap with DL symbol and UL symbol.

Observation 3. For Case-C, UE cannot know which symbols are impacted by the transient period.

Proposal 1. RAN1 to consider the following two alternatives to avoid incorrect reception and HARQ combining by the UE or any unnecessary transmission from the UE.

- Alt. 1: gNB ensures no overlapping occurs by scheduling

- Alt. 2: gNB configures which option is implemented and which symbols are impacted.

# 3. Conclusions

In this contribution, we discussed the impacts of RAN4’s transient period between SBFD symbols and non-SBFD symbols. Our observations and proposal are

Observation 1. In legacy TDD systems, the TDD BS transmitter transient period is ensured by the explicit guard symbols between DL symbols and UL symbols and the timing advance.

Observation 2. For Case-A, B, and D, the transient period does not overlap with DL symbol and UL symbol.

Observation 3. For Case-C, UE cannot know which symbols are impacted by the transient period.

Proposal 1. RAN1 to consider the following two alternatives to avoid incorrect reception and HARQ combining by the UE or any unnecessary transmission from the UE.

- Alt. 1: gNB ensures no overlapping occurs by scheduling

- Alt. 2: gNB configures which option is implemented and which symbols are impacted.

# Reference

[1] R1-2503213 (R4-2504767) “LS on the transient period for SBFD operation”; Source: RAN4; RAN4#114bis