3GPP TSG RAN WG1 #121			R1-2503544

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:	7

Source: 	Samsung

Title: 	Discussion on differentiation of sDCI mTRP, mDCI mTRP and sTRP

Document for:	Discussion and Decision

Introduction

In RAN2#124 meeting (RAN1#115 meeting), RAN2 received the reply LS on MIMOevo from RAN1 [1]. In the reply LS, for the issue about simultaneous unified TCI state(s) update for a list of cells, RAN1 gave the answer as following.

Question 2b: Is there any restrictions in configuring the serving cells of one list for sDCI mTRP, mDCI mTRP or sTRP operation? Answer 2b: It has been agreed in RAN1 that cells operated as sDCI mTRP, mDCI mTRP and sTRP should not be configured in the same list, i.e., they should be configured with separate lists for simultaneous unified TCI state(s) update. Other than this, there is no further/other restriction(s).

In RAN2, how to capture the RAN1 agreed restriction was discussed and the solutions were proposed to separate the cells operated as sDCI mTRP, mDCI mTRP and sTRP. For the proposed solutions, RAN2 has following questions would like to check RAN1’s understanding.

In RAN1#120, RAN1 agreed to make an answer on Question 1a and 1b [2], but even until RAN1#120bis, RAN1 cannot make an aligned view on Question 2a and 2b. Hence, this contribution provides Samsung’s view on RAN1 would like to provide some answers for RAN2’s remaining questions.

Discussion

To distinguish the cells operated as sTRP and sDCI mTRP, it is proposed by RAN2 to add the following highlight sentence.

imultaneousU-TCI-UpdateList1, simultaneousU-TCI-UpdateList2, simultaneousU-TCI-UpdateList3, simultaneousU-TCI-UpdateList4 List of serving cells for which the Unified TCI States Activation/Deactivation MAC CE applies simultaneously, as specified in TS 38.321 [3] clause 6.1.3.47. The different lists shall not contain same serving cells. Network only configures in these lists serving cells that are configured with unifiedTCI-StateType. Network should not configure serving cells that are configured with a BWP with different number of coresetPoolIndexes in the same list. For each list, the network ensures that either applyIndicatedTCI-State is configured for all ControlResourceSets in all BWPs of all serving cells or applyIndicatedTCI-State is not configured in any ControlResourcesSet of any BWP of any serving cell.

But some companies understand that applyIndicateTCI-State with value “none” configured for ControlResourceSet indicates sTRP operation and different channels on a BWP may operate differently.

Question 2a: RAN2 would like to ask if RAN1 has any concern on the above highlight sentence?

Answer 2a:

RAN1 has a concern on the above highlight sentence, because the RRC parameter applyIndicatedTCI-State in ControlResourceSet is optionally present and may not be mandatorily configured for all ControlResourceSets in all BWPs of the serving cell operated as sDCI mTRP, hence it cannot clearly distinguish between sDCI mTRP and sTRP.

Question 2b: If there is any concern on Question 2a, what’s RAN1’s suggestion on how to separate the cells operated as sTRP and sDCI mTRP?

Answer 2b: As mentioned in Answer 2a, the RRC parameter applyIndicatedTCI-State cannot clearly distinguish between sDCI mTRP and sTRP. Instead, in RAN1#120bis, some possible options have been agreed as follows [3]:

Agreement To differentiate the cells operated as sTRP vs sDCI mTRP, down-select one of the options: - Alt-1: It is RAN1 consensus that RRC parameter(s) is used to indicate whether a serving cell is in sDCI mTRP operation or sTRP operation. - Option 1: RAN1 agrees that whether RRC parameter applyIndicatedTCI-State is configured in ControlResourceSet can be used to indicate whether a serving cell is in sDCI mTRP operation or sTRP operation. However, for each list, the network ensures that either at least one of the following RRC parameters is configured for all BWPs of all serving cells or none of the following RRC parameters is configured in any BWP of any serving cell. - “applyIndicatedTCI-State-r18” in ConfiguredGrantConfig, ControlResourceSet, PUSCH-Config, SRS-ResourceSet, PDCCH-ConfigCommon, PUCCH-ResourceExt-v1610, or - “applyIndicatedTCI-State-r18” and “applyIndicatedTCI-State2-r18” in CSI-AssociatedReportConfigInfo, or - “applyIndicatedTCI-StateDCI-1-0-r18” in TCI-InDCI-r18. - mappingPattern-r17”, “multipanelSchemeSDM-r18”, “multipanelSchemeSFN-r18” in PUSCH-Config - “repetitionSchemeConfig-r16” in PDSCH-Config - “searchSpaceLinkingId-r17” in searchSpace - “sfnSchemePDCCH-r17” and “sfnSchemePDSCH-r17” in MIMOParam-r17 - “cjt-Scheme-PDSCH-r18” in ServingCellConfig - Two SRS resource sets configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook' or ‘noncodebook’ - Option 2 (No concern): RAN1 agrees that whether RRC parameter applyIndicatedTCI-State is configured in ControlResourceSet can be used to indicate whether a serving cell is in sDCI mTRP operation or sTRP operation. - Alt 2: It is RAN1 consensus that the number of indicated TCI states can be used to indicate whether a serving cell is in sDCI mTRP operation or sTRP operation.

Our views on each of possible options can be described as follows:

- For Alt-1, it is based on the certain condition on RRC configuration.

  - For Option 1, it lists all the relevant RRC parameters to enable certain functionality of single-DCI based multi-TRP scheme. Hence, it works well to distinguish between single-TRP and single-DCI based multi-TRP, even though there are too many RRC parameters which have been defined from Rel-16 to Rel-18. Also, this option can address RAN2’s intention for their question, i.e., condition of RRC configuration/parameter(s). However, some companies argue that full list of RRC parameters are not needed, so some portion of RRC parameters can be only considered in RAN2 specification. Hence, we think that this could be the most promising solution which can be a good starting point to discuss the final reply LS to RAN2.

  - For Option 2, as mentioned in Answer 2a, the RRC parameter applyIndicatedTCI-State itself cannot clearly distinguish between single-TRP and single-DCI based multi-TRP. Hence, it doesn’t work well and should be precluded.

- For Alt-2, it is based on certain condition on dynamic indication of TCI state(s). In RAN1 specification, this condition has been widely used to describe a condition of single-DCI based multi-TRP, so the condition itself is not wrong. However, the thing is, the question from RAN2 is how to distinguish single-TRP and single-DCI based multi-TRP in terms of the condition of RRC configuration/parameter(s). Considering this aspect, even though we include this solution into the final reply LS to RAN2, it would not address RAN2’s question.

Having said that, for the answer of Question 2b, we proposed to select Option 1 in Alt-1.

Proposal. To RAN2, RAN1 can provide the following answers.

Question 2a: RAN2 would like to ask if RAN1 has any concern on the above highlight sentence?

Answer 2a: RAN1 has a concern on the above highlight sentence, because the RRC parameter applyIndicatedTCI-State in ControlResourceSet is optionally present and may not be mandatorily configured for all ControlResourceSets in all BWPs of the serving cell operated as sDCI mTRP, hence it cannot clearly distinguish between sDCI mTRP and sTRP.

Question 2b: If there is any concern on Question 2a, what’s RAN1’s suggestion on how to separate the cells operated as sTRP and sDCI mTRP?

Answer 2b: As mentioned in Answer 2a, the RRC parameter applyIndicatedTCI-State cannot clearly distinguish between sDCI mTRP and sTRP. Instead, it is possible to use all RRC parameters for supporting sDCI mTRP (i.e., relevant RRC parameters for TCI application as well as DL/UL multi-TRP schemes). It is RAN1 consensus that RRC parameter(s) is used to indicate whether a serving cell is in sDCI mTRP operation or sTRP operation. For each serving cell list, the network ensures that either at least one of the following RRC parameters is configured for all BWPs of all serving cells (i.e., the case of single-DCI based multi-TRP) or none of the following RRC parameters is configured in any BWP of any serving cell (i.e., single-TRP).

- “applyIndicatedTCI-State-r18” in ConfiguredGrantConfig, ControlResourceSet, PUSCH-Config, SRS-ResourceSet, PDCCH-ConfigCommon, PUCCH-ResourceExt-v1610, or

- “applyIndicatedTCI-State-r18” and “applyIndicatedTCI-State2-r18” in CSI-AssociatedReportConfigInfo, or

- “applyIndicatedTCI-StateDCI-1-0-r18” in TCI-InDCI-r18.

- mappingPattern-r17”, “multipanelSchemeSDM-r18”, “multipanelSchemeSFN-r18” in PUSCH-Config

- “repetitionSchemeConfig-r16” in PDSCH-Config

- “searchSpaceLinkingId-r17” in searchSpace

- “sfnSchemePDCCH-r17” and “sfnSchemePDSCH-r17” in MIMOParam-r17

- “cjt-Scheme-PDSCH-r18” in ServingCellConfig

- Two SRS resource sets configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook' or ‘noncodebook’

Conclusion

Proposal. To RAN2, RAN1 can provide the following answers.

Question 2a: RAN2 would like to ask if RAN1 has any concern on the above highlight sentence?

Answer 2a: RAN1 has a concern on the above highlight sentence, because the RRC parameter applyIndicatedTCI-State in ControlResourceSet is optionally present and may not be mandatorily configured for all ControlResourceSets in all BWPs of the serving cell operated as sDCI mTRP, hence it cannot clearly distinguish between sDCI mTRP and sTRP.

Question 2b: If there is any concern on Question 2a, what’s RAN1’s suggestion on how to separate the cells operated as sTRP and sDCI mTRP?

Answer 2b: As mentioned in Answer 2a, the RRC parameter applyIndicatedTCI-State cannot clearly distinguish between sDCI mTRP and sTRP. Instead, it is possible to use all RRC parameters for supporting sDCI mTRP (i.e., relevant RRC parameters for TCI application as well as DL/UL multi-TRP schemes). It is RAN1 consensus that RRC parameter(s) is used to indicate whether a serving cell is in sDCI mTRP operation or sTRP operation. For each serving cell list, the network ensures that either at least one of the following RRC parameters is configured for all BWPs of all serving cells (i.e., the case of single-DCI based multi-TRP) or none of the following RRC parameters is configured in any BWP of any serving cell (i.e., single-TRP).

- “applyIndicatedTCI-State-r18” in ConfiguredGrantConfig, ControlResourceSet, PUSCH-Config, SRS-ResourceSet, PDCCH-ConfigCommon, PUCCH-ResourceExt-v1610, or

- “applyIndicatedTCI-State-r18” and “applyIndicatedTCI-State2-r18” in CSI-AssociatedReportConfigInfo, or

- “applyIndicatedTCI-StateDCI-1-0-r18” in TCI-InDCI-r18.

- mappingPattern-r17”, “multipanelSchemeSDM-r18”, “multipanelSchemeSFN-r18” in PUSCH-Config

- “repetitionSchemeConfig-r16” in PDSCH-Config

- “searchSpaceLinkingId-r17” in searchSpace

- “sfnSchemePDCCH-r17” and “sfnSchemePDSCH-r17” in MIMOParam-r17

- “cjt-Scheme-PDSCH-r18” in ServingCellConfig

- Two SRS resource sets configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook' or ‘noncodebook’

References

- R1-2312371, Reply LS on MIMOevo, Samsung, RAN1#115.

- Chair’s note, RAN1#120.

- Chair’s note, RAN1#120bis.