3GPP TSG RAN WG1 #121			R1-2503545

St Julian’s, Malta, May 19th – 23th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [Draft] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | - | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on UE capability for SRS antenna switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-Core |  |  |  |  |  | Date: |  |  | 2025-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | A UE capability, srs-AntennaSwitching8T8R2SP-1Periodic, was introduced for supporting up to 2 semi-persistent SRS resource sets and up to 1 periodic SRS resource set for 8T8R antenna switching. However, when the specification describes an UE behavior which only can be configured with up to two SRS resource sets (i.e., the UE does not support up to 2 semi-persistent SRS resource sets and up to 1 periodic SRS resource set) for 1T1R, 2T2R, 4T4R, or 8T8R, a UE capability srs-AntennaSwitching2SP-1Periodic is only captured which is relevant on 1T1R, 2T2R, or 4T4R, and a UE capability relevant on 8T8R, srs-AntennaSwitching8T8R2SP-1Periodic, is not captured. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a UE capability srs-AntennaSwitching8T8R2SP-1Periodic to a relevant sentence on 8T8R. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The corresponding UE’s possible RRC configuration on 8T8R SRS antenna switching is not accurately described when the UE capability is not reported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6.2.1.2	UE sounding procedure for DL CSI acquisition

=============================== Unchanged Text Omitted ===================================

-	For 1T=1R, 2T=2R, 4T=4R or 8T=8R, up to two SRS resource sets each with one SRS resource can be configured, where the number of SRS ports for each resource is equal to 1, 2, 4 or 8 if the UE is not indicating srs-AntennaSwitching2SP-1Periodic or srs-AntennaSwitching8T8R2SP-1Periodic. Up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' and one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' can be configured and the two SRS resource sets configured with 'semi-persistent' are not activated at the same time, or up to two SRS resource sets can be configured, if the UE is indicating srs-AntennaSwitching2SP-1Periodic or srs-AntennaSwitching8T8R2SP-1Periodic, where each SRS resource set has one SRS resource, the number of SRS ports for each resource is equal to 1, 2, 4, or 8 or

=============================== Unchanged Text Omitted ===================================