3GPP TSG RAN WG1 #121	        		             R1-2503546

St. Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:	7

Source: 	Samsung

Title: 	Discussions on collision handling between SSB and RA occasion for LTM

Document for:	Discussion and Decision

# Introduction

In [1], RAN4 replied to an RAN1’s LS [2] regarding whether/how to handle a collision between an SSB and a valid RO occasion associated with different cells. Specifically, in their reply LS:

.

RAN4 would like to thank RAN1 for the LS on collision between SSB and RA occasion for LTM (in R1-2410911/R4-2500008). RAN4 discussed the question from RAN1 and reached the following consensus: - RACH shall be prioritized over SSB, if the collision is outside MG. - If collision happens within MG, it is up to UE implementation, i.e., UE may transmit or drop RACH when the RACH occasion colliding with the MG occasion. To RAN1: ACTION: RAN4 respectfully asks RAN1 to take into account above information.

In this contribution, we provide our views on potential impact(s) on RAN1’s work regarding how to handle the aforementioned collision according to RAN4’s reply LS.

# Discussions

Two scenarios are indicated in RAN4’s reply LS: (1) the collision is outside a measurement gap (MG), and (2) the collision is within a MG, i.e., the RACH occasion collides with the MG occasion.

It is evident that (2) is up to UE’s implementation (also pointed out in the reply LS), and there is no need to clarify this in RAN1’s specification(s).

For (1), the corresponding UE’s behaviour can be clarified in RAN1’s specification(s) – in particular the TS 38.213.

Proposal: adopt the following TP to clarify corresponding UE’s behaviour when a collision between an SSB and a RACH occasion associated with different cells occur outside a MG.

## 8.1 Random access preamble < Unchanged parts are omitted > For single cell operation or for operation with contiguous carrier aggregation in a same frequency band or for operation with non-contiguous carrier aggregation in a same frequency band if the UE is not provided with intraBandNC-PRACH-simulTx-r17, a UE - does not transmit PRACH and PUSCH/PUCCH/SRS in a same slot with respect to the smallest SCS configuration between the SCS configuration for the UL BWP with the PRACH and the SCS configuration for the UL BWP with the PUSCH/PUCCH/SRS transmissions - does not transmit PRACH and PUSCH/PUCCH/SRS when a first or last symbol of a PRACH transmission in a first slot is separated by less than N symbols from the last or first symbol, respectively, of a PUSCH/PUCCH/SRS transmission in a second slot; for a PRACH transmission with Npreamblerep>1 preamble repetitions, this applies to each preamble repetition - for a PRACH transmission with Npreamblerep>1 preamble repetitions, if the UE does not indicate prach-Repetition, the UE does not transmit a first repetition of the PRACH and a second repetition of the PRACH when a first or last symbol of the first repetition of the PRACH in a first slot is separated by less than N symbols from the last or first symbol, respectively, of the second repetition of the PRACH in a second slot; otherwise, the UE transmits the first repetition of the PRACH and the second repetition of the PRACH where N=2 for μ=0 or μ=1, N=4 for μ=2 or μ=3, N=16 for μ=5, N=32 for μ=6, and μ is the smallest SCS configuration between the SCS configuration for the UL BWP with the PRACH and the SCS configuration for the UL BWP with the PUSCH/PUCCH/SRS transmissions. For a PUSCH transmission with repetition Type B, this applies to each actual repetition for PUSCH transmission [6, TS 38.214]. For a UE provided with LTM-SSB-Config, the UE transmits a PRACH on a PRACH occasion and does not receive a SS/PBCH if the PRACH occasion and the SS/PBCH associated with different cells are overlapping in time outside a measurement gap. < Unchanged parts are omitted >

# Conclusions

In this contribution, we provide our views on potential RAN1’s specification impact(s) regarding how to handle the collision between a RACH occasion and an SSB associated with difference cells according to RAN4’s reply LS.

Proposal: adopt the following TP to clarify corresponding UE’s behaviour when a collision between an SSB and a RACH occasion associated with different cells occur outside a MG.

## 8.1 Random access preamble < Unchanged parts are omitted > For single cell operation or for operation with contiguous carrier aggregation in a same frequency band or for operation with non-contiguous carrier aggregation in a same frequency band if the UE is not provided with intraBandNC-PRACH-simulTx-r17, a UE - does not transmit PRACH and PUSCH/PUCCH/SRS in a same slot with respect to the smallest SCS configuration between the SCS configuration for the UL BWP with the PRACH and the SCS configuration for the UL BWP with the PUSCH/PUCCH/SRS transmissions - does not transmit PRACH and PUSCH/PUCCH/SRS when a first or last symbol of a PRACH transmission in a first slot is separated by less than N symbols from the last or first symbol, respectively, of a PUSCH/PUCCH/SRS transmission in a second slot; for a PRACH transmission with Npreamblerep>1 preamble repetitions, this applies to each preamble repetition - for a PRACH transmission with Npreamblerep>1 preamble repetitions, if the UE does not indicate prach-Repetition, the UE does not transmit a first repetition of the PRACH and a second repetition of the PRACH when a first or last symbol of the first repetition of the PRACH in a first slot is separated by less than N symbols from the last or first symbol, respectively, of the second repetition of the PRACH in a second slot; otherwise, the UE transmits the first repetition of the PRACH and the second repetition of the PRACH where N=2 for μ=0 or μ=1, N=4 for μ=2 or μ=3, N=16 for μ=5, N=32 for μ=6, and μ is the smallest SCS configuration between the SCS configuration for the UL BWP with the PRACH and the SCS configuration for the UL BWP with the PUSCH/PUCCH/SRS transmissions. For a PUSCH transmission with repetition Type B, this applies to each actual repetition for PUSCH transmission [6, TS 38.214]. For a UE provided with LTM-SSB-Config, the UE transmits a PRACH on a PRACH occasion and does not receive a SS/PBCH if the PRACH occasion and the SS/PBCH associated with different cells are overlapping in time outside a measurement gap. < Unchanged parts are omitted >

# References

[1] R1-2501697 Reply LS to RAN1 on collision between SSB and RA occasion for LTM, 3GPP RAN4#114, Feb. 17th – 21st, 2025.

[2] R1-2410911 LS on collision between SSB and RA occasion for LTM, 3GPP RAN1#119, Nov. 19th – 22nd, 2024.