3GPP TSG RAN WG1 #121		                         R1-2503552 St. Julian’s, Malta, May 19th–24th, 2025

Agenda item:	9.1.3

Source:	Samsung

Title:	Views on AI/ML based CSI prediction

Document for:	Discussion and Decision

# Introduction

In RAN#105, the Rel-19 WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface is revised as below. Among other things, one of the objectives is to further the study, and if necessary, specify consistency of training/inference.

*** text omitted*** Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): - Functionality-based LCM leveraged from other use cases, when necessary and applicable, - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025 ***text omitted***

In the following, we provide our views on the above issues.

# 2. Further study on AI/ML-based CSI prediction

## 2.1 On the need for network’s assistance for training/inference consistency

### Site/cell/location-specific models

RAN1#11

Agreement RAN1#116

For the evaluation of AI/ML-based CSI prediction using localized models in Release 19, consider the following options as a starting point to model the spatial correlation in the dataset for a local region:

- Option 1: The dataset is derived from UEs dropped within the local region, with spatial consistency modelling as per TR 38.901.

    - E.g., Dropped in a specific cell or within a specific boundary.

- Option 2: By using a scenario/configuration specific to the local region.

    - E.g., Indoor-outdoor ratio, LOS-NLOS ratio, TXRU mapping, etc.

Note: While modelling the spatial correlation, strive to ensure that the dataset distribution also correctly captures the decorrelation due to temporal variations in the channel. To report methods to generate training and testing dataset.

Agreement RAN1#117

For the evaluation of AI/ML-based CSI prediction using localized models in Release 19, regarding training,

- The k-th local model is trained on region #B_k (the k-th local region), 1<=k<=N.

- The generalized model is trained on Region #A that may be constructed via any of the following methods that is appropriate for the given generalized/local region modeling approach.

  - Region #A is the same as the union of regions #B_1, …, #B_N.

  - Region #A is a proper superset of the union of regions #B_1, …, #B_N.

  - Region #A is generated separately from regions #B_1, …, #B_N.

  - Note: companies to report which method was used.

For the evaluation of AI/ML-based CSI prediction using localized models in Release 19, regarding testing,

- The trained generalized model, local model, and the non-AI/ML benchmark are tested on the regions #B_1, …, #B_N.

- In case N>1, when reporting the results, companies may report the performance of the generalized model, the local models, and the non-AI/ML benchmark, by averaging the performance over the regions #B_1,…,B_N. Companies to report the value of N.

6 and RAN1#117 made the following agreement for evaluation of localized models. belong to Alt3.

Figure 1 illustrates the performance gain in terms of SGCS of the generic vs. site-(sector) specific prediction model over the baseline without prediction, i.e., sample-and-hold. The generic model is trained based on dataset collected from 57 sectors with 10 UE drops and 10 drops per sector, i.e., 5700 UEs dropped in total. The site/specific model is trained from dataset based on Alt1 with the same number of UEs within one sector. In the figure it can be observed that there is a  performance gain by site-specific model over the generic model especially at a large prediction horizon.

Figure 1 Gain of AI/ML-based CSI prediction over no prediction (sample-and-hold) for generic and sector-specific models (UE speed 30kmphr).

Observation#1: Site-specific AI/ML prediction model trained based on dataset collected from a single drop on a single sector with spatial consistency turned on shows better performance (SGCS) as compared to generic model.

RAN1#118bis further agreed to study potential NW-side additional conditions that may impact on UE’s assumption for CSI prediction.

Agreement

For consistency between training and inference, study to identify which potential NW-side additional conditions, if any, may impact on UE assumption for CSI prediction using UE-sided model, resulting non-negligible degradation on model generalization performance.

Note: Companies are encouraged to provide generalization performance evaluation on the impact of NW-side additional conditions, if considered.

Agreement

For generalization evaluation to identify potential NW-side additional conditions to ensure consistency between training and inference, consider one or more of the following aspects, if evaluated:

- [Optional evaluation] Various tilt angle (e.g., [102] degrees, [110] degrees)

- [Optional evaluation] Various TXRU mapping

  - With TXRU virtualization (e.g., for 32 port CSI-RS, [N1, N2, P] = [2, 8, 2] (baseline), [4,4,2] (optional) with (8x8x2) and (12 x 8 x 2) antenna elements)

  - Without TXRU virtualization (e.g., for 32 port CSI-RS, [N1, N2, P] = [2, 8, 2] (baseline), [4,4,2] (optional) with (2x8x2) and (4 x 4 x 2) antenna elements, respectively)

- Other configurations are not precluded

Note: for other evaluation assumptions, reuse baseline of TR 38.843

Note: report how to map antenna port to antenna elements

Note: report the backbone/structure of AI/ML model used

In the below, evaluation results for the various TXRU mapping are presented. Two TXRU mapping settings are considered

- Setting#1: [N1, N2, P] = [2, 8, 2] with virtualization (12x8x2) antenna elements

- Setting#2: [N1, N2, P] = [2, 8, 2] with virtualization (2x8x2) antenna elements, i.e., w/o virtualization.

For inference, 5 measurements, each 5ms apart, are considered and the predictions are made for future time instances which are 5ms, 10ms, 15ms and 20ms in the future of the latest measurement. For prediction, an LSTM-based model with model description in the Appendix is considered. A generic model is considered that is trained on a dataset from a large number of UMa realizations, i.e., 900 drops. UE speed is set to 10kmphr.

Table 1 Performance impact of TXRU mapping for AI/ML based CSI prediction on 60Kmhr with measurement and prediction at 1ms/1ms/5

| Case 1: Model trained on Setting#2and tested on Setting#2 |  |  |  | Case 2: Model trained on Setting#1 and tested on Setting#2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (10kmph) |  |  |  | (10kmph) |  |  |  |  |
| n (x5ms) | Sample and hold (SGCS) | AI Prediction SGCS (A) | Gain % | n (x5ms) | Sample and hold (SGCS) | AI Prediction SGCS (B) |  | % gain from Case 1 |
|  |  |  |  |  |  |  |  | (B-A)/A |
|  |  |  |  |  |  |  | Gain % |  |
| 1 | 0.9337 | 0.9824 | 5.22% | 1 | 0.9337 | 0.9674 | 3.61% | -1.53% |
| 2 | 0.8132 | 0.9396 | 15.54% | 2 | 0.8132 | 0.9231 | 13.51% | -1.76% |
| 3 | 0.7088 | 0.8682 | 22.49% | 3 | 0.7088 | 0.8224 | 16.03% | -5.28% |
| 4 | 0.6317 | 0.6969 | 10.32% | 4 | 0.6317 | 0.6568 | 3.97% | -5.75% |
| 5 | 0.5638 | 0.6074 | 7.73% | 5 | 0.5638 | 0.5773 | 2.39% | -4.96% |

Table 2 Performance impact of TXRU mapping for AI/ML based CSI prediction on 60Kmhr with measurement and prediction at 1ms/1ms/5

| Case 1: Model trained on Setting#2 and tested on Setting#2 |  |  |  | Case 2: Model trained on Setting#1 and tested on Setting#2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (60kmph) |  |  |  | (60kmph) |  |  |  |  |
| n (x1ms) | Sample and hold (SGCS) | AI Prediction SGCS (A) | Gain % | n (x5ms) | Sample and hold (SGCS) | AI Prediction SGCS (B) |  | % gain from Case 1 |
|  |  |  |  |  |  |  |  | (B-A)/A |
|  |  |  |  |  |  |  | Gain % |  |
| 1 | 0.9144 | 0.9769 | 6.84% | 1 | 0.9144 | 0.9742 | 6.54% | -0.28% |
| 2 | 0.7777 | 0.9212 | 18.45% | 2 | 0.7777 | 0.9085 | 16.82% | -1.38% |
| 3 | 0.6785 | 0.7886 | 16.23% | 3 | 0.6785 | 0.7525 | 10.91% | -4.58% |
| 4 | 0.6032 | 0.6894 | 14.29% | 4 | 0.6032 | 0.6151 | 1.97% | -10.78% |
| 5 | 0.5388 | 0.5588 | 3.71% | 5 | 0.5388 | 0.5062 | -6.05% | -9.41% |

Table 3 Performance impact of TXRU mapping for AI/ML based CSI prediction on 60Kmhr with measurement and prediction at 1ms/1ms/5

| Case 1: Model trained on Setting#1 and tested on Setting#1 |  |  |  | Case 2: Model trained on Setting#2 and tested on Setting#1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (60kmph) |  |  |  | (60kmph) |  |  |  |  |
| n (x1ms) | Sample and hold (SGCS) | AI Prediction SGCS (A) | Gain % | n (x5ms) | Sample and hold (SGCS) | AI Prediction SGCS (B) |  | % gain from Case 1 |
|  |  |  |  |  |  |  |  | (B-A)/A |
|  |  |  |  |  |  |  | Gain % |  |
| 1 | 0.9099 | 0.9715 | 6.77% | 1 | 0.9099 | 0.9527 | 4.70% | -1.94% |
| 2 | 0.7671 | 0.906 | 18.11% | 2 | 0.7671 | 0.8775 | 14.39% | -3.15% |
| 3 | 0.6613 | 0.7461 | 12.82% | 3 | 0.6613 | 0.6886 | 4.13% | -7.71% |
| 4 | 0.5782 | 0.6102 | 5.53% | 4 | 0.5782 | 0.5907 | 2.16% | -3.20% |
| 5 | 0.509 | 0.4982 | -2.12% | 5 | 0.509 | 0.474 | -6.88% | -4.86% |

##### Observation#2: For performance evaluation of UE-side CSI prediction with respect to data distribution mismatch that may arise from NW-side additional conditions, considering TRP antenna settings with different TXRU mapping, e.g., Setting#1: [N1, N2, P] = [2, 8, 2] with virtualization (2x8x2) antenna elements and Setting#2: [N1, N2, P] = [2, 8, 2] with virtualization (12x8x2) antenna elements

##### Case1: When an AI/ML model is trained and tested on dataset from a single TRP antenna setting, a similar level of performance gain (5%-22%) over Baseline#1 (sample-and-hold) is observed irrespective of the antenna settings.

##### Case 2: When an AI/ML model is trained on dataset from one TRP antenna setting and tested on dataset from a different TRP antenna setting, a lower performance gain over Baseline#1 (sample-and-hold) is observed and a significant performance loss of up to 10.8% is observed as compared to a model trained on a single antenna setting.

For UE-side CSI prediction, the AI/ML model is in general trained by the UE-side vendor. To assist the data collection for training, the network may indicate its setting in an implicit manner that protects its proprietary implementation and privacy aspects. As shown in Fig. 2, for example, the indicated information indirectly indicates the TRP corresponding to the UE’s CSI prediction, e.g., TRPs with different TXRU virtualization. This indication may also implicitly indicate other aspects such spatial-domain pattern for network-energy saving, antenna tilt consideration, etc. Then, if UE vendors train localized/site/scenario/setting specific AI/ML model, the network may indicate its setting for the UE so that the collected data can be categorized. This can be considered as network-side additional condition as per RAN1#114bis agreement. One issue here is that in practice the model training and inference most likely happen in different times (not in a single RRC connection rather in weeks, months). This makes the TCI framework to be insufficient because the network has flexibility to change the mapping between the TCI IDs and the actual transmission configurations across UEs and different RRC configurations. However, for the present issue, the network may have to keep the same mapping between its settings and the required indication for a much longer time scale to ensure the consistency between data collection for training and inference.

Figure 2 CSI prediction from different TRPs

##### Proposal#1: In CSI prediction use case using UE-sided model, consider implicit indication of TRP related aspects for network-side additional condition indication.

## Other Aspects with Specification Impact

### On Configurations for UE’s Data collection

For data collection purpose, RAN1#120 agreed (see below) to support periodic (P) and Semi-persistent (SP) CSI-RS resources for CMR. Moreover, for inference, RAN1 agreed to support P, SP and aperiodic (AP) CSI-RS for CMR.

Agreement

For CSI prediction using UE-side model, for data collection for training,

- For resource configuration,

  - For CSI-RS resource type for CMR, periodic and semi-persistent are supported.

    - FFS on support of aperiodic CSI-RS resource

  - FFS on whether to separately configure CSI-RS resource(s) used for measurements for model input and CSI-RS resource(s) used for measurements for ground-truth CSI

  - FFS whether/how to enhance CSI-RS resource configuration to reduce CSI-RS signalling overhead.

- FFS on CSI report configuration

Agreement

For CSI prediction using UE-side model, at least for inference, Rel-18 CSI framework is reused.

- For CSI-RS resource type for CMR, periodic, semi-persistent, and aperiodic CSI-RS are supported.

- For inference report,

  - for N4>=1, support Rel-18 codebook ('typeII-Doppler-r18')

Conclusion (RAN1#120bis)

For CSI prediction using UE-side model, for data collection for training, aperiodic CSI-RS resource for CMR is not supported.

Agreement (RAN1#120bis)

For CSI prediction using UE-side model, for training data collection, support

- CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report

  - FFS on how to indicate without CSI report in CSI-ReportConfig

The CSI report configuration for data collection may consider the following points:

- Generally, the UE is not configured to report CSI for data collection purpose, i.e., the higher layer parameter for reportQuantity can be set to ‘none’. Moreover, such configuration does not require parameter for codebookType.

- The UE consumes processing resources for measurements for data collection purpose. How to determine the occupied processing unit and the duration of the occupancy?

- How to determine the active CSI-RS resources and the active CSI-RS ports when the resources are referred by a report configuration for data collection. Whether the same scaling factor Kp∈{1,2,4}  is applied as in the case where CSI-ReportConfig configured with the higher layer parameter codebookType set to 'typeII-Doppler-r18'.

##### Observation#3: For the AI/ML based CSI prediction using UE-side model, for CSI report configuration for training data collection purpose,

- The UE is not expected to report CSI for data collection purpose, i.e., the higher layer parameter for reportQuantity can be set to ‘none’. Moreover, such configuration does not require parameter for codebookType.

- RAN1 need to address how to manage the processing unit and their occupancy for CSI report configuration for data collection purpose.

- RAN1 need to address how to determine the active CSI-RS resources and the active CSI-RS ports when the resources are referred by a report configuration for data collection. Whether the same scaling factor Kp∈{1,2,4}  is applied as in the case wherein the CSI-ReportConfig configured with the higher layer parameter codebookType set to 'typeII-Doppler-r18'.

Observation#4: For the AI/ML based CSI prediction using UE-side model, the CSI report configuration for training data collection purpose needs to be differentiated from other report configurations with reportQuantity set to ‘none’.

##### Proposal#2: For the AI/ML based CSI prediction using UE-side model, for CSI report configuration for training data collection purpose,

- The UE is not expected to report CSI for data collection purpose, i.e., the higher layer parameter reportQuantity is set to ‘none’, Codebook (‘codebookType’) is not configured.

- To differentiate the report configuration from other report configurations with reportQuantity set to ‘none’, consider explicit indication of resources (CMRs) for data collection purpose, e.g., higher layer parameter resourcesForDataCollection    ‘CSI-ResourceConfigId’.

5G NR specifies rules to determine the processing units counting and their occupancy (the duration for the occupied processing units) when the CSI report configuration is configured with higher layer parameter reportQuantity  is set to ‘none’. Such configurations in NR occupy OCPU=0  when ‘trs-Info’ is configured and OCPU=1, otherwise. Considering the CMRs for data collection involve higher number of ports, a different value of processing unit could be occupied. As detailed in the subsequent sections below, the processing unit could also be counted towards a separate processing resources pool than the legacy CPU pool, i.e., UE’s indicated NCPU. Thus, differentiating CSI report configuration from other CSI report configuration with reportQuantity  set to ‘none’ is needed.

Figure 3 CPU occupancy for report configuration for data collection purpose (Ks=1)

For CPU occupancy, as a starting point, the legacy design for CSI report configurations without a corresponding report can be considered. Considering only P/SP CMRs are supported for data collection, at least for up to 32 CSI RS ports (Rel-18 DD CSI), Ks=1 CSI-RS resource is supported in a CSI-RS resource set for channel measurement. In this particular case, for each of the CSI-RS transmission occasions, the CPU is occupied from the first symbol of the CSI-RS resource to Z'3 symbols after the last symbol of CSI-RS resource. The Rel-19 enhancement on eType II DD CSI, i.e., for CSI up to 128 ports, up to Ks=4 CSI-RS resources, is supported. In this case, the same principle as in Ks=1 can be applied. Another issue is how to count the active CSI-RS resources and active CSI-RS ports. In the legacy, for a P/SP CSI-RS resources, a CSI report configuration with the higher layer parameter codebookType set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18', the CSI-RS resource and the CSI-RS ports within the CSI-RS resource are counted Kp times, where the value of Kp∈{1,2,4} is indicated by UE capability. However, in the legacy the UE has to consider multiple transmission occasions of the CSI-RS resource of the CSI-RS resource set for CSI calculation. This is not a valid assumption for the case of data collection, as there is no associated CSI report and the UE is not required to consider multiple transmission occasions. In this case, Kp=1 can simply be considered (without the need for capability report).

##### Proposal#3: For the AI/ML based CSI prediction using UE-side model, for CSI report configuration for training data collection

- OCPU=Ks processing units are counted toward the legacy CSI processing unit (CPU), where Ks is the number of CSI-RS resource(s) in the CSI-RS resource set for channel measurement.

- For each of P/SP CSI-RS transmission occasions, the CPU is occupied from the first symbol of the earliest CSI-RS resource until Z'3 symbols after the last symbol of the latest CSI-RS resource for data collection.

  - FFS: whether new value for Z'3.

- For the active CSI-RS resources and active CSI-RS ports counting, a scaling factor is not applied, i.e., Kp=1.

### On Model Performance Monitoring

Agreement

For CSI prediction using UE-side model, for performance monitoring, support UE assisted performance monitoring subject to an additional UE capability, and UE assisted performance monitoring is based on Type 3 performance monitoring

Agreement

For CSI prediction using UE-side model, for performance monitoring type 3, support SGCS as a performance metric.

The following are among the agreements for model performance monitoring for CSI prediction.

For configurations for monitoring, RAN1 may adopt the solution followed for BM prediction using UE-side model, i.e., the CSI report configuration for monitoring is linked to the CSI report configuration for inference. Separate and linked configurations, allow the different time-domain properties (P,SP, AP) as well as separate triggering/activation of CSI report for inference and monitoring. This can be configured to any of the configuration types.

Agreement At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling: - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report - FFS on the timing related issues - UE measures the dedicated resource set(s) for monitoring.

##### Proposal#4: For the AI/ML based CSI prediction using UE-side model, support to reuse CSI framework for the configuration for monitoring result report in L1 signaling

- Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring

  - The CSI report Configuration ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration

Moreover, in RAN1#120bis, the following was agreed regarding reporting contents for UE assisted performance monitoring.

Agreement

For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, down-select one alternative by RAN1#121.

- Alt 1: one SGCS is calculated based on predicted CSI for one inference reporting, ground truth CSI

- Alt 2: one SGCS is calculated based on predicted CSI for one inference reporting, and ground truth CSI, another SGCS is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance.

- Alt 3: statistic of reporting contents (e.g., mean, x% CDF of SGCS values) of inference reporting instances within configured monitoring window

  - Monitoring window can be configured by NW

    - FFS on signalling details

- FFS on whether to report per prediction instance, selected prediction instance(s), averaged over prediction instances

- FFS on report frequency granularity (e.g., per wideband or per subband or averaged over subband or selected subband)

- FFS on whether to report per layer, the first layer, averaged over layer

- FFS on how to quantize and report mechanism

First, the statistical processing in Alt3 can be applied to both Alt1 and Alt2. Thus, it is not a parallel alternative per se. While we acknowledge that Alt2, as compared to Alt1, provides additional information for the network to make LCM decisions, e.g., trigger different configuration, it also requires more specification discussion and work. In particular, based on Alt2, the network would know whether the predicted CSI is better performing than the baseline and switch to a non-predictive CSI, if not. As an example for the additional specification impact, codebook configuration should be defined for predicted CSI, ground-truth CSI and baseline, e.g., Rel-16 eType II, as opposed to predicted CSI and ground truth. Timeline related issues should also be resolved between these three CSIs with additional complication on CPU occupancy rules. Additionally, for the fully flexible Alt2, the UE has to compute three PMIs, i.e., Predicted PMI (Rel-18 eType II), ground truth PMI (Rel-18 eType II) and possibly non-predictive PMI (Rel-16 eType II). Alt2, without restriction, would also add complexity to the UE as it has to compute additional PMI and buffer it.

Observation#5: For the AI/ML based CSI prediction using UE-side model, among the alternatives of the reporting contents of UE assisted performance monitoring,

- Alt2, as compared to Alt1, provides more information for the network to preform LCM decision, e.g., fall-back to non-predicted CSI.

- Alt2, if no restriction is applied, requires a large specification effort, e.g., codebook and CSI timeline for non-predicted CSI

- Alt2, if no restriction is applied, may increase UE complexity, UE requires to compute three PMIs for each of monitoring instances.

Considering RAN1#121 is the last meeting for Rel-19 normative work, if some restrictions are not applied, it may not be feasible to go with Alt2 and complete the normative work on time. One alternative configuration that may provide the benefits of Alt2 (with some restrictions, e.g., N4>1) is shown in the below figure. If the higher layer parameter, prediction delay (denoted by δ∈{-nref, 0, 1 ,2} in the figure), is configured to –nref, the first prediction instance for n4=0 will be closer to the reference slot or be in the reference slot. Thus, the latest CMR for inference can also be used as the first MMR for monitoring. The predicted PMI and the ground truth PMI for n4=0 would be the same. In this case, the UE may utilize the predicted PMI to compute the SGCS for precoders corresponding to  n4=0, W0n3, and ground-truth precoders corresponding to n4>0, Wn4n3n4=1N4-1, to approximate the second SGCS in Alt2.

Figure 4: configuration to achieve the benefits of Alt2 based on Alt1

Observation#6: For the AI/ML based CSI prediction using UE-side model, for Alt2 of the reporting contents of UE assisted performance monitoring, when the latest CMR for inference report and the earliest MMR for the corresponding monitoring report are located in the reference slot of the inference reporting instance, i.e., by configuring higher layer parameter predictionDelay as δ=-nref,

- The baseline CSI (non-predicted precoder) is the same as the precoder corresponding to n4=0 of predicted PMI W0n3 in the inference report.

- For n4∈1,..,N4-1, the UE may calculate the second SGCS values based on the predicted PMI in the inference report, i.e., precoders corresponding to n4=0, W0n3, and the ground-truth precoding vectors corresponding to n4>0, Wn4n3n4=1N4-1.

Note: The second SGCS in Alt2 is based on the ground truth CSI and the non-predicted CSI corresponding to the latest CSI-RS transmission occasion not later than the CSI reference resource of the inference-reporting instance.

Proposal#5: Considering RAN1#121 is the last meeting of Rel-19 normative work and to address UE complexity, for the reporting contents alternatives of UE assisted performance monitoring, support Alt 2, only if

- prediction delay is configured as δ=-nref.

- the latest CMR for inference report instance and the earliest MMR for the corresponding monitoring are in the reference slot of the inference reporting instance.

Agreement

For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, down-select one alternative by RAN1#121.

- Alt 1: one SGCS is calculated based on predicted CSI for one inference reporting, ground truth CSI

- Alt 2: one SGCS is calculated based on predicted CSI for one inference reporting, and ground truth CSI, another SGCS is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance.

- Alt 3: statistic of reporting contents (e.g., mean, x% CDF of SGCS values) of inference reporting instances within configured monitoring window

  - Monitoring window can be configured by NW

    - FFS on signalling details

- FFS on whether to report per prediction instance, selected prediction instance(s), averaged over prediction instances

- FFS on report frequency granularity (e.g., per wideband or per subband or averaged over subband or selected subband)

- FFS on whether to report per layer, the first layer, averaged over layer

- FFS on how to quantize and report mechanism

As indicated by the FFS points in the above agreement, time-frequency granularity for monitoring KPI calculation and/or reporting need to be determined. As an example, in the frequency domain, the frequency granularity for monitoring KPI calculation can be set the same as the PMI subband, or CQI subband, or scaled to a larger frequency unit(to reduce UE’s computation and buffering requirement (See Figure 4, for scaling factor of 2).  Similarly, in the time domain, the time granularity for monitoring KPI calculation can be set to be same as the time granularity for PMI (DD time unit), or scaled to a larger time unit(to reduce UE’s computation and buffering requirement (See Figure 4, for scaling factor of 2).

Figure 6 The relationship between time-frequency unit for monitoring and time-frequency unit for inference.

It is to be noted that the frequency-domain unit for PMI determination is provided by the parameters reportFreqConfiguration, which indicates the number of subbands Nsb (implicitly the size of subband) as well as a bit-map based selection and the parameter numberOfPMI-SubbandsPerCQI-Subband, which can be set R=1, 2. similarly, the time-domain unit for PMI determination is provided by the parameter vectorLengthDD. If these parameters are configured in the configuration for inference, the configuration for monitoring may define the monitoring time-frequency unit with respect to these parameters, e.g., with a scaling factor. As an example, the monitoring time-domain unit can be defined as a time-domain unit with slot length scaled by an integral scaling factor γDD from the slot length of the Doppler-time unit of the configured prediction instance.

However, to minimize the specification effort, we propose simplified solutions. In general, the prediction performance degrades as the prediction horizon (instance) gets further from the latest measurement. Depending on UE’s speed and the prediction performance, the network may adjust the length of the prediction horizon by triggering a CSI report configuration with different N4 value. Thus, it is important for the monitoring KPI to be reported per prediction instance. Moreover, the wideband reporting granularity is sufficient for the time-domain CSI prediction. However, the subband SGCS values can be averaged for a wideband SGCS computation. The prediction accuracy in terms of SGCS varies in a wide range across the MIMO layers. Thus, averaging across the MIMO layers masks the accuracy of the prediction. Thus, we propose the UE to report the monitoring KPI per MIMO layer, where the layers range up to the reported rank for the corresponding inference reporting instance.

Proposal#6: For the AI/ML based CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, support

- Performance monitoring KPI reporting per MIMO layer, l=0,…, v-1, where v is the reported rank for the corresponding inference reporting instance.

- Performance  monitoring KPI reporting per prediction instance, n4=0, 1,…, N4-1.

  - The UE determines the value N4 for monitoring report based on the higher layer parameter vectorLengthDD  in the CSI report configuration for inference.

- Performance monitoring KPI reporting per wideband frequency granularity by averaging across the subband level SGCS values calculated per PMI reporting subbands indexed by n3∈0,…, N3-1.

  - The UE determines the value N3 for monitoring report based on the higher layer parameter reportFreqConfiguration  and  numberOfPMI-SubbandsPerCQI-Subband  in the CSI report configuration for inference.

Figure 7 Monitoring measurement resources (MMRs)

Figure 4, illustrates the example for configuration for CMR and MMR for CSI prediction. The MMRs has to be received in the time window between the slots starting from the first slot to the last slot in the time domain for the associated predicted PMI. If there is many-to-one relationship between configurations for inference and configuration for monitoring, the MMRs configured in the CSI report configuration for monitoring may not fit in to the N4 DD time units of the predicted CSI. In these cases, the MMRs might be configured and received in the slots outside the prediction window of the linked CSI report for inference, the UE may ignore the MMRs for the corresponding configuration for monitoring.

##### Proposal#7: For the AI/ML based CSI prediction using UE-side model, for performance monitoring, the network configures at least one monitoring measurement resource (MMR) in the prediction window defined as the slots between the earliest slot to the last slot, in time domain, for the linked predicted PMIs.

- The UE measures up to N4 MMRs in the prediction window.

- The UE is not expected to measure MMR outside the prediction window.

Another issue is quantization of the SGCS values for the reporting. The SGCS can be reported based on a predefined/configured range. The network may configure the UE to report the value quantized within a certain range [ymin,ymax] with quantization level ∆y and reporting bitwidth B. The UE may determines the quantization level ∆y=ymax-ymin2B-1 . The reported bits bB-1,,…,b1,b0 wherein bB-1 and b0 reperenst the MSB and LSB bits, respectively, correspond to a value c=i=1B-1bi2i. For the determined  KPI value, KPIreported, the UE determines and report the bB-1,,…,b1,b0  corresponding to the value c=KPIreported-ymin∆y if KPIreported≥ymin0     if       KPIreported<ymin. It is beneficial to configure the range [ymin,ymax] per MIMO layer and per prediction instance.

##### Proposal#8: For the AI/ML based CSI prediction using UE-side model, for the quantization of the reported SGCS values, support linear quantization of the SGCS values as monitoring KPIs

- The network configures the UE to report SGCS with a certain range [ymin,ymax] with quantization level ∆y and reporting bitwidth B. The range [ymin,ymax] is configured per layer per prediction instance.

- The UE may determines the quantization level ∆y=ymax-ymin2B-1 .

- The reported bits bB-1,,…,b1,b0 wherein bB-1 and b0 reperenst the MSB and LSB bits, respectively, correspond to a value, c=i=1B-1bi2i.

- For the calculated value, KPIreported, the UE determines and report the bB-1,,…,b1,b0  corresponding to the value c=KPIreported-ymin∆y if KPIreported≥ymin0     if       KPIreported<ymin.

For configuration of ymin and ymax, they can be configured in the range from 0 to 1 with a certain step size, e.g.,  0.1.

RAN1 has to further address issues related to processing unit occupancy for monitoring configuration, UCI construction for L1 reporting monitoring content.

Proposal#9: For the AI/ML based CSI prediction using UE-side model, for monitoring report configuration, RAN1 to support L1 reporting and address the following issues

- Processing unit and occupancy

- UCI construction and reporting priority for monitoring quantities.

### On Processing Unit Management and Timeline

RAN1 agreed the below agreements for CSI processing criteria and timeline.

Agreement Introduce a dedicated AI/ML PU for AI/ML features for UE, - The AI/ML PU is used at least for quantifying the simultaneous processing of multiple CSI reports subject to CSI-related AI/ML use case(s), e.g., CSI compression (if supported), CSI prediction, BM spatial prediction, BM temporal prediction.

The UE may execute the configuration for AI/ML related features/functionalities partly or fully on a different platform than legacy CSI. As an example, for the typical processing blocks for CSI prediction shown below, the channel prediction is performed is generally executed on a different processing resources (hardware) than the legacy CSI.  In some implementations, the CSI measurement, pre-processing,  RI determination, PMI determination, and CQI determination are processed in the same HW unit as the channel prediction. In some implementations the subsequent process are handled by the same processing resource as the legacy CSI.

Observation#3: The UE may execute the configurations for AI/ML related features/functionalities partly or fully on a different hardware unit (processing resource) than legacy CSI.

Moreover, for AI/ML operation the UE may fetch (deploy) the AI/ML model from non-volatile storage to the memory unit near to where inference is executed. This step is equivalent to model/functionality activation. Since the available memory resource is limited, there may be limitations on the maximum number of simultaneously activated models/functionalities, buffering requirement for channel samples.

Agreement (RAN1#120bis) For CSI prediction using UE side model, for inference, consider following options for potential down selection - Option 1: only dedicated AI/ML PU (OAPU) is occupied - Option 2: only legacy CPU (OCPU) is occupied - Option 3: both dedicated AI/ML PU (OAPU) and legacy CPU (OCPU) are occupied - FFS whether OAPU and OCPU are based on defined rule and/or reported by UE - Note: The supported option(s) by UE is reported by UE capability, if multiple options are supported. The total number of dedicated AI/ML PU for AI/ML is reported by UE capability. - Note: The total number of Use case specific dedicated AI/ML PU could be discussed separately.

We also support the UE to report OAPU from a predefined candidate values. If the candidate values include the value ‘0’, then Option2 would also be indirectly supported.

Proposal#10: For CSI prediction using UE side model, for inference,

- Support Option 3: both dedicated AI/ML PU (OAPU) and legacy CPU (OCPU) are occupied

- Follow the legacy rule for OCPU as defined for the case when the UE is configured with CSI report configuration with codebookType set to ‘typeII-Doppler-r18’

- The UE reports OAPU from a specified candidate values.

# Conclusion

##### In this contribution, the following observations and proposals are made:

##### Observation#1: Site-specific AI/ML prediction model trained based on dataset collected from a single drop on a single sector with spatial consistency turned on and a large number of UE per drop shows better performance (SGCS) as compared to generic model.

##### Observation#2: For performance evaluation of UE-side CSI prediction with respect to data distribution mismatch that may arise from NW-side additional conditions, considering TRP antenna settings with different TXRU mapping, e.g., Setting#1: [N1, N2, P] = [2, 8, 2] with virtualization (2x8x2) antenna elements and Setting#2: [N1, N2, P] = [2, 8, 2] with virtualization (12x8x2) antenna elements

##### Case1: When an AI/ML model is trained and tested on dataset from a single TRP antenna setting, a similar level of performance gain (5%-22%) over Baseline#1 (sample-and-hold) is observed irrespective of the antenna settings.

##### Case 2: When an AI/ML model is trained on dataset from one TRP antenna setting and tested on dataset from a different TRP antenna setting, a lower performance gain over Baseline#1 (sample-and-hold) is observed and a significant performance loss of up to (13.4%) is observed as compared to a model trained on a single antenna setting.

##### Proposal#1: In CSI prediction use case using UE-sided model, consider implicit indication of TRP related aspects for network-side additional condition indication.

##### Observation#3: For the AI/ML based CSI prediction using UE-side model, for CSI report configuration for training data collection purpose,

1)       The UE is not expected to report CSI for data collection purpose, i.e., the higher layer parameter for reportQuantity can be set to ‘none’. Moreover, such configuration does not require parameter for codebookType.

2)       RAN1 need to address how to manage the processing unit and their occupancy for CSI report configuration for data collection purpose.

3)       RAN1 need to address how to determine the active CSI-RS resources and the active CSI-RS ports when the resources are referred by a report configuration for data collection. Whether the same scaling factor  is applied as in the case wherein the CSI-ReportConfig configured with the higher layer parameter codebookType set to 'typeII-Doppler-r18'.

Observation#4: For the AI/ML based CSI prediction using UE-side model, the CSI report configuration for training data collection purpose needs to be differentiated from other report configurations with reportQuantity set to ‘none’.

##### Proposal#2: For the AI/ML based CSI prediction using UE-side model, for CSI report configuration for training data collection purpose,

- The UE is not expected to report CSI for data collection purpose, i.e., the higher layer parameter reportQuantity is set to ‘none’, Codebook (‘codebookType’) is not configured.

- To differentiate the report configuration from other report configurations with reportQuantity set to ‘none’, consider explicit indication of resources (CMRs) for data collection purpose, e.g., higher layer parameter resourcesForDataCollection    ‘CSI-ResourceConfigId’.

##### Proposal#3: For the AI/ML based CSI prediction using UE-side model, for CSI report configuration for training data collection

-         processing units are counted toward the legacy CSI processing unit (CPU), where  is the number of CSI-RS resource(s) in the CSI-RS resource set for channel measurement.

-        For each of P/SP CSI-RS transmission occasions, the CPU is occupied from the first symbol of the earliest CSI-RS resource until  after the last symbol of the latest CSI-RS resource for data collection.

o      FFS: whether new value for

-        For the active CSI-RS resources and active CSI-RS ports counting, a scaling factor is not applied, i.e., .

##### Proposal#4: For the AI/ML based CSI prediction using UE-side model, support to reuse CSI framework for the configuration for monitoring result report in L1 signaling

- Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring

  - The CSI report Configuration ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration

Observation#5: For the AI/ML based CSI prediction using UE-side model, among the alternatives of the reporting contents of UE assisted performance monitoring,

- Alt2, as compared to Alt1, provides more information for the network to preform LCM decision, e.g., fall-back to non-predicted CSI.

- Alt2, if no restriction is applied, requires a large specification effort, e.g., codebook and CSI timeline for non-predicted CSI

- Alt2, if no restriction is applied, may increase UE complexity, UE requires to compute three PMIs for each of monitoring instances.

Observation#6: For the AI/ML based CSI prediction using UE-side model, for Alt2 of the reporting contents of UE assisted performance monitoring, when the latest CMR for inference report and the earliest MMR for the corresponding monitoring report are located in the reference slot of the inference reporting instance, i.e., by configuring higher layer parameter predictionDelay as ,

-        The baseline CSI (non-predicted precoder) is the same as the precoder corresponding to  of predicted PMI  in the inference report.

-        For , the UE may calculate the second SGCS values based on the predicted PMI in the inference report, i.e., precoders corresponding to   and the ground-truth precoding vectors corresponding to .

Note: The second SGCS in Alt2 is based on the ground truth CSI and the non-predicted CSI corresponding to the latest CSI-RS transmission occasion not later than the CSI reference resource of the inference-reporting instance.

Proposal#5: Considering RAN1#121 is the last meeting of Rel-19 normative work and to address UE complexity, for the reporting contents alternatives of UE assisted performance monitoring, support Alt 2, only if

-        prediction delay is configured as

-        the latest CMR for inference report instance and the earliest MMR for the corresponding monitoring are in the reference slot of the inference reporting instance.

Proposal#6: For the AI/ML based CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, support

-        Performance monitoring KPI reporting per MIMO layer,  where  is the reported rank for the corresponding inference reporting instance.

-        Performance  monitoring KPI reporting per prediction instance,

o        The UE determines the value  for monitoring report based on the higher layer parameter vectorLengthDD  in the CSI report configuration for inference.

-        Performance monitoring KPI reporting per wideband frequency granularity by averaging across the subband level SGCS values calculated per PMI reporting subbands indexed by

o        The UE determines the value  for monitoring report based on the higher layer parameter reportFreqConfiguration  and  numberOfPMI-SubbandsPerCQI-Subband  in the CSI report configuration for inference.

##### Proposal#7: For the AI/ML based CSI prediction using UE-side model, for performance monitoring, the network configures at least one monitoring measurement resource (MMR) in the prediction window defined as the slots between the earliest slot to the last slot, in time domain, for the linked predicted PMIs.

-          The UE measures up to  MMRs in the prediction window.

-        The UE is not expected to measure MMR outside the prediction window.

##### Proposal#8: For the AI/ML based CSI prediction using UE-side model, for the quantization of the reported SGCS values, support linear quantization of the SGCS values as monitoring KPIs

-        The network configures the UE to report SGCS with a certain range [] with quantization level  and reporting bitwidth . The range [] is configured per layer per prediction instance.

-          The UE may determines the quantization level  .

-          The reported bits  wherein and reperenst the MSB and LSB bits, respectively, correspond to a value, .

-          For the calculated value, , the UE determines and report the   corresponding to the value .

Proposal#9: For the AI/ML based CSI prediction using UE-side model, for monitoring report configuration, RAN1 to support L1 reporting and address the following issues

- Processing unit and occupancy

- UCI construction and reporting priority for monitoring quantities.

Observation#7: The UE may execute the configurations for AI/ML related features/functionalities partly or fully on a different hardware unit (processing resource) than legacy CSI.

Proposal#10: For CSI prediction using UE side model, for inference,

- Support Option 3: both dedicated AI/ML PU (OAPU) and legacy CPU (OCPU) are occupied

- Follow the legacy rule for OCPU as defined for the case when the UE is configured with CSI report configuration with codebookType set to ‘typeII-Doppler-r18’

- The UE reports OAPU from a specified candidate values.

# References

[1] RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Sep, 2024.

[2] RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Dec, 2021.

[3] TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18).