3GPP TSG RAN1 WG #121		                         R1-2503553

St. Julian’s, Malta, May 19th– 23th, 2025

Agenda item:	9.1.4.1

Source:	Samsung

Title:	Views on additional study for AI/ML based CSI compression

Document for:	Discussion and Decision

- Introduction

In RAN#105, the Rel-19 WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface is revised as below. One of the objectives of the work item [1] is to further the study the following as a sequel to the Rel-18 study:

*** text omitted*** Study objectives: - CSI feedback enhancement [RAN1]: - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing necessary specification impact analysis, as well as, other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843. ***text omitted***

In the following, we provide our view on the final notes of the study item.

# 2. Finalizing the Rel-19 Study on CSI Compression

## 2.1 Considerations to improve performance-complexity trade-off

Here, one of the objectives in the Rel-19 WID is to further study various aspects that may improve the trade-off across performance, complexity, and overhead. The complexity of AI/ML models in the Rel-18 [TR 38.843]      based evaluation (captured in the TR 38.843), falls within a large range (~0.1 – 1000M FLOPs). Moreover, the model level complexity in terms of FLOPs is multiple orders larger than the complexity of conventional algorithms for eType II PMI determination, which is below a 0.2M FLOPs as illustrated in [4] .  While acknowledging that the nature of parallelism for AI/ML model inference and eType II PMI determination are different, it is obvious that the evaluated models, if deployed directly, would incur substantially larger complexity - consequently may drain UE’s battery while increasing CSI computing latency and hardware requirements.

Table 1 summarizes the approaches considered to improve the performance-complexity tradeoff in the Rel-19 study. Among these approaches, RAN1 has concluded that updated use case improves performance, while conclusions are yet to be made with the other approaches.

##### Table 1 Approaches to improve performance-complexity trade-off

| # | Approach | Estimated impact on performance | Estimated impact on model complexity |
| --- | --- | --- | --- |
| 1 | Update on use case (e.g., TSF-domain Joint prediction and compression) | Concluded it improves |  |
| 2 | Site/cell/location specific models | Observation it improves | Observation it improves |
| 3 | Input-output pre-processing (dimension reduction) | Observation from companies captured. | Observation from companies captured |
| 4 | Model optimization e.g., pruning | Pending | Pending |

### Updated use case

The Rel-18 SI considered CSI compression in the spatial and frequency domains (SF), i.e., the CSI input for compression corresponds to multiple-antenna ports (multiple samples in space domain), a number of subbands (multiple samples in the frequency domain) and single measurement (a single time sample).

One approach to increase the compressibility of the input is to consider inputs form multiple time samples and exploit time-domain correlation, thus time-spatial-frequency (TSF)-domain compression. The multiple time samples may mean multiple measurements in the past or multiple prediction instances in the future of the report.  Another consideration is to jointly consider both CSI compression and prediction. In this case, the prediction could be at the UE-side and/or network-side.

In this regard, five additional sub-use case on top of the one captured in Rel-18 study (Case 0), were considered. Among the added 5 cases, Case 2 and Case 3 were down selected and further investigated.

| Case | Target CSI slot(s) | Whether the UE uses past CSI information | Whether the network uses past CSI information |
| --- | --- | --- | --- |
| 0 | Present slot | No | No |
| 1 | Present slot | Yes | No |
| 2 | Present slot | Yes | Yes |
| 3 | Future slot(s) | Yes | No |
| 4 | Future slot(s) | Yes | Yes |
| 5 | Present slot | No | Yes |

Figure 1 illustrates an exemplary implementation of Case 2 and the observed UPT gain. It can be concluded that Case 2 improves the performance as compared to both Rel-16 eType II codebook and Case 0, the gain over the latter being relatively moderate. However, the CSI reconstruction (decoding) at the network side for Case 2 is dependent on the past CSI, making the CSI report non-self-contained. This requires solutions and specification efforts to mitigate the availability of past CSI, e.g., due to UCI loss, CSI dropping, rank change. This is captured in the conclusion from RAN1#120.

Figure 1 Exemplary implementation of Case 2 and the observed UPT gain in Rel-19 study (R1-240725) under small payload size

Conclusion (RAN1#120) Under 10% UCI loss, Case 2 shows small to large performance drop (by Comparing Scenario B and Scenario A). Signaling support to mitigate the UCI loss can improve the performance and helps preserving the gain of Case 2 over Case 0 (by Comparing Scenario C and Scenario A and benchmark schemes).

Moreover, the gain from Case 2 over the two benchmarks decreases as the payload size increases.

Figure 2 Gain of Case 2 over Benchmark 2 (Case 0 CSI compression) for small, medium and large CSI payload sizes as captured in (R1-240725).

Observation#1 Case 2 shows moderate UPT gain over Benchmark 1 (Rel-16 eType II) and smaller UPT gain over Benchmark 2 (Case 0 CSI compression).

- Issues from non-self-contained CSI report (dependency on past reports) may further diminish the gain and usability.

- Higher gain observed for smaller payload sizes and gain decreases as payload size increases.

Similarly, Figure 3 illustrates an exemplary implementation of Case 3 and the observed UPT gain in Rel-19 study (R1-240725). Case 3 is shown to improve performance over the benchmark, i.e., the benchmark based on non-AI/ML based CSI prediction and Rel-18 DD codebook based reporting. The gain is shown to be more pronounced for cell-edge UEs and for the case with higher resource utilization. Unlike Case 2, the CSI report in Case 3 is self-contained making it easier to incorporate it to the NR CSI schemes.

Figure 3 Exemplary implementation of Case 3 and the observed UPT gain in Rel-19 study (R1-240725) mixed dataset.

Observation#2 Case 3 shows moderate UPT gain over the benchmark based on non-AI/ML based CSI prediction and Rel-18 DD codebook based reporting

- Higher gain is observed for cell-edge UEs.

  - Inter-vendor training collaboration

Among the inter-vendor training collaboration options, RAN1#120bis concluded that both Direction C and Direction A are feasible.

Conclusion - RAN1 concludes that both Direction A and Direction C are feasible. - RAN1 concludes that both sub-option 4-1 and sub-option 3a-1 are feasible. - This includes both 3a-1 with and without target CSI sharing.

Regarding model parameters/dataset sharing options, RAN1 has sent LS to RAN2 asking for the feasibility of the OTA and non-OTA approaches, wherein the latter is not visible in 3GPP’s air-interface,. RAN2 (R2-2503169) concluded that there is no consensus on the feasibility of OTA approach. On the other hand, RAN2 asked RAN3, SA2, SA5 to confirm RAN2’s assumption whether the non-OTA approach can be supported within in the Re-19 existing architectural framework. Thus, the conclusion on the feasibility of non-OTA approach is pending to the reply from the involved working groups.

Figure 4 RAN2’s depiction of the two approaches in the LS reply (R2-2503169)

In our view, the following guiding principles should be considered:

- UE-side vendor-agnostic AI/ML model/solution at the network: The network-side AI/ML solution/model is not expected to be tailored to each UE-side implementation. As an example, a single NW-part of two-sided model shall be able to serve different devices (device models) from different UE-side vendors (chipset, device vendors). However, this does not preclude the legacy consideration that there are different classes of UEs in terms of their AI/ML capability; thus, the AI/ML solution/models at the network may scale up with the number of specified classes of UEs rather than the UE-side vendors or device models.

- UE-side AI/ML solutions shall consider different classes of UEs: This is to acknowledge the fact that future UEs may come in different classes in relation to their AI/ML capability. As an example, in Direction C, there could be UEs which support pre-installed AI/ML models (which can occasionally be updated, e.g., through firmware update). On the other hand, in Direction A, there could be a class of UEs which support frequent updates, e.g., per-cell. However, the candidate models for update may be required to be tested/optimized by the same UE-side vendor. A third class, in Direction B , i.e., a high-capability class of UEs, may support model transfer (of standardized structure) from other vendor, e.g., network-vendor.

- Efficiency and convenience: The solution shall consider efficiency in terms of radio access and core network resource usage. As an example, dataset sharing consumes more radio access and core network resources as compared to model sharing, considering the customary case wherein larger size is required for training dataset as compared to reference model. Moreover, the dataset or model exchange mechanism shall consider convenience for the respective vendor in terms of sharing the data.

Figure 4 illustrates various implementation alternatives for Direction C and Direction A. As the simplest alternative, in Alt1, the UE directly deploys the reference model. This does not preclude the UE-side from adapting the model structure and/or complexity to its implementation. For this, the UE side may utilize the reference decoder and the same dataset used for the standardization of the reference model for its model training/adaptation. The network-side, however, may train its own proprietary decoder compatible to the reference encoder.  The network uses field data for its decoder training. RAN1 has observed this deployment provides gain as compared to the case wherein specified reference encoder and decoders are directly deployed.

In a yet another implementation alternative of Direction C, Alt2, the UE may train its encoder based on field data and specified reference decoder. In this alternative, the network would also train its decoder based on field data and specified reference encoder. The associated ID framework can be utilized to align the data used for training. This alternative, does not require to set up its data collection servers in a specific network, e.g., MNO’s network. UE side may update the encoder based on the preferred granularity. For the networks, that the UE side could not  collect data or could not update the model, it can falls back to something similar to Alt1.

A third alternative, Alt3, is based on Direction A. Direction A provides full flexibility for the network in how it initializes model training. Note that Direction C offers common initialization among vendors through the standardized reference model; an aspect that facilitates interoperability but may impact performance. In Alt3, the UE must be capable of switching through encoders whenever the network dictates to. Thus, the UE side must be capable of updating/switching the model and set up it servers in the corresponding networks to abide by the legal or regulatory constraints related to dataset/parameter sharing, e.g., data residency laws and privacy regulations.

Based on the analysis above, Alt1 is the most widely deployable alternative among the three, while Alt2 offers potential performance gains at the expense of reasonably curtailed additional effort.

Figure 5 Implementation options for Direction C and Direction A

Observation#3: Based on Direction C and Direction A of inter-vendor training collaboration options, at least three deployment alternatives identified

- Alt1: UE side directly deploys the specified reference encoder

  - UE-side may still adapt the encoder to its implementation using the specified reference decoder and dataset used to specify reference model.

- Alt2: UE side trains its encoder based on reference decoder and field data collected by the UE-side.

  - For field data collection, indication for consistency on NW-side additional conditions

- Alt3: UE side trains its encoder based on parameter/dataset shared by the network, additional field data collected by the UE-side

  - For field data collection, indication for consistency on NW-side additional conditions

For all alternatives, the network may train it decoder based on field data and/or specified reference encoder.

Observation#4: Among the deployment alternatives of Direction C and Direction A in Observation#3, deployment Alt1 is the widely deployable alternative.

  - Remaining Aspects yet to be Addressed

One of the motivations of Direction A is to enable UE-side vendor to train its encoder based on its preferred model backbone. To evaluate this aspect, we conducted an experiment on whether a UE-side backbone for an actual encoder could be different from the assumed network-side nominal encoder backbone in the NW-side training. As shown in the Figure 6, if the network-side assumed the transformer (TF) encoder in its initial training and shared the dataset {v, ZTF} to the UE-side, and the UE-side attempts to train a CNN model (DenseNet) based on the shared dataset a sever performance degradation is observer (particularly when UE attempts to train its encoder directly based on Alt2). As a yet another attempt to circumvent this error, we assumed the case wherein the network shares mixed dataset for the two backbones, i.e., backbone common dataset – yet a similar degradation was observed. Therefore, it is important for the network to share dataset specific to a certain backbone as well as the corresponding assumed encoder backbone information.

Figure 6

Table 2: Performance for Direction A (Option 4-1) with respect to backbone alignment/mismatch

Observation#5: For Option 4-1 of Direction A, performance degradation is observed when the NW-side encoder backbone associated with the dataset is different from the UE-side encoder backbone.

Proposal#1: For Option 4-1 of Direction A, consider NW-side sharing the encoder backbone assumption associated with the dataset as additional information.

# 2. Summary and Recommendation for Normative Work

Based on the above analysis it can be concluded that CSI compression shows a potential for non-trivial UPT/overhead gain. It is a good practice for 3GPP to adopt this technology as precursor for advanced CSI feedback in 6G. Moreover, the Rel-19 study has addressed issues related to the inter-vendor training collaboration and identified feasible solutions. The inter-vendor collaboration options are studied for Case 0. As this case is a baseline, we recommend it to be considered as first priority for normative work. Considering these, we make the following recommendations

Proposal#2: Conclude the Rel-19 study on two-sided model based CSI compression with recommendation on the following aspects for normative work in Rel-20

- Support at least Direction C from the inter-vendor training collaboration options

- CSI compression sub use Case 0 as first priority

  - CSI compression sub use Case 3 as second priority

- Necessary specification support for measurement and report for inference, network-side and UE-side data collection, network-side and UE-side monitoring, timeline and processing unit

- Pairing method to address consistency between training and inference regarding network-side additional conditions.

# Conclusion

In this contribution, the following observations and proposals are made:

Observation#1 Case 2 shows moderate UPT gain over Benchmark 1 (Rel-16 eType II) and smaller UPT gain over Benchmark 2 (Case 0 CSI compression).

- Issues from non-self-contained CSI report (dependency on past reports) may further diminish the gain and usability.

- Higher gain observed for smaller payload sizes and gain decreases as payload size increases.

Observation#2 Case 3 shows moderate UPT gain over the benchmark based on non-AI/ML based CSI prediction and Rel-18 DD codebook based reporting

- Higher gain is observed for cell-edge UEs.

Observation#3: Based on Direction C and Direction A of inter-vendor training collaboration options, at least three deployment alternatives identified

- Alt1: UE side directly deploys the specified reference encoder

  - UE-side may still adapt the encoder to its implementation using the specified reference decoder and dataset used to specify reference model.

- Alt2: UE side trains its encoder based on reference decoder and field data collected by the UE-side.

  - For field data collection, indication for consistency on NW-side additional conditions

- Alt3: UE side trains its encoder based on parameter/dataset shared by the network, additional field data collected by the UE-side

  - For field data collection, indication for consistency on NW-side additional conditions

For all alternatives, the network may train it decoder based on field data and/or specified reference encoder.

Observation#4: Among the deployment alternatives of Direction C and Direction A in Observation#3, deployment Alt1 is the widely deployable alternative.

Observation#5: For Option 4-1 of Direction A, performance degradation is observed when the NW-side encoder backbone associated with the dataset is different from the UE-side encoder backbone.

Proposal#1: For Option 4-1 of Direction A, consider NW-side sharing the encoder backbone assumption associated with the dataset as additional information.

Proposal#2: Conclude the Rel-19 study on two-sided model based CSI compression with recommendation on the following aspects for normative work in Rel-20

- Support at least Direction C from the inter-vendor training collaboration options

- CSI compression sub use Case 0 as first priority

  - CSI compression sub use Case 3 as second priority

- Necessary specification support for measurement and report for inference, network-side and UE-side data collection, network-side and UE-side monitoring, timeline and processing unit

- Pairing method to address consistency between training and inference regarding network-side additional conditions.

# References

[1] RP-242399,” Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm, Dec, 2023.

[2] RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm, Dec, 2021.

[3] TR 38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”.

[4] R1-240074, “Discussion for further study on AI/ML-based CSI compression”, Samsung, RAN1#116, Athens, Greece.