3GPP TSG RAN WG1 #121                                           R1-2503558

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:	9.2.4

Source: 	Samsung

Title: 	Views on Rel-19 asymmetric DL sTRP/UL mTRP scenarios

Document for:	Discussion and Decision

Introduction

The Rel-19 WID which has been finally revised in RAN#105 [1] includes the following objectives regarding the Rel-19 asymmetric DL sTRP/UL mTRP scenarios.

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

This contribution provides Samsung’s view regarding the Rel-19 asymmetric DL sTRP/UL mTRP scenarios highlighted above.

Potential issues on asymmetric DL sTRP/UL mTRP scenarios

The main objective on this agenda item is to specify necessary enhancements for supporting asymmetric DL sTRP/UL mTRP deployment scenarios, which is depicted as in Figure 1. TRP1 (i.e., both DL/UL capable TRP) in this scenario can operate both DL transmission and UL reception, while TRP2 (i.e., UL TRP) can only operate UL reception.

Figure 1. Asymmetric DL sTRP/UL mTRP scenario

Basic assumptions of UL TRP are as follows:

    - From UL TRP, there is neither DL control information nor DL RS including pathloss RS (PL RS), hence scheduling/control information for UL TRP could be transmitted from TRP1 (i.e., both DL/UL capable TRP), and pathloss between UL only and a UE can be calculated by PL RS from TRP1, and pathloss offset (PL offset) configuration is additionally considered which is defined as difference between two pathloss values where the 1st pathloss is between TRP1 and UE, and the 2nd pathloss is between TRP2 and UE, respectively.

    - Considering heterogeneous network deployment, i.e., the coverage of TRP1 (i.e., both DL/UL capable TRP) and TRP2 (i.e., UL TRP) could be different (e.g., TRP2 can provide a smaller coverage than TRP1).

Within the assumptions as above, the potential issues on asymmetric DL sTRP/UL mTRP scenarios considering UL TRP are listed in the following each of sub-sections.

  - Pathloss offset configuration and update via MAC-CE

In RAN1#117 [2], it was agreed that MAC-CE based update for PL offset included in TCI state as follows:

Agreement For the association between PL offset and joint/UL TCI state, support the following - Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s).

Based on the above agreement, it implies that multiple joint or UL TCI states can be updated by a single MAC-CE. Similar with other TCI state update/activation MAC-CE, we would like to introduce simultaneous update/activation on PL offset among multiple CCs, based on CC lists. If a MAC-CE indicates a serving cell configured as part of a CC list, this MAC-CE can apply to all the other serving cells in the CC list.

Proposal 1. Support to introduce a functionality of simultaneous update on PL offset among multiple CCs in the CC list via MAC-CE.

Also, if MAC-CE updates multiple PL offsets for multiple TCI states, the bitwidth of MAC-CE could be larger. We think that a same PL offset update can be applied to multiple TCI states which share same PL RS, or same power control parameters corresponding to a certain UL TRP. Hence, in order to reduce MAC-CE overhead, it is better to introduce TCI state groups where each of TCI state groups includes multiple TCI states which are corresponding to a certain UL TRP. Then, when a UE receives a MAC-CE updating PL offset of a certain TCI state which is included in a TCI state group, then same PL offset update is applied to all other TCI states in the TCI state group.

Proposal 2. Support to introduce “a TCI state group” which includes a number of TCI states, in order to update PL offset in those TCI states via a single MAC-CE simultaneously.

In addition, we would like to clarify which joint/UL TCI state(s) can be applied by PL offset update MAC-CE. There can be two distinguished groups of Joint/UL TCI state(s): 1) joint/UL TCI state(s) configured with PL offset (where the configured PL offset value can be zero or non-zero), 2) joint/UL TCI state(s) not configured with PL offset. Our understanding on the above cyan highlighted part in the agreement is that the PL offset update MAC-CE is only applicable to group 1), since it said the PL offset value configured in a joint/UL TCI state can be updated by MAC-CE. Regarding group 2), the PL offset update MAC-CE cannot be applied, and UE can assume the corresponding PL offset value as zero.

Proposal 3. Clarify that PL offset update MAC-CE can be only applied to joint/UL TCI state(s) which PL offset value is configured. In other words, PL offset update MAC-CE cannot be applied to joint/UL TCI state(s) which PL offset is not configured.

  - PL offset for other PRACH transmissions

Until now, RAN1 only considered applying PL offset to PRACH transmission when the PRACH is triggered by PDCCH order. BTW, there are some PRACH transmissions which are not triggered by PDCCH order (e.g., PRACH for PCell beam failure recovery procedure). Hence, we would like to clarify whether applying PL offset is also considered PRACH transmission other than triggered by PDCCH order.

Proposal 4. Clarify whether PL offset can be applied to PRACH transmission other than PDCCH order triggered PRACH or not (e.g., PRACH for PCell beam failure recovery procedure).

  - MPE report enhancement for UL transmission toward UL TRP

In Rel-17 unified TCI framework, beam specific MPE report has been introduced in FR2, and resource(s) represented by DL RS (either SSB or CSI-RS) for certain beam(s) and corresponding MPE value(s) can be reported via Enhanced Single Entry PHR MAC CE or Enhanced Multiple Entry PHR MAC CE.

In TS38.331, RRC IE for MPE resource pool has been defined as follows. So far, since beam management is mainly based on DL RS, only either SSB or CSI-RS is considered as MPE resource when beam specific MPE report has been introduced in FR2.

MPE-Resource-r17 ::= SEQUENCE { mpe-ResourceId-r17 MPE-ResourceId-r17, cell-r17 ServCellIndex OPTIONAL, -- Need R additionalPCI-r17 AdditionalPCIIndex-r17 OPTIONAL, -- Need R mpe-ReferenceSignal-r17 CHOICE { csi-RS-Resource-r17 NZP-CSI-RS-ResourceId, ssb-Resource-r17 SSB-Index } }

In Rel-19 asymmetric MTRP scenario, since we now consider UL TRP which may not support DL transmission at least when PL offset is configured, the corresponding MPE value is not identified for UL transmission toward UL TRP in FR2 from gNB side due to lack of DL RS which can be only possible MPE resource. It is beneficial on UL scheduling by achieving MPE value so that gNB can know how much power reduction is expected when a UE transmits UL transmission toward UL TRP. Hence, we would like to additionally introduce UL RS (i.e., SRS) as one of MPE resource for beam specific MPE report.

Proposal 5. For asymmetric MTRP scenario in FR2, support SRS as one of MPE resource for beam specific MPE report.

  - Text proposal on TS38.213

In RAN1#120bis [3], the following agreement was made for SRS resource set without followUnifiedTCI-StateSRS.

Agreement in RAN1#120bis For an SRS resource set not configured with followUnifiedTCI-StateSRS, regarding how to determine the PL offset and the CLPC adjustment state - Alt1: The PL offset and CLPC adjustment state indicated by the TCI state configured to the SRS resource with lowest ID (i.e., reusing the rule specified in Rel-17) and no additional enhancement. When the UE is configured with Rel-19 2 TA, the above design is also applied to TAG.

The above agreement includes not only introducing no additional enhancement (i.e., reusing the rule specified in Rel-17), but also applying the rule to PL offset. Hence, to reflect this in the current specification, we would like to suggest the following text proposal.

Proposal 6. Adopt the following text proposal expressed by yellow highlight in Clause 7.3.1.1.2 in TS38.213.

< Unchanged parts are omitted > - in clause 7.3.1, if p0AlphaSetforSRS is provided, - if followUnifiedTCI-StateSRS is provided for a SRS resource set, the values of PO_UE_SRS,b,f,cqs, αSRS,b,f,cqs, and SRS power control adjustment state l are provided by p0AlphaSetforSRS associated with the indicated TCI-State or TCI-UL-State, or by p0AlphaSetforSRS associated with the CandidateTCI-State or CandidateTCI-UL-State indicated in the LTM Cell Switch Command MAC CE - else, if followUnifiedTCI-StateSRS is not provided for a SRS resource set and for a SRS resource from the SRS resource set, the values of PO_UE_SRS,b,f,cqs, αSRS,b,f,cqs, and SRS power control adjustment state l are provided by p0AlphaSetforSRS associated with TCI-State or TCI-UL-State of an SRS resource with lowest SRS-ResourceId in the SRS resource set, and a RS index qd for obtaining a pathloss estimate for the SRS transmission is provided by pathlossReferenceRS-Id-r17 associated with or included in the TCI-State or TCI-UL-State of an SRS resource with lowest SRS-ResourceId in the SRS resource set, and the value of PLb,f,c,offset is provided by pl-Offset associated with TCI-State or TCI-UL-State of an SRS resource with lowest SRS-ResourceId in the SRS resource set. < Unchanged parts are omitted >

  - RRC parameter

The value range and description of the RRC parameter startingBitOfFormat2-3 shall be revised accordingly, based on the following agreement.

Agreement in RAN1#118bis About the extended value range 1~X of starting bit of blocks in DCI format 2_3 in Rel-19, support Alt1: - Alt1: X = 45 (to be captured in RAN2 spec) - This feature is a separate UE capability and is appliable to any Rel-19 UE who supports this UE capability, regardless this UE supports two separate SRS CLPC adjustment states or not. - Note: X=45 can be used for operations in FR1 in shared spectrum or FR2-2 and X = 43 otherwise

Hence, we would like to propose the following RRC parameter description and value range.

Proposal 7. Revise the description and value range for RRC parameter startingBitOfFormat2-3.

| Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range |
| --- | --- | --- | --- | --- |
| startingBitOfFormat2-3 | Existing |  | This parameter is used to indicate a starting bit position of DCI format 2_3. The value range of this parameter is extended from 1~31 to 1~X45. FFS: the value of X>31. | INTEGER (1..45) |

Conclusion

In this contribution, the following observation and proposals are given:

Proposal 1. Support to introduce a functionality of simultaneous update on PL offset among multiple CCs in the CC list via MAC-CE.

Proposal 2. Support to introduce “a TCI state group” which includes a number of TCI states, in order to update PL offset in those TCI states via a single MAC-CE simultaneously.

Proposal 3. Clarify that PL offset update MAC-CE can be only applied to joint/UL TCI state(s) which PL offset value is configured. In other words, PL offset update MAC-CE cannot be applied to joint/UL TCI state(s) which PL offset is not configured.

Proposal 4. Clarify whether PL offset can be applied to PRACH transmission other than PDCCH order triggered PRACH or not (e.g., PRACH for PCell beam failure recovery procedure).

Proposal 5. For asymmetric MTRP scenario in FR2, support SRS as one of MPE resource for beam specific MPE report.

Proposal 6. Adopt the following text proposal expressed by yellow highlight in Clause 7.3.1.1.2 in TS38.213.

< Unchanged parts are omitted > - in clause 7.3.1, if p0AlphaSetforSRS is provided, - if followUnifiedTCI-StateSRS is provided for a SRS resource set, the values of PO_UE_SRS,b,f,cqs, αSRS,b,f,cqs, and SRS power control adjustment state l are provided by p0AlphaSetforSRS associated with the indicated TCI-State or TCI-UL-State, or by p0AlphaSetforSRS associated with the CandidateTCI-State or CandidateTCI-UL-State indicated in the LTM Cell Switch Command MAC CE - else, if followUnifiedTCI-StateSRS is not provided for a SRS resource set and for a SRS resource from the SRS resource set, the values of PO_UE_SRS,b,f,cqs, αSRS,b,f,cqs, and SRS power control adjustment state l are provided by p0AlphaSetforSRS associated with TCI-State or TCI-UL-State of an SRS resource with lowest SRS-ResourceId in the SRS resource set and a RS index qd for obtaining a pathloss estimate for the SRS transmission is provided by pathlossReferenceRS-Id-r17 associated with or included in the TCI-State or TCI-UL-State of an SRS resource with lowest SRS-ResourceId in the SRS resource set, and the value of PLb,f,c,offset is provided by pl-Offset associated with TCI-State or TCI-UL-State of an SRS resource with lowest SRS-ResourceId in the SRS resource set. < Unchanged parts are omitted >

Proposal 7. Revise the description and value range for RRC parameter startingBitOfFormat2-3.

| Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range |
| --- | --- | --- | --- | --- |
| startingBitOfFormat2-3 | Existing |  | This parameter is used to indicate a starting bit position of DCI format 2_3. The value range of this parameter is extended from 1~31 to 1~X45. FFS: the value of X>31. | INTEGER (1..45) |

References

- RP-242394, WID revision: NR MIMO Phase 5, RAN#105.

- Chair’s note, RAN1#117.

- Chair’s note, RAN1#120bis.