3GPP TSG RAN WG1 #121		R1-2503566

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:	9.4.1

Source:	Samsung

Title:	Views on Physical channels design – modulation aspects

Document for:	Discussion and decision

# Introduction

Ambient Internet of Things (A-IoT) was studied by 3GPP in Rel-19 with solutions captured in [1]. RAN WGs start a Rel-19 WI with general scopes and objectives in [2].

This contribution discusses issues on modulation aspects of A-IoT physical channels design, including R2D CP handling, OFDM boundary alignment, indication of M values, and R2D chip duration aspects.

# R2D modulation: OOK-4

## CP handling

In legacy NR OFDM waveform generation, at transmitter side, the CP is copied from the end of the symbol and then prepended at the start of the corresponding symbol. The CP is removed at the receiver when capturing the samples for the FFT window and, therefore, it serves as a guard time to mitigate the inter-symbol interference. By prepending the CP in a circular manner, it also allows to maintain sub-carrier orthogonality and, thereby, preventing inter-carrier interference.

For R2D signal generation with OOK modulation, the CP insertion can be still beneficial in terms of reusing the existing OFDM hardware for OOK signal generation and also for other NR UEs served using OFDM waveform by maintaining the sub-carrier orthogonality. If reusing the same hardware for OFDM from NR Uu, CP will be inserted in the time domain before each OFDM symbol. However, the inserted CP may break the normal line coding scheme by introducing false rising/falling transition edge at the start of, the end of, or within the CP duration.

Therefore, R2D CP handling was discussed in A-IoT SI phase as a key issue to be solved. Method Type 1 that removal of CP at device without specified transmit-side was agreed for supported M values <= 12 in RAN1#120bis, and the candidate methods for M values >12 are still under discussion, as follows [3]:

Agreement For further down-selection among CP handing which retains subcarrier orthogonality, at least for PRDCH, at least Method Type 1 is supported - For supported M values <= 12 - RAN1 will not further pursue additional CP handling design - For supported M values > 12 - RAN1 will further down-select one from the followings - Option 1: Candidate 3 of M2-1-1 (as per agreements from RAN1#120) - Insert padding chips only at the end OOK chips of OFDM symbol - The last 2 out of M OOK chips at the end of an OFDM symbol are always ‘ON’ - Option 3: RAN1 will not further pursue additional CP handling design

For Method Type 1, A-IoT devices are expected to determine CP position, and remove corresponding sampling points during the CP position by assuming alignment between start of R2D transmission and NR OFDM symbol boundary. The CP removal can be performed by device counting the indices of sampling points and removing sampling points with indices corresponding to CP duration, e.g. sampling points 0~9, 138~146, 275~283, etc.. Consequently, the SFO impact will introduce residual error edges after CP removal. Even if considering synchronization calibration according to Manchester line encoding waveform, there are still residual SFO. Figure 1 illustrates an example that how synchronization drift impacts the performance of CP removal and R2D decoding.

Figure 1 Examples of incorrect CP removal and false detection due to synchronization drift

Alt M1-1 proposes to solve the issue of different CP length between LCP of NR OFDM symbol 0, 7 and NCP of other OFDM symbols. According to 38.211, the length of LCP is 4.68μs and the length of NCP is 5.2μs under 15 KHz SCS. Assuming sampling frequency of 1.92Msps as baseline, LCP and NCP correspond to 10 and 9 sampling points, respectively. Therefore, even if assuming the offset is not accumulated across OFDM slots by device implementation, there will be an offset of 1 or 2 sampling points for some code chips.

The combination of SFO impact and CP length difference degrades accuracy of CP removal. For large M values of M=24, each code chip corresponds to 5.33 sampling points, and a typical sampling offset of 25%~67% code chip length can be observed between the practical CP position and the device counted CP position. Under the large offset, it is difficult for device to accurately distinguish data chips and CP chips. Therefore, Alt M1-1 is not appropriate for large M values.

Observation 1: Alt M1-1 is not appropriate for large M values, e.g. M=24. There exists a potential offset between practical CP position and device expected CP position, due to not distinguishing LCP/NCP and also SFO impact. With M=24, a typical sampling offset of 25% ~ 67% of code chip length are observed, which may severely impacts R2D reception performance.

Alt M1-2 proposes to distinguish code chip and false chip during CP position by the duration between transition edges. An example is illustrated in Figure 2, wherein T denotes the nominal chip length. The actual chip length may have a slight variation from its nominal value, i.e., T±δ. Δ denotes the nominal CP length, which may also have a slight variation due to inaccurate clocks at the transmitter or receiver, signal propagation, or sampling timing error.

Figure 2 Example of Alt M1-2 for CP removal based on chip duration

Based on the rising/falling edge detection, the OOK chips can be detected as:

- A chip with duration T+Δ, e.g., C1 in symbol n, due to CP having the same value with the chip.

- A chip with duration Δ, e.g., CP in symbol n+1, due to CP having different value with the neighboring chips.

- A chip with duration T, 2T, …, etc., which are normal chips.

The receiver detects chips based on the detected chip duration as follows:

- If a detected chip duration is Δ, which may be further associated with relative variation parameter δ such that the detected chip duration falls into [Δ–δ, Δ+δ], the receiver identifies that the corresponding chip as a CP and ignores it as invalid.

- If a detected chip duration is T+Δ, which may be further associated with relative variation parameter δ such that the detected chip duration falls into [T+Δ–δ, T+Δ+δ], the receiver identifies that the corresponding chip is composed of CP plus a normal chip and detects it as a single chip.

- Otherwise, the detection decision is made based on the nominal chip duration T, i.e., an integer multiples of T with some variation ±δ.

Therefore, the performance of Alt M1-2 for CP handling may depend on two aspects: how strong the CP duration Δ differentiates with normal code chip length T or 2T, and how strong the time offset δ impacts the differentiation among {T+Δ, Δ, T or 2T}. For M=24, T corresponds to 5.33 sampling points, thus the length 2T is very similar to Δ = LCP/NCP with only 0.67~1.67 sampling point difference. Considering potential time drift of δ and issue of code chip length with non-integer sampling points, device is difficult to distinguish OOK chip within CP from data chips. Therefore, Alt M1-2 is not applicable for M=24.

Observation 2: Alt M1-2 is not appropriate for M = 24. The difference between LCP/NCP and two line code chips has only 0.67~1.67 sampling points time length, thus become non-distinguishable at receiver side.

Combinations of Alt M1-1 and Alt M1-2 was also considered. However, for M=24 under which Alt M1-1 cannot work, the introduction of Alt M1-2 could only introduce limited improvement due to the similar length between 2 chips and LCP/NCP. In addition, both alternatives are impacted by SFO, and the combination cannot resist the SFO impact as well. Therefore, Method type 1 may work for small M values, but are not preferred for large M values > 12, e.g. M=24.

Moreover, error propagation of CP removal should also be taken into consideration. Once a residual OOK chip after CP removal being incorrectly decoded as part of code chip, all the following code chips might be mistaken. Given that no FEC is supported for R2D, it becomes more difficult for the device to correct the propagated error. Therefore, in most cases of our observation, only if all the CPs during a R2D transmission being correctly removed, the R2D transmission can be decoded successfully.

With small TBS, e.g. 20-bit, which typically across 2 or 3 CP positions of OFDM symbols according to R-TAS design, only 2 or 3 times of CP removal are performed, thus final performance can achieve the 10-2 BLER target. However, for R2D transmissions with large payload size, the impact of incorrect CP removal under Method type 1 is increased. For instance, R2D transmission with 200-bit TBS needs CP removal of ~18 times. Therefore, the BLER performance of Method Type 1 degrades severely when R2D payload size increased.

Figure 3 illustrates performance of Method type 1 under M=24 with message size of {20+CRC-6, 200+CRC-16} bits. Both Alt M1-1 and Alt M1-2 are used in combination, device will detect inserted CP according to both position of sampling points and also the length of OOK chip between two adjacent voltage transition edges, in order to avoid false detection of remaining CP chip due to inaccurate CP removal impacted by SFO. In this evaluation, message size is 20 bits with CRC-6 attached, and sampling rate is 1.92 MHz at device side. More simulation assumptions are provided in the Appendix.

It can be observed that with M=24, the performance of small TBS of 20 bits can achieve 10% and 1% BLER targets, but for large TBS of 200 bits, the target SNR at 10-1 BLER requirement increases ~7dB compared with 20-bit case, and extremely increases at 10-2 BLER requirement. Therefore, the performance and feasibility of Method Type 1 for large payload size needs to be carefully evaluated and discussed.

Observation 3: With M=24, the performance of Method type 1 degrades with TBS increasing, due to increased number of residual error of CP removal per OFDM symbol. Compared with small message size of TBS=20 bits, the SNR threshold of TBS=200 bits increases around 7 dB at 10% target BLER, and extremely increases at 1% target BLER.

Figure 3 Performance of MT1, Alt M1-1+M1-2, M=24

For Method Type 2, the candidate option agreed in RAN1#120bis includes that the last 2 out of M OOK chips at the end of an OFDM symbol are always ‘ON’. Given that LCP/NCP length are slightly smaller than 2 OOK chips under M=24, it is ensured the voltage level within CP is fixed to ‘ON’ by copying the end OOK chips of an OFDM symbol. Figure 4 illustrates 3 examples of introducing padding chips to resolve CP handling issue. In the examples, line encoded chips corresponding to information bits are colored by green, and the last 2 chips out of M=24 OOK chips and copied CP are colored by blue.

Figure 4 Example of Option 1: the last 2 out of M OOK chips at the end of an OFDM symbol are always ‘ON’

After fixing ‘ON’ status to the last 2 chips, false rise/falling edge will not happen within CP, and may happen at the boundary between the 22-nd and last 2 chips of an OFDM symbol, and at the boundary between CP and the first code chip. Therefore, a long OOK chip containing CP, e.g. {last 2 chips in OFDM symbol n-1 + CP in OFDM symbol n} as blue marked line in Figure 4, will occur in PRDCH transmission. The total length of long OOK chip is approximately {4, 5, 6} times of code chip length depending on whether the 22-nd chip in OFDM symbol n-1 and the first chip in OFDM symbol n are also ‘ON’, it becomes easier to be distinguished by the receiver and can better resist SFO impacts. Figure 5 illustrates an example that with large M values, the long OOK chip containing CP are easily detected by device, and the boundary can be used to calculate residual SFO and assist to calibrate clock drift. In addition, the long OOK chip can also be tracked to fix potential CP removal error and avoids error propagation.

Figure 5 Detection of long OOK chip for clock calibration, SFO resistance and CP removal error correction using Option 1

Figure 6 illustrates performance of option 1 and option 3 under M=24. The evaluation parameters are the same as for Method Type 1 with message size of 20 bits and 200 bits, respectively. It can be observed that with TBS=20 bits, Option 1 outperforms 2.1 dB than option 3 at 10% target BLER, and the performance gap increased to ~8.4 dB with TBS = 200 bits. Compared with option 1, the SNR-BLER performance of option 1 is more stable with TBS increasing, given that the long OOK chip can assist to calibrate clock timing and OFDM symbol boundary.

Observation 4: With M=24, TBS=20 bits, Option 1 outperforms 2.1 dB than Option 3 at 10% BLER target. With M=24, TBS=200 bits, Option 1 outperforms 8.4 dB than Option 3 at 10% BLER target.

Therefore, the following is proposed for CP handling methods:

Proposal 1: For supported M values >12, Option 1: the last 2 out of M OOK chips at the end of an OFDM symbol are always ‘ON’ is applied for R2D transmissions at least for PRDCH.

(a) TBS = 20 bits				  (b) TBS = 200 bits

Figure 6 Performance of Option 1 (MT2) and Option 3 (MT1) with M=24

## OFDM symbol boundary alignment

OFDM symbol boundary alignment was discussed with agreement that end of R2D transmission should be aligned with OFDM symbol boundary by padding, but how to generate the padding are still under discussion. The following was agreed in RAN1#120bis [3]:

Agreement When the generated number of chips for the R2D transmission does not fully occupy the last OFDM symbol, padding is used. - Padding is to be down-selected among the following alternatives: - Alt 1a: The content of padding is up to reader implementation and transparent to device. - Note: the timeline determination of any timing relationship refers to the end of padding. - Note: it implies the device should be aware of the duration of padding or the last OFDM symbol boundary by implementation. FFS how accurately the device can be aware. - Alt 1b: The content of padding is up to reader implementation and transparent to device. - Note: the timeline determination of any timing relationship refers to the start of the padding. - Alt 2: Specify the detailed content of padding - Note: the timeline determination of any timing relationship refers to the end of padding. - Note: the end chip(s) of the padding content shall follow the CP handling solution determined in RAN1, and may be affected by other agreements.

For R2D waveform generation using a DFT-s-OFDM based transmitter, when the generated number of chips for the R2D transmission does not fully occupy the last OFDM symbol, as illustrated by Figure 7, at least padding can be used for OFDM symbol boundary alignment, otherwise it will impact interference estimation from adjacent readers or coexisting NR system.

Figure 7 A illustration of a partially occupied last OFDM symbol

In our view, both options under Alt 1 are self-contradicting and not preferable.

- Alt 1a: It is self-contradicting in the sense that it is a reader implementation option, which is transparent to device, but it requires a device to accurately determine the end of padding signal for timing purpose by detecting the device-transparent padding signal. It is not testable given that it is an implementation option and a device cannot be required to detect it.

- Alt 1b: It is self-contradicting in the sense that the reference time for determining timing relationship is the start of the padding signal, while the reader is still in transmission until the end of padding signal. The only way to make this option workable is to define TRD considering the worst case, i.e., at least one OFDM symbol duration considering that padding signal can occupy up to one OFDM symbol duration.

- In any of the above Alt 1a and Alt 1b, it is not preferable to leave padding signal as an implementation option as it makes device detection of padding signal more complex compared to detecting a known padding signal. Also, an implementation-based padding signal, e.g., based on Manchester-rule violation, can be easily confused with PRDCH signal due to device SFO impact and error due to channel.

Observation 5: Padding option Alt 1a and Alt 1b are not preferable given that they are self-contradicting. Also, implementation-based option will make the device determination more complex and erroneous compared to a defining padding signal.

In our view, specifying padding signal is the most preferable way given that (i) it is easy for the device to determine a known padding signal compared to unknown implementation-based signal, (ii) it is clearer solution in determining the timing relationship, and (iii) it is testable.

Observation 6: Specifying padding signal is preferable given (i) easiness in device implementation, (ii) clarity in determining the timing relationship, and (iii) testability.

The content of padding can generally be consisted of bits or code chips with pre-defined value, e.g. padding with all-zero bits, all-one bits, padding with an alternation of high-low voltage level chips, or padding with low voltage level chips.

The length of padding is also relevant with CRC attachment and the existence of postamble. If bit-level padding is used, it can be added before CRC attachment, e.g., the length of padding bits can be determined and added in the baseband before the CRC attachment, if CRC is attached, such that the end of the PRDCH transmission after line encoding is aligned with the next OFDM symbol boundary. If zero-chips or alternation of zero-chip and one-chip are used for padding, it can be added after CRC attachment, but it needs to be distinguished with PRDCH including CRC to ensure correct CRC check.

Proposal 2: When the generated number of chips for the R2D transmission do not fully occupy the last OFDM symbol, padding is used based on one of the following options:

- Option 1) zero-bit padding or one-bit padding before CRC attachment

- Option 2) zero-chip padding or alternation of zero-chip and one-chip padding after CRC attachment

If the PRDCH is attached with a postamble, the length of padding bits is determined such that the end of postamble is aligned with the next OFDM symbol boundary.

Proposal 3: When PRDCH is attached with postamble and padding is performed, the end of postamble is aligned with the next OFDM symbol boundary.

## Indication of M values

The supported M values for R2D transmission were discussed in RAN1#120bis as follows:

Agreement For R2D, for the OOK-4 modulation for M-chip per OFDM symbol transmission, the maximum M value is 24. - RAN1 will further determine the set of M values up to the maximum M value. - The maximum M value applicable to the PRDCH is 24. - The maximum M value applicable to the R-TAS is not larger than 24. Agreement For R2D, at least for PRDCH, the set of M values is {2, 6, 12, 24}. - FFS: whether/how CAP use same M values set as PRDCH.

In our view, the CAP should use the same M value corresponding to the M value used in the following PRDCH. First of all, there is no clear motivation to do otherwise. Secondly, using the same M value between CAP and the following PRDCH helps the device to acquire a precise chip synchronization without any unnecessary steps.

Proposal 4: CAP using the same M value as the following PRDCH is the most simple and precise method for a device to acquire chip length without any unnecessary steps.

## R2D chip duration

Calculation of R2D chip duration was discussed in RAN1#120 with no consensus. In our understanding, the M OOK chips are contained in the non-CP part of an OFDM symbol, and then CP is additionally prepended at the beginning of the OFDM symbol. Therefore, the chip duration is independent of CP length, i.e. R2D chip duration of non-CP part is denoted as C = (1/SCS) / M.

In the CP part of an OFDM symbol, since CP length is not integer multiple of OOK chip with given M values in [1], the waveform of CP by copying the end OOK chips of non-CP part contains partial of an OOK chip. Therefore, it should be clarified that the R2D chip duration calculated for non-CP part should not limit practical OOK chip length within CP.

In addition, the calculation of chip duration mainly focused on PRDCH data rate. Regarding chip length or chip rate definition for X-ambles, since the sequence design for SIP and CAP are still under discussion, the R2D chip rate discussion should be applied to PRDCH only, and the chip duration for R2D preamble, if needed, are discussed separately.

Proposal 5: For PRDCH chip duration,

- For non-CP part, chip duration is denoted as C = (1/SCS) / M.

- Within CP part, the R2D OOK chip duration can be larger or less than C after CP insertion by copying code chip(s) in the end of an OFDM symbol.

- Definition of chip duration for R2D preamble is depending on preamble design and discussed separately.

# D2R modulation: OOK and BPSK

In RAN1 120, the following agreement on D2R modulation scheme was made [4].

Agreement For a D2R transmission, it is up to device implementation to use OOK or BPSK modulation. It is assumed that a device operates consistently with the same D2R modulation in the same inventory/command round. Agreement For D2R BPSK modulation - After line coding or square wave, chip “1” is mapped to the real-valued modulation symbol {1} and chip “0” is mapped to the real-valued modulation symbol {-1} in the baseband. Agreement For D2R OOK modulation - After line coding or square wave, chip “1” is mapped to the real-valued modulation symbol {1} and chip “0” is mapped to the real-valued modulation symbol {0} in baseband.

The remaining issues for D2R modulation include a set of supported D2R chip lengths and a signaling for indicating D2R chip length, which is to be discussed in AI 9.4.2 and AI 9.4.4.

The D2R modulation is neither restricted by underlying OFDM symbol boundaries as in R2D nor has CP handling issue. Thus, it is rather straightforward compared to R2D. The OOK chip duration or BPSK symbol duration, which is referred to as D2R chip duration, will be indicated in the R2D control information from a predefined set of D2R chip durations per agreement made in the previous meeting. The set of supported D2R chip durations, although it is not restricted by OFDM symbol durations, can be based on R2D chip durations such that a device can take a reference time duration from the preceding PRDCH. For instance, the set of supported D2R chip durations can be the same as the set of supported R2D chip durations, or the set of supported D2R chip durations has an integer multiplication or division relationship from the set of supported R2D chip rates.

Observation 7: The remaining issues for D2R modulation including 1) a set of supported D2R chip lengths and 2) a signaling for indicating D2R chip length can be discussed in AI 9.4.2 and AI 9.4.4.

# Conclusion

In this contribution, we discussed modulation aspects for physical channel design with the following observations and proposals:

Observation 1: Alt M1-1 is not appropriate for large M values, e.g. M=24. There exists a potential offset between practical CP position and device expected CP position, due to not distinguishing LCP/NCP and also SFO impact. With M=24, a typical sampling offset of 25% ~ 67% of code chip length are observed, which may severely impacts R2D reception performance.

Observation 2: Alt M1-2 is not appropriate for M = 24. The difference between LCP/NCP and two line code chips has only 0.67~1.67 sampling points time length, thus become non-distinguishable at receiver side.

Observation 3: With M=24, the performance of Method type 1 degrades with TBS increasing, due to increased number of residual error of CP removal per OFDM symbol. Compared with small message size of TBS=20 bits, the SNR threshold of TBS=200 bits increases around 7 dB at 10% target BLER, and extremely increases at 1% target BLER.

Observation 4: With M=24, TBS=20 bits, Option 1 outperforms 2.1 dB than Option 3 at 10% BLER target. With M=24, TBS=200 bits, Option 1 outperforms 8.4 dB than Option 3 at 10% BLER target.

Observation 5: Padding option Alt 1a and Alt 1b are not preferable given that they are self-contradicting. Also, implementation-based option will make the device determination more complex and erroneous compared to a defining padding signal.

Observation 6: Specifying padding signal is preferable given (i) easiness in device implementation, (ii) clarity in determining the timing relationship, and (iii) testability.

Observation 7: The remaining issues for D2R modulation including 1) a set of supported D2R chip lengths and 2) a signaling for indicating D2R chip length can be discussed in AI 9.4.2 and AI 9.4.4.

Proposal 1: For supported M values >12, Option 1: the last 2 out of M OOK chips at the end of an OFDM symbol are always ‘ON’ is applied for R2D transmissions at least for PRDCH.

Proposal 2: When the generated number of chips for the R2D transmission do not fully occupy the last OFDM symbol, padding is used based on one of the following options:

- Option 1) zero-bit padding or one-bit padding before CRC attachment

- Option 2) zero-chip padding or alternation of zero-chip and one-chip padding after CRC attachment

Proposal 3: When PRDCH is attached with postamble and padding is performed, the end of postamble is aligned with the next OFDM symbol boundary.

Proposal 4: CAP using the same M value as the following PRDCH is the most simple and precise method for a device to acquire chip length without any unnecessary steps.

Proposal 5: For PRDCH chip duration,

- For non-CP part, chip duration is denoted as C = (1/SCS) / M.

- Within CP part, the R2D OOK chip duration can be larger or less than C after CP insertion by copying code chip(s) in the end of an OFDM symbol.

- Definition of chip duration for R2D preamble is depending on preamble design and discussed separately.

# References:

- TR 38.769-201, Study on solutions for ambient IoT (Internet of Things)

- RP-250896, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, 3GPP TSG RAN Meeting #107, Incheon, Korea, March 12th-14th, 2025

- RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #120bis, Wuhan, China, April 7th – 11nd, 2025.

- RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #120, Athens, Greece, February 17th – 21st, 2025.

# Appendix

The evaluation parameters for R2D CP handling are provided as below.

Table 1 Evaluation assumptions for R2D CP handling

| System Parameter | Value |
| --- | --- |
| Sampling rate | Reader: 15.36 MHz Device: 1.92 MHz |
| Message size | 20 bit + CRC-6, 200 bit+CRC-16 |
| M value | 24 |
| SFO | [0.1~1]*10^5 ppm |
| Channel type | TDL-A 30ns |
| Bandwidth | 1~6 PRB according to signal BW |