3GPP TSG RAN WG1 #121		R1-2503567

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:	9.4.2

Source:	Samsung

Title:	Views on Physical channels design – line coding, FEC, CRC, repetition aspects

Document for:	Discussion and decision

- Introduction

As the Internet of Things (IoT) continues to expand, billions of connected devices are being deployed across industries. It is expected that the number of connected devices will reach approximately 40 billion by 2030, which is about 4.7 times the expected world population (~8.5 billion) at that time[1]. However, one of the biggest challenges facing IoT deployments is the increasing maintenance cost and environmental impact associated with battery replacement and manual recharging. Many IoT devices used in smart cities, industrial automation, and remote monitoring, are deployed in locations where battery replacement is impractical or even impossible. This has led to rising concerns regarding operational costs, efficiency, and sustainability.

This challenge has paved the way for a market that demands new technologies supporting batteryless devices with no energy storage capability or devices with energy storage that do not need to be replaced or recharged manually. Therefore, solutions for Ambient IoT within 3GPP systems have been studied, including a Rel-18 RAN-level study that produced TR 38.848 and a Rel-19 RAN-WG level study that led to TR 38.769. The Rel-18 RAN-level study defined use cases, deployment scenarios and connectivity topologies, design targets and other aspects for Ambient IoT within 3GPP. They also conducted preliminary feasibility assessments for device power consumption, complexity, coverage, latency and other relevant factors. The Rel-19 RAN-WG level study explored detailed solutions, evaluating their feasibility and addressing key aspects such as basic device architectures, the impact of device availability due to energy harvesting, physical channel and signal design, multiplexing/multiple access schemes, as well as carrier waveform and characteristic identification.

In this contribution, we share our view on the detailed discussion of the physical channels design aspects, including line coding, FEC, CRC and repetition.

- FEC

The following agreements were endorsed during the previous meetings:

Agreement (RAN1#120) RAN1 concludes that generator polynomials of LTE CC with constraint length of 7 and coding rate of 1/3 from TS36.212 are reused as the mother code for Ambient IoT. Agreement (RAN1#120b) For the initial values of the shift register of the CC encoder, the initial values of the shift register of the CC encoder are set to the values corresponding to the last (K-1) information bits defined in TS 36.212. - Note: K is the constraint length. - For discussion on timing relationships in 9.4.4, the entire processing time for encoding (including finishing CRC encoding) should be taken into account

Additional code rate for D2R

During the previous meetings, additional code rates beyond the LTE convolutional code of 1/3 were discussed, including the possibility of supporting higher or lower code rates (e.g., 1/2, 1/6).

In order to achieve a higher code rate, the encoded bits need to be punctured based on a puncturing pattern. However, puncturing can still increase the complexity on the device side, as devices are required to apply puncturing according to the pattern. Moreover, even with a simple puncturing pattern, decoding performance can still be degraded under low SNR conditions. Considering that no FEC already has been adopted for high SNR scenarios (i.e., cases where the reader and the device are in close proximity), the additional benefit of supporting puncturing is limited.

A lower code rate can be obtained by repeatedly transmitting the codeword, given that only the LTE convolutional generator polynomial with a coding rate of 1/3 is reused. However, in Rel-19 Ambient IoT, block-level repetition—where the bits after CRC attachment and before FEC encoding are repeated—has already been agreed. Therefore, additionally repeating the coded bits after FEC encoding would be redundant and inefficient.

Observation 1: Given that no FEC is already supported in D2R, there is limited justification for introducing puncturing to achieve higher code rates.

Observation 2: Since block-level repetition has already been agreed upon, additionally repeating coded bits is unnecessary and redundant.

Proposal 1: There is no consensus to support FEC code rate 1/2 in Rel-19.

Configurations for FEC

Applying or not applying FEC to D2R transmission is controlled by the reader. We believe that the use of FEC should remain consistent within the same inventory or command round. In RFID systems, for example, all tags within a query round are configured to use the same modulation type and data rate. Additionally, Rel-19 Ambient IoT considers only proximity determination solution 1. Therefore, it may be challenging for the reader to determine whether each device is close enough to operate without FEC, based on D2R reception on a per-device basis. For these reasons, it is appropriate for the reader to configure the use of FEC uniformly for all devices via A-IoT paging.

Observation 3: It can be difficult for the reader to determine whether each device is close enough to operate without FEC, based on proximity determination solution 1.

Proposal 2: The use of FEC is configured via A-IoT paging. It is assumed that all devices operate in the same manner with respect to FEC usage within the same inventory or command round.

- Note: This issue can be addressed under A.I. 9.4.4 or within this A.I.

- CRC

The following agreements were made regarding CRC attachment in the previous meeting:

Agreement (RAN1#120b) When CRC is attached to a PRDCH or PDRCH transmission, when the number of information bits is ≤ 24 bits, CRC-6 is used. Otherwise, when the number of information bits is > 24 bits, CRC-16 is used. Agreement (RAN1#120b) There is no case in D2R or R2D where CRC is not attached. Conclusion (RAN1#120b) RAN1 will not address the FFS in the agreement from RAN1#120: Agreement When CRC is attached to a PRDCH or PDRCH transmission, - When the number of information bits is ≤ X bits, CRC-6 is used. Otherwise, when the number of information bits is > X bits, CRC-16 is used. Down-selection by RAN1#120bis from the following for X considering the balance of overhead and probability of undetected error: - Alt. 1: 24 - Alt. 2: 56 - FFS impact of segmentation, if any - Note: impact may not be in RAN1

Based on the outcome made in the previous meeting, it is understood that fundamental aspects of CRC design are complete.

One potential remaining issue could be joint or separate CRC for R2D control and data, if L1 R2D control information is supported. For PRDCH, if any R2D control information is supported and carried by PRDCH, the study identified that either 1) joint CRC is attached to R2D control information and R2D data, or 2) separate CRC is attached to R2D control information and R2D data, respectively.

Figure 5 Different options of CRC attachment for PRDCH

If the R2D control information is provided, it must have a separate CRC so that a device can first read R2D control information to acquire information necessary to read the following R2D data, such as the intended recipient of the R2D data and other transmission parameters.

If a single CRC is attached for both the R2D control and data, the device needs to receive the entire PRDCH, perform demodulation of the entire PRDCH, check CRC and, then, read the header information. If the device identifies that the message is intended to the device, then the device reads the rest of the signal. In many cases, however, the received PRDCH may not be addressed to the device and the described procedure results in waste. Clearly, there is no motivation for defining L1 control information, if joint CRC is attached for L1 control and data.

If CRC is separately attached to the R2D control and R2D data, a device first processes and reads the header information and, if the device identifies that the message is intended to the device, then the device processes and reads the following payload. Given that A-IoT device operates using limited energy, this option is more preferable to extend the device’s sustainable operation time by avoiding unnecessary processing.

Observation 4: If L1 R2D control information is defined, it must have a separate CRC so that a device can first read R2D control information to acquire information necessary to read the following R2D data.

The R2D control information field has a fixed size for a device to identify the end of R2D control information. If the R2D control information field has a fixed size, it is also reasonable to have a fixed CRC size.

Proposal 3: If L1 R2D control information is supported, the L1 control information is attached with a separate CRC-6. If L1 R2D control information is not supported, no need for further discussion.

- Repetition

[H] Proposal 5.1-v2 For the number of block-level repetitions, down-select from the following options: - Option 1: {1, 2}. - Huawei, Samsung, Docomo, Spreadtrum, Ericsson - Option 2: {1, 2, 4}. - LGE, CMCC, ZTE, Panasonic, OPPO, MediaTek, NEC, Docomo, Spreadtrum, Ericsson, CATT - Note: When the number of block-level repetition is 1, it indicates no repetition. Proposal1 For the number of block-level repetitions, {1, 2, 4} repetitions are supported. - Note: When the number of block-level repetition is 1, it indicates no repetition. Proposal2 For the number of block-level repetitions, {1, 2, [4]} repetitions are supported. - Note: When the number of block-level repetition is 1, it indicates no repetition. - Note: no other candidate value than 4 is considered

During the RAN1#120b meeting, the number of block-level repetitions for D2R transmission was discussed, but no consensus was reached. Block-level repetition is intended to enhance D2R coverage. However, coverage enhancement can also be achieved through other means such as FEC and lower data rates. In particular, the maximum Tb of 266.67 us, agreed in the previous meeting, can provide sufficient coverage in most cases. Moreover, evaluation results from the SI show that the R2D link, not D2R, is the bottleneck in Ambient IoT coverage. Therefore, there is limited motivation to support four repetitions for the D2R link. Additionally, considering that repeated blocks are transmitted within a single PDRCH and the device’s limited energy budget, a repetition of 2 may offer a more reasonable and energy-efficient balance.

Proposal 4: For the number of block-level repetitions, {1, 2} repetitions are supported.

- Note: When the number of block-level repetition is 1, it indicates no repetition.

- Signalling details can be discussed under A.I. 9.4.4 or within this A.I.

- D2R chip duration and small frequency shift factor R

Down-selection of Tchip, Tb and R

During the previous meeting, the potential values of D2R chip duration Tchip, bit duration Tb, and the small frequency shift factor R were discussed, and the down-selection is expected to be finalized in this meeting.

Agreement (RAN1#120b) The potential values of D2R chip duration Tchip, bit duration Tb, and SFS factor R, are shown in the following table: - FFS further down-selections of Tchip, Tb, and R to be supported - Note: Detailed indication signaling of D2R scheduling information is discussed in AI 9.4.4. Tchip (μs). Tb (μs). 133.33; 66.67; 33.33; 16.67; 11.11; 8.33; 5.56; 4.17; 2.78; 2.08; 1.39; 1.04; 0.69; 0.52. 266.67; R=1; R=2; R=4; R=8; R=12; R=16; R=24; R=32; R=48; R=64; R=96; R=128; R=256. 133.33; R=1; R=2; R=4; R=6; R=8; R=12; R=16; R=24; R=32; R=48; R=64; R=96; R=128. 66.67; R=1; R=2; R=4; R=6; R=8; R=12; R=16; R=24; R=32; R=48; R=64. 33.33; R=1; R=2; R=4; R=6; R=8; R=12; R=16; R=24; R=32. 22.22; R=1; R=2; R=4; R=8; R=16. 16.67; R=1; R=2; R=4; R=6; R=8; R=12; R=16. 11.11; R=1; R=2; R=4; R=8. 8.33; R=1; R=2; R=4; R=6; R=8. 5.56; R=1; R=2; R=4. 4.17; R=1; R=2; R=4. 2.78; R=1; R=2. 2.08; R=1; R=2. 1.39; R=1. 1.04; R=1. Note: For further down-selection regarding the bit duration, - bit duration 266.67μs, 133.33μs and 66.67μs to be kept, which provides better multiplexing capability in the frequency domain. - Down-select to 1 between bit duration 1.39μs and 1.04μs, which achieves the competitive peak data rate larger than 640 kbps. - The remaining bit durations will be further discussed..

The current table shows two groups of candidate R values: one with R = 2n, and the other with R = 2n.

When comparing the two groups, the device implementation complexity should be taken into consideration. In the case of R=2n, device-side signal generation can be implemented using a simple binary divider. In contrast, R=2n cases require additional implementation, such as non-binary fractional dividers, which add to both circuit complexity and power consumption. This makes the R=2n option less suitable for Ambient IoT systems, where simplicity and ultra-low power operation are critical.

In addition to implementation concerns, spectral issues can also arise when using R=2n, particularly due to odd harmonics. For instance, when R=6, its harmonic components may overlap with those of R=4, especially at the 3rd or 5th harmonic. This can result in spectral collision between FDMA bins, reducing the effectiveness of receiver filtering and degrading channel separation. To avoid such issues, the number of usable R=2n values will be limited, making R=2n a cleaner design option.

Proposal 5: For D2R small frequency shift, only cases where R=2n are to be supported.

Assuming R = 2n, the table can be simplified as follows. First, for Tb values other than 266.67 us, 133.33 us, 66.67 us, 1.39 us and 1.04 us, a down-selection can be performed by retaining only one value among those that provide the same FDMA capability. If the minimum chip duration is assumed to be 0.52 us, the set of Tb values can be down-selected to {266.67, 133.33, 66.67, 33.33., 16.67, 8.33, 4.17, 2.08, 1.04} us. This allows the relationship between Tb and Tchip to be expressed as Tb = Tchip∙2n+1, which is beneficial for system-wide time structure alignment. This results in Table 1.

| Tchip(μs) Tb(μs) | 133.33 | 66.67 | 33.33 | 16.67 | 8.33 | 4.17 | 2.08 | 1.04 | 0.52 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 266.67 | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 | R=64 | R=128 | R=256 |
| 133.33 |  | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 | R=64 | R=128 |
| 66.67 |  |  | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 | R=64 |
| 33.33 |  |  |  | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 |
| 16.67 |  |  |  |  | R=1 | R=2 | R=4 | R=8 | R=16 |
| 8.33 |  |  |  |  |  | R=1 | R=2 | R=4 | R=8 |
| 4.17 |  |  |  |  |  |  | R=1 | R=2 | R=4 |
| 2.08 |  |  |  |  |  |  |  | R=1 | R=2 |
| 1.04 |  |  |  |  |  |  |  |  | R=1 |

Table 1. Potential values of Tb, Tchip and R

If the minimum chip duration is assumed to be 0.69 us, two sets of Tb values can be considered: (1) {266.67, 133.33, 66.67, 22.22, 11.11, 5.56, 2.78, 1.39} us, (2) {266.67. 133.33. 66.67. 33.33. 16.67. 8.33. 4.17. 1.39}us. These approaches lead to Table 2 and Table 3, respectively. In the first case, a larger number of Tchip values is required, which increases device-side implementation complexity. In the second case, the 1.39 us bit duration no longer maintains a power-of-two relationship with other Tb values, breaking system-wide timing alignment. In comparison, when the minimum Tchip is set to 0.52 us, all Tb values maintain the structure Tb = Tchip∙2n+1, which simplifies hardware design and ensures consistent timing alignment across the system.

Therefore, from our perspective, the down-selection should be based on Table 1, where the minimum chip duration is set to 0.52 us.

| Tchip(μs) Tb(μs) | 133.33 | 66.67 | 33.33 | 16.67 | 11.11 | 8.33 | 5.56 | 4.17 | 2.78 | 2.08 | 1.39 | 1.04 | 0.69 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 266.67 | R=1 | R=2 | R=4 | R=8 |  | R=16 |  | R=32 |  | R=64 |  | R=128 |  |
| 133.33 |  | R=1 | R=2 | R=4 |  | R=8 |  | R=16 |  | R=32 |  | R=64 |  |
| 66.67 |  |  | R=1 | R=2 |  | R=4 |  | R=8 |  | R=16 |  | R=32 |  |
| 22.22 |  |  |  |  | R=1 |  | R=2 |  | R=4 |  | R=8 |  | R=16 |
| 11.11 |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |  | R=8 |
| 5.56 |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |
| 2.78 |  |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |
| 1.39 |  |  |  |  |  |  |  |  |  |  |  |  | R=1 |

Table 2. Potential values of Tb, Tchip and R

| Tchip(μs) Tb(μs) | 133.33 | 66.67 | 33.33 | 16.67 | 8.33 | 4.17 | 2.08 | 1.04 | 0.69 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 266.67 | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 | R=64 | R=128 |  |
| 133.33 |  | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 | R=64 |  |
| 66.67 |  |  | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 |  |
| 33.33 |  |  |  | R=1 | R=2 | R=4 | R=8 | R=16 |  |
| 16.67 |  |  |  |  | R=1 | R=2 | R=4 | R=8 |  |
| 8.33 |  |  |  |  |  | R=1 | R=2 | R=4 |  |
| 4.17 |  |  |  |  |  |  | R=1 | R=2 |  |
| 1.39 |  |  |  |  |  |  |  |  | R=1 |

Table 3. Potential values of Tb, Tchip and R

Proposal 6: Down-selection of Tb, Tchip and R according to the table below.

| Tchip(μs) Tb(μs) | 133.33 | 66.67 | 33.33 | 16.67 | 8.33 | 4.17 | 2.08 | 1.04 | 0.52 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 266.67 | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 | R=64 | R=128 | R=256 |
| 133.33 |  | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 | R=64 | R=128 |
| 66.67 |  |  | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 | R=64 |
| 33.33 |  |  |  | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 |
| 16.67 |  |  |  |  | R=1 | R=2 | R=4 | R=8 | R=16 |
| 8.33 |  |  |  |  |  | R=1 | R=2 | R=4 | R=8 |
| 4.17 |  |  |  |  |  |  | R=1 | R=2 | R=4 |
| 2.08 |  |  |  |  |  |  |  | R=1 | R=2 |
| 1.04 |  |  |  |  |  |  |  |  | R=1 |

Signaling design

Agreement (RAN1#120) The following is supported for indicating the D2R chip duration: - The D2R chip duration is indicated in R2D control information from predefined a set of D2R chip duration values Agreement (RAN1#120b) At least for CBRA, for FDMA of multiple Msg1 transmissions in response to a R2D transmission triggering random access, support only the same data rate for the FDMed Msg1 transmissions. Agreement (RAN1#120b) For scheduling D2R transmission, any scheduling information related to resource allocation that needs to be signaled is indicated by higher-layer signaling via the corresponding PRDCH. MSG1 Resource Selection for FDMA (RAN2#129) - RAN2 assumes that device randomly selects among FDMA occasions as the baseline.

According to the agreements above, D2R scheduling information related to resource allocation is indicated by higher-layer signaling via the corresponding PRDCH. Tchip and R are parameters that determine the frequency-domain resource for D2R transmission. Therefore, these parameters shall also be provided via higher-layer signaling via the corresponding PRDCH. The device can then calculate the bit duration Tb based on the relation Tb = 2∙Tc∙R, if needed.

Proposal 7: The configuration of Tchip and R shall be provided via higher-layer signaling via the corresponding PRDCH.

For A-IoT msg1 transmission, it is agreed that a device randomly selects among FDMA occasions when responding to an R2D transmission that triggers random access. In addition, for all FDMed A-IoT Msg1 transmissions, the same data rate (i.e., the same Tb) should be used across devices. To ensure this consistency, the reader can transmit a Tb value via an R2D message that triggers random access. Based on the indicated Tb, the device can randomly select one Tchip value from the corresponding available set, thereby determining its FDMA frequency resource in a randomized manner.

Proposal 8: Different Tb values lead to different multiplexing capacities in the frequency domain. For each Tb, the set of available Tb values shall be predefined accordingly.

Proposal 9: The reader can transmit a Tb value via an R2D message that triggers random access. Based on the indicated Tb, the device can randomly select one Tchip value from the corresponding set.

Conclusion

In this contribution, we made the following observations and proposals:

Observation 1: Given that no FEC is already supported in D2R, there is limited justification for introducing puncturing to achieve higher code rates.

Observation 2: Since block-level repetition has already been agreed upon, additionally repeating coded bits is unnecessary and redundant.

Proposal 1: There is no consensus to support FEC code rate 1/2 in Rel-19.

Observation 3: It can be difficult for the reader to determine whether each device is close enough to operate without FEC, based on proximity determination solution 1.

Proposal 2: The use of FEC is configured via A-IoT paging. It is assumed that all devices operate in the same manner with respect to FEC usage within the same inventory or command round.

- Note: This issue can be addressed under A.I. 9.4.4 or within this A.I.

Observation 4: If L1 R2D control information is defined, it must have a separate CRC so that a device can first read R2D control information to acquire information necessary to read the following R2D data.

Proposal 3: If L1 R2D control information is supported, the L1 control information is attached with a separate CRC-6. If L1 R2D control information is not supported, no need for further discussion.

Proposal 4: For the number of block-level repetitions, {1, 2} repetitions are supported.

- Note: When the number of block-level repetition is 1, it indicates no repetition.

- Signalling details can be discussed under A.I. 9.4.4 or within this A.I.

Proposal 5: For D2R small frequency shift, only cases where R=2n are to be supported.

Proposal 6: Down-selection of Tb, Tchip and R according to the table below.

| Tchip(μs) Tb(μs) | 133.33 | 66.67 | 33.33 | 16.67 | 8.33 | 4.17 | 2.08 | 1.04 | 0.52 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 266.67 | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 | R=64 | R=128 | R=256 |
| 133.33 |  | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 | R=64 | R=128 |
| 66.67 |  |  | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 | R=64 |
| 33.33 |  |  |  | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 |
| 16.67 |  |  |  |  | R=1 | R=2 | R=4 | R=8 | R=16 |
| 8.33 |  |  |  |  |  | R=1 | R=2 | R=4 | R=8 |
| 4.17 |  |  |  |  |  |  | R=1 | R=2 | R=4 |
| 2.08 |  |  |  |  |  |  |  | R=1 | R=2 |
| 1.04 |  |  |  |  |  |  |  |  | R=1 |

Proposal 7: The configuration of Tchip and R shall be provided via higher-layer signaling via the corresponding PRDCH.

Proposal 8: Different Tb values lead to different multiplexing capacities in the frequency domain. For each Tb, the set of available Tb values shall be predefined accordingly.

Proposal 9: The reader can transmit a Tb value via an R2D message that triggers random access. Based on the indicated Tb, the device can randomly select one Tchip value from the corresponding set.

References:

- Sinha, Satyajit. "State of IoT 2024: Number of connected IoT devices growing 13% to 18.8 billion globally." IoT Analytics (2024).

- RP-243326 “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”.