3GPP TSG RAN WG1 #121    		                           R1-2503582

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:		9.11.1

Source:			Samsung

Title:			Discussion on downlink coverage enhancement for NR-NTN

Document for:		Discussion and decision

Introduction

In the previous RAN1#120bis [1], the following agreements were achieved for link level and system level enhancements.

Agreement When PDCCH CSS type-0 repetition is performed, for SIB1 link level enhancement, support PDSCH repetition of SIB1 transmitted within the same slot as the type0-CSS PDCCH repetition. - UE supporting SIB1 PDSCH coverage enhancement assumes that the PDCCH and associated PDSCH to be repeated in both slots where the corresponding PDCCHs are transmitted. - Each PDSCH SIB1 repetition is within the same slot of each PDCCH candidate for scheduling DCI - The two associated PDSCHs have the same RV FFS: Whether it is supported that type-0 PDCCH repetition is not performed while the PDSCH-SIB1 repetition is performed, and if so whether/how to handle the slot determination. Agreement For enabling/disabling SIB1 PDSCH repetition, RAN1 to consider the following options: - Option 1: Using reserved bit(s) in PBCH payload. - Option 2: Using scheduling PDCCH. - Option 3: The enabling/disabling of SIB1 PDSCH repetition is implicitly indicating by the enabling/disabling of Type-0 CSS PDCCH repetition. Agreement For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1: - Enabling/disabling using reserved bit(s) in PBCH payload - No UE behavior is defined for UE in connected mode specifically for the case where the network changes its signaling between enabling and disabling PDCCH repetition for Type0 PDCCH CSS. Agreement For Msg4 PDSCH repetition scheme, the Msg4 PDSCH is repeated in N consecutive slots: - The same resource allocation is assumed for all repetitions - The supported repetition factors are: 2 and 4 - The network configures a single value between 2 and 4 at a given time - The RV cycling is used for each repetition - If N=4: Starting RV; RV applicable to the nth repetition/transmission. n mod 4 = 0; n mod 4 = 1; n mod 4 = 2; n mod 4 = 3. 0; 0; 2; 3; 1. 2; 2; 3; 1; 0. 3; 3; 1; 0; 2. 1; 1; 0; 2; 3. Agreement For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1, support repeated PDCCH candidates in the two consecutive slots n0 and n0+1 associated with the same SSB index ( n0 as defined in section 13 of TS 38.213) at least for M=2. - Repeated PDCCH candidates share the same aggregation level (AL), coded bits and same candidate index - Note: if the network repeats the Type 0 PDCCH across two consecutive slots, a legacy UE might decode the PDCCH and associated PDSCH in one slot and skip PDCCH monitoring in the other slot. Note: further discuss the potential solution for M=1 and M=1/2. Working assumption For PDCCH CSS other than Type-0 CSS and other than Type-3 CSS for common search spaces other than SearchSpaceZero, intra-slot PDCCH repetition is supported. RAN1 to down select between option 1 and option 2: Option 1: Use same CORESET and two different SS (SS Set1 and SS Set2) - Linking two PDCCH candidates (adopt the same mechanism for SS linking specified in Release 17) - FFS: Blind decoding limit Option 2: Use same CORESET associated with one SS which is repeated by introducing symbol domain offset X - FFS: Blind decoding limit - FFS: details configuration and signalling Nokia expressed the concern on the above working assumption that this will take physical resources away from intra-slot scheduling for legacy PDSCH. Agreement For enabling/disabling Msg4 PDSCH repetition, RAN1 to down-select among the following options: - Option 1: UE specific PDSCH with Msg4 repetition activation indicated via PDCCH- DCI Format 1_0 - FFS: indication details. - FFS: whether/how network is informed by the UE that certain conditions are met to trigger Msg4 PDSCH repetition (e.g. RSRP detected at UE is less than x dB) - Option 2: The enabling/disabling of Msg4 PDSCH repetition is implicitly indicated by the enabling/disabling of SIB1 PDSCH repetition. - Option 3: The enabling/disabling is indicated by SIB1 configuration - FFS: whether/how network is informed by the UE that certain conditions are met to trigger Msg4 PDSCH repetition (e.g. RSRP detected at UE is less than x dB) FFS: Whether UE reports its capability

This contribution discusses remaining issues for the link level and the system level enhancements.

- Discussion

Type 0A/1/2 PDCCH CSS repetition

Working assumption For PDCCH CSS other than Type-0 CSS and other than Type-3 CSS for common search spaces other than SearchSpaceZero, intra-slot PDCCH repetition is supported. RAN1 to down select between option 1 and option 2: Option 1: Use same CORESET and two different SS (SS Set1 and SS Set2) - Linking two PDCCH candidates (adopt the same mechanism for SS linking specified in Release 17) - FFS: Blind decoding limit Option 2: Use same CORESET associated with one SS which is repeated by introducing symbol domain offset X - FFS: Blind decoding limit - FFS: details configuration and signalling

To enable Type 0A/1/2 PDCCH CSS repetition, SIB1 or SIB19 can provide relevant configurations. Therefore, the existing configurations for intra-slot level PDCCH repetition introduced in Rel-17 MIMO can be used as a baseline. For example, enabling the linkage between two search spaces through RRC for providing PDCCH repetition is feasible. Regarding the linked search spaces, specific restrictions apply, such as requiring identical search space linking IDs, identical search space set types, identical DCI formats, identical periodicities/offsets/durations, identical numbers of candidates per aggregation level, and identical numbers of monitoring occasions within a slot. In this sense, it is unnecessary to introduce another mechanism like option 2.

Proposal 1: Support option 1 to enable intra-slot PDCCH repetition for PDCCH CSS other than Type-0 CSS and other than Type-3 CSS

SIB1 PDSCH repetition

Agreement When PDCCH CSS type-0 repetition is performed, for SIB1 link level enhancement, support PDSCH repetition of SIB1 transmitted within the same slot as the type0-CSS PDCCH repetition. - UE supporting SIB1 PDSCH coverage enhancement assumes that the PDCCH and associated PDSCH to be repeated in both slots where the corresponding PDCCHs are transmitted. - Each PDSCH SIB1 repetition is within the same slot of each PDCCH candidate for scheduling DCI - The two associated PDSCHs have the same RV FFS: Whether it is supported that type-0 PDCCH repetition is not performed while the PDSCH-SIB1 repetition is performed, and if so whether/how to handle the slot determination.

One remaining issue is whether it is supported to perform PDSCH-SIB1 repetition without performing type-0 PDCCH repetition. Although the average worst-case coverage gaps are 3.9 dB for PDCCH and 6.5 dB for SIB1 PDSCH, as indicated by LLS results and observations from RAN1#117 [2], supporting this case would be overly optimized. This is because it would necessitate new signaling prior to SIB1 transmission and a new method for determining slot occasions for SIB1 PDSCH repetition without type-0 PDCCH repetition. Given that this meeting is the final opportunity to finalize features for downlink coverage enhancement, it is preferable to focus on resolving remaining details for key features rather than introducing new cases.

Proposal 2: Not support that type-0 PDCCH repetition is not performed while the PDSCH-SIB1 repetition is performed

Type0 PDCCH CSS repetition

Agreement For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1, support repeated PDCCH candidates in the two consecutive slots n0 and n0+1 associated with the same SSB index ( n0 as defined in section 13 of TS 38.213) at least for M=2. - Repeated PDCCH candidates share the same aggregation level (AL), coded bits and same candidate index - Note: if the network repeats the Type 0 PDCCH across two consecutive slots, a legacy UE might decode the PDCCH and associated PDSCH in one slot and skip PDCCH monitoring in the other slot. Note: further discuss the potential solution for M=1 and M=1/2.

In RAN1#120bis [1], it was agreed to support inter-slot level Type 0 PDCCH CSS repetition at least for M=2. Regarding the note, some companies argued that M=1 and M=1/2 are feasible when digital beamforming is considered without any further consideration, while other companies think that it is not. Actually, three options have been discussed to support inter-slot Type 0 PDCCH repetition for M=1 and M=1/2.

- Option 1 is to rely on gNB implementation, such as digital beamforming or other implicit operations.

- Option 2 is to support repeated PDCCH candidates in two non-consecutive slots, n0 and n0 + X.

- Option 3 is to support repeated PDCCH candidates in two consecutive slots associated with different SSB indexes.

If there is no convergence among these three options in this RAN1 meeting, it is preferable not to support M=1 and M=1/2 in Rel-19. Supporting only M=2 does not have any critical issues as it can improve coverage for Type 0 PDCCH transmission.

Proposal 3: Support the unified solution regardless of M values. If there is no consensus, M=2 is only supported in Rel-19

In last RAN1#120bis [1], it is supported that reserved bit(s) in PBCH payload is used to enable or disable PDCCH repetition for Type0 PDCCH CSS. Regarding the number of reserved bit(s), only one 1 reserved bit should be enough to indicate. Therefore, the above agreement should be updated as follows.

- Enabling/disabling using a reserved bit(s) in PBCH payload

Proposal 4: Support to update the agreement as follows

Agreement For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1: - Enabling/disabling using a reserved bit(s) in PBCH payload - No UE behavior is defined for UE in connected mode specifically for the case where the network changes its signaling between enabling and disabling PDCCH repetition for Type0 PDCCH CSS.

Msg4 PDSCH repetition

Agreement For enabling/disabling Msg4 PDSCH repetition, RAN1 to down-select among the following options: - Option 1: UE specific PDSCH with Msg4 repetition activation indicated via PDCCH- DCI Format 1_0 - FFS: indication details. - FFS: whether/how network is informed by the UE that certain conditions are met to trigger Msg4 PDSCH repetition (e.g. RSRP detected at UE is less than x dB) - Option 2: The enabling/disabling of Msg4 PDSCH repetition is implicitly indicated by the enabling/disabling of SIB1 PDSCH repetition. - Option 3: The enabling/disabling is indicated by SIB1 configuration - FFS: whether/how network is informed by the UE that certain conditions are met to trigger Msg4 PDSCH repetition (e.g. RSRP detected at UE is less than x dB) FFS: Whether UE reports its capability Agreement For Msg4 PDSCH repetition scheme, the Msg4 PDSCH is repeated in N consecutive slots: - The same resource allocation is assumed for all repetitions - The supported repetition factors are: 2 and 4 - The network configures a single value between 2 and 4 at a given time - The RV cycling is used for each repetition - If N=4: Starting RV; RV applicable to the nth repetition/transmission. n mod 4 = 0; n mod 4 = 1; n mod 4 = 2; n mod 4 = 3. 0; 0; 2; 3; 1. 2; 2; 3; 1; 0. 3; 3; 1; 0; 2. 1; 1; 0; 2; 3.

One of the remaining issues for Msg4 PDSCH repetition is determining how to enable and disable it. There are three proposed options.

For option 1, DCI format 1_0 dynamically indicates whether Msg4 PDSCH repetition is enabled or disabled. This approach offers flexibility, as the repetition number (either 1 or the configured value) can be specified via the DCI format. However, further discussion is needed to identify which DCI field should be used for this indication. If a consensus is reached on the DCI field, Option 1 could be considered viable. Otherwise, it may not be feasible if the gNB intends to configure only a single value for Msg4 PDSCH repetition. For option 2, enabling or disabling Msg4 PDSCH repetition is implicitly tied to SIB1 PDSCH repetition. For example, if SIB1 PDSCH repetition is enabled, the UE assumes that Msg4 PDSCH repetition is also enabled. However, the benefit of this approach is unclear, as the actual repetition number for Msg4 PDSCH would still be provided in SIB1. For option 3, SIB1 explicitly indicates whether Msg4 PDSCH repetition is enabled or disabled. This is a straightforward solution, especially since the gNB typically configures a single value (between 2 and 4) for Msg4 PDSCH repetition, which is likely to be included in SIB1. Therefore, Option 3 should serve as the baseline, while Option 1 can be considered optional if additional flexibility is desired.

To implement option 1, leveraging the TDRA table is a practical solution. Under current specifications, for DCI formats containing TC-RNTI, the TDRA table defaults to either Default A or Pdsch-TimeDomainAllocationList, as defined in PDSCH-ConfigCommon. By adding a column for the repetition factor (similar to PUSCH repetition) to pdsch-TimeDomainAllocationList, the UE can determine the repetition factor alongside other parameters such as SLIV, K0, PDSCH mapping type, and dmrs-TypeA-Position. Given the importance of coverage in NTN scenarios, most PDSCH scheduling will likely utilize all available symbols per slot. This eliminates the need for varying starting symbols or durations for Msg4 PDSCH transmissions. Beyond the TDRA field, the MCS table HARQ process number field could also serve as an indicator for Msg4 PDSCH repetition. Note that 1 bit is enough to indicate whether repetition is enabled or not. Thus, other DCI field like MCS field can be considered as it is unlikely to provide high MCS level for transmitting Msg4 PDSCH in NTN.

For UE capability reporting, Msg3 PUSCH can be as a container for reporting support for Msg4 PDSCH repetition, similar to how PUCCH repetition is handled for Msg4 PDSCH.

Proposal 5: Support options 1 and 3 for Msg4 PDSCH repetition with the followings

- Enabling PDSCH repetition using TDRA table or other DCI field

- Msg3 PUSCH is used for UE capability reporting

Conclusion

This contribution discusses remaining issues for downlink coverage enhancements. Followings are proposals in this contribution.

Proposal 1: Support option 1 to enable intra-slot PDCCH repetition for PDCCH CSS other than Type-0 CSS and other than Type-3 CSS

Proposal 2: Not support that type-0 PDCCH repetition is not performed while the PDSCH-SIB1 repetition is performed

Proposal 3: Support the unified solution regardless of M values. If there is no consensus, M=2 is only supported in Rel-19

Proposal 4: Support to update the agreement as follows

Agreement For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1: - Enabling/disabling using a reserved bit(s) in PBCH payload - No UE behavior is defined for UE in connected mode specifically for the case where the network changes its signaling between enabling and disabling PDCCH repetition for Type0 PDCCH CSS.

Proposal 5: Support options 1 and 3 for Msg4 PDSCH repetition with the followings

- Enabling PDSCH repetition using TDRA table or other DCI field

- Msg3 PUSCH is used for UE capability reporting

Reference

[1] RAN1#120bis Chair Note

[2] RAN1#117 Chair Note