3GPP TSG RAN WG1 #121    		                           R1-2503585

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:		9.11.4

Source:			Samsung

Title:			Discussion on uplink capacity/throughput enhancement for IoT-NTN

Document for:		Discussion and decision

- Introduction

In RAN1#120bis [1], it was discussed on potential enhancements to enable multiplexing of multiple UEs in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH. Following agreements/conclusions were achieved. This contribution discusses on remaining details.

Agreement For the 3.75kHz SCS symbol-based OCC scheme, the granularity of spreading for data is one symbol. Agreement Dynamic activation / deactivation of OCC is supported by DCI. - FFS: details of signalling by DCI Conclusion RAN1 assumes no specification change to support pairing UEs with different modulation orders. Agreement For 3.75kHz SCS OCC for NPUSCH format 1, RAN1 down selects between the following mappings between DMRS sequence samples and active TDM DMRS slots: - Option 1: Sequential mapping of samples of the original DMRS sequence to active DMRS slots - Option 2: Dropping of samples of the original DMRS sequence in blanked slots Agreement For NPUSCH Format 1 single-tone 15kHz SCS, for CDM DMRS with legacy pattern: - DMRS symbols are spread before the OCC is applied - Option 1_1: according to the formula: ru,OCCMn+m=runqm, 0≤n<X/M, m=0, …, M-1 Where: M is the OCC length, q is the assigned OCC codeword for the UE, run is the reference signal sequence defined in TS36.211 section 10.1.4.1.1 and X is the total number of slots in the NPUSCH transmission after OCC is applied Agreement The OCC sequence index is signalled using DCI format N0. Agreement The RRC parameters for IoT-NTN UL capacity enhancements are: RAN2 Parent IE; Parameter name in the spec; New or existing?; Description; Value range; UE-specific or Cell-specific. NPUSCH-ConfigDedicated-NB; npusch-OCC-Enabled; new; The parameter is used to enable OCC for NPUSCH format 1 single tone.; ENUMERATED {‘true’}; UE-specific. Agreement For indicating OCC sequence index and activation / deactivation, the following fields may be repurposed or constrained to be not present in the DCI: - Modulation and coding scheme - Repetition number - Redundancy version - Number of scheduled TB for Unicast - Subcarrier indication - Resource reservation

- Discussion

Maximum OCC length for IDLE mode

Agreement For 3.75kHz SCS OCC for NPUSCH format 1, RAN1 down selects between the following mappings between DMRS sequence samples and active TDM DMRS slots: - Option 1: Sequential mapping of samples of the original DMRS sequence to active DMRS slots - Option 2: Dropping of samples of the original DMRS sequence in blanked slots

For option 1, samples of the original DMRS sequence are sequentially mapped to active DMRS slots. For example, if there are two active slots (e.g., slots 0 and 1), the first sample of the DMRS sequence is placed in slot 0, and the next sample in slot 1. This continues for all active slots. Thus, proponents of option 1 have argued that it can maintains the original order of the DMRS sequence, and spreads the DMRS symbols across active slots, which may preserve orthogonality between UEs but could introduce phase continuity challenges if slots are staggered. So, it would require careful alignment of UE transmissions to avoid interference. While for option 2, samples of the original DMRS sequence are dropped (punctured) in blanked slots. Only the remaining samples in active slots are used. It would shorten the DMRS sequence by omitting samples in inactive slots, and reduces potential interference with neighboring cells since fewer DMRS symbols are transmitted. However, it simplifies implementation as it avoids reordering samples but may affect channel estimation accuracy if critical samples are dropped.

For orthogonality, option 1 preserves sequence order but may risk inter-cell interference if UEs have staggered transmissions, while option 2 minimizes this risk by truncating sequences. For specification impact, option 2 has lower specification impact as it reuses legacy DMRS patterns without requiring new definitions. For implementation, option 2 would provide simplicity and reduced interference, while Option 1 can maintain sequence integrity. The choice between these options hinges on balancing orthogonality, implementation complexity, and inter-cell interference management. Since this meeting is the final meeting to finish WI, it is preferable to have simple solution with minimum specification change. In that sense, option 2 is slightly preferred.

Proposal 1: Support option 2 for mapping between DMRS sequence samples and active TDM DMRS slots in case of 3.75kHz SCS OCC for NPUSCH format 1.

Signaling aspect for OCC

Agreement Dynamic activation / deactivation of OCC is supported by DCI. - FFS: details of signalling by DCI Agreement The OCC sequence index is signalled using DCI format N0. Agreement For indicating OCC sequence index and activation / deactivation, the following fields may be repurposed or constrained to be not present in the DCI: - Modulation and coding scheme - Repetition number - Redundancy version - Number of scheduled TB for Unicast - Subcarrier indication - Resource reservation

Regarding how to indicate OCC sequence index, 1 bit is enough to indicate as the maximum OCC length is two. Without reducing scheduling flexibility, it is preferable to consider new DCI field by replacing other configured DCI fields such as number of scheduled TB for unicast or resource reservation as those DCI fields are optional. It does not mean that OCC sequence index is repurposed by those DCI field. But, it can be configured optionally like other existing DCI field.

Proposal 2: Consider new DCI field for indicating OCC sequence index in DCI format N0.

Conclusion

This contribution discussed remaining details for uplink capacity/throughput evaluation. Followings are proposals in this contribution.

Proposal 1: Support option 2 for mapping between DMRS sequence samples and active TDM DMRS slots in case of 3.75kHz SCS OCC for NPUSCH format 1.

Proposal 2: Consider new DCI field for indicating OCC sequence index in DCI format N0.

References

[1] RAN1#120bis Chair Note