3GPP TSG RAN WG1 #121						R1-2503595

St Julian's, Malta, May 19th – 23th, 2025

Agenda item:	9.15.5

Source:	Samsung

Title:	Discussion on UE features for LP-WUS/WUR

Document for:	Discussion and Decision

- Introduction

According to the WID on Low-power wake-up signal and receiver for NR [1], the objectives related to LP-WUS/LP-SS design, LP-WUS operation for RRC idle/inactive state, and LP-WUS operation for RRC CONNECTED state are discussed during Rel-19 LP-WUS/WUR work item. The latest UE feature agreements are captured in the appendix.

In this contribution, we discuss the UE features for above objectives.

- LP-WUS operation in RRC IDLE/INACTIVE mode

FG 62-1 and 62-1a

We suggest the merging of the possible “M” values for OOK-4 operation into the first component as a mandatory feature for all UEs supporting LP-WUS. This is because, it is difficult and inefficient for the gNB to group the UEs based on their supported M values especially for OOK-based LP-WUS signalling. For the supported values of M corresponding to each SCS, M=4 with 30kHz is still a working assumption and has not been confirmed yet, so we keep it in the brackets.

For components 2 in FG 62-1, we believe the existing proposals by FL are sufficient, thus we suggest removing highlights and brackets. However, for FG 62-1a, which channel/signal should be mandatory to perform RRM measurement should be further discussed considering the related issues (e.g., whether LP-SS transmission in RRC IDLE/INACTIVE mode can be configured by the gNB, the case when the overlaid OFDM sequence is not configured by the gNB for LP-SS with M=1). Thus, we keep the brackets for component

For components 3, and 4 in FG 62-1/62-1a, these components are related to RAN2/RAN4 requirement for MR RRM measurement relaxation and RRM measurement offloading to LR. In RAN1, how to measure the quantities based on LP-SS have been mainly discussed rather than designing the requirement or conditions for RRM measurement, we suggest to leave them for RAN2/RAN4 UE feature discussion.

Since the potential candidate values for wake-up delay have not been agreed yet, we suggest removing the brackets around component 5 in FG 62-1/62-1a, but keeping the potential candidate values still as an FFS.

Proposal 1: Update FGs 62-1 and 62-1a as follows:

- RRM measurement relaxation/offloading related components can be left to RAN2/RAN4 UE feature discussion

| 62. NR_LPWUS | 62-1 | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OOK signal with M=1,2, [and 4] [2. The support of LP-SS based RRM measurement] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [53. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal is not supported | Per Band | n/a | n/a | n/a | Component 53 candidate values: FFS | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 62. NR_LPWUS | 62-1a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OFDM overlaid sequence with M=1,2, [and 4] [2. FFS: The support of LP-SS based RRM measurement and/or [X3. FFS: The support of SSB-based RRM measurement] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [54. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence is not supported | Per band | n/a | n/a | n/a | Component 54 candidate values: FFS | Optional with capability signalling |

- LP-WUS operation in RRC CONNECTED mode

FG 62-2 and 62-2a

In this section, we focus attention on the RRC CONNECTED mode. Similar to FG 62-1 and FG 62-1a, we suggest that these feature groups should be supported per band and thus suggest the removal of the brackets. We also suggest the merging of the possible “M” values for OOK-4 operation into the first component as a mandatory feature for all UEs supporting LP-WUS. This is because, it is difficult and inefficient for the gNB to group the UEs based on their supported M values especially for OOK-based LP-WUS signalling. For the supported values of M corresponding to each SCS, M=4 with 30kHz is still a working assumption and has not been confirmed yet, so we keep it in the brackets.

Since the potential candidate values for minimum time gap between LP-WUS reception and UE to start PDCCH monitoring have not been agreed yet, we suggest removing the highlight for component 3 in FG 62-2 and FG 62-2a, but keeping the potential candidate values still as FFS.

We don’t see the need for adding new components based on SCS since these can follow those of the MR and thus suggest the removal of the corresponding FFS.

Proposal 2: Update FGs 62-2 and 62-2a as follows:

| 62. NR_LPWUS | 62-2 | LP-WUS operation in CONNECTED mode based on OOK signal | 1. LP-WUS operation in CONNECTED mode based on OOK signal with M=1,2, [and 4] 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. Support of LP-WUS frequency resource within active DL BWP FFS: M values FFS: SCS FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in CONNECTED mode based on OOK signal is not supported | [Per band] | n/a | n/a | n/a | Component 2 candidate values: FFS Component 3 candidate values: FFS | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 62. NR_LPWUS | 62-2a | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence | 1. LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) with M=1,2, [and 4] 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. Support of LP-WUS frequency resource within active DL BWP FFS: M values FFS: SCS FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence is not supported | [Per band] | n/a | n/a | n/a | Component 2 candidate values: FFS Component 3 candidate values: FFS | Optional with capability signalling |

FG 62-3

In the last RAN1 meeting #120-bis, a new UE capability was added for LP-WUS frequency resource selection outside active DL BWP, i.e. FG 62-3. We suggest to support this FG per band.

Proposal 3: Update FG 62-3 for RRC CONNECTED UEs as follows:

| 62. NR_LPWUS | 62-3 | LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode | 1. Support of LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode | FG 62-2 or 62-2a | YES | n/a | LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode is not supported | [Per band] | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Conclusion

The proposals made in this contribution are summarized below:

Proposal 1: Update FGs 62-1 and 62-1a as follows:

- RRM measurement relaxation/offloading related components can be left to RAN2/RAN4 UE feature discussion

| 62. NR_LPWUS | 62-1 | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OOK signal with M=1,2, [and 4] [2. The support of LP-SS based RRM measurement] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [53. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal is not supported | Per Band | n/a | n/a | n/a | Component 53 candidate values: FFS | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 62. NR_LPWUS | 62-1a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OFDM overlaid sequence with M=1,2, [and 4] [2. FFS: The support of LP-SS based RRM measurement and/or [X3. FFS: The support of SSB-based RRM measurement] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [54. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence is not supported | Per band | n/a | n/a | n/a | Component 54 candidate values: FFS | Optional with capability signalling |

Proposal 2: Update FGs 62-2 and 62-2a as follows:

| 62. NR_LPWUS | 62-2 | LP-WUS operation in CONNECTED mode based on OOK signal | 1. LP-WUS operation in CONNECTED mode based on OOK signal with M=1,2, [and 4] 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. Support of LP-WUS frequency resource within active DL BWP FFS: M values FFS: SCS FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in CONNECTED mode based on OOK signal is not supported | [Per band] | n/a | n/a | n/a | Component 2 candidate values: FFS Component 3 candidate values: FFS | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 62. NR_LPWUS | 62-2a | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence | 1. LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) with M=1,2, [and 4] 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. Support of LP-WUS frequency resource within active DL BWP FFS: M values FFS: SCS FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence is not supported | [Per band] | n/a | n/a | n/a | Component 2 candidate values: FFS Component 3 candidate values: FFS | Optional with capability signalling |

Proposal 3: Update FG 62-3 for RRC CONNECTED UEs as follows:

| 62. NR_LPWUS | 62-3 | LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode | 1. Support of LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode | FG 62-2 or 62-2a | YES | n/a | LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode is not supported | [Per band] | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Reference

[1] RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR).

- Appendix

The agreements from the previous RAN1 #120bis meeting are captured below:

Agreement:

- Define new FG 62-3 for the support of LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode

- Add a component in FG 62-2/2a for the support of LP-WUS frequency resource within active DL BWP

| 62-3 | LP-WUS frequency resource outside active DL BWP in addition to the one within active DL BWP for LP-WUS operation in CONNECTED mode | 1. Support of LP-WUS frequency resource outside active DL BWP in addition to the one within active DL BWP for LP-WUS operation in CONNECTED mode | FG 62-2 or 62-2a | YES | n/a | LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode is not supported | [Per band] | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement:

Adopt “Per band” for FG 62-1 and 62-1a:

Agreement:

Update FG 62-1 and 62-1a as follows:

| 62-1 | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OOK [2. The support of LP-SS based RRM measurement and/or SSB-based measurement, corresponding to component 1] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [5. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal is not supported | Per Band | n/a | n/a | n/a | Component 1 candidate values: {OOK-based, OFDM-based, both} Component 5 candidate values: FFS [For UE supporting both X-1 and X-2, UE is not allowed to report different candidate values for component 1] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 62-1a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OFDM overlaid sequence [2. FFS: The support of LP-SS based RRM measurement and/or [X. FFS: The support of SSB-based RRM measurement] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [5. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: {OOK-based, OFDM-based, both} Component 5 candidate values: FFS [For UE supporting both X-1 and X-2, UE is not allowed to report different candidate values for component 1] | Optional with capability signalling |