3GPP TSG-RAN WG1 Meeting #121	 											R1-2503603

St Julians, Malta, May 19th – 23rd, 2025

Agenda item:		9.15.13

Source:				Samsung

Title:					UE features for Rel-19 LTE-based 5G Broadcast

Document for:	    Discussion & Decision

# 1 Introduction

In this paper, we discuss the details on UE features for Rel-19 LTE-based 5G Broadcast.

# 2 Discussion

## 2.1. UE capability for Rel-19 LTE-based 5G Broadcast

UE capability information for supporting UE features for Rel-19 LTE-based 5G Broadcast should be optional with capability signalling, allowing UE implementations with the required flexibility and enabling network to most appropriately provision the broadcast transmission. As noted in RAN1#120 meeting, some UE categories, depending on factors such as value of N, bandwidth, MCS, may not be able to receive an interleaved MTCH with M=16 or M=32. It is, therefore, required that maximum value of M and maximum value for N supported by the UE for time interleaving should also be included in the UE capability information signalling. It can be noted that maximum supported TBS for broadcast is related to that of maximum supported TBS for unicast (i.e., it is a scaled value) and it is already indicated through the ue-Category. While most of the other UE capability parameters like number of RBs, BW, SCS are already addressed by legacy releases of specifications.

Proposal 1: UE capability for Rel-19 LTE-based 5G Broadcast feature support with parameters for maximum supported values of M and N is optional with capability signalling.

Table 1: UE features for Rel-19 LTE-based 5G Broadcast

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3. LTE_terr_bcast_Ph2 | 3-1 | Time-interleaving | 1. Support of PMCH transmission pattern, excluding MCCH and MSI, with time interleaving 2. Support of TBS determination for the scaled TB 3. Support of determining the starting point for reading from the circular buffer (k0) for each subframe 4. Maximum supported value of M 5. Maximum supported value of N | Support of fembmsDedicatedCell | Yes | UE is not able to support time-interleaving for LTE-based 5G broadcast | Per band | No | Note: One TB is mapped to N non-consecutive subframes. Two transmissions of the same TB are separated by (M-1) subframes. M:{4, 8,16,32} N:{2,4,8,16} | Optional with capability signalling |

## 2.2. MBMS Interest Indication for Rel-19 LTE-based 5G Broadcast

Apart from UE capability information message that provides the static capabilities of the UE, MBMS Interest Indication message provides the UE’s interest in broadcast servcie and dynamic baseband processing resource capability information. The basedband processing resources are typically shared between the unicast operation on LTE (NW-A) and LTE-based 5G Broadcast reception (NW-B) and. As in the legacy, dynamic baseband processing resource capability information makes NW-A aware, and accordingly unicast configuration is appropriately configured for the UE [Refer to 36.213 clause 11.1].

We understand there is a need for a new Rel-19 IE in the MBMS interest indication to convey UE’s interest and baseband resource capability corresponding to Rel-19 LTE-based 5G Broadcast.

- Rel-19 5G Broadcast is different from legacy 5G Broadcast, network need to be informed about UE’s interest in Rel-19 5G Broadcast for proper provisioning of broadcast services w/ required configurations

- Sub-carrier spacing utilized for Rel-19 5G Broadcast has different set of values as compared to legacy 5G Broadcast

- Bandwidth parameter for Rel-19 5G Broadcast also need to accommodate a few new values for 30/35/40 resource blocks

- Currently utilized value of M for receiving Rel-19 5G Broadcast affects the baseband processing resources available for unicast drastically as compared to legacy (e.g. large utilized value of M implies less HARQ buffers availability for unicast reception)

Below we depict a new Rel-19 IE and pertinent parameters that required for MBMS Interest Indication message corresponding to Rel-19 LTE-based 5G Broadcast reception.

MBMSInterestIndication message

-- ASN1START

<Omitted part>

MBMS-ROM-Info-r15 ::= SEQUENCE {

mbms-ROM-Freq-r15						ARFCN-ValueEUTRA-r9,

mbms-ROM-SubcarrierSpacing-r15		ENUMERATED {kHz15, kHz7dot5, kHz1dot25},

mbms-Bandwidth-r15					ENUMERATED {n6, n15, n25, n50, n75, n100}

}

MBMS-ROM-Info-r16 ::= SEQUENCE {

mbms-ROM-Freq-r16					ARFCN-ValueEUTRA-r9,

mbms-ROM-SubcarrierSpacing-r16		ENUMERATED {kHz2dot5, kHz0dot37},

mbms-Bandwidth-r16					ENUMERATED {n6, n15, n25, n50, n75, n100}

}

MBMS-ROM-Info-r19 ::= SEQUENCE {

mbms-ROM-Freq-r19					ARFCN-ValueEUTRA-r9,

mbms-ROM-SubcarrierSpacing-r19		ENUMERATED {kHz2dot5, kHz1dot25},

mbms-Bandwidth-r19					ENUMERATED {n6, n15, n25, n30, n35, n40, n50, n75,n100}

pmch-TimeInterleavingUsed-M-r19     ENUMERATED {n4, n8, n16, n32…}

}

-- ASN1STOP

Proposal 2: MBMS Interest Indication message includes a Rel-19 Information Element to convey UE’s interest and baseband resource capability corresponding to Rel-19 LTE-based 5G Broadcast. Rel-19 IE in MBMS Interest Indication message carries following parameters utilized for receiving Rel-19 LTE-based 5G Broadcast service(s):

- Frequency

- Sub-carrier spacing  = {2.5 kHz, 1.25 kHz}

- Bandwidth = {n6, n15, n25, n30, n35, n40, n50, n75,n100}

- Currently utilized value of M (i.e. Pmch-TimeInterleavingUsed-M) = {n4, n8, n16, n32…}

# 3 Conclusion

Based on the discussion above, request RAN1 to consider the following proposals:

Proposal 1: UE capability for Rel-19 LTE-based 5G Broadcast feature support with parameters for maximum supported values of M and N is optional with capability signalling.

Table 1: UE features for Rel-19 LTE-based 5G Broadcast

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3. LTE_terr_bcast_Ph2 | 3-1 | Time-interleaving | 1. Support of PMCH transmission pattern, excluding MCCH and MSI, with time interleaving 2. Support of TBS determination for the scaled TB 3. Support of determining the starting point for reading from the circular buffer (k0) for each subframe 4. Maximum supported value of M 5. Maximum supported value of N | Support of fembmsDedicatedCell | Yes | UE is not able to support time-interleaving for LTE-based 5G broadcast | Per band | No | Note: One TB is mapped to N non-consecutive subframes. Two transmissions of the same TB are separated by (M-1) subframes. M:{4, 8,16,32} N:{2,4,8,16} | Optional with capability signalling |

Proposal 2: MBMS Interest Indication message includes a Rel-19 Information Element to convey UE’s interest and baseband resource capability corresponding to Rel-19 LTE-based 5G Broadcast. Rel-19 IE in MBMS Interest Indication message carries following parameters utilized for receiving Rel-19 LTE-based 5G Broadcast service(s):

- Frequency

- Sub-carrier spacing  = {2.5 kHz, 1.25 kHz}

- Bandwidth = {n6, n15, n25, n30, n35, n40, n50, n75,n100}

- Currently utilized value of M (i.e. Pmch-TimeInterleavingUsed-M) = {n4, n8, n16, n32…}