3GPP TSG RAN WG1#121	R1-2503605

St. Julian’s, Malta, May 19th – 23rd, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.901 | CR | DRAFT | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for Rel-19 7-24GHz Channel model |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation, ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_7_24GHz_CHmod |  |  |  |  |  | Date: |  |  | 2025-05-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To capture the findings of the SI: Study on channel modelling enhancements for 7-24GHz for NR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Channel modelling is updated considering new measurements and information. Added new UE antenna model, support of near field channel propagation, and support of spatial non-stationarity. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Fail to complete the SI. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 2, 4, 6.1, 6.2, 6.3, 6.4, 7.2, 7.3, 7.4, 7.4.1, 7.4.2, 7.4.3, 7.4.3.1, 7.5, 7.6, 7.6.3.1, 7.6.9, 7.7.5.1, 7.8.1, 7.8.2, 7.8.3, A, A.1, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 1	Scope

The present document captures the findings of the study item, "Study on channel model for frequency spectrum above 6 GHz" [2] and from further findings of the study item, "Study on New Radio Access Technology [22]",the study item "Study on Channel Modeling for Indoor Industrial Scenarios [23]", and "New SID: Study on channel modelling enhancements for 7-24GHz for NR [24]". The channel models in the present document address the frequency range 0.5-100 GHz. The purpose of this TR is to help TSG RAN WG1 to properly model and evaluate the performance of physical layer techniques using the appropriate channel model(s). Therefore, the TR will be kept up-to-date via CRs in the future.

This document relates to the 3GPP evaluation methodology and covers the modelling of the physical layer of both Mobile Equipment and Access Network of 3GPP systems.

This document is intended to capture the channel model(s) for frequencies from 0.5GHz up to 100GHz.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TD RP-151606: "Study on channel model for frequency spectrum above 6 GHz".

[3]	3GPP TR 36.873 (V12.2.0): "Study on 3D channel model for LTE".

[4]	3GPP RP-151847: "Report of RAN email discussion about >6GHz channel modelling", Samsung.

[5]	3GPP TD R1-163408: "Additional Considerations on Building Penetration Loss Modelling for 5G System Performance Evaluation", Straight Path Communications.

[6]	ICT-317669-METIS/D1.4: "METIS channel model, METIS 2020, Feb, 2015".

[7]	Glassner, A S: "An introduction to ray tracing. Elsevier, 1989".

[8]	McKown, J. W., Hamilton, R. L.: "Ray tracing as a design tool for radio networks, Network, IEEE, 1991(6): 27-30".

[9]	Kurner, T., Cichon, D. J., Wiesbeck, W.: "Concepts and results for 3D digital terrain-based wave propagation models: An overview", IEEE J.Select. Areas Commun., vol. 11, pp. 1002–1012, 1993.

[10]	Born, M., Wolf, E.: "Principles of optics: electromagnetic theory of propagation, interference and diffraction of light", CUP Archive, 2000.

[11]	Friis, H.: "A note on a simple transmission formula", proc. IRE, vol. 34, no. 5, pp. 254–256, 1946.

[12]	Kouyoumjian, R.G., Pathak, P.H.: "A uniform geometrical theory of diffraction for an edge in a perfectly conducting surface" Proc. IEEE, vol. 62, pp. 1448–1461, Nov. 1974.

[13]	Pathak, P.H., Burnside, W., Marhefka, R.: "A Uniform GTD Analysis of the Diffraction of Electromagnetic Waves by a Smooth Convex Surface", IEEE Transactions on Antennas and Propagation, vol. 28, no. 5, pp. 631–642, 1980.

[14]	IST-WINNER II Deliverable 1.1.2 v.1.2, "WINNER II Channel Models", IST-WINNER2, Tech. Rep., 2007 (http://www.ist-winner.org/deliverables.html).

[15]	3GPP TR36.101: "User Equipment (UE) radio transmission and reception".

[16]	3GPP TR36.104: "Base Station (BS) radio transmission and reception".

[17]	Asplund, H., Medbo, J., Göransson, B., Karlsson, J., Sköld, J.: "A simplified approach to applying the 3GPP spatial channel model", in Proc. of PIMRC 2006.

[18]	ITU-R Rec. P.1816: "The prediction of the time and the spatial profile for broadband land mobile services using UHF and SHF bands".

[19]	ITU-R Rec. P.2040-1: "Effects of building materials and structures on radiowave propagation above about 100 MHz", International Telecommunication Union Radiocommunication Sector ITU-R, 07/2015.

[20]	ITU-R Rec. P.527-3: "Electrical characteristics of the surface of the earth", International Telecommunication Union Radiocommunication Sector ITU-R, 03/1992.

[21]	Jordan, E.C., Balmain, K.G.: "Electromagnetic Waves and Radiating Systems", Prentice-Hall Inc., 1968.

[22]	3GPP TD RP-162469: "Study on New Radio (NR) Access Technology".

[23]	3GPP TD RP-182138: "SID on Channel Modeling for Indoor Industrial Scenarios".

[24]	3GPP TD RP-234018: "New SID: Study on channel modelling enhancements for 7-24GHz for NR"

[25]	3GPP TD R1-2503060: "Data source descriptions for 7 – 24 GHz SI".

< Unchanged text omitted >

# 4	Introduction

At TSG RAN #69 meeting the Study Item Description on "Study on channel model for frequency spectrum above 6 GHz" was approved [2]. This study item covers the identification of the status/expectation of existing information on high frequencies (e.g. spectrum allocation, scenarios of interest, measurements, etc), and the channel model(s) for frequencies up to 100 GHz. This technical report documents the channel model(s). The new channel model has to a large degree been aligned with earlier channel models for <6 GHz such as the 3D SCM model (TR 36.873) or IMT-Advanced (ITU-R M.2135). The new model supports comparisons across frequency bands over the range 0.5-100 GHz. The modelling methods defined in this technical report are generally applicable over the range 0.5-100 GHz, unless explicitly mentioned otherwise in this technical report for specific modelling method, involved parameters and/or scenario.

Subsequently, at the TSG RAN #81 meeting the Study Item Description "Study on Channel Modeling for Indoor Industrial Scenarios" was approved [23]. The findings from this study item is also captured in the present technical report. The Industrial channel model was developed by considering new measurements and information in the literature. An overview list of all such contributions and sources is available in tdoc R1-1909706.

Subsequently, at the TSG RAN #102 meeting the Study Item Description "Study on channel modelling enhancements for 7-24GHz for NR" was approved [24]. The findings from this study are also captured in the present technical report. Channel modeling was further updated by considering new measurements and information in the literature. An overview of the information sources is available in tdoc R1-2403060 [25].

The channel model is applicable for link and system level simulations in the following conditions:

-	For system level simulations, supported scenarios are urban microcell street canyon, urban macrocell, indoor office, rural macrocell, indoor factory, and suburban macrocell.

-	Bandwidth is supported up to 10% of the center frequency but no larger than 2GHz.

-	Mobility of either one end of the link or both ends of the link is supported

-	For the stochastic model, spatial consistency is supported by correlation of LSPs and SSPs as well as LOS/NLOS state.

-	Large array support is based on far field assumption and stationary channel over the size of the array.

-	Near field channel modeling and spatial non-stationarity is supported.

< Unchanged text omitted >

# 6	Status/expectation of existing information on high frequencies

## 6.1	Channel modelling works outside of 3GPP

This clause summarizes the channel modelling work outside of 3GPP based on the input from companies.

Groups and projects with channel models:

-	METIS (Mobile and wireless communications Enablers for the Twenty-twenty Information Society)

-	MiWEBA (Millimetre-Wave Evolution for Backhaul and Access)

-	ITU-R M

-	COST2100

-	IEEE 802.11

-	NYU WIRELESS: interdisciplinary academic research center

-	Fraunhofer HHI has developed the QuaDRiGa channel model, Matlab implementation is available at http://quadriga-channel-model.de

Groups and projects which intend to develop channel models:

-	5G mmWave Channel Model Alliance: NIST initiated, North America based

-	mmMAGIC (Millimetre-Wave Based Mobile Radio Access Network for Fifth Generation Integrated Communications): Europe based

-	IMT-2020 5G promotion association: China based

METIS Channel Models:

-	Identified 5G requirements (e.g., wide frequency range, high bandwidth, massive MIMO, 3-D and accurate polarization modelling)

-	Performed channel measurements at various bands between 2GHz and 60 GHz

-	Provided different channel model methodologies (map-based model, stochastic model or hybrid model). For stochastic model, the proposed channel is focused on outdoor square, Indoor cafeteria and indoor shopping mall scenarios.

MiWEBA Channel Models:

-	Addressed various challenges: Shadowing, spatial consistency, environment dynamics, spherical wave modelling, dual mobility Doppler model, ratio between diffuse and specular reflections, polarization

-	Proposed Quasi-deterministic channel model

-	Performed channel measurements at 60 GHz

-	Focused on university campus, street canyon, hotel lobby, backhaul, and D2D scenarios.

ITU-R M Channel Models:

-	Addressed the propagation loss and atmospheric loss on mmW

-	Introduced enabling antenna array technology and semiconductor technology

-	Proposed deployment scenarios, focused on dense urban environment for high data rate service: indoor shopping mall, indoor enterprise, in home, urban hotspot in a square/street, mobility in city.

COST2100 and COST IC1004 Channel Models:

-	Geometry-based stochastic channel model that reproduce the stochastic properties of MIMO channels over time, frequency and space. It is a cluster-level model where the statistics of the large scale parameters are always guaranteed in each series of channel instances.

NYU WIRELESS Channel Models:

-	Conducted many urban propagation measurements on 28/38/60/73 GHz bands for both outdoor and indoor channels, measurements are continuing.

-	Proposed 3 areas for 5G mmWave channel modelling which are small modifications or extensions from 3GPP's current below 6GHz channel models

-	1) LOS/NLOS/blockage modelling (a squared exponential term); 2). Wideband power delay profiles (time clusters and spatial lobes for a simple extension to the existing 3GPP SSCM model); 3). Physics-based path loss model (using the existing 3GPP path loss equations, but simply replacing the "floating" optimization parameter with a deterministic 1 m "close-in" free space reference term in order to provide a standard and stable definition of "path loss exponent" across all different parties, scenarios, and frequencies).

802.11 ad/ay Channel Models:

-	Conducted ray-tracing methodology on 60 GHz band indoor channels, including conference room, cubicle, living room scenarios

-	Intra cluster parameters were proposed in terms of ray excess delay and ray power distribution

-	Human blockage models were proposed in terms of blockage probability and blockage attenuation

5G mmWave Channel Model Alliance:

-	Will provide a venue to promote fundamental research into measurement, analysis, identification of physical parameters, and statistical representations of mmWave propagation channels.

-	Divided into six collaborative working groups that include a Steering Committee; Modelling Methodology Group; Measurement Methodology Group; and groups that focus on defining and parameterizing Indoor, Outdoor, and Emerging Usage Scenarios.

-	Sponsored by Communications Technology Research Laboratory within the NIST.

mmMAGIC:

-	Brings together major infrastructure vendors, major European operators, leading research institutes and universities, measurement equipment vendors and one SME.

-	Will undertake extensive radio channel measurements in the 6-100 GHz range.

-	Will develop and validate advanced channel models that will be used for rigorous validation and feasibility analysis of the proposed concepts and system, as well as for usage in regulatory and standards fora.

IMT-2020 5G promotion association

-	Jointly established by three ministries of China based on the original IMT-Advanced promotion group

-	Members including the main operators, vendors, universities and research institutes in China

-	The major platform to promote 5G technology research in China and to facilitate international communication and cooperation

QuaDRiGa (Fraunhofer HHI)

-	QuaDRiGa (QUAsi Deterministic RadIo channel GenerAtor) was developed at the Fraunhofer Heinrich Hertz Institute within the Wireless Communications and Networks Department to enable the modelling of MIMO radio channels for specific network configurations, such as indoor, satellite or heterogeneous configurations.

-	Besides being a fully-fledged 3D geometry-based stochastic channel model (well aligned with TR36.873), QuaDRiGa contains a collection of features created in SCM(e) and WINNER channel models along with novel modelling approaches which provide features to enable quasi-deterministic multi-link tracking of users (receiver) movements in changing environments. QuaDRiGa supports Massive MIMO modelling enabled through a new multi-bounce scattering approach and spherical wave propagation. It will be continuously extended with features required by 5G and frequencies beyond 6 GHz. The QuaDRiGa model is supported by data from extensive channel measurement campaigns at 10 / 28 / 43 / 60 / 82 GHz performed by the same group.

ATIS Next G Alliance

-	Published a report that includes a collection of results from several measurement campaigns relevant to deployment scenarios and use cases for 7-24 GHz channel modeling enhancements and channel modeling for integrated sensing and communications.

## 6.2	Scenarios of interest

Brief description of the key scenarios of interest identified (see note):

(1)	UMi (Street canyon, open area) with O2O and O2I: This is similar to 3D-UMi scenario, where the BSs are mounted below rooftop levels of surrounding buildings. UMi open area is intended to capture real-life scenarios such as a city or station square. The width of the typical open area is in the order of 50 to 100 m.

Example: [Tx height:10m, Rx height: 1.5-2.5 m, ISD: 200m]

(2)	UMa with O2O and O2I: This is similar to 3D-UMa scenario, where the BSs are mounted above rooftop levels of surrounding buildings.

Example: [Tx height:25m, Rx height: 1.5-2.5 m, ISD: 200m, 500m]

(3)	Indoor: This scenario is intended to capture various typical indoor deployment scenarios, including office environments, and shopping malls. The typical office environment is comprised of open cubicle areas, walled offices, open areas, corridors etc. The BSs are mounted at a height of 2-3 m either on the ceilings or walls. The shopping malls are often 1-5 stories high and may include an open area (or "atrium") shared by several floors. The BSs are mounted at a height of approximately 3 m on the walls or ceilings of the corridors and shops.

Example: [Tx height: 2-3m, Rx height: 1.5m, area: 500 square meters]

(4)	Backhaul, including outdoor above roof top backhaul in urban area and street canyon scenario where small cell BSs are placed at lamp posts.

(5)	D2D/V2V. Device-to-device access in open area, street canyon, and indoor scenarios. V2V is a special case where the devices are mobile.

(6)	Other scenarios such as Stadium (open-roof) and Gym (close-roof).

(7)	Indoor industrial scenarios

(8) Suburban macro (SMa) scenarios: In suburban macro-cells base stations are located above the surrounding environment to allow wide area coverage, and mobile stations are outdoors at street level and within commercial and residential buildings. Buildings are typically low residential detached houses with one or two floors, or blocks of apartments/condos or commercial buildings with a few floors. Occasional open areas such as parks or playgrounds between the houses make the environment rather open. Streets do not form urban-like regular strict grid structure. In suburban areas, vegetation is more prevalent than in urban areas with a high variability of foliage density across different suburban areas.

Note:	The scenarios of interest are based on the plenary email discussion and different from the supported scenarios in clause 7. The indoor industrial scenarios were identified at a later stage in the TSG RAN #81 meeting. SMa scenarios were identified at a later stage as part of Study on channel modelling enhancements for 7-24GHz for NR.

## 6.3	Channel measurement capabilities

The measurement capability as reported by each company is summarized in the following table.

Table 6.3-1: Channel measurement capabilities

|  | 6 - 20 GHz | 20 - 30 GHz | 30 - 60 GHz | >60 GHz |
| --- | --- | --- | --- | --- |
| Urban macro | CMCC Nokia/Aalborg | Nokia/Aalborg | NYU |  |
| Urban micro | Aalto University CMCC Ericsson Intel/Fraunhofer HHI Nokia/Aalborg NTT DOCOMO Orange | AT&T Aalto University CMCC Huawei Intel/Fraunhofer HHI Nokia/Aalborg NTT DOCOMO NYU Qualcomm Samsung CATT KT ETRI ITRI/CCU ZTE | AT&T Huawei Intel/Fraunhofer HHI NTT DOCOMO Qualcomm CATT ETRI ITRI/CCU ZTE | AT&T Aalto University Huawei Intel/Fraunhofer HHI NYU |
| Indoor | Aalto University CMCC Ericsson Huawei Intel/Fraunhofer HHI Nokia/Aalborg NTT DOCOMO Orange | AT&T Alcatel-Lucent Aalto University BUPT CMCC Huawei Intel/Fraunhofer HHI Nokia/Aalborg NTT DOCOMO NYU Qualcomm Samsung CATT KT ETRI ITRI/CCU ZTE | AT&T Ericsson Huawei Intel/Fraunhofer HHI NTT DOCOMO NYU Qualcomm CATT ETRI ITRI/CCU ZTE | AT&T Aalto University Huawei Intel/Fraunhofer HHI NYU |
| O2I | Ericsson Huawei Intel/Fraunhofer HHI Nokia/Aalborg NTT DOCOMO Orange | AT&T Alcatel-Lucent Ericsson Huawei Intel/Fraunhofer HHI NTT DOCOMO NYU Samsung KT | AT&T Ericsson Huawei Intel/Fraunhofer HHI NTT DOCOMO | AT&T Huawei Intel/Fraunhofer HHI |

## 6.4	Modelling objectives

The requirements for channel modelling are as follows.

-	Channel model SI should take into account the outcome of RAN-level discussion in the '5G' requirement study item

-	Complexity in terms of Description, Generating channel coefficients, development complexity and Simulation time should be considered.

-	Support frequency range up to 100 GHz.

-	The critical path of the SI is 6 – 100 GHz

-	Take care of mmW propagation aspects such as blocking and atmosphere attenuation.

-	The model should be consistent in space, time and frequency

-	Support large channel bandwidths (up to 10% of carrier frequency)

-	Aim for the channel model to cover a range of coupling loss considering current typical cell sizes, e.g. up to km-range macro cells. Note: This is to enable investigation of the relevance of the 5G system using higher frequency bands to existing deployments.

-	Accommodate UT mobility

-	Mobile speed up to 500 km/h.

-	Develop a methodology considering that model extensions to D2D and V2V may be developed in future SI.

-	Support large antenna arrays

-	Support near-field channel propagation and spatial non-stationarity

< Unchanged text omitted >

# 7	Channel model(s) for 0.5-100 GHz

< Unchanged text omitted >

## 7.2	Scenarios

The detailed scenario description in this clause can be used for channel model calibration.

UMi-street canyon and UMa

Details on UMi-street canyon and UMa scenarios are listed in Table 7.2-1.

Table 7.2-1: Evaluation parameters for UMi-street canyon and UMa scenarios

| Parameters |  | UMi - street canyon | UMa |
| --- | --- | --- | --- |
| Cell layout |  | Hexagonal grid, 19 micro sites, 3 sectors per site (ISD = 200m) | Hexagonal grid, 19 macro sites, 3 sectors per site (ISD = 500m, see note 1) |
| BS antenna height |  | 10m | 25m |
| UT location | Outdoor/indoor | Outdoor and indoor | Outdoor and indoor |
|  | LOS/NLOS | LOS and NLOS | LOS and NLOS |
|  | Height | Same as 3D-UMi in TR36.873 | Same as 3D-UMa in TR36.873 |
| Indoor UT ratio |  | 80% | 80% |
| UT mobility (horizontal plane only) |  | 3km/h | 3km/h |
| Min. BS - UT distance (2D) |  | 10m | 35m |
| UT distribution (horizontal) |  | Uniform | Uniform |
| NOTE 1: UMa scenarios with ISDs between 200-500m can be used for evaluations. |  |  |  |

Indoor-office

Details on indoor-office scenarios are listed in Table 7.2-2 and presented in Figure 7.2-1. More details, if necessary, can be added to Figure 7.2-1.

Table 7.2-2: Evaluation parameters for indoor-office scenarios

| Parameters |  | Indoor - office open office | Indoor - office mixed office |
| --- | --- | --- | --- |
| Layout | Room size (WxLxH) | 120mx50mx3m |  |
|  | ISD | 20m |  |
| BS antenna height |  | 3 m (ceiling) |  |
| UT location | LOS/NLOS | LOS and NLOS |  |
|  | Height | 1 m |  |
| UT mobility (horizontal plane only) |  | 3 km/h |  |
| Min. BS - UT distance (2D) |  | 0 |  |
| UT distribution (horizontal) |  | Uniform |  |

Note:	The only difference between the open office and mixed office models in this TR is the line of sight probability.

Figure 7.2-1: Layout of indoor office scenarios.

RMa

The rural deployment scenario focuses on larger and continuous coverage. The key characteristics of this scenario are continuous wide area coverage supporting high speed vehicles. This scenario will be noise-limited and/or interference-limited, using macro TRPs. Details of RMa scenario is described in Table 7.2-3.

Table 7.2-3: Evaluation parameters for RMa

| Parameters | RMa |
| --- | --- |
| Carrier Frequency | Up to 7Ghz |
| BS height | 35m |
| Layout | Hexagonal grid, 19 Macro sites, 3sectors per site, ISD = 1732m or 5000m |
| UT height | 1.5m |
| UT distribution | Uniform |
| Indoor/Outdoor | 50% indoor and 50% in car |
| LOS/NLOS | LOS and NLOS |
| Min BS - UT distance(2D) | 35m |

Indoor Factory (InF)

The indoor factory (InF) scenario focuses on factory halls of varying sizes and with varying levels of density of "clutter", e.g. machinery, assembly lines, storage shelves, etc. Details of the InF scenario are listed in Table 7.2-4.

Table 7.2-4: Evaluation parameters for InF

|  |  | InF |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameters |  | InF-SL (sparse clutter, low BS) | InF-DL (dense clutter, low BS) | InF-SH (sparse clutter, high BS) | InF-DH (dense clutter, high BS) | InF-HH (high Tx, high Rx) |
| Layout | Room size | Rectangular: 20-160000 m2 |  |  |  |  |
|  | Ceiling height | 5-25 m | 5-15 m | 5-25 m | 5-15 m | 5-25 m |
|  | Effective clutter height hc | < Ceiling height, 0-10 m |  |  |  |  |
|  | External wall and ceiling type | Concrete or metal walls and ceiling with metal-coated windows |  |  |  |  |
| Clutter type |  | Big machineries composed of regular metallic surfaces. For example: several mixed production areas with open spaces and storage/commissioning areas | Small to medium metallic machinery and objects with irregular structure. For example: assembly and production lines surrounded by mixed small-sized machineries. | Big machineries composed of regular metallic surfaces. For example: several mixed production areas with open spaces and storage/commissioning areas | Small to medium metallic machinery and objects with irregular structure. For example: assembly and production lines surrounded by mixed small-sized machineries. | Any |
| Typical clutter size, dclutter |  | 10 m | 2 m | 10 m | 2 m | Any |
| Clutter density r (percentage of surface area occupied by clutter) |  | Low clutter density (<40%) | High clutter density (≥40%) | Low clutter density (<40%) | High clutter density (≥40%) | Any |
| BS antenna height |  | Clutter-embedded, i.e. the BS antenna height is below the average clutter height |  | Above clutter |  | Above clutter |
| UT location | LOS/NLOS | LOS and NLOS |  |  |  | 100% LOS |
|  | Height | Clutter-embedded |  |  |  | Above clutter |

SMa

Details on SMa scenarios are listed in Table 7.2-5.

Table 7.2-5: Evaluation parameters for SMa scenarios

| Parameters |  | SMa |
| --- | --- | --- |
| Cell layout |  | Hexagonal grid, 19 macro sites, 3 sectors per site (ISD = 1299m and 1732m, see note) Up to two floors for residential buildings, up to five floors for commercial buildings. Building distribution are 90% residential and 10% commercial buildings |
| BS antenna height hBS |  | 35 ms |
| UT location | Outdoor/indoor | Outdoor and indoor |
|  | LOS/NLOS | LOS and NLOS |
|  | Height hUT | 1.5 m for outdoor, 1.5 or 4.5 m for residential buildings, 1.5/4.5/7.5/10.5/13.5 m for commercial buildings |
| Indoor UT ratio |  | 80% |
| UT mobility (horizontal plane only) |  | Indoor UTs: 3km/h Outdoor UTs: 40km/h |
| Min. BS - UT distance (2D) |  | 35m |
| UT distribution (horizontal) |  | Uniform |
| UT distribution (vertical) |  | Uniform distribution across all floors for a building type |
| Penetration model |  | Low-loss A penetration model |
| NOTE: For evaluation in study items/work items, if needed, admission control policies may be considered in conjunction with the ISD choice to address out of coverage UEs |  |  |

## 7.3	Antenna modelling

This clause captures the antenna array structures considered in this SI for calibration.

The BS antenna is modelled by a uniform rectangular panel array, comprising MgNg panels, as illustrated in Figure 7.3-1 with Mg being the number of panels in a column and Ng being the number of panels in a row. Furthermore the following properties apply:

-	Antenna panels are uniformly spaced in the horizontal direction with a spacing of dg,H and in the vertical direction with a spacing of dg,V.

-	On each antenna panel, antenna elements are placed in the vertical and horizontal direction, where N is the number of columns, M is the number of antenna elements with the same polarization in each column.

-	Antenna numbering on the panel illustrated in Figure 7.3-1 assumes observation of the antenna array from the front (with x-axis pointing towards broad-side and increasing y-coordinate for increasing column number).

-	The antenna elements are uniformly spaced in the horizontal direction with a spacing of dH and in the vertical direction with a spacing of dV.

-	The antenna panel is either single polarized (P =1) or dual polarized (P =2).

The rectangular panel array antenna can be described by the following tuple .

Figure 7.3-1: Cross-polarized panel array antenna model

The antenna radiation power pattern of each antenna element is generated according to Table 7.3-1.

Table 7.3-1: Radiation power pattern of a single antenna element

| Parameter | Values |
| --- | --- |
| Vertical cut of the radiation power pattern (dB) |  |
| Horizontal cut of the radiation power pattern (dB) |  |
| 3D radiation power pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 8 dBi |

The UE antenna can be modelled by placing the antenna element/array at the placement candidate locations relative to the centre of the UE. The size of the device can be set to (X cm, Y cm, Z cm), in depth, width, and height respectively.

For cases when a candidate antenna placement location is used for one antenna field pattern:

-	Reference radiation pattern of the UE antenna model is vertically polarized with all the gain in the theta filed component, Fθ″″θ″,ϕ″=A″θ″,ϕ″ and Fϕ″″θ″,ϕ″=0.

For cases when a candidate antenna placement location is used for two distinct antenna polarization field patterns:

-	Reference radiation pattern of the UE antenna model is,

-	For first antenna field pattern: Fθ″″θ″,ϕ″=A″θ″,ϕ″ and Fϕ″″θ″,ϕ″=0.

-	For second antenna field pattern: Fθ″″θ″,ϕ″=0 and Fϕ″″θ″,ϕ″=A″θ″,ϕ″.

Each polarized field component of the reference radiation pattern Fθ″″θ″,ϕ″ and Fϕ″″θ″,ϕ″ should be rotated according to the orientation of the each of UE antennae to get Fu,θ''θ',ϕ', Fu, ϕ''θ',ϕ' and based on the orientation of the UE in the global coordinate system to get Fu,θθ,ϕ and Fu,ϕθ,ϕ using the method specified in Clause 7.3.2  equation (7.3-3) and Clause 7.1.3 equation (7.1-11).

For handheld UE devices, the following additional properties apply:

-	Four corners of the rectangle and the center of edges of the rectangle that reflect UE form factor are identified as potential locations of the UE antenna and potential locations are enumerated as illustrated in Figure 7.3-2.

-	Antenna are assumed to be oriented along the direction determined by the vector connecting the centre of the rectangle to the antenna location.

-	Reference UE orientation vector of the handheld UE is perpendicular to the plane of the flat UE handheld device, and reference point for near field phase calculation of the UE is assumed to be the center of the plane of the UE handheld.

-	Device dimensions for UE antenna modelling are (15 cm, 7 cm, 0 cm).

Figure 7.3-2: Handheld UE antenna placement candidate locations relative to centre of device (top down view)

-	[placeholder for reference orientation of handheld UE]

-	The antenna radiation power pattern of each antenna element for handheld UE is generated according to Table 7.3-2.

Table 7.3-2: Radiation power pattern of a single antenna element for handheld UE

| Parameter | Values |
| --- | --- |
| Vertical cut of the radiation power pattern (dB) | AdB″θ″, ϕ″=0°=-min12θ″-90°θ3dB2,SLAV with θ3dB=125°,SLAV=22.5dB and θ″∈0°, 180° |
| Horizontal cut of the radiation power pattern (dB) | AdB″θ″=90°, ϕ″=-min12ϕ″ϕ3dB2,Amax with ϕ3dB=125°,Amax=22.5 dB and ϕ″∈-180°, 180°max |
| 3D radiation power pattern (dB) | AdB″θ″,ϕ″=-min-AdB″θ″, ϕ″=0°+AdB″θ″=90°, ϕ″,Amax |
| Maximum directional gain of an antenna element GE,max | 5.3 dBi |
| NOTE: For UE antenna modelling of handheld devices, optional antenna imbalance can be modelled. If modelled, randomized loss is applied per UE antenna port. No imbalance is modelled by default. |  |

For consumer premise equipment (CPE) devices, the following properties apply:

-	Total 9 candidate antenna locations in 4 corners of the vertical square plane, 4 center of the edges of the vertical square plane, and center of the vertical square plane. The figure below captures the side view on the device. Note that a candidate antenna location (e.g., 9) can be used for multiple antennas.

-	Device dimensions for UE antenna modelling are (0 cm, 20 cm, 20 cm)

Figure 7.3-3: CPE UE antenna placement candidate locations relative to centre of device (side view)

### 7.3.1	Antenna port mapping

Legacy BS array antennas, i.e. uniform linear arrays with fix phase shifts between its M elements to obtain a beamtilt in vertical direction are modelled using complex weights

(7.3-1)

where m=1, …, M, θetilt is the electrical vertical steering angle defined between 0° and 180° (90° represents perpendicular to the array).  denotes the wavelength and  the vertical element spacing.

### 7.3.2	Polarized antenna modelling

In general the relationship between radiation field and power pattern is given by:

.	(7.3-2)

The following two models represent two options on how to determine the radiation field patterns based on a defined radiation power pattern.

Model-1:

In case of polarized antenna elements assume  is the polarization slant angle where  degrees corresponds to a purely vertically polarized antenna element and degrees correspond to a pair of cross-polarized antenna elements. Then the antenna element field components in  and  direction are given by

(7.3-3)

where , .

Note that the zenith and the azimuth field components , ,  and  are defined in terms of the spherical basis vectors of an LCS as defined in Clause 7.1. The difference between the single-primed and the double-primed components is that the single-primed field components account for the polarization slant and the double-primed field components do not. For a single polarized antenna (purely vertically polarized antenna) we can write  and  where is the 3D antenna radiation power pattern as a function of azimuth angle  and zenith angle  in the LCS as defined in Table 7.3-1 converted into linear scale.

Model-2:

In case of polarized antennas, the polarization is modelled as angle-independent in both azimuth and elevation, in an LCS. For a linearly polarized antenna, the antenna element field pattern, in the vertical polarization and in the horizontal polarization, are given by

(7.3-4)

and

,	(7.3-5)

respectively, where  is the polarization slant angle and is the 3D antenna element power pattern as a function of azimuth angle,  and elevation angle, in the LCS. Note that  degrees correspond to a purely vertically polarized antenna element. The vertical and horizontal field directions are defined in terms of the spherical basis vectors,  and  respectively in the LCS as defined in Clause 7.1.2. Also =, = and =  as defined in Table 7.1-1.

< Unchanged text omitted >

## 7.4	Pathloss, LOS probability and penetration modelling

### 7.4.1	Pathloss

The pathloss models are summarized in Table 7.4.1-1 and the distance definitions are indicated in Figure 7.4.1-1 and Figure 7.4.1-2. Note that the distribution of the shadow fading is log-normal, and its standard deviation for each scenario is given in Table 7.4.1-1.

| Figure7.4.1-1: Definition of d2D and d3D for outdoor UTs | Figure 7.4.1-2: Definition of d2D-out, d2D-in and d3D-out, d3D-in for indoor UTs. |
| --- | --- |

Note that

(7.4-1)

Table 7.4.1-1: Pathloss models

| Scenario | LOS/NLOS | Pathloss [dB], fc is in GHz and d is in meters, see note 6 | Shadow fading std [dB] | Applicability range, antenna height default values |
| --- | --- | --- | --- | --- |
| RMa | LOS | PLRMa-LOS=PL110m≤d2D≤dBPPL2dBP≤d2D≤10km, see note 5 PL1=20log10(40πd3Dfc/3)+min(0.03h1.72,10)log10(d3D) -min(0.044h1.72,14.77)+0.002log10(h)d3D PL2=PL1(dBP)+40log10(d3D/dBP) | For 10 m ≤ d2D < dBP, σSF=4 For dBP ≤ d2D < 10 km, σSF=6 | hBS=35m hUT=1.5m W=20m h=5m h = avg. building height W = avg. street width The applicability ranges: 5m≤h≤50m 5m≤W≤50m 10m≤hBS≤150m 1m≤hUT≤10m |
|  | NLOS | PLRMa-NLOS=max(PLRMa-LOS,PLRMa-NLOS'), for 10m≤d2D≤5km PLRMa-NLOS'=161.04-7.1log10(W)+7.5log10(h) -(24.37-3.7(h/hBS)2)log10(hBS) +(43.42-3.1log10(hBS))(log10(d3D)-3) +20log10(fc)-(3.2(log10(11.75hUT))2-4.97) | σSF=8 |  |
| UMa | LOS | PLUMa-LOS=PL110m≤d2D≤dBP'PL2dBP'≤d2D≤5km, see note 1 PL1=28.0+22log10(d3D)+20log10(fc) PL2=28.0+40log10(d3D)+20log10(fc) -9log10((dBP')2+(hBS-hUT)2) | σSF=4 | 1.5m≤hUT≤22.5m hBS=25m |
|  | NLOS | PLUMa-NLOS=max(PLUMa-LOS,PLUMa-NLOS'), for 10m≤d2D≤5km PLUMa-NLOS'=13.54+39.08log10(d3D)+ 20log10(fc)-0.6(hUT-1.5) | σSF=6 | 1.5m≤hUT≤22.5m hBS=25m Explanations: see note 3 |
|  |  | Optional PL=32.4+20log10fc+30log10d3D | σSF=7.8 |  |
| UMi - Street Canyon | LOS | PLUMi-LOS=PL110m≤d2D≤dBP'PL2dBP'≤d2D≤5km, see note 1 PL1=32.4+21log10(d3D)+20log10(fc) PL2=32.4+40log10(d3D)+20log10(fc) -9.5log10((dBP')2+(hBS-hUT)2) | σSF=4 | 1.5m≤hUT≤22.5m hBS=10m |
|  | NLOS | PLUMi-NLOS=max(PLUMi-LOS,PLUMi-NLOS'), for 10m≤d2D≤5km PLUMi-NLOS'=35.3log10(d3D)+22.4 + 21.3log10(fc)-0.3(hUT-1.5) | σSF=7.82 | 1.5m≤hUT≤22.5m hBS=10m Explanations: see note 4 |
|  |  | Optional PL=32.4+20log10fc+31.9log10d3D | σSF=8.2 |  |
| InH - Office | LOS | PLInH-LOS=32.4+17.3log10(d3D)+20log10(fc) | σSF=3 | 1m≤d3D≤150m |
|  | NLOS | PLInH-NLOS=max(PLInH-LOS,PLInH-NLOS') PLInH-NLOS'=38.3log10d3D+17.30+24.9log10fc | σSF=8.03 | 1m≤d3D≤150m |
|  |  | Optional PLInH-NLOS'=32.4+20log10fc+31.9log10d3D | σSF=8.29 | 1m≤d3D≤150m |
| InF | LOS | PLLOS=31.84+21.50log10d3D+19.00log10fc | σSF=4.3 | 1≤d3D≤600 m |
|  | NLOS | InF-SL: PL=33+25.5log10d3D+20log10fc PLNLOS=maxPL,PLLOS | σSF=5.7 |  |
|  |  | InF-DL: PL=18.6+35.7log10d3D+20log10fc PLNLOS=maxPL,PLLOS,PLInF-SL | σSF=7.2 |  |
|  |  | InF-SH: PL=32.4+23.0log10d3D+20log10fc PLNLOS=maxPL,PLLOS | σSF=5.9 |  |
|  |  | InF-DH: PL=33.63+21.9log10d3D+20log10fc PLNLOS=maxPL,PLLOS | σSF=4.0 |  |
| SMa | LOS | For 10 m ≤ d < dBP, PL=20log1040πdfc3+min0.03h1.72,10log10d- min0.044h1.72, 14.77+0.002log10hd, see note 5 For dBP ≤ d < 5000 m, PL2=PL1dBP+40log10ddBP, see note 5 | For 10 m ≤ d < dBP, σSF=4 For dBP ≤ d < 5000 m, σSF=6 | 0.8 ≤fc≤ 37 GHz 10 m ≤ d < dBP dBP ≤ d < 5000 m 25 m < hBS < 35 m h = 10 m |
|  | NLOS | PL=161.04-7.1log10W+7.5log10h-24.37-3.7hhBS2log10hBS+43.42-3.1log10hBSlog10d-3+20log10fc-3.2log1011.75hUT2-4.97 | σSF=8 | 0.8 ≤fc≤ 37 GHz 10 m ≤ d < 5000 m 1 m < hUT < 14 m h = 10 m, W = 10 m |
| NOTE 1: Effective breakpoint distance d'BP = 4 h'BS h'UT fc/c, where fc is the centre frequency in Hz, c = 3.0108 m/s is the propagation velocity in free space, and h'BS and h'UT are the effective antenna heights at the BS and the UT, respectively. The effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – hE, h'UT = hUT – hE, where hBS and hUT are the actual antenna heights, and hE is the effective environment height. For UMi hE = 1.0m. For UMa hE=1m with a probability equal to 1/(1+C(d2D, hUT)) and chosen from a discrete uniform distribution uniform(12,15,…,(hUT-1.5)) otherwise. With C(d2D, hUT) given by Cd2D,hUT=0,hUT<13mhUT-13101.5gd2D,13m≤hUT≤23m, where gd2D=0,d2D≤18m54d2D1003exp-d2D150,18m<d2D. Note that hE depends on d2D and hUT and thus needs to be independently determined for every link between BS sites and UTs. A BS site may be a single BS or multiple co-located BSs. NOTE 2: The applicable frequency range of the PL formula in this table is 0.5 < fc < fH GHz, where fH = 30 GHz for RMa, fH = 37 GHz for SMa, and fH = 100 GHz for all the other scenarios. It is noted that RMa pathloss model for >7 GHz is validated based on a single measurement campaign conducted at 24 GHz. NOTE 3: UMa NLOS pathloss is from TR36.873 with simplified format and PLUMa-LOS = Pathloss of UMa LOS outdoor scenario. NOTE 4: PLUMi-LOS = Pathloss of UMi-Street Canyon LOS outdoor scenario. NOTE 5: Break point distance dBP = 2π hBS hUT fc/c, where fc is the centre frequency in Hz, c = 3.0 108 m/s is the propagation velocity in free space, and hBS and hUT are the antenna heights at the BS and the UT, respectively. NOTE 6: fc denotes the center frequency normalized by 1GHz, all distance related values are normalized by 1m, unless it is stated otherwise. |  |  |  |  |

### 7.4.2	LOS probability

The Line-Of-Sight (LOS) probabilities are given in Table 7.4.2-1.

Table 7.4.2-1 LOS probability

| Scenario | LOS probability (distance is in meters) |
| --- | --- |
| RMa |  |
| UMi - Street canyon |  |
| UMa | where |
| Indoor - Mixed office |  |
| Indoor - Open office |  |
| InF-SL InF-SH InF-DL InF-DH | PrLOS,subsced2D=exp-d2Dksubsce where ksubsce=-dclutterln1-rfor InF-SL and InF-DL-dclutterln1-r∙hBS-hUThc-hUTfor InF-SH and InF-DH The parameters dclutter, r, and hc are defined in Table 7.2-4 |
| InF-HH | PrLOS=1 |
| SMa | PLOS=1, d2D≤10 mexp-d2Dkcommercialexp-d2Dkresidentialexp-d2Dkvegetation, d2D>10 m where 1kcommercial=-ln1-rcommericialdclutter∙hcommercial-hUThBS-hUT 1kresidential=-ln1-rresidentialdclutter∙maxhresidential-hUT ,0hBS-hUT 1kvegetation=-ln1-rvegetationdclutter∙hvegetation-hUThBS-hUT dclutter=30 m, hcommercial = 20 m, hresidential= 8 m, hvegetation = 15 m rcommericial=2%, rresidential=18%, rvegetation=0% (no vegetation), 10% (sparse vegetation), or 20% (dense vegetation) |
| NOTE: The LOS probability is derived with assuming antenna heights of 3m for indoor, 10m for UMi, and 25m for UMa |  |

< Unchanged text omitted >

### 7.4.3	O2I penetration loss

#### 7.4.3.1	O2I building penetration loss

The pathloss incorporating O2I building penetration loss is modelled as in the following:

PL=PLb+PLtw+PLin+N0,σP2	(7.4-2)

where PLb is the basic outdoor path loss given in Clause 7.4.1, where d3D is replaced by d3D-out+d3D-in . PLtw is the building penetration loss through the external wall, PLin is the inside loss dependent on the depth into the building, and σP is the standard deviation for the penetration loss.

PLtw is characterized as:

PLtw=PLnpi-10log10i=1Npi×10Lmateriali-10	(7.4-3)

PLnpi is an additional loss is added to the external wall loss to account for non-perpendicular incidence; Lmateriali=amateriali+bmateriali⋅f, is the penetration loss of material i, example values of which can be found in Table 7.4.3-1; pi is proportion of i-th materials, where i=1Npi=1; and N is the number of materials.

Table 7.4.3-1: Material penetration losses

| Material | Penetration loss [dB] | Notes on Material |
| --- | --- | --- |
| Standard multi-pane glass | Lglass=2+0.2f | Reference thickness of 3 cm assumed |
| IRR glass | LIRRglass=25.4+0.11f, see note 2 | Single coating is assumed |
| Concrete | Lconcrete=5+4f | Reference thickness of 23 cm assumed |
| Wood | Lwood=4.85+0.12f | Reference thickness of 3.3 cm assumed |
| Plywood | Lplywood=1.03+0.17f | Reference thickness of 1.27 cm assumed |
| NOTE 1: f is in GHz NOTE 2: Values marked with this note were updated as part of Release 19 Study on channel modelling enhancements for 7-24 GHz for NR. |  |  |

Table 7.4.3-2 gives PLtw, PLin and σP for two O2I penetration loss models. The O2I penetration is UT-specifically generated, and is added to the SF realization in the log domain.

Table 7.4.3-2: O2I building penetration loss model

|  | Path loss through external wall: PLtw in [dB] | Indoor loss: PLin in [dB] | Standard deviation: σP in [dB] |
| --- | --- | --- | --- |
| Low-loss model | 5-10log100.3⋅10-Lglass10+0.7⋅10-Lconcrete10 | 0.5 d2D-in | 4.4 |
| High-loss model | 5-10log100.7⋅10-LIRRglass10+0.3⋅10-Lconcrete10 | 0.5 d2D-in | 6.5 |
| Low-loss A model | 5-10log100.3⋅10-Lglass10+0.7⋅10-Lplywood10 | 0.5 d2D-in | 4.4 |

d2D-in is minimum of two independently generated uniformly distributed variables between 0 and 25 m for UMa and UMi-Street Canyon, and between 0 and 10 m for RMa. d2D-in shall be UT-specifically generated.

Both low-loss and high-loss models are applicable to UMa and UMi-Street Canyon.

Only the low-loss model is applicable to RMa.

Only the high-loss model is applicable to InF.

The Low-loss A model is applicable to SMa.

The composition of low and high loss is a simulation parameter that should be determined by the user of the channel models, and is dependent on the use of metal-coated glass in buildings and the deployment scenarios. Such use is expected to differ in different markets and regions of the world and also may increase over years to new regulations and energy saving initiatives. Furthermore, the use of such high-loss glass currently appears to be more predominant in commercial buildings than in residential buildings in some regions of the world (see note).

Note:	One example survey for the US market can be found in [5]. The survey does not necessarily be representative for all the scenarios. Other ratios outside of the survey should not be precluded.

For backwards compatibility with TR 36.873 [3], the following building penetration model should be used for UMa and UMi single-frequency simulations at frequencies below 6 GHz.

Table 7.4.3-3. O2I building penetration loss model for single-frequency simulations <6 GHz

| Parameter | Value |
| --- | --- |
| PLtw | 20 dB |
| PLin | 0.5 d2D-in with d2D-in being a single, link-specific, uniformly distributed variable between 0 and 25 m |
| σP | 0 dB |
| σSF | 7 dB (note: replacing the respective value in Table 7.4.1-1) |

< Unchanged text omitted >

## 7.5	Fast fading model

< Unchanged text omitted >

Step 12: Apply pathloss and shadowing for the channel coefficients.

< Unchanged text omitted >

Table 7.5-6 Part-1: Channel model parameters for UMi-Street Canyon and UMa

| Scenarios |  | UMi - Street Canyon |  |  | UMa |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O2I | LOS | NLOS | O2I |
| Delay spread (DS) lgDS=log10(DS/1s) | lgDS | -0.18 log10(1+ fc) - 7.28, see note 8 | -0.22 log10(1+ fc) - 6.87, see note 8 | -6.62 | -7.067 - 0.0794 log10(fc), see note 8 | -6.47 - 0.134 log10(fc), see note 8 | -6.62 |
|  | lgDS | 0.39, see note 8 | 0.19 log10(1+ fc) + 0.22, see note 8 | 0.32 | 0.57 + 0.026 log10(fc), see note 8 | 0.39 | 0.32 |
| AOD spread (ASD) lgASD=log10(ASD/1) | lgASD | -0.05 log10(1+ fc) + 1.21 | -0.23 log10(1+ fc) + 1.53 | 1.25 | 0.95, see note 8 | 1.12, see note 8 | 0.84, see note 8 |
|  | lgASD | 0.41 | 0.11 log10(1+ fc) + 0.33 | 0.42 | 0.31, see note 8 | 0.42, see note 8 | 0.42 |
| AOA spread (ASA) lgASA=log10(ASA/1) | lgASA | -0.07 log10(1+ fc) + 1.66, see note 8 | -0.07 log10(1+ fc) + 1.76, see note 8 | 1.76 | 1.76, see note 8 | 2.04 - 0.25 log10(fc), see note 8 | 1.76 |
|  | lgASA | 0.021 log10(1+ fc) + 0.26, see note 8 | 0.05 log10(1+ fc) + 0.27, see note 8 | 0.16 | 0.19, see note 8 | 0.17 - 0.03 log10(fc), see note 8 | 0.16 |
| ZOA spread (ZSA) lgZSA=log10(ZSA/1) | lgZSA | -0.1 log10(1+ fc) + 0.73 | -0.04 log10(1+ fc) + 0.92 | 1.01 | 0.95 | -0.2856 log10(fc) + 1.445, see note 8 | 1.01 |
|  | lgZSA | -0.04 log10(1+ fc) + 0.34 | -0.07 log10(1+ fc) + 0.41 | 0.43 | 0.16 | 0.17, see note 8 | 0.43 |
| Shadow fading (SF) [dB] | SF | See Table 7.4.1-1 | See Table 7.4.1-1 | 7 | See Table 7.4.1-1 | See Table 7.4.1-1 | 7 |
| K-factor (K) [dB] | K | 9 | N/A | N/A | 9 | N/A | N/A |
|  | K | 5 | N/A | N/A | 3.5 | N/A | N/A |
| Cross-Correlations | ASD vs DS | 0.5 | 0 | 0.4 | 0.4 | 0.4 | 0.4 |
|  | ASA vs DS | 0.8 | 0.4 | 0.4 | 0.8 | 0.6 | 0.4 |
|  | ASA vs SF | -0.4 | -0.4 | 0 | -0.5 | 0 | 0 |
|  | ASD vs SF | -0.5 | 0 | 0.2 | -0.5 | -0.6 | 0.2 |
|  | DS vs SF | -0.4 | -0.7 | -0.5 | -0.4 | -0.4 | -0.5 |
|  | ASD vs ASA | 0.4 | 0 | 0 | 0 | 0.4 | 0 |
|  | ASD vs  | -0.2 | N/A | N/A | 0 | N/A | N/A |
|  | ASA vs  | -0.3 | N/A | N/A | -0.2 | N/A | N/A |
|  | DS vs  | -0.7 | N/A | N/A | -0.4 | N/A | N/A |
|  | SF vs  | 0.5 | N/A | N/A | 0 | N/A | N/A |
| Cross-Correlations 1) | ZSD vs SF | 0 | 0 | 0 | 0 | 0 | 0 |
|  | ZSA vs SF | 0 | 0 | 0 | -0.8 | -0.4 | 0 |
|  | ZSD vs K | 0 | N/A | N/A | 0 | N/A | N/A |
|  | ZSA vs K | 0 | N/A | N/A | 0 | N/A | N/A |
|  | ZSD vs DS | 0 | -0.5 | -0.6 | -0.2 | -0.5 | -0.6 |
|  | ZSA vs DS | 0.2 | 0 | -0.2 | 0 | 0 | -0.2 |
|  | ZSD vs ASD | 0.5 | 0.5 | -0.2 | 0.5 | 0.5 | -0.2 |
|  | ZSA vs ASD | 0.3 | 0.5 | 0 | 0 | -0.1 | 0 |
|  | ZSD vs ASA | 0 | 0 | 0 | -0.3 | 0 | 0 |
|  | ZSA vs ASA | 0 | 0.2 | 0.5 | 0.4 | 0 | 0.5 |
|  | ZSD vs ZSA | 0 | 0 | 0.5 | 0 | 0 | 0.5 |
| Delay scaling parameter r |  | 3 | 2.1 | 2.2 | 2.5 | 2.3 | 2.2 |
| XPR [dB] | XPR | 9 | 8.0 | 9 | 8 | 7 | 9 |
|  | XPR | 3 | 3 | 5 | 4 | 3 | 5 |
| Number of clusters |  | 12 | 19 | 12 | 12 | 20 | 12 |
| Number of rays per cluster |  | 20 | 20 | 20 | 20 | 20 | 20 |
| Cluster DS () in [ns] |  | 5 | 11 | 11 | max(0.25, 6.5622 -3.4084 log10(fc)) | max(0.25, 6.5622 -3.4084 log10(fc)) | 11 |
| Cluster ASD () in [deg] |  | 3 | 10 | 5 | 3.74, see note 8 | 1.82, see note 8 | 1.82, see note 8 |
| Cluster ASA () in [deg] |  | 17 | 22 | 8 | 11 | 15 | 8 |
| Cluster ZSA () in [deg] |  | 7 | 7 | 3 | 7 | 7 | 3 |
| Per cluster shadowing std  [dB] |  | 3 | 3 | 4 | 3 | 3 | 4 |
| Correlation distance in the horizontal plane [m] | DS | 7 | 10 | 10 | 30 | 40 | 10 |
|  | ASD | 8 | 10 | 11 | 18 | 50 | 11 |
|  | ASA | 8 | 9 | 17 | 15 | 50 | 17 |
|  | SF | 10 | 13 | 7 | 37 | 50 | 7 |
|  |  | 15 | N/A | N/A | 12 | N/A | N/A |
|  | ZSA | 12 | 10 | 25 | 15 | 50 | 25 |
|  | ZSD | 12 | 10 | 25 | 15 | 50 | 25 |
| fc is carrier frequency in GHz; d2D is BS-UT distance in km. NOTE 1: DS = rms delay spread, ASD = rms azimuth spread of departure angles, ASA = rms azimuth spread of arrival angles, ZSD = rms zenith spread of departure angles, ZSA = rms zenith spread of arrival angles, SF = shadow fading, and K = Ricean K-factor. NOTE 2: The sign of the shadow fading is defined so that positive SF means more received power at UT than predicted by the path loss model. NOTE 3: All large scale parameters are assumed to have no correlation between different floors. NOTE 4: The following notation for mean (μlgX=mean{log10(X) }) and standard deviation (σlgX=std{log10(X) }) is used for logarithmized parameters X. NOTE 5: For all considered scenarios the AOD/AOA distributions are modelled by a wrapped Gaussian distribution, the ZOD/ZOA distributions are modelled by a Laplacian distribution and the delay distribution is modelled by an exponential distribution. NOTE 6: For UMa and frequencies below 6 GHz, use fc = 6 when determining the values of the frequency-dependent LSP values NOTE 7: For UMi and frequencies below 2 GHz, use fc = 2 when determining the values of the frequency-dependent LSP values NOTE 8: Values marked with this note were updated as part of Release 19 Study on channel modelling enhancements for 7-24 GHz for NR. |  |  |  |  |  |  |  |

< Unchanged text omitted >

Table 7.5-6 Part-4: Channel model parameters for SMa

| Scenarios |  | SMa |  |  |
| --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O2I |
| Delay spread (DS) lgDS=log10(DS/1s) | lgDS | –7.23 | -7.12 | -7.12 |
|  | lgDS | 0.38 | 0.33 | 0.33 |
| AOD spread (ASD) lgASD=log10(ASD/1) | lgASD | 0.55 | 0.55 | 0.55 |
|  | lgASD | 0.25 | 0.25 | 0.25 |
| AOA spread (ASA) lgASA=log10(ASA/1) | lgASA | 1.07 | 1.39 | 1.39 |
|  | lgASA | 0.13 | 0.34 | 0.34 |
| ZOA spread (ZSA) lgZSA=log10(ZSA/1) | lgZSA | 0.95 | 1.14 | 1.14 |
|  | lgZSA | 0.16 | 0.16 | 0.16 |
| Shadow fading (SF) [dB] | SF | 4 | 8 | 8 |
| K-factor (K) [dB] | K | 9 | N/A | N/A |
|  | K | 7 | N/A | N/A |
| Cross-Correlations | ASD vs DS | 0 | 0 | 0 |
|  | ASA vs DS | 0.8 | 0.7 | 0.7 |
|  | ASA vs SF | –0.5 | 0 | 0 |
|  | ASD vs SF | –0.5 | -0.4 | -0.4 |
|  | DS vs SF | –0.6 | -0.13 | -0.13 |
|  | ASD vs ASA | 0 | 0 | 0 |
|  | ASD vs  | 0 | N/A | N/A |
|  | ASA vs  | 0 | N/A | N/A |
|  | DS vs  | 0 | N/A | N/A |
|  | SF vs  | 0 | N/A | N/A |
| Cross-Correlations 1) | ZSD vs SF | 0 | 0 | 0 |
|  | ZSA vs SF | -0.8 | -0.47 | -0.47 |
|  | ZSD vs K | 0 | N/A | N/A |
|  | ZSA vs K | 0 | N/A | N/A |
|  | ZSD vs DS | 0.5 | -0.5 | -0.5 |
|  | ZSA vs DS | 0 | 0 | 0 |
|  | ZSD vs ASD | 0.5 | 0.5 | 0.5 |
|  | ZSA vs ASD | 0 | -0.1 | -0.1 |
|  | ZSD vs ASA | -0.3 | 0 | 0 |
|  | ZSA vs ASA | 0.4 | 0.36 | 0.36 |
|  | ZSD vs ZSA | 0 | 0 | 0 |
| Delay scaling parameter r |  | 2.4 | 2.4 | 1.5 |
| XPR [dB] | µXPR | 8 | 4 | 4 |
|  | σXPR | 4 | 3 | 5 3 |
| Number of clusters N |  | 15 | 15 | 14 |
| Number of rays per cluster M |  | 20 | 20 | 20 |
| Cluster DS (cDS) in [ns] |  | N/A | max(0.25, 6.5622 -3.4084 log10(fc)) | max(0.25, 6.5622 -3.4084 log10(fc)) |
| Cluster ASD (cASD) in [deg] |  | FFS | 1.5 | 1.5 |
| Cluster ASA (cASA) in [deg] |  | 5 | 5 | 10 |
| Cluster ZSA (cZSA) in [deg] |  | N/A | 7 | 7 |
| Per cluster shadowing std  [dB] |  | 3 | 3 | 3 |
| Correlation distance in the horizontal plane [m] | DS | 6 | 40 | 40 |
|  | ASD | 15 | 30 | 30 |
|  | ASA | 20 | 30 | 30 |
|  | SF | 40 | 50 | 50 |
|  | K | 10 | N/A | N/A |
|  | ZSA | 15 | 50 | 50 |
|  | ZSD | 15 | 50 | 50 |
| fc is carrier frequency in GHz; d2D is BS-UT distance in km. NOTE 1: DS = rms delay spread, ASD = rms azimuth spread of departure angles, ASA = rms azimuth spread of arrival angles, ZSD = rms zenith spread of departure angles, ZSA = rms zenith spread of arrival angles, SF = shadow fading, and K = Ricean K-factor. NOTE 2: The sign of the shadow fading is defined so that positive SF means more received power at UT than predicted by the path loss model. NOTE 3: The following notation for mean (μlgX=mean{log10(X) }) and standard deviation (σlgX=std{log10(X) }) is used for logarithmized parameters X. NOTE 4: Void. NOTE 5: For all considered scenarios the AOD/AOA distributions are modelled by a wrapped Gaussian distribution, the ZOD/ZOA distributions are modelled by a Laplacian distribution and the delay distribution is modelled by an exponential distribution. |  |  |  |  |

< Unchanged text omitted >

Table 7.5-12: ZSD and ZOD offset parameters for SMa

| Scenarios |  | LOS | NLOS | O2I |
| --- | --- | --- | --- | --- |
| ZOD spread (ZSD) lgZSD=log10(ZSD/1) | lgZSD | 0.14 | 0.14 | 0.14 |
|  | lgZSD | 0.16 | 0.16 | 0.16 |
| ZOD offset | µoffset,ZOD | 0 | atanhBS-hcld2D-atanhBS-hUTd2DhUT<hcl0hUT≥hcl hcl= [3.5] |  |

Notes for Tables 7.5-7, 7.5-8, 7.5-9, 7.5-10, 7.5-11, 7.5-12:

NOTE 1:	fc is center frequency in GHz; d2D is BS-UT distance in m.

NOTE 2:	hBS and hUT are antenna heights in m for BS and UT respectively.

NOTE 3:	The following notation for mean (μlgX=mean{log10(X)}) and standard deviation (σlgX=std{log10(X)}) is used for logarithmized parameters X.

NOTE 4:	For frequencies below 6 GHz, use fc = 6 when determining the values of the frequency-dependent ZSD and ZOD offset parameters in Table 7.5-7 and 7.5-10

NOTE 5: The ZSD parameters for O2I links are the same parameters that are used for outdoor links, depending on the LOS condition of the outdoor link part in Tables 7.5-7 and 7.5-8.

## 7.6	Additional modelling components

The additional modelling components in this clause are to support advanced simulations, such as simulations with very large arrays and large bandwidth, simulations affected by oxygen absorption (frequencies between 53 and 67 GHz), simulations in which spatial consistency is important (e.g. high number of closely located user), simulations of mobility, simulations of blockage effects, simulations of absolute time of arrival, simulations of dual mobility, simulations that include sources of EM interference, simulations of devices embedded in machinery or enclosures, simulations of near-field channel effects, and simulations of spatial non-stationarity. These modelling components affect some of the steps between Step 1 and Step 12 in Clause 7.5.

These extensions are computationally more expensive and might not be required in all evaluation cases.

< Unchanged text omitted >

### 7.6.3	Spatial consistency

#### 7.6.3.1	Spatial consistency procedure

< Unchanged text omitted >

Table 7.6.3.1-2 Correlation distance for spatial consistency

| Correlation distance in [m] | RMa |  |  | UMi |  |  | UMa |  |  | SMa |  |  | Indoor | InF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | LOS | NLOS | O2I | LOS | NLOS | O2I | LOS | NLOS | O2I | LOS | NLOS | O2I |  |  |
| Cluster and ray specific random variables | 50 | 60 | 15 | 12 | 15 | 15 | 40 | 50 | 15 | 40 | 50 | 15 | 10 | 10 |
| LOS/NLOS state | 60 |  |  | 50 |  |  | 50 |  |  | 50 |  |  | 10 | dclutter2 |
| Indoor/outdoor state | 50 |  |  | 50 |  |  | 50 |  |  | 50 |  |  | N/A | N/A |

< Unchanged text omitted >

### 7.6.9	Absolute time of arrival

To support simulations in which absolute time of arrival is important, the propagation time delay due to the total path length is considered in step 11 of the fast fading model as follows. The impulse response in NLOS is determined using equation (7.6-43) instead of (7.5-27) and the impulse response in LOS is determined using equation (7.6-44) instead of (7.5-30), where c is the speed of light. Δτ is generated from a lognormal distribution with parameters according to Table 7.6.9-1. Δτ is generated independently for links between the same UT and different BS sites. For InF and InH scenarios, the excess delay in NLOS, , should further be upper bounded by 2Lc, where L is the largest dimension of the factory hall and office room, i.e. L = max(length, width, height). For UMi, UMa, SMa, and RMa scenarios, the excess delay is not upper bounded.

Hu,sNLOSτ,t=n=12i=13m∈RiHu,s,n,mNLOStδτ-τn,i-d3Dc-Δτ+n=3NHu,s,nNLOStδτ-τn-d3Dc-Δτ (7.6.-43)

Hu,sNLOSτ,t=n=12i=13m∈RiHu,s,n,mNLOStδτ-τn,i-d3Dc-Δτ+n=3NHu,s,nNLOStδτ-τn-d3Dc-Δτ

Hu,sLOSτ,t=1KR+1Hu,sNLOSτ,t+KRKR+1Hu,s,1LOStδτ-τ1-d3Dc.	(7.6-44)

where, in LOS, Hu,sNLOSτ,t=n=12i=13m∈RiHu,s,n,mNLOStδτ-τn,i-d3Dc+n=3NHu,s,nNLOStδτ-τn-d3Dc.

Table 7.6.9-1: Parameters for the absolute time of arrival model in NLOS

| Scenarios |  | InH | InF-SL, InF-DL | InF-SH, InF-DH | UMi | UMa | RMa | SMa |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| lgΔτ=log10Δτ1s |  | μlgΔτ |  | -8.6 |  | -7.5 |  | -7.5 |  | -7.5 |  | -7.4 |  | -8.33 |  | [-7.702] |
|  |  | σlgΔτ |  | 0.1 |  | 0.4 |  | 0.4 |  | 0.5 |  | 0.2 |  | 0.26 |  | [0.4] |
| Correlation distance in the horizontal plane [m] |  | 10 | 6 | 11 | 15 | 50 | 50 | [50] |

< Unchanged text omitted >

### 7.6.13	Near-field channel model

The near-field channel model is to support the simulations that involve the impacts of the spherical wavefront from the perspective of antenna array.

To model the antenna element-wise channel parameters, in the Step 11 in Section 7.5, the following updates are considered to generate the channel impulse response:

For the NLOS channel impulse response, determine the NLOS channel coefficient for the two strongest clusters, say n = 1 and 2, instead of the equation 7.5-28 and for the N – 2 weakest clusters, say n = 3, 4,…, N, using Hu,s,nNLOSt=m=1MHu,s,n,mNLOSt instead of equation 7.5-22.

-	To model the antenna element-wise phase at TRP side, the NLOS channel coefficient, Hu,s,n,mNLOSt, is given by:

Hu,s,n,mNLOSt=PnMFrx,u,θθn,m,ZOA,ϕn,m,AOAFrx,u,ϕθn,m,ZOA,ϕn,m,AOATexpjΦn,mθθkn,m-1expjΦn,mθϕkn,m-1expjΦn,mϕθexpjΦn,mϕϕFtx,s,θθn,m,ZOD,ϕn,m,AODFtx,s,ϕθn,m,ZOD,ϕn,m,AOD

expj2πd1,n,m-d1,n,m∙ rtx,n,m-dtx,sλ0expj2πrrx,n,mT∙drx,uλ0expj2πrrx,n,mT∙vλ0t	(7.6.13-1)

where, the  rtx,n,m is the spherical unit vector with azimuth departure angle and elevation departure angle for ray m of cluster n. dtx,s is the vector pointing from reference point to transmit antenna element s, wherein the reference point is the physical center of the antenna array/center at Tx side. The d1,n,m is the distance calculated as:

d1,n,m=sTRP,n*d3D+Δτ*c+τn,i*c, for the two strongest clusters, say n = 1 and 2sTRP,n*d3D+Δτ*c+τn*c,for the N – 2 weakest clusters, say n = 3, 4,…, N,

where the d3D refers to the 3D distance between reference point at TRP and UT side. The Δτ refers to the excess delay, which is only applicable in NLOS and generated according to the Section 7.6.9, otherwise Δτ is assumed to be 0.  The τn,i refers to the delay of ith sub-cluster mapping to the rays defined in Table 7.5-5. For the sTRP,n, if the nth cluster is one of the k1 strongest cluster, sTRP,n= 1, otherwise sTRP,n is generated according to the Betaα,β. The corresponding value of k1 and Beta distribution is defined in Table 7.6-13-1, respectively.

Table 7.6-13-1: Parameters for Uma, Umi, Indoor-Office and Indoor-Factory

| Scenarios |  | UMa | UMi | InH | InF |
| --- | --- | --- | --- | --- | --- |
| k1 |  | 2 | 2 | 4 | 4 |
| Beta distribution | α | 1.93 | 1.53 | 1.25 | 1.38 |
|  | β | 1.33 | 1.42 | 1.27 | 1.26 |

Optionally,

-	To model the antenna element-wise phase at UT side additionally, the Hu,s,n,mNLOSt is given by:

Hu,s,n,mNLOSt=PnMFrx,u,θθn,m,ZOA,ϕn,m,AOAFrx,u,ϕθn,m,ZOA,ϕn,m,AOATexpjΦn,mθθkn,m-1expjΦn,mθϕkn,m-1expjΦn,mϕθexpjΦn,mϕϕFtx,s,θθn,m,ZOD,ϕn,m,AODFtx,s,ϕθn,m,ZOD,ϕn,m,AOD

expj2πd2,n,m-d2,n,m∙ rrx,n,m-drx,uλ0expj2πd1,n,m-d1,n,m∙ rtx,n,m-dtx,sλ0expj2πrrx,n,mT∙vλ0t	(7.6.13-2)

where rrx,n,m is the spherical unit vector with azimuth arrival angle and elevation arrival angle for ray m of cluster n. drx,u is the vector pointing from reference point to receive antenna element u, wherein the reference point is the physical center of the antenna array/center at Rx side. The d2,n,m is the distance calculated as:

d2,n,m=sUE,n*d3D+Δτ*c+τn,i*c, for the two strongest clusters, say n = 1 and 2sUE,n*d3D+Δτ*c+τn*c,for the N – 2 weakest clusters, say n = 3, 4,…, N,

where sUE,n=1, if the nth cluster is one of the k1 strongest cluster, otherwise sUE,n =1-sTRP,n.

-	To model the antenna element-wise antenna field patterns additionally, the Hu,s,n,mNLOSt is given by:

Hu,s,n,mNLOSt=PnMFrx,u,θθn,m,ZOA,u,ϕn,m,AOA,uFrx,u,ϕθn,m,ZOA,u,ϕn,m,AOA,uTexpjΦn,mθθkn,m-1expjΦn,mθϕkn,m-1expjΦn,mϕθexpjΦn,mϕϕFtx,s,θθn,m,ZOD,s,ϕn,m,AOD,sFtx,s,ϕθn,m,ZOD,s,ϕn,m,AOD,s

expj2πd2,n-d2,n∙ rrx,n,m-drx,uλ0expj2πd1,n-d1,n∙ rtx,n,m-dtx,sλ0expj2πrrx,n,mT∙vλ0t	(7.6.13-3)

In the LOS channel impulse response, determine the LOS channel coefficient as:

-	To model the antenna element-wise phase, the equation 7.5-29 in Section 7.5 is replaced by:

Hu,s,1LOSt=Frx,u,θθLOS,ZOA,ϕLOS,AOAFrx,u,ϕθLOS,ZOA,ϕLOS,AOAT100-1Ftx,s,θθLOS,ZOD,ϕLOS,AODFtx,s,ϕθLOS,ZOD,ϕLOS,AOD

exp-j2πd3Dλ0exp-j2πru,s-d3Dλ0expj2πrrx,LOST∙vλ0t	(7.6.13-4)

where, the ru,s refers to the vector determined by the location of the uth antenna element at receiver and the sth antenna element at transmitter. The d3D refers to the 3D distance between reference point at TRP and UT side, wherein the the reference point is the physical center of the antenna array/center of the device.

Optionally,

-	To model the element-wise antenna field patterns additionally, the equation 7.5-29 in Section 7.5 is replaced by:

Hu,s,1LOSt=Frx,u,θθLOS,ZOA,u,s,ϕLOS,AOA,u,sFrx,u,ϕθLOS,ZOA,u,s,ϕLOS,AOA,u,sT100-1Ftx,s,θθLOS,ZOD,u,s,ϕLOS,AOD,u,sFtx,s,ϕθLOS,ZOD,u,s,ϕLOS,AOD,u,s

exp-j2πd3Dλ0exp-j2πru,s-d3Dλ0expj2πrrx,LOST∙vλ0t	(7.6.13-5)

where θLOS,ZOA,u,s, ϕLOS,AOA,u,s, θLOS,ZOD,u,s, ϕLOS,AOD,u,s are the respective antenna element-wise elevation arrival angles, azimuth arrival angles, elevation departure angles and azimuth departure angles of LOS path between the transmit antenna element s and receive antenna element u.

### 7.6.14	Spatial non-stationarity channel model

#### 7.6.14.1	Spatial non-stationarity channel model at TRP side

##### 7.6.14.1.1	Introduction

The spatial non-stationarity (SNS) channel model is to support the simulations that involve the impacts of the antenna element-wise power variation at TRP. Two alternative models (i.e., Physical blocker-based Model and Stochastic based Model) are provided for the SNS modelling. [Both approaches have their own use cases]

##### 7.6.14.1.2	Physical blocker-based Model

To model the antenna element-wise power variation at TRP side for each UT, the following additional steps between Step 9 and 10 in Section 7.5 are introduced to calculate the antenna element-wise power variation, i.e., for NLOS cluster,  LdB,u,n,mNLOS  if the TRP is the receiver or  LdB,s,n,mNLOS if the TRP is the transmitter, and LdB,u,s,1LOS for LOS ray.

Step I: Determine blockers

The number of blockers (K), their vertical and horizontal extensions (hkand wk), locations (xk,yk,zk) are determined according to ‘Step a’ in the Section 7.6.4.2:

Note:

-	Recommended parameters for typical blockers are provided in Table 7.6.14.1.1-1.

-	The blocking effect diminishes with increasing distance to the blocker. For implementation purposes it may be sufficient to consider only the K nearest blockers from a BS.

Table 7.6.14.1.1-1: Blocker parameters

|  | Typical set of blockers | Blocker dimensions | Mobility pattern |
| --- | --- | --- | --- |
| Indoor; Outdoor; InF | Human | Cartesian: w=0.3m; h=1.7m | Stationary or up to 3 km/h |
| Outdoor | Vehicle | Cartesian: w=4.8m; h=1.4m | Stationary or up to 100 km/h |
| InF | AGV | Cartesian: w=3m; h=1.5m | Up to 30 km/h |
| InF | Industrial robot | Cartesian: w=2m; h=0.2m | Up to 3 m/s |
| Outdoor | Billboard | Cartesian: w=2.4m; h=3.6m | Stationary |
| Outdoor | Street lamp | Cartesian: w=0.4m; h=0.8m | Stationary |
| Outdoor | Building edge | Cartesian: w=50 m; h=20 m | Stationary |
| Indoor | Pillar | Cartesian: w=0.3m; h=3m | Stationary |

Step II: Determine the antenna element-wise power variation per ray for each element

The antenna element-wise power variation caused by each blocker to the mth ray of nth cluster for each element is modelled using a simple knife edge diffraction model and is given by:

LdB,u,n,mNLOS or LdB,s,n,mNLOS or LdB,u,s,1LOS =-20log10(1-(Fh1+Fh2)(Fw1+Fw2)),-20log100.5-F, if blocker is building edge	(7.6.14.1.1-1)

where F represents one of Fh1, Fh2, Fw1, and Fw2 which account for knife edge diffraction at the four edges, and are given by equation 7.6-30 in Section 7.6.4.2 by assuming that:

-	D1h1|h2|w1|w2D1h1h2w1|w2 are the projected (onto the side and top view planes) distances between the uth antenna element at receiver and four edges of the corresponding blocker, and D2h1|h2|w1|w2 D2h1h2w1|w2 are the projected (onto the side and top view planes) distances between the sth antenna element at transmitter and four edges of the corresponding blocker.

-	The side view plane is perpendicular to the horizontal ground plane. The top view is perpendicular to the side view.

-	For each cluster and each element at BS side, the blocker screen is rotated around its centre such that the arrival (if the BS is receiver)/or departure (if the BS is transmitter) direction of the corresponding path is always perpendicular to the screen. It should be noted that different rotations are required for each individual ray and each antenna element. Meanwhile, the base and top edges of the screens are always parallel to the horizontal plane. As the screen is perpendicular to each ray, r is the distance between the uth antenna element at receiver and the sth antenna element at transmitter for the direct path in LOS, and r' is the distance between the blocker screen and the uth antenna element at receiver or the sth antenna element at transmitter, projected onto the incoming ray direction, for all the other (NLOS) paths.

Step III: Apply the antenna element-wise power variation in coefficient generation

The antenna element-wise power variation is considered in the Step 11 for coefficient generation in Section 7.5 with following:

-	For the NLOS cluster, the channel coefficients Hu,s,nNLOSt are calculated according to the equation 7.5-28 in Section 7.5 by applying the LdB,u,n,mNLOS or LdB,s,n,mNLOS.

-	In the LOS case, the LOS channel coefficient Hu,s,1LOSt is calculated according to the equation 7.5-29 in Section 7.5 by applying the LdB,u,s,1LOS for LOS path.

##### 7.6.14.1.3	Stochastic based Model

To model the antenna element-wise power variation at TRP side for each UT, the following additional steps between Step 9 and 10 in Section 7.5 are introduced to calculate the antenna element-wise power attenuation factor, i.e., αu,n if the TRP is the receiver or αs,n if the TRP is the transmitter.

Step I: Determine the invisibility condition for the nth cluster of UT

For the nth cluster, the cluster is assumed to be impacted by the invisibility if the xn<Prsns, where the xn is generated per cluster using:

xn ~ unif(0,1).	(7.6.14.1.2-1)

The Prsns is generated per UT using the truncated normal distribution,

Prsns=Nμ, σ2.	(7.6.14.1.2-2)

where 0≤Prsns≤1,which is defined in Table 7.6.14.1.2-1.

Table 7.6.14.1.2-1: Parameters for Prsns

|  | UMa |
| --- | --- |
| μ | 0.56 |
| σ | 0.20 |

Step II: Generate the visibility region (VR)

For the nth cluster which is assumed to be impacted by the invisibility, the visibility region is generated as a rectangle region with dimension a*b by using:

a~unif(Vn*W,W),	(7.6.14.1.2-3)

b=Vn*H*W/ a,	(7.6.14.1.2-4)

where H is the height of TRP antenna (i.e., in vertical direction) and W is the width of antenna (i.e., in horizontal direction). The Vn is generated as:

Vn=A*e-maxPn-PnR+Β+δ.	(7.6.14.1.2-5)

where [Pn refers to the power of cluster in section 7.5]. The parameters A, B and δ are defined in Table 7.6.14.1.2-2.

Table 7.6.14.1.2-2: Parameters for A, B, R, and δ

|  |  | UMa |
| --- | --- | --- |
| A |  | 0.15 |
| B |  | 0.45 |
| R |  | 33 |
| δ (Normal distribution) | μ | 0 |
|  | σ | 0.0015 |

Each visibility region is assumed to be randomly located at a one of the corner of TRP antenna, which is determined with following conditions:

-	The probability of VR located at either left or right part in horizontal domain of antenna array is 0.5.

-	The probability of VR located at the upper part of antenna array is [0.8] and the probability of VR at the lower part of antenna array is [0.2].

Step III: Determine the antenna element-wise power attenuation factor

For the nth cluster which is assumed not to be impacted by the invisibility, the αu,n=1 or αs,n=1.

For the nth cluster which is assumed to be impacted by the invisibility, the αu,n=αi,n or αs,n=αi,n.

αi,n=1                             ,xi-x0,n≤a,yi-y0,n≤be-xi-xa,nDn∙C        ,xi-x0,n>a,yi-y0,n≤b  e-yi-yb,nDn∙C        ,xi-x0,n≤a,yi-y0,n>b e-xi-xa,n2+yi-yb,n2Dn∙C                          ,others  	(7.6.14.1.2-6)

where, Dn=xA,n-xa,n2+yB,n-yb,n2 , xA,n, yB,n is the coordinate of the other corner of the antenna array on the diagonal with reference corner x0,n, y0,n, and xa,n, yb,n is the coordinate of the other corner of the visibility region on the diagonal with reference corner x0,n, y0,n. The xs, ys denotes the coordinate of the antenna element [out of visibility region]. C is the roll-off factor between the visible and invisible regions and defined in Table 7.6.14.1.2-3.

Table 7.6.14.1.2-3: Parameters for C

|  | UMa |
| --- | --- |
| C | 13 |

Step IV: Apply the antenna element-wise power attenuation factor in coefficient generation

The antenna element-wise power variation is considered in the Step 11 for coefficient generation in Section 7.5 with following:

-	For the NLOS cluster, the channel coefficients Hu,s,nNLOSt are calculated according to the equation 7.5-28 in Section 7.5 by applying the αu,n or αs,n.

-	In the LOS case, the LOS channel coefficient Hu,s,1LOSt is calculated according to the equation 7.5-29 in Section 7.5 by applying the αu,1 or αs,1 for LOS path.

#### 7.6.14.2	Spatial non-stationarity channel model at UT side

The spatial non-stationarity (SNS) channel model is to support the simulations that involve the impacts of the antenna element-wise power variation at UT.

The ratio of UT with impact of SNS is 90% and for each UT, the probability of each usage scenario is defined in Table 7.6.14.2-1:

Table 7.6.14.2-1: The probability of each scenario

| Usage scenario | One hand grip | Dual hand grip | Head and one hand grip |
| --- | --- | --- | --- |
| Probability | 58% | 29% | 13% |

The antenna element-wise power variation at UT side, i.e., LdB,uUT if the UT is receiver or LdB,sUT if the UT is transmitter, is defined in Table 7.6.14.2-2 for each candidate antenna location in Section 7.3.

Table 7.6.14.2-2: The power attenuation for each candidate antenna location

| Antenna index | Power attenuation (dB) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | One hand grip |  |  | Dual hand grip |  |  | Head and one hand grip |  |  |
|  | Below 1 GHz | (1-8.4) GHz | 14.5-15.5 GHz | Below 1 GHz | (1-8.4) GHz | 14.5-15.5 GHz | Below 1 GHz | (1-8.4) GHz | 14.5-15.5 GHz |
| 1 | - | 0.7 | 1.0 | - | 11.0 | 3.8 | - | 3.7 | 4.0 |
| 2 | - | 4.1 | 2.4 | - | 1.1 | 3.8 | - | 4.1 | 3.7 |
| 3 | - | 3.3 | 3.8 | - | 10.5 | 3.8 | - | 4.3 | 3.3 |
| 4 | 13.6 | 7.2 | 3.6 | 5.8 | 5.6 | 2.6 | 15.1 | 7.8 | 3.3 |
| 5 | - | 10.8 | 3.8 | - | 1.5 | 1.0 | - | 11.7 | 3.3 |
| 6 | - | 9.1 | 2.4 | - | 1.4 | 1.0 | - | 10.1 | 3.7 |
| 7 | - | 0.7 | 1.0 | - | 1.3 | 1.0 | - | 2.9 | 4.0 |
| 8 | 2.4 | 0.6 | 1.0 | 5.6 | 6.2 | 2.6 | 4.9 | 4.2 | 4.0 |
| NOTE 1: For below 1GHz, the model is only applicable for 2-antenna system with antenna location index 4 and 8. NOTE 2: The selection of number of antenna and antenna index will be determined as part of the assumption for simulation. |  |  |  |  |  |  |  |  |  |

The antenna element-wise power variation is applied in the Step 11 for coefficient generation in Section 7.5.

< Unchanged text omitted >

## 7.7	Channel models for link-level evaluations

< Unchanged text omitted >

### 7.7.5	Extension for MIMO simulations

Extended MIMO link-level channel models can be constructed according to two alternative methods described in the following.

#### 7.7.5.1	CDL extension: Scaling of angles

The angle values of CDL models are fixed, which is not very suitable for MIMO simulations for several reasons; The PMI statistics can become biased, and a fixed precoder may perform better than open-loop and on par with closed-loop or reciprocity beamforming. Furthermore, a CDL only represents a single channel realization. The predefined angle values in the CDL models can be generalized by introducing angular translation and scaling. By translation, mean angle, μϕ,model, can be changed to μϕ,desired and angular spread, ASmodel, can be changed to match ASdesired by scaling. The translated and scaled cluster angles can be obtained according to the following equation:

ϕn,m,scaled=ϕn,intermediate-μϕ,intermediate+μϕ,desired+c{ASA, ASD,ZSA,ZSD}αm	(7.7-5)

ϕLOS,scaled=ϕLOS,intermediate-μϕ,intermediate+μϕ,desired	(7.7-5a)

ϕn,intermediate=s∙WrapTo180ϕn,model-μϕ,model	(7.7-5b)

ϕLOS,intermediate=s∙WrapTo180ϕLOS,model-μϕ,model	(7.7-5c)

in which:

-	μϕ,model is the cluster mean angle calculated using Annex A.4 with ϕn,model  and ϕLOS,model as the input cluster angles,

-	μϕ,intermediate is the intermediate cluster mean angle calculated using Annex A.4 with ϕn,intermediate and ϕLOS,intermediate as input cluster angles,

-	s is a scale factor chosen to change the distribution of the angles based on calculation in Annex A.5. Table 7.7.5.1-1 shows required scale factor for typical desired angular spread values for AOD, AOD, ZOA, and ZOD,

-	ϕn,model	is the tabulated CDL cluster angle,

-	ASmodel	is the rms cluster angular spread of the tabulated CDL including the offset ray angles, calculated using the angular spread definition in Annex A.3,

-	μϕ,desired	is the desired cluster mean angle,

-	ASdesired	is the desired rms cluster angular spread,

-	ϕn,m,scaled	 is the resulting scaled ray angle that corresponds to mth subpath angle of nth cluster,

-	cASA, ASD,ZSA,ZSD is the cluster-wise rsm angular spread of either custer ASA, ASD, ZSA, or ZSD from Table 7.7.1-1, 7.7.1-2, 7.7.1-3, 7.7.1-4, and 7.7.1-5,

-	αm is the offset angles from Table 7.5-3,

-	WrapTo180⋅ is a function which wraps an azimuth angle to the half-open interval (-180, 180].

Table 7.7.5.1-1: Scale factor values for each CDL model

| CDL Type | Desired AOD Spread（°） | Scale Factor (AOD) | Desired AOA Spread（°） | Scale Factor (AOA) | Desired ZOA Spread（°） | Scale Factor (ZOA) | Desired ZOD Spread（°） | Scale Factor (ZOD) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CDL-A | 5 | 0.0680 | 30 | 0.3531 | 5 | 0.0352 | 1 | 0.0352 |
|  | 10 | 0.1360 | 45 | 0.5268 | 10 | 0.1056 | 3 | 0.1056 |
|  | 15 | 0.2041 | 60 | 0.6981 | 15 | 0.1761 | 5 | 0.1761 |
|  | 25 | 0.3405 |  |  |  |  |  |  |
| CDL-B | 5 | 0.1238 | 30 | 0.5417 | 5 | 0.1940 | 1 | 0.1940 |
|  | 10 | 0.2475 | 45 | 0.8081 | 10 | 0.5822 | 3 | 0.5822 |
|  | 15 | 0.3710 | 60 | 1.0709 | 15 | 0.9705 | 5 | 0.9705 |
|  | 25 | 0.6168 |  |  |  |  |  |  |
| CDL-C | 5 | 0.1281 | 30 | 0.4307 | 5 | 0.3643 | 1 | 0.3643 |
|  | 10 | 0.2568 | 45 | 0.6447 | 10 | 1.0929 | 3 | 1.0929 |
|  | 15 | 0.3864 | 60 | 0.8585 | 15 | 1.8219 | 5 | 1.8219 |
|  | 25 | 0.6513 |  |  |  |  |  |  |
| CDL-D | 5 | 0.3231 | 30 | 9.8888 | 5 | 0.4477 | 1 | 0.4477 |
|  | 10 | 0.6652 | 45 | N/A | 10 | 1.3469 | 3 | 1.3469 |
|  | 15 | 1.0594 | 60 | N/A | 15 | 2.2579 | 5 | 2.2579 |
|  | 25 | 5.8637 |  |  |  |  |  |  |
| CDL-E | 5 | 0.3950 | 30 | 2.9733 | 5 | 0.9714 | 1 | 0.9714 |
|  | 10 | 0.8009 | 45 | N/A | 10 | 2.9180 | 3 | 2.9180 |
|  | 15 | 1.2330 | 60 | N/A | 15 | 4.8774 | 5 | 4.8774 |
|  | 25 | 2.3627 |  |  |  |  |  |  |
| NOTE: Values of Table 7.7.5.1-1 were computed based on scaling factor calculation method described in Annex A.3. |  |  |  |  |  |  |  |  |

The angular scaling is applied on the cluster angles including offsets from the tabulated cluster angles. Typical angular spreads for different scenarios can be obtained from the system-level model.

Example scaling values are:

-	AOD spread (ASD) for each CDL model: {5, 10, 15, 25} degrees.

-	AOA spread (ASA) for each CDL model: {30, 45, 60} degrees.

-	ZOA spread (ZSA) for each CDL model: {5, 10, 15} degrees.

-	ZOD spread (ZSD) for each CDL model: {1, 3, 5} degrees.

The angular scaling and translation can be applied to some or all of the azimuth and zenith angles of departure and arrival.

ϕn,model-μϕ,model

Note: The azimuth angles may need to be wrapped around to be within [0, 360] degrees, while the zenith angles may need to be clipped to be within [0, 180] degrees.

< Unchanged text omitted >

## 7.8	Channel model calibration

### 7.8.1	Large scale calibration

For large scale calibration, fast fading is not modeled. The calibration parameters can be found in Table 7.8-1. The calibration results based on TR 38.900 V14.0.0 can be found in R1-165974.

Table 7.8-1: Simulation assumptions for large scale calibration

| Parameter | Values |
| --- | --- |
| Scenarios | UMa, UMi-Street Canyon, Indoor-office (open office) |
| Sectorization | 3 sectors per cell site: 30, 150 and 270 degrees |
| BS antenna configurations | Mg = Ng = 1; (M,N,P) = (10, 1, 1), dV = 0.5λ |
| BS port mapping | The 10 elements are mapped to a single CRS port |
| BS antenna electrical downtilting | 102 degrees for UMa and UMi Street Canyon 110 degrees for indoor |
| Antenna virtualization | DFT precoding according to TR 36.897 with application of panning and tilting angles |
| BS Tx power | 44 dBm for UMi-Street Canyon, 49 for UMa at 6GHz 35 dBm at 30GHz and 70 GHz for UMa and UMi-Street canyon 24 dBm for Indoor for all carrier frequencies |
| Bandwidth | 20MHz for 6GHz, and 100MHz for 30GHz and 70 GHz |
| UT antenna configurations | 1 element (vertically polarized), Isotropic antenna gain pattern |
| Handover margin (for calibration) | 0dB |
| UT distribution | Following TR36.873 for UMa and UMi, (3D dropping) uniform dropping for indoor with minimum distance (2D) of 0 m |
| UT attachment | Based on pathloss considering LOS angle |
| UT noise figure | 9 dB |
| Fast fading channel | Fast fading channel is not modelled |
| O2I penetration loss | 50% low loss and 50% high loss |
| Carrier Frequency | 6 GHz, 30 GHz, 70GHz |
| Wrapping method for UMa and UMi | geographical distance based wrapping (mandatory) radio distance (optional) |
| Metrics | 1) Coupling loss – serving cell (based on LOS pathloss) |
|  | 2) Geometry (based on LOS pathloss) with and without white noise |

Additional calibration parameters can be found in Table 7.8-1A. It is assumed that parameters from Table 7.8-1 is used if unspecified by the additional calibration parameters in Table 7.8-1A. In addition, calibration of UMa and UMi-Street Canyon at 6 GHz carrier frequency using simulation assumptions in Table 7.8-1 with updated channel modeling is part of the additional calibration. The calibration results based on additional calibration parameters can be found in R1-250XXXX.

Table 7.8-1A: Simulation assumptions for large scale calibration

| Parameter | Values |
| --- | --- |
| Scenarios | UMa, UMi-Street Canyon, SMa |
| Carrier Frequency | 7 GHz |
| BS antenna electrical downtilting | 95 degrees for SMa for ISD = 1299m [93] degrees for SMa for ISD = 1732m |
| BS antenna configurations | Mg = Ng = 1; (M,N,P) = (10, 1, 1), dV = 0.5λ |
| BS port mapping | Mp = 1, Np = 1, i.e., 10 elements are mapped to a single port Mp and Np are the number of vertical, horizontal TXRUs within a panel and polarization |
| BS Tx power | 49 dBm for SMa |
| Bandwidth | [20] MHz |
| UT distribution | For SMA, 20% of UT outdoor, 80% of UT indoor. Among indoor UT, 90% of indoor UT are within residential buildings, and 10% of indoor UT in commercial buildings. Indoor UTs are uniformly distributed across all floors for a building type. |
| O2I penetration loss | For SMa, [low-loss model] |
| Wrapping method | For UMa, UMi-Street Canyon, SMa: - geographical distance based wrapping (mandatory) - radio distance (optional) |

### 7.8.2	Full calibration

The calibration parameters for full calibration including the fast fading modelling can be found in Table 7.8-2. Unspecified parameters in Table 7.8-2 are the same as those in Table 7.8-1. When P=2, X-pol (+/-45 degree) is used for BS antenna configuration 1 and X-pol (0/+90 degree) is used for UT antenna configuration. The calibration results based on TR 38.900 V14.0.0 can be found in R1-165975.

Table 7.8-2: Simulation assumptions for full calibration

| Parameter | Values |  |
| --- | --- | --- |
| Scenarios | UMa, UMi-street Canyon, Indoor-office (open office) |  |
| Carrier Frequency | 6 GHz, 30 GHz, 60GHz, 70GHz |  |
| Bandwidth | 20MHz for 6GHz, and 100MHz for 30GHz, 60 GHz and 70 GHz |  |
| BS Tx power | 44 dBm for UMi-Street Canyon, 49 for UMa at 6GHz 35 dBm at 30GHz, 60 GHz and 70 GHz for UMa and UMi-Street canyon 24 dBm for Indoor for all carrier frequencies |  |
| BS antenna configurations | Config 1: M = 4,N = 4,P = 2, Mg = 1, Ng = 2, dH = dV = 0.5λ, dH,g = dV,g = 2.5λ … calibration metrics 1), 2), 3) are calibrated Config 2: Mg = Ng = 1, M = N = 2, P = 1 … calibration metrics 1), 2), 4) are calibrated |  |
| BS port mapping | Config 1: all 16 elements for each polarization on each panel are mapped to a single CRS port; panning angles of the two subarrays: (0,0) degs; same downtilt angles as used for the large-scale calibrations Config 2: each antenna element is mapped to one CRS port |  |
| UT antenna configurations | Mg = Ng = 1, M = N = 1, P = 2 |  |
| UT distribution | Following TR36.873 for UMa and UMi, (3D dropping) uniform dropping for indoor with minimum distance (2D) of 0 m |  |
| UT attachment | Based on RSRP (formula) from CRS port 0 |  |
| Polarized antenna modelling | Model-2 in TR36.873 |  |
| UT array orientation | ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 90 degree, ΩUT, = 0 degree |  |
| UT antenna pattern | Isotropic |  |
| Metrics | 1) Coupling loss – serving cell |  |
|  | 2) Wideband SIR before receiver without noise |  |
|  | 3) CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA) from the serving cell (according to circular angle spread definition of TR 25.996) |  |
|  | 4) CDF of largest (1st) PRB singular values (serving cell) at t=0 plotted in 10*log10 scale. CDF of smallest (2nd) PRB singular values (serving cell) at t=0 plotted in 10*log10 scale. CDF of the ratio between the largest PRB singular value and the smallest PRB singular value (serving cell) at t=0 plotted in 10*log10 scale. See note. |  |
| NOTE: The PRB singular values of a PRB are the eigenvalues of the mean covariance matrix in the PRB. |  |

Additional full calibration parameters can be found in Table 7.8-2A. It is assumed that parameters from Table 7.8-2 is used if unspecified by the additional full calibration parameters in Table 7.8-2A. In addition, calibration of UMa and UMi-Street Canyon at 6 GHz carrier frequency using simulation assumptions in Table 7.8-2 with updated channel modeling is part of the additional calibration. For calibration of UMa and UMi-Street Canyon at 6 GHz, the following is additionally assumed.

-	SCS of 15 kHz

-	UT attachment is based on RSRP (formula) from BS port 0

-	BS antenna configuration 1 and 2 both apply

-	UT antenna configuration, pattern, and polarization modeling are labeled as UT antenna config A

The calibration results based on additional calibration parameters can be found in R1-250XXXX.

Table 7.8-2A: Simulation assumptions for full calibration

| Parameter | Values |
| --- | --- |
| Scenarios | UMa, UMi-Street Canyon, SMa |
| Carrier Frequency | 7 GHz |
| BS antenna electrical downtilting | 95 degrees for SMa for ISD = 1299m [93] degrees for SMa for ISD = 1732m |
| BS antenna configurations | Config 3 for 7 GHz: Mg = Ng = 1, M = 8, N = 16, P = 2, dH = dV = 0.5λ … calibration metrics 1), 2), 3), 4) are calibrated (optional) Config 4 for UMa at 7 GHz: Mg = Ng = 1, M = 64, N = 16, P = 2, Mg = 1, Ng = 1, dH = dV = 0.5λ … calibration metrics 1), 2), 3), 4) are calibrated |
| BS Polarized antenna modelling | Model-2 in Clause 7.3.2 |
| BS port mapping | Config 3 for 7 GHz: Mp = 8, Np = 16, each antenna element is mapped to one port (optional) Config 4 for UMa at 7 GHz: Mp = 16, Np = 16 Mp and Np are the number of vertical, horizontal TXRUs within a panel and polarization |
| BS Tx power | 49 dBm for SMa |
| Bandwidth | [20] MHz for 7 GHz (optional) 200 MHz for 7 GHz |
| UT attachment | Based on RSRP (formula) from BS port 0 |
| UT distribution | For SMA, 20% of UT outdoor, 80% of UT indoor. Among indoor UT, 90% of indoor UT are within residential buildings, and 10% of indoor UT in commercial buildings. Indoor UTs are uniformly distributed across all floors for a building type. |
| UT array orientation | ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 45 degree, ΩUT, = 0 degree |
| UT antenna configurations | Config B for 7 GHz: 4 antenna port with single/linear polarization for calibration based on handheld device antenna model using candidate antenna locations (1,7,3,5) as described in Clause 7.3 |
| UT antenna pattern | Config B for 7 GHz: Based on directional antenna for UT described in Clause 7.3 |
| UT Polarized antenna modelling | Config B for 7 GHz: Based on directional antenna for UT described in Clause 7.3 |
| O2I penetration loss | For SMa, low-loss A model |
| SCS assumption | 30 kHz |

### 7.8.3	Calibration of additional features

The calibration parameters for the calibration of oxygen absorption, large bandwidth and large antenna array, spatial consistency, and blockage can be respectively found in Table 7.8-3, 7.8-4, 7.8-5, and 7.8-6. Unspecified parameters in these tables are the same as those in Tables 7.8-1 and 7.8-2. When P=2, X-pol (+/-45 degree) is used for BS antenna configuration 1 and X-pol (0/+90 degree) is used for UT antenna configuration. The calibration results based on TR 38.900 V14.0.0 can be found in R1-1700990.

The additional calibration parameters for the calibration of near field channel modeling, BS side spatial non-stationarity, and UT side spatial non-stationarity can be respectively found in Table 7.8-7, 7.8-8, and 7.8-9.  For the assumption on the aperture size of the antenna array, the following guidance is considered:

-	Up to 1.5 m for UMa with maximum antenna elements in the array is 5k for single Polarization.

-	Up to 1 m for UMi with maximum antenna elements in the array is 2.22k for single Polarization.

-	Up to 0.71 m for Indoor factory with maximum antenna elements in the array is 1.12k for single Polarization.

-	Up to 0.25 (for rectangular antenna array), 0.5 (for linear antenna array) m for Indoor office with maximum antenna elements in the array is 256, 80 for single Polarization, respectively.

The additional calibration results can be found in [R1-25XXXX].

Table 7.8-3: Simulation assumptions for calibration for oxygen absorption

| Parameter | Values |
| --- | --- |
| Scenarios | UMi-street Canyon |
| Carrier Frequency | 60 GHz |
| BS antenna configurations | M = 4, N = 4, P = 2, Mg = 1, Ng = 2, dH = dV = 0.5λ, dH,g = dV,g = 2.5λ |
| BS port mapping | all 16 elements for each polarization on each panel are mapped to a single CRS port; panning angles of the two subarrays: (0,0) degs; same downtilt angles as used for the large-scale calibrations |
| Calibration method | Drop multiple users in the multiple cells randomly, and collect the following metrics for each user after attachment. |
| Metrics | 1) CDF of coupling loss (serving cell) 2) Wideband SINR before receiver – determined from RSRP (formula) from CRS port 0 3) CDF of Delay Spread from the serving cell |

Table 7.8-4: Simulation assumptions for calibration for large bandwidth and large antenna array

| Parameter | Values |
| --- | --- |
| Scenarios | UMi-street Canyon |
| Carrier Frequency | 30GHz |
| Bandwidth | 2GHz |
| BS antenna configurations | M = 8, N = 8, P = 2, Mg = 1, Ng = 4, dH = dV = 0.5λ, dH,g = dV,g = 4λ |
| BS port mapping | all 64 elements for each polarization on each panel are mapped to a single CRS port; panning angles of the two subarrays: (0,0) degs; same downtilt angles as used for the large-scale calibrations |
| The number of rays per cluster | NMPC = 40 |
| Calibration method | Drop multiple users in the multiple cells randomly, and collect the following metrics for each user after attachment. |
| Metrics | 1) CDF of coupling loss (serving cell) 2) Wideband SINR before receiver – determined from RSRP (formula) from CRS port 0 3) CDF of largest (1st) PRB singular values (serving cell) at t=0 plotted in 10*log10 scale |

Table 7.8-5: Simulation assumptions for calibration for spatial consistency

| Parameter | Values |
| --- | --- |
| Scenarios | UMi-street Canyon |
| Carrier Frequency | 30 GHz |
| BS antenna configurations | M = 4, N = 4, P = 2, Mg = 1, Ng = 2, dH = dV = 0.5λ, dH,g = dV,g = 2.5λ |
| BS port mapping | all 16 elements for each polarization on each panel are mapped to a single CRS port; panning angles of the two subarrays: (0,0) degs; same downtilt angles as used for the large-scale calibrations |
| UE distribution | Following TR36.873, 3D dropping uniform dropping for indoor with minimum distance of 0 m For Config1: 100% UE indoor and in the 1st floor For Config2: 100% UE outdoor |
| Mobility | Config1: UE is stationary Config2: UE is moving with random direction and fixed speed, e.g., 30 km/h |
| Calibration method | For Config1: Drop multiple UEs in a single cell, determine all permutations of pairs of UEs, collect the variables for each pair and bin them into certain distance ranges, e.g., 1m/2m/10m, to get enough samples. Collect the following metrics 1) –6). For Config2: Drop multiple users in the single cell, and collect metric 1)-2) and 7)-9) for each user after attachment. |
| Metrics | 1) CDF of coupling loss (serving cell) 2) Wideband SINR before receiver – determined from RSRP (formula) from CRS port 0 3) Cross-correlation coefficient of delay for the third cluster between paired UEs, see note 1 4) Cross-correlation coefficient of AOA for the third cluster between paired UEs 5) Cross-correlation coefficient of LOS/NLOS status between paired UEs 6) Cross correlation coefficient of the channel response in Step 11 on the first non-DC subcarrier in an OFDM symbol on antenna port 0 received on the first UE antenna over multiple realizations between paired UEs 7) CDF of average varying rate of power for the third cluster, see note 2 8) CDF of average varying rate of delay for the third cluster (ns) 9) CDF of average varying rate of AOA for the third cluster (degree) |
| NOTE 1 For the UT pair at a certain distance range, the variables collected by two UEs can be denoted as X and Y, respectively, then the cross-correlation coefficient for real number can be written as [E(XY)-E(X)E(Y)] /sqrt([E(X^2)-E(X)^2])/ sqrt([E(Y^2)-E(Y)^2]), the cross-correlation coefficient for complex number can be written as ¦[E(XY*)-E(X)E(Y)*]/sqrt([E(XX*)-E(X)E(X)*])/sqrt([E(YY*)-E(Y)E(Y)*])¦. NOTE 2 For the average varying rate, we assume the collecting interval, e.g., 100ms, and then get the samples for a certain UE, the varying rate can be written as the standard variance of the samples per 100ms. |  |

Table 7.8-6: Simulation assumptions for calibration for blockage

| Parameter | Values |
| --- | --- |
| Scenarios | UMi-street Canyon |
| Carrier Frequency | 30 GHz |
| BS antenna configurations | M = 4, N = 4, P = 2, Mg = 1, Ng = 2, dH = dV = 0.5λ, dH,g = dV,g = 2.5λ |
| BS port mapping | all 16 elements for each polarization on each panel are mapped to a single CRS port; panning angles of the two subarrays: (0,0) degs; same downtilt angles as used for the large-scale calibrations |
| Calibration method | For Model A: Drop multiple users in the multiple cells, and collect the following metrics 1) – 3) for each user after attachment. Optional self-blocking feature is made mandatory in the Landscape mode only for calibration purposes. For Model B: Drop a BS in (0,0,30) and a UT in (100,0,1.5), Jump directly to Step 11 and replace the channel with CDL-E. Drop a blocking screen of size h=10m, w = 2m in (80,10,1.5) Move the UT from (100,0,1.5) to (100,20,1.5) in small increments. For each UT position, translate all the AODs and AOAs of CDL-E such that the specular (LOS path) of CDL-E is pointing along the direct path between the BS and UT. Collect metric 4) |
| Metrics | 1) CDF of coupling loss (serving cell) 2) Wideband SINR before receiver – determined from RSRP (formula) from CRS port 0 3) CDF of ASA from the serving cell 4) RSRP as a function of UT position |

Table 7.8-7: Simulation assumptions for calibration for near field channel modeling

| Parameter |  | Values |
| --- | --- | --- |
| Scenarios |  | UMi-street Canyon, Indoor-office (open office) |
| Carrier Frequency |  | 7 GHz (optional) 15 GHz |
| Bandwidth |  | 20 MHz |
| BS antenna height |  | UMi-street Canyon: 10m Indoor-office: 3m |
| BS antenna downtilt |  | 102 degrees for UMi-street Canyon 180 degrees for indoor (i.e., array panel downward facing) |
| BS Tx power |  | 44 dBm for UMi-Street Canyon 24 dBm for Indoor |
| BS antenna configurations and port mapping |  | For UMi scenario: Config 1 for 7 GHz (M, N, P, Mg, Ng; MP, NP) = (24, 32, 2, 1, 1; 8, 32), dH = 0.5λ, dV = 0.7λ; (optional) (M, N, P, Mg, Ng; MP, NP) = (64, 16, 2, 1, 1; 16, 16), dH = dV = 0.5λ Config 2 for 15 GHz: (M, N, P, Mg, Ng; MP, NP) = (64, 16, 2, 1, 1; 8, 16), dH = dV = 0.5λ For Indoor-office scenario for 7 GHz: (M, N, P, Mg, Ng; MP, NP) = (8, 24, 2, 1, 1; 8, 8), dH = dV = 0.5λ (optional) (M, N, P, Mg, Ng; MP, NP) = (64, 16, 2, 1, 1; 16, 16), dH = dV = 0.5λ Mp and Np are the number of vertical, horizontal TXRUs within a panel and polarization |
| BS Polarized antenna modeling |  | Model-2 in Clause 7.3.2 |
| UE Location | Outdoor/indoor | UMi-street Canyon: 100% outdoor Indoor-office: 100% indoor |
|  | LOS/NLOS | 100% LOS |
|  | UE antenna height | UMi-street Canyon: 1.5m Indoor-office: 1m |
| UT antenna configurations |  | Config A: Mg = Ng = 1, M = 2, N = 1, P = 2, dH = dV = 0.5λ Config B: 4 antenna port with single/linear polarization for calibration based on handheld device antenna model using candidate antenna locations (1,7,3,5) as described in Clause 7.3 |
| UT antenna pattern |  | Isotropic |
| UT Polarized antenna modelling |  | For UT Config A: Model-2 in Clause 7.3.2 For UT Config B: following the clause 7.3 |
| UT orientation |  | ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 45 degree, ΩUT, = 0 degree |
| Applicability of Nearfield Modeling |  | Component of NF channel to be considered: - Phase of Direct path - Phase of Non-direct path, applied only at the BS side (optionally) |
| Calibration method |  | Drop multiple users in the multiple cells at the fixed horizontal distance Z from the BS, and collect the metric for a each distance Z For UMi-street Canyon: Z = 10, 30, 50, 100 m (optional) Z = 20, 40, 80 m For Indoor: Z = 0, 2, 6, 10 m (optional) Z = 4, 8 m |
| Metric |  | CDF of the ratio between the 2nd, 3rd ,4th, ..., xth (smallest, see note 1) PRB singular value and the 1st PRB (largest) singular value (see note 2) of the serving cell at t=0 plotted in 10*log10 scale. |
| NOTE 1: The value of x is the minimum value of (number of BS antenna ports, number of UT antenna ports) NOTE 2: The PRB singular values of a PRB are the eigenvalues of the mean covariance matrix in the PRB. |  |  |

Table 7.8-8: Simulation assumptions for calibration for BS side spatial non-stationarity

| Parameter | Values |
| --- | --- |
| Scenarios | UMi-street Canyon |
| Carrier Frequency | 7 GHz (optional) 15 GHz |
| Bandwidth | 20 MHz |
| BS Antenna height | 10m for UMi-Street Canyon |
| BS Tx power | 44 dBm for UMi-Street Canyon |
| BS Antenna Configuration and port mapping | Config 1 for 7 GHz: (M, N, P, Mg, Ng; MP, NP) = (24, 32, 2, 1, 1; 8, 32), dH = 0.5λ, dV = 0.7λ; (optional) (M, N, P, Mg, Ng; MP, NP) = (64, 16, 2, 1, 1; 16, 16), dH = dV = 0.5λ Config 2 for 15 GHz: (M, N, P, Mg, Ng; MP, NP) = (64, 32, 2, 1, 1; 8, 16), dH = dV = 0.5λ Mp and Np are the number of vertical, horizontal TXRUs within a panel and polarization |
| BS Polarized antenna modeling | Model-2 in Clause 7.3.2 |
| UT Antenna Configuration | Config 1: 4 antenna port with single/linear polarization for calibration based on handheld device antenna model using candidate antenna locations (1,7,3,5) as described in Clause 7.3 |
| UT antenna pattern | Isotropic |
| UT Polarized antenna modelling | Following the Clause 7.3 |
| UT orientation | ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 45 degree, ΩUT, = 0 degree |
| UT attachment | Based on RSRP (formula) from BS port 0 |
| UT distribution | Following TR36.873 for UMi, (3D dropping) |
| Spatial non-stationarity modeling at the BS | Case 1: Spatial blocker-based approach (e.g. K = [1 per sector]) Case 2: Stochastic-based approach |
| Metric | (1) CDF of coupling loss (serving cell). 2) CDF of the ratio between the 2nd, 3rd ,4th, ..., xth (smallest, see note 1) PRB singular value and the 1st PRB (largest) singular value (see note 2) of the serving cell at t=0 plotted in 10*log10 scale. |
| NOTE 1: The value of x is the minimum value of (number of BS antenna ports, number of UT antenna ports) NOTE 2: The PRB singular values of a PRB are the eigenvalues of the mean covariance matrix in the PRB. |  |

Table 7.8-9: Simulation assumptions for calibration for UT side spatial non-stationarity

| Parameter | Values |
| --- | --- |
| Scenarios | UMi-street Canyon |
| Carrier Frequency | 7 GHz (optional) 15 GHz |
| Bandwidth | 20 MHz |
| BS Antenna height | 10m for UMi-Street Canyon |
| BS Tx power | 44 dBm for UMi-Street Canyon |
| BS Antenna Configuration and port mapping | (M, N, P, Mg, Ng; MP, NP) = (8, 16, 2, 1, 1; 8, 16), dH = dV =0.5λ Mp and Np are the number of vertical, horizontal TXRUs within a panel and polarization |
| BS Polarized antenna modeling | Model-2 in Clause 7.3.2 |
| UT Antenna Configuration | Config A for 7GHz: 4 antenna port with single/linear polarization for calibration based on handheld device antenna model using candidate antenna locations (1, 7, 3, 5) as described in Clause 7.3 Config B for 15 GHz: 16 antenna port with dual polarization based on handheld device antenna model using all candidate antenna locations (1,2,3,4,5,6,7,8) as described in Clause 7.3. |
| UT antenna pattern | Based on directional antenna for UT described in Clause 7.3 for each supported UT antenna configuration |
| UT Polarized antenna modelling | Following the Clause 7.3 |
| UT orientation | Case with blockage: - For one-hand blockage, ΩUT,α = 0 – 360 deg, ΩUT,β = 45 deg, ΩUT,γ = 0 deg, - For dual-hand blockage, ΩUT,α = 0 – 360 deg, ΩUT,β = 45 deg, ΩUT,γ = 90 deg, - For hand and head blockage, ΩUT,α = 0 – 360 deg, ΩUT,β = 90 deg, ΩUT,γ = 0 deg, Case without blockage: - ΩUT,α = 0 – 360 deg, ΩUT,β = 45 deg, ΩUT,γ = 0 deg, |
| UT attachment | Based on RSRP (formula) from BS port 0 |
| UT distribution | Following TR36.873 for UMi, (3D dropping) |
| Spatial non-stationarity modeling at the UT | Case 1: one hand grip Case 2: dual hand grip Case 3: head and one hand grip See note 3 |
| Metric | CDF of coupling loss (serving cell) received per UT port CDF of the ratio between the 2nd, 3rd ,4th, ..., xth (smallest, see note 1) PRB singular value and the 1st PRB (largest) singular value (see note 2) of the serving cell at t=0 plotted in 10*log10 scale. |
| NOTE 1: The value of x is the minimum value of (number of BS antenna ports, number of UT antenna ports) NOTE 2: The PRB singular values of a PRB are the eigenvalues of the mean covariance matrix in the PRB. NOTE 3: For each UE, the selection of case is determiend by the probability defined in Table 7.5.14-2. |  |

< Unchanged text omitted >

# Annex A:
Further parameter definitions

## A.1	Calculation of angular spread

Based on the circular standard deviation in directional statistics, the following expression for the angular spread AS in radians is proposed

AS=-2lnn=1Nm=1Mexpjφn,mPn,mn=1Nm=1MPn,m	(A-1)

where Pn,m is the power for the mth subpath of the nth path and φn,m is the subpaths angle (either AOA, AOD, ZOA, ZOD) given in radians.

## A.2	Calculation of mean angle

The power weighted mean angle is given by

μφ=argn=1Nm=1Mexpjφn,mPn,m	(A-2)

where Pn,m is the power for the mth subpath of the nth path and φn,m is the subpaths angle (either AOA, AOD, ZOA, ZOD) given in radians.

## A.3	Calculation of cluster angular spread

The cluster angular spread in degrees is given by

AS=180π-2lnexpjπ180φLOSPLOS+n=1Nexpjπ180φnPnPLOS+n=1NPn	(A-3)

where Pm is the power for the nth cluster path, φn is the cluter path angle (either AOA, AOD, ZOA, ZOD) given in degrees, PLOS is the power for the LOS path, and φLOS is the LOS path angle (either AOA, AOD, ZOA, ZOD) given in degrees. If LOS path does not exist, PLOS=0 is assumed.

## A.4	Calculation of cluster mean angle

The power weighted cluster mean angle in degrees is given by

μφ=180πargexpjπ180φLOSPLOS+n=1Nexpjπ180φnPn	(A-4)

where arg{z} is the angle in radians measured from the positive real axis to the line joining the origin and complex value z, Pm is the power for the nth cluster path, φn is the cluter path angle (either AOA, AOD, ZOA, ZOD) given in degrees, PLOS is the power for the LOS path, and φLOS is the LOS path angle (either AOA, AOD, ZOA, ZOD) given in degrees. If LOS path does not exist, PLOS=0 is assumed.

## A.5	Calculation of scaling factor for changing CDL model angular spread

The following expression for the computing scaling factor, s, to achieve a specific angular spread, AS, in degrees is given by

s=minx≥0{x: ASx=ASdesired}	(A-5)

ASx=180π-2lnexpjx∙π180WrapTo180(φLOS)PLOS+n=1Nexpjx∙π180WrapTo180(φn)PnPLOS+n=1NPn	(A-6)

where Pm is the power for the nth cluster path, φn is the input cluter path angle (either AOA, AOD, ZOA, ZOD) given in degrees, PLOS is the power for the input LOS path, and φLOS is the input LOS path angle (either AOA, AOD, ZOA, ZOD) given in degrees. If input LOS path does not exist, PLOS=0 is assumed.

< Unchanged text omitted >