3GPP TSG-RAN WG1 Meeting #121	R1-2503607

St Julian’s, Malta, May 19th – May 23rd, 2025

Agenda Item:	9.12.1

Source:	Ericsson

Title:	Multi-cell PxSCH scheduling with a single DCI

Document for:	Discussion, Decision

# 1	Introduction

In RAN plenary 106, a revised WI for Rel-19 for enhancements of multi-carrier scheduling was approved [1], with the following two main objectives:

Specify the support of the following for multi-cell PUSCH/PDSCH scheduling with a single DCI [RAN1]

- Different SCS/carrier type among co-scheduled cells by the single DCI.

- One or multiple PUSCHs/PDSCHs per scheduled cell by the single DCI.

  - The maximum number of PUSCHs/PDSCHs for a scheduled cell is 8.

  - Note: Type-1 HARQ-ACK codebook is not enhanced for Rel-19 multi-cell scheduling.

  - Note: The maximum number of sub-codebooks for Type-2 HARQ-ACK codebook is not increased for Rel-19 multi-cell scheduling.

  - Note: UE does not expect to be configured with both single-cell multi-PUSCH/PDSCH scheduling and multi-cell multi-PUSCH/PDSCH scheduling on the same or different cells within a same PUCCH group.

- Note: No new DCI format is introduced.

In the previous meetings, the normative work for this work item had a great progress and many agreements were made as listed in the Appendix. In this contribution, we discuss our view on the remaining design issues for supporting the above enhancements.

# 2	Discussions

## 2.1	Spec impact on TS38.300

For Rel-18 multi-cell PUSCH/PDSCH scheduling with a single DCI, only single PDSCH or PUSCH scheduled per scheduled cell is supported. According to the WID, the enhancement for multi-cell PUSCH/PDSCH scheduling with a single DCI in Rel-19 supports one or more PDSCH(s)/PUSCH(s) on one or more serving cells with the single DCI. In other words, more than one PDSCH/PUSCH per scheduled cell or multiple PUSCHs/PDSCHs for at least one of scheduled cells in the set needs to be supported, which has the impact in TS38.300 subclause 10.11.

In addition, only same SCS/carrier type among co-scheduled cells by a single DCI is supported in Rel-18. Therefore, the spec impact in TS38.300 subclause 10.11 also needs to be reflected.

Therefore, we propose the following:

Adopt below TP for 38.300, subclause 10.11 to reflect that multi-cell multi-PDSCHs/PUSCHs could be scheduled with different SCS/carrier type among co-scheduled cells by a single DCI.

-----------------------------Begin TP for 38.300, subclause 10.11----------------------------- ## 10.11 Multi-cell scheduling by a single DCI Multi-cell scheduling by a single DCI allows the PDCCH of a serving cell to schedule one or more PDSCH(s)/PUSCH(s) on one or more serving cells with the single DCI but with the following restrictions: - When a serving cell is configured with a PDCCH which schedules PDSCH(s)/PUSCH(s) on a cell set, the PUSCH/PDSCH on serving cells in the cell set is always scheduled by a PDCCH on the serving cell; - When SpCell is configured with a PDCCH which schedules PDSCH(s)/PUSCH(s) on serving cells in a cell set, that SpCell's PDSCH and PUSCH cannot be scheduled by a PDCCH on an SCell; - When an SCell is configured with a PDCCH which schedules PDSCH(s)/PUSCH(s) on serving cells in a cell set, SpCell is not included in the cell set; - The scheduling PDCCH and the scheduled PDSCH(s)/PUSCH(s) can use the same or different numerologies; - The co-scheduled PDSCH(s) with a PDCCH use the same or different numerologyies; - The co-scheduled PUSCH(s) with a PDCCH use the same or different numerologyies. -----------------------------End TP for 38.300, subclause 10.11-----------------------------

## 2.2	Interaction with other features (Repetition, TBoMs)

When combining Rel-18 single cell multi-PxSCH scheduling with Rel-18 DCI 0_3/1_3 multi-cell PxSCH to design the enhanced DCI 0_3/1_3, there are features that are not supported by both. In this case, it is important to decide whether these features are supported for the enhanced DCI 0_3/1_3 for ensuring proper operation.

One feature is the repetition of PUSCH/PDSCH that is not supported for multi-PxSCH TDRA and consequently is not supported for Rel-18 single cell multi-PxSCH scheduling by a DCI 0_1/1_1. However, Rel-18 DCI 0_3/1_3 multi-cell PxSCH scheduling seems to support repetition.

From our view, since multi-PxSCH TDRA is the main component to enable multi-PxSCH on a scheduled cell for enhanced DCI 0_3/1_3, it would be very complicated if repetition is supported for enhanced DCI 0_3/1_3.

The other feature is the support TB transmission over multiple slots (TBoMs). In our understanding, TBoMs is not supported for Rel-18 single cell multi-PxSCH scheduling by a DCI 0_1/1_1. However, it is supported for Rel-18 DCI 0_3/1_3 multi-cell PxSCH scheduling. Similarly to repetition case, the support of TBoMs for enhanced DCI 0_3/1_3 scheduling complicates the design.

Due to the expected additional complexity and uncertainty in convincing benefits or necessity, we propose not to support these features for the enhanced DCI 0_3/1_3.

Repetition and TB transmission over multiple slots (TBoMs) are not supported for the enhanced DCI 0_3/1_3.

## 2.3	RRC parameters for TDRA Table

In Rel-18, it was decided to not allow multi-PUSCH/multi-PDSCH on a scheduled cell by a DCI 0_3/1_3, respectively. This restriction is described by disallowing configuration of pdsch-TimeDomainAllocationListForMultiPDSCH and pusch-TimeDomainAllocationListForMultiPUSCH for DCI formats 1_3 and 0_3, as shown in Table 5.1.2.1.1-1 and Table 6.1.2.1.1-1C in TS38.214, respectively.

Based on the following agreement, the main structure of TDRA table for enabling multi-PxSCH per scheduled cell by the enhanced DCI 0_3/1_3 in Rel-19 was determined. As the maximum number of time domain resource assignments in DCI format 0_3/1_3 need further discussions, new joint TDRA tables are needed instead of reusing the existing Rel-18 joint TDRA tables. In addition, due to the above-mentioned restriction, two new TDRA tables for multi-PUSCH/multi-PDSCH on a scheduled cell by a DCI 0_3/1_3, i.e., pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 and pusch-TimeDomainAllocationListForMultiPUSCH-DCI-0-3 are needed. These two TDRAs are similar comparing to the existing configurable TDRA tables, i.e., pdsch-TimeDomainAllocationListForMultiPDSCH-r17 and pusch-TimeDomainAllocationListForMultiPUSCH-r16 for multi-PDSCH scheduling using DCI format 1_1 and multi-PUSCH scheduling using DCI format 0_1, respectively. The corresponding endorsed collection of RRC parameter can be found in [2].

Agreement RAN1#118bis - A single TDRA field in DCI format 0_3/1_3 indicates one row from a joint TDRA table. - Each row in the table contains only one TDRA index for each BWP of each cell within the set of cells. Each TDRA index points to one or multiple time domain resource allocations in the TDRA table applicable for multi-PUSCH/PDSCH scheduling by DCI format 0_3/1_3 for the corresponding cell.

The remaining issue is to determine the maximum number of time domain resource assignments in DCI format 0_3/1_3.

Moreover, in our view, UE should support the maximum schedulable PUSCHs/PDSCHs per cell, i.e., 8, by default. Apart from the supported maximum number of co-scheduled cells per set of cells similar to Rel-18 MCE, UE could indicate the supported maximum total number of schedulable PUSCHs/PDSCHs scheduled by a DCI format 0_3/1_3. As such, it is more efficient for network to configure PUSCHs/PDSCHs across all schedulable cells based on the indication from UE.

Therefore, we propose the following

Support the same maximum number of time domain resource assignments in DCI format 0_3 and 1_3 with candidate value set of {16, 32, 64}.

The max number of schedulable PUSCHs/PDSCHs across all the co-schedulable cells is configured with candidate value set of {16, 32}.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Adopt below TP for 38.300, subclause 10.11 to reflect that multi-cell multi-PDSCHs/PUSCHs could be scheduled with different SCS/carrier type among co-scheduled cells by a single DCI.

Proposal 2	Repetition and TB transmission over multiple slots (TBoMs) are not supported for the enhanced DCI 0_3/1_3.

Proposal 3	Support the same maximum number of time domain resource assignments in DCI format 0_3 and 1_3 with candidate value set of {16, 32, 64}.

Proposal 4	The max number of schedulable PUSCHs/PDSCHs across all the co-schedulable cells is configured with candidate value set of {16, 32}.

# References

RP-242904		Revised WID: Multi-carrier enhancements for NR Phase 3; Lenovo

R1-2503156		Collection of Rel-19 higher layers parameters list Post RAN1#120bis; Moderator (Ericsson).

# Appendix

In the following, the agreements and conclusions made in the previous meetings are listed.

## RAN1#118bis

Agreement

- For multiple PUSCHs/PDSCHs scheduled on a cell by a DCI format 0_3/1_3,

- Common FDRA is applied to the PUSCHs/PDSCHs on the cell as Rel-16/17 multi-PUSCH/PDSCH scheduling.

- Common MCS is applied to the PUSCHs/PDSCHs on the cell as Rel-16/17 multi-PUSCH/PDSCH scheduling.

- HARQ process number indicated for the cell is applied to the first scheduled PUSCH/PDSCH and then incremented by 1 for subsequent PUSCHs/PDSCHs on the cell (with modulo operation if needed) as Rel-16/17 multi-PUSCH/PDSCH scheduling.

Agreement

- In DCI format 0_3/1_3, for each block of NDI field, consider the following options:

  - Option 1: the number of bits is equal to the maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell by the DCI format 0_3/1_3.

  - Option 2: the number of bits is equal to the actual number of scheduled PUSCHs/PDSCHs on the corresponding cell by the DCI format 0_3/1_3.

  - Option 3: if the number of scheduled PUSCH/PDSCH is 1, then one bit NDI is applied; otherwise, option 1 is applied.

Agreement

- In DCI format 0_3/1_3, for each block of RV field, consider the following options:

  - Option 1: the number of bits is determined based on the maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell by the DCI format 0_3/1_3 and number of bits for RV configured for the corresponding cell.

  - Option 2: the number of bits is determined based on the actual number of scheduled PUSCHs/PDSCHs on the corresponding cell by the DCI format 0_3/1_3 and number of bits for RV configured for the corresponding cell.

  - Option 3: if the number of scheduled PUSCH/PDSCH is 1, then option 2 is applied; otherwise, option 1 is applied.

Agreement

- A single TDRA field in DCI format 0_3/1_3 indicates one row from a joint TDRA table.

- Each row in the table contains only one TDRA index for each BWP of each cell within the set of cells. Each TDRA index points to one or multiple time domain resource allocations in the TDRA table applicable for multi-PUSCH/PDSCH scheduling by DCI format 0_3/1_3 for the corresponding cell.

Agreement

- Time domain HARQ-ACK bundling is supported.

Agreement

- Consider at least the case that up to two different SCS can be scheduled by a DCI format 0_3/1_3 in Rel-19.

- Consider at least the following cases for scheduled cells in Rel-19:

- Case 1: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR1 licensed FDD cell(s) with SCS1 and FR1 licensed TDD cell(s) with SCS2.

  - SCS1 can be same or different to SCS2.

- Case 2: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR1 licensed FDD cell(s) with SCS1 and FR2-1 cell(s) with SCS2.

  - SCS1 can be same or different to SCS2.

- Case 3: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR1 licensed TDD cell(s) with SCS1 and FR2-1 cell(s) with SCS2.

  - SCS1 can be same or different to SCS2.

- Case 4: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR1 licensed FDD cell(s) with different SCS.

- Case 5: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR1 licensed TDD cell(s) with different SCS.

- Case 6: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR2-1 cell(s) with different SCS.

## RAN1#119

Agreement

- Time-domain HARQ-ACK bundling is configured per cell as Rel-17.

Agreement

- Specification supports the maximum number of PUSCHs/PDSCHs for a scheduled cell by a DCI format 0_3/1_3 is 8.

- Payload size of a DCI format 0_3/1_3 exceeding 140 is not supported in Rel-19.

Agreement

- Type-2 HARQ-ACK codebook is generated by concatenating a first sub-codebook and a second sub-codebook.

- The first sub-codebook comprises HARQ-ACK information bits for PDSCH(s) scheduled by DCI(s) with each scheduling a single PDSCH, or each scheduling a single cell with multiple PDSCHs on it and nrofHARQ-BundlingGroups configured as 1, and HARQ-ACK information bit(s) for DCI(s) having associated HARQ-ACK information without scheduling PDSCH reception.

- The second sub-codebook comprises HARQ-ACK information bits for PDSCHs scheduled by DCI(s) with each scheduling more than one cell, or each scheduling a single cell with multiple PDSCHs on it without nrofHARQ-BundlingGroups or nrofHARQ-BundlingGroups configured larger than 1.

- Separate DAI counting is applied for DCI(s) associated with the first sub-codebook and DCI(s) associated with the second sub-codebook as Rel-18.

- Note: For providing HARQ-ACK information corresponding to SCell dormancy indication, the UE assumes that the UE receives a PDSCH on the serving cell associated with fields in DCI format 1_3 used for SCell dormancy indication as Rel-18.

Agreement

- For the second sub-codebook, the number of HARQ-ACK information bits for each DCI format 1_3 is equal to M, to select one of the following options.

  - Option 1: M is the maximum number of HARQ-ACK information bits which can be generated for a DCI format 1_3 across all the configured cell set(s) in the PUCCH group for the UE. M is implicitly derived based on RRC configuration.

  - Option 2: M is explicitly configured by RRC parameter for the PUCCH group.

  - Other options are not precluded.

Agreement

- For NDI indication in DCI format 0_3/1_3 for TB1, further study

  - Option 1: For each block of NDI field, the number of bits is equal to the maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell by the DCI format 0_3/1_3.

    - Maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell is determined by TDRA table for the cell.

  - Option 2a: Total number of bits of the NDI field is equal to the maximum number of schedulable PUSCHs/PDSCHs by the DCI format 0_3/1_3 across all the co-schedulable cells.

    - The number of bits of a block of NDI field corresponding to a scheduled cell is equal to the actual number of the scheduled PUSCHs/PDSCHs by the DCI format 0_3/1_3 on the cell.

    - Some reserved bits are needed when the actual number of the scheduled PUSCHs/PDSCHs by the DCI format 0_3/1_3 is smaller than the maximum number of schedulable PUSCHs/PDSCHs by the DCI format 0_3/1_3 across all the co-schedulable cells.

- Other options are not precluded.

- Note: For Option 1 and 2a, DCI format 0_3/1_3 has NcellUL or NcellDL blocks of NDI field for TB1, same as in Rel-18.

Agreement

- For RV indication in DCI format 0_3/1_3 for TB1, further study

  - Option 1: For each block of RV field, the number of bits is equal to the maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell by the DCI format 0_3/1_3 and number of bits for RV configured for the cell.

    - Maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell is determined by TDRA table for the cell.

  - Option 2a: Total number of bits of the RV field is determined based on the maximum number of schedulable PUSCHs/PDSCHs by the DCI format 0_3/1_3 across all the co-schedulable cells.

    - The number of bits of a block of RV field corresponding to a scheduled cell is determined based on the actual number of the scheduled PUSCHs/PDSCHs by the DCI format 0_3/1_3 on the cell and number of bits for RV configured for the cell.

    - Some reserved bits are needed when the actual number of the scheduled PUSCHs/PDSCHs by the DCI format 0_3/1_3 is smaller than the maximum number of schedulable PUSCHs/PDSCHs by the DCI format 0_3/1_3 across all the co-schedulable cells.

- Other options are not precluded.

- Note: For Option 1 and 2a, DCI format 0_3/1_3 has NcellUL or NcellDL blocks of RV field for TB1, same as in Rel-18.

## RAN1#120

Agreement

- For determining the timing of a PUCCH carrying HARQ-ACK information corresponding to a set of co-scheduled PDSCHs by a DCI format 1_3, follow Rel-18 operation.

- If the UE is not provided subslotLengthForPUCCH, the DL slot 𝑛𝐷 is the DL slot ending last, amongst the DL slots where the more than one PDSCH are scheduled by the DCI format 1_3.

  - FFS: RAN1 spec impact till RAN1#120-bis

- If the UE is provided subslotLengthForPUCCH, no spec change is necessary.

- Note: Specification of this feature shall not impact the existing UE processing PDSCH timeline requirement for any individual PDSCH, as specified in 5.3.1 of TS38.214.

Agreement

- For the second sub-codebook, the number of HARQ-ACK information bits for each DCI format 1_3 is equal to M.

- M is the maximum number of HARQ-ACK information bits which can be generated for a DCI format 1_3 across all the configured cell set(s) in the PUCCH group for the UE. M is implicitly derived based on RRC configuration.

Agreement

- For NDI indication in DCI format 0_3/1_3 for a TB,

  - Option 1 is adopted.

Agreement

- For RV indication in DCI format 0_3/1_3 for a TB,

  - Option 1 is adopted.

Agreement

- For multi-PUSCH/PDSCH scheduling using a DCI format 0_3/1_3, 1 bit RV indication is determined according to Table 7.3.1.2.3-1 of TS 38.212.

  - Note: This is aligned with Rel-18 DCI format 0_3/1_3 for cells configured with 1 bit RV by numberOfBitsForRV-DCI-0-3/1-3.

Agreement

- For the second sub-codebook, the HARQ-ACK information bits for a DCI format 1_3 are ordered firstly according to same ordering as in Rel-17 multi-PDSCHs scheduling for PDSCH receptions on a same serving cell, then according to ascending order of associated serving cell indexes.

Agreement

- For Type-1C fields in DCI format 0_3, when the TDRA field indicates more than one scheduled PUSCHs on the scheduled cell with the smallest cell index:

  - The CSI request field applies to the PUSCH determined based on Rel-17 multi-PUSCH scheduling on same serving cell.

  - Note for background: When the TDRA field of DCI format 0_3 indicates only one scheduled PUSCH on the scheduled cell with the smallest cell index, DCI interpretation and UE procedure is same as in Rel-18.

Working Assumption

For a DCI format 1_3 with fields repurposed for SCell dormancy indication and scheduling one or more PDSCHs, if TDRA field indicates multiple SLIVs for the serving cell with smallest serving cell index with invalid FDRA, the HARQ-ACK information bit for SCell dormancy indication is ACK for the first SLIV and followed by NACK bits for the remaining SLIVs.

Agreement

- Regarding presence of UL-SCH field,

  - 1 bit UL-SCH field is always present in DCI format 0_3.

- UL-SCH field and CSI request field in a DCI format 0_3 are applied to the same PUSCH.

Agreement

- For NDI/RV indication in DCI format 0_3/1_3 for a TB,

  - Within each block of NDI field, the NDI bits are placed in the LSBs based on the SLIV position in the indicated TDRA row. Padding bits, if any, are placed in the MSBs within the block.

  - Within each block of RV field, the RV bits are placed in the LSBs based on the SLIV position in the indicated TDRA row. Padding bits, if any, are placed in the MSBs within the block.

## RAN1#120bis

Conclusion

- All bits in the NDI of TB1 and RV of TB1 corresponding to the cell with smallest serving cell index with invalid FDRA are used for SCell dormancy indication.

Conclusion

- For a DCI format 1_3 with fields repurposed for SCell dormancy indication and without scheduling any PDSCH, one bit of ACK is generated for SCell dormancy indication and included in the first sub-codebook regardless of the number of SLIVs indicated by the DCI format 1_3 for the cell with fields repurposed for SCell dormancy indication.

Agreement

- For a DCI format 1_3 with fields repurposed for SCell dormancy indication and scheduling one or more PDSCHs, if TDRA field indicates multiple SLIVs for the serving cell with smallest serving cell index with invalid FDRA and nrofHARQ-BundlingGroups is not provided for the serving cell, the HARQ-ACK information bit for SCell dormancy indication is ACK for the first SLIV and followed by NACK bits for the remaining SLIVs.

- For a DCI format 1_3 with fields repurposed for SCell dormancy indication and scheduling one or more PDSCHs, if TDRA field indicates multiple SLIVs for the serving cell with smallest serving cell index with invalid FDRA and nrofHARQ-BundlingGroups is provided for the serving cell, the HARQ-ACK information bit for SCell dormancy indication is ACK for the first TBG, and followed by NACK bits for the remaining TBGs, if any.

- Note: Related working assumption made in RAN1#120 meeting does not need to be confirmed.

Agreement

For determining the number M of HARQ-ACK information bits for each DCI format 1_3 corresponding to the second HARQ-ACK sub-codebook in a PUCCH transmission:

- M is a maximum number over all the configured cell set(s) S in a PUCCH group of the sum of NTB,cDL⋅NPDSCH,cmax (if nrofHARQ-BundlingGroups is not provided for a serving cell c) or NTB,cDL⋅NHARQ-ACK,cTBG,max (if nrofHARQ-BundlingGroups is provided for the serving cell c) across serving cells of a respective cell set S that can be co-scheduled by a DCI format 1_3;

  - NPDSCH,cmax is the maximum number of SLIVs amongst all rows of the multi-PDSCH TDRA table configured on the active BWP of the serving cell c;

  - NHARQ-ACK,cTBG,max is the maximum number of transport block groups (TBGs) for first TBs (and for second TBs, if configured) for the serving cell c if harq-ACKSpatialBundlingPUCCH is not provided, or the maximum number of PDSCH reception groups on the serving cell c if harq-ACKSpatialBundlingPUCCH is provided, and is provided by RRC parameter nrofHARQ-BundlingGroups;

  - NTB,cDL is the value of maxNrofCodeWordsScheduledByDCI for serving cell c when harq-ACKSpatialBundlingPUCCH is not provided; otherwise NTB,cDL=1.