3GPP TSG RAN WG1 #121                     	R1-2503611

St. Julians, MT, May 19th – 23rd, 2025

Agenda Item:	9.4.4

Source:	InterDigital, Inc.

Title:	Discussion on multiplexing/multiple access, scheduling information, and timing relationships

Document for:	Discussion and Decision

# Introduction

RAN approved a work item on solutions for Ambient IoT in NR in Rel-19 [2]. Ambient IoT (AIoT) is a new 3GPP IoT technology which relies on ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications[1]. The scope for this work item in Rel-19 is as follows:

This WI is to standardize RAN aspects of Ambient IoT, a new 3GPP IoT technology, suitable for deployment in a 3GPP system, which relies on ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications. The work shall provide clear differentiation, i.e. addressing use cases and scenarios that cannot otherwise be fulfilled based on existing 3GPP LPWA IoT technology e.g. NB-IoT including with reduced peak Tx power.

General Scope

The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope:

- The overall objective shall be to standardize the following Ambient IoT device:

Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally.

- Deployment scenario 1 with Topology 1, according to D1T1-B.

- FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum.

- Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor.

- Traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command).

- Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769:

  - Case 1-4 for D1T1-B

- Proximity determination via Solution 1 in TR 38.769 only.

- Device (un)availability via Direction 1 in TR 38.769 only.

Specific RAN WG1 objectives are included below [2]:

- PRDCH and PDRCH, which are the only physical channels in R2D and D2R, respectively.

- R2D and D2R signal(s)

- Multiplexing/multiple access in R2D is by only TDMA, and in D2R is by only TDMA and FDMA.

- R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations.

- R2D transmission supports only the Manchester line code in TR 38.769

- D2R transmission supports:

  - Either the Manchester line code in TR 38.769 or no line code (one to be down-selected); and

  - A corresponding small frequency shift method according to the options in TR 38.769.

- R2D does not support FEC. D2R supports only convolutional code with generator polynomials as per TS 36.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis).

- PRDCH and PDRCH both support transmission without CRC, and with CRC as per the generator polynomials in TS 38.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis) for 6-bit CRC and 16-bit CRC. Cases to use which length of CRC, or no CRC, to be decided in RAN1.

- D2R supports physical layer repetition transmission. R2D does not support physical-layer repetition transmission.

This contribution discusses InterDigital Inc.’s views on design aspects related to multiplexing, multiple-access, scheduling, and timing relationships.

# Discussion

## Multiple Access Design

### TDM Support

In RAN1#120bis, the following agreements were made regarding TDM multiplexing of Msg1 transmissions triggered by a single PRDCH trigger [3]:

Agreement

For Msg 1 transmission determined by one R2D transmission triggering random access, support X=1 and X=2 time domain resource(s) for D2R transmission(s) for Msg1 only, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s) in Rel-19.

- All devices support the above

- Note: the impact of specification support (at least including signalling overhead) for X=2 to a reader supporting only X=1 should be minimized

Only support X=1 time domain resource for D2R transmission for Msg3 in response to a PRDCH for Msg2 transmission.

Agreement

For X=1 or X=2, from device perspective, the starting time for the first Msg1 time domain resource is Toffset1 after the end of the corresponding R2D transmission triggering random access.

- FFS Toffset1 is predefined in the spec or indicated implicitly or explicitly by the R2D transmission triggering random access.

- FFS detailed value(s) of Toffset1, considering at least the device processing time including FEC impact.

- FFS: how to define the end of the R2D transmission in 9.4.1.

Working assumption

For X=2, from device perspective, for determination of the starting time for the second Msg1 time domain resource:

- Derived based on the starting time of the first Msg1 time domain resource plus Toffset2

- FFS Toffset2 is predefined in the spec or derived by a rule or indicated implicitly or explicitly by the R2D transmission triggering random access.

Agreement

Use 1 bit to indicate the value of X (X=1 or X=2) time domain resource(s) for Msg1 transmission(s).

A timing diagram is shown in Figure 1: Timing diagram for a CBRA scenario in which a device at the edge of reader coverage transmits Msg1 in the first TDM instance, and a device near the reader transmits Msg1 in the second TDM instance for a CBRA scenario in which a device at the edge of reader coverage transmits Msg1 in the first TDM instance, and a device near the reader transmits Msg1 in the second TDM instance.

The diagram indicates that the minimum value of Toffset1 need only be large enough to allow the device to receive Ms0 and prepare transmission for Msg1.  The processing time for both of these operations should be predefined according to processing latency requirements for Rel-19 A-IoT devices.

Observation 1:	The minimum value for Toffset1 can be pre-defined based on processing latency requirements for a Rel-19 A-IoT device.

As can be seen the arrival of Msg1 transmitted by DevA arrives 2⋅Tpropmax+Toffset1 after the reader transmits Msg0, where Tpropmax is the maximum propagation latency for transmission from a device to the given reader.  The arrival of Msg1 transmitted by DevB arrives 2⋅Tpropmin+Toffset1+Toffset2, where Tpropmin is the minimum propagation latency for transmission from a device to the given reader.  For a Msg1 duration of TM1 there is a probability of collision between the arriving Msg1s in each instance if:

Toffset2≥2⋅Tpropmax -Tpropmin+TM1.

The minimum propagation latency may be assumed to be zero, and for a maximum link distance 15 m in topology 1, a maximum propagation latency of should be expected to be 50 ns which would not significantly impact the timing offset.  In the case of CBRA, the transmission latency for M1 can be implicitly determined by the device from the modulation and coding information configured for Msg1 transmission by the paging message, including preamble duration, Nchippreamble, coding rate, R, and chip duration Tchip.

TM1=Nchippreamble+ N16⋅2R+  ⋅Tchip

Based on these determinations the minimum value for Toffset2 can be derived from a pre-defined value for the maximum propagation latency between a reader and a device in Rel-19, and modulation and coding information configured for Msg1 transmission.

Observation 2:	At least for CBRA, the minimum value for Toffset2 can be derived from a pre-defined value for the maximum propagation latency between a reader and device in Rel-19 and the transmission latency of Msg1, implicitly determined from modulation and coding information provided by the reader as part of the paging procedure.

Additionally, the sum of both Toffset1 and Toffset2 must not exceed maximum duration a device can support Msg1 transmission after reception for Msg0, Toffsetmax.  In this case, the maximum value of Toffset1 can be determined in the case where Toffset2 is at minimum:

Toffset1≤Toffsetmax-2⋅Tpropmax -Tpropmin-TM1.

A value for Toffsetmax may be pre-defined based on device counting and/or timing performance requirements.  For this reason, the maximum value for Toffset1 can be based on pre-defined values and implicitly determined from modulation and coding configuration for Msg1. In the case where X=1 the Toffset1≤Toffsetmax

Observation 3:	At least for CBRA:

- With X=1, the maximum value for Toffset1 can be derived from a pre-defined value for the counting/time performance requirements for a Rel-19 device.

- With X=2, the maximum value for Toffset1 can be derived from a pre-defined value for the maximum propagation latency between a reader and device in Rel-19, the transmission latency of Msg1, and the counting/timing performance requirements for a Rel-19 device and implicitly determined from modulation and coding information provided by the reader as part of the paging procedure.

Figure 1: Timing diagram for a CBRA scenario in which a device at the edge of reader coverage transmits Msg1 in the first TDM instance, and a device near the reader transmits Msg1 in the second TDM instance.

A timing diagram is shown in for a CBRA scenario in which a device at the edge of reader coverage transmits Msg1 in the second TDM instance, and a device near the reader transmits Msg1 in the first TDM instance.

As the diagram shows, a reader cannot transmit a responding Msg2 before it has completed receiving Msg1 in the second instance assuming maximum propagation latency, this implies that Msg2 cannot be transmitted earlier than 2⋅Tpropmax+Toffset1+Toffset2+TM1+TG.  Assuming a device near the reader begins to monitor for a Msg1 response immediately after transmitting Msg1, the maximum monitoring duration, Tmonitormax, the maximum supportable value for Toffset2≤Tmonitormax-TG-2⋅Tpropmax-Tpropmin, where TG may be a reader implemented guard interval to allow Tx/Rx processing and Tx/Rx switching.  Similar to device processing latency, the maximum supported monitoring window for a device, may be pre-defined based on e.g., timing synchronization requirements for the device.  Reader guard interval may be based on pre-defined processing and timing performance requirements for the reader.

Observation 4:	The maximum value for Toffset2 can be derived from pre-defined values for the maximum propagation latency between a reader and device in Rel-19, processing and timing requirements for the reader, and timing requirements for the device.

It should be noted that while in CBRA, there is no need to explicitly indicate the duration of Msg1 transmission, it may be necessary for a reader to indicate the expected duration of Msg1 transmission in each TDM instance in the case of CFRA, where the Msg1 payload may not be pre-defined.

Observation 5:	In the case of CFRA, if the Msg1 payload size for each TDM instance cannot be pre-determined, it may be necessary for the reader to signal Toffset1 and Toffset2.

Figure 2: Timing diagram for a CBRA scenario in which a device at the edge of reader coverage transmits Msg1 in the second TDM instance, and a device near the reader transmits Msg1 in the first TDM instance.

Based on the above analysis, at least for CBRA, it is possible for a device to determine a suitable value for Toffset1 and Toffset2 without explicit indication from the reader.  In the case of CFRA, it may be necessary for a reader to explicitly indicate the expected duration of Msg1 transmission. Additionally, for increased reader implementation flexibility it may be preferrable for a reader to indicate a possible range of values even for CBRA.  For these reasons, Toffset1 andToffset2 should be explicitly indicated by reader as part of an initial paging message.

Proposal 1:	In addition to the indication of X={1,2} TDMA occasions for Msg1 transmission, a reader should explicitly indicate Toffset1 and Toffset2 as part of its initial paging message.

Proposal 2:	Whether the values of Toffset1 and Toffset2 are explicitly indicated or implicitly determined, their configuration should be subject to the following constraints:

- When X= 1:

  - Tprocmax≤Toffset1≤Toffsetmax

- When X=2:

  - Tprocmax≤Toffset1≤Toffsetmax-2⋅Tpropmax -Tpropmin-TM1

  - 2⋅Tpropmax -Tpropmin+TM1≤Toffset2≤Tmonitormax-TG-2⋅Tpropmax-Tpropmin

### FDM Support

The following agreement was made in RAN1#120 regarding support of FDM for Msg3 transmissions [3]:

Agreement

Support the following option(s) for frequency domain resource for Msg3 transmission:

- Option 2: The frequency domain resource for Msg3 can be determined based on explicit indication in the PRDCH for Msg2 transmission for one or multiples devices.

- FFS: support option 1 as a default if PRDCH does not provide the indication or based on a rule

  - Option 1: the frequency domain resource for Msg3 reuses the frequency domain resource for Msg1 for the same device

As noted, increased reader flexibility is supported if the reader can indicate the available frequency resources for Msg1 transmission.  Likewise, allowing the reader to schedule Msg3 transmissions on different frequency resources may also assist in co-existing support and interference management; however, this explicit signaling can result in increased control signaling overhead, particularly in cases where all devices can transmit Msg3 on the same frequency resource as their respective Msg1 transmissions.  For this reason, the default assumption should be that when a frequency resource is not explicitly indicated for Msg3 transmission by the reader, the device assumes the Msg3 transmission uses the same frequency resource as for Msg1.

Proposal 3:	When a frequency resource for Msg3 is not explicitly indicated by the reader to a device, the device assumes the same frequency resource used for Msg1 transmission is reused for Msg3.

## PRDCH L1 Control Design

During the study item phase, the design of PRDCH with L1 control multiplexing was studied [1]:

For R2D, the only physical channel is PRDCH, which carries any higher-layer payload (including system information, if defined), and L1 R2D control information, if defined. The following design options have been studied for PRDCH:

Option 1: 	If there is no L1 R2D control information carried by PRDCH, then PRDCH transmission with only R2D data is considered.

Note: In this option, R2D control information (if any) maybe carried by higher-layer signalling.

Option 2: 	If any L1 R2D control information is supported & carried by PRDCH, then for the CRC attachment (appended if there is non-zero length CRC), following two options have been studied.

In addition, two variants of option 2 were noted relating to the design of CRC for error detection:

Option 2-1: Joint CRC is attached to L1 R2D control information and R2D data.

Option 2-2: Separate CRC is attached to L1 R2D control information and R2D data, respectively.

In general, the benefit of a separate fixed formal for L1 control signaling should be to enable more flexible configuration of PRDCH data transmission.  Configuration parameters related to things like modulation, repetition rate, and chip duration, when transmitted in L1 control enable early detection of the appropriate receiver configuration for the link adaptive portion of the PRDCH transmission.

For A-IoT there is no expectation that there should be significant link adaptation needed.  In this case, there may be of limited benefit from including a fixed format L1 control.

Observation 6:	Fixed format L1 control may not significantly increase configuration flexibility for R2D transmissions.

One potential benefit of including fixed format L1 control is a reduction in device implementation complexity, since early detection of relevant information like target device ID may enable more efficient process pipelining and reduce PRDCH reception latency.  This could also potentially reduce energy consumption from recovering transmissions not intended for the device receiving the PRDCH.

Observation 7:	Fixed format L1 control may reduce the reception latency and energy consumption for a device receiving PRDCH transmission.

Additionally, support for fixed format L1 control may allow better forward compatibility with future releases which may require more flexible configuration of PRDCH transmission.

Observation 8:	If L1 control is not included, this may limit flexibility of future designs.

For Rel-19, a single framework should be selected to limit spec impact and avoid complex device implementation.

Proposal 4:	Down-select between fixed format L1 control multiplexed with PRDCH data, or a single PRDCH transmission with higher-layer control and data multiplexed together.

The following agreement was made in RAN1#120 regarding the indication of payload size for PDRCH transmission [3]:

Agreement

The payload size (i.e. TBS-like) for PDRCH transmission with variable size is explicitly indicated in the corresponding R2D control information.

While a reader may indicate its expectation that a device should transmit a PDRCH of a certain size, it may not always be feasible for a device to fulfill this request.  High on PRDCH receive processing latency and PDRCH transmission processing latency may result in a situation where the device is not capable of transmitting the indicated message in the time domain resources indicated for PDRCH transmission.  A device may zero-pad its PDRCH transmission in these cases, but this can result in inefficient spectrum use where significant resources are allocated for null data transmission. In these cases, it should be possible for the device to indicate to the reader that the request transmission may not be fulfilled with limited overhead.

Proposal 5:	In scenarios when a device cannot transmit a PDRCH as indicated by the reader, the device can indicate its inability in place of a PDRCH transmission.

# Conclusion

The following observations are provided for discussion in RAN1#120bis:

Observation 1:	The minimum value for Toffset1 can be pre-defined based on processing latency requirements for a Rel-19 A-IoT device.

Observation 2:	At least for CBRA, the minimum value for Toffset2 can be derived from a pre-defined value for the maximum propagation latency between a reader and device in Rel-19 and the transmission latency of Msg1, implicitly determined from modulation and coding information provided by the reader as part of the paging procedure.

Observation 3:	At least for CBRA:

- With X=1, the maximum value for Toffset1 can be derived from a pre-defined value for the counting/time performance requirements for a Rel-19 device.

- With X=2, the maximum value for Toffset1 can be derived from a pre-defined value for the maximum propagation latency between a reader and device in Rel-19, the transmission latency of Msg1, and the counting/timing performance requirements for a Rel-19 device and implicitly determined from modulation and coding information provided by the reader as part of the paging procedure.

Observation 4:	The maximum value for Toffset2 can be derived from pre-defined values for the maximum propagation latency between a reader and device in Rel-19, processing and timing requirements for the reader, and timing requirements for the device.

Observation 5:	In the case of CFRA, if the Msg1 payload size for each TDM instance cannot be pre-determined, it may be necessary for the reader to signal Toffset1 and Toffset2.

Observation 6:	Fixed format L1 control may not significantly increase configuration flexibility for R2D transmissions.

Observation 7:	Fixed format L1 control may reduce the reception latency and energy consumption for a device receiving PRDCH transmission.

Observation 8:	If L1 control is not included, this may limit flexibility of future designs.

The following proposals are provided for discussion in RAN1#120bis:

Proposal 1:	In addition to the indication of X={1,2} TDMA occasions for Msg1 transmission, a reader should explicitly indicate Toffset1 and Toffset2 as part of its initial paging message.

Proposal 2:	Whether the values of Toffset1 and Toffset2 are explicitly indicated or implicitly determined, their configuration should be subject to the following constraints:

- When X= 1:

  - Tprocmax≤Toffset1≤Toffsetmax

- When X=2:

  - Tprocmax≤Toffset1≤Toffsetmax-2⋅Tpropmax -Tpropmin-TM1

  - 2⋅Tpropmax -Tpropmin+TM1≤Toffset2≤Tmonitormax-TG-2⋅Tpropmax-Tpropmin

Proposal 3:	When a frequency resource for Msg3 is not explicitly indicated by the reader to a device, the device assumes the same frequency resource used for Msg1 transmission is reused for Msg3.

Proposal 4:	Down-select between fixed format L1 control multiplexed with PRDCH data, or a single PRDCH transmission with higher-layer control and data multiplexed together.

Proposal 5:	In scenarios when a device cannot transmit a PDRCH as indicated by the reader, the device can indicate its inability in place of a PDRCH transmission.

# References

R1-2410929,	“Study on solutions for ambient IoT (Internet of Things) (Rel-19)” v2.0.0

RP-243327,	“New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei)

R1-2501357	FL summary #3 on other aspects for Rel-19 Ambient IoT	Moderator (vivo)

R1-2502500	FL summary #3 on other aspects for Rel-19 Ambient IoT	Moderator (vivo)