3GPP TSG-RAN WG1 Meeting #121	R1-2503752

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	9.7.1

Source:	InterDigital, Inc.

Title:	Discussion on ISAC deployment scenarios

Document for:	Discussion and Decision

# Introduction

In RAN#102, a SID to study channel models for Integrated Sensing and Communication (ISAC) [1] in preparation for Rel. 19 was approved, defining the scope, encompassing various use case types, sensing targets, sensing modes and frequencies.

In the previous RAN1 meeting #120b [2] and the email discussions post-RAN1 #120b [3], the companies agreed on the large-scale and full-scale calibration parameters for different sensing scenarios and targets. In this contribution, we provide our inputs for the remaining aspects of ISAC deployment scenarios, channel calibration and the calibration results.

# ISAC channel calibration

Spatial consistency for UAV:

In the email discussions, RAN1 agreed on spatial consistency calibration parameters for urban grid and indoor office scenarios for automotive and human sensing targets respectively. In the following table, we propose the spatial consistency parameters for UAV sensing target calibration.

Table 1 Parameters for spatial consistency for UAV

| Parameter | Values |
| --- | --- |
| Scenario | UMa-AV |
| Cell layout | Hexagonal grid, 19 macro sites, 3 sectors per site (ISD = 500m) (refer to TR 36.777) |
| Sensing mode | TRP-UE bistatic |
| BS antenna configuration | Single dual-pol isotropic antenna |
| UT antenna configuration | (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1) |
| Polarized antenna model | Model-2 in clause 7.3.2 in TR 38.901 |
| Scattering point | Single point |
| Calibration link and calibration method | - Calibration link: same target associated to different UT as following figure Figure 1 Distance groups described in [3] - Calibration method Step1: In single drop simulation, drop one Target and multiple UTs. Step2: Generate the channel of each link based on Target level spatial consistency method. Step3: Determine Target-UT groups based on the 3D distance between UTs. Step4: Bin all the link pairs into certain distance groups, e.g., - 0m <= the location distance of link pair < 1m -> 0m group - 1m <= the location distance of link pair < 2m -> 1m group - 2m <= the location distance of link pair < 3m -> 2m group - 3m <= the location distance of link pair < 4m -> 3m group …… - (N)m <= the location distance of pair < (N+1)m -> (N)m group Note: N is at least twice the maximum correlation distance associated with the channel parameters in the scenario Step5: Calculate the correlation coefficient metric of each distance groups. Step6: Draw the x-y cure, where x=0, …, N, y= correlation coefficient metric related to (x)m group |
| ST and UT distribution | 1 target uniformly distributed (across multiple drops) within the center cell. Vertical distribution: Fixed height value of 200 m. The overall number of UTs is 10 uniformly distributed in the center cell. All of the UTs are terrestrial UTs, all outdoors. |
| Metrics | - Cross-correlation coefficient of delay for the third cluster of channel between link pairs. - Cross-correlation coefficient of ZoA and AoA (for UT) for the third cluster of channel between link pairs. - Cross-correlation coefficient of LOS/NLOS status of channel between link pairs. |
| Note: Other necessary but unspecified parameters in this table are the same as those in the table for UMa-AV scenario calibration. Note : Spatial consistency is not modelled between TRP-target link and target -UT link |  |

Proposal 1: Use Table 1 for the spatial consistency calibration for UMa-AV deployment scenario for UAV sensing.

# Large scale calibration results

In this section, CDF of coupling loss for sensing target channel and background channel is generated. Large scale calibration parameters follow the agreed values shown in Appendix for UAV, human and AGV as targets. In Figure 1 to figure 14, coupling loss results for different frequency range, targets and sensing modes are presented.

Figure 1: CDF of coupling loss of target channel for human sensing target, Indoor office scenario (FR1, TRP UE bistatic)

Figure 2: CDF of coupling loss of target channel for human sensing target, Indoor office scenario (FR2, TRP UE bistatic)

Figure 3: CDF of coupling loss of target channel for AGV target, InF scenario (FR1, TRP UE bistatic)

Figure 4: CDF of coupling loss of target channel for AGV target, InF scenario (FR2, TRP UE bistatic)

Figure 5: CDF of coupling loss of target channel for UAV target, Uma-AV scenario (FR1, TRP bistatic)

Figure 6: CDF of coupling loss of target channel for UAV target, Uma-AV scenario (FR1, TRP monostatic)

Figure 7: CDF of coupling loss of target channel for UAV target, Uma-AV scenario (FR2, TRP bistatic)

Figure 8: CDF of coupling loss of target channel for UAV target, Uma-AV scenario (FR2, TRP monostatic)

Figure 9: CDF of coupling loss of target channel for UAV target, Uma-AV scenario (FR1, TRP- (aerial) UE bistatic)

Figure 10: CDF of coupling loss of target channel for UAV target, Uma-AV scenario (FR2, TRP - (aerial) UE bistatic)

Figure 11: CDF of coupling loss of target channel for UAV target, Uma-AV scenario (FR1, TRP- (terrestrial) UE bistatic)

Figure 12: CDF of coupling loss of target channel for UAV target, Uma-AV scenario (FR2, TRP - (terrestrial) UE bistatic)

Figure 13: CDF of coupling loss of background channel for UAV target, Uma-AV scenario (FR1, TRP monostatic)

Figure 14: CDF of coupling loss of background channel for UAV target, Uma-AV scenario (FR2, TRP monostatic)

# Remaining aspects of deployment scenario

In this section, we discuss some remaining topics for deployment scenario parameters.

Micro-Doppler Characteristics:

In the SID [1], the primary goal of the ISAC channel modelling framework is defined to be able to differentiate between sensing targets and unintended objects or EOs. One approach to achieve this differentiation is based on the reflection characteristics of the objects, which are captured by their Radar Cross Section (RCS).

However, the RCS of an object is influenced by a variety of factors, including:

- Frequency/wavelength of transmitted signal

- Angle of incidence and scattering to the target object

- Shape of target object

- Size of target object

- Material property of target object

- Mobility of target object

- Distance between Tx/Rx and object

For EOs, particularly EO type-1, which are defined as having physical characteristics comparable to those of a sensing target, distinguishing between objects based on RCS can be challenging due to their similar physical attributes.

Micro-Doppler characteristics captures the variations in the phase of the received reference signal due to the object's motion characteristics including vibrations, rotation, movement, etc. [4]. These variations can be unique for different objects considering the unique motion characteristics of the objects. Proposals related to micro-Doppler function are shown in our companion contribution [5].

Observation 1: Micro-Doppler signatures can be used to distinguish between sensing targets and the unintended targets

Proposal 2: Incorporate the micro-Doppler signatures of the sensing targets, unintended targets for different deployment scenarios.

# Conclusion

In this contribution, we discussed the calibration parameters and results for scenarios associated with different sensing targets. Based on the discussion, we made the following observations and proposals:

Observation 1: Micro-Doppler signatures can be used to distinguish between sensing targets and the unintended targets

Proposal 1: Use Table 1 for the spatial consistency calibration for UMa-AV deployment scenario for UAV sensing

Table 1 Parameters for spatial consistency for UAV

| Parameter | Values |
| --- | --- |
| Scenario | UMa-AV |
| Cell layout | Hexagonal grid, 19 macro sites, 3 sectors per site (ISD = 500m) (refer to TR 36.777) |
| Sensing mode | TRP-UE bistatic |
| BS antenna configuration | Single dual-pol isotropic antenna |
| UT antenna configuration | (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1) |
| Polarized antenna model | Model-2 in clause 7.3.2 in TR 38.901 |
| Scattering point | Single point |
| Calibration link and calibration method | - Calibration link: same target associated to different UT as following figure Distance groups described in [3] - Calibration method Step1: In single drop simulation, drop one Target and multiple UTs. Step2: Generate the channel of each link based on Target level spatial consistency method. Step3: Determine Target-UT groups based on the 3D distance between UTs. Step4: Bin all the link pairs into certain distance groups, e.g., - 0m <= the location distance of link pair < 1m -> 0m group - 1m <= the location distance of link pair < 2m -> 1m group - 2m <= the location distance of link pair < 3m -> 2m group - 3m <= the location distance of link pair < 4m -> 3m group …… - (N)m <= the location distance of pair < (N+1)m -> (N)m group Note: N is at least twice the maximum correlation distance associated with the channel parameters in the scenario Step5: Calculate the correlation coefficient metric of each distance groups. Step6: Draw the x-y cure, where x=0, …, N, y= correlation coefficient metric related to (x)m group |
| ST and UT distribution | 1 target uniformly distributed (across multiple drops) within the center cell. Vertical distribution: Fixed height value of 200 m. The overall number of UTs is 10 uniformly distributed in the center cell. All of the UTs are terrestrial UTs, all outdoors. |
| Metrics | - Cross-correlation coefficient of delay for the third cluster of channel between link pairs. - Cross-correlation coefficient of ZoA and AoA (for UT) for the third cluster of channel between link pairs. - Cross-correlation coefficient of LOS/NLOS status of channel between link pairs. |
| Note: Other necessary but unspecified parameters in this table are the same as those in the table for UMa-AV scenario calibration. Note : Spatial consistency is not modelled between TRP-target link and target -UT link |  |

Proposal 2: Incorporate the micro-Doppler signatures of the sensing targets, unintended targets for different deployment scenarios.

# References

[1] RP-234069, “New SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR”, Nokia

[2] 3GPP Chairman’s note, RAN1#120b, Wuhan, China, April 2025

[3] R1-2503150, “Email summary on Post-120bis-ISAC-01”, Moderator (AT&T), Wuhan, China, April 2025

[4] C. Gu, T. Inoue and C. Li, "Analysis and Experiment on the Modulation Sensitivity of Doppler Radar Vibration Measurement," in IEEE Microwave and Wireless Components Letters, vol. 23, no. 10, pp. 566-568, Oct. 2013, doi: 10.1109/LMWC.2013.2250269.

[5] R1-2503753, “Discussion on ISAC channel modelling, ” RAN1#121, St. Julian’s, Malta, May, 2025.

# Appendix

During the previous RAN1 meeting #120b [2], the following agreements were reached concerning the deployment parameters for ISAC channel modelling.

Agreement For the purposes of large scale calibration for UAV sensing targets, the following revised calibration parameters are proposed below in Table x. Note that the change bars are against the agreements from RAN1#120. Table x. Simulation assumptions for large scale calibration for UAV sensing targets Parameters; Values. Scenario; UMa-AV. Sensing mode; TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic. Target type; UAV of small size (0.3m x 0.4m x 0.2m). Sectorization; Single 360-degree sector can be assumed. Carrier Frequency; FR1: 6 GHz FR2: 30 GHz. BS antenna configurations; Single dual-pol isotropic antenna. BS Tx power; FR1: 56dBm FR2: 41dBm. Bandwidth; FR1: 100MHz FR2: 400MHz. BS noise figure; FR1: 5dB FR2: 7dB. UT antenna configurations; Single dual-pol isotropic antenna; (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1). UT noise figure; FR1: 9dB FR2: 10dB. UT height; 1.5m for terrestrial UTs,. UT Tx power; 23dBm. UT Distribution; • The overall number of UTs is 30 uniformly distributed in the center cell. • All of the UTs are either terrestrial UTs or aerial UTs, all outdoors. • Vertical distribution of aerial UE: Fixed height value of 200 m. • FR1 is assumed for aerial UE.. Sensing target distribution; 1 target uniformly distributed (across multiple drops) within the center cell. Vertical distribution: Fixed height value of 200 m.. Component A of the RCS for each scattering point; -12.81 dBsm. Minimum 3D distances between pairs of Tx/Rx and sensing target; 10 m. Wrapping Method; No wrapping method is used if interference is not modelled, otherwise geographical distance based wrapping. Coupling loss for target channel; power scaling factor (pathloss, shadow fading, and RCS component A included): LTX-SPST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2. Sensing Tx/Rx selection; Best N = 4 Tx-Rx pairs to be selected for the target. NOTE1: Based on the Tx-Rx pairs with the smallest power scaling factor of the target channel.. Metrics; Coupling loss for target channel Coupling loss for background channel (in case of monostatic sensing, this is the coupling loss between Tx and one reference point) Note: CDFs can be separately generated for target channel, background channel. Agreement For the purposes of full calibration for UAV sensing targets, the following calibration parameters are proposed below in Table x. Table x. Simulation assumptions for full calibration for UAV sensing targets Parameters; Values. Scenario; UMa-AV. Sensing mode; TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic. Target type; UAV of small size (0.3m x 0.4m x 0.2m). Sectorization; Single 360-degree sector can be assumed. Carrier Frequency; FR1: 6 GHz FR2: 30 GHz. BS antenna configurations; Single dual-pol isotropic antenna. BS Tx power; FR1: 56dBm FR2: 41dBm. Bandwidth; FR1: 100MHz FR2: 400MHz. BS noise figure; FR1: 5dB FR2: 7dB. UT antenna configurations; Single dual-pol isotropic antenna; (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1). UT noise figure; FR1: 9dB FR2: 10dB. UT height; 1.5m for terrestrial UTs. UT Tx power; 23dBm. UT Distribution; - The overall number of UTs is 30 uniformly distributed in the center cell. - All of the UTs are either terrestrial UTs or aerial UTs, all outdoors. - Vertical distribution of aerial UE: Fixed height value of 200 m. - FR1 is assumed for aerial UE.. Sensing target distribution; 1 target uniformly distributed (across multiple drops) within the center cell. Vertical distribution: Fixed height value of 200 m.. RCS for each scattering point; Component A: -12.81 dBsm Component B1: 0 dB Component B2: 3.74 dB for standard deviation The same values are used for monostatic RCS and bistatic RCS. Minimum 3D distances between pairs of Tx/Rx and sensing target; 10 m. Wrapping Method; No wrapping method is used if interference is not modelled, otherwise geographical distance based wrapping. Fast fading model; TR 36.777 Annex B.1.3. (u, std) for XPR of target; Mean 13.75 dB, deviation 7.07 dB. The power threshold for path dropping after concatenation for target channel; FFS. The power threshold for removing clusters in step 6 in section 7.5, TR 38.901 for background channel; FFS. Coupling loss for target channel; By definition, need to consider all direct and indirect paths. The following parameters are included in the calculation: - power scaling factor (pathloss, shadow fading, and RCS component A included) - for small scale RCS B1/B2 and power of rays in Tx-target/target-Rx links (Pn',m',n,mk,p), Tx/Rx antenna pattern, 3 polarization matrixes, i.e., Pn',m',n,mk,pFrx,u,θθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pFrx,u,ϕθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pTCPMrx,n',m'k,pCPMn',m',n,mk,pCPMtx,n, mk,p ∙Ftx,s,θθtx,n,m,ZODk,p,ϕtx,n,m,AODk,pFtx,s,ϕθtx,n,m,ZODk,p,ϕtx,n,m,AODk,p. Sensing Tx/Rx selection; Best N = 4 Tx-Rx pairs to be selected for the target. NOTE1: Based on the Tx-Rx pairs with the smallest power scaling factor of the target channel.. Absolute delay; The model of UMa scenario defined in TR 38.901 7-24GHz channel modeling [ref] is reused for UMa-AV for all sensing modes.. Metrics; Coupling loss for target channel Coupling loss for background channel (in case of monostatic sensing, this is the linear sum of coupling losses between Tx/Rx and all reference points) Note: CDFs can be separately generated for target channel, background channel CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA). Definition of Delay Spread is similar to the definition of angle spread in Annex A of TR 25.996, Definition of Angle Spread can ref to Annex A of TR 25.996.. Agreement For the purposes of large scale calibrations for Automotive sensing targets, the following parameters are proposed below in Table x. - FFS: which type of UE is used for UT in different sensing mode - FFS: impact of spatial consistency, if any, in case of vehicle with 5 scattering points - FFS: cell layout for ISD = 250 m Table x. Simulation assumptions for large scale calibration for Automotive sensing targets Parameters; Values. Scenario; For FR1: Urban Grid (ISD=500m, BS height=25m) Highway (ISD=1732m, BS height=35m) For FR2: Urban Grid (ISD=250m, BS height=25m) Highway (ISD=500m, BS height=35m). Sensing mode; TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic, UE monostatic. Target type; Vehicle type 2 [TR37.885]. Sectorization; Single 360-degree sector can be assumed. Carrier Frequency; FR1: 6 GHz FR2: 30 GHz. BS antenna configurations; Single dual-pol isotropic antenna. BS Tx power; FR1: 56dBm FR2: 41dBm. Bandwidth; FR1: 100MHz FR2: 400MHz. BS noise figure; FR1: 5dB FR2: 7dB. UT antenna configurations; Single dual-pol isotropic antenna, (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1). UT noise figure; FR1: 9dB FR2: 10dB. UT height; 1.5m for pedestrian type UE 5m for RSU type UE 1.6m for vehicle type UE. UT Tx power; 23dBm. UT Distribution; Per TR37.885. Sensing target distribution; Per TR37.885: - Option A - Vehicle type distribution: 100% vehicle type 2. - Clustered dropping is not used. - Highway: one target uniformly distributed (across multiple drops) within the simulation region. Vehicle speed is 140 km/h in all the lanes as baseline. - Urban Grid: one target is uniformly distributed (across multiple drops) within the center road grid. Vehicle speed is 60 km/h in all the lanes as baseline. NOTE: vehicle is dropped with 5 scattering points (front/left/right/back/roof) and each point has one location, or vehicle is dropped with 1 scattering points. Component A of the RCS for each scattering point; -20dBsm. Minimum 3D distances between pairs of Tx/Rx and sensing target; 10 m. Wrapping Method; As defined in urban grid/highway scenario. Coupling loss for target channel; Power scaling factor (pathloss, shadow fading, and RCS component A included) LTX-SPST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2. Sensing Tx/Rx selection; Best N= Tx-Rx pairs to be selected for the target. For urban grid N = 4 For Highway N = 4 NOTE: Based on the Tx-Rx pair with the smallest power scaling factor of the target channel.. Metrics; Coupling loss for target channel Coupling loss for background channel (in case of monostatic sensing, this is the coupling loss between Tx and one reference point) Note: CDFs can be separately generated for target channel, background channel.

Additionally, during the email discussions post RAN 120b [3], following agreements were endorsed.

- Proposal 4-2-2 Proposal 4-2-2: For the purposes of large scale calibrations for Automotive sensing targets, the following parameters are updated below in Table x based on the agreements in RAN1#120-bis. Table x. Simulation assumptions for large scale calibration for Automotive sensing targets Parameters; Values. Scenario; For FR1: Urban Grid (ISD=500m, BS height=25m) Highway (ISD=1732m, BS height=35m) For FR2: Urban Grid (ISD=250m, BS height=25m) Highway (ISD=500m, BS height=35m) For Urban Grid ISD =250m - The layout is configured as follows: - Red triangles: BS with 250m ISD, 18 BSs are located.. Sensing mode; TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic, UE monostatic. Target type; Vehicle type 2 [TR37.885]. Sectorization; Single 360-degree sector can be assumed. Carrier Frequency; FR1: 6 GHz FR2: 30 GHz. BS antenna configurations; Single dual-pol isotropic antenna. BS Tx power; FR1: 56dBm FR2: 41dBm. Bandwidth; FR1: 100MHz FR2: 400MHz. BS noise figure; FR1: 5dB FR2: 7dB. UT antenna configurations; Single dual-pol isotropic antenna, (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1). UT noise figure; FR1: 9dB FR2: 10dB. UT height; 1.5m for pedestrian type UE 5m for RSU type UE 1.6m for vehicle type UE. UT Tx power; 23dBm. UT Distribution; For Highway: - Vehicle Type UT distribution follows vehicle UE dropping as in Table A.1.2-1 from TR36.885. - RSU-type UTs are uniformly allocated with 100m spacing in the middle of the freeway as per TR36.885. For Urban grid: - Vehicle Type UT distribution follows vehicle UE dropping as in Table A.1.2-1 from TR36.885. - Pedestrian type UT, the dropping using equally spaced along the sidewalk with a fixed inter-pedestrian X m dropped per TR36.885 - Total number of pedestrian UEs is 16 in the centre grid. - Pedestrian UE is in the middle of the sidewalk - The inter-pedestrian UE distance (m) (i.e., X) is calculated by ‘A/16’, where ‘A’ is the total length of sidewalk where the pedestrian UEs are dropped under the assumption of ‘N’ road grids (i.e., ‘{(250m – 17m) + (433m – 17m)} * 2 * N’). For example, if the pedestrian UEs are dropped in ‘14’ road grids, the inter-pedestrian UE distance (m) is ‘36.344’. - N=1; - RSU-type UT: the dropping is at the center of intersection per TR36.885. NOTE: A single UT type is used per calibration, e.g., pedestrian type UT, RSU type UT, or vehicle type UT. Sensing target distribution; Per TR37.885: - Option A - Vehicle type distribution: 100% vehicle type 2. - Clustered dropping is not used. - Highway: one target uniformly distributed (across multiple drops) within the simulation region. Vehicle speed is 140 km/h in all the lanes as baseline. - Urban Grid: one target is uniformly distributed (across multiple drops) within the center road grid. Vehicle speed is 60 km/h in all the lanes as baseline. NOTE: vehicle is dropped with 5 scattering points (front/left/right/back/roof) and each point has one location, or vehicle is dropped with 1 scattering points. In the case of vehicle with 5 scattering points, spatial consistency is enabled with the following assumptions: - The correlation for LOS/NLOS condition of the 5 points is assumed equal to 1. LOS/NLOS condition can be calculated based on the distance of the Tx/Rx to the centroid of the ST, then apply the LOS/NLOS condition to each of the 5 points. - The correlation for stochastic cluster paths of the 5 points is assumed equal to 1. The stochastic cluster paths can be calculated between the Tx/Rx and the centroid of the ST, then the stochastic cluster paths are added to each of the 5 points.. Component A of the RCS for each scattering point; -20dBsm. Minimum 3D distances between pairs of Tx/Rx and sensing target; 10 m. Wrapping Method; As defined in urban grid/highway scenario. Coupling loss for target channel; Power scaling factor (pathloss, shadow fading, and RCS component A included) LTX-SPST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2. Sensing Tx/Rx selection; Best N= Tx-Rx pairs to be selected for the target. For urban grid N = 4 For Highway N = 4 NOTE: Based on the Tx-Rx pair with the smallest power scaling factor of the target channel.. Metrics; Coupling loss for target channel Coupling loss for background channel (in case of monostatic sensing, this is the coupling loss between Tx and one reference point) Note: CDFs can be separately generated for target channel, background channel. - Proposal 4-3-2 Proposal 4-3-2: For the purposes of full calibrations for Automotive sensing targets, the following parameters are proposed below in Table x. Table x. Simulation assumptions for full calibration for Automotive sensing targets Parameters; Values. Scenario; For FR1: Urban Grid (ISD=500m, BS height=25m) Highway (ISD=1732m, BS height=35m) For FR2: Urban Grid (ISD=250m, BS height=25m) Highway (ISD=500m, BS height=35m) For Urban Grid ISD =250m - The layout is configured as follows: - Red triangle: BS with 250m ISD, 18 BSs are located.. Sensing mode; TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic, UE monostatic. Target type; Vehicle type 2 [TR37.885]. Sectorization; Single 360-degree sector can be assumed. Carrier Frequency; FR1: 6 GHz FR2: 30 GHz. BS antenna configurations; Single dual-pol isotropic antenna. BS Tx power; FR1: 56dBm FR2: 41dBm. Bandwidth; FR1: 100MHz FR2: 400MHz. BS noise figure; FR1: 5dB FR2: 7dB. UT antenna configurations; Single dual-pol isotropic antenna, (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1). UT noise figure; FR1: 9dB FR2: 10dB. UT height; 1.5m for pedestrian type UE 5m for RSU type UE 1.6m for vehicle type UE. UT Tx power; 23dBm. UT Distribution; For Highway: - Vehicle Type UT distribution follows vehicle UE dropping as in Table A.1.2-1 from TR36.885. - RSU-type UTs are uniformly allocated with 100m spacing in the middle of the freeway as per TR36.885. For Urban grid: - Vehicle Type UT distribution follows vehicle UE dropping as in Table A.1.2-1 from TR36.885. - Pedestrian type UT, the dropping using equally spaced along the sidewalk with a fixed inter-pedestrian X m dropped per TR36.885 - Total number of pedestrian UEs is 16 in the centre grid. - Pedestrian UE is in the middle of the sidewalk - The inter-pedestrian UE distance (m) (i.e., X) is calculated by ‘A/16’, where ‘A’ is the total length of sidewalk where the pedestrian UEs are dropped under the assumption of ‘N’ road grids (i.e., ‘{(250m – 17m) + (433m – 17m)} * 2 * N’). For example, if the pedestrian UEs are dropped in ‘14’ road grids, the inter-pedestrian UE distance (m) is ‘36.344’. - N=1; - RSU-type UT: the dropping is at the center of intersection per TR36.885. NOTE: A single UT type is used per calibration, e.g., pedestrian type UT, RSU type UT, or vehicle type UT .. Sensing target distribution; Per TR37.885: - Option A - Vehicle type distribution: 100% vehicle type 2. - Clustered dropping is not used. - Highway: one target uniformly distributed (across multiple drops) within the simulation region. Vehicle speed is 140 km/h in all the lanes as baseline. - Urban Grid: one target is uniformly distributed (across multiple drops) within the center road grid. Vehicle speed is 60 km/h in all the lanes as baseline. NOTE: vehicle is dropped with 5 scattering points (front/left/right/back/roof) and each point has one location, or vehicle is dropped with 1 scattering points. In the case of vehicle with 5 scattering points, spatial consistency is enabled with the following assumptions: - The correlation for LOS/NLOS condition of the 5 points is assumed equal to 1. LOS/NLOS condition can be calculated based on the distance of the Tx/Rx to the centroid of the ST, then apply the LOS/NLOS condition to each of the 5 points. - The correlation for stochastic cluster paths of the 5 points is assumed equal to 1. The stochastic cluster paths can be calculated between the Tx/Rx and the centroid of the ST, then the stochastic cluster paths are added to each of the 5 points.. RCS for each scattering point; Based on the agreed RCS.. Minimum 3D distances between pairs of Tx/Rx and sensing target; 10 m. Minimum 3D distance between sensing targets; 10 m. Wrapping Method; As defined in urban grid/highway scenario. Fast fading model (full calibration); Procedures based on 37.885. (u, std) for XPR of target; (21.12, 6.88) dB.. The power threshold for path dropping after concatenation for target channel; [-40dB] FFS: Other power thresholds.. The power threshold for removing clusters in step 6 in section 7.5, TR 38.901 for background channel; -25dB. Coupling loss for target channel; By definition, need to consider all direct and indirect paths. The following parameters are included in the calculation: - power scaling factor (pathloss, shadow fading, and RCS component A included) - for small scale RCS B1/B2 and power of rays in Tx-target/target-Rx links (Pn',m',n,mk,p), Tx/Rx antenna pattern, 3 polarization matrixes, i.e., Pn',m',n,mk,pFrx,u,θθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pFrx,u,ϕθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pTCPMrx,n',m'k,pCPMn',m',n,mk,pCPMtx,n, mk,pd112+d2222 ∙Ftx,s,θθtx,n,m,ZODk,p,ϕtx,n,m,AODk,pFtx,s,ϕθtx,n,m,ZODk,p,ϕtx,n,m,AODk,p. Sensing Tx/Rx selection; Best N= Tx-Rx pairs to be selected for the target. For urban grid N = 4 For Highway N = 4 NOTE: Based on the Tx-Rx pair with the smallest power scaling factor of the target channel.. Absolute delay; The model of UMa scenario defined in TR 38.901 7-24GHz channel modelling [Table 7.6.9-1] is reused for Highway (FR2)/Urban Grid for all sensing modes. The model of RMa scenario defined in TR 38.901 7-24GHz channel modelling [Table 7.6.9-1] is reused for Highway (1732m ISD) for all sensing modes.. Metrics; Coupling loss for target channel Coupling loss for background channel (in case of monostatic sensing, this is the linear sum of the coupling loss between Tx/Rx and all reference points) Note: CDFs can be separately generated for target channel, background channel CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA). Definition of Delay Spread is similar to the definition of angle spread in Annex A of TR 25.996, Definition of Angle Spread can ref to Annex A of TR 25.996.. - Proposal 5-3-1 Proposal 5-3-1: For the purposes of large scale calibration for Human (indoor and outdoor) sensing targets, the following calibration parameters are proposed below in Table x. Table x. Simulation assumptions for large scale calibration for Human sensing targets Parameters; Indoor Values; Outdoor Values. Scenario; Indoor office 12 sectors per 120m * 50m * 3m ISD = 20m InF-SH Hall size: 300x150 m Room height: 10m 18 BSs on a square lattice with spacing D, located D/2 from the walls. -big hall (L=300m x W=150m): D=50m; UMa (ISD = 500m), UMi (ISD= 200m). Sensing mode; TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic, UE monostatic. Target type; Adult Pedestrian: 0.5m x 0.5m x 1.75m. Note: Height of scattering point 1.5m. Sectorization; Single 360-degree sector can be assumed; Single 360-degree sector can be assumed. Carrier Frequency; FR1: 6 GHz FR2: 30 GHz; FR1: 6 GHz FR2: 30 GHz. BS antenna configurations; Single dual-pol isotropic antenna; Single dual-pol isotropic antenna. BS Tx power; FR1: 24dBm FR2: 23dBm; FR1: 56dBm FR2: 41dBm. Bandwidth; FR1: 100MHz FR2: 400MHz; FR1: 100MHz FR2: 400MHz. BS noise figure; FR1: 5dB FR2: 7dB; FR1: 5dB FR2: 7dB. UT antenna configurations; Single dual-pol isotropic antenna; (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1); Single dual-pol isotropic antenna; (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1). UT noise figure; FR1: 9dB FR2: 10dB; FR1: 9dB FR2: 10dB. UT height; 1m; 1.5m. UT Tx power; 23dBm; 23dBm. UT Distribution; Per Table 7.8-1 Indoor-Office Number of UTs: 20; Per Table 7.8-1. Number of UTs/cell: 10. Sensing target distribution; 100% indoor, 1 target uniformly distributed (across multiple drops) over the convex hull of the TRP deployment; 100% outdoor. 1 target uniformly distributed (across multiple drops) within the center cell.. Component A of the RCS for each scattering point; -1.37 dBsm; -1.37 dBsm. Minimum 3D distances between pairs of Tx/Rx and sensing target; Min distances defined in TR 38.901 and TR36.843 and TR38.859; Min distances defined in TR 38.901 and TR36.843 and TR38.859. Wrapping Method; N/A; No wrapping method is used if interference is not modelled, otherwise geographical distance based wrapping. Coupling loss for target channel; Power scaling factor (pathloss, shadow fading, and RCS component A included) LTX-SPST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2. Sensing Tx/Rx selection; Best N = 4 Tx-Rx pairs to be selected for the target. Based on Tx-Rx pair with the smallest power scaling factor of the target channel.; Best N = 4 Tx-Rx pairs to be selected for the target. Based on Tx-Rx pair with the smallest power scaling factor of the target channel.. Metrics; Coupling loss for target channel Coupling loss for background channel (in case of monostatic sensing, this is the coupling loss between Tx and one reference point) Note: CDFs can be separately generated for target channel, background channel. - Proposal 5-4-2 Proposal 5-4-2: For the purposes of full calibration for Human sensing targets, the following calibration parameters are proposed below in Table x. Table x. Simulation assumptions for full calibration for Human sensing targets Parameters; Indoor Values; Outdoor Values. Scenario; Indoor office 12 sectors per 120m * 50m * 3m ISD = 20m InF-SH Hall size: 300x150 m Room height: 10m 18 BSs on a square lattice with spacing D, located D/2 from the walls. -big hall (L=300m x W=150m): D=50m; UMa (ISD = 500m), UMi (ISD= 200m). Sensing mode; TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic, UE monostatic. Target type; Adult Pedestrian: 0.5m x 0.5m x 1.75m Note: Height of scattering point 1.5m. Sectorization; Single 360-degree sector can be assumed; Single 360-degree sector can be assumed. Carrier Frequency; FR1: 6 GHz FR2: 30 GHz; FR1: 6 GHz FR2: 30 GHz. BS antenna configurations; Single dual-pol isotropic antenna; Single dual-pol isotropic antenna. BS Tx power; FR1: 24dBm FR2: 23dBm; FR1: 56dBm FR2: 41dBm. Bandwidth; FR1: 100MHz FR2: 400MHz; FR1: 100MHz FR2: 400MHz. BS noise figure; FR1: 5dB FR2: 7dB; FR1: 5dB FR2: 7dB. UT antenna configurations; Single dual-pol isotropic antenna; (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1); Single dual-pol isotropic antenna; (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1). UT noise figure; FR1: 9dB FR2: 10dB; FR1: 9dB FR2: 10dB. UT height; 1 m; 1.5 m. UT Tx power; 23dBm; 23dBm. UT Distribution; Per Table 7.8-2 Indoor-Office Number of UTs: 20; Per Table 7.8-2 Number of UTs/cell: 10. Sensing target distribution; 100% indoor, 1 target uniformly distributed (across multiple drops) over the convex hull of the TRP deployment; 100% outdoor. 1 target uniformly distributed (across multiple drops) within the center cell.. Minimum 3D distances between pairs of Tx/Rx and sensing target; Min distances defined in TR 38.901 and TR36.843 and TR38.859; Min distances defined in TR 38.901 and TR36.843 and TR38.859. RCS for each scattering point; Component A: -1.37 dBsm Component B1: 0 dB Component B2: 3.94 dB The same values are used for monostatic RCS and bistatic RCS; Component A: -1.37 dBsm Component B1: 0 dB Component B2: 3.94 dB The same values are used for monostatic RCS and bistatic RCS. Fast fading model; For BS to UE link: Follow the procedure defined in TR38.901 For BS to target link: Follow the procedure defined in TR38.901 for target to UE link: Follow the procedure defined in 38.858; For BS to UE link: Follow the procedure defined in TR38.901 For BS to target link: Follow the procedure defined in TR38.901 for target to UE link: Follow the procedure defined in 38.858. (u, std) for XPR of target; FFS; FFS. The power threshold for path dropping after concatenation for target channel; [-40dB] FFS: Other power thresholds.; [-40dB] FFS: Other power thresholds.. The power threshold for removing clusters in step 6 in section 7.5, TR 38.901 for background channel; [-25dB] FFS: Other power thresholds.; [-25dB] FFS: Other power thresholds.. Coupling loss for target channel; By definition, need to consider all direct and indirect paths. The following parameters are included in the calculation: - power scaling factor (pathloss, shadow fading, and RCS component A included) - for small scale RCS B1/B2 and power of rays in Tx-target/target-Rx links (Pn',m',n,mk,p), Tx/Rx antenna pattern, 3 polarization matrixes, i.e., Pn',m',n,mk,pFrx,u,θθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pFrx,u,ϕθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pTCPMrx,n',m'k,pCPMn',m',n,mk,pCPMtx,n, mk,p CPMrx,n',m'k,pCPMn',m',n,mk,pCPMtx,n, mk,pd112+d2222∙Ftx,s,θθtx,n,m,ZODk,p,ϕtx,n,m,AODk,pFtx,s,ϕθtx,n,m,ZODk,p,ϕtx,n,m,AODk,p. Tx-Rx pairing; Best N = 4 Tx-Rx pairs to be selected for the target. NOTE1: Based on the Tx-Rx pairs with the smallest power scaling factor of the target channel.. Absolute delay; The model of InH, InF-SH scenarios defined in TR 38.901 7-24GHz channel modeling [Table 7.6.9-1].; The model of UMa/UMi scenario defined in TR 38.901 7-24GHz channel modeling [Table 7.6.9-1].. Metrics; Coupling loss for target channel Coupling loss for background channel (in case of monostatic sensing, this is the linear sum of coupling losses between Tx/Rx and all reference points) Note: CDFs can be separately generated for target channel, background channel CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA). Definition of Delay Spread is similar to the definition of angle spread in Annex A of TR 25.996, Definition of Angle Spread can ref to Annex A of TR 25.996.. - Proposal 6-2-1 Proposal 6-2-1: For the purposes of large scale calibration for AGV sensing targets, the following calibration parameters are proposed below in Table x. Table x. Simulation assumptions for large scale calibration for AGV indoor sensing targets (full calibration) Parameters; Values. Scenario; InF: 18 TRPs per the big hall: L = 300 m x W = 150 m - X-axis is pointing down to the floor - The antenna array is mounted in the Y-Z plane with boresight along the X-axis - The X-axis/Y-axis/Z-axis refer to LCS - 8 m for high BS scenario. Sensing mode; TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic, UE monostatic. Target type; Option 2: 1.5m x 3.0m x 1.5m. Sectorization; Single 360-degree sector can be assumed. Carrier Frequency; FR1: 6 GHz FR2: 30 GHz. BS antenna configurations; Single dual-pol isotropic antenna. BS Tx power; FR1: 24dBm FR2: 23dBm. Bandwidth; FR1: 100MHz FR2: 400MHz. BS noise figure; FR1: 5dB FR2: 7dB. UT antenna configurations; Single dual-pol isotropic antenna; (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1). UT noise figure; FR1: 9dB FR2: 10dB. UT height; 1.5m. UT Tx power; 23dBm. UT Distribution; Per Table 7.8-7 Indoor Factory. Number of UTs: 30. Sensing target distribution; 100% indoor, 1 target uniformly distributed (across multiple drops) over the convex hull of the TRP deployment. Component A of the RCS for each scattering point; -1.37 dBsm Note: For calibration purposes, other value(s) are not precluded.. Coupling loss for target channel; Power scaling factor (pathloss, shadow fading, and RCS component A included) LTX-SPST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2. Tx-Rx pairing; Best N = 4 Tx-Rx pairs to be selected for the target. Based on Tx-Rx pair with the smallest power scaling factor of the target channel.. Metrics; Coupling loss for target channel Coupling loss for background channel (in case of monostatic sensing, this is the coupling loss between Tx and one reference point) Note: CDFs can be separately generated for target channel, background channel. - Proposal 6-3-2 Proposal 6-3-2: For the purposes of full calibration for AGV sensing targets, the following calibration parameters are proposed below in Table x. Table x. Simulation assumptions for full calibration for AGV indoor sensing targets (full calibration) Parameters; Values. Scenario; InF: 18 TRPs per the big hall: L = 300 m x W = 150 m - X-axis is pointing down to the floor - The antenna array is mounted in the Y-Z plane with boresight along the X-axis - The X-axis/Y-axis/Z-axis refer to LCS - 8 m for high BS scenario. Sensing mode; TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic, UE monostatic. Target type; Option 2: 1.5m x 3.0m x 1.5m. Sectorization; Single 360-degree sector can be assumed. Carrier Frequency; FR1: 6 GHz FR2: 30 GHz. BS antenna configurations; Single dual-pol isotropic antenna. BS Tx power; FR1: 24dBm FR2: 23dBm. Bandwidth; FR1: 100MHz FR2: 400MHz. BS noise figure; FR1: 5dB FR2: 7dB. UT antenna configurations; Single dual-pol isotropic antenna; (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1). UT noise figure; FR1: 9dB FR2: 10dB. UT height; 1.5m. UT Tx power; 23dBm. UT Distribution; Per Table 7.8-7 Indoor-Factory. Number of UTs: 30. Sensing target distribution; 100% indoor, 1 target uniformly distributed (across multiple drops) over the convex hull of the TRP deployment. RCS for each scattering point; The values/pattern of component A*B1 are generated by the following parameters φcenter; φ3dB, n; θcenter; θ3dB,n; Gmax; σmax; Applicable Range of θ; Applicable Range of φ. Front; 0°; 13.68°; 90°; 13.68°; 13.02; 23.29; [30°,180°]; [45°,135°]. Left; 90°; 15.53°; 75°; 20.03°; 7.33; 17.60; [30°,180°]; [135°,225°]. Back; 180°; 12.49°; 90°; 11.89°; 11.01; 21.28; [30°,180°]; [225°,315°]. Right; 270°; 15.53°; 75°; 20.03°; 7.33; 17.60; [30°,180°]; [-45°,45°]. Roof; /; /; 0°; 11.44°; 11.79; 22.06; [0°,30°]; [0°,360°]. - When θ is in the range [0°,30° ), σHdB φ=0 The standard deviation of component B2 is 2.51 dB The same values are used for monostatic RCS and bistatic RCS. Fast fading model (Full calibration); For BS to UE link: Follow the procedure defined in TR38.901 For BS to target link: Follow the procedure defined in TR38.901 for target to UE link: Follow the procedure defined in TR 38.858. (u, std) for XPR of target; FFS. The power threshold for path dropping after concatenation for target channel; [-40dB] FFS: Other power thresholds.. The power threshold for removing clusters in step 6 in section 7.5, TR 38.901 for background channel; [-25dB] FFS: Other power thresholds.. Coupling loss for target channel; By definition, need to consider all direct and indirect paths. The following parameters are included in the calculation: - power scaling factor (pathloss, shadow fading, and RCS component A included) - for small scale RCS B1/B2 and power of rays in Tx-target/target-Rx links (Pn',m',n,mk,p), Tx/Rx antenna pattern, 3 polarization matrixes, i.e., Pn',m',n,mk,pFrx,u,θθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pFrx,u,ϕθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pTCPMrx,n',m'k,pCPMn',m',n,mk,pCPMtx,n, mk,p CPMrx,n',m'k,pCPMn',m',n,mk,pCPMtx,n, mk,pd112+d2222∙Ftx,s,θθtx,n,m,ZODk,p,ϕtx,n,m,AODk,pFtx,s,ϕθtx,n,m,ZODk,p,ϕtx,n,m,AODk,p. Tx-Rx pairing; Best N = 4 Tx-Rx pairs to be selected for the target. Based on Tx-Rx pair with the smallest power scaling factor of the target channel.. Absolute delay; The model of InF scenario defined in TR 38.901 7-24GHz channel modeling [Table 7.6.9-1] is reused for all sensing modes.. Metrics; Coupling loss for target channel Coupling loss for background channel (in case of monostatic sensing, this is the linear sum of coupling losses between Tx/Rx and all reference points) Note: CDFs can be separately generated for target channel, background channel CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA). Definition of Delay Spread is similar to the definition of angle spread in Annex A of TR 25.996, Definition of Angle Spread can ref to Annex A of TR 25.996.. - Proposal 8.1.1.1 Proposal 8-1-1-1: RAN1 may calibrate EO Type-2 for ISAC in Rel-19. Interested companies can provide results. For the purposes of EO Type-2 calibration, the following calibration parameters are proposed below in Table x. Table x. Simulation assumptions for calibration of EO type-2 Parameter; Values. Scenario; Urban grid. Cell layout; For FR1: ISD = 500m, the layout is defined as below: The BSs are placed at the top of buildings at one corner. Specifically, the road grids shall be shifted by (ISD3-10,32*ISD3-10) m in horizontal plane, or the BSs are shifted by (-ISD3+10,-32*ISD3+10)m in horizontal plane equivalently. For FR2 ISD=250m, the cell layout is as follows:. Sensing mode; TRP- pedestrian UE bistatic TRP- RSU type UE bistatic TRP monostatic TRP-TRP bistatic RSU type UE - RSU type UE bistatic (optional). EO deployment; 9 buildings with size 413m x 230m x 20m illustrated by the green blocks as in the figure shown in the row of cell layout.. ST distribution; one target is uniformly distributed (across multiple drops) within the center of the outside lane of the grid.. UT distribution; - For pedestrian UT - Pedestrian type UE, the dropping using equally spaced along the sidewalk with a fixed inter-pedestrian X m dropped per TR36.885. - Total number of pedestrian UEs is 16 in the centre grid. - Pedestrian UE is in the middle of the sidewalk - The inter-pedestrian UE distance (m) (i.e., X) is calculated by ‘A/16’, where ‘A’ is the total length of sidewalk where the pedestrian UEs are dropped under the assumption of ‘N’ road grids (i.e., ‘{(250m – 17m) + (433m – 17m)} * 2 * N’). For example, if the pedestrian UEs are dropped in ‘14’ road grids, the inter-pedestrian UE distance (m) is ‘36.344’. - N=1; - For RSU type UT - The dropping is at the center of intersection per TR36.885.. EM-parameters; Table 7.6.8-1 Material properties [TR38.901] Material class; Relative permittivity; Conductivity; Frequency range in [GHz]. Concrete; 5.31; 0; 0.0326; 0.8095; 1-100.. Metrics; CDF curves: - Full coupling loss: calculate the coupling loss for each Tx-EO-ST-LOS-Rx and Tx-LOS-ST-EO-Rx rays. - CDF of the Delay. - CDF of the AoA, AoD, ZoA, ZoD. Additional CDF curves: - Coupling loss for target channel - CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA). Definition of Delay Spread is similar to the definition of angle spread [TR 25.996, Annex A].. Note: Other necessary but unspecified parameters in this table are the same as those in the table for urban grid scenario calibration.. - Proposal 8-1-2 Proposal 8-1-2: RAN1 may calibrate spatial consistency for ISAC in Rel-19. Interested companies can provide results. For the purposes of spatial consistency calibration, the following calibration parameters are proposed below in Table x. Table x. Simulation assumptions for calibration of spatial consistency Parameter; Values. Scenario; Urban grid, Indoor office. Cell layout; Urban grid: ISD = 500m, the same layout with 37.885 Indoor office: Office size (WxLxH): 120mx50mx3m, ISD=20m (refer to TR 38.901). Sensing mode; TRP- pedestrian UE bistatic in urban grid and TRP-UE bistatic in indoor office. BS antenna configuration; Single dual-pol isotropic antenna. UT antenna configuration; (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1). Polarized antenna model; Model-2 in clause 7.3.2 in TR 38.901. Scattering point; Single point. Calibration link and calibration method; - Calibration link: same target associated to different UT as following figure - Calibration method Step1: In single drop simulation, drop one Target and multiple UTs. Step2: Generate the channel of each link based on Target level spatial consistency method. Step3: Determine Target-UT groups based on the distance between UTs. Step4: Bin all the link pairs into certain distance groups, e.g., - 0m <= the location distance of link pair < 1m -> 0m group - 1m <= the location distance of link pair < 2m -> 1m group - 2m <= the location distance of link pair < 3m -> 2m group - 3m <= the location distance of link pair < 4m -> 3m group …… - (N)m <= the location distance of pair < (N+1)m -> (N)m group Note: N is at least twice the maximum correlation distance associated with the channel parameters in the scenario Step5: Calculate the correlation coefficient metric of each distance groups. Step6: Draw the x-y cure, where x=0, …, N, y= correlation coefficient metric related to (x)m group. ST and UT distribution; - Urban grid one target is uniformly dropped within the center grid in the outside lane with length of 433m, the UT is distributed by a uniform distribution distance of [0,1]m in the walk road where is in the same street with the target. As following figure. Note: The ST-UT link only consider LOS condition discarding NLOSv condition. - Indoor office 1 Target is uniform dropped in office region 10 UTs are dropped uniformly per cell, i.e., totally 120 UTs are uniformly dropped in office region.. Metrics; - Cross-correlation coefficient of delay for the third cluster of channel between link pairs. - Cross-correlation coefficient of AOA (for UT) for the third cluster of channel between link pairs. - Cross-correlation coefficient of LOS/NLOS status of channel between link pairs(only for indoor office scenario).. Note: Other necessary but unspecified parameters in this table are the same as those in the table for urban grid scenario calibration and indoor office scenario calibration.. - Proposal 8-3 Proposal 8-3: For the purposes of full calibration for UAV sensing targets, the following update is proposed below based on the agreement from RAN1#120-bis for Coupling loss for target channel: Coupling loss for target channel; By definition, need to consider all direct and indirect paths. The following parameters are included in the calculation: - power scaling factor (pathloss, shadow fading, and RCS component A included) - for small scale RCS B1/B2 and power of rays in Tx-target/target-Rx links (Pn',m',n,mk,p), Tx/Rx antenna pattern, 3 polarization matrixes, i.e., Pn',m',n,mk,pFrx,u,θθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pFrx,u,ϕθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pTCPMrx,n',m'k,pCPMn',m',n,mk,pCPMtx,n, mk,p CPMrx,n',m'k,pCPMn',m',n,mk,pCPMtx,n, mk,pd112+d2222∙Ftx,s,θθtx,n,m,ZODk,p,ϕtx,n,m,AODk,pFtx,s,ϕθtx,n,m,ZODk,p,ϕtx,n,m,AODk,p.